Stage corrections KRW04 - Jack Lewis

Stage corrections - Jack Lewis

plot.or("krw",or)

Removed two large unexplained outliers

krw04.or <- krw04.or[-c(18,22),]

plot.or("krw",04)

plot.or("krw",04,x="stage")

plot.or("krw",04,x="stage",linear=T)

Coefficients:

Estimate Std. Error t value Pr(>|t|)

(Intercept)  0.005172   0.006467   0.800    0.431

elecstg     -0.009052   0.001136  -7.969  1.9e-08 ***

The trend had not been taken care of in the adjusted file I received:

plot.or("krw",04,x="stage",cor=T,reread=F)

This calls for starting over with stage-proportional adjustments.

0.00517-0.009052*(1:17)

[1] -0.003882 -0.012934 -0.021986 -0.031038 -0.040090 -0.049142 -0.058194

[8] -0.067246 -0.076298 -0.085350 -0.094402 -0.103454 -0.112506 -0.121558

[15] -0.130610 -0.139662 -0.148714

Corrected from 0 at 1 foot to -0.14 at 16 feet

07apr2004:0550 to 26apr2004:1540 reconstructed from SFM using linear ratio.

I then applied two stage proportional adjustments from 20apr:1200 to the OR

on 18may, and from the OR on 18may to 26apr2004:1540.  I didn't start on

07apr because that warped the recession in an unnatural way.

The result is most errors are now less than 0.04 and trend is gone:

plot.or("krw",04,x="stage",cor=T)

krw04.or <- krw04.or[-c(18,22),]

plot.or("krw",04,x="stage",cor=T,reread=F)

plot.or("krw",04,cor=T)

Turbidity corrections - Jack Lewis

There is much bad turbidity data this year.  Clark attempted to reconstruct

much of it, but mostly with little basis.  Mostly what he did is reasonable

but extremely speculative.  I've replaced most of it with missing.

15Nov2003:1730 to 06Dec2003:0300

replaced bad with missing (Clark had reconstructed most of this by freehand)

18Dec2003:1620 to 21Dec2003:1320

replaced bad with missing (Clark had adjusted)

30Dec2003:0010 to 01Jan2003:0200

reconstructed from SFM using linear ratio then adjusted several times; this is going out a bit on a limb;  retained a couple of the spikes but this is also speculative.

02Jan to 03Jan: accepted Clark's reconstruction

10Jan2003:0730 to 23Jan2004:1840

replaced bad with missing (Clark had fabricated it)

26Jan2004:1030 to 27Jan2004:1200

replaced bad with missing (Clark had reconstructed)

08Feb to 16Feb: accepted Clark's reconstruction

19Feb2004:0340 to 22Feb2004:1110

redid Clark's reconstruction using SFM linear method

27Feb2004:2120 to 29Feb2004:1330

Redid Clark’s reconstruction using multiple interpolation

03Mar2004:0920 to 25Mar2004:1310

replaced bad with missing (Clark had reconstructed it)

26Mar2004:0620 to 27Mar2004:1200

replaced Clark's reconstruction with raw coded questionable

27Mar2004:1210 to end of year

replaced bad with missing (Clark had fabricated much of this)