FTR 2007 Annual SAND and FINES load estimation

ftr07.flo <- read.flo("ftr",07)

ftr07.lab <- read.sand("ftr",07)

ftr07.sed <- merge.flo.sand("ftr",07)

# Now create the ssc column: it will represent only FINES

ftr07.sed$ssc <- ftr07.sed$totssc - ftr07.sed$sand

# Since it's named ssc, we can use existing functions to calc the fines load

head(ftr07.sed,6)

chr dump bottle   sand totssc turb turbcode     q labcode1 labcode2    ssc

1 (12/13/06 10:10:00)    5     10 109.76 237.87  220        0 283.7        0        0 128.11

2 (12/13/06 10:50:00)    5     11 208.30 480.81  345        0 332.2        0        0 272.51

3 (12/13/06 12:10:00)    5     12 115.72 435.82  361        0 347.0        0        0 320.10

4 (12/13/06 13:00:00)    5     13  88.98 274.35  290        0 349.3        0        0 185.37

5 (12/13/06 14:30:00)    5     14  48.80 192.98  211        0 324.4        0        0 144.18

6 (12/13/06 15:40:00)    5     15  42.98 164.59  150        0 296.1        0        0 121.61

summary(ftr07.sed$ssc)

Min. 1st Qu.  Median    Mean 3rd Qu.    Max.

0.0   156.3   230.7   277.1   358.0   772.7

dim(ftr07.flo)

[1] 32271     9

table(ftr07.flo$turbcode)

0     2     5

27688    46  4537

Only 46 bad values of questionable turbidity and none bad out of 27668 intervals to be estimated.  These occurred on Nov 10 and Nov 11 during an early season flush

trb07.flo <- ftr07.flo[!is.na(ftr07.flo$turb), ]

flo07.flo <- ftr07.flo[is.na(ftr07.flo$turb), ]

# Develop model for good turb data

range(trb07.flo$turb)

[1]    0 684

table(ftr07.sed$labcode2)

0  1  4  9

41  5  2  1

# 4=spilled < 15% of sediment, 9=spilled > 15% of sediment

# The 9 is of possible concern

# From FTR07 Annual Load.rtf none of the bots with labcode 4 or 9 are outliers, keep them

trb07.sed <- ftr07.sed

turbsscplot("trb",07,type="loess")

# goes negative at low turbidities, eliminate the zero concentrations

trb07.sed <- ftr07.sed[ftr07.sed$ssc > 0, ]

four()

turbsscplot("trb",07,txt="dump",type="linear")

turbsscplot("trb",07,txt="dump",type="loess")

turbsscplot("trb",07,txt="dump",type="power")

turbsscplot("trb",07,txt="dump",type="logxy")

# Linear and loess go negative at low turbidities; don't even bother with them

turb1 <- turbsrc("trb",07,060801,0000,070731,2400,type="power",var=F)

turb2 <- turbsrc("trb",07,060801,0000,070731,2400,type="logxy",var=F)

total(turb1,turb2)

start            end       type surr    yhat  n    r2      s cv

turb1 10/29/06 12:10 06/10/07 14:30      power turb 1787945 47 0.907 51.561 NA

turb2 10/29/06 12:10 06/10/07 14:30 logxy:mvue turb 1855383 47 0.873  0.195 NA

# These differ by only 3.7%

# Power model wins on basis of better fit especially at upper end

start            end  type surr    yhat  n    r2      s cv

turb1 10/29/06 12:10 06/10/07 14:30 power turb 1787945 47 0.907 51.561 NA

Total fines load estimated by turbidity = 1787945 kg

# Now estimate the remaining 46 points using flow

# Range of flow for which SSC is be estimated, in cubic meters per second

range(flo07.flo$q)

[1] 0.9786 2.5910

# Range of flow in sample data

range(flo07.sed$q)

[1] 168.9 888.3

# The sample data are way out of the necessary range

# Earlier analysis estimated that only 37 kg of sediment was discharged during

# these 46 intervals, compared to 2613 metric tons for the year, so we can ignore it

Total fines load = 1787945 kg

# A very reliable estimate for fines

==================================================

# Now compute the total annual load based on this same smaller sample

# using only the 47 bottles that had sand fractions done

trb07.sed$ssc <- trb07.sed$totssc

four()

turbsscplot("trb",07,txt="dump",type="linear")

turbsscplot("trb",07,txt="dump",type="loess")

turbsscplot("trb",07,txt="dump",type="power")

turbsscplot("trb",07,txt="dump",type="logxy")

# This looks even better than the relationships with fines

# Again we can ignore the linear and loess models, which go negative

turb1 <- turbsrc("trb",07,060801,0000,070731,2400,type="power",var=F)

turb2 <- turbsrc("trb",07,060801,0000,070731,2400,type="logxy",var=F)

total(turb1,turb2)

start            end       type surr    yhat  n    r2      s cv

turb1 10/29/06 12:10 06/10/07 14:30      power turb 2595327 47 0.950 50.740 NA

turb2 10/29/06 12:10 06/10/07 14:30 logxy:mvue turb 2632680 47 0.927  0.144 NA

# The power and logxy modesl agree well, with the power model being slightly superior

total(turb1)

start            end  type surr    yhat  n   r2     s cv

turb1 10/29/06 12:10 06/10/07 14:30 power turb 2595327 47 0.95 50.74 NA

Total load = 2595327 kg

# Again we can ignore the 46 intervals with missing turbidity

This total compares very well with the previous estimates:

2613382 based on sum of storms and inter-storm loads

2552487 based on one annual regression model using all 177 samples (those with ssc > 0)

==================================================

==================================================

Total load 2595327

Total fines 1787945

Total sand  807382

Percent sand = 31.1%

Percent fines= 68.9%