KRW 2007 Annual SAND and FINES load estimation

krw07.flo <- read.flo("krw",07)

krw07.lab <- read.sand("krw",07)

krw07.sed <- merge.flo.sand("krw",07)

# Now create the ssc column: it will represent only FINES

krw07.sed$ssc <- krw07.sed$totssc - krw07.sed$sand

# Since it's named ssc, we can use existing functions to calc the fines load

head(krw07.sed,6)

chr dump bottle   sand totssc turb turbcode     q labcode1 labcode2    ssc

1 (12/10/06 09:30:00)    7      5  10.81 112.57   94        0 102.9        0        0 101.76

2 (12/10/06 15:10:00)    7      7  20.43 221.53  194        0 219.5        0        0 201.10

3 (12/10/06 16:50:00)    7      8  10.85 228.60  217        0 203.9        0        0 217.75

4 (12/10/06 18:50:00)    7      9   9.25 164.11  151        0 178.4        0        0 154.86

5 (12/10/06 21:40:00)    7     10   4.09 102.75   99        0 150.8        0        0  98.66

6 (12/13/06 15:00:00)    8      3 109.08 262.24  180        0 314.4        0        0 153.16

table(krw07.flo$turbcode)

0     1     2     5     7

11290   119   165 19812     2

There are 165 values of bad turbidity and 119 questionable

out of 31388 intervals to be estimated.  There is a great deal of interpolated

turbidity data starting in late January.  The questionable data occurred on

Dec 22, Apr 12, and Apr 18 and are perhaps best estimated using discharge.

trb07.flo <- krw07.flo[krw07.flo$turbcode %in% c(0,5,7), ]

flo07.flo <- krw07.flo[krw07.flo$turbcode %in% c(1,2), ]

dim(trb07.flo)

[1] 31104     9

dim(flo07.flo)

[1] 284   4

31104 + 284

[1] 31388

# Develop model for good turb data

range(trb07.flo$turb)

[1]  -1 797

trb07.flo$turb[trb07.flo$turb < 0] <- 0

table(krw07.sed$labcode2)

0  1  4  9

56 20  1  1

# Just one bad lab code (9) to worry about

# (4=spilled < 15% of sediment, 9=spilled > 15% of sediment)

# Eliminate bad turbidity and labcode 9

trb07.sed <- krw07.sed[!is.na(krw07.sed$turb) & krw07.sed$labcode2 != 9, ]

range(trb07.sed$turb)

[1]  94 709

table(trb07.sed$turbcode)

0  5

76  1

two()

turbsscplot("trb",07,type="loess",txt="dump")

turbsscplot("trb",07,type="loess")

# Dump 10 is highly questionable because initial filter weights were lost and had to be estimated

# Dump 12: bots 17 and 21 were eliminated in estimating storm loads

# Eliminate outlier bottles in dumps 10 and 12 (rec nums 29,32,39,43,47)

trb07.sed <- trb07.sed[-c(29,32,39,43,47),]

four()

turbsscplot("trb",07,type="power",txt="dump")

turbsscplot("trb",07,type="logxy",txt="dump")

turbsscplot("trb",07,type="linear",txt="dump")

turbsscplot("trb",07,type="loess",txt="dump")

turb1 <- turbsrc("trb",07,060801,0000,070731,2400,type="power",var=F)

turb2 <- turbsrc("trb",07,060801,0000,070731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",07,060801,0000,070731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",07,060801,0000,070731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2      s cv

turb1 09/16/06 12:50 04/22/07 12:00      power turb 4445390 72 0.918 61.574 NA

turb2 09/16/06 12:50 04/22/07 12:00 logxy:mvue turb 4567410 72 0.815  0.316 NA

turb3 09/16/06 12:50 04/22/07 12:00      loess turb 3743102 72 0.917 62.248 NA

turb4 09/16/06 12:50 04/22/07 12:00     linear turb 3971349 72 0.912 63.739 NA

> sum(turb3$predssc < 0)

[1] 25785

> sum(turb4$predssc < 0)

[1] 27359

# Can't use loess or linear: too many negative predictions

# The power model or logxy model differ by 3%, but power model wins

total(turb1)

start            end  type surr    yhat  n    r2      s cv

turb1 09/16/06 12:50 04/22/07 12:00 power turb 4445390 72 0.918 61.574 NA

Total fines load by turbidity = 4445390 kg

# A fairly good estimate; may be biased low by dump 10's estimated filter weights

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo07.flo$q)

[1]  37.46 112.10

> range(krw07.sed$q)

[1]   39.14 1078.00

two()

qsscplot("krw",07,type="loess",txt="dump")

qsscplot("krw",07,type="loess")

The data to be estimated are below 112cfs = 3.17 cumecs.

# Limit the regression data to less than 10 cumecs, and labcodes != 9

flo07.sed <- krw07.sed[krw07.sed$q < 350 & krw07.sed$labcode2 != 5, ]

four()

qsscplot("flo",07,type="linear")

qsscplot("flo",07,type="loess")

qsscplot("flo",07,type="power")

qsscplot("flo",07,type="logxy")

flow1 <- flowsrc("flo",07,060801,0000,070731,2400,type="logxy")

flow2 <- flowsrc("flo",07,060801,0000,070731,2400,type="power")

flow3 <- flowsrc("flo",07,060801,0000,070731,2400,type="loess")

flow4 <- flowsrc("flo",07,060801,0000,070731,2400,type="linear")

total(flow1,flow2,flow3,flow4)

start            end       type surr     yhat  n    r2      s     cv

flow1 11/21/06 11:20 04/18/07 17:00 logxy:mvue flow 40661.83 26 0.079  0.643 25.298

flow2 11/21/06 11:20 04/18/07 17:00      power flow 34061.68 26 0.181 76.511 27.890

flow3 11/21/06 11:20 04/18/07 17:00      loess flow 34355.00 26 0.180 77.919     NA

flow4 11/21/06 11:20 04/18/07 17:00     linear flow 34972.24 26 0.182 76.470 26.969

> sum(flow3$predssc <0)

[1] 0

> sum(flow4$predssc <0)

[1] 0

#Based on CV, logxy model is best. Range of estimates is 19%

#The logxy model looks better

total(flow1)

start            end       type surr     yhat  n    r2     s     cv

flow1 11/21/06 11:20 04/18/07 17:00 logxy:mvue flow 40661.83 26 0.079 0.643 25.298

Total fines load by discharge = 40662 kg,  CV = 25.3 %

==================================================

# Total fines load

4445390 + 40662 = 4486052 kg

==================================================

# Compute total load based on the reduced sample (those with sand fractions only)

trb07.sed$ssc <- trb07.sed$totssc

flo07.sed$ssc <- flo07.sed$totssc

four()

turbsscplot("trb",07,type="power",txt="dump")

turbsscplot("trb",07,type="logxy",txt="dump")

turbsscplot("trb",07,type="linear",txt="dump")

turbsscplot("trb",07,type="loess",txt="dump")

turb1 <- turbsrc("trb",07,060801,0000,070731,2400,type="power",var=F)

turb2 <- turbsrc("trb",07,060801,0000,070731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",07,060801,0000,070731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",07,060801,0000,070731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2      s cv

turb1 09/16/06 12:50 04/22/07 12:00      power turb 5451446 72 0.934 61.066 NA

turb2 09/16/06 12:50 04/22/07 12:00 logxy:mvue turb 5466548 72 0.936  0.159 NA

turb3 09/16/06 12:50 04/22/07 12:00      loess turb 4798051 72 0.934 61.625 NA

turb4 09/16/06 12:50 04/22/07 12:00     linear turb 4941124 72 0.932 61.837 NA

> sum(turb3$predssc < 0)

[1] 24069

> sum(turb4$predssc < 0)

[1] 25627

total(turb1)

start            end  type surr    yhat  n    r2      s cv

turb1 09/16/06 12:50 04/22/07 12:00 power turb 5451446 72 0.934 61.066 NA

Total load by turbidity = 5451446 kg

four()

turbsscplot("flo",07,type="power",txt="dump")

turbsscplot("flo",07,type="logxy",txt="dump")

turbsscplot("flo",07,type="linear",txt="dump")

turbsscplot("flo",07,type="loess",txt="dump")

flow1 <- flowsrc("flo",07,060801,0000,070731,2400,type="logxy")

flow2 <- flowsrc("flo",07,060801,0000,070731,2400,type="power")

flow3 <- flowsrc("flo",07,060801,0000,070731,2400,type="loess")

flow4 <- flowsrc("flo",07,060801,0000,070731,2400,type="linear")

total(flow1,flow2,flow3,flow4)

start            end       type surr     yhat  n    r2       s     cv

flow1 11/21/06 11:20 04/18/07 17:00 logxy:mvue flow 47598.16 26 0.215   0.446 16.826

flow2 11/21/06 11:20 04/18/07 17:00      power flow 45516.01 26 0.167 104.142 28.455

flow3 11/21/06 11:20 04/18/07 17:00      loess flow 44305.95 26 0.172 105.749     NA

flow4 11/21/06 11:20 04/18/07 17:00     linear flow 46788.99 26 0.166 104.165 27.459

> sum(flow3$predssc <0)

[1] 0

> sum(flow4$predssc <0)

[1] 0

total(flow1)

start            end       type surr     yhat  n    r2     s     cv

flow1 11/21/06 11:20 04/18/07 17:00 logxy:mvue flow 47598.16 26 0.215 0.446 16.826

Total load by discharge = 47598 kg,  CV = 16.83 %

Total load based on reduced samples = 5451446 + 47598 = 5499044

This total compares very well with the previous estimate:

5634119 based on sum of storms and inter-storm loads

==================================================

==================================================

Total load 5499044

Total fines 4486052

Total sand  1012992

Percent sand = 18.4%

Percent fines= 81.6%