KRW03 event loads

krw03.flo <- read.flo("krw",03)

krw03.lab <- read.lab("krw",03)

krw03.sed <- merge.flo("krw",03)

KRW,301,021214,0100,021215,1630   427240 kg  Very good

KRW,302,021215,1640,021217,0330  1276531 kg   Very good

KRW,303,021219,0950,021224,1000   452624 kg  Fair

KRW,304,021227,0300,021230,0650  5168054 kg  Excellent

KRW,305,021230,0700,030103,0000  2688777 kg  Excellent

KRW,306,030113,1140,030115,1800   281519 kg  Good

KRW,307,030219,0300,030221,1800  1058095 kg  Poor

KRW,308,030313,0830,030317,1200   590748 kg  Good

KRW,309,030325,1610,030330,1200   543654 kg  Poor

KRW,310,030412,0020,030416,0300   371381 kg  Good

KRW,311,030423,1900,030425,0150   321101 kg  Fair

KRW,312,030425,0200,030427,1800  1189895 kg  Fair

KRW,313,030429,1730,030502,0000   340202 kg  Fair

Storm 301: dump 4: 021214,0100,021215,1630

Good turbidity and very good sample coverage after initial rise

One minor interpolation of turbidity at end of storm

Sampling in fixed-time mode

First sample of year has too high an SSC, most likely due to sediment sitting

in intake and an inadequate purge.  Eliminate it from analyses.

four()

turbsscplot("krw",03,dump=4,bot=5:17)

turbsscplot("krw",03,dump=4,bot=5:17,type="power")

turbsscplot("krw",03,dump=4,bot=5:17,type="logxy")

turbsscplot("krw",03,dump=4,bot=5:17,type="loess")

turb1 <- turbsrc("krw",03,021214,0100,021215,1630,dump=4,bot=5:17)

turb2 <- turbsrc("krw",03,021214,0100,021215,1630,dump=4,bot=5:17,type="power")

turb3 <- turbsrc("krw",03,021214,0100,021215,1630,dump=4,bot=5:17,type="logxy")

turb4 <- turbsrc("krw",03,021214,0100,021215,1630,dump=4,bot=5:17,type="loess")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s    cv

turb1 12/14/02 01:00 12/15/02 16:30     linear turb 424752.9 12 0.996 18.123 1.397

turb2 12/14/02 01:00 12/15/02 16:30      power turb 427240.3 12 0.994 20.716 1.550

turb3 12/14/02 01:00 12/15/02 16:30 logxy:mvue turb 426757.6 12 0.997  0.048 1.759

turb4 12/14/02 01:00 12/15/02 16:30      loess turb 426239.4 12 0.996 17.901    NA

# turb1 and turb4 each have just 4 negative predicted SSC values at the start

ttsplot("krw",03,turb2)

start            end  type surr     yhat  n    r2      s   cv

turb2 12/14/02 01:00 12/15/02 16:30 power turb 427240.3 12 0.994 20.716 1.55

Total load = 427240 kg,  CV = 1.55 %  Very good

Storm 302: dump 5: KRW,302,021215,1640,021217,0330

Excellent turbidity; Sample coverage is good on first peak, but there's a

20-hr gap between samples 10 and 11, 2nd peak;  On fixed-time mode sampling.

Need to include an extra bottle after defined end of storm

four()

turbsscplot("krw",03,dump=5,bot=1:11)

turbsscplot("krw",03,dump=5,bot=1:11,type="power")

turbsscplot("krw",03,dump=5,bot=1:11,type="logxy")

turbsscplot("krw",03,dump=5,bot=1:11,type="loess")

Loess is the only one that captures the slight curvature

turb1 <- turbsrc("krw",03,021215,1640,021217,0330,dump=5,bot=1:13)

turb2 <- turbsrc("krw",03,021215,1640,021217,0330,dump=5,bot=1:13,type="power")

turb3 <- turbsrc("krw",03,021215,1640,021217,0330,dump=5,bot=1:13,type="logxy")

turb4 <- turbsrc("krw",03,021215,1640,021217,0330,dump=5,bot=1:13,type="loess")

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2      s    cv

turb1 12/15/02 16:40 12/17/02 03:30     linear turb 1261671 13 0.990 36.403 1.797

turb2 12/15/02 16:40 12/17/02 03:30      power turb 1260753 13 0.987 42.184 2.264

turb3 12/15/02 16:40 12/17/02 03:30 logxy:mvue turb 1270674 13 0.969  0.164 5.492

turb4 12/15/02 16:40 12/17/02 03:30      loess turb 1276531 13 0.995 26.453    NA

ttsplot("krw",03,turb4)

start            end  type surr    yhat  n    r2      s cv

turb4 12/15/02 16:40 12/17/02 03:30 loess turb 1276531 13 0.995 26.453 NA

Total load = 1276531 kg   Very good

Storm 303: dump 5-6: KRW,303,021219,0950,021224,1000

Two peaks; second peak much larger

Turbidity was reconstructed by Clark on rising limb of first peak

and latter half of recession of second peak, based on Hach turbidities

four()

turbsscplot("krw",03,021219,0950,021224,1000)

qsscplot("krw",03,021219,0950,021224,1000)

# Sample 24 has too low SSC relative to both turbidity and discharge.

# Eliminate it.  Must be an ISCO problem.

turbsscplot("krw",03,021219,0950,021224,1000,dump=rep(5:6,c(13,1)),bot=c(22,23,25:35,1))

turbsscplot("krw",03,dump=4:10)

Dump 5 is anomalous relative to 4,6,7,8,9, and 10

I'm not sure we can trust the turbidity reconstructions.

turbsscplot("krw",03,dump=5,bot=c(23,25:28),type="loess")

Sample 22 has SSC=43.5, Sample 2 on 12/25 has SSC = 32

time1 <- lineartime("krw",03,021219,0950,021219,1530,ssc1=43)

turb1 <- turbsrc("krw",03,021219,1540,021220,1120,dump=5,bot=c(23,25:28))

time2 <- cubictime("krw",03,021220,1130,021224,1000,ssc2=36)

ttsplot("krw",03,time1,turb1,time2)

#The last piece is reliable. The rest is speculative.

total(time1,turb1,time2)

start            end   type surr      yhat n    r2       s     cv

time1 12/19/02 09:50 12/19/02 15:30 linear time  16111.92 1    NA      NA     NA

turb1 12/19/02 15:40 12/20/02 11:20 linear turb 198209.07 5 0.699 105.079 12.451

time2 12/20/02 11:30 12/24/02 10:00 linear time 238302.63 9    NA      NA     NA

Total load = 452624 kg   Fair

Storm 304: dump 6: KRW,304,021227,0300,021230,0650

Turbidity data are good.  Clark reconstructed a bit at the start of rise and

interpolated a section at the end.  Excellent sample coverage except last bit.

ISCO still on fixed time sampling.  Need to include sample 2 before start

four()

turbsscplot("krw",03,dump=6,bot=2:25)

turbsscplot("krw",03,dump=6,bot=2:25,type="power")

turbsscplot("krw",03,dump=6,bot=2:25,type="loess")

turbsscplot("krw",03,dump=6,bot=2:25,type="loess",degree=2)

# Only loess captures the curvature. We also have enough sample to use lineartime

turb1 <- turbsrc("krw",03,021227,0300,021230,0650,dump=6,bot=2:25,type="loess",degree=2)

start        end

-9.378863 162.770162

time1 <- lineartime("krw",03,021227,0300,021230,0650,ssc1=32,ssc2=163)

total(turb1,time1)

Warning: Objects are non-sequential: check for gaps and overlaps

start            end   type surr    yhat  n    r2      s cv

turb1 12/27/02 03:00 12/30/02 06:50  loess turb 5168054 24 0.984 68.152 NA

time1 12/27/02 03:00 12/30/02 06:50 linear time 5072082 23    NA     NA NA

ttsplot("krw",03,time1)

Hard to say which is better.  Difference is less than 2%

total(turb1)

start            end  type surr    yhat  n    r2      s cv

turb1 12/27/02 03:00 12/30/02 06:50 loess turb 5168054 24 0.984 68.152 NA

Total load = 5168054 kg  Excellent.

Storm 305: dump 6-7: KRW,305,021230,0700,030103,0000

Very good turbidity and sample coverage except for last part of recession

Minor reconstructions on last part of recession

ISCO still on time mode.

turbsscplot(krw",03,021230,0700,030103,0000)

turb1 <- turbsrc("krw",03,021230,0700,030103,0000)

ttsplot("krw",03,turb1)

total(turb1)

start     end   type surr    yhat  n   r2      s    cv

turb1 12/30/02 07:00 1/03/03 linear turb 2688777 27 0.99 46.657 1.223

Total load = 2688777 kg,  CV = 1.22 %  Excellent

Storm 306: dump 7: KRW,306,030113,1140,030115,1800

Very good turbidity and sample coverage; minor turbidity adjustment on recession

Need to include bottle 15 from just before start

turbsscplot("krw",03,dump=7,bot=15:24)

I'm going to assume sample 21 had an ISCO problem

four()

turbsscplot("krw",03,dump=7,bot=c(15:20,22:24))

turbsscplot("krw",03,dump=7,bot=c(15:20,22:24),type="power")

turbsscplot("krw",03,dump=7,bot=c(15:20,22:24),type="logxy")

turbsscplot("krw",03,dump=7,bot=c(15:20,22:24),type="loess")

turb1 <- turbsrc("krw",03,030113,1140,030115,1800,dump=7,bot=c(15:20,22:24))

turb2 <- turbsrc("krw",03,030113,1140,030115,1800,dump=7,bot=c(15:20,22:24),type="loess")

total(turb1,turb2)

start            end   type surr     yhat n    r2      s   cv

turb1 01/13/03 11:40 01/15/03 18:00 linear turb 276233.0 9 0.990 23.059 3.15

turb2 01/13/03 11:40 01/15/03 18:00  loess turb 281519.0 9 0.992 23.016   NA

ttsplot("krw",03,turb2)

Not too good a fit at the peak

start            end  type surr     yhat n    r2      s cv

turb2 01/13/03 11:40 01/15/03 18:00 loess turb 281519.0 9 0.992 23.016 NA

Total load = 281519 kg  Good

Storm 307: dump 11: KRW,307,030219,0300,030221,1800

Excellent turbidity but no samples!  The little peak of Feb 16 had a few samples,

but we have to go back to dump 6, Dec 31, to find turbidities this high.

turbsscplot("krw",03,txt="dump")

# Dumps 11-18 seem to have higher SSC for a given turbidity, so using dump 6

to characterize this event might underestimate the load. The only other

alternative would be to extrapolate. Turbidities in this event range to 550.

Starting with dump 11, ISCO was having volume problems, both high and low.

four()

turbsscplot("krw",03,dump=6,type="loess",degree=2)

turbsscplot("krw",03,dump=c(6,11))

turbsscplot("krw",03,dump=c(11,13),txt="dump",type="power")

turb1 <- turbsrc("krw",03,030219,0300,030221,1800,dump=6,type="loess",degree=2)

turb2 <- turbsrc("krw",03,030219,0300,030221,1800,dump=c(6,11))

turb3 <- turbsrc("krw",03,030219,0300,030221,1800,dump=c(11,13),type="power")

total(turb1,turb2,turb3)

start            end   type surr    yhat  n    r2       s    cv

turb1 02/19/03 03:00 02/21/03 18:00  loess turb 1005544 49 0.983  67.553    NA

turb2 02/19/03 03:00 02/21/03 18:00 linear turb 1058095 62 0.969  97.144 2.721

turb3 02/19/03 03:00 02/21/03 18:00  power turb 2321812 25 0.908 140.978 6.939

# Because of the volume problems starting at dump 11, prefer to use dump 6

# But the loess model leads to negative predictions

endpoints(turb1)

start       end

-16.94204  20.44454

ttsplot("krw",03,turb2)

total(turb2)

start            end   type surr    yhat  n    r2      s    cv

turb2 02/19/03 03:00 02/21/03 18:00 linear turb 1058095 62 0.969 97.144 2.721

Total load = 1058095 kg,  CV = 2.67 %  Poor

Storm 308: dumps 13-14: KRW,308,030313,0830,030317,1200

Excellent turbidity and sample coverage

Quality of SSC is questionable because of volume problems

Dump 13 bots 1-7 volumes high, 8-12 volumes low

four()

turbsscplot("krw",03,030313,0830,030317,1200)

turbsscplot("krw",03,030313,0830,030317,1200,type="power")

turbsscplot("krw",03,030313,0830,030317,1200,type="logxy")

turbsscplot("krw",03,030313,0830,030317,1200,type="loess")

turb1 <- turbsrc("krw",03,030313,0830,030317,1200,type="power")

turb2 <- turbsrc("krw",03,030313,0830,030317,1200,type="logxy",bias="qmle")

total(turb1,turb2)

start            end       type surr     yhat  n    r2       s     cv

turb1 03/13/03 08:30 03/17/03 12:00      power turb 578635.5 14 0.885 186.927 10.855

turb2 03/13/03 08:30 03/17/03 12:00 logxy:qmle turb 571684.9 14 0.910   0.377 13.086

ttsplot("krw",03,turb1)

# Poor fit to first peak

# Alternative: break into two parts

turb3 <- turbsrc("krw",03,030313,0830, 030314,2250,type="pairs")

turb4 <- turbsrc("krw",03,030314,2300, 030317,1200,dump=rep(13:14,c(8,2)),bot=c(5:12,1:2))

ttsplot("krw",03,turb3,turb4)

start            end   type surr      yhat  n    r2       s     cv

turb3 03/13/03 08:30 03/14/03 22:50  pairs turb  30936.19  5    NA      NA     NA

turb4 03/14/03 23:00 03/17/03 12:00 linear turb 559812.09 10 0.874 209.541 15.381

Total load = 590748 kg  Good (provided you trust the pumped samples)

Storm 309: dump 14-15: KRW,309,030325,1610,030330,1200

No samples until dump 15: very late on recession :-(

Bot 4 low volume, bot 5-10 high volume (full), water in ISCO base.

Good turbidity data for the most part.

Questionable spike at start of storm removed.

Recession was reconstructed by Clark by freehand/interpolation.

Need a regression that goes up to 350 NTU; unless we go back to dump 6,

we'll have to extrapolate somewhat.

four()

turbsscplot("krw",03,dump=11:18,txt="dump")

turbsscplot("krw",03,dump=11:18,txt="dump",type="power")

turbsscplot("krw",03,dump=14:18,txt="dump")

turbsscplot("krw",03,dump=14:18,txt="dump",type="power")

turb1 <- turbsrc("krw",03,030325,1610,030330,1200,dump=11:18,type="power")

turb2 <- turbsrc("krw",03,030325,1610,030330,1200,dump=14:18,type="power")

total(turb1,turb2)

start            end  type surr     yhat  n    r2      s    cv

turb1 03/25/03 16:10 03/30/03 12:00 power turb 664800.1 99 0.897 91.888 2.813

turb2 03/25/03 16:10 03/30/03 12:00 power turb 543653.7 74 0.914 49.371 3.081

Because of the volume problems and the concave extrapolation, I don't trust dump 13

ttsplot("krw",03,turb2)

start            end  type surr     yhat  n    r2      s    cv

turb2 03/25/03 16:10 03/30/03 12:00 power turb 543653.7 74 0.914 49.371 3.081

Total load = 543654 kg,  CV = 3.09 %  Poor

Storm 310: dumps 16-17: KRW,310,030412,0020,030416,0300

Good turbidity until late on recession which was reconstructed

Fairly good sample coverage, but a little sparse in a few places

Sample volume problems: bots 7, 10-14 low volume; bot 8 high volume.

# Include bottle 7 from before storm start

two()

turbsscplot("krw",03,dump=rep(16:17,c(8,2)),bot=c(7:14,1,2))

turbsscplot("krw",03,dump=rep(16:17,c(8,2)),bot=c(7:14,1,2),type="power")

turb1 <- turbsrc("krw",03,030412,0020,030416,0300,dump=rep(16:17,c(8,2)),bot=c(7:14,1,2))

turb2 <- turbsrc("krw",03,030412,0020,030416,0300,dump=rep(16:17,c(8,2)),bot=c(7:14,1,2),type="power")

total(turb1,turb2)

start            end   type surr     yhat  n    r2      s    cv

turb1 04/12/03 00:20 04/16/03 03:00 linear turb 375844.3 10 0.966 40.109 5.911

turb2 04/12/03 00:20 04/16/03 03:00  power turb 371381.2 10 0.976 34.179 4.973

endpoints(turb1)

start       end

-8.829484 44.132790

ttsplot("krw",03,turb2)

total(turb2)

start            end  type surr     yhat  n    r2      s    cv

turb2 04/12/03 00:20 04/16/03 03:00 power turb 371381.2 10 0.976 34.179 4.973

Total load = 371381 kg,  CV = 4.95 %  Good

Storm 311: dump 17: KRW,311,030423,1900,030425,0150

Good turbidity, only the initial rise was reconstructed

NO SAMPLES

four()

turbsscplot("krw",03,dump=16:17)

turbsscplot("krw",03,dump=rep(16:17,c(8,2)),bot=c(7:14,1,2),type="power")

turbsscplot("krw",03,dump=16:18)

turbsscplot("krw",03,dump=rep(16:18,c(14,21,6)),bot=c(1:14,1:21,1:6))

turb1 <- turbsrc("krw",03,030423,1900,030425,0150,dump=16:17)

turb2 <- turbsrc("krw",03,030423,1900,030425,0150,dump=rep(16:17,c(8,2)),bot=c(7:14,1,2),type="power")

turb3 <- turbsrc("krw",03,030423,1900,030425,0150,dump=16:18)

turb4 <- turbsrc("krw",03,030423,1900,030425,0150,dump=rep(16:18,c(14,21,6)),bot=c(1:14,1:21,1:6))

total(turb1,turb2,turb3,turb4)

start            end   type surr     yhat  n    r2      s    cv

turb1 04/23/03 19:00 04/25/03 01:50 linear turb 321038.1 23 0.964 31.885 2.902

turb2 04/23/03 19:00 04/25/03 01:50  power turb 321100.8 10 0.976 34.179 3.355

turb3 04/23/03 19:00 04/25/03 01:50 linear turb 341702.6 36 0.875 69.834 4.062

turb4 04/23/03 19:00 04/25/03 01:50 linear turb 338506.1 29 0.903 68.998 4.325

# Although there were no samples in this event, there is little sample-dependent

variation in the estimates.  Therefore I think we can use this storm in analyses

but which version?  Best to not use the dump 18 outliers.  Go with the model used

in storm 310

ttsplot("krw",03,turb2)

total(turb2)

start            end  type surr     yhat  n    r2      s    cv

turb2 04/23/03 19:00 04/25/03 01:50 power turb 321100.8 10 0.976 34.179 3.355

Total load = 321101 kg,  CV = 3.36 %  Fair

Storm 312: dump 18: KRW,312,030425,0200,030427,1800

Excellent turbidity but no samples until dump 18, late on recession

Samples 1 and 2 are outliers,  sample 2 had > 15% organics

Fortunately they are outliers in opposite directions, limiting influence.

turbsscplot("krw",03,030425,0200,030427,1800)

turbsscplot("krw",03,dump=17:18)

turbsscplot("krw",03,dump=16:18)

turb1 <- turbsrc("krw",03,030425,0200,030427,1800,dump=17:18)

turb2 <- turbsrc("krw",03,030425,0200,030427,1800,dump=16:18)

total(turb1,turb2)

start            end   type surr    yhat  n    r2      s    cv

turb1 04/25/03 02:00 04/27/03 18:00 linear turb 1179375 22 0.835 81.988 6.490

turb2 04/25/03 02:00 04/27/03 18:00 linear turb 1135561 36 0.875 69.834 4.317

ttsplot("krw",03,turb2)

# We can improve the fit to the recession (samples 3-6)

turb3 <- turbsrc("krw",03,030425,0200,030425,2210,dump=17:18)

turb4 <- turbsrc("krw",03,030425,2220,030427,1800)

ttsplot("krw",03,turb3,turb4)

total(turb3,turb4)

start            end   type surr     yhat  n    r2      s    cv

turb3 04/25/03 02:00 04/25/03 22:10 linear turb 935933.9 22 0.835 81.988 7.536

turb4 04/25/03 22:20 04/27/03 18:00 linear turb 253961.0  4 0.995 12.214 2.785

Total load = 1189895 kg,  CV = 6.03 %  Fair

Storm 313: dump 18: KRW,313,030429,1730,030502,0000

Very good turbidity, but only 3 samples

No notes about volume problems during this period.

four()

turbsscplot("krw",03,030429,1730,030502,0000)

# Include neighboring bottles to extend range

turbsscplot("krw",03,dump=18,bot=6:13)

# Exclude outlier bot 13

turbsscplot("krw",03,dump=18,bot=6:12)

turb1 <- turbsrc("krw",03,030429,1730,030502,0000)

turb2 <- turbsrc("krw",03,030429,1730,030502,0000,dump=18,bot=6:13)

turb3 <- turbsrc("krw",03,030429,1730,030502,0000,dump=18,bot=6:12)

total(turb1,turb2,turb3)

start     end   type surr     yhat n    r2      s    cv

turb1 04/29/03 17:30 5/02/03 linear turb 340201.5 3 0.999  1.999 0.710

turb2 04/29/03 17:30 5/02/03 linear turb 339142.2 8 0.512 40.135 6.559

turb3 04/29/03 17:30 5/02/03 linear turb 311989.5 7 0.872 22.511 4.623

Pretty good agreement

ttsplot("krw",03,turb1)

total(turb1)

start     end   type surr     yhat n    r2     s   cv

turb1 04/29/03 17:30 5/02/03 linear turb 340201.5 3 0.999 1.999 0.71

Total load = 340202 kg,  CV = 0.7 %  Fair (only 3 samples)