KRW 2008 Annual load estimation

krw08.flo <- read.flo("krw",08)

krw08.lab <- read.lab("krw",08)

krw08.sed <- merge.flo("krw",08)

nonstorms <- drop.storms("krw",08)

dim(nonstorms)

[1] 41891     4

head(nonstorms, 5)

chr     q turb turbcode

1 (08/19/07 06:20:00) 8.842   NA        2

2 (08/19/07 06:30:00) 8.842   NA        2

3 (08/19/07 06:40:00) 8.842   NA        2

4 (08/19/07 06:50:00) 8.842   NA        2

5 (08/19/07 07:00:00) 8.842   NA        2

> table(nonstorms$turbcode)

0     2     5

15261 15831 10799

15831 values of bad turbidity out of 41891 intervals to be estimated

trb08.flo <- nonstorms[!is.na(nonstorms$turb), ]

flo08.flo <- nonstorms[is.na(nonstorms$turb), ]

dim(trb08.flo)

[1] 20982     4

dim(flo08.flo)

[1] 20909   4

20982+20909

[1] 41891

# Develop model for good turb data

range(trb08.flo$turb)

[1]  -5 262

# The negative turbidity could be a problem with some model types

# Add 6 to eliminate 0 and negative values

trb08.flo$turb <- 6 + trb08.flo$turb

table(krw08.sed$labcode2)

0   1   4   6

134  27   1  13

# Code 4 means spilled < 15% of sediment mass during or after filtering

# Code 6 means > 15% organic debris or foreign material

# Eliminate bad turbidity and tracking code 7

trb08.sed <- krw08.sed[!is.na(krw08.sed$turb) & krw08.sed$turb < 300, ]

turbsscplot("trb",08)

# Eliminate the egregious and influential outliers

trb08.sed <- trb08.sed[trb08.sed$dump!=3 | trb08.sed$bottle!=3,]

trb08.sed <- trb08.sed[trb08.sed$ssc < 499, ]

trb08.sed <- trb08.sed[trb08.sed$turb > 2, ]  # outlier in log-log space

table(trb08.sed$turbcode)

0   5

113  25

turbsscplot("trb",08)

turbsscplot("trb",08,type="power") # pretty darn linear

turbsscplot("trb",08,type="logxy")

# Try a few different models for comparison

turb1 <- turbsrc("trb",08,070801,0000,080731,2400,type="linear",var=F)

turb2 <- turbsrc("trb",08,070801,0000,080731,2400,type="power",var=F)

turb3 <- turbsrc("trb",08,070801,0000,080731,2400,type="logxy",var=F)

total(turb1,turb2,turb3)

start            end       type surr    yhat   n    r2      s cv

turb1 10/11/07 18:00 03/25/08 17:50     linear turb 1273572 138 0.920 29.596 NA

turb2 10/11/07 18:00 03/25/08 17:50      power turb 1288822 138 0.921 29.545 NA

turb3 10/11/07 18:00 03/25/08 17:50 logxy:mvue turb 1313401 138 0.887  0.484 NA

> sum(turb1$predssc < 0)

[1] 34

# Best to not use linear because of the negative predictions

# The power model has the higher r-square of the remaining 2 models

total(turb2)

start            end  type surr    yhat   n    r2      s cv

turb2 10/11/07 18:00 03/25/08 17:50 power turb 1288822 138 0.921 29.545 NA

Total load = 1288822 kg

# A relatively good estimate

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo08.flo$q)

[1]  7.614 143.100

> range(krw08.sed$q)

[1]  12.63 1172.00

# Eliminate high flow data that could tweak regression and tracking code 7s

flo08.sed <- krw08.sed[krw08.sed$q < 260, ]

qsscplot("flo",08)

# Eliminate outlier

flo08.sed <- flo08.sed[flo08.sed$ssc < 400, ]

flow1 <- flowsrc("flo",08,040801,0000,080731,2400,type="logxy",var=F)

flow2 <- flowsrc("flo",08,040801,0000,080731,2400,type="power",var=F)

flow3 <- flowsrc("flo",08,040801,0000,080731,2400,type="loess",var=F)

flow4 <- flowsrc("flo",08,040801,0000,080731,2400,type="linear",var=F)

total(flow1,flow2,flow3,flow4)

start            end       type surr     yhat   n    r2      s cv

flow1 08/19/07 06:20 06/24/08 16:50 logxy:mvue flow 206893.3 119 0.357  1.038 NA

flow2 08/19/07 06:20 06/24/08 16:50      power flow 207266.7 119 0.340 59.613 NA

flow3 08/19/07 06:20 06/24/08 16:50      loess flow 187005.8 119 0.341 59.732 NA

flow4 08/19/07 06:20 06/24/08 16:50     linear flow 195582.0 119 0.340 59.605 NA

> sum(flow3$predssc <0)

[1] 6583

> sum(flow4$predssc <0)

[1] 0

#Agreement among models OK excluding loess which had too many neg predictions

#logxy looks like the best model to me, highest r2 anyway

start            end       type surr     yhat   n    r2     s cv

flow1 08/19/07 06:20 06/24/08 16:50 logxy:mvue flow 206893.3 119 0.357 1.038 NA

Total load = 206893 kg

==================================================

# Total non-storm load

1288822 + 206893 = 1495715 kg = 1496 metric tons

# Total storm load = 3846716 kg = 3847 metric tons

# Total annual load

3847 + 1496 = 5343 metric tons total annual load

# Storm load is 3847/5343 = 72.0 % of annual load