FTR06 Event Loads

stn,storm,sdate,stime,edate,etime

FTR,601,051130,2300,051204,1200  1894509 kg  Poor (no samples)

FTR,602,051218,0430,051220,0520   236483 kg  Very good

FTR,603,051220,0530,051221,1320   103929 kg  Good

FTR,604,051222,0630,051224,2100   157992 kg  Fair

FTR,605,051228,0130,051230,0730  2886531 kg  Excellent

FTR,606,051230,0740,060103,1150  2303001 kg  Very good

FTR,607,060110,1010,060113,1040   175249 kg  Very good

FTR,608,060113,1050,060117,1050   152618 kg  Very good

FTR,609,060117,1100,060118,0550   100466 kg  Good

FTR,610,060118,0600,060120,1150   138489 kg  Good

FTR,611,060120,1200,060122,1200    57084 kg  Good

FTR,612,060128,0700,060130,0410   126936 kg  Very good

FTR,613,060130,0420,060201,0130    56293 kg  Good

FTR,614,060201,0140,060203,2350   518918 kg  Excellent

FTR,615,060226,1950,060301,1820   294464 kg  Very good

FTR,616,060301,1830,060305,0200    85850 kg  Fair

FTR,617,060305,0210,060307,0010   280785 kg  Very good

FTR,618,060307,0020,060308,1950   341433 kg  Excellent

FTR,619,060316,0810,060318,1200    69342 kg  Very good

FTR,620,060415,1000,060418,1200    92654 kg  Good

-----------------------------------------------------

SUM of events                   10073026 kg  Very good

-----------------------------------------------------

ftr06.flo <- read.flo("ftr",06)

ftr06.lab <- read.lab("ftr",06)

#mismatches("ftr",06)

#Dump 10 bottle 1 has no matching flo record;

#Dump 21 bottles 1 - 9 have no matching ssc records (data was lost)

# I can find no notes or other documentation to resolve the mismatches

ftr06.sed <- merge.flo("ftr",06)

-----------------------------------------------------

Storm 601: dump 5: FTR,601,051130,2300,051204,1200

# Turbidity and stage data are excellent, but NO ISCO samples

# Stage peak 6.5 ft max turbidity 1200

# Try using dump 9 which had a storm of even greater magnitude

four()

turbsscplot("ftr",06,dump=9)

turbsscplot("ftr",06,dump=9,type="power")

turbsscplot("ftr",06,dump=9,type="logxy")

turbsscplot("ftr",06,dump=9,type="loess")

turb1 <- turbsrc("ftr",06,051130,2300,051204,1200,dump=9)

turb2 <- turbsrc("ftr",06,dump=9,051130,2300,051204,1200,type="power")

turb3 <- turbsrc("ftr",06,dump=9,051130,2300,051204,1200,type="logxy",bias="qmle")

turb4 <- turbsrc("ftr",06,dump=9,051130,2300,051204,1200,type="loess")

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2       s    cv

turb1 11/30/05 23:00 12/04/05 12:00     linear turb 1852819 23 0.959 285.051 7.155

turb2 11/30/05 23:00 12/04/05 12:00      power turb 1894509 23 0.961 276.952 4.713

turb3 11/30/05 23:00 12/04/05 12:00 logxy:qmle turb 1922891 23 0.968   0.234 5.342

turb4 11/30/05 23:00 12/04/05 12:00      loess turb 1812327 23 0.960 287.079    NA

# Range of estimates is 6%

ttsplot("ftr",06,turb2)

total(turb2)

start            end  type surr    yhat  n    r2       s    cv

turb2 11/30/05 23:00 12/04/05 12:00 power turb 1894509 23 0.961 276.952 4.713

Total load = 1894509 kg,  CV = 4.71 %  Poor to fair

-----------------------------------------------------------

Storm 602: dumps 6-7: 051218,0430,051220,0520

# Double-peaked event with good quality stage and turbidity data

# Turbidity vs SSC has hysteresis

# Turbidity spike between bottles 6 and 9 was removed

# Very good sample coverage of this event

four()

turbsscplot("ftr",06,051218,0430,051220,0520)

turbsscplot("ftr",06,051218,0430,051218,1930)

turbsscplot("ftr",06,051218,0430,051218,1930,type="power")

turbsscplot("ftr",06,051218,1940,051220,0520)

turb0 <- turbsrc("ftr",06,051218,0430,051220,0520,type="logxy")

start            end       type surr     yhat  n    r2     s     cv

turb0 12/18/05 04:30 12/20/05 05:20 logxy:mvue turb 285442.3 16 0.947 0.325 11.082

Total load = 285442 kg,  CV = 11.08 %

turb1 <- turbsrc("ftr",06,051218,0430,051218,1930,type="power")

turb2 <- turbsrc("ftr",06,051218,1940,051220,0520)

ttsplot("ftr",06,turb1,turb2)

total(turb1,turb2)

start            end   type surr      yhat n    r2      s    cv

turb1 12/18/05 04:30 12/18/05 19:30  power turb  60060.71 9 0.990 30.920 4.084

turb2 12/18/05 19:40 12/20/05 05:20 linear turb 187926.28 7 0.996 13.195 2.422

Total load = 247987 kg,  CV = 2.09 %

# Stats look good but fit to rising limb doesn't, and there's a big discontinuity

head(turb2$predssc)

[1] 790.4857 751.8468 720.5678 672.7292 634.0903 599.1313 566.0123 532.8933 505.2941 472.1751

time1 <- lineartime("ftr",06,051218,0430,051218,1930,ssc1=10,ssc2=825)

ttsplot("ftr",06,time1,turb2)

total(time1,turb2)

start            end   type surr      yhat n    r2      s    cv

time1 12/18/05 04:30 12/18/05 19:30 linear time  48556.57 9    NA     NA    NA

turb2 12/18/05 19:40 12/20/05 05:20 linear turb 187926.28 7 0.996 13.195 2.422

Total load = 236483 kg  Very good

-----------------------------------------------------------

Storm 603: dump 7-8: 051220,0530,051221,1320

# Good turb and stage data, but only 5 samples and missed one on rising limb

four()

turbsscplot("ftr",06,051220,0530,051221,1320)

turbsscplot("ftr",06,051220,0530,051221,1320,type="logxy")

turbsscplot("ftr",06,051220,0330,051221,1320)

turbsscplot("ftr",06,051220,0330,051221,1320,type="logxy")

# There is probably hysteresis but can't tell since bot 2 is missing

turb0 <- turbsrc("ftr",06,051220,0530,051221,1320)

start            end   type surr     yhat n   r2      s    cv

turb0 12/20/05 05:30 12/21/05 13:20 linear turb 100615.5 5 0.97 26.889 9.172

Total load = 100616 kg,  CV = 9.17 %

turb1 <- turbsrc("ftr",06,051220,0530,051220,0920,dump=7,bot=c(1,3))

turb2 <- turbsrc("ftr",06,051220,1050,051221,1320)

tail(turb1$predssc,4)

[1] 464.2109 489.1637 516.3849 536.8008

head(turb2$predssc,4)

[1] 339.9522 319.7478 308.7273 292.1964

time1 <- lineartime("ftr",06,051220,0930,051220,1040,ssc1=545,ssc2=359)

ttsplot("ftr",06,turb1,time1,turb2)

total(turb1,time1,turb2)

start            end   type surr     yhat n r2     s    cv

turb1 12/20/05 05:30 12/20/05 09:20 linear turb 26200.42 2  1   Inf   Inf

time1 12/20/05 09:30 12/20/05 10:40 linear time 22555.64 0 NA    NA    NA

turb2 12/20/05 10:50 12/21/05 13:20 linear turb 55172.84 4  1 1.974 1.179

Total load = 103929 kg  Good

# Only 3.3% higher than the simple linear regression estimate

-----------------------------------------------------------

Storm 604: dump 8,051222,0630,051224,2100

# Double peaked event with good data all around

# Good sample coverage with samples on both peaks

four()

turbsscplot("ftr",06,051222,0630,051224,2100)

turbsscplot("ftr",06,dump=8)

qsscplot("ftr",06,051222,0630,051224,2100)

qsscplot("ftr",06,dump=8)

I suspect something is wrong with the lab data.  Better get the original spreadsheet before proceeding.  An hour of troubleshooting failed to illuminate the problem.

But it's clear that there is an error of some sort on bottles 11 and 13.

four()

turbsscplot("ftr",06,dump=8,bot=c(2:10,12))

turbsscplot("ftr",06,dump=8,bot=c(2:10,12),type="power")

turbsscplot("ftr",06,dump=8,bot=c(2:10,12),type="loess",degree=2)

qsscplot("ftr",06,dump=8,bot=c(2:10,12))

turb1 <- turbsrc("ftr",06,051222,0630,051224,2100,dump=8,bot=c(2:10,12))

turb2 <- turbsrc("ftr",06,051222,0630,051224,2100,dump=8,bot=c(2:10,12),type="power")

turb3 <- turbsrc("ftr",06,051222,0630,051224,2100,dump=8,bot=c(2:10,12),type="loess",degree=2)

# Finally use the model selected for storm 605 below, ignoring dump 8

turb4 <- turbsrc("ftr",06,051222,0630,051224,2100,dump=c(rep(9,24),10),bot=c(1:24,2),type="power")

total(turb1,turb2,turb3,turb4)

start            end   type surr     yhat  n    r2       s     cv

turb1 12/22/05 06:30 12/24/05 21:00 linear turb 153443.7 10 0.882  39.185  9.767

turb2 12/22/05 06:30 12/24/05 21:00  power turb 151554.6 10 0.900  36.245  8.003

turb3 12/22/05 06:30 12/24/05 21:00  loess turb 157992.4 10 0.915  38.916     NA

turb4 12/22/05 06:30 12/24/05 21:00  power turb 181027.8 24 0.963 270.862 16.020

# This is a diverse set of models, I like the loess model

# It's encouraging that the model based on dump 9 is only 14% higher than turb3

ttsplot("ftr",06,turb3)

total(turb3)

start            end  type surr     yhat  n    r2      s cv

turb3 12/22/05 06:30 12/24/05 21:00 loess turb 157992.4 10 0.915 38.916 NA

Total load = 157992 kg  Fair

-----------------------------------------------------------

Storm 605: dump 9,051228,0130,051230,0730

# Good stage data

# Turbidity data had lots of noise on recession limb, smoothed by interpolation

# Excellent sample coverage until fairly low on recession, then nothing for 36 hrs

four()

turbsscplot("ftr",06,dump=c(rep(9,24),10),bot=c(1:24,2))

turbsscplot("ftr",06,dump=c(rep(9,24),10),bot=c(1:24,2),type="power")

turbsscplot("ftr",06,dump=c(rep(9,24),10),bot=c(1:24,2),type="logxy")

turbsscplot("ftr",06,dump=c(rep(9,24),10),bot=c(1:24,2),type="sqrt")

# The power fit looks great

turb1 <- turbsrc("ftr",06,051228,0130,051230,0730,dump=c(rep(9,24),10),bot=c(1:24,2))

turb2 <- turbsrc("ftr",06,051228,0130,051230,0730,dump=c(rep(9,24),10),bot=c(1:24,2),type="power")

turb3 <- turbsrc("ftr",06,051228,0130,051230,0730,dump=c(rep(9,24),10),bot=c(1:24,2),type="logxy",bias="qmle")

turb4 <- turbsrc("ftr",06,051228,0130,051230,0730,dump=c(rep(9,24),10),bot=c(1:24,2),type="sqrt")

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2       s    cv

turb1 12/28/05 01:30 12/30/05 07:30     linear turb 2825989 24 0.961 279.485 3.869

turb2 12/28/05 01:30 12/30/05 07:30      power turb 2886531 24 0.963 270.862 2.872

turb3 12/28/05 01:30 12/30/05 07:30 logxy:qmle turb 3160209 24 0.972   0.229 6.214

turb4 12/28/05 01:30 12/30/05 07:30  sqrt:duan turb 2846060 24 0.977   2.789    NA

ttsplot("ftr",06,turb2)

total(turb2)

start            end  type surr    yhat  n    r2       s    cv

turb2 12/28/05 01:30 12/30/05 07:30 power turb 2886531 24 0.963 270.862 2.872

Total load = 2886531 kg,  CV = 2.87 %  Excellent

-----------------------------------------------------------

Storm 606: dump 10,051230,0740,060103,1150

# Two-peak storm with good stage and turbidity data

# Excellent sample coverage until final falling limb

# No samples for 64 hours

four()

turbsscplot("ftr",06,051230,0740,060103,1150)

turbsscplot("ftr",06,051230,0740,060103,1150,type="power")

turbsscplot("ftr",06,051230,0740,060103,1150,type="logxy")

turbsscplot("ftr",06,051230,0740,060103,1150,type="sqrt")

# I'm going to assume sample 9 is an error and omit it

turb1 <- turbsrc("ftr",06,051230,0740,060103,1150,dump=c(rep(10,22),11),bot=c(2:8,10:24,1))

turb2 <- turbsrc("ftr",06,051230,0740,060103,1150,type="power",dump=c(rep(10,22),11),bot=c(2:8,10:24,1))

turb3 <- turbsrc("ftr",06,051230,0740,060103,1150,type="logxy",bias="qmle",dump=c(rep(10,22),11),bot=c(2:8,10:24,1))

turb4 <- turbsrc("ftr",06,051230,0740,060103,1150,type="sqrt",dump=c(rep(10,22),11),bot=c(2:8,10:24,1))

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2       s    cv

turb1 12/30/05 07:40 01/03/06 11:50     linear turb 2256074 23 0.970 133.251 3.870

turb2 12/30/05 07:40 01/03/06 11:50      power turb 2303001 23 0.972 127.412 2.824

turb3 12/30/05 07:40 01/03/06 11:50 logxy:qmle turb 2287476 23 0.966   0.191 4.536

turb4 12/30/05 07:40 01/03/06 11:50  sqrt:duan turb 2287185 23 0.975   2.014    NA

# Excellent agreement between these estimates (2%)

ttsplot("ftr",06,turb2)

total(turb2)

start            end  type surr    yhat  n    r2       s    cv

turb2 12/30/05 07:40 01/03/06 11:50 power turb 2303001 23 0.972 127.412 2.824

Total load = 2303001 kg,  CV = 2.82 %  Very good

# Would be excellent but for the lack of samples on the recession

-----------------------------------------------------------

Storm 607: dump 12,060110,1010,060113,1040

# Good quality stage and turbidity data, one quick downspike removed

# Very good sample coverage even though a couple are missing

# Looks like maybe every other sample was discarded on recession

turbsscplot("ftr",06,060110,1010,060113,1040)

turbsscplot("ftr",06,060110,1010,060113,1040,type="power")

turbsscplot("ftr",06,060110,1010,060113,1040,type="logxy")

turbsscplot("ftr",06,060110,1010,060113,1040,type="sqrt")

# All models look good; I like the logxy for overall fit

turb1 <- turbsrc("ftr",06,060110,1010,060113,1040)

turb2 <- turbsrc("ftr",06,060110,1010,060113,1040,type="power")

turb3 <- turbsrc("ftr",06,060110,1010,060113,1040,type="logxy",bias="qmle")

turb4 <- turbsrc("ftr",06,060110,1010,060113,1040,type="sqrt")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat n    r2      s    cv

turb1 01/10/06 10:10 01/13/06 10:40     linear turb 174068.7 9 0.973 43.031 7.924

turb2 01/10/06 10:10 01/13/06 10:40      power turb 179694.0 9 0.973 43.971 6.737

turb3 01/10/06 10:10 01/13/06 10:40 logxy:qmle turb 175248.9 9 0.982  0.186 7.675

turb4 01/10/06 10:10 01/13/06 10:40  sqrt:duan turb 174988.1 9 0.979  1.251    NA

# Range of variation is 3.2%

ttsplot("ftr",06,turb3)

total(turb3)

start            end       type surr     yhat n    r2     s    cv

turb3 01/10/06 10:10 01/13/06 10:40 logxy:qmle turb 175248.9 9 0.982 0.186 7.675

Total load = 175249 kg,  CV = 7.67 %  Very good

-----------------------------------------------------------

Storm 608: dumps 12-13,060113,1050,060117,1050

# Very good stage and turbidity data

# Good sample coverage (6 bottles), need to include bot 12(13)

# Looks like odd bottles were discarded on recession

four()

turbsscplot("ftr",06,dump=c(12,rep(13,7)),bot=c(13,1:7))

turbsscplot("ftr",06,dump=c(12,rep(13,7)),bot=c(13,1:7),type="power")

turbsscplot("ftr",06,dump=c(12,rep(13,7)),bot=c(13,1:7),type="logxy")

turbsscplot("ftr",06,dump=c(12,rep(13,7)),bot=c(13,1:7),type="sqrt")

turb1 <- turbsrc("ftr",06,060113,1050,060117,1050,dump=c(12,rep(13,7)),bot=c(13,1:7))

turb2 <- turbsrc("ftr",06,060113,1050,060117,1050,dump=c(12,rep(13,7)),bot=c(13,1:7),type="power")

turb3 <- turbsrc("ftr",06,060113,1050,060117,1050,dump=c(12,rep(13,7)),bot=c(13,1:7),type="logxy",bias="qmle")

turb4 <- turbsrc("ftr",06,060113,1050,060117,1050,dump=c(12,rep(13,7)),bot=c(13,1:7),type="sqrt")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat n    r2      s    cv

turb1 01/13/06 10:50 01/17/06 10:50     linear turb 151879.8 6 0.982 17.208 7.124

turb2 01/13/06 10:50 01/17/06 10:50      power turb 152618.0 6 0.993 10.924 3.911

turb3 01/13/06 10:50 01/17/06 10:50 logxy:qmle turb 155357.9 6 0.985  0.116 4.909

turb4 01/13/06 10:50 01/17/06 10:50  sqrt:duan turb 155401.5 6 0.981  0.712    NA

# Range of variation is 2.3%

ttsplot("ftr",06,turb2)

total(turb2)

start            end  type surr   yhat n    r2      s    cv

turb2 01/13/06 10:50 01/17/06 10:50 power turb 152618 6 0.993 10.924 3.911

Total load = 152618 kg,  CV = 3.91 % Very good to good

-----------------------------------------------------------

Storm 609: dump 13,060117,1100,060118,0550

# Very Good turbidity and stage

# Fair sample coverage (only 4 bottles).  Again, it looks like odd bottles

# on recession limb were discarded. These should have been kept.

four()

turbsscplot("ftr",06,060117,1100,060118,0550)

turbsscplot("ftr",06,060117,1100,060118,0550,type="power")

turbsscplot("ftr",06,dump=13)

turbsscplot("ftr",06,dump=13,bot=6:14)

# No benefit from including extra bottles

# This event has slightly lower SSC for given turb relative to dump 13

turb1 <- turbsrc("ftr",06,060117,1100,060118,0550)

start            end   type surr    yhat n r2    s   cv

turb1 01/17/06 11:00 01/18/06 05:50 linear turb 96793.8 4  1 3.01 0.66

Total load = 96794 kg,  CV = 0.66 %

endpoints(turb1)

start       end

-18.89566  66.22064

# Better not use turb1 due to negative predictions

turb2 <- turbsrc("ftr",06,060117,1100,060118,0550,type="power")

ttsplot("ftr",06,turb2)

total(turb2)

start            end  type surr     yhat n r2    s    cv

turb2 01/17/06 11:00 01/18/06 05:50 power turb 100465.9 4  1 0.89 0.183

Total load = 100466 kg,  CV = 0.18 %  Good

-----------------------------------------------------------

Storm 610: dump 13,060118,0600,060120,1150

# Very Good turbidity and stage

# Good enough sample coverage (6 bottles).  Again, it looks like a bottle

# on recession limb was discarded.

four()

turbsscplot("ftr",06,060118,0600,060120,1150)

turbsscplot("ftr",06,060118,0600,060120,1150,type="power")

turbsscplot("ftr",06,dump=13,bot=12:21)

turbsscplot("ftr",06,dump=13)

# bottle 14, which occurred on a spike, seems anomalous but is consistent with sample s from storm 610

turb0 <- turbsrc("ftr",06,060118,0600,060120,1150,type="power")

ttsplot("ftr",06,turb0)

total(turb0)

start            end  type surr     yhat n    r2      s    cv

turb0 01/18/06 06:00 01/20/06 11:50 power turb 135887.3 6 0.977 15.479 5.343

Total load = 135887 kg,  CV = 5.34 %

# Alternative break it into two

turb1 <- turbsrc("ftr",06,060118,0600,060118,1610,dump=13,bot=9:14,type="power")

turb2 <- turbsrc("ftr",06,060118,1620,060120,1150,type="power")

ttsplot("ftr",06,turb1,turb2)

total(turb1,turb2)

start            end  type surr      yhat n    r2      s   cv

turb1 01/18/06 06:00 01/18/06 16:10 power turb  16829.11 4 1.000  2.954 2.72

turb2 01/18/06 16:20 01/20/06 11:50 power turb 121659.53 5 0.991 10.739 3.63

Total load = 138489 kg,  CV = 3.21 %  Good to very good

# 2% higher than single regression model

-----------------------------------------------------------

Storm 611: dump 13-14,060120,1200,060122,1200

# Very Good turbidity and stage

# Inadequate sample coverage (only 3 bottles).  Again, it looks like a bottle

# on recession limb was discarded.  Should have kept it.

four()

turbsscplot("ftr",06,060120,1200,060122,1200)

turbsscplot("ftr",06,dump=13,bot=18:24)

turbsscplot("ftr",06,dump=13,bot=18:24,type="power")

turbsscplot("ftr",06,dump=13)

turb1 <- turbsrc("ftr",06,060120,1200,060122,1200)

turb2 <- turbsrc("ftr",06,060120,1200,060122,1200,dump=13,bot=18:24)

turb3 <- turbsrc("ftr",06,060120,1200,060122,1200,dump=13,bot=18:24,type="power")

turb4 <- turbsrc("ftr",06,060120,1200,060122,1200,dump=13)

total(turb1,turb2,turb3,turb4)

start            end   type surr     yhat  n    r2      s    cv

turb1 01/20/06 12:00 01/22/06 12:00 linear turb 57161.69  3 0.984 11.733 9.851

turb2 01/20/06 12:00 01/22/06 12:00 linear turb 57083.53  5 0.992  8.206 5.839

turb3 01/20/06 12:00 01/22/06 12:00  power turb 59115.05  5 0.993  8.029 4.544

turb4 01/20/06 12:00 01/22/06 12:00 linear turb 50916.95 18 0.979 14.750 7.760

# Range of variation 3.6% between first 3 models. turb4 probably not a good model.

# Linear model matches bottes 21:24 better than power model

ttsplot("ftr",06,turb2)

total(turb2)

start            end   type surr     yhat n    r2     s    cv

turb2 01/20/06 12:00 01/22/06 12:00 linear turb 57083.53 5 0.992 8.206 5.839

Total load = 57084 kg,  CV = 5.84 %  Good to fair

-----------------------------------------------------------

Storm 612: dump 15,060128,0700,060130,0410

# Very good stage and turbidity data, one quick downspike removed

# Good sample coverage (7 bottles, none discarded)

# Need to go back a few hours to include bottle 1

four()

turbsscplot("ftr",06,dump=15,bot=1:8)

turbsscplot("ftr",06,dump=15,bot=1:8,type="power")

turbsscplot("ftr",06,dump=15,bot=1:8,type="logxy")

turbsscplot("ftr",06,dump=15,bot=1:8,type="sqrt")

turb1 <- turbsrc("ftr",06,060128,0700,060130,0410,dump=15,bot=1:8)

turb2 <- turbsrc("ftr",06,060128,0700,060130,0410,dump=15,bot=1:8,type="power")

turb3 <- turbsrc("ftr",06,060128,0700,060130,0410,dump=15,bot=1:8,type="logxy")

turb4 <- turbsrc("ftr",06,060128,0700,060130,0410,dump=15,bot=1:8,type="sqrt")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat n    r2      s    cv

turb1 01/28/06 07:00 01/30/06 04:10     linear turb 121375.6 8 0.994 12.056 3.104

turb2 01/28/06 07:00 01/30/06 04:10      power turb 126683.4 8 0.997  8.709 1.678

turb3 01/28/06 07:00 01/30/06 04:10 logxy:mvue turb 126936.0 8 0.998  0.042 1.739

turb4 01/28/06 07:00 01/30/06 04:10  sqrt:duan turb 124151.5 8 0.998  0.278    NA

# Range of variation is 4.5%, any of turb2 or turb3 or turb4 would do

ttsplot("ftr",06,turb4)

total(turb3)

start            end       type surr     yhat n    r2     s    cv

turb3 01/28/06 07:00 01/30/06 04:10 logxy:mvue turb 126936.0 8 0.998 0.042 1.739

Total load = 126936 kg,  CV = 1.74 %  Very good

-----------------------------------------------------------

Storm 613: dump 15,060130,0420,060201,0130

# Very good stage and turbidity data

# Fair sample coverage (5 bottles and 4 of these are recession)

four()

turbsscplot("ftr",06,060130,0420,060201,0130)

turbsscplot("ftr",06,060130,0420,060201,0130,type="power")

turbsscplot("ftr",06,060130,0420,060201,0130,type="logxy")

turbsscplot("ftr",06,060130,0420,060201,0130,type="sqrt")

turb1 <- turbsrc("ftr",06,060130,0420,060201,0130)

turb2 <- turbsrc("ftr",06,060130,0420,060201,0130,type="power")

turb3 <- turbsrc("ftr",06,060130,0420,060201,0130,type="logxy")

turb4 <- turbsrc("ftr",06,060130,0420,060201,0130,type="sqrt")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat n    r2     s    cv

turb1 01/30/06 04:20 02/01/06 01:30     linear turb 56292.57 5 0.996 4.336 2.684

turb2 01/30/06 04:20 02/01/06 01:30      power turb 57079.27 5 0.993 5.804 3.340

turb3 01/30/06 04:20 02/01/06 01:30 logxy:mvue turb 56834.50 5 0.995 0.065 3.283

turb4 01/30/06 04:20 02/01/06 01:30  sqrt:duan turb 56289.05 5 0.997 0.213    NA

# Range of variation is small as expected (1.4%)

ttsplot("ftr",06,turb1)

total(turb1)

start            end   type surr     yhat n    r2     s    cv

turb1 01/30/06 04:20 02/01/06 01:30 linear turb 56292.57 5 0.996 4.336 2.684

Total load = 56293 kg,  CV = 2.68 %  Good

-----------------------------------------------------------

Storm 614: dump 15,060201,0140,060203,2350

# Very good stage and turbidity data and very good sample coverage

# Will help to grab an extra sample at each end to estimate lower turbs

four()

turbsscplot("ftr",06,dump=c(rep(15,12),16),bot=c(13:24,1))

turbsscplot("ftr",06,dump=c(rep(15,12),16),bot=c(13:24,1),type="power")

turbsscplot("ftr",06,dump=c(rep(15,12),16),bot=c(13:24,1),type="logxy",bias="qmle")

turbsscplot("ftr",06,dump=c(rep(15,12),16),bot=c(13:24,1),type="loess")

# These models all look good

turb1 <- turbsrc("ftr",06,060201,0140,060203,2350,dump=c(rep(15,12),16),bot=c(13:24,1))

turb2 <- turbsrc("ftr",06,060201,0140,060203,2350,dump=c(rep(15,12),16),bot=c(13:24,1),type="power")

turb3 <- turbsrc("ftr",06,060201,0140,060203,2350,dump=c(rep(15,12),16),bot=c(13:24,1),type="logxy",bias="qmle")

turb4 <- turbsrc("ftr",06,060201,0140,060203,2350,dump=c(rep(15,12),16),bot=c(13:24,1),type="loess")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s    cv

turb1 02/01/06 01:40 02/03/06 23:50     linear turb 518487.6 13 0.989 30.744 3.459

turb2 02/01/06 01:40 02/03/06 23:50      power turb 527267.3 13 0.988 33.422 3.484

turb3 02/01/06 01:40 02/03/06 23:50 logxy:qmle turb 517528.5 13 0.978  0.177 5.979

turb4 02/01/06 01:40 02/03/06 23:50      loess turb 518918.4 13 0.989 31.425    NA

# Range of variation is 1.9%

# turb1 starts negative, but it's for only 5 data points

# turb4 is all positive

ttsplot("ftr",06,turb4)

total(turb4)

start            end  type surr     yhat  n    r2      s cv

turb4 02/01/06 01:40 02/03/06 23:50 loess turb 518918.4 13 0.989 31.425 NA

Total load = 518918 kg  Excellent

-----------------------------------------------------------

Storm 615: dump 16,060226,1950,060301,1820

# Very good stage and turbidity data, one short downspike removed

# Excellent sample coverage (17 samples)

four()

turbsscplot("ftr",06,060226,1950,060301,1820,type="power")

turbsscplot("ftr",06,dump=16,bot=c(4:8,10:20),type="power")

turbsscplot("ftr",06,dump=16,bot=c(4:8,10:20),type="logxy")

turbsscplot("ftr",06,dump=16,bot=c(4:8,10:12,15:20))

Samples 9, 13 and 14 are outliers.  We'll see how sensitive the estimates are to the handling of these bottles.

turb1 <- turbsrc("ftr",06,060226,1950,060301,1820,type="power")

turb2 <- turbsrc("ftr",06,060226,1950,060301,1820,dump=16,bot=c(4:8,10:20),type="power")

turb3 <- turbsrc("ftr",06,060226,1950,060301,1820,dump=16,bot=c(4:8,10:20),type="logxy",bias="qmle")

turb4 <- turbsrc("ftr",06,060226,1950,060301,1820,dump=16,bot=c(4:8,10:12,15:20))

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s    cv

turb1 02/26/06 19:50 03/01/06 18:20      power turb 298862.7 17 0.858 99.179 8.970

turb2 02/26/06 19:50 03/01/06 18:20      power turb 294464.3 16 0.880 93.681 8.572

turb3 02/26/06 19:50 03/01/06 18:20 logxy:qmle turb 300181.6 16 0.964  0.237 8.768

turb4 02/26/06 19:50 03/01/06 18:20     linear turb 284688.9 14 0.992 14.735 1.848

# Only 5.4% difference between largest and smallest estimate

# The variation between the CVs just reflects the variation due to outlier handling

# My inclination is to discard bottle 9 since it has so much leverage

# But keep bottles 13 and 14 which tend to cancel each other and keep the CV honest

# If bottles 13 and 14 are both bad samples turb4 would be a better estimate but

# we're only 3.5% too high if turb2 is selected

ttsplot("ftr",06,turb2)

total(turb2)

start            end  type surr     yhat  n   r2      s    cv

turb2 02/26/06 19:50 03/01/06 18:20 power turb 294464.3 16 0.88 93.681 8.572

Total load = 294464 kg,  CV = 8.57 % very good

-----------------------------------------------------------

Storm 616: dump 17,060301,1830,060305,0200

# Very good stage and turbidity data

# Fair sample coverage

# Yes 5 bottles, but missed the peak; no samples for last 53 hrs

four()

turbsscplot("ftr",06,060301,1830,060305,0200)

turbsscplot("ftr",06,dump=17)

# Bottle 5 is anomalous; get rid of it

turbsscplot("ftr",06,dump=17,bot=c(1:4,6:7))

turbsscplot("ftr",06,dump=17,bot=c(1:4,6:7),type="power")

turb1 <- turbsrc("ftr",06,060301,1830,060305,0200,dump=17,bot=c(1:4,6:7))

turb2 <- turbsrc("ftr",06,060301,1830,060305,0200,dump=17,bot=c(1:4,6:7),type="power")

total(turb1,turb2)

start            end   type surr     yhat n    r2      s     cv

turb1 03/01/06 18:30 03/05/06 02:00 linear turb 71050.08 6 0.977 15.241 17.255

turb2 03/01/06 18:30 03/05/06 02:00  power turb 85850.26 6 0.987 11.602  6.778

# These two estimates don't agree very well (21% difference)

ttsplot("ftr",06,turb2)

total(turb2)

start            end  type surr     yhat n    r2      s    cv

turb2 03/01/06 18:30 03/05/06 02:00 power turb 85850.26 6 0.987 11.602 6.778

Total load = 85850 kg,  CV = 6.78 %  Fair

-----------------------------------------------------------

Storm 617: dump 17,060305,0210,060307,0010

# Double-peaked event

# Excellent turbidity and stage data

# Excellent sample coverage (11 samples)

four()

turbsscplot("ftr",06,060305,0210,060307,0010)

turbsscplot("ftr",06,060305,0210,060307,0010,type="power")

turbsscplot("ftr",06,060305,0210,060307,0010,type="logxy")

turbsscplot("ftr",06,060305,0210,060307,0010,type="loess",degree=2,span=0.90)

turb1 <- turbsrc("ftr",06,060305,0210,060307,0010)

turb2 <- turbsrc("ftr",06,060305,0210,060307,0010,type="power")

turb3 <- turbsrc("ftr",06,060305,0210,060307,0010,type="logxy")

turb4 <- turbsrc("ftr",06,060305,0210,060307,0010,type="loess",degree=2,span=0.90)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s    cv

turb1 03/05/06 02:10 03/07/06 00:10     linear turb 270865.7 11 0.990 24.797 3.472

turb2 03/05/06 02:10 03/07/06 00:10      power turb 276763.1 11 0.993 20.488 2.347

turb3 03/05/06 02:10 03/07/06 00:10 logxy:mvue turb 277305.9 11 0.980  0.112 3.603

turb4 03/05/06 02:10 03/07/06 00:10      loess turb 280784.6 11 0.997 17.147    NA

# Range of varation is just 3.7%

# turb4 handles matches bottle 16 the best

ttsplot("ftr",06,turb4)

total(turb4)

start            end  type surr     yhat  n    r2      s cv

turb4 03/05/06 02:10 03/07/06 00:10 loess turb 280784.6 11 0.997 17.147 NA

Total load = 280785 kg  Very good to Excellent

-----------------------------------------------------------

Storm 618: dump 17,060307,0020,060308,1950

# Very good stage and turbidity.  One small stage bump fixed

# Excellent sample coverage (8 bottles)

# Back up to include bottle 16 to strengthen low end of regression

four()

turbsscplot("ftr",06,dump=17,bot=16:24)

turbsscplot("ftr",06,dump=17,bot=16:24,type="power")

turbsscplot("ftr",06,dump=17,bot=16:24,type="logxy")

turbsscplot("ftr",06,dump=17,bot=16:24,type="loess",degree=2)

turb1 <- turbsrc("ftr",06,060307,0020,060308,1950,dump=17,bot=16:24)

turb2 <- turbsrc("ftr",06,060307,0020,060308,1950,dump=17,bot=16:24,type="power")

turb3 <- turbsrc("ftr",06,060307,0020,060308,1950,dump=17,bot=16:24,type="logxy")

turb4 <- turbsrc("ftr",06,060307,0020,060308,1950,dump=17,bot=16:24,type="loess",degree=2)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat n    r2      s    cv

turb1 03/07/06 00:20 03/08/06 19:50     linear turb 337231.3 9 0.998 10.234 1.288

turb2 03/07/06 00:20 03/08/06 19:50      power turb 342230.6 9 0.998 10.252 1.088

turb3 03/07/06 00:20 03/08/06 19:50 logxy:mvue turb 341433.1 9 0.999  0.029 1.047

turb4 03/07/06 00:20 03/08/06 19:50      loess turb 339746.7 9 0.999  7.966    NA

# Range of variation is 1.5%

ttsplot("ftr",06,turb3)

total(turb3)

start            end       type surr     yhat n    r2     s    cv

turb3 03/07/06 00:20 03/08/06 19:50 logxy:mvue turb 341433.1 9 0.999 0.029 1.047

Total load = 341433 kg,  CV = 1.05 %  Excellent!

# It doesn't get any better than this

-----------------------------------------------------------

Storm 619: dump 18,060316,0810,060318,1200

# Very good stage and turbidity data

# Adequate sample coverage

# Only 4 samples but we can pick up an extra bottle before and after

four()

turbsscplot("ftr",06,dump=18,bot=3:8)

turbsscplot("ftr",06,dump=18,bot=3:8,type="power")

turbsscplot("ftr",06,dump=18,bot=3:8,type="logxy")

turbsscplot("ftr",06,dump=18,bot=4:8,type="loess")

turb1 <- turbsrc("ftr",06,060316,0810,060318,1200,06,dump=18,bot=3:8)

turb2 <- turbsrc("ftr",06,060316,0810,060318,1200,06,dump=18,bot=3:8,type="power")

turb3 <- turbsrc("ftr",06,060316,0810,060318,1200,06,dump=18,bot=3:8,type="logxy")

turb4 <- turbsrc("ftr",06,060316,0810,060318,1200,06,dump=18,bot=4:8,type="loess")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat n    r2     s    cv

turb1 03/16/06 08:10 03/18/06 12:00     linear turb 69341.83 6 0.987 7.446 3.514

turb2 03/16/06 08:10 03/18/06 12:00      power turb 69943.12 6 0.984 8.318 3.766

turb3 03/16/06 08:10 03/18/06 12:00 logxy:mvue turb 70274.31 6 0.981 0.142 7.594

turb4 03/16/06 08:10 03/18/06 12:00      loess turb 70276.51 5 0.994 6.447    NA

# Not very sensitive to exclusion of bottle 3

# Range of variation is only 1.4%

ttsplot("ftr",06,turb1)

total(turb1)

start            end   type surr     yhat n    r2     s    cv

turb1 03/16/06 08:10 03/18/06 12:00 linear turb 69341.83 6 0.987 7.446 3.514

Total load = 69342 kg,  CV = 3.51 %  Very good

-----------------------------------------------------------

Storm 620: dump 20,060415,1000,060418,1200

# Almost a two-peaked storm; more like a peak and a hump

# Very good turbidity and stage data

# Sharp turbidity spike at start of rise has a different turb/SSC relation

# Will need to break this event into two segments to limit influence of samples 4-6

# Very good sample coverage on first peak; only 1 sample on 2-day hump

# Bring in bottles 2 and 3 to help nail down lower end of regression

# And maybe bots 15 and 16?

four()

turbsscplot("ftr",06,dump=20,bot=2:16,type="power")

turbsscplot("ftr",06,dump=20,bot=2:7)

turbsscplot("ftr",06,dump=20,bot=8:14)

turbsscplot("ftr",06,dump=20,bot=8:16)

turb0 <- turbsrc("ftr",06,060415,1000,060418,1200,dump=20,bot=2:16,type="power")

turb1 <- turbsrc("ftr",06,060415,1000,060415,1400,dump=20,bot=2:7)

turb2a <- turbsrc("ftr",06,060415,1410,060418,1200,dump=20,bot=8:14)

turb2b <- turbsrc("ftr",06,060415,1410,060418,1200,dump=20,bot=8:16)

total(turb0)

start            end  type surr     yhat  n    r2      s    cv

turb0 04/15/06 10:00 04/18/06 12:00 power turb 84736.57 15 0.971 19.754 5.884

Total load = 84737 kg,  CV = 5.88 %

total(turb1,turb2a)

start            end   type surr      yhat n    r2      s     cv

turb1  04/15/06 10:00 04/15/06 14:00 linear turb  1539.711 6 0.990  4.923  2.669

turb2a 04/15/06 14:10 04/18/06 12:00 linear turb 91114.340 7 0.982 17.948 10.601

Total load = 92654 kg,  CV = 10.42 %  Good

total(turb1,turb2b)

start            end   type surr      yhat n    r2      s    cv

turb1  04/15/06 10:00 04/15/06 14:00 linear turb  1539.711 6 0.990  4.923 2.669

turb2b 04/15/06 14:10 04/18/06 12:00 linear turb 99280.060 9 0.984 17.576 6.752

Total load = 100820 kg,  CV = 6.65 %

# The 3 estimates vary by 19% but the first isn't really a contender

# Still the last estimate is 11% higher than the middle one

# The preferred estiamate is middle one (last one misses on bottle 14, which strongly

affects 2 entire days).  See plots below:

ttsplot("ftr",06,turb2a,turb1)

ttsplot("ftr",06,turb2b,turb1)