HHB 2006 Annual load estimation

hhb06.flo <- read.flo("hhb",06)

hhb06.lab <- read.lab("hhb",06)

hhb06.sed <- merge.flo("hhb",06)

nonstorms <- drop.storms("hhb",06)

dim(nonstorms)

[1] 23631     4

head(nonstorms, 5)

chr     q turb turbcode

1 (10/08/05 14:00:00) 2.471    1        7

2 (10/08/05 14:10:00) 2.471    1        5

3 (10/08/05 14:20:00) 2.480    1        5

4 (10/08/05 14:30:00) 2.471    1        0

5 (10/08/05 14:40:00) 2.462    1        0

table(nonstorms$turbcode)

0     1     5     7

22529   911    75   116

911 values of questionable turbidity, none bad (code 2)

Most were 15-20Oct2005 or 25-29Mar2006

The Oct values are below 12 NTU and inconsequential

The Mar values are mostly corroborated by the SSC

trb06.flo <- nonstorms[!is.na(nonstorms$turb), ]

dim(trb06.flo)

[1] 23631     4

# Develop model for good turb data

range(trb06.flo$turb)

[1]   1 206

range(hhb06.sed$turb)

[1]    1 1072

table(hhb06.sed$labcode2)

0   1   4

250   5   5

# No bad lab codes to worry about

# (4=sediment spilled, but less than 15%)

sum(is.na(hhb06.sed$turb))

trb06.sed <- hhb06.sed[!is.na(hhb06.sed$turb), ]

table(hhb06.sed$turbcode)

0   1   5   7

242   6   7   5

# 5=interpolated, 6=reconstructed, 7=freehanded

turbsscplot("hhb",06,type="loess")

# Let's limit ourselves to the prediction range (1-206 NTU) which eliminates most outliers too

# Eliminate them

trb06.sed <- hhb06.sed[hhb06.sed$turb < 250, ]

turbsscplot("trb",06,txt="dump")

# The above will predict negative SSC below about 20 NTU

turbsscplot("trb",06,type="logxy",txt="dump")

# A couple of mega-outliers; should be eliminated before using this

# These are bottle 1 of dumps 1 and 2

trb06.sed <- hhb06.sed[hhb06.sed$turb < 250, ]

trb06.sed <- trb06.sed[!(trb06.sed$bottle ==1 & trb06.sed$dump %in% 1:2), ]

# Much better;  this will be our model

# But also look at power model

turbsscplot("trb",06,type="power",txt="dump")

turb1 <- turbsrc("trb",06,050801,0000,060731,2400,type="power",var=F)

turb2 <- turbsrc("trb",06,050801,0000,060731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",06,050801,0000,060731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",06,050801,0000,060731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat   n    r2      s cv

turb1 10/08/05 14:00 05/10/06 17:30      power turb 826269.6 201 0.863 38.905 NA

turb2 10/08/05 14:00 05/10/06 17:30 logxy:mvue turb 813181.5 201 0.935  0.234 NA

turb3 10/08/05 14:00 05/10/06 17:30      loess turb 634788.1 201 0.862 39.221 NA

turb4 10/08/05 14:00 05/10/06 17:30     linear turb 679543.1 201 0.859 39.493 NA

# Small 1.6% difference between the only two viable models (power and logxy)

> sum(turb3$predssc < 0)

[1] 14981

> sum(turb4$predssc < 0)

[1] 15773

# Can't use the linear or loess models because of negative predictions

# The logxy model is clearly the best fit

total(turb2)

start            end       type surr     yhat   n    r2     s cv

turb2 10/08/05 14:00 05/10/06 17:30 logxy:mvue turb 813181.5 201 0.935 0.234 NA

Total load = 813182 kg

# Looks to be a very reliable estimate

====================================================================

# Total non-storm load

813182 kg = 813 metric tons

# Total storm load = 14263477 kg = 14263 metric tons

# Total annual load

14263 + 813 = 15076 metric tons total annual load

# Storm load is 14263/15076 = 94.6% of annual load