SFM06 Event Loads

Note turbidity data was excellent all winter, but there were 8 events with no samples or

very poor sample coverage - either the sampler was full or there were ISCO/battery

problems.  The only reason these were not rated "poor" is that the relation between turb

and ssc for surrounding dumps was fairly tight.

Discharge measurments on Dec 20,24,28 seem to indicate backwater effect (from debris?).

But applying a rating equation based on them gives discharges that are much too low

relative to KRW, so I've disregarded those points but did apply a new rating curve based

on all the other HY03-07 measurements.  It's not much different from the HY05 curve.

See sfm/SFM06 rating equation discussion.doc

stn,storm,sdate,stime,edate,etime

SFM,601,051130,2250,051204,1200	1285591 kg	Excellent

SFM,602,051218,0430,051220,0600	 329845 kg  Very good

SFM,603,051220,0610,051221,1410	 300104 kg  Good

SFM,604,051222,0700,051224,2200	 820793 kg	Excellent

SFM,605,051228,0130,051230,0830	2958505 kg  Excellent!!

SFM,606,051230,0840,060103,1150     2502414 kg	Excellent

SFM,607,060110,1000,060113,1440	2193775 kg  Fair

SFM,608,060113,1450,060117,1040	1019422 kg  Very good

SFM,609,060117,1050,060118,0550	 296886 kg  Excellent

SFM,610,060118,0600,060120,1300	 480463 kg  Very good

SFM,611,060120,1310,060122,1200      621586 kg  Very good

SFM,612,060128,0700,060130,0410	 814458 kg  Good

SFM,613,060130,0420,060201,0140	 324056 kg  Fair (no samples)

SFM,614,060201,0150,060204,0050	1495190 kg	Fair (no samples)

SFM,615,060226,1910,060301,1820	1193499 kg  Very good

SFM,616,060301,1830,060305,0200	 844506 kg	Good

SFM,617,060305,0210,060307,0000	2125489 kg  Fair (no samples)

SFM,618,060307,0010,060308,2020	 929842 kg  Fair (no samples)

SFM,619,060316,0800,060318,1200	 542990 kg  Fair (no samples)

SFM,620,060415,0940,060418,1200      568554 kg  Fair (no samples)

-----------------------------------------------------

SUM of events                      21647968 kg

-----------------------------------------------------

sfm06.flo <- read.flo("sfm",06)

sfm06.lab <- read.lab("sfm",06)

mismatches("sfm",06)

# There are 220 unmatched samples in flo file!

# There is also one unmatched sample in lab file: dump 1 bot 19

# The FF shows it but there is no such datum in the flo file

# It was apparently collected during a non-storm event on Nov 8 or 9

sfm06.sed <- merge.flo("sfm",06)

Storm 601: dump 4-5:  SFM,601,051130,2250,051204,1200

# Excellent turbidity and sample coverage

turbsscplot("sfm",06, 051130,2250,051204,1200)

Pretty nice. There are a couple of subtle hysteresis loops that could be dealt with but it may not be worth it.

turb1 <- turbsrc("sfm",06, 051130,2250,051204,1200)

turb2 <- turbsrc("sfm",06, 051130,2250,051204,1200,type="pairs")

time1 <- lineartime("sfm",06, 051130,2250,051204,1200)

time2 <- cubictime("sfm",06, 051130,2250,051204,1200)

ttsplot("sfm",06,turb1)

start            end   type surr    yhat  n    r2      s    cv

turb1 11/30/05 22:50 12/04/05 12:00 linear turb 1285591 25 0.977 80.374 3.246

turb2 11/30/05 22:50 12/04/05 12:00  pairs turb 1287985 25    NA     NA    NA

time1 11/30/05 22:50 12/04/05 12:00 linear time 1296262 25    NA     NA    NA

time2 11/30/05 22:50 12/04/05 12:00 linear time 1295750 25    NA     NA    NA

# These are all within 1% of each other

Total load = 1285591 kg,  CV = 3.25 % Excellent

Storm 602: dump 6:  SFM,602,051218,0430,051220,0600

Sample coverage is OK, 11 samples; missing a couple

turbsscplot("sfm",06, 051218,0430,051220,0600)

turb1 <- turbsrc("sfm",06, 051218,0430,051220,0600)

turb2 <- turbsrc("sfm",06, 051218,0430,051220,0600,type="power")

turb3 <- turbsrc("sfm",06, 051218,0430,051220,0600,type="logxy")

turb4 <- turbsrc("sfm",06, 051218,0430,051220,0600,type="sqrt")

start            end       type surr     yhat  n    r2      s    cv

turb1 12/18/05 04:30 12/20/05 06:00     linear turb 330977.1 12 0.980 26.294 2.315

turb2 12/18/05 04:30 12/20/05 06:00      power turb 329845.2 12 0.981 26.291 2.395

turb3 12/18/05 04:30 12/20/05 06:00 logxy:mvue turb 331157.6 12 0.986  0.121 4.050

turb4 12/18/05 04:30 12/20/05 06:00  sqrt:duan turb 329858.6 12 0.985  0.792    NA

linear model predicts negative at start of storm; Use power func

ttsplot("sfm",06,turb2)

start            end  type surr     yhat  n    r2      s    cv

turb2 12/18/05 04:30 12/20/05 06:00 power turb 329845.2 12 0.981 26.291 2.395

Total load = 329845 kg,  CV = 2.39 %  Very good

Storm 603: dump 6-7: SFM,603,051220,0610,051221,1410

Pretty good sample coverage, but 21-hr gap on recession, and a bad? sample

par(mfrow=c(1,2))

turbsscplot("sfm",06,051220,0610,051221,1410)

# Sample 22 is a major outlier for the whole year; omit it

# Omit sample 22 and add bottle 3 collected at 051221,1500 hrs

turbsscplot("sfm",06,dump=rep(6:7,c(5,2)),bot=c(16:20,1,3))

turb1 <- turbsrc("sfm",06,051220,0610,051221,1410,dump=rep(6:7,c(5,2)),bot=c(16:20,1,3))

ttsplot("sfm",06,turb1)

start            end   type surr     yhat n    r2      s    cv

turb1 12/20/05 06:10 12/21/05 14:10 linear turb 300104.3 7 0.989 37.187 4.829

Total load = 300104 kg,  CV = 4.83 %  Good

Storm 604: dump 7 SFM,604,051222,0700,051224,2200

Very good sample coverage

par(mfrow=c(1,2))

turbsscplot("sfm",06,051222,0700,051224,2200)

# We've got hysteresis; pairs will handle it nicely in this case

turbsscplot("sfm",06,051222,0700,051224,2200,type="pairs")

turb1 <- turbsrc("sfm",06,051222,0700,051224,2200)

turb2 <- turbsrc("sfm",06,051222,0700,051224,2200,type="pairs")

start            end   type surr     yhat  n    r2      s    cv

turb1 12/22/05 07:00 12/24/05 22:00 linear turb 875763.0 14 0.973 89.951 5.654

turb2 12/22/05 07:00 12/24/05 22:00  pairs turb 820792.5 14    NA     NA    NA

ttsplot("sfm",06,turb2)

start            end  type surr     yhat  n r2  s cv

turb2 12/22/05 07:00 12/24/05 22:00 pairs turb 820792.5 14 NA NA NA

Total load = 820793 kg  Excellent

Storm 605: dump 9: SFM,605,051228,0130,051230,0830

Excellent coverage until ran out of bottles late on recession

Sensor actually maxed out for 6 data points at peak (samples 11-12)

Should use linear time from 0920 to 1020 on 051228

four()

turbsscplot("sfm",06,051228,0130,051230,0830)

# Nice hysteresis loop

turbsscplot("sfm",06,051228,0130,051230,0830,type="pairs")

turbsscplot("sfm",06,051228,0130,051228,0910,type="loess",degree=2)

turbsscplot("sfm",06,051228,1030,051230,0830,type="loess",degree=2,span=0.8)

# This is gorgeous data

turb1 <- turbsrc("sfm",06,051228,0130,051228,0910,type="loess",degree=2)

turb2 <- turbsrc("sfm",06,051228,1030,051230,0830,type="loess",degree=2,span=0.8)

tail(turb1$predssc,4)

[1] 2748.763 2785.530 2827.706 2844.248

head(turb2$predssc,4)

[1] 2530.814 2420.555 2322.523 2191.499

time1 <- lineartime("sfm",06,051228,0920,051228,1020,ssc1=2861,ssc2=2640)

ttsplot("sfm",06,turb1,time1,turb2)

start            end   type surr      yhat  n    r2      s cv

turb1 12/28/05 01:30 12/28/05 09:10  loess turb  781313.4  9 1.000 25.215 NA

time1 12/28/05 09:20 12/28/05 10:20 linear time  409198.0  2    NA     NA NA

turb2 12/28/05 10:30 12/30/05 08:30  loess turb 1767993.7 13 0.998 33.142 NA

Total load = 2958505 kg   Excellent

Storm 606: dumps 10-11 SFM,606,051230,0840,060103,1150

This storm had a landslide the night of Jan 01; blocked river and shut

off the flow for a couple hours, then released a big slug of sediment.

I believe I heard the slide occurred on BLM property.

Excellent coverage until no more bottles in the last 18 hrs.

This event has two peaks and each one has a turb/ssc hysteresis loop

four()

turbsscplot("sfm",06,dump=10,bot=1:13,type="pairs")

turbsscplot("sfm",06,dump=rep(10:11,c(10,1)),bot=c(13:22,1),type="pairs")

turbsscplot("sfm",06,dump=11)

turbsscplot("sfm",06,dump=rep(10:11,c(7,24)),bot=c(16:22,1:24))

# Stategy: use separate regressions for

(1) rising limb of first turbidity peak, including bottle 1

(2) falling limb of first turbidity peak

(3) rising limb of second turbidity peak, including bottle 13

(4) all the rest, including the landslide

This requires breaking storm into two parts since these

routines are limited to handling 2 turbidity regressions at a time

# Use the following models

four()

turbsscplot("sfm",06,dump=10,bot=1:6,type="loess",degree=2)

turbsscplot("sfm",06,051230,1830,051231,1200,type="loess",degree=2)

turbsscplot("sfm",06,dump=10,bot=13:15)

turbsscplot("sfm",06,051231,1550,060103,1150,type="loess",degree=2)

# First peak

turb1 <- turbsrc("sfm",06,051230,0840,051230,1820,dump=10,bot=1:6,type="loess",degree=2)

turb2 <- turbsrc("sfm",06,051230,1830,051231,1200,type="loess",degree=2)

ttsplot("sfm",06,turb1,turb2)

# Second peak

turb3 <- turbsrc("sfm",06,051231,1200,051231,1540,dump=10,bot=13:15)

turb4 <- turbsrc("sfm",06,051231,1550,060103,1150,type="loess",degree=2)

ttsplot("sfm",06,turb3,turb4)

# There's a discontinuity betwen bots 15 and 16, we can fix it

with a linear segment

# Also a slight refinement is needed to make turb2 and turb3 meet

smoothly

turb2 <- turbsrc("sfm",06,051230,1830,051231,1310,type="loess",degree=2)

turb3 <- turbsrc("sfm",06,051231,1320,051231,1400,dump=10,bot=13:15)

time1 <- lineartime("sfm",06,051231,1410,051231,1540,ssc1=921,ssc2=turb4$predssc[1])

ttsplot("sfm",06,turb2,turb3)  # plot not shown

ttsplot("sfm",06,turb3,time1,turb4)

# Total all the pieces

total(turb1,turb2,turb3,time1,turb4)

start            end   type surr       yhat  n    r2       s   cv

turb1 12/30/05 08:40 12/30/05 18:20  loess turb  288111.13  6 0.999  42.424   NA

turb2 12/30/05 18:30 12/31/05 13:10  loess turb 1012893.32  7 0.999  13.151   NA

turb3 12/31/05 13:20 12/31/05 14:00 linear turb   29838.98  3 1.000   5.879 0.49

time1 12/31/05 14:10 12/31/05 15:40 linear time   95625.92  1    NA      NA   NA

turb4 12/31/05 15:50 01/03/06 11:50  loess turb 1075944.86 31 0.975 116.273   NA

Total load = 2502414 kg		Excellent

# How much of this came after bottle 24?

turb5 <- turbsrc("sfm",06,060102,1910,060103,1150,051231,1550,060103,1150,type="loess",degree=2)

start            end  type surr     yhat  n    r2       s cv

turb5 01/02/06 19:10 01/03/06 11:50 loess turb 131217.3 31 0.975 116.273 NA

131217/2502414 = 5.24% of the load

Storm 607: dumps 13-14: SFM,607,060110,1000,060113,1440

Double turbidity peak, still very high turbidity from landslide

Ran out of bottles early in this event. Missed 24 hrs including the main peak.

May need to include some bottles from previous event in regressions.

four()

turbsscplot("sfm",06,060110,1000,060113,1440)

turbsscplot("sfm",06,dump=13,bot=c(9,19:21))

turbsscplot("sfm",06,dump=rep(13:14,c(1,8)),bot=c(23,1:8),type="loess")

turbsscplot("sfm",06,dump=rep(c(11,13,14),c(19,1,8)),bot=c(6:24,23,1:8))

We'll need two regressions; the question is how much to weight sample 23

versus the bottles from dump 11 (the prior storm).  We'll run it using

both regressions above to see how much they differ.

turb1 <- turbsrc("sfm",06,060110,1000,060110,1650,dump=13,bot=c(9,19:21))

turb2 <- turbsrc("sfm",06,060110,1700,060113,1440,type="loess")

turb3 <- turbsrc("sfm",06,060110,1700,060113,1440,type="power")

turb4 <- turbsrc("sfm",06,060110,1700,060113,1440,type="pairs")

turb5 <- turbsrc("sfm",06,060110,1700,060113,1440,dump=rep(c(11,13,14),c(19,1,8)),bot=c(6:24,23,1:8))

start            end   type surr    yhat  n    r2       s    cv

turb2 01/10/06 17:00 01/13/06 14:40  loess turb 2133643  9 0.999  25.276    NA

turb3 01/10/06 17:00 01/13/06 14:40  power turb 2083886  9 0.990  63.757 2.723

turb4 01/10/06 17:00 01/13/06 14:40  pairs turb 2138057  9    NA      NA    NA

turb5 01/10/06 17:00 01/13/06 14:40 linear turb 1775783 28 0.971 110.966 2.068

They differ by 17% despite the low CVs

The pattern of the final samples 1-8 suggests the second peak might have had

significantly higher SSC for a given turb than either dumps 11 or 13. Therefore

go with the loess estimate.

ttsplot("sfm",06,turb1,turb2)

The problem is the discontinuity between bottles 21 and 23. We don't

have a good way to transition between the two relationships, so use

linear time.

endpoints(turb1)

start       end

459.1106 1821.3434

turb1 <- turbsrc("sfm",06,060110,1000,060110,1650,dump=13,bot=c(9,19:21))

time1 <- lineartime("sfm",06,060110,1700,060110,1940,ssc1=1821,ssc2=2032)

turb2 <- turbsrc("sfm",06,060110,1950,060113,1440,type="loess")

ttsplot("sfm",06,turb1,time1,turb2)

total(turb1,time1,turb2)

start            end   type surr       yhat n    r2      s    cv

turb1 01/10/06 10:00 01/10/06 16:50 linear turb   75633.37 4 0.998 28.771 1.577

time1 01/10/06 17:00 01/10/06 19:40 linear time  117052.95 0    NA     NA    NA

turb2 01/10/06 19:50 01/13/06 14:40  loess turb 2001088.29 9 0.999 25.276    NA

Total load = 2193775 kg   fair

Storm 608: dumps 14-15: SFM,608,060113,1450,060117,1040

Excellent sample coverage, but also include bots 7-8 just before storm

four()

turbsscplot("sfm",06,060113,1040,060117,1040)

# Sample 24 may have had slight sensor fouling but we'll accept it

# Slight hysteresis might justify 2 regressions here

turbsscplot("sfm",06,060113,1040,060114,2020)

turbsscplot("sfm",06,060113,1040,060114,2020,type="power")

turbsscplot("sfm",06,060114,2030,060117,1040)

turb1 <- turbsrc("sfm",06,060113,1450,060114,2020,060113,1040,060114,2020,type="power")

turb2 <- turbsrc("sfm",06,060114,2030,060117,1040)

ttsplot("sfm",06,turb1,turb2)

# Not a smooth transition but acceptable

start            end   type surr     yhat  n    r2      s    cv

turb1 01/13/06 14:50 01/14/06 20:20  power turb 470476.2  5 0.997 22.053 1.778

turb2 01/14/06 20:30 01/17/06 10:40 linear turb 548945.9 17 0.943 50.685 3.170

Total load = 1019422 kg,  CV = 1.89 %   Very good

Storm 609: dump 15: SFM,609,060117,1050,060118,0550

Excellent sample coverage

four()

turbsscplot("sfm",06,060117,1050,060118,0550)

turbsscplot("sfm",06,060117,1050,060117,1900)

turbsscplot("sfm",06,060117,1910,060118,0550)

turbsscplot("sfm",06,060117,1900,060118,0550)

It doesn't get much better than that!

turb1 <- turbsrc("sfm",06,060117,1050,060117,1900)

turb2 <- turbsrc("sfm",06,060117,1910,060118,0550)

ttsplot("sfm",06,turb1,turb2)

total(turb1,turb2)

start            end   type surr      yhat n    r2      s    cv

turb1 01/17/06 10:50 01/17/06 19:00 linear turb  77519.32 6 0.999 18.311 1.405

turb2 01/17/06 19:10 01/18/06 05:50 linear turb 219366.51 7 1.000  6.555 0.376

Total load = 296886 kg,  CV = 0.46 %  Excellent

Storm 610: dump 15: SFM,610,060118,0600,060120,1300

Good sample coverage but missed one bottle late on recession

four()

turbsscplot("sfm",06,060118,0600,060120,1300)

turbsscplot("sfm",06,060118,0600,060120,1300,type="power")

turbsscplot("sfm",06,060118,0600,060120,1300,type="pairs")

turb1 <- turbsrc("sfm",06,060118,0600,060120,1300)

turb2 <- turbsrc("sfm",06,060118,0600,060120,1300,type="power")

turb3 <- turbsrc("sfm",06,060118,0600,060120,1300,type="pairs")

start            end   type surr     yhat n    r2      s    cv

turb1 01/18/06 06:00 01/20/06 13:00 linear turb 488572.2 7 0.982 12.639 1.619

turb2 01/18/06 06:00 01/20/06 13:00  power turb 486622.1 7 0.983 12.069 1.479

turb3 01/18/06 06:00 01/20/06 13:00  pairs turb 480462.8 7    NA     NA    NA

ttsplot("sfm",03,turb3)

start            end  type surr     yhat n r2  s cv

turb3 01/18/06 06:00 01/20/06 13:00 pairs turb 480462.8 7 NA NA NA

Total load = 480463 kg  Very good

Storm 611: dump 16: SFM,611,060120,1310,060122,1200

No samples during the short rapid rising turbidity segment

Excellent coverage on the turbidity recession including the hydro peak.

four()

turbsscplot("sfm",06,060120,1310,060122,1200)

turbsscplot("sfm",06,060120,1310,060122,1200,type="power")

turbsscplot("sfm",06,060120,1310,060122,1200,type="loess")

turbsscplot("sfm",06,060120,1310,060122,1200,type="loess",degree=2)

turb1 <- turbsrc("sfm",06,060120,1310,060122,1200)

turb2 <- turbsrc("sfm",06,060120,1310,060122,1200,type="power")

turb3 <- turbsrc("sfm",06,060120,1310,060122,1200,type="loess")

turb4 <- turbsrc("sfm",06,060120,1310,060122,1200,type="loess",degree=2)

start            end   type surr     yhat  n    r2      s    cv

turb1 01/20/06 13:10 01/22/06 12:00 linear turb 652880.6 12 0.992 63.656 4.023

turb2 01/20/06 13:10 01/22/06 12:00  power turb 658050.5 12 0.995 54.686 2.646

turb3 01/20/06 13:10 01/22/06 12:00  loess turb 629695.8 12 0.996 48.041    NA

turb4 01/20/06 13:10 01/22/06 12:00  loess turb 621585.9 12 0.998 40.448    NA

start            end  type surr     yhat  n    r2      s cv

turb4 01/20/06 13:10 01/22/06 12:00 loess turb 621585.9 12 0.998 40.448 NA

Total load = 621586 kg	Very good

How much of this was on the unsampled rising limb?

turb5 <- turbsrc("sfm",06,060120,1310,060120,1800,060120,1310,060122,1200,type="loess",degree=2)

start            end  type surr     yhat  n    r2      s cv

turb5 01/20/06 13:10 01/20/06 18:00 loess turb 78240.85 12 0.998 40.448 NA

Total load = 78241 kg = 12.6% of 621586, so not that tragic to have missed

Storm 612: dump 17: SFM,612,060128,0700,060130,0410

Very poor sample coverage; only 4 samples early in the event

Will need to use samples from the peak of 060127 (which did not qualify as a storm)

four()

turbsscplot("sfm",06,060128,0700,060130,0410)

turbsscplot("sfm",06,dump=17)

turbsscplot("sfm",06,dump=16:17)

turbsscplot("sfm",06,dump=15:17)

turb1 <- turbsrc("sfm",06,060128,0700,060130,0410)

turb2 <- turbsrc("sfm",06,060128,0700,060130,0410,dump=17)

turb3 <- turbsrc("sfm",06,060128,0700,060130,0410,dump=16:17)

turb4 <- turbsrc("sfm",06,060128,0700,060130,0410,dump=15:17)

total(turb1,turb2,turb3,turb4)

start            end   type surr     yhat  n    r2       s    cv

turb1 01/28/06 07:00 01/30/06 04:10 linear turb 832296.0  5 0.986  66.620 4.669

turb2 01/28/06 07:00 01/30/06 04:10 linear turb 766566.6 24 0.968  97.108 3.427

turb3 01/28/06 07:00 01/30/06 04:10 linear turb 810727.7 48 0.962 120.534 2.791

turb4 01/28/06 07:00 01/30/06 04:10 linear turb 814457.9 72 0.967  99.671 1.875

Less than 10% difference between these estimates, not so bad.

ttsplot("sfm",06,turb4)

start            end   type surr     yhat  n    r2      s    cv

turb4 01/28/06 07:00 01/30/06 04:10 linear turb 814457.9 72 0.967 99.671 1.875

Total load = 814458 kg,  CV = 1.87 %  Good

Storm 613: dump 17: SFM,613,060130,0420,060201,0140

No samples :-(

four()

turbsscplot("sfm",06,dump=15:16)

turbsscplot("sfm",06,dump=17)

turbsscplot("sfm",06,dump=18)

turbsscplot("sfm",06,dump=15:18)

ttsplot("sfm",06,

turb1 <- turbsrc("sfm",06,060130,0420,060201,0140,dump=15:16)

turb2 <- turbsrc("sfm",06,060130,0420,060201,0140,dump=17)

turb3 <- turbsrc("sfm",06,060130,0420,060201,0140,dump=18)

turb4 <- turbsrc("sfm",06,060130,0420,060201,0140,dump=15:18)

total(turb1,turb2,turb3,turb4)

start            end   type surr     yhat  n    r2      s    cv

turb1 01/30/06 04:20 02/01/06 01:40 linear turb 325513.9 48 0.987 64.394 3.418

turb2 01/30/06 04:20 02/01/06 01:40 linear turb 324645.9 24 0.968 97.108 8.091

turb3 01/30/06 04:20 02/01/06 01:40 linear turb 312403.5  9 0.996  8.058 1.801

turb4 01/30/06 04:20 02/01/06 01:40 linear turb 324056.1 81 0.970 93.877 3.810

Less than 5% difference between lowest and highest estimates, not bad

ttsplot("sfm",06,060130,0420,060201,0140,dump=15:18)

start            end   type surr     yhat  n   r2      s   cv

turb4 01/30/06 04:20 02/01/06 01:40 linear turb 324056.1 81 0.97 93.877 3.81

Total load = 324056 kg,  CV = 3.81 %  Fair

Storm 614: dump 18: SFM,614,060201,0150,060204,0050

No samples again!  Use dumps 15-18 as before

turb1 <- turbsrc("sfm",06,060201,0150,060204,0050,dump=15:16)

turb2 <- turbsrc("sfm",06,060201,0150,060204,0050,dump=17)

turb3 <- turbsrc("sfm",06,060201,0150,060204,0050,dump=18)

turb4 <- turbsrc("sfm",06,060201,0150,060204,0050,dump=15:18)

total(turb1,turb2,turb3,turb4)

start            end   type surr    yhat  n    r2      s    cv

turb1 02/01/06 01:50 02/04/06 00:50 linear turb 1565480 48 0.987 64.394 1.479

turb2 02/01/06 01:50 02/04/06 00:50 linear turb 1412059 24 0.968 97.108 3.591

turb3 02/01/06 01:50 02/04/06 00:50 linear turb 1442380  9 0.996  8.058 2.086

turb4 02/01/06 01:50 02/04/06 00:50 linear turb 1495190 81 0.970 93.877 1.737

Max and min differ by 11%, not so bad

ttsplot("sfm",06,turb4)

start            end   type surr    yhat  n   r2      s    cv

turb4 02/01/06 01:50 02/04/06 00:50 linear turb 1495190 81 0.97 93.877 1.737

Total load = 1495190 kg,  CV = 1.74 %  Fair

Storm 615: dump 19: SFM,615,060226,1910,060301,1820

Good sample coverage despite missing (discarded?) bottles

four()

turbsscplot("sfm",06,060226,1910,060301,1820)

qsscplot("sfm",06,060226,1910,060301,1820)

# Bottle 4 is an outlier in both plots.  Omit it and split into two periods

turbsscplot("sfm",06,dump=19,bot=c(1:3,5:24))

turbsscplot("sfm",06,dump=rep(19:20,c(4,4)),bot=c(18,20,22,24,1:4))

four()

turbsscplot("sfm",06,dump=19,bot=c(1:3,5:24),type="power")

turbsscplot("sfm",06,dump=rep(19:20,c(4,4)),bot=c(18,20,22,24,1:4),type="power")

turbsscplot("sfm",06,dump=rep(19:20,c(4,4)),bot=c(18,20,22,24,1:4),type="logxy")

turbsscplot("sfm",06,dump=rep(19:20,c(4,4)),bot=c(18,20,22,24,1:4),type="loess")

turb1 <- turbsrc("sfm",06,060226,1910,060228,0440,dump=19,bot=c(1:3,5:24),type="power")

turb2 <- turbsrc("sfm",06,060228,0450,060301,1820,dump=rep(19:20,c(4,4)),bot=c(18,20,22,24,1:4),type="power")

turb3 <- turbsrc("sfm",06,060228,0450,060301,1820,dump=rep(19:20,c(4,4)),bot=c(18,20,22,24,1:4),type="logxy")

turb4 <- turbsrc("sfm",06,060228,0450,060301,1820,dump=rep(19:20,c(4,4)),bot=c(18,20,22,24,1:4),type="loess")

total(turb2,turb3,turb4)

start            end       type surr     yhat n    r2       s    cv

turb2 02/28/06 04:50 03/01/06 18:20      power turb 797092.0 8 0.982 152.007 7.786

turb3 02/28/06 04:50 03/01/06 18:20 logxy:mvue turb 840373.5 8 0.990   0.113 4.241

turb4 02/28/06 04:50 03/01/06 18:20      loess turb 849229.3 8 0.985 149.709    NA

ttsplot("sfm",06,turb1,turb3)

start            end       type surr     yhat  n    r2       s    cv

turb1 02/26/06 19:10 02/28/06 04:40      power turb 353125.4 17 0.976 128.474 3.043

turb3 02/28/06 04:50 03/01/06 18:20 logxy:mvue turb 840373.5  8 0.990   0.113 4.241

Total load = 1193499 kg,  CV = 3.12 %  Very good

Storm 616: dump 20: SFM,616,060301,1830,060305,0200

Very good coverage until no samples in the last 24 hrs

four()

turbsscplot("sfm",06,060301,1830,060305,0200)

turbsscplot("sfm",06,dump=20,bot=4:18)

turbsscplot("sfm",06,dump=20,bot=4:18,type="power")

turbsscplot("sfm",06,dump=20,bot=17:24)

turbsscplot("sfm",06,060301,1830,060305,0200)

turb1 <- turbsrc("sfm",06,060301,1830,060303,0800,dump=20,bot=4:18)

turb2 <- turbsrc("sfm",06,060301,1830,060303,0800,dump=20,bot=4:18,type="power")

turb3 <- turbsrc("sfm",06,060301,1830,060303,0800,dump=20,bot=4:18,type="loess",degree=2)

turb4 <- turbsrc("sfm",06,060303,0810,060305,0200,dump=20,bot=17:24)

total(turb1,turb2,turb3)

start            end   type surr     yhat  n    r2      s    cv

turb1 03/01/06 18:30 03/03/06 08:00 linear turb 608195.4 15 0.951 43.174 2.531

turb2 03/01/06 18:30 03/03/06 08:00  power turb 609282.9 15 0.953 42.473 2.489

turb3 03/01/06 18:30 03/03/06 08:00  loess turb 608938.3 15 0.954 46.102    NA

These are very similar

ttsplot("sfm",06,turb2,turb4)

total(turb2,turb4)

start            end   type surr     yhat  n    r2      s    cv

turb2 03/01/06 18:30 03/03/06 08:00  power turb 609282.9 15 0.953 42.473 2.489

turb4 03/03/06 08:10 03/05/06 02:00 linear turb 235223.2  8 0.965 19.103 5.391

Total load = 844506 kg,  CV = 2.34 %  Good

# How much after bottle 24

turb5 <- turbsrc("sfm",06,060303,2045,060305,0200,dump=20,bot=17:24)

start            end   type surr     yhat n    r2      s    cv

turb5 03/03/06 20:50 03/05/06 02:00 linear turb 104135.1 8 0.965 19.103 10.17

104135/844506 = 12.3% of the event load

Storm 617: dump 20: SFM,617,060305,0210,060307,0000

No samples :-(  Maximum turbidity is 1500

#Eliminate outlier bottle 4 from dump 19

sfm06.lab <- sfm06.lab[-316,]

sfm06.sed <- merge.flo("sfm",06)

four()

turbsscplot("sfm",06,dump=19,type="loess")

turbsscplot("sfm",06,dump=19:21,type="loess")

turbsscplot("sfm",06,dump=17:22)

turbsscplot("sfm",06)

turb1 <- turbsrc("sfm",06,060305,0210,060307,0000,dump=19,type="loess")

turb2 <- turbsrc("sfm",06,060305,0210,060307,0000,dump=19:21)

turb3 <- turbsrc("sfm",06,060305,0210,060307,0000,dump=17:22)

turb4 <- turbsrc("sfm",06,060305,0210,060307,0000,dump=1:27)

total(turb1,turb2,turb3,turb4)

start     end   type surr    yhat   n    r2       s    cv

turb1 03/05/06 02:10 3/07/06  loess turb 2181916  17 0.972 145.972    NA

turb2 03/05/06 02:10 3/07/06 linear turb 2250656  65 0.948 114.877 1.992

turb3 03/05/06 02:10 3/07/06 linear turb 2125489 111 0.944 115.366 1.591

turb4 03/05/06 02:10 3/07/06 linear turb 2097623 446 0.943 140.898 0.846

Only 7% difference between highest and lowest

turb2 is higher because of the greater influence of bottle 18 on a smaller sample.

I'm guessing turb3 is the most representative

ttsplot("sfm",06,turb3)

start     end   type surr    yhat   n    r2       s    cv

turb3 03/05/06 02:10 3/07/06 linear turb 2125489 111 0.944 115.366 1.591

Total load = 2125489 kg,  CV = 1.59 %   Fair

Storm 618: dump 20 SFM,618,060307,0010,060308,2020

Again, no samples!  Use same method as for 617; max turb = 750

Actually the field forms say there were 3 bottles pumped manually during

this event, labelled A1-A3, taken on 07Mar at 0940, 1340, and 1830.

But I don't have the concentrations at this time (JL)

turb1 <- turbsrc("sfm",06,060307,0010,060308,2020,dump=19,type="loess")

turb2 <- turbsrc("sfm",06,060307,0010,060308,2020,dump=19:21)

turb3 <- turbsrc("sfm",06,060307,0010,060308,2020,dump=17:22)

turb4 <- turbsrc("sfm",06,060307,0010,060308,2020,dump=1:27)

total(turb1,turb2,turb3,turb4)

start            end   type surr     yhat   n    r2       s    cv

turb1 03/07/06 00:10 03/08/06 20:20  loess turb 801582.0  17 0.972 145.972    NA

turb2 03/07/06 00:10 03/08/06 20:20 linear turb 968333.4  65 0.948 114.877 2.712

turb3 03/07/06 00:10 03/08/06 20:20 linear turb 929841.6 111 0.944 115.366 2.170

turb4 03/07/06 00:10 03/08/06 20:20 linear turb 950728.4 446 0.943 140.898 1.301

21% difference between turb2 and turb1.  Go with turb3 again.

ttsplot("sfm",06,turb3)

total(turb3)

Total load = 929842 kg,  CV = 2.17 %  Fair

Storm 619: dump 21: SFM,619,060316,0800,060318,1200

Again, NO SAMPLES!  Max turb = 880  Use same methods as for 617 and 618

turb1 <- turbsrc("sfm",06,060316,0800,060318,1200,dump=19,type="loess")

turb2 <- turbsrc("sfm",06,060316,0800,060318,1200,dump=19:21)

turb3 <- turbsrc("sfm",06,060316,0800,060318,1200,dump=17:22)

turb4 <- turbsrc("sfm",06,060316,0800,060318,1200,dump=1:27)

total(turb1,turb2,turb3,turb4)

start            end   type surr     yhat   n    r2       s    cv

turb1 03/16/06 08:00 03/18/06 12:00  loess turb 443587.4  17 0.972 145.972    NA

turb2 03/16/06 08:00 03/18/06 12:00 linear turb 558814.9  65 0.948 114.877 3.767

turb3 03/16/06 08:00 03/18/06 12:00 linear turb 542989.7 111 0.944 115.366 2.963

turb4 03/16/06 08:00 03/18/06 12:00 linear turb 568715.2 446 0.943 140.898 1.748

28% difference between turb4 and turb1

ttsplot("sfm",06,turb3)

total(turb3)

turb3 03/16/06 08:00 03/18/06 12:00 linear turb 542989.7 111 0.944 115.366 2.963

Total load = 542990 kg,  CV = 2.96 %  Fair

Storm 620: dump 26: SFM,620,060415,0940,060418,1200

No samples AGAIN!!  Turbidity hit 1650.

Use same methods as for storms 617-619

turb1 <- turbsrc("sfm",06,060415,0940,060418,1200,dump=19,type="loess")

turb2 <- turbsrc("sfm",06,060415,0940,060418,1200,dump=19:21)

turb3 <- turbsrc("sfm",06,060415,0940,060418,1200,dump=17:22)

turb4 <- turbsrc("sfm",06,060415,0940,060418,1200,dump=1:27)

total(turb1,turb2,turb3,turb4)

start            end   type surr     yhat   n    r2       s    cv

turb1 04/15/06 09:40 04/18/06 12:00  loess turb 485903.5  17 0.972 145.972    NA

turb2 04/15/06 09:40 04/18/06 12:00 linear turb 583431.6  65 0.948 114.877 4.057

turb3 04/15/06 09:40 04/18/06 12:00 linear turb 568554.4 111 0.944 115.366 3.178

turb4 04/15/06 09:40 04/18/06 12:00 linear turb 598965.4 446 0.943 140.898 1.861

23% diff between turb4 and turb1

ttsplot("sfm",06,turb3)

total(turb3)
               start            end   type surr     yhat   n    r2       s    cv

turb3 04/15/06 09:40 04/18/06 12:00 linear turb 568554.4 111 0.944 115.366 3.178

Total load = 568554 kg,  CV = 3.18 %  Fair