SFM 2008 Annual SAND and FINES load estimation

sfm08.flo <- read.flo("sfm",08)

sfm08.lab <- read.sand("sfm",08)

sfm08.sed <- merge.flo.sand("sfm",08)

# Now create the ssc column: it will represent only FINES

sfm08.sed$ssc <- sfm08.sed$totssc - sfm08.sed$sand

dim(sfm08.sed)

[1] 73 11

dim(sfm08.flo)

[1] 36545     9

table(sfm08.flo$turbcode)

0     1     2     5     6     7

7136    42 22170  5446  1748     3

22170 values of bad turbidity out of 36545 intervals to be estimated

# This reflects all the fouling of the OBS-3

trb08.flo <- sfm08.flo[!is.na(sfm08.flo$turb), ]

flo08.flo <- sfm08.flo[is.na(sfm08.flo$turb), ]

> dim(trb08.flo)

[1] 14375     9

> dim(flo08.flo)

[1] 22170     9

14375+22170

[1] 36545

# Develop model for good turb data

range(trb08.flo$turb)

[1]   9 651

range(sfm08.sed$turb)

[1]  59 591

# There is an absence of low turbidity samples

table(sfm08.sed$labcode2)

0  1  4  6

66  3  1  3

# Codes 4 is minor errors

# Code 1 is < 15% organics

# Code 6 is > 15% organics

sfm08.sed[sfm08.sed$labcode2 >= 4, ]

chr dump bottle  sand  totssc turb turbcode      q labcode1 labcode2     ssc

10 (01/03/08 19:40:00)    6      5  1.60   94.66   62        0  104.0        0        6   93.06

44 (01/31/08 20:20:00)    8     20 34.73 1359.09  286        0 1044.0        0        4 1324.36

46 (02/01/08 03:20:00)    8     22 58.34  930.96  152        0  594.7        0        6  872.62

49 (02/01/08 21:30:00)    9      2 21.53  236.21   60        5  241.4        0        6  214.68

# Turns out none of these are outliers but there are others

two()

turbsscplot("sfm",08,type="logxy",txt="dump")

turbsscplot("sfm",08,type="logxy")

# Eliminate outliers 3(15) and 4(3)

trb08.sed <- sfm08.sed[sfm08.sed$ssc > 10, ]

four()

turbsscplot("trb",08)

turbsscplot("trb",08,type="loess") # slightly curved

turbsscplot("trb",08,type="power")

turbsscplot("trb",08,type="logxy")

# Try a few different models for comparison

turb1 <- turbsrc("trb",08,070801,0000,080731,2400,type="power",var=F)

turb2 <- turbsrc("trb",08,070801,0000,080731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",08,070801,0000,080731,2400,type="linear",var=F)

turb4 <- turbsrc("trb",08,070801,0000,080731,2400,type="loess",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2       s cv

turb1 11/09/07 19:50 03/17/08 22:20      power turb 6912614 71 0.934 152.069 NA

turb2 11/09/07 19:50 03/17/08 22:20 logxy:mvue turb 6701024 71 0.829   0.359 NA

turb3 11/09/07 19:50 03/17/08 22:20     linear turb 5987553 71 0.937 149.061 NA

turb4 11/09/07 19:50 03/17/08 22:20      loess turb 5455784 71 0.938 148.302 NA

> sum(turb3$predssc < 0)

[1] 8499

> sum(turb4$predssc < 0)

[1] 9594

# Can't use the linear or loess models because of the negative predictions

# The power model has the higher r-square of the remaining 2 models

total(turb1)

start            end  type surr    yhat  n    r2       s cv

turb1 11/09/07 19:50 03/17/08 22:20 power turb 6912614 71 0.934 152.069 NA

Total fines load by turbidity = 6912614 kg

# A fair estimate.  There's a lot of scatter in the regression

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo08.flo$q)

[1]   1.42 152.00

> range(sfm08.sed$q)

[1]   81.13 1125.00

# Problem is no samples at flows under 81cfs.

two()

qsscplot("sfm",08,txt="dump")

qsscplot("sfm",08)

# Eliminate the same samples as in the turbidity analysis

# And eliminate potential high-leverage points above 20cms = 700cfs range

flo08.sed <- sfm08.sed[sfm08.sed$ssc >10 & sfm08.sed$q < 700,]

four()

qsscplot("flo",08)

qsscplot("flo",08,type="power")

qsscplot("flo",08,type="linear")

qsscplot("flo",08,type="loess")

# Note all predictions are made below 4.3 cms (152 cfs)

flow1 <- flowsrc("flo",08,040801,0000,080731,2400,type="logxy",var=F)

flow2 <- flowsrc("flo",08,040801,0000,080731,2400,type="power",var=F)

flow3 <- flowsrc("flo",08,040801,0000,080731,2400,type="linear",var=F)

flow4 <- flowsrc("flo",08,040801,0000,080731,2400,type="loess",var=F)

total(flow1,flow2,flow3,flow4)

start            end       type surr      yhat  n    r2       s cv

flow1 08/12/07 15:30 04/22/08 10:10 logxy:mvue flow 1444325.1 66 0.420   0.627 NA

flow2 08/12/07 15:30 04/22/08 10:10      power flow 1405016.3 66 0.313 455.336 NA

flow3 08/12/07 15:30 04/22/08 10:10     linear flow 1218238.3 66 0.314 455.241 NA

flow4 08/12/07 15:30 04/22/08 10:10      loess flow  534398.2 66 0.327 454.342 NA

> sum(flow3$predssc < 0)

[1] 7996

> sum(flow4$predssc < 0)

[1] 14479

# Cannot use linear or loess due to the many negative predictions

# The logxy and power models agree well, but the estimate is highly dependent

# on the truncation of the sample that was done (at 700 cfs)

# There's no strong basis fro choosing one model over the other

hist(flo08.flo$q)

# Note there were no samples below 80 cfs so this is a pretty wild estimate

# logxy looks like the best model to me, highest r2 by far, and favors low values

total(flow1)

start            end       type surr    yhat  n   r2     s cv

flow1 08/12/07 15:30 04/22/08 10:10 logxy:mvue flow 1444325 66 0.42 0.627 NA

Total fines load by discharge = 1444325 kg

Total fines load by turbidity = 6912614 kg

Total fines load = 1444325 + 6912614 = 8356939 kg

==================================================

# Now compute total load with the same samples (i.e. just those with sand fractions)

trb08.sed$ssc <- trb08.sed$totssc

flo08.sed$ssc <- flo08.sed$totssc

four()

turbsscplot("trb",08,type="power",txt="dump")

turbsscplot("trb",08,type="logxy",txt="dump")

turbsscplot("trb",08,type="linear",txt="dump")

turbsscplot("trb",08,type="loess",txt="dump")

turb1 <- turbsrc("trb",08,070801,0000,080731,2400,type="power",var=F)

turb2 <- turbsrc("trb",08,070801,0000,080731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",08,070801,0000,080731,2400,type="linear",var=F)

turb4 <- turbsrc("trb",08,070801,0000,080731,2400,type="loess",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2       s cv

turb1 11/09/07 19:50 03/17/08 22:20      power turb 7314356 71 0.928 169.662 NA

turb2 11/09/07 19:50 03/17/08 22:20 logxy:mvue turb 7090642 71 0.822   0.370 NA

turb3 11/09/07 19:50 03/17/08 22:20     linear turb 6334296 71 0.930 166.697 NA

turb4 11/09/07 19:50 03/17/08 22:20      loess turb 5763852 71 0.932 166.130 NA

> sum(turb3$predssc < 0)

[1] 8880

> sum(turb4$predssc < 0)

[1] 9807

# Can't use the linear or loess models because of the negative predictions

# The power model has the higher r-square of the remaining 2 models

total(turb1)

start            end  type surr    yhat  n    r2       s cv

turb1 11/09/07 19:50 03/17/08 22:20 power turb 7314356 71 0.928 169.662 NA

Total load by turbidity from reduced sample = 7314356 kg

four()

qsscplot("flo",08)

qsscplot("flo",08,type="power")

qsscplot("flo",08,type="linear")

qsscplot("flo",08,type="loess")

flow1 <- flowsrc("flo",08,040801,0000,080731,2400,type="logxy",var=F)

flow2 <- flowsrc("flo",08,040801,0000,080731,2400,type="power",var=F)

flow3 <- flowsrc("flo",08,040801,0000,080731,2400,type="linear",var=F)

flow4 <- flowsrc("flo",08,040801,0000,080731,2400,type="loess",var=F)

total(flow1,flow2,flow3,flow4)

start            end       type surr      yhat  n    r2       s cv

flow1 08/12/07 15:30 04/22/08 10:10 logxy:mvue flow 1472558.6 66 0.433   0.627 NA

flow2 08/12/07 15:30 04/22/08 10:10      power flow 1446068.5 66 0.322 485.207 NA

flow3 08/12/07 15:30 04/22/08 10:10     linear flow 1164659.1 66 0.322 484.990 NA

flow4 08/12/07 15:30 04/22/08 10:10      loess flow  415470.2 66 0.336 483.820 NA

> sum(flow3$predssc < 0)

[1] 8387

> sum(flow4$predssc < 0)

[1] 15172

# Same situation as with the fines model. Stay with logxy for consistency

start            end       type surr    yhat  n    r2     s cv

flow1 08/12/07 15:30 04/22/08 10:10 logxy:mvue flow 1472559 66 0.433 0.627 NA

Total load by discharge from reduced sample = 1472559 kg

Total load by turbidity from reduced sample = 7314356 kg

Total load from reduced sample = 1472559 + 7314356 = 8786915

This is 14% higher than the 7720048 kg based on sum of storms and inter-storm loads

That's not surprising since so much extrapolation was required for the reduced sample

==============================================

Total load 8786915

Total fines 8356939

Total sand  429976

Percent sand = 4.9%

Percent fines= 95.1%

Applying these percentages to the more reliable total load (7720048) gives

Total fines = 7342278

Total sand = 377770

==============================================

%sand based on turbidity regressions alone = 401742/7314356 = 5.5%

%sand based on discharge regressions alone = 28234/1472559 = 1.9%