FILE FORMATS

The core data files consist of ASCII TEXT only.  These files have various extensions but are simply text, and can be opened with any text editor or easily imported into any spreadsheet, database, or statistical program.. A TTS database consists of the following core data file types:

- Station Information File (stationinfo.txt)

- Observer Record File (file names ending in .or)

- Stage-Discharge Rating File (file names ending in .sdr)

- Laboratory Data File (file names ending in .isc)

- Sand Data File (file names ending in .sand)

- Appended/Corrected Output (file names ending in .flo)

- Campbell Data Logger File (file names ending in .dat)

- Ten-minute SSC File (file names ending in ssc.txt)

# Station Information File (stationinfo.txt)

Each line of the file is a list of comma-separated values:

- unique 3-letter station name

- watershed area in hectares (needed only for reconstructions of discharge that use the time-linear difference method )

- units of stage measurement, either ft or m

- names of staff plate readings, if any, that are routinely read by observers at the site. These will be shown in the plot legend. This list specifies the order of staff plate readings in the observer record file. For sites using Paradox field forms, the observer record file will be automatically created with stilling well, right staff, left staff, and average (of right and left) staff, so the list must have four names corresponding to these staff plate readings.

- if there is more than one staff plate listed in item 4 above, the default that is to be displayed on plots must be repeated at the end of the list.

Example station information file:

sfc,424,ft,stilling well,right staff,left staff,average,left staff
zie,25.3,ft,stilling well,right staff,left staff,average,stilling well
hyp,2750,m,right staff,left staff,right staff
ftr,3445,ft,right staff
ujc,1500,m,left staff

# Observer Record File (.or)

Each line of the file is a list of comma-separated values. The staff plate readings must correspond to the list of names starting at item 4 in the station information file. However, the default reading is only listed once in the observer record file. For example, an observer record file for station hyp as defined in the example station information file, would contain the following values:

- 4-digit year

- 2-digit month

- 2-digit day

- 4-digit military time of the staff plate reading(s)

- dump (data file) number of the electronic data recorded at the time of the staff plate reading(s)

- one or more staff plate reading(s)

Example observer record file for station hyp, containing a right and left staff plate reading as defined in the example station information file above:

2003,12,01,1240,3,0.06,0.06
2003,12,08,1210,4,0.05,0.04
2003,12,10,1500,4,0.11,0.10

# Stage-Discharge Rating File (.sdr)

Each line of the file is a list of comma-separated values indicating the starting date for each stage-discharge rating and the corresponding expression. Use the special expression "DO NOT CALCULATE" if you have inserted discharge values in the FLO file that you do not want to be modified.

- 4-digit year

- 2-digit month

- 2-digit day

- 4-digit military time of the staff plate reading(s)

- A mathematical expression to compute discharge from stage. Expressions must contain a single variable, either s or x. These are not equations, so they should not begin with "q =". Spaces are ignored. Expressions may include the arithmetic operators + , - , * , / , and ^; and the mathematical functions exp, ln, log10, log2, abs, sqrt, sin, cos, tan, cot, sec, csc, arcsin, arccos, arctan, and round. In addition, expressions may include the one-character relational operators =, <, and >. Relational expressions evaluate to 0 if false or 1 if true, and are included to permit different expressions to be applied for different stage ranges (see example below). The absence of two-character relational operators requires a little extra work (see the last 3 expressions below, which are all equivalent if stage is recorded to 3 decimals).

Example stage-discharge rating equation file:

2000,08,01,0000, DO NOT CALCULATE
2001,08,01,0000, 4*s^2.5
2002,08,02,0000, 81.56*s^(1.4174 - 0.2927*ln(s)) 
2003,02,05,1200, (s<2)*4*s^2.5 + (s=2)*4*s^2.5 + (s>2)*3.5*s^2.1
2003,08,01,0000, 4*s^2.5 + (s>2)*(3.5*s^2.1 - 4*s^2.5)
2004,08,01,0000, (s<2.0005)*4*s^2.5 + (s>2.0005)*3.5*s^2.1

# Laboratory Data File (.isc)

Each line of the file is a list of comma-separated values:

- dump number

- bottle number

- suspended sediment concentration (mg/l), use NA for missing values

- lab quality code or other additional columns (optional)

Example laboratory data file:

10,002, 1.499E+02,00
10,003, 1.379E+02,00
11,001, NA,50
11,002, 6.745E+00,07
11,003, 3.083E+00,00

# Sand Data File (.sand)

Each line of the file is a list of comma-separated values:

- dump number

- bottle number

- suspended sand concentration (mg/l), use NA for missing values

- suspended sediment concentration (mg/l), use NA for missing values

- lab quality code or other additional columns (optional)

Example sand data file:

11,5,99.82,330.48,0

11,6,81.93,294.76,0

11,7,44.13,200.02,0

12,13,97.74,257.55,0

12,14,58.04,283.45,0

12,15,34.33,179.61,0

# Appended/Corrected Output (.flo)

Each line of the file is a list of comma-separated values:

- 4-digit year

- 2-digit month

- 2-digit day of month

- 4-digit military time

- dump number

- bottle number

- threshold and sampling codes

- threshold code: R=rising, F=falling, B=baseflow (below minimum stage)

- sampling code: X=none, T=threshold, A=auxiliary, D=aux w/DIS, S=startup, E=equal time-interval sample above turbidity sensor limit

- raw stage (units defined in station information file)

- corrected stage

- stage code

- discharge (units determined by stage-discharge rating)

- raw turbidity

- corrected turbidity

- turbidity code

- optional fields such as water and air temperature, rainfall, etc.

Example appended/corrected output file with optional water temperature:

2003,11,12,0510,6,0,BX,0.270,0.262,4,1.514E-1,0007,0007,0,9.23
2003,11,12,0520,6,0,BX,0.270,0.262,4,1.514E-1,0008,0008,0,9.23
2003,11,12,0530,6,0,BX,0.270,0.262,4,1.514E-1,0007,0007,0,9.23
2003,11,12,0540,6,0,BX,0.270,0.262,4,1.514E-1,0020,0007,5,9.23
2003,11,12,0550,6,0,BX,0.270,0.262,4,1.514E-1,0010,0007,5,9.22

# Campbell Data Logger File (.dat)

Each line of the file is a list of comma separated values:

- 3-digit station identifier

- 4-digit year

- day number (1 = Jan. 1)

- 4-digit military time

- dump number

- bottle number

- threshold code

- 1 = rising, 2 = falling, 3 = baseflow (below minimum stage)

- sample code

- 0 = none, 1 = threshold, 2 = auxiliary, 3 = aux w/DIS, 4 = startup, 
5 = equal time-interval sample above turbidity sensor limit

- stage

- turbidity

- optional fields such as water and air temperature, rainfall, etc.

Example Campbell data logger file with optional water temperature:

200,2003,50,1800,20,1,2,1,4.903,676,9.1

200,2003,50,1810,20,0,2,0,5.149,642,9.1

200,2003,50,1820,20,0,2,0,5.087,643,9.1

200,2003,50,1830,20,0,2,0,5.108,615,9.1

200,2003,50,1840,20,0,2,0,5.012,609,9.1

200,2003,50,1850,20,0,2,0,4.739,603,9.1

# Ten-minute SSC File (ssc.txt)

Each line of the file is a list of values separated by white space:

- Date (mm/dd/yy)

- Time (hh:mm)

- Flow (cubic feet per second)

- Turbidity (units depend on sensor)

- Turbidity quality code (see Stage and Turbidity Codes in TTS Adjuster manual

- SSC (mg/L)

- Method – identifies the surrogate variable used to derive SSC

- 1 = turbidity (linear or non-linear regression of SSC on turbidity)

- 2 = flow (linear or non-linear regression of SSC on discharge)

- 3 = time (SSC values interpolated linearly between samples)

Example ten-minute SSC file:

DATE     TIME  FLOW      TURB TURBQUAL  SSC METHOD

02/19/03 07:00  3.747E+01 40   0  3.176E+01 1

02/19/03 07:10  3.907E+01 40   0  3.176E+01 1

02/19/03 07:20  4.054E+01 41   0  3.311E+01 1

02/19/03 07:30  4.125E+01 41   0  3.311E+01 1

02/19/03 07:40  4.296E+01 42   0  3.447E+01 1

02/19/03 07:50  4.511E+01 43   0  3.583E+01 1

02/19/03 08:00  4.832E+01 44   0  3.721E+01 1