SFM 2004 Annual load estimation

These loads were calc’ed using the following rating equation, built from HY03-05 measurements:

q <- ifelse(s<2.4302, 1.2326*s^3.1572, 2.1831*s^2+28.325*s-61.386)

sfm04.flo <- read.flo("sfm",04)

sfm04.lab <- read.lab("sfm",04)

sfm04.sed <- merge.flo("sfm",04)

nonstorms <- drop.storms("sfm",04)

dim(nonstorms)

[1] 41282     3

head(nonstorms, 5)

chr     q turb turbcode

1 (08/30/03 05:50:00) 1.901   NA        2

2 (08/30/03 06:00:00) 1.901   NA        2

3 (08/30/03 06:10:00) 1.901   NA        2

4 (08/30/03 06:20:00) 1.901   NA        2

5 (08/30/03 06:30:00) 1.901   NA        2

table(nonstorms$turbcode)

0     1     2     4     5

15044  1306 20588   208  4136

20588 values of bad turbidity out of 41282 intervals to be estimated

trb04.flo <- nonstorms[!is.na(nonstorms$turb), ]

flo04.flo <- nonstorms[is.na(nonstorms$turb), ]

dim(trb04.flo)

[1] 20694     3

dim(flo04.flo)

[1] 20588     3

20694+20588

[1] 41282

# Develop model for good turb data

range(trb04.flo$turb)

[1]   3 542

table(sfm04.sed$labcode2)

0   1

200   7

# No bad lab codes to worry about

# Eliminate bad turbidity and turbidity > 550

trb04.sed <- sfm04.sed[!is.na(sfm04.sed$turb) & sfm04.sed$turb < 550, ]

table(trb04.sed$turbcode)

0   1   4   5

145   3   6   9

turbsscplot("trb",04)

# Eliminate bots 45-48 from dump 5 and try different models

trb04.sed <- sfm04.sed[!is.na(sfm04.sed$turb) & sfm04.sed$turb < 550 & sfm04.sed$bottle < 45, ]

table(trb04.sed$turbcode)

0   1   4   5

143   3   6   7

turbsscplot("trb",04,type="power")

turb1 <- turbsrc("trb",04,030801,0000,040731,2400,type="power",var=F)

start            end  type surr     yhat   n   r2     s cv

turb1 11/29/03 04:00 05/29/04 15:30 power turb 602366.6 159 0.93 55.85 NA

Total load = 602367 kg

# A relatively good estimate

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo04.flo$q)

[1]  1.482 56.800

> range(sfm04.sed$q)

[1]   7.292 817.700

# Eliminate high discharges to avoid influence on regression

flo04.sed <- sfm04.sed[sfm04.sed$q <= 62, ]

qsscplot("flo",04)

The scatter is huge. We just have to pray there isn't much sediment

to be estimated here.

flow1 <- flowsrc("flo",04,030801,0000,040731,2400,var=F)

total(flow1)

start            end       type surr    yhat  n    r2     s cv

flow1 08/30/03 05:50 07/01/04 10:00 logxy:mvue flow 59029.7 51 0.248 1.008 NA

Total load = 59030 kg

# Compare with power model and loess models

qsscplot("flo",04,type="power")

qsscplot("flo",04,type="loess")

flow2 <- flowsrc("flo",04,030801,0000,040731,2400,type="power",var=F)

flow3 <- flowsrc("flo",04,030801,0000,040731,2400,type="loess",var=F)

total(flow1,flow2,flow3)

start            end       type surr     yhat  n    r2      s cv

flow1 08/30/03 05:50 07/01/04 10:00 logxy:mvue flow 59029.70 51 0.248  1.008 NA

flow2 08/30/03 05:50 07/01/04 10:00      power flow 49969.42 51 0.265 56.318 NA

flow3 08/30/03 05:50 07/01/04 10:00      loess flow 54690.45 51 0.263 56.820 NA

# The power model is the lesser evil (has slightly better stats)

# All estimates are poor. Luckily this part of the load is quite small

total(flow2)

start            end  type surr     yhat  n    r2      s cv

flow2 08/30/03 05:50 07/01/04 10:00 power flow 49969.42 51 0.265 56.318 NA

Total load = 49969 kg

# Total non-storm load 602367 + 49969

602367 + 49969 = 652336 kg = 652 metric tons

# Total storm load = 6412559 metric tons

6413 + 652 = 7065 metric tons total annual load

# Storm load is 6413/7065 = 90.8%of annual load