FTR08 Event Loads

stn,storm,sdate,stime,edate,etime

FTR,801,080104,0240,080106,1440  656629 kg  Excellent

FTR,802,080108,0600,080111,1200   77697 kg  Good

FTR,803,080130,2230,080202,0630  876387 kg  Excellent

FTR,804,080202,0640,080206,0000  224155 kg  Fair

FTR,805,080224,0100,080227,0000  158482 kg  Fair (possibly poor)

FTR,806,080312,1850,080317,1200   88634 kg  Fair (possibly poor)

-----------------------------------------------------

SUM of events:                  2081984 kg  Very good

-----------------------------------------------------

ftr08.flo <- read.flo("ftr",08)

ftr08.lab <- read.lab("ftr",08)

mismatches("ftr",08)

ftr08.sed <- merge.flo("ftr",08)

Storm 801: dump 6-7,080104,0240,080106,1440

# Good looking stage data

# However the OR on 04Jan was a bit of an anomaly; I shifted stage by nearly -0.08,

# but not enough to completely eliminate the original stage difference of -0.118

# Very good turbidity data and outstanding sample coverage

four()

turbsscplot("ftr",08,080104,0240,080106,1440)

turbsscplot("ftr",08,080104,0240,080104,0920)

turbsscplot("ftr",08,080104,0240,080104,0920,type="loess")

turbsscplot("ftr",08,080104,0930,080106,1440)

turb0 <- turbsrc("ftr",08,080104,0240,080106,1440)

start            end   type surr     yhat  n   r2       s    cv

turb0 01/04/08 02:40 01/06/08 14:40 linear turb 647429.5 19 0.97 108.829 7.649

Total load = 647429 kg,  CV = 7.65 %

# That's a good start but we do better by modeling the hysteresis

turb1 <- turbsrc("ftr",08,080104,0240,080104,0920)

turb2 <- turbsrc("ftr",08,080104,0930,080106,0920)

ttsplot("ftr",08,turb2,turb1)

start            end   type surr     yhat  n    r2      s    cv

turb1 01/04/08 02:40 01/04/08 09:20 linear turb 229024.2  6 0.996 51.495 2.192

turb2 01/04/08 09:30 01/06/08 09:20 linear turb 419801.6 13 0.996 35.668 3.909

Total load = 648826 kg,  CV = 2.64 %

# Only drawback is the peak is discontinous where the two models meet

> endpoints(turb1)

start        end

20.85615 2274.04536

> endpoints(turb2)

start        end

1818.50760   41.55121

time0 <- lineartime("ftr",08,080104,0240,080106,1440,ssc1=21,ssc2=42)

total(time0)

start            end   type surr     yhat  n r2  s cv

time0 01/04/08 02:40 01/06/08 14:40 linear time 656628.9 19 NA NA NA

Total load = 656629 kg  Excellent

# 1.2% higher than 2-part linear model

-----------------------------------------------------

Storm 802: dump 7,080108,0600,080111,1200

# Very small event

# Good stage data, but was shifted down by 0.04 despite staff plate

# reading that agreed with electronic stage.  The shift was made because

# of the many other staff plate readings that indicated the estage was too high.

# There are also a couple of stage anomalies indicative of debris

# Turbidity looks good and sample coverage OK for an event of this size

one()

turbsscplot("ftr",08,080108,0600,080111,1200)

turb1 <- turbsrc("ftr",08,080108,0600,080111,1200)

ttsplot("ftr",08,turb1)

total(turb1)

start            end   type surr     yhat n    r2      s    cv

turb1 01/08/08 06:00 01/11/08 12:00 linear turb 77696.87 8 0.896 17.157 9.144

Total load = 77697 kg,  CV = 9.14 %  Good

-----------------------------------------------------

Storm 803: dump 9-10,080130,2230,080202,0630

# Big event with excellent data all around

# Excellent sample coverage

four()

turbsscplot("ftr",08,080130,2230,080202,0630)

turbsscplot("ftr",08,080130,2230,080202,0630,type="power")

turbsscplot("ftr",08,080130,2230,080202,0630,type="logxy")

turbsscplot("ftr",08,080130,2230,080202,0630,type="loess")

turb1 <- turbsrc("ftr",08,080130,2230,080202,0630)

turb2 <- turbsrc("ftr",08,080130,2230,080202,0630,type="power")

turb3 <- turbsrc("ftr",08,080130,2230,080202,0630,type="pairs")

endpoints(turb1)

start      end

24.60406 36.21871

time1 <- lineartime("ftr",08,080130,2230,080202,0630,ssc1=25,ssc2=36)

total(turb1,turb2,turb3,time1)
               start            end   type surr     yhat  n    r2      s    cv

turb1 01/30/08 22:30 02/02/08 06:30 linear turb 876387.4 16 0.990 54.239 2.873

turb2 01/30/08 22:30 02/02/08 06:30  power turb 899218.2 16 0.989 58.098 2.359

turb3 01/30/08 22:30 02/02/08 06:30  pairs turb 872631.0 16    NA     NA    NA

time1 01/30/08 22:30 02/02/08 06:30 linear time 903992.4 16    NA     NA    NA

# Estimates vary ty 3.6%

# time1 is a slight overestimate since it linearizes segments that are concave upward

# The pairs estimate should be quite accurate, but we don't need anything that exotic

ttsplot("ftr",08,turb1)

total(turb1)

start            end   type surr     yhat  n   r2      s    cv

turb1 01/30/08 22:30 02/02/08 06:30 linear turb 876387.4 16 0.99 54.239 2.873

Total load = 876387 kg,  CV = 2.87 %  Excellent

-----------------------------------------------------

Storm 804: dump 10,080202,0640,080206,0000

# A small event with very good stage and turbidity data

# Very good sample coverage for such a small event

# Expand to include sample 1 from tail of storm 803

four()

turbsscplot("ftr",08,080202,0640,080206,0000)

turbsscplot("ftr",08,dump=10,bot=1:8)
turbsscplot("ftr",08,dump=10,bot=1:8,type="logxy")

qsscplot("ftr",08,dump=10,bot=1:8,type="logxy")

# Odd that the relationships are so poor; suspect ISCO or lab problem

# The spreadsheet says "dump 10 out of order", not sure how to use that

turb1 <- turbsrc("ftr",08,080202,0640,080206,0000,dump=10,bot=1:8)

turb2 <- turbsrc("ftr",08,080202,0640,080206,0000,dump=10,bot=1:8,type="logxy",bias="qmle")

flow1 <- flowsrc("ftr",08,080202,0640,080206,0000,dump=10,bot=1:8,bias="qmle")

time1 <- lineartime("ftr",08,080202,0640,080206,0000,ssc1=41,ssc2=66)

total(turb1,turb2,flow1,time1)

start     end       type surr     yhat n    r2      s     cv

turb1 02/02/08 06:40 2/06/08     linear turb 220832.5 8 0.328 74.821 21.637

turb2 02/02/08 06:40 2/06/08 logxy:qmle turb 224155.0 8 0.429  0.464 18.632

flow1 02/02/08 06:40 2/06/08 logxy:qmle flow 242652.8 8 0.389  0.480 19.259

time1 02/02/08 06:40 2/06/08     linear time 243742.0 7    NA     NA     NA

# Remarkable agreement given such poor models, they vary by only 9%

ttsplot("ftr",08,turb1)

ttsplot("ftr",08,turb2)

ttsplot("ftr",08,flow1)

ttsplot("ftr",08,time1)

# Take the regression model with lowest CV

total(turb2)

start     end       type surr   yhat n    r2     s     cv

turb2 02/02/08 06:40 2/06/08 logxy:qmle turb 224155 8 0.429 0.464 18.632

Total load = 224155 kg,  CV = 18.63 %  Fair

-----------------------------------------------------

Storm 805: dump 11,080224,0100,080227,0000

# Small event with good turbidity and stage data

# There is a questionable turbidity pulse on 24Feb from 0620 to 0920

# but we see a lot of these for the remainder of HY08

# Very good sample coverage, but reach past end for bottle 17

four()

turbsscplot("ftr",08,dump=11,bot=5:17)

turbsscplot("ftr",08,dump=c(1:5,7:8,10:14),txt="dump")

# Bottles 13 and 14 have much higher than expected SSC for the given turbidity

qsscplot("ftr",08,dump=11,bot=5:17)

# Discharge or time may be a better surrogate than turbidity this time

# Let's compare models with each surrogate (turb, flow, time)

four()

turbsscplot("ftr",08,dump=11,bot=5:12,type="power")

turbsscplot("ftr",08,dump=11,bot=13:17,type="logxy")

qsscplot("ftr",08,dump=11,bot=5:14)

qsscplot("ftr",08,dump=11,bot=14:17)

turb1 <- turbsrc("ftr",08,080224,0100,080224,1830,dump=11,bot=5:12,type="power")

turb2 <- turbsrc("ftr",08,080224,1840,080227,0000,dump=11,bot=13:17, type="logxy",bias="qmle")

ttsplot("ftr",08,turb2,turb1)

# HUGE discontinuity at the change in models; this is unacceptable:

start            end       type surr      yhat n    r2     s     cv

turb1 02/24/08 01:00 02/24/08 18:30      power turb  11146.12 8 0.997 6.103  1.506

turb2 02/24/08 18:40 02/27/08 00:00 logxy:qmle turb 158004.70 5 0.948 0.439 31.657

Total load = 169151 kg,  CV = 29.57 %

flow1 <- flowsrc("ftr",08,080224,0100,080225,0500,dump=11,bot=5:14)

flow2 <- flowsrc("ftr",08,080225,0510,080227,0000,dump=11,bot=14:17,bias="qmle")

ttsplot("ftr",08,flow1,flow2)

total(flow1,flow2)

start            end       type surr      yhat  n    r2     s     cv

flow1 02/24/08 01:00 02/25/08 05:00 logxy:mvue flow 100750.71 10 0.958 0.297 13.316

flow2 02/25/08 05:10 02/27/08 00:00 logxy:qmle flow  57731.02  4 0.979 0.294 19.893

Total load = 158482 kg,  CV = 11.14 % Fair

# Finally lineartime

> endpoints(flow1)

start        end

8.401467 341.697699

> endpoints(flow2)

start       end

283.79651  23.02932

time1 <- lineartime("ftr",08,080224,0100,080227,0000,ssc1=8,ssc2=23)

ttsplot("ftr",08,time1)

total(time1)

start     end   type surr     yhat  n r2  s cv

time1 02/24/08 01:00 2/27/08 linear time 154753.4 12 NA NA NA

Total load = 154753 kg

# The three estimates are 169151(turb), 158482(flow), 154753(time)

# These vary by 9% but we are trusting the SSC on bots 13 and 14

# If there is an error, they could be WAY off and this storm would be cabbage

# My preference is the flow model but it COULD be a very poor estimate

-----------------------------------------------------

Storm 806: dump 12,080312,1850,080317,1200

# Small multi-peak affair with good stage and turbidity data

# The stage was shifted down stage-proportionally (see Data Corrections)

# Very good sample coverage, bring bot 14 from after storm

four()

turbsscplot("ftr",08,dump=12,bot=1:14)

turbsscplot("ftr",08,dump=12,bot=1:14,type="logxy")

qsscplot("ftr",08,dump=12,bot=1:14)

qsscplot("ftr",08,dump=12,bot=1:14,type="loess",degree=2)

# Bottle 11 is unexpectedly high for the given turbidity or discharge

# As in the previous event, the result will depend heavily on how much

# weight this sample is given.

# Need to look at some more models

four()

turbsscplot("ftr",08,dump=12,bot=1:10,type="loess")

turbsscplot("ftr",08,dump=12,bot=11:14,type="logxy")

qsscplot("ftr",08,dump=12,bot=1:10)

qsscplot("ftr",08,dump=12,bot=11:14)

turb1 <- turbsrc("ftr",08,080312,1850,080315,0840,type="loess")

turb2 <- turbsrc("ftr",08,080315,0850,080317,1200,dump=12,bot=11:14,type="logxy")

ttsplot("ftr",08,turb2,turb1)

total(turb1,turb2)

start            end       type surr     yhat  n    r2      s     cv

turb1 03/12/08 18:50 03/15/08 08:40      loess turb 38518.39 10 0.877 12.601     NA

turb2 03/15/08 08:50 03/17/08 12:00 logxy:mvue turb 50116.07  4 0.930  0.458 30.397

Total load = 88634 kg  Fair

flow1 <- flowsrc("ftr",08,080312,1850,080315,0840)

flow2 <- flowsrc("ftr",08,080315,0850,080317,1200,dump=12,bot=11:14)

ttsplot("ftr",08,flow1,flow2)

start            end       type surr     yhat  n    r2     s     cv

flow1 03/12/08 18:50 03/15/08 08:40 logxy:mvue flow 44272.52 10 0.741 0.423 18.008

flow2 03/15/08 08:50 03/17/08 12:00 logxy:mvue flow 58406.44  4 0.901 0.543 36.543

Total load = 102679 kg,  CV = 22.19 %

# The huge discontinuities in both of these estimates are unavoidable unless we

# regard bottle 11 as an error

# If we regard bottle 11 as an error we can do

turb0 <- turbsrc("ftr",08,080312,1850,080317,1200,dump=12,bot=c(1:10,12:14),type="power")

flow0 <- flowsrc("ftr",08,080312,1850,080317,1200,dump=12,bot=c(1:10,12:14),bias="qmle")

total(turb0,flow0)

start            end       type surr     yhat  n    r2     s     cv

turb0 03/12/08 18:50 03/17/08 12:00      power turb 64960.94 13 0.793 14.54  7.748

flow0 03/12/08 18:50 03/17/08 12:00 logxy:qmle flow 85504.89 13 0.638  0.52 20.125

ttsplot("ftr",08,turb0)

# Underestimates final recession considerably

ttsplot("ftr",08,flow0)

# Finally we could do a lineartime model

# This truncates some peaks and troughs but doesn't overweight bottle 11

endpoints(turb0)

start       end

1.357850 14.939192

time1 <- lineartime("ftr",08,080312,1850,080317,1200,ssc1=1,ssc2=15)

start            end   type surr   yhat  n r2  s cv

time1 03/12/08 18:50 03/17/08 12:00 linear time 104199 13 NA NA NA

Total load = 104199 kg

# The three estimates so far are 88634(turb), 102679(flow), 104199(time)

# Single model without bot 11: 64961(turb), 85504(flow)

# Take the middle model 88634