KRW DISCHARGE ERROR ANALYSIS HY03 to HY08

qfunkrw <- function(s) ifelse(s < 1.3255, -1.304*s + 7.883*s^2, -27.321 + 26.928*s + 2.134*s^2)

# I used this rating equation for all years at KRW

avg.err <- function(e) (exp(e) - exp(-e))/2

qkrw <- read.csv("krw/Q and rating curves/KRWratingdata0308.csv")

attach(qkrw[-21,])

plot(stg,q)

x0 <- seq(0,14.5,0.2)

y0 <- qfunkrw(x0)

lines(x0,y0,col=1)

# Develop model for log(q)

fit1 <- lm(log(q) ~ log(stg))

fit2 <- lm(log(q) ~ log(stg) + I((log(stg))^2))

fit3 <- lm(log(q) ~ log(stg) + I((log(stg))^2) + I((log(stg))^3))

y1 <- predict(fit1,newdata=data.frame(stg=x0))

y2 <- predict(fit2,newdata=data.frame(stg=x0))

y3 <- predict(fit3,newdata=data.frame(stg=x0))

lines(x0,exp(y1),col=2)

lines(x0,exp(y2),col=3)

lines(x0,exp(y3),col=4)

legend(locator(1),c(”Rating equation”,”log(q) ~ poly(1)”,”log(q) ~ poly(2)”,”log(q)  ~ poly(3)”),lty=c(1,1,1),col=1:4)

The order 3 polynomial (fit3) in log(stg) is close to the original rating.  How does it compare in terms of RMSE

q1 <- qfunkrw(stg)

q2 <- exp(predict(fit3,newdata=data.frame(stg)))

sqrt(mean((q-q1)^2))

[1] 16.46421

sqrt(mean((q-q2)^2))

[1] 18.71268

# The log approximation has slightly greater error than the original

KRW03

# There is one NA in discharge; fill it in

krw03.flo$q[36777] <- krw03.flo$q[36776]

m1 <- cbind(log(stg),log(stg)^2,log(stg)^3)

m2 <- cbind(log(krw03.flo$stg), log(krw03.flo$stg)^2, log(krw03.flo$stg)^3)

weighthalfwidth(fit3, m1, m2, krw03.flo$q)

[1] 0.1002996

avg.err(0.1002996)

[1] 0.1004679

# That probably overestimates the error slightly

0.1005*16.46/18.71

[1] 0.08841422

# Adjusted error estimate 8.80%

KRW04

m1 <- cbind(log(stg),log(stg)^2,log(stg)^3)

m2 <- cbind(log(krw04.flo$stg), log(krw04.flo$stg)^2, log(krw04.flo$stg)^3)

weighthalfwidth(fit3, m1, m2, krw04.flo$q)

[1] 0.09590604

# That probably overestimates the error slightly

0.09590604*16.46/18.71

[1] 0.08437271

# Adjusted error estimate 8.44%

KRW05

m1 <- cbind(log(stg),log(stg)^2,log(stg)^3)

m2 <- cbind(log(krw05.flo$stg), log(krw05.flo$stg)^2, log(krw05.flo$stg)^3)

weighthalfwidth(fit3, m1, m2, krw05.flo$q)

[1] 0.09032653

# That probably overestimates the error slightly

0.09032653*16.46/18.71

[1] 0.07946417

# Adjusted error estimate 7.95%

KRW06

m1 <- cbind(log(stg),log(stg)^2,log(stg)^3)

m2 <- cbind(log(krw06.flo$stg), log(krw06.flo$stg)^2, log(krw06.flo$stg)^3)

weighthalfwidth(fit3, m1, m2, krw06.flo$q)

[1] 0.09444138

# That probably overestimates the error slightly

0.09444138*16.46/18.71

[1] 0.08308419

# Adjusted error estimate 8.31%

KRW07

m1 <- cbind(log(stg),log(stg)^2,log(stg)^3)

m2 <- cbind(log(krw07.flo$stg), log(krw07.flo$stg)^2, log(krw07.flo$stg)^3)

weighthalfwidth(fit3, m1, m2, krw07.flo$q)

[1] 0.09169705

# That probably overestimates the error slightly

0.09169705*16.46/18.71

[1] 0.08066988

# Adjusted error estimate 8.07%

KRW08

m1 <- cbind(log(stg),log(stg)^2,log(stg)^3)

m2 <- cbind(log(krw08.flo$stg), log(krw08.flo$stg)^2, log(krw08.flo$stg)^3)

weighthalfwidth(fit3, m1, m2, krw08.flo$q)

[1] 0.09074786

# That probably overestimates the error slightly

0.09074786*16.46/18.71

[1] 0.07983484

# Adjusted error estimate 7.98%