SFM 2007 Annual load estimation

sfm07.flo <- read.flo("sfm",07)

sfm07.lab <- read.lab("sfm",07)

sfm07.sed <- merge.flo("sfm",07)

nonstorms <- drop.storms("sfm",07)

dim(nonstorms)

[1] 34044     4

head(nonstorms, 5)

chr     q turb turbcode

1 (09/20/06 00:10:00) 3.116   NA        2

2 (09/20/06 00:20:00) 3.116   NA        2

3 (09/20/06 00:30:00) 3.116   NA        2

4 (09/20/06 00:40:00) 3.089   NA        2

5 (09/20/06 00:50:00) 3.116   NA        2

table(nonstorms$turbcode)

0     2     5

17674 16268   102

There are 17674 values of bad turbidity out of 34044 intervals to be estimated

trb07.flo <- nonstorms[!is.na(nonstorms$turb), ]

flo07.flo <- nonstorms[is.na(nonstorms$turb), ]

dim(trb07.flo)

[1] 17776     4

dim(flo07.flo)

[1] 16268     4

17776 + 16268

[1] 34044

# Develop model for good turb data

range(trb07.flo$turb)

[1]   2 286

table(sfm07.sed$labcode2)

0   1   3   5   9

171  36   1   2   1

# Just three bad lab codes to worry about

# (5=low volume, 9=spilled > 15% of sediment)

# Eliminate bad turbidity, turbidity above 300, and labcodes 5 and 9

trb07.sed <- sfm07.sed[!is.na(sfm07.sed$turb) & sfm07.sed$turb < 300 & sfm07.sed$labcode2 < 5, ]

table(trb07.sed$turbcode)

0   5

136   6

turbsscplot("trb",07,type="loess")

turbsscplot("trb",07,type="power")

turbsscplot("trb",07,type="logxy")

turb1 <- turbsrc("trb",07,060801,0000,070731,2400,type="power",var=F)

turb2 <- turbsrc("trb",07,060801,0000,070731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",07,060801,0000,070731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",07,060801,0000,070731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat   n    r2      s cv

turb1 11/22/06 13:30 06/09/07 12:30      power turb 531761.3 142 0.901 36.812 NA

turb2 11/22/06 13:30 06/09/07 12:30 logxy:mvue turb 520378.9 142 0.931  0.309 NA

turb3 11/22/06 13:30 06/09/07 12:30      loess turb 390743.1 142 0.902 36.593 NA

turb4 11/22/06 13:30 06/09/07 12:30     linear turb 441585.0 142 0.901 36.741 NA

> sum(turb3$predssc < 0)

[1] 8885

> sum(turb4$predssc < 0)

[1] 8885

# Can't use loess or linear: too many negative predictions

# The power model or logxy model are very similar

# By far most of the predictions are for turbidity < 50, so logxy preferred

total(turb2)

start            end       type surr     yhat   n    r2     s cv

turb2 11/22/06 13:30 06/09/07 12:30 logxy:mvue turb 520378.9 142 0.931 0.309 NA

Total load = 520379 kg

# A fairly good estimate

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo07.flo$q)

[1]   0.8853 149.0000

> range(sfm07.sed$q)

[[1]   10.68 1029.00

qsscplot("sfm",07,type="loess")

The data to be estimated are below 149cfs= 4.2cumecs.

# Limit the regression data to less than 10 cumecs, and labcodes < 5

flo07.sed <- sfm07.sed[sfm07.sed$q < 350 & sfm07.sed$labcode2 < 5, ]

qsscplot("flo",07)

qsscplot("flo",07,type="power")

flow1 <- flowsrc("flo",07,060801,0000,070731,2400,type="logxy",var=F)

flow2 <- flowsrc("flo",07,060801,0000,070731,2400,type="power",var=F)

flow3 <- flowsrc("flo",07,060801,0000,070731,2400,type="loess",var=F)

flow4 <- flowsrc("flo",07,060801,0000,070731,2400,type="linear",var=F)

total(flow1,flow2,flow3,flow4)

start            end       type surr     yhat   n    r2       s cv

flow1 09/20/06 00:10 02/10/07 04:50 logxy:mvue flow 387107.9 171 0.676   0.785 NA

flow2 09/20/06 00:10 02/10/07 04:50      power flow 444951.1 171 0.416 235.565 NA

flow3 09/20/06 00:10 02/10/07 04:50      loess flow 239078.3 171 0.416 235.921 NA

flow4 09/20/06 00:10 02/10/07 04:50     linear flow 350444.5 171 0.414 235.750 NA

> sum(flow3$predssc <0)

[1] 12698

> sum(flow4$predssc <0)

[1] 12993

#Must use power or logxy to avoid negative predictions

#The logxy model looks better

start            end       type surr     yhat   n    r2     s cv

flow1 09/20/06 00:10 02/10/07 04:50 logxy:mvue flow 387107.9 171 0.676 0.785 NA

Total load = 387108 kg

==================================================

# Total non-storm load

520379 + 387108 = 907487 kg = 907 metric tons

# Quality is only fair, too much had to be estimated from discharge

# Total storm load = 6343932 kg = 6344 metric tons

# Total annual load

6344 + 907 = 7251 metric tons total annual load

# Storm load is 6344/7251 = 87.5% of annual load