FTR 2008 Annual SAND and FINES load estimation

ftr08.flo <- read.flo("ftr",08)

ftr08.lab <- read.sand("ftr",08)

ftr08.sed <- merge.flo.sand("ftr",08)

# Now create the ssc column: it will represent only FINES

ftr08.sed$ssc <- ftr08.sed$totssc - ftr08.sed$sand

dim(ftr08.flo)

[1] 32957     9

table(ftr08.flo$turbcode)

0     1     2     5

28453    11  4273   220

4273 values of bad turbidity (and 11 questionable) out of 32957 intervals to be estimated

trb08.flo <- ftr08.flo[!is.na(ftr08.flo$turb), ]

flo08.flo <- ftr08.flo[is.na(ftr08.flo$turb), ]

dim(trb08.flo)

[1] 28677	9

dim(flo08.flo)

[1] 4280	9

28677 + 4280

[1] 32957

# Develop model for good turb data

range(trb08.flo$turb)

[1]    0 1226

range(ftr08.sed$turb)

[1]  130 1209

table(ftr08.sed$labcode2)

0  1  6 NA

31  7  1 10

# Code 6 means > 15% organic debris or foreign material

ftr08.sed[ftr08.sed$labcode2 == 6, ]

chr dump bottle  sand totssc turb turbcode     q labcode1 labcode2    ssc

9 (12/04/07 00:40:00)    3      7 10.58 112.26  147        0 332.4        0        6 101.68

turbsscplot("ftr",08,txt="dump")

attach(ftr08.sed[ftr08.sed$labcode2 ==6, ])

points(turb,ssc,pch=16,cex=1.2,col=2)

# The point with code 6 is right on the line; leave it in

trb08.sed <- ftr08.sed

four()

turbsscplot("trb",08)

turbsscplot("trb",08,type="loess")

turbsscplot("trb",08,type="power")

turbsscplot("trb",08,type="logxy")

# Eliminate the egregious outliers from dumps 8 and 10

trb08.sed <- ftr08.sed[ftr08.sed$ssc > 50, ]

turbsscplot("trb",08)

turbsscplot("trb",08,type="loess")

turbsscplot("trb",08,type="power")

turbsscplot("trb",08,type="logxy")

# Try a few different models for comparison

turb1 <- turbsrc("trb",08,070801,0000,080731,2400,type="linear",var=F)

turb2 <- turbsrc("trb",08,070801,0000,080731,2400,type="loess",var=F)

turb3 <- turbsrc("trb",08,070801,0000,080731,2400,type="power",var=F)

turb4 <- turbsrc("trb",08,070801,0000,080731,2400,type="logxy",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2      s cv

turb1 10/18/07 07:50 05/04/08 12:20     linear turb 1429089 46 0.980 52.780 NA

turb2 10/18/07 07:50 05/04/08 12:20      loess turb 1030888 46 0.981 52.570 NA

turb3 10/18/07 07:50 05/04/08 12:20      power turb 1956197 46 0.978 55.494 NA

turb4 10/18/07 07:50 05/04/08 12:20 logxy:mvue turb 1762208 46 0.951  0.187 NA

turb1 <- turbsrc("trb",08,070801,0000,080731,2400,type="linear",var=F)

turb2 <- turbsrc("trb",08,070801,0000,080731,2400,type="loess",var=F)

turb3 <- turbsrc("trb",08,070801,0000,080731,2400,type="power",var=F)

turb4 <- turbsrc("trb",08,070801,0000,080731,2400,type="logxy",var=F)

total(turb1,turb2,turb3,turb4)

# Can't use linear or loess because of all the negative predictions

# The other two models vary by 11%

# The power model looks like the better fit

total(turb3)

start            end  type surr    yhat  n    r2      s cv

turb3 10/18/07 07:50 05/04/08 12:20 power turb 1956197 46 0.978 55.494 NA

Total fines load by turbidity = 1956197 kg

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo08.flo$q)

[1]  0.06277 18.79000

> range(ftr08.sed$q)

[1]   7.063 962.200

# The latter = 0.20 to 27 m3/sec, so the range is roughly appropriate

four()

qsscplot("ftr",08)

qsscplot("ftr",08,type="loess")

qsscplot("ftr",08,type="power")

qsscplot("ftr",08,type="logxy")

# Very ill-conditioned data set; try some loess models

qsscplot("ftr",08,type="loess",degree=2)

qsscplot("ftr",08,type="loess",degree=2,span=1.5)

qsscplot("ftr",08,type="loess",span=0.9)

qsscplot("ftr",08,type="loess",span=0.8)

# Need to limit the range to less than 20 cms

# And eliminate outlier bottle 4 (dump 10)

flo08.sed <- ftr08.sed[ftr08.sed$q < 700 & !(ftr08.sed$dump ==10 & ftr08.sed$bottle==4), ]

four()

qsscplot("flo",08,type="loess",degree=1)

qsscplot("flo",08,type="loess",degree=1,span=0.9)

qsscplot("flo",08,type="loess",degree=2)

qsscplot("flo",08,type="power")

flow1 <- flowsrc("flo",08,070801,0000,080731,2400,type="loess",span=1.0,var=F)

flow2 <- flowsrc("flo",08,070801,0000,080731,2400,type="loess",span=0.9,var=F)

flow3 <- flowsrc("flo",08,070801,0000,080731,2400,type="loess",degree=2,var=F)

flow4 <- flowsrc("flo",08,070801,0000,080731,2400,type="power",var=F)

total(flow1,flow2,flow3,flow4)

start            end  type surr     yhat  n    r2       s cv

flow1 09/18/07 15:40 11/03/07 13:30 loess flow 3105.911 37 0.688 132.845 NA

flow2 09/18/07 15:40 11/03/07 13:30 loess flow 4241.157 37 0.694 131.951 NA

flow3 09/18/07 15:40 11/03/07 13:30 loess flow 5230.739 37 0.692 133.907 NA

flow4 09/18/07 15:40 11/03/07 13:30 power flow   39.863 37 0.673 134.882 NA

# Why such a discrepancy?

hist(flo08.flo$q)

# The great majority of the predictions are at flow < 1 cms where there are only 5 samples

# Better use a model that goes thru the middle of those 5; i.e. one of the loess models

total(flow2)

start            end  type surr     yhat  n    r2       s cv

flow2 09/18/07 15:40 11/03/07 13:30 loess flow 4241.157 37 0.694 131.951 NA

Total fines load by discharge = 4241 kg

Total fines load = 1956197 + 4241 = 1960438 kg

==================================================

# Now compute the total annual load based on this same smaller sample

# using only the 49 bottles that had sand fractions done, minus outliers

trb08.sed$ssc <- trb08.sed$totssc

turbsscplot("trb",08)

turbsscplot("trb",08,type="loess")

turbsscplot("trb",08,type="power")

turbsscplot("trb",08,type="logxy")

turb1 <- turbsrc("trb",08,070801,0000,080731,2400,type="linear",var=F)

turb2 <- turbsrc("trb",08,070801,0000,080731,2400,type="loess",var=F)

turb3 <- turbsrc("trb",08,070801,0000,080731,2400,type="power",var=F)

turb4 <- turbsrc("trb",08,070801,0000,080731,2400,type="logxy",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr      yhat  n    r2       s cv

turb1 10/18/07 07:50 05/04/08 12:20     linear turb 1958614.5 46 0.957 112.731 NA

turb2 10/18/07 07:50 05/04/08 12:20      loess turb  854715.4 46 0.961 108.162 NA

turb3 10/18/07 07:50 05/04/08 12:20      power turb 2831646.8 46 0.954 118.332 NA

turb4 10/18/07 07:50 05/04/08 12:20 logxy:mvue turb 2392516.3 46 0.912   0.273 NA

> sum(turb1$predssc < 0)

[1] 25373

> sum(turb2$predssc < 0)

[1] 26794

# Cannot use linear or loess due to negative predictions.  Best model is power

total(turb3)

start            end  type surr    yhat  n    r2       s cv

turb3 10/18/07 07:50 05/04/08 12:20 power turb 2831647 46 0.954 118.332 NA

Total load by turbidity = 2831647 kg

flo08.sed$ssc <- flo08.sed$totssc

four()

qsscplot("flo",08,type="loess",degree=1)

qsscplot("flo",08,type="loess",degree=1,span=0.9)

qsscplot("flo",08,type="loess",degree=2)

qsscplot("flo",08,type="power")

flow1 <- flowsrc("flo",08,070801,0000,080731,2400,type="loess",span=1.0,var=F)

flow2 <- flowsrc("flo",08,070801,0000,080731,2400,type="loess",span=0.9,var=F)

flow3 <- flowsrc("flo",08,070801,0000,080731,2400,type="loess",degree=2,var=F)

flow4 <- flowsrc("flo",08,070801,0000,080731,2400,type="power",var=F)

total(flow1,flow2,flow3,flow4)

start            end  type surr     yhat  n    r2       s cv

flow1 09/18/07 15:40 11/03/07 13:30 loess flow 2275.631 37 0.733 178.092 NA

flow2 09/18/07 15:40 11/03/07 13:30 loess flow 3976.547 37 0.740 176.484 NA

flow3 09/18/07 15:40 11/03/07 13:30 loess flow 5686.549 37 0.738 179.087 NA

flow4 09/18/07 15:40 11/03/07 13:30 power flow   39.164 37 0.723 179.179 NA

# Use flow2 model for same reasons as above for the fines estimation by discharge

total(flow2)

start            end  type surr     yhat  n   r2       s cv

flow2 09/18/07 15:40 11/03/07 13:30 loess flow 3976.547 37 0.74 176.484 NA

Total load by discharge = 3977 kg

Total load from reduced sample = 2831647 + 3977 = 2835624 kg

This total compares fairly well with the previous estimate:

2612747 based on sum of storms and inter-storm loads

=====================================================

Total load 2835624

Total fines 1960438

Total sand 875186

Percent sand 30.9%

Percent fines 69.1%