FTR rating equation re-analysis: Lewis 12/13/08

Read the discharge measurements made in HY2003-2007

ftrq <- read.csv("ftr/Qmeas0307.csv")

hy         id       date  stg       q qual

1  2003  11/9/2002  11/9/2002 0.69    5.35 good

2  2003 12/14/2002 12/14/2002 2.41  166.72 poor

3  2003 12/15/2002 12/15/2002 2.04  110.89 good

4  2003 12-28-02 a 12/28/2002 5.24  814.52 fair

5  2003 12-28-02 b 12/28/2002 6.39 1089.22 fair

6  2003   1/5/2003   1/5/2003 1.60   36.81 good

7  2003  4/13/2003  4/13/2003 2.21  119.17 good

8  2004 10/12/2003 10/12/2003 0.97    0.29 fair

9  2004   1/4/2004   1/4/2004 2.00   69.67 good

10 2004  2/29/2004  2/29/2004 1.89   63.89 good

11 2005  9/19/2004  9/19/2004 0.97    0.33 good

12 2005  11/6/2004  11/6/2004 1.11    3.81 fair

13 2005   1/3/2005   1/3/2005 1.96   71.93 fair

14 2005   1/4/2005   1/4/2005 1.80   50.59 fair

15 2005   1-5-05 a   1/5/2005 1.64   29.36 good

16 2005   1-5-05 b   1/5/2005 1.64   34.26 fair

17 2005   1/6/2005   1/6/2005 1.57   24.06 fair

18 2005  3/21/2005  3/21/2005 2.25  125.47 fair

19 2005  3/22/2005  3/22/2005 2.07   93.99 good

20 2005  3/29/2005  3/29/2005 3.72  194.04 fair

21 2006  9/18/2005  9/18/2005 0.67    0.58 fair

22 2006 12/11/2005 12/11/2005 1.16   15.31 good

23 2006  3/12/2006  3/12/2006 1.83   50.26 good

24 2007  9/10/2006  9/10/2006 0.54    0.16 fair

25 2007 11/12/2006 11/12/2006 0.79    2.35 fair

26 2007 11/18/2006 11/18/2006 1.14    6.65 good

27 2007 12/29/2006 12/29/2006 1.82   40.67 good

28 2007  2/25/2007  2/25/2007 3.49  389.37 good

Plot the data using last digit of water year:

ftrq$sym <- as.character(ftrq$hy %% 1000)

attach(ftrq)

plot(stg, q, type="p", pch=sym)

Looks nice except for the 3/29/05 measurement at 3.72 ft. On the spreadsheet, there were no indications of a problem with this measurement.  Most velocities were measured at 0.2 and 0.8 depths. The measurements taken on 3/21/05 and 3/22/05 are not outliers.  And the field notes don't mention any channel changes or other clues.

I'm going to assume that the 3/29/05 measurement is an error.

detach(ftrq)

ftrq <- ftrq[-20, ]

attach(ftrq)

scatter.smooth(stg,q, type="p", pch=sym, span=0.55)

points(stg, q, pch=sym, col=2)

That's a nice fit but, for TTS adjuster and R software, we need equations.

First, let's take a look at how well Clark's equations fit the data at the low end of this.

plot(stg,q, type="p", pch=sym, col = sym, xlim=c(0,2.6),ylim=c(-10,200))

abline(0,0,lty=8)

x0 <- seq(0,2.6,0.1)

y03 <- 25.328*x0^2+18.389*x0-30

y04 <- 28.364*x0^2-2.9171*x0-11

y05 <- 28.487*x0^2-1.1462*x0-25

lines(x0,y03,col=3)

lines(x0,y04,col=4)

lines(x0,y05,col=5)

lines(loess.smooth(stg,q,span=0.55))

legend(0.25,180,c("Clark03","Clark04","Clark05","Loess03-07"),col=c(3,4,5,1),lty=1)

Clark's curves don't fit the data well in this range, not even the data for the particular year each curve was made for.

The loess curve suggests two segments with a slope break at about 1.5 ft.

Fit a segment regression using function segmented from package segmented.

# After loading "segmented" package from menu:

detach(ftrq)

attach(ftrq[ , ])

obj <- lm(q ~ stg)

segreg <- segmented(obj, Z=stg, psi=3)  # fits segmented linear regression

Call: segmented.lm(obj = obj, Z = stg, psi = 3)

Meaningful coefficients of the linear terms:

(Intercept)         stg       U.stg

-29.87975    39.06299   188.95273

Estimated Break-Point(s) for the variable(s) stg : 1.848

plot(stg,q)

abline(-29.88,36.063,col=2)

abline(-29.88-1.848*188.95, 39.063+188.95,col=2)

Fair, but curves will do better. Use a power function to keep the first segment positive.

Use a quadratic for the second segment.  Split the data at stg=1.75.

fit1 <- nls(q ~ a*stg^b, data=ftrq, subset=stg < 1.75, start=list(a=1,b=1))

a        b

4.400749 4.094276

x1 <- seq(0,1.75,0.05)

y1 <- 4.401*x1^4.094

plot(stg,q)

lines(x1,y1,col=2)

fit2 <- lm(q ~ stg + I(stg^2), data=ftrq, subset=stg > 1.75)

(Intercept)          stg     I(stg^2)

-290.705      172.608        6.911

x2 <- seq(1.75,6.5,0.05)

y2 <- -290.7 + 172.61*x2 + 6.911*x2^2

lines(x2,y2,col=2)

Looks very good. Just need a slight refinement to improve the intersection (not visible at scale shown here).

Include 3 more points in fit1 data set:

fit1 <- nls(q ~ a*stg^b, data=ftrq, subset=stg < 1.85, start=list(a=4,b=4))

a        b

5.012794 3.770439

Find the intersection of the two models

uniroot(

function(x) 5.012794*x^3.770439 - (-290.705+172.608*x+6.911*x^2),

interval=c(1.5,2)

)

$root

[1] 1.836462  # Stage height at intersection of the curves

Replot the data

plot(stg,q,main="FTR HY03-07",xlab="stage (ft)",ylab="discharge (cfs)")

# Add the loess smooth in green and the piecewise curves in red for comparison

lines(loess.smooth(stg,q,span=0.55),col=3)

x1 <- seq(0,1.85,0.05)

y1 <- 5.013*x1^3.770

lines(x1,y1,col=2)

x2 <- seq(1.85,6.5,0.05)

y2 <- -290.7 + 172.61*x2 + 6.911*x2^2

lines(x2,y2,col=2)

legend(1.25,950,c("2-pc power/quadratic","loess (span=0.55)"),col=2:3,lty=1)

# Zoom in on lower end

plot(stg,q, type="p", pch=sym, col = sym, xlim=c(0,2.6),ylim=c(-10,200))

abline(0,0,lty=8)

x0 <- seq(0,2.6,0.1)

y03 <- 25.328*x0^2+18.389*x0-30

y04 <- 28.364*x0^2-2.9171*x0-11

y05 <- 28.487*x0^2-1.1462*x0-25

lines(x0,y03,col=3)

lines(x0,y04,col=4)

lines(x0,y05,col=5)

lines(x1,y1,col=2)

lines(x2,y2,col=2)

legend(0.25,180,c("Clark03","Clark04","Clark05","Lewis03-07"),col=c(3,4,5,2),lty=1)

This is quite an improvement.  There is scant evidence that we should use different rating curves each year.

Finally compare these curves at full scale.

plot(stg,q, type="p", pch=sym, col = sym, cex=0.75)

abline(0,0,lty=8)

x0 <- seq(0,6.5,0.1)

y03 <- 25.328*x0^2+18.389*x0-30

y04 <- 28.364*x0^2-2.9171*x0-11

y05 <- 28.487*x0^2-1.1462*x0-25

lines(x0,y03,col=3)

lines(x0,y04,col=4)

lines(x0,y05,col=5)

lines(x1,y1,col=2)

lines(x2,y2,col=2)

legend(1,1000,c("Clark03","Clark04","Clark05","Lewis03-07"),col=c(3,4,5,2),lty=1)

It would be useful to collect more discharge measurements above 3 ft.