Quartz Valley Indian Reservation

Quality Assurance Project Plan

for Water Quality Sampling and Analysis

CWA 106 grant identification # I-96927206-0

Prepared by

Tribal Environmental Protection Department

Quartz Valley Indian Reservation

13601 Quartz Valley Road

Fort Jones, CA  96032

Technical Assistance Provided by both:

Kier Associates

&

Rebekah Sluss

Quartz Valley Indian Reservation

Quality Assurance Project Plan

Water Quality Sampling and Analysis

CWA 106 grant identification # I-96927206-0

Approval Page

Quartz Valley Indian Reservation

13601 Quartz Valley Road

Fort Jones, CA  96032

This QAPP has been approved by:

__________________________________________	Date:____________

Crystal Bowman

QVIR Environmental  Director

__________________________________________	Date:_____________

Harold Bennett

QVIR Vice Chairman

| For EPA use: |  |
| --- | --- |
| Approved by EPA Project Manager: __________________________________________ Janis Gomes CWA 106 Project Officer | Date:___________ |
| Expedited Review? ___ Yes | ___ No |
| Received by QA Office: __________________________ | Date: ________________________ |
| Reviewed by: __________________________ | Date: ________________________ |
| Approved: __________________________ Eugenia McNaughton Region 9 Quality Assurance Manager | Date: ________________________ |

  - Table of Contents

1.0 PROJECT MANAGEMENT ................................................ 1-68

1.1 	Title and Approval Page ............................................ 1-2

1.2	Table of Contents ................................................. 3-7

1.3	Distribution List ...................................................  8

1.4	Project Organization ............................................... 9-11

1.5 	Problem Definition/Background ......................................11-30

1.6	Project/Task Description and Schedule .............................. 31-51

1.7 	Quality Objectives and Criteria for Measurement Data .................. 51-61

                - Objectives and Project Decisions ............................51-52

Action Limits/Levels .......................................53-55

Measurement Performance Criteria/Acceptance Criteria .........56-61

1.8 	Special Training Requirements/Certification ..........................  62

1.9 	Documents and Records .......................................... 62-68

Field Documentation and Records ...........................62-65

Laboratory Documentation and Records ...................... 65-66

Technical Reviews and Evaluations .......................... 66-67

Biannual and Annual Reports ...............................67-68

2.0 DATA GENERATION AND ACQUISITION ...................................68-96

2.1	Sampling Design (Experimental Design) ..............................69-72

2.2 	Sampling Methods ................................................ 72-78

Surface Water Sampling ....................................72-74

Field Health and Safety Procedures ..........................74

Field Measurements .......................................74

Field Variances ...........................................75

Decontamination Procedures ...............................75

Disposal of Residual Materials ..............................75-76

Quality Assurance for Sampling ............................. 76-78

2.3 	Sample Handling and Custody ......................................79

Sample Containers & Preservatives ..........................79

Sample Packaging and Shipping ............................79-80

Sample Custody ..........................................80-81

Sample Disposal ..........................................81

2.4 	Analytical Methods ............................................... 81

Field Measurement Methods ................................ 81

Laboratory Analyses Methods (Off-Site) ......................81

2.5 	Quality Control Requirements ...................................... 82

Field Sampling Quality Control .............................. 82

Field Measurement Quality Control ........................... 84

Laboratory Analyses Quality Control (Off-Site) ................. 84

Background Samples ......................................86

2.6 	Instrument/Equipment Testing, Inspection, and Maintenance ............ 91-96

Field Measurement Instruments/Equipment ................... 91

Laboratory Analysis Instruments/Equipment .................. 91

2.7 	Instrument/Equipment Calibration and Frequency ..................... 91

Field Measurement Instrument/Equipment ....................91

Laboratory Analysis Instruments/Equipment .................. 91

2.8 	Inspection and Acceptance of Supplies and Consumables .............. 95-96

Field Sampling Supplies and Consumables ................... 95

Field Measurement Supplies and Consumables ................95

Laboratory Analyses (Off-Site) Supplies and Consumables ......95

Data Acquisition Requirements (Non-Direct Measurements) ............. 96

2.10 	Data Management ...............................................96

3.0 ASSESSMENT AND OVERSIGHT ........................................96-98

3.1 	Assessment/Oversight and Response Actions ........................96

Field Oversight ...........................................96

Readiness Reviews .................................96

Field Activity Audits ................................97

Post Sampling Event Review .........................97-98

Laboratory Oversight ......................................98

Reports to Management ...........................................98

4.0 DATA REVIEW AND USABILITY ...........................................99-102

4.1 	Data Review, Verification, and Validation Requirements ................99

Field Sampling and Measurement Data ....................... 99

Laboratory Data .......................................... 99

4.2 	Verification and Validation Methods .................................101-102

Field Sampling and Measurement Data .......................101

Laboratory Data .......................................... 102

Reconciliation with User Requirements .............................. 102

- REFERENCES .............................................................................................................................103-104

List of Tables

Table 1.5.1 Lakes and Headwaters of Shackleford Mill………………..12

Table 1.5.2 Top 10 Pesticides in Shackleford/Mill and Scott River……………….17

Table 1.5.3 Groundwater Testing Results……………….21

Table 1.5.4 Surface Water Testing Results……………….22

Table 1.5.5 Surface Water Field Parameters………………..22

Table 1.5.6 Scott River Ongoing Research and Monitoring……………….25-30

Table 1.6.1 QVIR Monitoring Locations and Sample ID……………….34-35

Table 1.6.2: Sample locations and parameters……………….43-46

Table 1.6.3:  Sample Locations, Rationale, & accessibility for QVIR water quality monitoring program……………….47-52

Table 1.7.1: Water Quality Parameters and Action Levels………………..55-57

Table 1.7.2 Precision of sampling equipment used by the QVIR EPD………………..62-63

Table 2.2.1. Required sample containers, volumes, preservation methods and holding times for water samples requiring laboratory analysis……………..79-80

Table 2.5.1: Summary of Field and QC Samples Water Monitoring Program

Quartz Valley Indian Reservation……………….89

Table 2.5.2:  Quality Control Requirements for Surface Water Field Measurements…...91-92

Table 2.7.1: Field Equipment Calibration, Maintenance, Testing and Inspection……....94-96

List of Figures

Figure 1	Water Quality Program organization………………..11

Figure 2	Scott River Basin………………..13

Figure 3.1	Map of Sample Locations- Shackleford-Mill Sub basin………………..36

Figure 3.2	Map of Sample Locations- Shackleford-Mill Sub basin………………..37

Figure 4	Map of Sample Locations- Scott River Canyon………………..38

Figure 5	Map of Sample Locations – Scott River East Fork……………….39

Figure 6	Map of Sample Locations – Scott River South Fork………………..40

Figure 7	Schedule for Implementation………………..53

List of Appendices

Appendix A: Shackleford Creek Limiting Factors

Appendix B: ECORP WQ Study Documents

Appendix B1:  Prior groundwater test results before ECORP BWA

Appendix B2: ECORPs SAP

Appendix B3: CLS Lab QA information

Appendix B4: Complete copy of lab analytical report for ECORP testing

Appendix B5: Habitat unit characterization from ECORP

Appendix B6: Flow Measurement from ECORP

Appendix C: Laboratory Documents

Appendix C1: Sample Labels from Labs

Appendix C2: Sample Chain of Custody and Custody Seals

Appendix C3: Labs’ & Consultant QA information

Appendix C3-1: Jon Lee Consulting

Appendix C3-2: North Coast Laboratories

Appendix C3-3: Aquatic Analysts

Appendix D: QVIR Water Quality Checklists and Worksheets

Appendix D1: Field Activities Review Checklist

Appendix D2: Laboratory Data Review Checklist

Appendix D3: Field Water Quality Data Sheet

Appendix E: Field Equipment Manuals and Instructions

Appendix E1: Trimble GEO Explorer

Appendix E2: AquaCalc 5000 Open Channel Flow Computer

Appendix E3: Onset HOBO Water Temp Pro Loggers
	Appendix E4: Van Dorn Sample Bottle Instructions

Appendix E5: YSI 556 MultiProbe System Manual

Appendix E6: YSI 6600 EDS MultiProbe System Manual

Appendix E7: Model WQ770 Turbidity Meter

Appendix F:  Existing Protocols

Appendix F1: Rapid Bioassessment Protocols

Appendix F2: NCWAP Methods Manual

Appendix G: Existing Water Quality Standards

Appendix G1: Basin Plan MCL Tables

Appendix G2: EPA Surface Water Quality Standards

1.3  Distribution List

The following is a list of individuals who will receive copies of the approved QAPP and any subsequent revisions or changes.

| Crystal Bowman QVIR Environmental Director Quartz Valley Indian Reservation 13601 Quartz Valley Road Fort Jones, CA 96032 Ph: 530-468-5907 Fax: 530-468-5908 crystalqvir@yahoo.com | Tim Wilhite GAP Project Officer USEPA c/o Klamath Nat’l Forest 1312 Fairlane Road Yreka, CA 96097-9549 Ph: 530-841-4577 Fax: 530-841-4571 Wilhite.Timothy@epamail.epa.gov |
| --- | --- |
| QVIR Environmental Department Staff Quartz Valley Indian Reservation 13601 Quartz Valley Road Fort Jones, CA 96032 Ph: 530-468-5907 Fax: 530-468-5908 | Eugenia McNaughton, Manager Region 9 Quality Assurance Office U.S. EPA Region 9 75 Hawthorne Street Mail Code: MTS-3 San Francisco, CA 94105 (415) 972-3411 mcnaugton.eugenia@epa.gov |
| QVIR Tribal Council Quartz Valley Indian Reservation 13601 Quartz Valley Road Fort Jones, CA 96032 Ph: 530-468-5907 Fax: 530-468-5908 | Jon Lee Jon Lee Consulting 2337 15th Street Eureka, CA 95501 707-441-9347 jlee@humboldt1.com |
| Janis Gomes CWA 106 Project Officer USEPA REGION 9 75 Hawthorne Street Mail Code: WTR-10 San Francisco, CA 94105 Ph: 415-972-3517 gomes.janis@epa.gov | North Coast Laboratories Ltd. 5680 West End Road Arcata CA 95521-9202 Ph: 707-822-4649 Fax: 707-822-6831 www.northcoastlabs.com |
| Aquatic Research Inc. 3927 Aurora Ave. N Seattle, WA 98103 Ph: 206-632-2715 info@aquaticresearchinc.com | Jim Sweet Aquatic Analysts Salmon, WA Ph: 509-493-8222 jwsweet@aol.com |

1.4 Project Organization

| Title/Responsibility | Name | Phone Number |
| --- | --- | --- |
| EPA Project Manager | Janis Gomes CWA 106 Project Officer | (415) 972-3517 |
| QVIR Project Manager | Crystal Bowman, QVIR Environmental Director | (530) 468-5907 |
| Quality Assurance Manager QVIR | Crystal Bowman QVIR Environmental Director | (530) 468-5907 |
| Lab - Organics | Aquatic Research Inc. | (206) 632-2715 |
| Lab- Phytoplankton | Jim Sweet Aquatic Analysts | (509) 493-8222 |
| Lab- Pesticides | North Coast Laboratories Ltd. | (707) 822-4649 |
| Consultant | Jon Lee Jon Lee Consulting | (707) 441-9347 |
| QVIR Data Manager | Kayla Super QVIR Directors Assistant | (530) 468-5907 |
| QVIR Field Technicians | Vacant Position QVIR CWA106 Coordinator Tonya Lindsey QVIR Water Rights Coordinator | (530) 468-5907 (530) 468-5907 |

- Project Organization

The Quartz Valley Indian Reservation’s Tribal EPD (QVIREPD) is completing this QAPP to define how quality control (QC) procedures are implemented and to define how the QVIREPD and its staff will work together on quality assurance (QA) to insure that data are properly collected and analyzed, managed and stored for on-going use, and results published in a timely fashion. Because of the systematic planning process documented in this QAPP, the QVIREPD Water Quality Monitoring Program will supply quality assured data for management decisions related to the aquatic environment within QVIR jurisdiction and the Scott River watershed.

The QVIR Water Quality Monitoring Program is organized as shown in Figure 1. The QVIR Environmental Director has ultimate control over and responsibility for the WQ program, and acts as both the QA Officer and Project Manager.  The QVIR Environmental Director is responsible for program coordination, schedule and budget management, technical oversight, report preparation, and overall program quality.

The QA Officer will have responsibility and authority for:

  - Ongoing review of monitoring methods and equipment calibration,

  - Auditing field notebooks, databases, chain of custody forms, and

  - Insuring adherence to field and laboratory QA/QC programs.

In short, the QA Officer will insure that QC procedures developed in this QAPP are carried out. The Data Manager and Water Quality Technicians will work under the supervision of the QA Officer and follow procedures as defined in this QAPP.

The Data Manager will:

  - Transfer results from the field or laboratory into databases,

  - Properly store data and archive to insure against loss,

  - Run preliminary analysis of data, and provide charts for reports, and

  - Assist with report preparation.

The WQ Technicians will:

  - Collect field samples

  - Fill out forms to record results and field conditions,

  - Care for and calibrate equipment,

  - Properly fix and ship samples needing laboratory analysis,

Any time there are problems perceived by the Data Manager or the WQ Technician with any part of the WQ Monitoring Program, they are to notify the QVIR Project Manager so they can work collaboratively on resolving them. The QA Officer will also periodically conduct audits to detect QA/QC problems or deficiencies.

If any tests of surface or groundwater exceed action levels, then the QVIR Environmental Director will be notified so that she can inform the QVIR Tribal Council.  Following notification of the Tribal Council, the QVIR EPD would then inform the North Coast Regional Water Quality Control Board staff and work cooperatively with that agency for abatement of problems.

The QVIR EPD will send water quality samples needing laboratory analysis to Aquatic Research Inc., an accredited laboratory by the U.S. EPA. Water samples tested for pesticides will be sent to North Coast Laboratories, Ltd. A benthic macro-invertebrate consulting firm, Aquatic Analyst, will provide technical assistance in the sampling and identification of aquatic macro-invertebrates. Phytoplankton and algae samples will be sent to Jim Sweet to be processed and analyzed.

QVIR Program Environmental Director/

Quality Assurance Officer

QVIR Data Manager

QVIR Field Technicians

Consultant

Aquatic Research Inc.

North Coast Laboratories, Ltd.

Figure 1

1.5 	Problem Definition/Background

The Quartz Valley Indian Reservation lies in a rural, sparsely populated area within the Scott River watershed in the Klamath Mountain Province, and is one of four major tributaries of the Klamath River, contributing about 5% of the entire Klamath’s runoff (yearly average of 615,000 acre feet). The Scott River watershed is a large area with substantial variation in geology, geomorphology, and climatology. The watershed drains approximately 520,617 acres (812.2 mi2 or 2,107 km2). Major tributaries to the 58 mile long Scott River in the Scott Valley include: Shackleford / Mill, Kidder, Etna, French, and Moffett Creeks, and also the South and East Forks (SRWC SAP 2005).

Quartz Valley Indian Reservation has several streams on tribal land. Shackleford, Shackleford-Mill, and Sniktaw Creeks all run through the Reservation.

The headwaters of the largest creeks, Shackelford and Mill are in the Marble Mountains, a 242,500 acre Wilderness Area on the western edge of Scott Valley.  Campbell, Cliff, and Summit Lakes are the headwaters of Shackleford Creek.  The headwaters of Mill Creek consist of the two Mill Creek ponds.  The size, elevation and depth of each lake are listed in Table 1.5.1.   The Tribe has interest in the health of these aquatic ecosystems because of their role in producing cold water fish and other culturally significant flora and fauna.  Chinook and coho salmon as well as steelhead trout return to these creeks to spawn and rely on a healthy Scott River for juvenile rearing and adult migration.

| Table 1.5.1 : Lakes at Headwaters of Shackleford-Mill Creeks (Data from the Klamath National Forest |  |  |  |
| --- | --- | --- | --- |
| Lake | Elevation | Acres | Depth |
| Campbell | 5800’ | 33 | 3’ |
| Cliff | 6100’ | 52 | 175’ |
| Summit | 6050’ | 5 | 15’ |
| Summit Meadow | 6050’ | 1.3 | 4’ |
| Mill Creek (West) | 6450’ | 4.5 | 8’ |
| Mill Creek (East) | 6350’ | 1.5 | 15’ |

The Scott River was listed as sediment and temperature impaired under Section 303(d) of the Clean Water Act by the North Coast Regional Water Quality Control Board and the U.S. Environmental Protection Agency (EPA) in 1997.   Total Maximum Daily Loads (TMDL’s) were recently approved by the Regional Water Board (December 2005), the State Water Board (June 2006) and the US EPA (September 2006).  Water quality conditions have affected the habitat of anadromous salmonids populations in the Scott River watershed. Coho salmon in the region were listed as threatened under the federal Endangered Species Act (ESA) in 1997 by the National Marine Fisheries Service (NMFS) and by the California Fish and Game Commission in 2002.  Additionally, the Klamath River is also listed for temperature, nutrients, and dissolved oxygen; the Klamath TMDL is due to be completed by January 2007.

(SRWC SAP 2005)

Figure 2 Scott River Basin

Decline of the fishery

Historically, spring-run Chinook salmon were abundant in the rivers of the Klamath Basin, considerably outnumbering the fall Chinook run (Hume in Snyder 1931). “Salmon ascend the river in large numbers, before the waters subside in the spring”, remarked Gibbs in 1851 (SRWC SAP 2005).  Fall Chinook, winter and summer steelhead were also widespread in the Scott River Basin. (Maria, personal communication in SRWC SAP 2005).  Today, the spring Chinook and summer steelhead run is virtually nonexistent in the Scott River (KRBFTF, 1991. p. 2-87, 2-99, and 4-15; USFS, 2000b, p.3-9; USFS, 2000a).   Data indicate that the fall Chinook population within the Scott River basin has experienced a decline since at least the 1960s (Hardy and Addley, 2001, p.12).  The Scott River produces approximately 9.2% of the natural fall Chinook salmon in the Klamath River basin (SRWC, 2004, p.6-1).

Coho salmon would have flourished in the numerous ponds created by beavers throughout the valley and in both forks of the Scott River (SRWC SAP 2005 & Belchik, personal communication).  Brown et al. (1994) state that California coho populations are probably less than 6% of what they were in the 1940s, and there has been at least a 70% decline since the 1960s. Coho salmon occupy only 61% of the SONCC Coho ESU streams that were previously identified as historical coho salmon streams (CDFG, 2002, p.2)

Land Use Factors

Consideration of factors limiting salmon and steelhead production, water quality and attainment of other beneficial uses in Shackleford and Mill Creeks must be tiered.  There are some over-arching factors, such as flow depletion, that can then cause secondary water quality problems as transit time increases and stagnation of water occurs.  Limiting factors are most often linked to two current land use activities; logging and agriculture, although mining also occurred in the past.

Historically, gold was mined in the Quartz Valley area, both in the valley itself and in the Scott and Oro Fino valleys nearby. The type of mining performed in Northern California during the late 1800s was hydraulic mining, not chemical (like cyanide-leach mining), so less chemical contamination is associated with it. Surface and groundwater in the Quartz Valley could potentially be contaminated with heavy metals that naturally occur in association with gold but are discarded in tailings, like arsenic. Dredge tailings from hydraulic mining can also serve as a source of sediment pollution.

Much of the land in Siskiyou County was logged, beginning in the latter half of the 19th century. Erosion due to clear-cutting and logging roads in the area, whether still used and maintained or abandoned, could contribute significant amounts of sediment to the Shackleford Creek and Scott River system.

Beginning around 1850, ranching became the most prevalent use of land around the Quartz Valley. Grazing of cattle is still performed by many landowners on the valley floors in the Scott River watershed, which could contribute to erosion of streams and bacterial contamination of surface waters when cattle are permitted access to streams. Land around the upper reaches of Shackleford Creek has historically been used for cattle grazing during the summers, which could contribute fecal coliforms to the surface water. Land in the Quartz Valley floor is also used for agriculture, which may contribute contaminants such as pesticides, nitrates, and phosphates to the surface water.

Private industrial timber lands border the Reservation and Shackleford Creek.  This activity could result in herbicide and pesticide contamination of surface and ground water.  Fruitgrowers, one of the largest owners of private land in Siskiyou County, has been unwilling to share data or studies they have conducted pertaining to Shackleford Creek. They have since relinquished their west side Scott River holdings to Timber Vest. QVIR Environmental Director met with Timber Vest in December of 2006 and has established a working relationship which will includes data sharing and property access for QVIR EPD.

On Reservation land, no grazing or agriculture is performed. The area around Shackleford Creek and the Reservation does contain paved roads and has vehicle traffic. Oils and other contaminants likely to come from automotive traffic may be present.

No heavy industry is present in the area, so organic contaminants from pesticide manufacture, petroleum refinement and other industrial applications are unlikely.

Logging and Roads:  Upland areas within the Shackleford Creek watershed are extensively logged and have high road densities (see Appendix A).  Compaction of soils and changes in routing of storm water on logged areas and logging roads are known to:

- Increase peak discharge (Montgomery and Buffington, 1993; Jones and Grant, 1996),

- Increase sediment yield (Hagans et al., 1986, de la Fuente and Elder, 1998), and

- Decrease large wood available for recruitment to streams (Reeves et al., 1993; Schuett-Hames et al., 1999).

The potential changes in aquatic conditions related to upland disturbance are described below, while the description of conditions in Shackleford Creek uplands based on GIS and other data can be found in Appendix A.

Increased Peak Flows: Elevated peak discharge can increase median particle size distribution to those greater than optimal for salmonid use, wash out large wood, and trigger bank failures and channel scour.  Channel changes can include decreased pool frequency and depth (Buffington and Montgomery, 1993).  Wider and shallower channels also are more subject to warming.  Although less well studied, hydrologic changes associated with compaction of a watershed can also lead to decreased summer base flows.

Increased Sediment Yield:  Sediment yield is a noted problem in the Scott River watershed (NCRWQCB, 2003; 2005).  Fine sediment comes primarily from surface or gully erosion and Sommarstrom et al. (1990) identified road cuts and road fills on decomposed granitic soils as a major source of fines in the Scott River watershed.

Fine Sediment:  High levels of sand and fine sediment can fill interstitial spaces in stream gravels, decrease salmonid egg and alevin survival and reduce aquatic insect habitat.  Decreased aquatic invertebrate production can diminish food resources for juvenile salmonids.  Smaller sediment particles are highly mobile and may cause diminished pool frequency and depth, thus reducing salmonid juvenile carrying capacity and adult salmonid holding habitat.

Mass Wasting:  The coarse and fine sediment yielded by mass wasting can cause channel aggradation, loss of pool habitat, changes in median particle size, decreased spawning gravel quality and channel adjustments that facilitate stream warming.

Large Wood Depletion:  Changes in riparian conditions can increase ambient air temperature over streams and reduce relative humidity, thus leading to stream warming (Bartholow, 1989; Pool and Berman, 2001).  Cold air moving down slope from Marble Mountain peaks in winter may also cause elevated risk for the formation of anchor ice along streams where canopy is lacking.  Pools formed by large wood are extremely important as nursery areas for coho salmon juveniles (Reeves et al., 1988) that must spend one year in freshwater before migrating to the ocean.  Large wood depletion can, therefore, cause diminished aquatic habitat complexity, reduced pool frequency and lower carrying capacity for juvenile coho.  Large coniferous trees in riparian zones may take decades or centuries to grow to sufficient size to be useful in buffering air temperatures and providing wood of sufficient size to provide lasting habitat value (Shuett-Hames et al., 1999).

Agricultural Water Diversion: Flow depletion in Shackleford Creek due to water extraction for agriculture causes warming as water volume is reduced and loss of surface flow. Decreasing flows may cause riffles to become shallow or the formation of isolated pools.  Growth of periphyton covering stream substrate will increase with warming water temperatures and would also be increased by nutrient rich agricultural return water.  High rates of photosynthesis by algae in low flow conditions can cause large nocturnal and diurnal fluctuations in pH and dissolved oxygen.  The secondary effects related to high photosynthetic activity in stagnant, de-watered reaches are not targeted because loss of flow is an over-riding impact.

Pesticides:  Many of the leading pesticides used in Scott Valley and the Shackleford/Mill Creek drainage are herbicides that have no recognized levels of exposure for human health risk set by the U.S. Environmental Protection Agency or the State of California (EDW, 2005).  However, two of the pesticides used in Scott Valley were recently acknowledged, July 2006, by the US EPA to be harmful to salmonids.  Table 1.5.2 show the top pesticides used in the Shackleford/Mill and Scott River basins, respectively with those harmful to salmon and steelhead in bold.

| Table 1.5.2. Top ten pesticides used in the Shackleford-Mill Creek watershed and the Scott River watershed from 1990 to 2004. Data from the California Pesticide Use Reporting Database. |  |  |
| --- | --- | --- |
| Use Rank | Shackleford/Mill | Scott River |
| 1 | Paraquat Dichloride | Paraquat Dichloride |
| 2 | Trifluralin | Hexazinone |
| 3 | Hexazinone | Diuron |
| 4 | Metribuzin | Glycophosphate |
| 5 | Glycophosphate | 2,4-D Dimethylamine Salt |
| 6 | 2,4-D Dimethylamine Salt | Metribuzin |
| 7 | 2.4-D Butoxyethanol Ester | 2.4-D Butoxyethanol Ester |
| 8 | Norflurazon | Trifluralin |
| 9 | MCPA, Dimethylamine Salt | 2,4-D, Isooctyl Ester |
| 10 | Atrazine | Chloropyrifos |

Purpose of Water Quality Investigations:

What was once a historically productive fishery in Scott Valley has declined to numbers precluding tribal members from utilizing their fishing rights on Reservation waters and limiting their take for sustenance throughout the Scott River Valley.  The Indian people of Quartz Valley traditionally depended on the land and waters to provide for their physical and cultural needs.  The state of the watershed today prevents this dependency.  The Tribe’s priority is a restored watershed that supports healthy populations of salmonids.  This water quality study is a first step in understanding conditions in areas that have not been studied which may have contributed to population decreases of anadromous salmonids.  It is also an opportunity to work collaboratively with other agencies and tribes to share information and to ultimately restore the watershed to historical conditions.

The goal of this QAPP is to provide the Quartz Valley Indian Community with a quantitative assessment of the water quality of the resources effecting the Reservation to further expand the Tribe’s scientific knowledge for tribal members, fisheries, future planning, and watershed activities.   Additionally, these analyses will help identify any surface water contamination problems that could affect fish habitat, since wild salmon are an important resource to the Tribe and a vital piece of the Tribe’s cultural heritage.

This QAPP will be used to develop baseline information in order to document water quality changes over time, screen for potential water quality problems, and to provide a scientific foundation in order to actively participate in the management of the Shackleford-Mill Creek watershed and the broader Scott River Watershed.

Principal data users/decision makers who will use the data to make decisions:

The first step to attainment of the goal of this QAPP is baseline data collection for water bodies in the Reservation’s watershed.  Quality assured water quality data collected by the QVIR EPD will be used in management of the Shackleford-Mill Creek watershed. Data will be shared with the U.S. EPA and NCRWQCB staff through timely reports on findings, including for use in TMDL updates. Other agencies and entities cooperating in Shackleford-Mill Creek watershed management, including the Klamath National Forest, may also receive QVIR EPD data after it has undergone QA/QC and analysis. The QVIR EPD will also share data with tribal members through annual reports and with the public upon request.

Brief Summary of existing information:

Ground water has been sampled for the QVIR residents in the past as a part of routine well installation and maintenance. Records of well water analysis for the wells are few, and lab analysis Quality Assurance information is not known. Prior groundwater test results are included in Appendix B1.

Ground and surface water were sampled in 2005 by QVIR environmental staff and ECORP Consulting as part of a preliminary Baseline Watershed Assessment. Shackleford Creek was tested using hand-held field meters for dissolved oxygen, pH, turbidity and temperature. These parameters were chosen due to their importance to salmonid habitat. Samples of water were also taken and analyzed for phosphates, nitrates, organochloride pesticides, heavy metals, and some volatile organic chemicals.  Test results are compiled in Table 1.5.3 through Table 1.5.5.  ECORPs SAP and the Lab’s SOP are included in the Appendix B.

Recent fish surveys have confirmed the presence of Chinook and coho salmon in the Scott River, and report that the Scott River has the largest number of native coho of the Lower Klamath River tributaries. These surveys are unable to provide information on the decline in salmonid populations during the 20th century.

In recent years, Geographic Information Systems (GIS) technology and other remote sensing methods have contributed to the knowledge of the basin. Geologic and sediment study data are available for the greater Scott River basin, some of which is applicable to or includes specific information about the Quartz Valley. GIS datasets may also cover the Quartz Valley, but to date, few analyses performed with this data have focused on the Quartz Valley. Currently, the Scott River TMDL development work is utilizing the available GIS data for the watershed, but recent changes to land use, forestation, or roads may still not be reflected in the data available.

Quantitative testing was previously performed on QVIR water resources only for newly drilled wells, which were tested for the presence or absence of coliform bacteria, in accordance with Siskiyou County health regulations.

In February 2005, a preliminary assessment of the QVIR groundwater and surface water resources was conducted with the assistance of ECORP Consulting. The assessment had two components: chemical analysis, for both surface and groundwater, and an assessment of fish and wildlife habitat in the QVIR reach of Shackleford Creek. Funding for this assessment was obtained through the Bureau of Indian Affairs, by the QVIR Tribal EPD office.  During these tests, 15 wells on QVIR land were tested and samples were taken from four stream sites.

Groundwater Results

In groundwater, A suite of 12 metals was tested, which included mercury, antimony, barium, beryllium, cadmium, chromium, copper, iron, arsenic, lead, selenium and thallium. Copper, iron and barium were detected.  In two wells, iron concentrations exceeded the proposed limit, found in the US EPA secondary drinking water standards.  Copper was detected in samples from all the wells on the main reservation. Although it did not exceed the standard proposed above, it was the only analyte of note detected in the samples. Because it occurred in every well, it is likely the copper is naturally occurring in the water bearing rock. A complete copy of the lab analytical report is included as Appendix B4.

The other analytes tested in groundwater during the preliminary sampling event were total coliforms, fecal coliforms, chlorine dioxide, chlorite, and chloramines. Field parameters were measured with the YSI Multiparameter, but many readings were of poor data quality. A calibration check after the sampling event showed the YSI unit to be out of calibration. As a result, the readings taken during this sampling event do not provide a good baseline for temperature, conductivity, TDS and pH. In addition to the analytes listed above, an abandoned well was tested for hexane extractable materials (HEM), which result from contamination with petroleum products such as oil or gasoline. Empty 55-gallon drums found near the abandoned well were thought to have previously contained petroleum products. No HEMs were detected.

Two wells had a total coliform count out of compliance with the standards of the Siskiyou County Health office. These two wells tested positive for total coliforms, though neither tested positive for fecal coliforms. No other wells were found to have either total coliforms or fecal coliforms.

| Table 1.5.3: Groundwater Analysis Results |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling Site | Coliforms | Metals | Concentration in micrograms/L | Nitrate mg/L | Nitrite | Chlorines | Oils and Grease |
| Well #1 | Total -/Fecal - | Copper | 18 | n/a | n/a | ND | n/a |
| Well #2 | Total -/Fecal - | Copper | 14 | n/a | n/a | ND | n/a |
| Well #3 | Total -/Fecal - | Copper | 16 | n/a | n/a | ND | n/a |
| Well #4 | Total -/Fecal - | Copper | 17 | n/a | n/a | ND | n/a |
| Well #5 | Total -/Fecal - | Copper | 110 | n/a | n/a | ND | n/a |
| Well #6 | Total +/Fecal - | Copper | 62 | n/a | n/a | ND | n/a |
| Well #7 | Total -/Fecal - | Copper | 40 | n/a | n/a | ND | n/a |
| Well #8 | Total -/Fecal - | Copper | 34 | n/a | n/a | ND | n/a |
| Well #9 | Total -/Fecal - | Copper | 27 | n/a | n/a | ND | n/a |
| Well #10 | Total +/Fecal - | Copper | 23 | n/a | n/a | ND | n/a |
| Well #11 | Total -/Fecal - | Copper | 18 | n/a | n/a | ND | n/a |
| Well #12 | Total -/Fecal - | Iron | 290 | n/a | n/a | ND | n/a |
| Well #13 (Cram Gulch) | Total -/Fecal- | Barium | 46 | n/a | n/a | ND | n/a |
| Iron |  |  |  | 740 |  |  |  |
| Well #14 (Cram Gulch) | Total -/Fecal- | Barium | 47 | n/a | n/a | ND | n/a |
| Iron |  |  |  | 270 |  |  |  |
| Well #15 | Total -/Fecal - | Iron | 600 | 2.6 | ND | ND | ND |
| * n/a -- samples from this site were not tested for this analyte |  |  |  |  |  |  |  |
| + ND -- analyte not present/not detectable |  |  |  |  |  |  |  |

## Surface Water Analysis

No information on surface water testing results prior to 2005 is available for the upper reaches of Shackleford Creek. Some modern aerial photographs, including FLIR data, are available for the lower reaches of the stream.

During the February 2005 preliminary assessment surface water samples were analyzed for nitrates and nitrites, organophosphate pesticides, and heavy metals (see Table 1.5.4). Iron and copper were again the only metals detected. No organophosphates were detected. Two out of the four stream sample sites had very low nitrate concentrations; none of the sites tested positive for nitrite. High concentrations of nitrates can be indicative of runoff containing fertilizers or potential septic tank seepage.

At the time of sample collection, field parameters were also taken with a YSI Multiparameter meter. Temperature, TDS, pH, conductivity, and dissolved oxygen were recorded (see Table 1.5.5).

Table 1.5.4: Surface Water Analysis Results

| Sampling Site | Metals | Concentration (micrograms/L) | Nitrate mg/L | Nitrite | Organophosphate Pesticides |
| --- | --- | --- | --- | --- | --- |
| Stream Site 1 | Copper | 29 | 0.52 | ND | ND |
|  | Iron | 270 |  |  |  |
| Stream Site 2 | Copper | 16 | 0.5 | ND | ND |
|  | Iron | 150 |  |  |  |
| Stream Site 3 | Copper | 16 | ND | ND | ND |
| Stream Site 4 | Copper | 12 | ND | ND | ND |

n/a -- samples from this site were not tested for this analyte

+ ND -- analyte not present/not detectable

Table 1.5.5: Field Parameters at Surface Water Sample Locations

| Sampling Site | DO | Conductivity | TDS | Salinity | PH | Temperature |
| --- | --- | --- | --- | --- | --- | --- |
| Stream Site 1 | 13 | 61 | 0.066 |  | 7.91 | 4 |
| Stream Site 2 | 13 | 63 | 0.068 | 0.05 | 7.67 | 4.10 |
| Stream Site 3 | 12.74 | 63 | 0.068 | 0.05 | 7.94 | 4.14 |
| Stream Site 4 | 12.41 | 62 | 0.067 | 0.05 | 7.52 | 4.2 |

No analytes were found to be out of compliance with proposed water quality standards during this sampling event. In addition, all organophosphate pesticides were below the detection limit. All four stream sites tested positive for copper, in low concentrations. Two sites also tested positive for iron, in moderate concentrations, but were still under the level proposed for drinking water, coming from the US EPA secondary drinking water standards.

## Habitat Assessment and Stream Condition Inventory

This description describes the condition of Shackleford Creek prior to flooding events that occurred during the winter of 2005-2006. Some changes, widening and additional braids, have occurred but it has not been formally habitat typed since ECORP in 2005.

The section of Shackleford Creek that runs through QVIR property is a wide, shallow, and braided with primarily gravel and cobble substrate. The channel divides into two major braids, and one minor braid. Most habitat units are low-gradient riffle (LGR) or high-gradient riffle (HGR) with one short glide (GLD) section and one root wad enhanced lateral scour pool. One secondary channel pool was also noted. In stream cover is minimal in most portions of the stream, with few large trees situated near enough to the stream to provide cover. The depth in most of the stream was less than two feet during the stream survey, conducted during low- to moderate- flow conditions, so the stream depth does not offer significant cover to aquatic species in most of the stream. Under the Rosgen classification system for streams, this section of Shackleford Creek is type D3, a multiple channel system with a cobble and gravel substrate.

In the upper reaches of the stream near the upstream QVIR boundary, the main stream channel is slightly entrenched; at this location, the north bank is higher than the south, and the bank was slight undercut. Further downstream, the banks were of more equal height and did not show the same erosion as the upstream reaches. The width-to-depth ratio of the stream is very high, and the overflow are around the stream is quite wide.

In several locations, the stream contained excellent gravels for salmonid spawning. Several redds had been found in this section of stream during surveys in prior years; several likely redds were also noted during this habitat assessment. Several of the habitat reaches in the stream ranged from good to ideal spawning habitat. Silt depth in the channel was minimal; the banks of the stream showed slightly greater silt depth, but generally less than 0.2 feet.

At the time the habitat survey was conducted, a stonefly hatch was noted. Future surveys could include a benthic macroinvertebrate component to evaluate the availability of larvae as a food source for Coho fry. Included in Appendix B5 is the complete habitat unit characterization.

Flow measurements were also made at the endpoints of the QVIR reach of Shackleford Creek. The flow where Shackleford Creek enters QVIR land was 40.5 cubic feet per second (cfs), where the stream width was 27.0 feet. The flow just below the road crossing where Shackleford Creek leaves QVIR land was 39.6 cfs, and the stream width was 42.5 feet. At the point in time when the flow measurements were taken, the stream appeared to lose approximately one cfs within the QVIR boundaries; this calculated loss, however, is within the margin of error of the flow instrumentation, and more flow measurements are necessary to draw conclusions about stream loss or baseflow. Complete flow measurement data are included as Appendix B6.

Several agencies have been collecting water quality data in the Scott River watershed for years.  The type of water quality information collected by each agency is compiled in Table 1.5.6.  QVIR EPD will work collaboratively with these agencies to collect and share data empowering the Tribe to fulfill their role as co-managers of the watershed.

| Table 1.5.6 SCOTT RIVER RESEARCH AND MONITORING - September 2006 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method | Who | Locations | Frequency | Season | Years | Contact | Sources | Format | Notes |
| STREAMFLOW |  |  |  |  |  |  |  |  |  |  |
|  | gage station | USGS | RM 20.5 | daily ave | all | 1941- | Christine O'Neill (530) 246-5282 | www.water.usgs.gov | electronic table | http://s601dcascr.wr.usgs.gov/rt-cgi/gen_stn_pg?station=11519500 |
|  | gage station | DWR | above N. Fork French Creek | daily ave | 4/1-9/30 | 1969- | John Clements |  | Hard? | Keith Dick former water master |
|  | gage station | DWR | E. Fork & S. Fork | 15min | all | 2002- | John Clements | WY 2002 RCD | Hard/electronic | Summer flow only |
|  | gage station | DWR | Shackleford, Shackleford-Mill, French | 15min | all | 2003 | John Clements |  |  | Summer flow only |
|  | gage station | DWR | Scott Main @ Johnson’s Bar | 15min | all | 2003 | John Clements |  |  | Summer flow only |
|  | Water stage recorders at flumes & ditches | DWR |  |  |  |  | Water master Joe Scott |  | Hard? |  |
|  | gage station | USFWS/RCD | Shackleford, Kidder, Mill(Shackleford) | 15 min | all | 2002 | D. Quigley |  |  |  |
|  | Pressure Transducer | RCD | Sugar Creek | 15 min | July-October | 2002/2003 | D. Quigley | RCD office | Hard/electronic |  |
|  | aquacalcPro | RCD | 8 mainstem locations | 10-14 days | May - Dec | 2006 | D. Quigley | Worksheet | Hard | Repeat of SWRCB 1972 |
| TEMPERATURE -AIR |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Callahan, Quartz Hill, Scott Mt., Middle Boulder 3 | Hourly | all |  |  |  | electronic table |  |
|  | manual record of thermometer- high and low | USFS | Fort Jones, USFS | Daily | all | 1938-present | Philip McNeil (530) 468-5331 |  | hard copy table |  |
| PRECIPITATION |  |  |  |  |  |  |  |  |  |  |
|  |  | USFS, CDF, DWR | Callahan, Quartz Hill, Scott Mt., Middle Boulder 3 | hourly | all | varies | CDEC | www.cdec.water.ca.gov | electronic table |  |
|  |  | DWR | Scott Mountain | Daily | all | 1985-present | CDEC | www.cdec.water.ca.gov | electronic table |  |
|  |  | private | French Creek |  |  |  | Carl Schwarzenberg | Siskiyou RCD | hard copy table |  |
|  |  | RCD | Various sub watersheds | event | all | 2002-present | D. Quigley | Siskiyou RCD | electronic table |  |
|  |  | DWR | Fort Jones | Monthly |  | 1938-present | CDEC | www.cdec.water.ca.gov | electronic table |  |
| SNOWPACK |  |  |  |  |  |  |  |  |  |  |
|  | Snow water content (adjusted) | DWR, USFS | Scott Mt., Middle Boulder 3, Dynamite Meadow | Daily | all | varies | Philip McNeil (530) 468-5331 | www.cdec.water.ca.gov | electronic table |  |
|  | Snow water content (adjusted) | USFS | Box Camp, Marble Valley, Middle Boulder 1, Swampy John, Etna Mt., Log Lake | Monthly |  | varies | Philip McNeil (530) 468-5331 | www.cdec.water.ca.gov | electronic table |  |
| GROUNDWATER LEVELS |  |  |  |  |  |  |  |  |  |  |
|  | Well level stage | SRWC | 30 sites, valley floor | Monthly | annual | 2005- | Mark Horney (NRCS) |  |  |  |
|  | Well level stage | DWR | 3 sites in Scott Valley | Point in time | Fall & Spring | 1981- | Red Bluff | http://well.water.ca.gov/eXterra/GetBasinWell.cfm?GWBasinCode=’1-005.00’ (Bundy, 1999) |  | SS. Call DWR to get info, may go back to 1950’s and chemical analysis. Check Lorrie Bundy’s Water Balance |
| WATER QUALITY |  |  |  |  |  |  |  |  |  |  |
| Temperature | thermographs "Hobo temps" | RCD |  | 1.6 and 1.0 hourly | 5/15-10/15 | 1995-present | Danielle Quigley (530) 467-3975 | KRIS and listed cooperators | electronic table and hard copy tables | Conner, Farber, Power, Quigley "Water Temperatures in the Scott River Watershed" 2001 |
| Temperature | thermographs "Hobo temps" | Timber Products |  | 1.6 and 1.0 hourly | 5/15-10/15 | 1995-present | Stu Farber |  |  | Conner,Farber,Power,Quigley "Water Temperatures in the Scott River Watershed" 2001 |
| Temperature | thermographs "Hobo temps" | Timber Products |  | 1.6 and 1.0 hourly | 5/15-10/15 | 1995-present | Kelly Conner |  |  | Conner,Farber,Power,Quigley "Water Temperatures in the Scott River Watershed" 2001 |
| Temperature | thermographs "Hobo temps" | USFS |  | 1.6 and 1.0 hourly |  | 1995-present | Jim Kilgore (530) 468-1274 |  |  | Conner,Farber,Power,Quigley "Water Temperatures in the Scott River Watershed" 2001 |
| Dissolved oxygen | DO meter - night and day |  | low flow areas | periodically |  |  |  |  |  |  |
| pH | pH meter or paper |  |  | periodically |  |  |  |  |  |  |
| Conductivity | conductivity meter |  |  | periodically | 8/1 - 9/15 |  |  |  |  |  |
| SEDIMENT |  |  |  |  |  |  |  |  |  |  |
| Sediment | McNeil | RCD | RM 18 to 56, and tribs | Point in time | 8/1 - 9/15 | 1989, 2000,2006 | Danielle Quigley (530) 467-3975 | final report | hard | Sommarstrom and Associates Scott River Granitic Sediment Study 1990 |
|  | v* | RCD,USFS, DFG | French Creek | BIANNUAL |  |  | Jim Kilgore' |  |  |  |
| Nutrients | Hydrolab | BOR | RM 20.5 to mouth of Scott | daily? | all? |  | Bill Wood (541) 883-6935 WWOOD@mp.usbr.gov |  |  |  |
| Nutrients |  |  |  | periodically | 6/1 - 10/1 |  |  |  |  |  |
| Macro-invertebrate collection | rapid bioassessment | RCD | mainstem and tributary | semi-annual | spring, fall | 2000/2003 | Danielle Quigley (530) 467-3975 |  |  |  |
| Metals, etc. |  | BOR |  |  |  | 1999 | Bill Wood |  |  |  |
| PHYSICAL ATTRIBUTES |  |  |  |  |  |  |  |  |  |  |
| Cross sections | Total Station survey | RCDNRCS | 16, mainstem mostly at bridges | every 5-10 years | 6/1 - 11/15 | 1998, 2006 | Lorrie Bundy (NRCS) | Total station survey | AutoCAD drawing, hard copy data file |  |
| Riparian condition | visual (photos, alive/dead, vigor) | RCD |  | periodically | 8/1 - 10/31 |  | Gary Black (530) 467-3975 | worksheets | hard copy tables |  |
| Riparian condition |  | RCD (Alvin Lewis) | Mainstem RM…. – RM…. |  | 1990? |  |  |  | FWS Report | 1991? |
| Photo points | Cantara Project | RCD | about 26 mainstem | annually | 6/1-8/1 | 96-01 | Danielle Quigley (530) 467-3975 | Hard copy | RCD Office | Fay Lane Restoration Binder |
|  | Riparian Conditions | RCD | about 30 mainstem and tributaries | annually | 6/1-8/1 | 00-01 | Danielle Quigley (530) 467-3975 | Electronic/hard copy | RCD Office |  |
| Road Erosion Inventory |  | RCD | Shackleford | Once |  | 1999 | Gary Black (530) 467-3975 | Hard copy | RCD Office |  |
|  |  |  | SouthFork | Once |  | 2001 | Gary Black | Hard copy | RCD Office |  |
|  |  |  | Etna | Once |  | 2001/2001 | Gary Black | In Progress | RCD Office |  |
|  |  |  | Mill(Scott Bar) | Once |  | 2001/2001 | Gary Black | In Progress | RCD Office |  |
|  |  |  | French Creek | Once |  |  |  | Hard copy | RCD Office |  |
| FISH SURVEYS |  |  |  |  |  |  |  |  |  |  |
| Fall Chinook spawning redd surveys | visual in waders | DFG, USFS, Americorps | RM 0 - RM 48 | annual | 10/15 - high water | 1992-present? | Mark Hampton, CDFG |  |  |  |
| Fall Chinook spawning redd surveys |  | USFWS | Mainstem Klamath | Annual |  | 1992-present? | Arcata Office USFWS |  |  | DM |
| Adult Coho Spawning Ground Survey | Visual in waders | RCD/USFS/CDFG/volunteers | Various tribs | annual | Nov-Jan | 2001-present | Mark Pisano (CDFG)/Danielle Quigley | Hard copy |  |  |
| juvenile fish counts | visual by snorkeling | USFS, DFG | French Creek and …. | periodically | 6/1-9/30 |  | Jim Kilgore (530) 468-5351 |  |  | DM |
| juvenile outmigrant trapping | Fyke & pipe trap | RCD | French, Etna, Sugar, East Fork | once-pilot | Oct - June | 2005-2006 | Erich Yokel (RCD) 467-3247 | final report |  |  |
| juvenile outmigrant trapping | Rotary Screw Trap | CDFG | Scott R. near Scott Bar | annual | Feb - July | 2000-present | Bill Chesney (CDFG) 530-842-2168 | final report |  |  |
| Fish Rescue Data |  | DFG |  |  |  |  | Jim Whelan (CDFG) |  |  |  |
| Aerial Redd Counts |  | DFG |  |  |  | 1964-1979 | Jim Whelan (CDFG) |  |  |  |
| Aerial photos | airplane B/W | NRCS |  | periodically |  |  | Ayn Perry (530) 842-6121 | photos | photocopies or order photo series |  |
| Aerial photos |  | USFS |  |  |  |  | Philip McNeil (530) 468-1213 |  |  |  |

*Table obtained from the Siskiyou Resource Conservation District

1.6	Project/Task Description and Schedule

The QVIR EPD will implement a Water Quality Monitoring Program to collect quality assured water quality data for management decisions related to the aquatic environment within the QVIR and the Scott River watershed.  Monitoring is scheduled to begin in Spring 2007 upon US EPA approval of this QAPP. Water quality data collection will help establish baseline water quality conditions and quantitatively assess the quality of QVIR water resources and initiate long-term trend monitoring. After one year of sampling in Spring 2008, a report will be issued to US EPA.  To the degree they are useful, these quality assured data will be provided for to the North Coast Regional Water Quality Control Board for use in TMDL implementation.

The QVIR EPD will be sampling surface water for various parameters critical to fish health at numerous locations on Shackleford, Mill and Sniktaw Creeks in the Reservation’s watershed.  The headwater lakes of Shackleford and Mill Creeks will be sampled as well. Additional sample sites have been selected in the Scott River, one just downstream of the confluence of Shackleford/Mill Creek, one farther downstream at Johnson’s Bar, and sites upstream on both the East and South Forks that are above any agricultural influences.  These upstream sites were selected to use as a comparison with the pre-agricultural conditions of Shackleford/Mill Creeks and to use as baselines for other Scott River sample results.

Water quality sampling will take place in the following water bodies with varying numbers of stations in each (Figure 1.6.1):

- The Scott River will be sampled at 3 locations: downstream the convergence with Shackleford Creek (Wing Property), Johnson’s Bar, South Fork at USFS lower boundary.

- Seven tributaries to the East Fork Scott River will be sampled: Crater, Houston, Rail, Kangaroo, Grouse, Mule and Mill (East Fork) Creeks.

- Four tributaries to the main stem Scott River will be sampled: Kelsey, Canyon, Boulder and Scott Bar Mill Creeks at the USFS lowest boundary.

- The headwaters of Shackleford Creek will be sampled at 9 locations: inlet, outlet and in the lakes of Summit, Cliff and Campbell.

- Shackleford Creek will be sampled at 7 locations: convergence of Campbell/Cliff and Summit Lakes, convergence of Long High Creek, convergence with Back Meadows Creek, at Wilderness boundary, on Timber Vest property, at QVIR boundary, and at an Indian Trust property near the confluence with the Scott.

- The headwaters of Mill Creek will be sampled at 6 locations: inlet, outlet and in East and West Mill Creeks ponds.

- Mill Creek will be sampled at 4 locations: convergence of West and East Mill Creek Ponds, at USFS boundary, at BLM parcel, at Quartz Valley Elementary School.

- Sniktaw Creek will be sampled at 2 locations: top and bottom of Case property

- Water quality parameters to be sampled for each water body are listed in Table 1.6.2.  These include hand held instrument readings, stage and flow gauges, and continuous automated probe sampling. The sampling frequency at each location by parameter can also be found in Table 1.6.2. Water quality sampling may not be feasible on the middle sections of Shackleford Creek because of loss of surface flow late in summer and before fall rains. The continuous data recorder in the Scott River downstream of the mouth of Shackleford Creek will be fixed to a cable, protected by a metal pipe, which will suspend the probe to avoid damage to equipment posed by powerful Scott River flows during winter.

Monitoring will help discover whether there are water quality problems with waters   within or adjacent to the QVIR and the QVIR EPD will report any findings of action levels of contaminants and work to abate any identified problems as described above. Turbidity monitoring data will likely be useful for tracking recovery of water quality for the TMDL implementation.

Monitoring Locations, Methods and Timing of Samples

Monitoring methods described below have been selected to best determine whether beneficial uses of water are being attained on the QVIC Reservation and in the Shackleford/Mill Creek watershed and what trends for parameters limiting attainment of beneficial uses over time.

Sample sites are located on public, private and tribal lands.  There are two sample sites located on private lands where permission has been secured from the owners. Much of the middle reaches of both Shackleford and Mill Creeks are under private ownership and are not accessible at this time.  Timber Vest, a timber company, own a majority of the land in the middle reaches of Shackleford and Mill Creeks.  QVIR Environmental Director has met and has received permission to monitor on Timber Vest property. We in turn have agreed to share the data collected at all of our sampling locations.

The QVIR monitoring locations (Table 1.6.1 and Figures 3 and 4) are arrayed so as to allow a comprehensive assessment of Shackleford & Mill Creeks and to collect baseline data to facilitate participation in co-management of the watershed.  Upper Shackleford Creek locations start at the headwaters within the U.S. Forest Service designated Marble Mountain Wilderness Area at the inlets, outlets, and in the center of Campbell Lake, Summit Lake, and Cliff Lake.  Samples will also be collected at the junction of outlets from Summit Lake and Cliff/Campbell Lakes, at the confluence of the outlets of Log Lake, Long High Creek, and Back Meadows Creek and additionally at the Wilderness boundary. Middle and lower Shackleford Creek locations are on Timber Vest property, at the top of the QVIR ownership and on Indian trust land just below the convergence with Mill Creek (ST) and above the convergence with the Scott River.

Upper Mill Creek locations also begin at the headwaters in the USFS Marble Mountain Wilderness Area at the inlets, outlets, and in the center of the two Mill Creek Ponds and downstream at the Wilderness Boundary before the Creek crosses into private land.  An additional sample site on Mill Creek is located approximately two miles outside of the Wilderness boundary on Bureau of Land Management property.  A sample location on Lower Mill Creek is at the Quartz Valley Elementary School above the convergence of Shackleford and Mill Creeks. This site offers a great opportunity for QVES students to participate in sampling events.

Sniktaw Creek will be sampled at two locations on the Reservation upstream and downstream of an area where excess solid waste has been stockpiled.

Scott River sampling locations are located just downstream of the confluence of Shackleford/Mill Creek (Wing property), continuous real-time YSI Sonde monitoring at the USGS gage, upstream of the confluence with the Klamath at Johnson’s Bar, and sites upstream on both the East and South Forks. The following tributaries to the mainstem will be sampled at the lowest USFS boundary: Kelsey, Canyon, Boulder and Mill Creek at Scott Bar (a.k.a. “Scott Bar” Mill). Because land on the East Fork is all private and inaccessible at this time, the major tributaries flowing into the Scott River must be sampled instead.  Samples will be taken on USFS land on Crater, Houston, Rail, Kangaroo, Grouse, Mule, and Mill Creeks.  The South Fork main-stem will be sampled at the lowest USFS Boundary.

| Table 1.6.1: Locations for QVIR water quality monitoring with station identification codes. |  |
| --- | --- |
| ID Number | Monitoring Location Scott River Watershed |
| SLI | Summit Lake Inlet |
| SL | Summit Lake |
| SLO | Summit Lake Outlet |
| CLI | Cliff Lake Inlet |
| CL | Cliff Lake |
| CLO | Cliff Lake Outlet |
| CALI | Campbell Lake Inlet |
| CAL | Campbell Lake |
| CALO | Campbell Lake Outlet |
| SCC | Shackleford/Campbell Convergence |
| SLHC | Shackleford at Long High Creek |
| SBMC | Shackleford at Back Meadows Creek |
| ST | Upper Shackleford at Wilderness trailhead |
| SR | Shackleford at Timber Vest property |
| SQVIR | Shackleford on QVIR |
| STR | Shackleford at Indian Trust Property (near confluence) |
| NMCI | West Mill Creek Pond Inlet |
| NMC | West Mill Creek Pond |
| NMCO | West Mill Creek Pond Outlet |
| SMCI | East Mill Creek Pond Inlet |
| SMC | East Mill Creek Pond |
| SMCO | East Mill Creek Pond Outlet |
| WEMC | West/East Mill Creek Convergence |
| MFS | Mill Creek at USFS Boundary |
| MM | Middle Mill Creek on BLM |
| ML | Lower Mill Creek at Quartz Valley Elementary School |
| SCU | Sniktaw Creek upstream end of Case property |
| SCD | Sniktaw Creek downstream Case property |
| SRS | Scott River near Shackleford confluence (Wing property - continuous sonde) |
| SRJB | Scott River Johnson's Bar |
| ESRC | East Fork Scott River- Crater – USFS boundary |
| ESRH | East Fork Scott River- Houston – USFS boundary |
| ESRR | East Fork Scott River- Rail – USFS boundary |
| ESRK | East Fork Scott River- Kangaroo – USFS boundary |
| ESRG | East Fork Scott River- Grouse – USFS boundary |
| ESRM | East Fork Scott River- Mule – USFS boundary |
| ESRML | East Fork Scott River- Mill – USFS boundary |
| SFSR | South Fork Scott River- USFS boundary |
| MSRK | Main Stem Scott River - Kelsey Creek – USFS boundary |
| MSRC | Main Stem Scott River - Canyon Creek – USFS boundary |
| MSRB | Main Stem Scott River - Boulder Creek – USFS boundary |
| MSRM | Main Stem Scott River – Mill Creek (at Scott Bar) – USFS boundary |

Quartz Valley Indian Reservation

Map of Sample Locations – Shackleford Mill Subbasin

Figure 3.1

Quartz Valley Indian Reservation

Map of Sample Locations – Shackleford Mill Sub basin Figure3.2

Quartz Valley Indian Reservation

Map of Sample Locations – Scott River Canyon

Figure 4

Quartz Valley Indian Reservation

Map of Sample Locations – Scott River East Fork

Figure 5

Quartz Valley Indian Reservation

Map of Sample Locations – Scott River South Fork

Figure 6

Sample Frequency and Parameters

Each location will be sampled for the following on a weekly basis from April 1 through October 31 at the 35 locations listed on Table 1.6.1 and shown on Figures 3 and 4 : flow; temperature; pH; conductivity; dissolved oxygen; macroinvertebrates; and nutrients (grab sample)- phytoplankton, total phosphorus, dissolved phosphorus, total nitrogen, ammonium nitrogen, nitrate + nitrite, chlorophyll. Shackleford, Mill Creeks and the Scott River will be sampled for pesticide trifluralin.  The Scott River will also be tested for trifluralin and diuron. Additional pesticide testing may be necessary if new pesticides posing a threat to human or aquatic life are introduced into the Scott basin or Shackleford and Mill sub-basins.

Temperature probes will be placed at each designated sample site for continuous monitoring recorded hourly.

Stratified temperature monitoring and nutrient sampling will be conducted at all sampling locations in the lakes and ponds on a weekly basis between April 1 and October 31.  Discharge will also be measured at each location during each sampling event.

A YSI Sonde will monitor temp, pH, conductivity, turbidity and dissolved oxygen in the Scott River just downstream from the mouth of Shackleford-Mill Creek hourly on a year round continuous basis.  It will have real-time monitoring preliminary data available through the QVIR web page, accessible at <qvir.com>. Private property access has been granted to the QVIR EPD Environmental Director by the Wing family. Access to use the USGS gage satellite for real-time monitoring is still pending USGS approval.

Most of the sample locations in this study are accessible by maintained paved or dirt roads, either County or Forest Service roads.  The QVIR EPD’s 4-wheel drive vehicle will be used when sampling these locations.  The remaining sample locations will be accessed via hiking trails in the Marble Mountain Wilderness.  Table 1.6.3 describes sample locations, rationale, and accessibility.  Sampling technicians will hike in to wilderness locations using pack animals to carry sampling equipment.  All samples will be collected from the shoreline except samples collected at the lakes.  Lake samples will be obtained using an inflatable kayak to access the lake’s center.

All sampling locations will be recorded using global positioning system (GPS) equipment following the procedures included in Appendix E1.  Additionally, photo documentation will occur at each sampling location during every sampling event.

A parameter may be removed from the monitoring program if the sampling results indicate it is not of concern or added if new land uses develop after the monitoring program begins or the monitoring data indicates other potential parameters to include.

| ID Table 1.6.2: Sample locations and parameters | Monitoring Locations Shackleford-Mill Watershed | Frequency- April 1 - Oct 31 | Photo Points | Flow | Temp. | pH | Conductivity | Turbidity | DO | Macro-invertebrates | Total Phosphorus | Dissolved Phosphorus | Total Nitrogen | Ammonium Nitrogen | Nitrate + Nitrite | Phytoplankton | Chlorophyll | Trifluralin | Pesticides- Diuron |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SLI | Summit Lake Inlet | Weekly | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |  |  |
| SL | Summit Lake | Weekly | X |  | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |  |  |
| SLO | Summit Lake Outlet | Weekly | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| CLI | Cliff Lake Inlet | Weekly | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |  |  |
| CL | Cliff Lake | Weekly | X |  | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |  |  |
| CLO | Cliff Lake Outlet | Weekly | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| CALI | Campbell Lake Inlet | Weekly | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |  |  |
| CAL | Campbell Lake | Weekly | X |  | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |  |  |
| CALO | Campbell Lake Outlet | Weekly | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| SCC | Shackleford/Campbell Convergence | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| SLHC | Shackleford at Long High Creek | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| SBMC | Shackleford at Back Meadows Creek | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| ST | Upper Shackleford at Wilderness trailhead | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| SR | Shackleford at Timber Vest | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| SQVIR | Shackleford on QVIR | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| STR | Shackleford at Charlie Thom’s Property | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| NMCI | West Mill Creek Pond Inlet | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |  |
| NMC | West Mill Creek Pond | Weekly | X |  | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |  |
| NMCO | West Mill Creek Pond Outlet | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| SMCI | East Mill Creek Pond Inlet | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |  |
| SMC | East Mill Creek Pond | Weekly | X |  | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |  |
| SMCO | East Mill Creek Pond Outlet | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| WEMC | West/East Mill Creek Convergence | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| MFS | Mill Creek at USFS Boundary | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| MM | Middle Mill Creek on BLM | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| ML | Lower Mill Creek at Quartz Valley Elementary School | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| SCU | Sniktaw Creek upstream end of Case property | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| SCD | Sniktaw Creek downstream Case property | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |  |
| SRS | Scott River downstream ¼ mile Shackleford confluence | Weekly | X | X | Year Round Continuous- Hourly | Year Round Continuous- Hourly | Year Round Continuous- Hourly | Year Round Continuous- Hourly | Year Round Continuous- Hourly | X | X | X | X | X | X | X | X | X | X |  |  |  |
| SRJB | Scott River Johnson's Bar | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| ESRC | East Fork Scott River- Crater | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| ESRH | East Fork Scott River- Houston | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| ESRR | East Fork Scott River- Rail | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| ESRK | East Fork Scott River- Kangaroo | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| ESRG | East Fork Scott River- Grouse | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| ESRM | East Fork Scott River- Mule | Weekly | X | X | Hourly | X | X | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  |
| ESRML MSRK MSRC MSRB | East Fork Scott River- Mill Mainstem Scott River – Kelsey Creek Mainstem Scott River –Canyon Creek Mainstem Scott River- Boulder Creek | Weekly Weekly Weekly Weekly | X X X X | X X X X | Hourly Hourly Hourly Hourly | X X X X | X X X X | X X X X | X X X X | X X X X | X X X X | X X X X | X X X X | X X X X | X X X X | X X X X | X X X X | X X X X | X X X X |  |  |  |
| SFSR MSRM | South Fork Scott River- USFS Boundary Mainstem Scott River – “Scott Bar” Mill Creek | Weekly Weekly | X X | X X | Hourly Hourly | X X | X X | X X | X X | X X | X X | X X | X X | X X | X X | X X | X X | X X | X X |  |  |  |

| Table 1.6.3: Sample Locations, Rationale, & accessibility for QVIR water quality monitoring program |  |  |  |
| --- | --- | --- | --- |
| ID Number | Monitoring Location Scott River Watershed | Rationale | Accessibility |
| SLI | Summit Lake Inlet | Background sample, headwaters of Shackleford Creek before entering Lake, used as pristine comparison to detect pollution to the lake and the greater Scott and Klamath watershed. | USFS Shackleford Road Shackleford Creek Trail |
| SL | Summit Lake | Background sample, captures water quality of lake before it becomes Shackleford. This could identify any water quality problems such as nutrient pollution associated with grazing. | USFS Shackleford Road Shackleford Creek Trail |
| SLO | Summit Lake Outlet | Captures water quality upon leaving lake and becoming Shackleford Creek Captures existing flows exiting the lake. | USFS Shackleford Road Shackleford Creek Trail |
| CLI | Cliff Lake Inlet | Background sample, headwaters of Shackleford Creek before entering Lake, used as pristine comparison to detect pollution to the lake and the greater Scott and Klamath watershed. | USFS Shackleford Road Shackleford Creek Trail |
| CL | Cliff Lake | Background sample, captures water quality of lake before it flows into Campbell Lake. This could identify any water quality problems such as nutrient pollution associated with grazing. | USFS Shackleford Road Shackleford Creek Trail |
| CLO | Cliff Lake Outlet | Captures water quality upon leaving lake and entering Campbell Lake Captures existing flows exiting the lake. | USFS Shackleford Road Shackleford Creek Trail |
| CALI | Campbell Lake Inlet | Background sample, water entering is from Cliff Lake, used to detect pollutants entering Campbell Lake and the greater Scott and Klamath watershed. | USFS Shackleford Road Shackleford Creek Trail |
| CAL | Campbell Lake | Background sample, captures water quality of lake before it becomes Shackleford. This could identify any WQ problems such as nutrient pollution associated with grazing. | USFS Shackleford Road Shackleford Creek Trail |
| CALO | Campbell Lake Outlet | Captures water quality upon leaving lake and becoming Shackleford Creek Captures existing flows exiting the lake. | USFS Shackleford Road Shackleford Creek Trail |
| SCC | Summit/Campbell_Cliff Shackleford Convergence | Captures quality of Shackleford Creek downstream of Summit, Campbell and Cliff Lakes | USFS Shackleford Road Shackleford Creek Trail |
| SLHC | Shackleford at Long High Creek | Captures Long High Creek influence on Shackleford | USFS Shackleford Road Shackleford Creek Trail |
| SBMC | Shackleford at Back Meadows Creek | Captures Back Meadows Creek influence on Shackleford | USFS Shackleford Road Shackleford Creek Trail |
| ST | Upper Shackleford at Wilderness trailhead | Captures water quality before Shackleford leaves wilderness and the influence of unnamed tributary upstream of wilderness boundary. This is also the last site before Shackleford enters industrial logging property | USFS Shackleford Road Shackleford Creek Trail |
| SR | Shackleford at Timber Vest | This sampling site is in the middle of industrial logging property and will identify water quality problems stemming from this type of land use | USFS Shackleford Road |
| SQVIR | Shackleford on QVIR | This sampling site is downstream from industrial logging property and will identify water quality problems stemming from this type of land use | USFS Shackleford Road |
| STR | Shackleford/Mill at Tribal Trust (near confluence) Property | Site is downstream from Shackleford/Mill convergence and will monitor water quality of creeks after confluence. Captures water quality associated with agricultural land uses | Quartz Valley Road to Dangel Lane |
| NMCI | West Mill Creek Pond Inlet | Background sample, headwaters of Mill Creek before entering Lake, used as pristine comparison to detect pollution to the lake and the greater Scott and Klamath watershed. | Quartz Valley Road to Mill Creek Road to Mill Creek Trail |
| NMC | West Mill Creek Pond | Background sample, captures water quality of lake before it becomes Mill Creek. This could identify any water quality problems such as nutrient pollution associated with grazing. | Quartz Valley Road to Mill Creek Road to Mill Creek Trail |
| NMCO | West Mill Creek Pond Outlet | Monitors water quality upon leaving pond and becoming Mill Creek Captures existing flows exiting the lake. | Quartz Valley Road to Mill Creek Road to Mill Creek Trail |
| SMCI | East Mill Creek Pond Inlet | Background sample, headwaters of Mill Creek before entering Lake, used as pristine comparison to detect pollution to the lake and the greater Scott and Klamath watershed. | Quartz Valley Road to Mill Creek Road to Mill Creek Trail |
| SMC | East Mill Creek Pond | Background sample, captures water quality of lake before it becomes Mill Creek. This could identify any water quality problems such as nutrient pollution associated with grazing. | Quartz Valley Road to Mill Creek Road to Mill Creek Trail |
| SMCO | East Mill Creek Pond Outlet | Captures water quality upon leaving pond and becoming Mill Creek Captures existing flows exiting the lake. | Quartz Valley Road to Mill Creek Road to Mill Creek Trail |
| WEMC | West/East Mill Creek Convergence | Monitors influence of both forks of Mill Creek, used to detect the interaction of nutrient pollution | Quartz Valley Road to Mill Creek Road to Mill Creek Trail |
| MFS | Mill Creek at USFS Boundary | Last sampling site before leaving wilderness and entering industrial forest land | Quartz Valley Road to Mill Creek Road |
| MM | Middle Mill Creek on BLM | Monitors water quality in the midst of industrial forest land | Quartz Valley Road to Mill Creek Road |
| ML | Lower Mill Creek at Quartz Valley Elementary School | Monitors water quality above Shackleford convergence and in the midst of agricultural land use | Quartz Valley Road in Mugginsville |
| SCU | Sniktaw Creek upstream end of Case property | Background sample, upstream of suspected contamination source. Downstream of agricultural land use. | Quartz Valley Road at Big Meadows Drive |
| SCD | Sniktaw Creek downstream Case property | Downstream of suspected contamination source Captures water quality of Sniktaw leaving the Reservation, before entering the Scott. Agricultural land use. | Quartz Valley Road at Big Meadows Drive |
| SRS | Scott River ¼ mile downstream Shackleford confluence | Captures water quality of Scott River after Shackleford/Mill enters. Captures water quality associated with agricultural land use before it enters federal land. | Scott River Road – granted access @ Wing’s property, may be using USGS gage satellite |
| SRJB | Scott River Johnson's Bar | Downstream sample above confluence with the Klamath Comparable parameters to Karuk and Yurok Tribe data | Scott River Road |
| ESRC | East Fork Scott River- Crater | Background sample pre agriculture. Can’t sample on Scott due to property issues | USFS road 41N03 |
| ESRH | East Fork Scott River- Houston | Background sample pre agriculture. Can’t sample on Scott due to property issues | HWY 3 to USFS road 41N03 |
| ESRR | East Fork Scott River- Rail | Background sample pre agriculture. Can’t sample on Scott due to property issues | HWY 3 to USFS road 41N08 |
| ESRK | East Fork Scott River- Kangaroo | Background sample pre agriculture. Can’t sample on Scott due to property issues | HWY 3 to USFS road 40N08 |
| ESRG | East Fork Scott River- Grouse | Background sample pre agriculture. Can’t sample on Scott due to property issues | HWY 3 to USFS road 40N06 |
| ESRM | East Fork Scott River- Mule | Background sample pre agriculture. Can’t sample on Scott due to property issues | HWY 3 |
| ESRML | East Fork Scott River- Mill | Background sample pre agriculture. Can’t sample on Scott due to property issues | USFS road (#?) off of HWY 3 |
| SFSR | South Fork Scott River- USFS Boundary | Background sample pre agriculture, | Gazelle Callahan Road |
| MSRK | Kelsey Creek – tributary to main stem Scott River | Background sample to capture water quality of the tributary before entering the Scott River | Scott River Rd. to USFS road 44N41 |
| MSRC | Canyon Creek – tributary to the main stem Scott River | Background sample to capture water quality of the tributary before entering the Scott River | Scoot River Rd. to Canyon Creek Rd. |
| MSRB | Boulder Creek – tributary to the main stem Scott River | Background sample to capture water quality of the tributary before entering the Scott River | Scott River Rd. to USFS road 44N53Y |
| MSRM | “Scott Bar” Mill Creek – tributary to the main stem Scott River | Background sample to capture water quality of the tributary before entering the Scott River | Scott River Rd. to USFS road 45N27 |

Figure 7:  Schedule for Implementation

QAPP Development

Fall 2006

QAPP Submission, Review, and Approval

Fall 2006 – Winter 2007

Water Quality Sampling

April 1 – October 31, 2007

Laboratory Analysis and Reporting

April – November 2007

Data validation/evaluation

November 2007 – January 2008

Project Report Development

January – February 2008

Project Report Due Date

March 15, 2008

1.7	Quality objectives and Criteria for Measurement Data

- The primary goal of this QAPP is to ensure that high quality data be generated by the QVIR EPD Water Quality program that this data can be used to answer questions about the quality of waters within QVIRs watershed and to foster their protection or improvement over time.  Specific questions to be answered through this study include:

- Do chemical and biological baseline levels in Shackleford and Mill Creek support fish health?

- What pollutants are present in the surface water of Shackleford-Mill Creek Watershed that would be detrimental to the health of fish populations or the ecosystem?

- Do any of these pollutants exceed the national, state, and regional water quality objectives set for this basin?

- Do the lakes support current adjudicated water rights in the Shackleford and Mill sub basins?

The Tribe’s primary concern with surface water is to minimize the effects of their human activity in the watershed, to bolster the health of the ecosystem, to preserve cultural resources, and to return fish populations to a sustainable level enabling tribal members to utilize their fishing rights on the Reservation.  Current numbers of returning salmonids will not support a fishery on the Reservation as it once did.

Decisions to be made using the data

The surface water monitoring program is designed to characterize the surface water resources of the Quartz Valley Indian Reservation. The baseline data generated from the first year of quarterly sampling will provide valuable information about the current condition of the Scott River basin’s water resources, particularly the Shackleford/Mill sub-basin. On-going monitoring, conducted for the following 4 years, will allow the Tribe to begin to track changes in water quality over time and to assess potential future environmental impacts to the Reservation’s surface waters. The long-term use of the surface water monitoring program is to provide information to help the Tribe establish water quality standards and other tribal regulations and ordinances for the Quartz Valley Indian Reservation.

Decisions to be made with the data include:

- If data for any analyte or field parameter (from an individual location or single quarterly sampling event) are found to exceed the project action limits, then the Tribal Council will be notified.

- If data are found to exceed the project action limits and appear to be increasing with time, then the Tribal Council will be notified and a plan for future investigations of potential sources will be discussed.

- If waters flowing onto the reservation are impaired (i.e., exceed project action limits or the national water quality standards), the issue will be brought to the attention of the Tribal Council for possible discussion with the US EPA Project Officer.

The Tribe will determine if any action is needed to reduce surface water pollution from tribal lands. Some examples of actions that could result from findings of poor water quality on the Reservation are:

- Remediation activities for point sources to stop contamination if a single point source is suspected

- Stream and watershed restoration activities (e.g. planting native flora for erosion control)

- Pollution prevention planning and establishment of educational programs on the Reservation to reduce anthropogenic sources of pollution

- Dedication of Tribes adjudicated water rights to bolster in-stream flows and improve water quality

The Tribe will also use this information to act as co-managers in the Scott River Watershed with federal, state, and local agencies.  The information will be shared with these agencies in order to track changes over time and to ultimately improve the quality and quantity of fish populations in the Watershed.

Action Limits/Levels

| Table 1.7.1: Water Quality Parameters and Action Levels |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Units | Water Body | Data Uses | Action Level |  |  |  | Laboratory Detection Limit |
| Flow | cfs | All | Baseline Long-Term Monitoring, TMDL | Minimum instream flow requirements for salmonids has yet to be determined |  |  |  | AquaCalc |
| Temp. | °C | All | Baseline Long-Term Monitoring, TMDL | MWMT a |  |  | 24 C | HOBO YSI |
| pH | pH | All | Baseline Long-Term Monitoring | Max |  |  | Min |  |
|  |  |  |  | 8.5 b |  |  | 7 b | YSI |
| Conductivity | μS/cm | All | Baseline Long-Term Monitoring | 90% Upper Limit |  |  | 50% Upper Limit |  |
|  |  |  |  | 400 b |  |  | 275 b | YSI |
| Turbidity | NTU | All | Baseline Long-Term Monitoring, TMDL | 5 NTU e above ambient turbidity levels |  |  |  | YSI WQ770 |
| Dissolved Oxygen | mg/L | All | Baseline Long-Term Monitoring | Min | 90% Upper Limit |  | 50% Upper Limit | YSI |
|  |  |  |  | 7.0 b |  |  | 9.0 b |  |
| Macro-invertebratesd | - | All | Baseline Long-Term Monitoring | N/A |  |  |  | N/A |
| Total Phosphorus | μg/L | All | Baseline Long-Term Monitoring | 10.00 c |  |  |  | 0.05 ppm |
| Dissolved Phosphorus | mg/L | All | Baseline Long-Term Monitoring | - |  |  |  | 0.050 ppm |
| Total Nitrogen | mg/L | All | Baseline Long-Term Monitoring | 0.12 c |  |  |  | 0.40 ppm |
| Ammonium Nitrogen | μg/L | All | Baseline Long-Term Monitoring | - |  |  |  | 0.10 ppm |
| Nitrate + Nitrite | mg/L | All | Baseline Long-Term Monitoring | 10 f |  |  |  | 400 ppb |
| Phytoplankton | - | All | Baseline Long-Term Monitoring TMDL | - |  |  |  | 100 specimens |
| Chlorophyll | μg/L | All | Baseline Long-Term Monitoring | 100-200 g |  |  |  | 2 ppb |
| Pesticides- Trifluralin | μg/L | All except lakes | Baseline Long-Term Monitoring | 0.08 f |  | 1.0 |  | 1.0 |
| Pesticides- Diuron | μg/L | All except lakes | Baseline Long-Term Monitoring | 1.0 f |  |  |  | 1.0 |

d California Department of Fish and Game: Aquatic Bioassessment Laboratory.  December, 2003. California Stream Bioassessment Procedure: (Protocol Brief for Biological and Physical/Habitat Assessment in Wadeable Streams).

e Action Level adopted from Berg, L. 1982 The effect of short term pulses of suspended sediment on the behavior of juvenile salmonids. Pages 177-196 in G.F. Hartman et al. editors. Proceedings of the Carnation Creek Workshop: a ten year review. Department of Fisheries and Oceans, Nanaimo, B.C. Same data results from:  Lloyd, D.S. 1987. Turbidity as a water quality standard for salmonid habitats in Alaska. North

American Journal of Fisheries Management. 7:34–45. ***This will become a fixed number once year round data has been collected in the Scott River and its headwaters to establish ambient turbidity.

f EPA, 2006. 2006 Edition of the Drinking Water and Health Advisories Standards pg 10.  *Need a lab with a lower detection limit.

g National Nutrient Guidance for Rivers and Streams, US EPA 2000. Chapter 7 Nutrient and Algal Criteria Development

Identify Data Needs and establish acceptance criteria

- Data Quality Indicators

In order to support project decisions, data generated must be of known and acceptable quality. To define acceptable data quality for this project, data quality indicators (DQIs) were identified for each analytical parameter, and decisions were made regarding how each DQI would be assessed. The DQIs include: precision, accuracy/bias, representativeness, comparability, completeness, and sensitivity.

The general approach to assessing each DQI is described below. Some DQIs will be assessed quantitatively, while others will be assessed qualitatively. For quantitative assessments, example calculations have been provided and the QC samples (to assess each DQI) have been identified.

The frequency of the QC samples and the measurement performance criteria for each QC sample for each type of analysis are provided in Tables 2.5.1 through 2.5.2. For quantitative assessment of laboratory methodology, the laboratory’s QA Manual and analytical SOPs have been reviewed by the tribe’s project team, and the associated laboratory QC (types & frequencies of QC samples and QC acceptance limits) have been determined to be adequate to meet the data quality needs of the project. As such, the laboratory QC have been accepted as the project’s measurement performance criteria for the analytical component, while project-specific criteria have been defined to assess the field sampling component.

For field measurements, the DQIs to be assessed quantitatively include precision and accuracy alone. The associated acceptance criteria (types & frequencies of QC checks and acceptance limits) for the project are summarized in Table 2.5.2.

Data quality will be assured by:

- Proper study design,

- Following standard methods,

- Using well calibrated equipment,

- Taking and maintaining good field records,

- Following chain of custody procedures for laboratory analysis,

- Prompt data entry in standard programs and formats,

- Data archiving with back ups to insure against loss, and

- Proper oversight of QA/QC procedures.

Accuracy is the degree of agreement of a measurement with the true value. Accuracy includes a combination of random error (precision) and systematic error (bias) that result from sampling and analytical operations. Accuracy of water quality and quantity measurements contained in this QAPP is a function of the equipment used during sampling, which are listed in Table 1.7.2.

Accuracy/bias will be assessed as related to recovery, as well as in regards to potential contamination sources. Both of these terms will be evaluated quantitatively.

Accuracy/bias related to recovery is an assessment of the laboratory analytical methods alone. For Laboratory Control Samples (LCS), it will be expressed as % Recovery by the following equation:

% Recovery = X x 100 T

where,

X = Measured concentration

T = True spiked concentration

or, for Matrix Spike (MS) samples, by the following equation:

% Recovery = (B - A) x 100 T

where,

B = Measured concentration of spiked sample

A = Measured concentration of unspiked sample

T = True spiked concentration

The frequency of the LCS and/or MS samples associated with the analytical parameters will be one for every 20 samples or 5%. No LCS or MS samples will be analyzed as part of the field measurements.

Accuracy/bias as related to contamination involves both a field sampling and laboratory component. To assess all steps of the project (from sample collection through analysis), field blanks will be collected and analyzed. Field blanks are planned to be collected at a frequency of 5% (or 1 blank/20 field samples) for off-site analysis of metals and anions. To assess potential laboratory contaminant sources alone, laboratory blanks will be prepared and analyzed at a one per batch or 5% frequency. No blanks will be analyzed as part of the field measurements.

Precision is a measure of agreement among replicate measurements of the same property, under prescribed similar conditions. The precision of the sampling equipment is listed as a percentage in Table 1.7.2.

Precision will be assessed quantitatively with duplicate samples and expressed as relative percent difference (RPD) by the following equation:

RPD (%) = |X1 - X2| x 100 (X1 + X2)/2

where,

RPD (%) = relative percent difference

X1 = Original sample concentration

X2 = Duplicate sample concentration

|X1 - X2| = Absolute value of X1 - X2

To assess precision associated with all steps of the project (from sample collection through analysis) field duplicates will be collected and analyzed. Field duplicates will be collected at a frequency of 10% (1 duplicate/10 field samples) for each analytical parameter and 5% (1 duplicate each of 2 days/10 field samples) for each field measurement parameter. To assess laboratory precision alone, laboratory duplicates will be prepared and analyzed at a 5% frequency.

Completeness is a measure of the amount of valid data obtained, expressed as a percentage of the number of valid measurements that were planned to be collected. Lack of completeness may result in an inability to support Data Quality Objectives or to provide adequate data for assessment and decision support by the QVIR.

To assess the term quantitatively, % Completeness will be expressed by the following equation:

% Completeness = N x 100

T

where,

N = Number of usable results

T = Total targeted number of samples planned to be collected

All data collected in this project will be used to determine the quality of water in the Scott River Watershed and the Shackleford/Mill Creek sub-basin. Due to a variety of circumstances, sometimes not all samples scheduled to be collected can be collected (e.g., a creek may be dry, etc.) or the data from the samples cannot be used (e.g., samples are or bottles are broken in transit, sample holding times are grossly exceeded, etc.). For this surface water sampling project, the overall completeness goal has been set at 90% for each analytical parameter and field measurement type. If the completeness goal is not met, re-sampling and/or re-analyzing will be conducted.

At this point in time, no sampling locations have been deemed more critical to the overall project goal than any other. As such, there will be no qualitative assessment of

completeness to ensure that samples from critical locations have been collected and their associated data has been deemed usable to support the project objectives.

Representativeness is a qualitative measure of the degree to which data accurately and precisely represent a characteristic of the sampled population or environmental condition.

Representativeness will be assessed both quantitatively and qualitatively.

To assess this term quantitatively, an overall evaluation will be made of how well the precision and accuracy/bias assessments met their associated measurement performance criteria. An additional assessment will involve ensuring that a temperature blank sample has accompanied each cooler of samples that has a temperature requirement associated with its preservation (see Table 2.2.1) and that the temperature of these temperature blank samples are 4°C ± 2°C when received at the laboratory.

To assess this term qualitatively, no actual QC samples are involved. Instead, the evaluation will involve verifying that documented sample collection and analytical methods (including sample handling and chain-of-custody procedures, sample preservation, and sample holding time protocols) were followed.

The procedures identified throughout this QA Project Plan were chosen to optimize the potential for obtaining samples that reflect the true state of the environment, within practical limits. Long-term monitoring will increase the representativeness of the project in that it would enable an assessment of changes over time.

Comparability qualitatively expresses the confidence with which one data set can be compared to another. The use of standard, published methods in this project allows data to be compared to data from other regional projects and using the same methods throughout allows for comparison of data collected by the QVIR EPD in the future. Sampling methodologies are described in Sections 2.1 and 2.2 below. Expressing data using consistent units of measure also addresses comparability. Units of measure for each water quality parameter are listed in Table 1.7.1.

Sensitivity- the ability of a method to detect and quantify an analytical parameter of concern at the concentration level of interest will be assessed semi quantitatively. No actual QC samples are involved. Instead, the laboratory to perform the analyses has provided their QLs and DLs and demonstrated that these are lower than the project action limits (as shown in Table 1.7.1) or the majority of the analytical parameters. For field measurements, the sensitivity is defined by the instrument manufacturer.

| Table 1.7.2: Precision of sampling equipment by the QVIR EPD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Matrix | Parameter | Measurement Method | Precision | Accuracy | Measurement Range |
| Water | Depth | Keck Water Level Meter | ± 1.0% | .01 ft | 0 - 100 feet |
| Water | Discharge | Aqua Calc flow meter | 0.001 | n/a | Operating range: 0.1 - 18 fps Volume range: 999,999.99 CFS Temp range: -20°C to 70°C |
| Water | Temperature | Onset HOBO Water Temp Pro Loggers | ±0.2°C at 0° to 50°C (±0.36°F at 32° to 120° | ±0.2°C at 0° to 50°C (±0.36°F at 32° to 120° | 0° to 50°C (32° to 122°F) in water (non-freezing) |
| Water | Temperature | YSI 556 & 6600 MPS Multi Probe System: YSI Precision ™ Thermistor | 0.1°C | ± 0.15°C | YSI 556= -5 to 45°C YSI 6600= -5 to 60°C |
| Water | pH | YSI 556 & 6600 MPS Multi Probe System: YSI Glass Combination electrode | 0.01 units | ±0.2 units | 0 to 14 units |
| Water | Dissolved Oxygen | YSI 556 & 6600 MPS Multi Probe System Steady state polarographic | 0.01 mg/L | ±2% @ 0 to 20 mg/L ±6% @ 20 to 50 mg/L | 0 to 50 mg/L |
| Water | Conductivity | YSI 556 & 6600 MPS Multi Probe System: YSI 4-electrode cell with autoranging | 0.001 mS/cm to 0.1 mS/cm range-dependent | ± 0.5% + 0.001 mS/cm | YSI 556= 0 to 200 mS/cm YSI 6600= 0 to 100mS/cm |
| Water | Turbidity | YSI 6600 MPS Multi Probe System | 0.01 NTU | ± 2% | 0-1000 NTU |
| Water | Turbidity | Model WQ770 | n/a | +5 % of full scale | 0-50 NTU and 0-1000 NTU |

1.8	Special Training Requirements/Certificates

No special training of field personnel is required for this project. The QVIR Environmental Director is an experienced scientist who has been leading and training employees in conducting water quality investigations for over five years.  She has been trained by US Forest Service, Siskiyou and Shasta Resource Conservation District’s, and the Northern California Resource Center to calibrate, deploy and download HOBO temp loggers, flow meters, and hydolabs / data sondes according to established protocols.  She has been trained to sample benthic macroinvertebrates under the guidance of Jim Harrington from California Department of Fish and Game. In addition, the field staff will be attending a 3-day bioassessment workshop, which will include sampling procedures for benthic macroinvertebrates. The QVIR Environmental Director will oversee initial sampling events to ensure that field staff is following the guidelines of this QAPP.

The WQ Technician will keep clear records about how instructions from the Director were followed and make notes about any conditions that might cause anomalies in data. The QVIR EPD QA Officer will inspect the field and sampling equipment and periodically audit the WQ Technician to make sure that proper maintenance is taking place and is being documented.

The collection of all surface and ground water samples using hand held equipment will use standard field methods as described in this QAPP, which are derived from recognized U.S. EPA (1983; 2004) and U.S. Geologic Survey (USGS, 1998) protocols.

1.9	Documents and Records

QA Project Plan Distribution

It is the responsibility of the QVIR Environmental Director/QA Officer to prepare and maintain amended versions of the QA Project Plan and to distribute the amended QA Project Plan to the individuals listed in Section 1.3.  This QAPP, once approved, will be kept in printed form for ease of reference of the WQ Technician, QA Officer and QVIR Environmental Director. When updated plans are approved, one copy of an older version will be retained in the QVIR EPD library, but clearly stamped to indicate that it is no longer current. In addition, each page of the QAPP will be clearly labeled as to the version and date of revision.

Field Documentation and Records

In the field, records will be documented in several ways, including field logbooks, photographs, pre-printed forms (such as labels and chain-of-custody forms), corrective action reports, and field audit checklists and reports. Field activities must be conducted according to this QAPP.  All documentation generated by the sampling program will be kept on file in the office of the Quartz Valley Environmental Program.

Field Notebooks

Bound field logbooks will be used to record field observations, sampling site conditions, and on-site field measurements. These books will be kept in a permanent file in the QVIR EPD Office. At a minimum, information to be recorded in the field logbooks at each sample collection/measurement location includes:

- Sample location and description,

- Site or sampling area sketch showing sample location and measured distances,

- Sampler’s names,

- Date and time of sample collection,

- Designation of sample as composite or grab (for this project, all are grab samples),

- Type (media or matrix) of sample (for this project, all are surface water samples),

- Type of sampling equipment used (for this project, only sample bottles will be used),

- Type of field measurement instruments used, along with equipment model and serial number,

- Field measurement instrument readings,

- Field observations and details related to analysis or integrity of samples (e.g., weather conditions, noticeable odors, color),

- Preliminary sample descriptions (e.g., clear water with strong ammonia-like odor),

- Sample preservation,

- Lot numbers of the sample containers, sample identification numbers and any explanatory codes,

- Shipping arrangements (overnight air bill number), and

- Name(s) of recipient laboratory(ies).

In addition to the sampling information, the following specific information will also be recorded in the field logbook for each day of sampling:

- Team members and their responsibilities,

- Time of arrival/entry on site and time of site departure,

- Other personnel on site,

- Deviations from the QAPP or SOPs required in the field, and

- Summary of any meetings or discussions with tribal, contractor, or federal agency personnel.

Separate instrument/equipment notebooks or logbooks will be maintained for each piece of equipment or instrument. These logbooks will be used to record field instrument calibration and maintenance information. Each logbook with include the name, manufacturer, and serial number of the instrument/equipment, as well as dates and details of all maintenance and calibration activities.

Photographs

Digital photographs will be taken at each sampling location and at other areas of interest near the sampling area for every sampling event. The photographs will serve to verify information entered into the field logbook. Digital photographs will be archived in a permanent digital file to be kept in the QVIR EPD office.

For each photograph taken, the following information will be written in the field logbook or recorded in a separate field photography logbook:

- Time, date, location, and weather conditions,

- Description of the subject photographed,

- Direction in which the picture was taken, and

- Name and affiliation of the photographer.

Labels

All samples collected will be labeled in a clear and precise way for proper identification in the field and for tracking in the laboratory. The Laboratory will provide sample labels (see Appendix C1) for this project. The samples will have preassigned, identifiable, and unique numbers. At a minimum, the sample labels will contain the following information:

- Sampling location or name,

- Unique sample number,

- Sample description (e.g., grab, composite),

- Date and time of collection,

- Initials/signature of sampler,

- Analytical parameter(s), and

- Method of preservation.

Each sample location will have a unique sample identification number.

Field Quality Control Sample Records

Field QC samples (duplicates and blanks) will be labeled as such in the field logbooks. They will be given unique (fictitious) sample identification numbers and will be submitted “blind” to the laboratory (i.e., only the field logbook entry will document their identification and the laboratory will not know these are QC samples). The frequency of QC sample collection will also be recorded in the field logbook.

Sample Chain-of-Custody Forms and Custody Seals

Chain-of-custody forms and custody seals (see Appendix C2) will be provided by the laboratory. The forms will be used to document collection and shipment of samples for off-site laboratory analysis, while the seals will serve to ensure the integrity of (i.e., there has been no tampering with) the individual samples.

All sample shipments will be accompanied by a chain-of-custody form. The forms will be completed and sent with each shipment of samples to the laboratory. If multiple coolers are sent to a laboratory on a single day, forms will be completed and sent with the samples for each cooler. The original form will be included with the samples and sent to the laboratory. Copies will be sent to the QVIR Environmental Director/QA Officer.

The chain-of-custody form will identify the contents of each shipment and maintain the custodial integrity of the samples. Generally, a sample is considered to be in someone's custody if it is either in someone's physical possession, in someone's view, locked up, or kept in a secured area that is restricted to authorized personnel. Until the samples are shipped, the custody of the samples will be the responsibility of the field personnel, who will sign the chain-of-custody form in the "relinquished by" box and note the date, time, and air bill number.

A self-adhesive custody seal will be placed across the lid of each sample container/bottle. The shipping containers in which samples are stored will also be sealed with self-adhesive custody seals any time they are not in someone's possession or view before shipping, as well as during shipping. All custody seals will be signed and dated.

Laboratory Documentation and Records

The analytical laboratory will keep a sample receiving log and all completed chain-of-custody forms submitted with the samples collected for this project. The analytical laboratory will also keep records of all analyses performed, as well as associated QC information, including: laboratory blanks, matrix spikes, laboratory control samples, and laboratory duplicates. Hard copy data of the analytical results will be maintained for six years by the laboratory.

The data generated by the laboratory for each sampling event will be compiled into individual data packages/reports. The data packages will include the following information:

- Project narrative including a discussion of problems or unusual events (including but not limited to the topics such as: receipt of samples in incorrect, broken, or leaking containers, with improperly or incompletely filled out chain-of-custody forms, with broken chain-of-custody seals, etc.; receipt and/or analysis of samples after the holding times have expired; summary of QC results exceeding acceptance criteria; etc.),

- Sample results and associated QLs,

- Copies of completed sample receiving logs and chain-of-custody forms, and,

- QC check sample records and acceptance criteria (to be included for all QC samples listed in Tables 2.5.1 through 2.5.2, including the temperature blank check).

All data packages will be reviewed by the Laboratory QA Officer to ensure the accurate documentation of any deviations from sample preparation, analysis, and/or QA/QC procedures; highlights of any excursions from the QC acceptance limits; and pertinent sample data. Once finalized, the Laboratory QA Officer will provide the data packages/reports to the Laboratory Project Manager who will sign them and submit them to the QVIR Environmental Director/QA Officer. Any problems identified by the Laboratory QA Officer will be documented in the narrative part of the tribe’s report.

Information about the documentation to be provided by the analytical laboratory is also contained in the laboratory’s QA Manual (Appendix C3).

Technical Reviews and Evaluations

As part of the QA efforts for the project, on-going technical reviews will be conducted and documented. These reviews are associated with both field activities and the data generated by the off-site laboratory.

Field Audit Reports

The QVIR Environmental Director/QA Officer will observe selected sampling events to ensure that sample collection and field measurements are going according to plan. The results of the observations will be documented in a designated QA Audit Logbook. Once back in the office, the QVIR QA Officer will formalize the audit in a Field Audit Report to be forwarded to the QVIR Environmental Director and the QVIR EPD Water Quality Technician/Field Sampler.

Corrective Action Reports (following Field Audits)

Corrective action reports will be prepared by the QVIR EPD Water Quality Technician/Field Sampler in response to findings identified by the QVIR Environmental Director/QA Officer during field visits and audits. The reports will focus on plans to resolve any identified deficiencies and non-compliance issues that relate to on-going activities and problems of a systematic nature, rather than on one time mistakes. Corrective Action reports do not have a specific format, but will be handled as an internal memorandum.

Field Activities Review Checklist

At the end of each sampling event, a technical review will be conducted of field sampling and field measurement documentation to ensure that all information is complete and any deviations from planned methodologies are documented. This review is described in Section 3.1. The review, as well as comments associated with potential impacts on field samples and field measurement integrity, will be documented on a Field Activities Review Checklist (as provided in Appendix D1.)

Laboratory Data Review Checklist

Following receipt of the off-site laboratory’s data package for each sampling event, The QVIR EPD QA Officer/Data Manager will conduct a technical review of the data to ensure all information is complete, as well as to determine if all planned methodologies were followed and QA/QC objectives were met. The results of this review, as well as comments associated with potential impacts on data integrity to support project decisions, will be documented on a Laboratory Data Review Checklist (as provided in Appendix D2).

Project Document Backup and Retention

Hardcopies of field notebooks, checklists, laboratory results and other paperwork will be maintained in the QVIR EPD office water quality file for six years.  After six years, project files will be placed in long term storage.  The Tribe’s policy is to maintain records indefinitely.

Electronic data will be backed up on CDs at year end and placed into project files for storage.  Additionally, an external hard-drive will be used to backup all project data from computer hard-drives.   These drives will be stored in a fireproof safe nightly.

Biannual and Annual Reports

The QVIR Environmental Director/QA Officer is responsible for the preparation of biannual and annual reports (summarizing the year’s activities) to be submitted to the US EPA Grants Project Officer.

The biannual report should include, at a minimum:

- Table summarizing the results (including both laboratory data and field measurements),

- Final laboratory data package (including QC sample results),

- Brief discussion of the field and laboratory activities, as well as any deviations or modifications to the plans,

- Copies of Field Audit Reports and any associated Corrective Action Reports,

- Copies of Field Activities Review Checklists and Data Review Checklists,

- Discussion of any problems noted with the data, either from laboratory or field measurements,

- Discussion of any data points showing exceedence of Action Levels, and

- Recommendations/changes for the next sampling event.

The annual reports should include, at a minimum:

- Description of the project,

- Table summarizing the results (of all project data collected to date, including both laboratory data and field measurements),

- Final laboratory data package (including QC sample results),

- Discussion of the field and laboratory activities, as well as any deviations or modifications to the plans,

- Trends observed as a result of the year’s monitoring efforts,

- Copies of Field Audit Reports and any associated Corrective Action Reports (for the fourth quarter),

- Copies of Field Activities Review Checklists and Data Review Checklists (for the fourth quarter),

- Evaluation of the data in meeting the project objectives, including data exceeding Action Levels,

- Recommendations to the Tribal Council regarding exceedence which are occurring on an on-going basis, and

- Recommendations/changes for future project activities (e.g., adding/deleting sampling locations and/or analyses, modifications to SOPs, amendments to the QA Project Plans, etc.).

2.0 Data Generation

This section of the QA Project Plan describes how the samples will be collected, shipped, and analyzed.

2.1  Sampling Design

A total of 42 locations will be sampled for this surface water monitoring program. All the locations will be in the Scott River Watershed.  Water quality sampling will take place in the following water bodies with varying numbers of stations in each (Table 1.6.1):

- The Scott River will be sampled at 3 locations: downstream the convergence with Shackleford Creek (Wing property), Johnson’s Bar, South Fork at USFS lower boundary.

- Three tributaries to the main stem Scott River will be sampled: Kelsey, Canyon, Boulder and “Scott Bar” Mill Creek at the USFS lowest boundary.

- Seven tributaries to the East Fork Scott River will be sampled: Crater, Houston, Rail, Kangaroo, Grouse, Mule and Mill (East Fork) Creeks.

- The headwaters of Shackleford Creek will be sampled at 9 locations: inlet, outlet and in the lakes of Summit, Cliff and Campbell.

- Shackleford Creek will be sampled at 7 locations: convergence of Campbell/Cliff and Summit Lakes, convergence of Long High Creek, convergence with Back Meadows Creek, at Wilderness boundary, at Timber Vest property, at QVIR boundary, at a Tribal Trust property.

- The headwaters of Mill Creek will be sampled at 6 locations: inlet, outlet and in East and West Mill Creeks ponds.

- Mill Creek will be sampled at 4 locations: convergence of West and East Mill Creek Ponds, at USFS boundary, at BLM parcel, at Quartz Valley Elementary School.

- Sniktaw Creek will be sampled at 2 locations: top and bottom of Case property.

The sample locations and ID for each sampling location are included in Table 1.6.1. The samples to be collected are summarized in Table 1.6.2.

Most of the sample locations in this study are accessible by maintained paved or dirt roads, either County or Forest Service roads.  The QVIR EPD's 4-wheel drive vehicle will be used when sampling these locations.  The remaining sample locations will be accessed via hiking trails in the Marble Mountain Wilderness.  Table 1.6.3 describes sample locations, rationale, and accessibility.  Sampling technicians will hike in to wilderness locations using pack animals to carry sampling equipment.  All samples will be collected from the shoreline except samples collected at the lakes.  Lake samples will be obtained using an inflatable kayak to access the lake’s center.

All sampling locations will be recorded using global positioning system (GPS) equipment following the procedures included in Appendix E1.  Additionally, photo documentation will occur at each sampling location during every sampling event.

Samples will be collected weekly for a five year period beginning April 1 and will conclude October 31 at the 42 locations listed on Table 1.6.1 and shown on Figures 3.1, 3.2, 4, 5 and 6.  Analyses will include pesticides, chlorophyll, phytoplankton, nutrients, and macro-invertebrates.  Samples from each location will also be field tested for temperature, pH, dissolved oxygen, conductivity (as specific conductance), and turbidity, see Table 1.6.2.   Discharge will also be measured at each location during each sampling event.

A parameter may be removed from the monitoring program if the sampling results indicate it is not of concern or added if new land uses develop after the monitoring program begins or the monitoring data indicates other potential parameters to include.

Water samples will be collected from Sniktaw Creek upstream and downstream of an accumulation of solid waste, a suspected contamination source (Sample ID- SCU and SCD).  This approach will essentially bracket the suspected contamination and indicate any effects on water quality.  Additionally, water samples will be collected from the most upstream points along Shackleford and Mill Creeks and the Scott River (Sample ID- SLI, CLI, CALI, SFSR, ESRM) to the most downstream points, downstream of the Shackleford-Mill Creek confluence with the Scott River (Sample ID- SR).  The Scott River will also be sampled further downstream at Johnson’s Bar above the confluence with the Klamath River (Sample ID SRJB).  Tributaries to the mainstem Scott River canyon will also be sampled (Sample ID – MSRK, MSRC, MSRB, and MSRM). This rationale is being used since the most upstream sampling points in the Scott River watershed are assumed to be the least impacted by current land use activities while the most downstream point is assumed to be most impacted by current land use.  However, the rationale could change depending on the results from subsequent sampling events.  If the sample collection order changes, this will be noted in the biannual reports to the US EPA Grants Project Manager and documented in an amendment to the QA Project Plan.  The types and frequency of field QC samples to be collected are discussed in Section 2.5 and listed in Tables 2.5.1 through 2.5.2.  These locations offer a linear comparison of parameters with Karuk and Yurok water quality sampling, from the headwaters of the Scott River to the ocean.

Sample locations along the tributaries of the East Fork Scott River were selected instead of locations in the East Fork Scott River itself due to accessibility issues.  The East Fork of the Scott River is privately owned with no access granted at this time to the QVIR EPD or other local and federal agencies.

Flow measurements will allow assessment of dilution of potential pollutants or relationships of sources to climatic events and flow.  The Scott River and Mill Creek are perennial and can be sampled year-round. Shackleford Creek sometimes loses surface flow on the Reservation during late summer and fall. Consequently, sampling at SQVIR will be dependent on flow, rainfall, and the water year.

Measurements in Scott River and its tributaries will represent conditions that are shaped by management of the Scott River watershed as a whole and are expected to reflect temperature and sediment impairment, particularly turbidity. Sediment pollution could stem from several sources such as forestry practices, agriculture and grazing and gravel mining. Sediment and temperature are the focus of the current Scott River TMDL that will allow participation of QVIR EPD staff in analysis of watershed wide sediment problems. Subsequent updates of this QAPP may include additional data collection related to sediment and temperature impairment to assist in TMDL Implementation monitoring.

Continuous hourly temperature monitoring will take place at all stream and river sample locations using a Hobo Temp Datalogger.  During weekly sample collection data will be downloaded from each datalogger for analysis later in the office.  Temperature data will provide QVIR EPD with long-term temperature trends in order to evaluate the habitat suitability for salmonids.  Data may also be applicable in the Scott River TMDL Implementation. Field technicians will follow manufacturer’s instructions on downloading data from the dataloggers, found in Appendix E.

Continuous, year-round, hourly monitoring for temperature, turbidity, pH, dissolved oxygen, and conductivity will take place on the Scott River just below the mouth of Shackleford-Mill Creek (Location ID- SR) using a YSI 6600.  Continuous monitoring will provide QVIR EPD with a holistic picture of water quality conditions of the Scott River throughout seasonal changes. Adult and juvenile salmonids must navigate year round through the lower Scott on their way to and from the Reservation waters therefore the condition of this habitat is critical to their sustainability. Year round turbidity measurements have never been collected on the Scott River.

Water quality and quantity parameters to be sampled for each water body are listed in Table 1.7.1, including action levels that were chosen to comply with NCRWQCB Basin Plan (2002) standards and those set by U.S. EPA for protection of beneficial uses under the Clean Water Act.

2.2 Sampling Methods

The YSI Handheld 556 MPS will be used to gather water quality data at each site. The probe will be calibrated and used according to procedures outlined in the units manual Appendix E5. At each site, measurements will be taken at a consistent distance from the surface of the water.

The grab sampling method will be used to gather samples from the Scott River and all Creeks at each of the different locations. The sample will be taken from flowing, not stagnant water, and the sampler will be facing upstream in the middle of the stream. Samples will be collected by hand if the stream is at a wadeable stage, or with a sample bottle holder during larger flows. The bottle will be uncapped and the cap protected from contamination.  Water samples will be collected 6 - 12 inches below the water’s surface.  At each sampling location, all sample bottles/containers designated for a particular analysis will be filled sequentially before containers designated for another analysis are filled. High mountain lake samples will be taken at the lake’s center, via an inflatable kayak.

Stratified water sampling will be done in the lakes using a Van Dorn Alpha Vertical Water Sampling Bottle.  Instructions for using the Van Dorn bottle sample collection are in Appendix E4. Stratified sampling will occur in the epilimnion and hypolimnion layers. The depth of the thermocline will be found with the handheld YSI probe. Based on the depth of the thermocline, samples will be taken at the proper depths to represent each layer. All sampling depths will be recorded along with the YSI information. The Van Dorn bottle will be rinsed with deionized water between stratified samples.

Continuously monitoring HOBO temperature loggers and the Multiprobe YSI sonde will be deployed in areas expected to retain water throughout the summer. Some site locations will require us to remove the temperature loggers once the creek disconnects. Loggers will be checked at each site location when grab samples are taken. Data will be downloaded once the temp loggers and sonde have been retrieved. HOBO temperature loggers will be calibrated, deployed and retrieved according to the US Fish and Wildlife Service protocol (See Appendix Misc.).

Benthic Macro-invertebrates will be sampled at all sites except the open water lake and pond sites.  Specific sampling locations at the sites will be located in a riffle typical of the stream. An ideal location will be at least 3 ft x 3 ft, have cobble-sized rocks, fast moving water, and a depth of 3 to 12 inches, Sampling will be done using a kick net.  The kick net will be positioned at the downstream end of the sampling area and the sampler will slowly walk upstream.  The net should be stretched out to its full 3-foot width with the bottom edge lying firmly against the stream bed. No water should wash under or over the net.  If needed, small rocks can be used to weigh down the bottom edge of the net.  A "kick" is a stationary sampling accomplished by using the toe or heel of a boot and dislodging the upper layer of the stream bed one meter at a time.  If larger substrate is encountered, such as a large piece of wood, the object should be picked up and rubbed by hand or a small brush to dislodge the attached organisms.  To avoid losing macroinvertebrates that should be part of the sample, the sampler should not stand in or disturb the sampling area before the kick seine is in place.   The kick seine will be lifted out of the water with a forward scooping motion.  The kick seine should be carried to the stream bank and spread it out flat on a piece of white plastic. Specimens will be placed in preserved sampling containers (500 ml) for identification by contracted consultant. This data will be evaluated using regional indices of biological  integrity.

If a QC sample is to be collected at a given location, all containers designated for a particular analysis for both the sample and QC sample will be filled sequentially before containers for another analysis are filled. For field duplicate samples, containers with two different sample designations will be filled alternately.

Preservatives will be added after sample collection, if required, to avoid losing the preservatives and dilution of preservatives during sampling.  Once the samples are collected and preserved, they will be kept chilled (if appropriate) and processed for shipment to the laboratory. Care will be taken to not touch the lip of the sample bottle during sample collection and preservation, so as not to potentially contaminate the sample. Table 2.2.1 summarizes the sample bottle/containers, volumes, and preservation requirements for each analysis and field measurement.

For other contaminants that require a preservative, guidelines presented in the QA manuals from contracted laboratories will be used (see Appendix C). If the option is given of a shorter hold time with no preservative, or a longer hold time with a preservative added to the sample, the longer hold time with a preservative will be the method chosen. After samples are taken, the bottles will be properly labeled, and placed into the appropriate cooler. All samples will be double-checked for the proper sample level, any potential leakage, and proper labeling before being sealed and shipped to the lab.  If the level of sample is different from the water level marked in the field at the time of sampling, the sample will be recorded as potentially tainted in the sampling log book.

Field Health and Safety Procedures

A brief tail-gate safety meeting will be held the first day of each sampling event to discuss emergency procedures (e.g., location of the nearest hospital or medical treatment facility), local contact information (e.g., names and telephone numbers of local personnel, fire department, police department), as well as to review the tribe’s contingency plan. All field sampling activities will be conducted with a buddy system (i.e., two field personnel will constitute the sampling team). This will allow for the presence of a second person to provide assistance and/or call in an emergency or accident for the other field person, if/when needed.

Level D personal protective equipment (PPE) will be used when collecting the surface water samples. At a minimum, safety glasses, plastic gloves, and steel-toed rain boots or waders will be worn. When wading, care will be taken to avoid slipping on rocks and algae. Also, due to weather conditions during the sampling events and the possibility of health concerns (e.g., heat stress) from working in high temperatures, field personnel will be advised to drink plenty of water and wear clothing (e.g., hat, long-sleeved shirt) that will cover and shade the body.

Potential routes of exposure related to field sampling and measurement activities are through the skin (e.g., from direct contact from the surface water) and/or by ingestion (e.g., from not washing up prior to eating). The use of Level D PPE, good hygiene, and following proper sampling procedures will minimize these potential exposures.

Field Measurements

Discharge will be measured using the AquaCalc flow meter. A tape measure will be stretched across the channel perpendicular to water flow. A minimum of ten measurements will be taken at each site from river right wetted edge to river left wetted edge. The water depth, the velocity at 40% of the water column (from the streambed up) and the distance from the wetted edge will be recorded for each measurement.

Surface water samples will be analyzed at each sample collection location for the following field measurement parameters: pH, dissolved oxygen, conductivity (as specific conductance), turbidity, and temperature.  Field measurements will be taken at each location prior to sample collection laboratory analysis. All field instruments will be calibrated (according to the manufacturer’s instructions) at the beginning of each date of sampling and checked at the end of each day. Field instrument calibration and sample measurement data will be recorded in the field logbook.

Field Variances

As conditions in the field vary, it may become necessary to implement minor modifications to the sampling procedures and protocols described in this QA Project Plan. If/when this is necessary; the QVIR EPD Field Sampler will notify the QVIR Environmental Director/QA Officer of the situation to obtain a verbal approval prior to implementing any changes. The approval will be recorded in the field logbook. Modifications will be documented in the Quarterly Reports to the US EPA Grants Project Officer.

Decontamination Procedures

For the currently planned sample collection activities, samples will be collected directly into sample bottles/containers provided from the laboratory. As such, no field decontamination of these bottles (used as the sampling equipment) is necessary. The bottles will be provided and certified clean by the laboratory according to procedures described in the laboratory’s QA Manual provided in Appendix C3.

In the case that there is a need to collect surface water samples by an alternative method decontamination of reusable sampling equipment coming in direct contact with the samples will be necessary. Decontamination will occur prior to each use of a piece of equipment and after use at each sampling location. Disposable equipment (intended for one-time use) will not be decontaminated but will be packaged for appropriate disposal. All reusable/non-disposable sampling devices will be decontaminated according to US EPA Region 9 recommended procedures using the following washing fluids in sequence:

- Non-phosphate detergent and tap water wash (using a brush, if necessary),

- Tap-water rinse, and

- Deionized/distilled water rinse (twice).

Equipment will be decontaminated in a predesignated area on plastic sheeting. Cleaned small equipment will be stored in plastic bags. Materials to be stored more than a few hours will also be covered.

Disposal of Residual Materials

In the process of collecting water samples for this project, various types of potentially contaminated wastes will be generated which may include the following:

- Used PPE,

- Disposable sampling bottles/containers or equipment,

- Decontamination fluids, and

- Excess water collected for sample container filling.

The USEPA's National Contingency Plan requires that management of the wastes generated during sampling comply with all applicable or relevant and appropriate requirements to the extent practicable.  Residuals generated for this project will be handled in a manner consistent with the Office of Emergency and Remedial Response (OERR) Directive 9345.3-02 (May 1991), which provides the guidance for the management of wastes. In addition, other legal and practical considerations that may affect the handling of the wastes will be considered, as follows:

- Used personal protective equipment (PPE) and disposable containers/equipment will be double bagged and placed in a municipal refuse dumpster. These wastes are not considered hazardous and can be sent to a municipal landfill. Any used PPE and disposable containers or equipment (even if it appears to be reusable) will be rendered inoperable before disposal in the refuse dumpster.

- Decontamination fluids generated in the sampling event could consist of deionized water, residual contaminants, and water with non-phosphate detergent. The volume and concentration of the decontamination fluid will be sufficiently low to allow disposal at the sampling area. The water (and water with detergent) will be poured onto the ground.

- Excess water collected for sample container filling will be poured onto the ground.

Quality Assurance for Sampling

Detailed instructions for collection of all field QC samples is discussed in Section 2.5 and listed in Tables 2.5.1 and 2.5.2.

Documentation of deviations from this QA Project Plan is the responsibility of the QVIR EPD QA Officer. Deviations noted during the field audit will be documented in the QA Audit Logbook, recorded in the Field Audit Reports, and discussed in the biannual reports.

Additional deviations from the QA Project Plan may be implemented as field variances or modifications. These deviations will be communicated to the QVIR Environmental Director/QA Officer by the QVIR EPD Water Quality Technician/Field Sampler for approval. The approval will be recorded in the field logbook, and the modifications will be documented in the Quarterly Reports.

| Table 2.2.1. Required sample containers, volumes, preservation methods, analysis method and holding times for water samples requiring laboratory analysis. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Analysis | Container Type | Sample Volume | Preservation Method | Maximum Holding Time | Laboratory Detection Limit | Analysis Method | Inorganic5 No. of:1 |  |
|  |  |  |  |  |  |  | Dup | MS |
| Macro-invertebrates | Plastic Bottle | 500 ml | 95% denatured CH3CH2OH (ethanol) | 15 days | N/A | Level 1 CSBP2 | N/A |  |
| Total Phosphorous (TPO4) | Poly | 250 ml | H2SO4 | 28 DAYS | 0.050 ppm | EPA 365.2 | 1 Dup and MS per analytical batch 500 ml |  |
| Dissolved Phosphorus | Same Bottle As T-P | 250 ml | H2SO4 | 28 DAYS | 0.050 ppm | EPA 365.2 | Same as bottle as T-P Dup and MS |  |
| Total Nitrogen | Poly | 125 ml | None | 28 DAYS | 0.40 ppm | EPA 351.3 | N/A |  |
| Ammonium Nitrogen | Poly | 500 ml | H2SO4 | 28 Days | .10 ppm | EPA 350.2 | 1 Dup and MS per analytical batch 1liter |  |
| Nitrate + Nitrite | Same Bottle As T-P | 125 ml | 4 DEG C, | 48 HRS | 400 ppb | EPA 300.0 | 1 Dup and MS per analytical batch |  |
| Phytoplankton | Poly | 250 ml | 1% Lugol solution | 1 year or more | 0.45 micrometer membrane filter | Standard Methods, 1992,10200.F.2.c | 10% of samples for duplication |  |
| Chlorophyll a | Amber Glass | 1 liter | None | 24 Hrs To Filtration | 2 ppb | SM10200H2B | 1 Dup and MS per analytical batch 2 liters |  |
| Pesticides- Trifluralin | Amber Glass | 1 liter | None | 7 days | 1.0 ug/l | EAP 8141A | Extra Liter for Duplicate |  |
| Pesticides- Diuron | Amber Glass | 1 liter | None | 7 days | 1.0 ug/l | EPA 632 | Extra Liter for Duplicate |  |

2.3 Sample Handling and Custody

This section describes the sample handling and custody procedures from sample collection through transport and laboratory analysis. It also includes procedures for the ultimate disposal of the samples.

Sample Containers & Preservatives

The QVIR Environmental Director has worked directly with the Laboratory Project Manager to determine the number of sample containers, and associated sizes/volumes and materials, needed for this monitoring project. The containers will be provided precleaned from the laboratory directly and require no washing or rinsing by the field samplers prior to sample collection.

Preservatives will also be provided by the laboratory. Sample bottles will not be pre-preserved. Instead, the preservative will be added to the sample containers by the field team immediately following sample collection.

Container and preservative information will be documented in the field logbook.

Sample Packaging and Shipping

All sample containers will be placed in a sturdy shipping container (e.g., a steel-belted cooler). The following outlines the packaging procedures that will be followed for this project:

- Line the bottom of the cooler with a large trash bag to minimize leakage of water.

- Place bubble wrap around the inside edge of the cooler to prevent breakage during shipment, and/or wrap bottles individually.

- Seal the drain plug of the cooler with fiberglass tape to prevent potential leakage from the cooler (should sample bottles or bagged ice leak.)

- Prepare bags of ice to be used to keep the samples cool during transport. Ice will be used. Pack the ice in doubled, zip-locked plastic bags.

- Check the sample bottle screw caps for tightness and, if not full, mark the sample volume level of liquid samples on the outside of the sample bottles with indelible ink.

- Secure sample bottle/container tops and place a custody seal over the container’s top.

- Ensure sample labels are affixed to each sample container and protected by a cover of clear tape.

- Wrap all glass sample containers in bubble wrap to prevent breakage.

- Seal all sample containers in heavy duty plastic zip-lock bags. Write the sample numbers on the outside of the plastic bags with indelible ink.

- Place sample containers (wrapped and sealed) into the cooler. Place the bagged ice on top and around the samples to chill them to the correct temperature.

- Fill the empty space in the cooler with bubble wrap, Styrofoam peanuts, or any other available inert material to prevent movement and breakage during shipment.

- Enclose the appropriate chain-of-custody(s) in a zip-lock plastic bag and affix to the underside of the cooler lid.

- Close the lid of the cooler. Tape the cooler shut with fiberglass strapping tape.

- Affix custody seals across the openings of the cooler both front and back to ensure that samples are not tampered with during transport. Include sample packer’s initials and date on the custody seals.

Daily, the QVIR Field Samplers will notify the Laboratory Project Manager of the sample shipment schedule. The laboratory will be provided with the following information:

  - Sampler’s name,

  - Name and location of the site or sampling area,

  - Names of the tribe and project,

  - Total number(s) and matrix of samples shipped to the laboratory,

  - Carrier, air bill number(s), method of shipment (e.g., priority next day),

  - Shipment date and when it should be received by the laboratory,

  - Irregularities or anticipated problems associated with the samples, and

  - Whether additional samples will be shipped or if this is the last shipment.

Sample Custody

The field sampler is responsible for custody of the samples until they are delivered to the laboratory or picked up for shipping. (Note: As few people as possible will handle the samples to ensure sample custody.) Chain-of-custody forms must be completed in the field. Each time one person relinquishes control of the samples to another person, both individuals must complete the appropriate portions of the chain-of-custody form (see Appendix C2) by filling in their signature as well as the appropriate date and time of the custody transfer.

During transport by a commercial carrier, the air bill will serve as the associated chain-of-custody. Once at the laboratory, the sample receipt coordinator will open the coolers and sign and date the chain-of-custody form. The laboratory personnel are then responsible for the care and custody of samples. The analytical laboratory will track sample custody through their facility using a separate sample tracking form, as discussed in the laboratory QA Manual included in Appendix C3.

A sample is considered to be in one’s custody if:

- The sample is in the sampler’s physical possession,

- The sample has been in the sampler’s physical possession and is within sight of the sampler,

- The sample is in a designated, secure area, and/or

- The sample has been in the sampler’s physical possession and is locked up.

Sample Disposal

Following sample analysis, each laboratory will store the unused portions for an established length of time (see lab QA/QC Manual’s in Appendix C3). At that time, the laboratory will properly dispose of all the samples (if applicable). Sample disposal procedures at the laboratory are discussed in the laboratory’s QA Manual included in Appendix C3.

Analytical Methods

The field measurement and off-site laboratory analytical methods are listed in Table 2.5.1 and discussed below.

Field Measurement Methods

See Section 2.2

Laboratory Analyses Methods (Off-Site)

Surface water samples will be analyzed at Aquatic Research Inc., North Coast Laboratories, Ltd., and Aquatic Analysts. Analyses will be performed following either EPA-approved methods or methods from Standard Methods for the Examination of Water and Wastewater, 20th Edition, as summarized in Table 2.2.1.  SOPs for the analytical methods are included in Appendix C. The Laboratory QA/QC Officer must notify the Laboratory Project Manager if there is any knowledge of the SOPs not being followed.

Benthic Macro invertebrates will be analyzed by Jon Lee Consulting.  Macro invertebrates are determined to the genus level.  The California Stream Bioassessment Procedure Taxonomic Level 1, as outlined in the CAMLnet Short List of Taxonomic Effort, is followed.  Samples are subsampled to a 300, 500, or higher specimen count - protocol dependent.  Each sample is placed into a 500 micron sieve.  Larger debris is carefully inspected for clinging organisms, thoroughly rinsed, and returned to the original container.  The sample is placed in water for approximately five minutes and strained.  The sample is moved to a gridded pan (25 cm.2 grids) and evenly spread among the grids.  The number of grids covered by the sample is recorded.  At least five grid numbers are randomly selected.  Macroinvertebrates are systematically removed from the selected grids and placed into vials containing 70% ethanol until the targeted number of specimens have been removed.  The number removed from each grid is recorded.  All specimens are removed from the last grid processed and this number is recorded.  The number of specimens removed from each grid provide an estimate of sample relative abundance.  The processed sample debris from each grid is placed into a container labeled remnant.  The remaining sample is returned to the original sample container and labeled original.

Both the laboratory and consultant will summarize the data and associated QC results in a data report, and provide this report to the QVIR Environmental Director within 21 days of receipt.  The QVIR Environmental Director/QA Officer will review the data reports and associated QC results to make decisions on data quality and usability in addressing the project objectives.

2.5 Quality Control Requirements

This section identifies the QC checks that are in place for the sample collection, field measurement, and laboratory analysis activities that will be used to access the quality of the data generated from this project.

Field Sampling Quality Control

Field sampling QC consists of collecting field QC samples to help evaluate conditions resulting from field activities. Field QC is intended to support a number of data quality goals:

- Combined contamination from field sampling through sample receipt at the laboratory (to assess potential contamination from field sampling equipment, ambient conditions, sample containers, sample transport, and laboratory analysis) - assessed using field blanks;

- Sample shipment temperature (to ensure sample integrity and representativeness that the sample arriving at the laboratory has not degraded during transport) - assessed using temperature blanks; and

- Combined sampling and analysis technique variability, as well as sample heterogeneity - assessed using field duplicates.

For the current project, the types and frequencies of field QC samples to be collected for each field measurement and off-site laboratory analysis are listed in Table 2.5.1 and 2.5.2. These include field blanks, temperature blanks (as included in a footnote to the table), and field duplicates.

Field Blanks - Field blanks will be collected to evaluate whether contaminants have been introduced into the samples during the sample collection due to exposure from ambient conditions or from the sample containers themselves. Field blank samples will be obtained by pouring deionized water into a sample container at the sampling location. Field blanks will not be collected if equipment blanks have been collected during the sampling event. If no equipment blanks are collected (and none are planned because samples will be collected directly into sample containers), one field blank will be collected for every 10 samples or a frequency of 10%.

Field blanks will be preserved, packaged, and sealed in the same manner described for

the surface water samples. A separate sample number and station number will be

assigned to each blank. Field blanks will be submitted blind to the laboratory for

invalidation of results, greater attention to detail during the next sampling event, or analysis of metals, hardness, and anions. No field blanks are planned for the other analytical parameters or field measurements as it is not expected that it would yield information critical to project data needs.

If target analytes are found in field blanks, sampling and handling procedures will be reevaluated and corrective actions taken. These may consist of, but are not limited to, obtaining sampling containers from new sources, training of personnel, discussions with the laboratory other procedures felt appropriate.

Temperature Blanks -For each cooler of samples that is transported to the analytical

laboratory, a 40-ml VOA vial (prepared by the laboratory) will be included that is marked

“temperature blank.” This blank will be used by the laboratory’s sample custodian to

check the temperature of samples upon receipt to ensure that samples were maintained

at the temperature appropriate for the particular analysis.  For the current project, temperature blanks will be included in all coolers containing samples requiring temperature preservation, as identified in Table 2.2.1.

Field Duplicate Samples - Field duplicate samples will be collected to evaluate the precision of sample collection through analysis. Field duplicates will be collected at designated sample locations by alternately filling two distinct sample containers for each analysis. Field duplicate samples will be preserved, packaged, and sealed in the same manner described for the surface water samples. A separate sample number and station number will be assigned to each duplicate. The samples will be submitted as “blind” (i.e., not identified as field duplicates) samples to the laboratory for analysis.

For the current project, field duplicates will be collected for each analytical parameter, and field measurement parameter, at the frequencies shown in Table 2.5.1. The duplicates samples will be collected at random locations for each sampling event. Criteria for field duplicates for the analytical and field measurement parameters are provided in Tables 2.5.1 through 2.5.2, respectively. If criteria are exceeded, field sampling and handling procedures will be evaluated, and problems corrected through greater attention to detail, additional training, revised sampling techniques, or whatever appears to be appropriate to correct the problems.

Field Measurement Quality Control

Quality control requirements for field measurements are provided in Table 2.5.2.

Laboratory Analyses Quality Control (Off-Site)

Laboratory QC is the responsibility of the personnel and QA/QC department of the contracted analytical laboratories. Each laboratory’s Quality Assurance Manuals detail the QA/QC procedures it follows. The following elements are part of standard laboratory quality control practices:

- Analysis of method blanks,

- Analysis of laboratory control samples,

- Instrument calibration (including initial calibration, calibration blanks, and calibration verification),

- Analysis of matrix spikes, and

- Analysis of duplicates.

The data quality objectives for Aquatic Analysts, California Laboratory Services, and North Coast Laboratories (including frequency, QC acceptance limits, and corrective actions if the acceptance limits are exceeded) are detailed in the QA Manuals and SOPs (as in Appendix C).  Any excursions from these objectives must be documented by the laboratory and reported to Quartz Valley Indian Reservation Project Manager/QA Officer.

The Tribe has reviewed each laboratory’s control limits and corrective action procedures and feels that these will satisfactorily meet tribal project data quality needs. A summary of this information is included in Tables 2.5.1 through 2.5.2. These include laboratory (or method) blanks, laboratory control samples, matrix spikes, and laboratory duplicates.

Method Blanks - A method blank is an analyte-free matrix, analyzed as a normal sample by the laboratory using normal sample preparation and analytical procedures. A method blank is used for monitoring and documenting background contamination in the analytical environment. Method blanks will be analyzed at a frequency of one per sample batch (or group of up to 20 samples analyzed in sequence using the same method).

Corrective actions associated with exceeding acceptable method blank concentrations include isolating the source of contamination and re-digesting and/or re-analyzing the associated samples. Sample results will not be corrected for blank contamination, as this is not required by the specific analytical methods. Corrective actions will be documented in the laboratory report’s narrative statement.

Laboratory Control Samples - Laboratory control samples (LCS) are laboratory-generated samples analyzed as a normal sample and by the laboratory using normal sample preparation and analytical procedures. An LCS is used to monitor the day-to-day performance (accuracy) of routine analytical methods. An LCS is an aliquot of clean water spiked with the analytes of known concentrations corresponding to the analytical method. LCS are used to verify that the laboratory can perform the analysis on a clean matrix within QC acceptance limits. Results are expressed as percent recovery of the known amount of the spiked analytical parameter.

One LCS is analyzed per sample batch. Acceptance criteria (control limits) for the LCS are defined by the laboratory and summarized in Tables 2.5.1 through 2.5.2. In general, the LCS acceptance criteria recovery range is 70 to 130 percent of the known amount of the spiked analytical parameter. Corrective action, consisting of a rerunning of all samples in the affected batch, will be performed if LCS recoveries fall outside of control limits. Such problems will be documented in the laboratory report’s narrative statement.

Matrix Spikes - Matrix spikes (MS) are prepared by adding a known amount of the analyte of interest to a sample. MS are used as a similar function as the LCS, except that the sample matrix is a real-time sample rather than a clean matrix. Results are expressed as percent recovery of the known amount of the spiked analytical parameter. Matrix spikes are used to verify that the laboratory can determine if the matrix is causing either a positive or negative influence on sample results.

One matrix spike is analyzed per sample batch. Acceptance criteria of the MS are defined by the laboratory and summarized in Tables 2.5.1 through 2.5.2.  In general, the MS acceptance criteria recovery range is of 70 to 130 percent of the known amount of the spiked analytical parameter. Generally, no corrective action is taken for matrix spike results exceeding the control limits, as long as the LCS recoveries are acceptable. However, the matrix effect will be noted in laboratory report’s narrative statement and documented in the Tribe’s reports for each sampling event.

Laboratory Duplicates - A laboratory duplicate is a laboratory-generated split sample used to document the precision of the analytical method. Results are expressed as relative percent difference between the laboratory duplicate pair.

One laboratory duplicate will be run for each laboratory batch or every 10 samples, whichever is more frequent. Acceptance criteria (control limits) for laboratory duplicates are specified in the laboratory QA Manual and SOPs and are summarized in Tables 2.5.1 through 2.5.2. If laboratory duplicates exceed criteria, the corrective action will be to repeat the analyses. If results remain unacceptable, the batch will be rerun. The discrepancy will be noted in the laboratory report’s narrative statement and documented in the Tribe’s reports for each sampling event.

Background Samples

Background samples are collected because there is a possibility that there are native or ambient levels of one or more target analytes present, and because one objective of the sampling event is to differentiate between on-site and off-site contributions to a parameter’s concentration. The background location for this monitoring program will be the most upstream (and thus assumed to be least impacted) sample collected at the following locations: Summit, Cliff, Campbell inlet and lake samples, West and East Mill Creek ponds inlet and lake samples, upstream of the Case property on Sniktaw, Crater, Houston, Rail, Kangaroo, Grouse, Mill, Mule, S. Fork Scott, Kelsey, Canyon and Boulder Creeks.  The analyses to be conducted on the background samples will be the same as that for the other surface water samples.

| Table 2.5.1: Summary of Field and QC Samples Water Monitoring Program Quartz Valley Indian Reservation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Matrix/ Media | Analytical Parameter |  | No. of Sampling Locations |  | Depth (surface, mid, or deep) | No. of Field Duplicates |  | Inorganic5 No. of: |  |  |  | No. of Field Blanks | Total No. of Samples |
|  |  |  |  |  |  |  |  | Dup |  |  | MS |  |  |
| Analyses: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Surface Water | Total Phosphorus | 42 |  | Surface (grab) |  |  | 4 |  | 1 | 1 |  | 1 | 49 |
| Surface Water | Dissolved Phosphorus | 42 |  | Surface (grab) |  |  | 4 |  | 1 | 1 |  | 1 | 49 |
| Surface Water | Total Nitrogen | 42 |  | Surface (grab) |  |  | 4 |  | 0 | 0 |  | 1 | 47 |
| Surface Water | Ammonium Nitrogen | 42 |  | Surface (grab) |  |  | 4 |  | 1 | 1 |  | 1 | 49 |
| Surface Water | Nitrate + Nitrite | 42 |  | Surface (grab) |  |  | 4 |  | 1 | 1 |  | 1 | 49 |
| Surface Water | Phytoplankton | 42 |  | Surface (grab) |  |  | 4 |  | 10% of samples |  |  | 1 | 51 |
| Surface Water | Chlorophyll | 42 |  | Surface (grab) |  |  | 4 |  | 1 | 1 |  | 1 | 49 |
| Surface Water | Pesticides- Trifluralin | 10 |  | Surface (grab) |  |  | 1 |  | 1 liter extra |  |  | 1 | 13 |
| Surface Water | Pesticides- Diuron | 10 |  | Surface (grab) |  |  | 1 |  | 1 liter extra |  |  | 1 | 13 |
| Surface Water | Macro-invertebrates | 42 |  | Surface (grab) |  |  | 4 |  | NAS |  |  | 1 | 47 |
| Field Measurements: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Surface Water | Temp. | 42 |  | Surface (grab) |  |  | 4 |  |  |  |  | 0 | 46 |
| Surface Water | pH | 42 |  | Surface (grab) |  |  | 4 |  |  |  |  | 0 | 46 |
| Surface Water | Conductivity | 42 |  | Surface (grab) |  |  | 4 |  |  |  |  | 0 | 46 |
| Surface Water | Turbidity | 42 |  | Surface (grab) |  |  | 4 |  |  |  |  | 0 | 46 |
| Surface Water | Dissolved Oxygen | 42 |  | Surface (grab) |  |  | 4 |  |  |  |  | 0 | 46 |

All analyses will be performed at an off-site laboratory. There will be no field screening analyses. Field measurements will be performed at each sample collection location.

2 Samples will be collected at depth of 6-12 inches. If depth of water is less than 12 inches, sample will be collected at mid depth and noted in the field logbook.

3 Field duplicates will be collected at a frequency of 10% of the samples collected for laboratory analysis. Field duplicates will be collected at a frequency of 10% or one per day, whichever is more frequent, for samples collected for field measurements.

4 Include number of associated analytical QC samples if collection of additional sample volume and/or bottles is necessary. If the QC samples listed are part of the analysis but no additional sample volume and/or bottles are needed, include “NAS” (for “no additional sample”) in the column. (Note: MS=matrix spike, MSD=matrix spike duplicate, dup=laboratory duplicate/replicate.)

5 Field blanks will be collected at a frequency of 10% of the of samples collected, or one per day, whichever is less frequent. Field blanks will not be collected, as they were determined not to be critical, to support laboratory analysis of Total Dissolved Solids, alkalinity, total coliform, e. coli or for field measurements.

6 Temperature blank samples will be submitted with each cooler of samples. These samples were not included in the sample count, as they are not carried though the analyses.

| Table 2.5.2: Quality Control Requirements for Surface Water Field Measurements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Field Parameters: Temperature, pH, Dissolved Oxygen, Turbidity, Conductivity |  |  |  |  |  |
| QC Sample | Data Quality Indicator (DQI) | Frequency/ Number | Methods/SOP QC Acceptance Limits | Acceptance Criteria/ Measurement Performance criteria | Corrective Action |
| Temperature- YSI 556 & 6600 MPS Multi Probe System: YSI Precision ™ Thermistor |  |  |  |  |  |
| Field Duplicate | Precision (S & A) | 1/5 field samples | N/A | ± 0.15°C | Collect & analyze 3rd sample. Qualify data if still exceeding criteria |
| QC Check Sample | Accuracy | N/A | N/A | N/A | None. Sensor not used if it didn’t meet annual calibration criteria. |
| Temperature- Onset HOBO Water Temp Pro Loggers |  |  |  |  |  |
| Field Duplicate | Precision (S & A) | 1/5 field samples | N/A | ±0.2°C | Collect & analyze 3rd sample. Qualify data if still exceeding criteria |
| QC Check Sample | Accuracy | N/A | N/A | N/A | None. Sensor not used if it didn’t meet annual calibration criteria. |
| pH- YSI 556 & 6600 MPS Multi Probe System: YSI Glass Combination electrode |  |  |  |  |  |
| Field Duplicate | Precision (S & A) | 1/5 field samples | N/A | ±0.2 units | Collect & analyze 3rd sample. Qualify data if still exceeding criteria |
| QC Check Sample6 | Accuracy | 1/batch each day | ±0.5 units of true value for both calibration check standards | ±0.5 units of true value | Qualify associated field data |
| Dissolved Oxygen- YSI 556 & 6600 MPS Multi Probe System Steady state polarographic |  |  |  |  |  |
| Field Duplicate | Precision (S & A) | 1/5 field samples | N/A | ±20% RPD | Collect & analyze 3rd sample. Qualify data if still exceeding criteria |
| QC Check Sample6 | Accuracy | 1/batch each day | ±0.5 mg/L of true value of full saturation standard | ±0.5 mg/L of true value | Qualify associated field data |
| Conductivity- YSI 556 & 6600 MPS Multi Probe System: YSI 4-electrode cell with autoranging |  |  |  |  |  |
| Field Duplicate | Precision (S & A) | 1/5 field samples | N/A | ±20% RPD | Collect & analyze 3rd sample. Qualify data if still exceeding criteria |
| QC Check Sample6 | Accuracy | 1/batch each day | ±10% of true value or ±20 μS/cm (whichever is greater) for both calibration check standards | ±10% of true value | Qualify associated field data |
| Turbidity- YSI 6600 MPS Multi Probe System |  |  |  |  |  |
| Field Duplicate | Precision (S & A) | 1/5 field samples | N/A | ±20% RPD | Collect & analyze 3rd sample. Qualify data if still exceeding criteria |
| QC Check Sample6 | Accuracy | 1/batch each day | ±20% or ±2 NTU of 20 NTU standard (whichever is greater) and ±1 NTU for 0 NTU standard | ±20% of true value | Qualify associated field data |
| Turbidity- Model WQ770 Turbidity Meter |  |  |  |  |  |
| Field Duplicate | Precision (S & A) | 1/5 field samples | N/A | ±20% RPD | Collect & analyze 3rd sample. Qualify data if still exceeding criteria |
| QC Check Sample6 | Accuracy | 1/batch each day | ±20% or ±2 NTU of 20 NTU standard (whichever is greater) and ±1 NTU for 0 NTU standard | ±20% of true value | Qualify associated field data |

Methods are provided in Appendix A-2.

2 Data Quality Indicators may be related to sampling (S) and/or analysis (A) activities.

3 For field duplicate samples, there are no method-specific QC acceptance limits. (NA - Not applicable.)

4 The information in this column supports acceptance criteria/measurement performance criteria introduced in Section 1.7 For this study, the field measurement’s QC acceptance limits (as determined from a calibration check sample analyzed half-way through the field day) were reviewed and found acceptable to meet the current data quality needs. As such, the field measurement’s QC acceptance limits and the project’s measurement performance criteria are equivalent.

5 Accuracy is not ensured through the analysis of a QC check. If the temperature sensor meets the annual calibration procedures and criteria presented in Table 2.7.1, the measurements are considered accurate enough to meet the needs of the current project.

6 Accuracy is ensured through the calibration and calibration check process presented in Table 2.7.1. The post calibration check sample(s) will be considered as QC check samples for the field measurements.

ALL SAMPLES ARE SURFACE WATER MATRIX. ALL SAMPLES ARE COLLECTED BY THE SAME PROCEDURE. NO ADDITIONAL QC CHECKS ARE PLANNED BEYOND THOSE IDENTIFIED ABOVE FOR ACCURACY AND PRECISION.

2.6 Instrument/Equipment Testing, Inspection, and Maintenance

Field Measurement Instruments/Equipment

Sampling equipment under the care of the QVIR Environmental Program will be maintained according to the manufacturer’s instructions. Maintenance logs will be kept in the office of the QVIR Environmental Director/QA Officer. Each piece of equipment will have its own maintenance log. The log will document any maintenance and service of the equipment. A log entry will include the following information:

- Name of person maintaining the instrument/equipment,

- Date and description of the maintenance procedure,

- Date and description of any instrument/equipment problem(s),

- Date and description of action to correct problem(s),

- List of follow-up activities after maintenance (i.e., system checks), and

- Date the next maintenance will be needed.

Laboratory Analysis Instruments/Equipment (Off-Site)

Inspection and maintenance of laboratory equipment is the responsibility of the Aquatic Analysts, Aquatic Research, and North Coast Laboratories and is described in each laboratory’s QA Manual included as Appendix C.

2.7 Instrument/Equipment Calibration and Frequency

Field Measurement Instrument/Equipment

Calibration and maintenance of field equipment/instruments will be performed according to the manufacturer’s instructions (see Appendix E) and recorded in an instrument/equipment logbook. Each piece of equipment/instrument will have its own logbook.

The project-specific criteria for calibration (frequency, acceptance criteria, and corrective actions associated with exceeding the acceptance criteria) are provided in Table 2.7.1.

Laboratory Analysis Instruments/Equipment

Laboratory instruments will be calibrated according to the appropriate analytical methods. Acceptance criteria for calibrations are found Aquatic Analysts, Aquatic Research, and North Coast Laboratories calibrations procedures are contained in each of their QA Manuals included as Appendix C.

| Table 2.7.1: Field Equipment Calibration, Maintenance, Testing and Inspection |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Analytical Parameter | Instrument | Calibration Activity | Maintenance & Testing/ Inspection Activity | Frequency | Acceptance Criteria | Corrective Action |
| Temperature (sensor) | YSI 556 & 6600 MPS Multi Probe System: YSI Precision ™ Thermistor |  | See Manufacturer’s manual | Initial Post: Once a week check and calibrate as needed | ± 0.15°C of true value at both endpoints | Remove from use if doesn’t pass calibration criteria |
| Temperature (sensor) | Onset HOBO Water Temp Pro Loggers | Initial: Water bath calibration against NIST thermometer (US Fish and Wildlife Protocol) | See Manufacturer’s manual |  | ±0.2°C of true value at both endpoints | Remove from use if doesn’t pass calibration criteria |
| pH (electrode) | YSI 556 & 6600 MPS Multi Probe System: YSI Glass Combination electrode | Initial: two-point calibration bracketing expected field sample range (using 7.0 and either 4.0 or 10.0 pH buffer, depending on field conditions); followed by one-point check with 7.0 pH buffer Post: single-point check with 7.0 pH buffer | See Manufacturer’s manual | Initial Post: Once a week check and calibrate as needed | Initial: Two point calibration done electronically; one-point check (using 7.0 pH buffer) ±0.1 pH units of true value Post: ±0.5 pH units of true value with both 7.0 pH and either 4.0 or 10.0 pH buffer | Recalibrate Qualify data |
| Dissolved oxygen (membrane electrode) | YSI 556 & 6600 MPS Multi Probe System Steady state polarographic | Initial: One-point calibration with saturated air (need temp, barometric pressure); followed by two-point check with saturated air and zero Post: single-point check at full saturation | See Manufacturer’s manual | Initial Post: Once a week check and calibrate as needed | Initial: one-point calibration done electronically; two-point check with high (saturated) standard ± 0.2 mg/L of true value and low (zero) standard <0.5 mg/L Post: ±0.5 mg/L of true saturated value | Recalibrate; change membrane and recalibrate Qualify data |
| Turbidity (sensor) | YSI 6600 MPS Multi Probe System | Initial: two point calibration using 0 NTU (or deionized water) and 20 NTU standards to bracket expected sample range; follow by one point check with 20 NTU standard Post: two point check with high (20 NTU) and low ()NTU standards) | See Manufacturer’s manual | Initial Post: Once a week check and calibrate as needed | Initial: two-point calibration done electronically; one-point check (using 20 NTU standard) ±10% of true value Post: two-point check with high (20 NTU) standard ±20% or ±2 NTU (whichever is greater) of true value and low (0 NTU) standard ±1 NTU of true value | See Manufacturer’s manual |
| Turbidity (sensor) | Model WQ770 Turbidity Meter | Initial: two point calibration using 0 NTU (or deionized water) and 20 NTU standards to bracket expected sample range; follow by one point check with 20 NTU standard Post: two point check with high (20 NTU) and low ()NTU standards) | See Manufacturer’s manual | Initial: beginning of each day Post: end of each day | Initial: two-point calibration done electronically; one-point check (using 20 NTU standard) ±10% of true value Post: two-point check with high (20 NTU) standard ±20% or ±2 NTU (whichever is greater) of true value and low (0 NTU) standard ±1 NTU of true value | See Manufacturer’s manual |
| Conductivity (sensor) | YSI 556 & 6600 MPS Multi Probe System: YSI 4-electrode cell with autoranging | Initial: one point calibration at high (using 500 mS/cm standard) end of expected field sample range; followed by two-point check with High (500 mS/cm) and low (100 mS/cm) standards | See Manufacturer’s manual | Initial Post: Once a week check and calibrate as needed | Initial: one point calibration done electronically; two point check with high standard ±5% of true value and low standard ±10% of true value Post: two point check with high and low standards ±10% of true value or ±20 mS/cm whichever is greater | Recalibrate Qualify data |
| Velocity | AquaCalc 5000 Open Channel Flow Computer | N/A Standard ratings are built into the AquaCalc **See Manual |  |  |  |  |

2.8 Inspection and Acceptance of Supplies and Consumables

Field Sampling Supplies and Consumables

Sample containers and preservatives will be provided by the analytical laboratory. Containers will be inspected for breakage and proper sealing of caps. Other equipment such as sample coolers and safety equipment will be acquired by the Tribe. If reusable sampling equipment is acquired in the future, materials/supplies necessary for equipment decontamination will be purchased by the Tribe; however, this is not necessary for the present study. Any equipment deemed to be in unacceptable condition will be replaced.

Field Measurement Supplies and Consumables

Field measurement supplies, such as calibration solutions, will be acquired from standard sources, such as the instrument manufacturer or reputable suppliers. Chemical supplies will be American Chemical Society reagent grade or higher. The lot number and expiration date on standards and reagents will be checked prior to use. Expired solutions will be discarded and replaced. The source, lot number, and expiration dates of all standards and reagents will be recorded in the field log books.

Laboratory Analyses (Off-Site) Supplies and Consumables

Each of the laboratory’s requirements for supplies and consumables are described in its QA Manual which is provided in Appendix C.

Data Acquisition Requirements (Non-Direct Measurements)

To supplement field measurements and laboratory analytical activities conducted under this project, other potential “external” data sources will be researched. These sources include, but are not limited to, the U.S. Geological Survey, the California Department of Water Resources, the U.S. Environmental Protection Agency, the United States Forest Service, the Karuk Tribe of California, the Hoopa Tribe, and the Yurok Tribe. The primary use of this external data will be to help focus the Tribe’s data collection efforts (for example, the information may be used to identify new sites in the Scott Valley River watershed for future sampling).

If it appears that the “external” data might facilitate water body evaluation, the data will first be reviewed to verify that they are of sufficient quality to meet the needs of the project by examining:

- the sample collection and location information;

- the data to see whether they are consistent with known tribally-collected data from the same general vicinity; and

- the QA/QC information associated with the data.

If the data are of insufficient or unknown quality, limitations will be placed on its use in supporting project decisions. In general, it is anticipated that decisions for the current project will be based on data collected by the Tribe following this current QA Project Plan.

Data Management

All data collected by the QVIR Environmental Program will be maintained in appropriate bound notebooks and electronic databases. Data from the laboratory will be requested in both hard copy and electronic form. The electronic and hard copy results will be compared to ensure that no errors occurred in either format. If discrepancies are noted, the laboratory will be contacted to resolve the issues.

3.0 ASSESSMENT AND OVERSIGHT

This section describes how activities will be checked to ensure that they are completed correctly and according to procedures outlined in this QA Project Plan.

Assessment/Oversight and Response Actions

During the course of the project, it is important to assess the project’s activities to ensure that the QA Project Plan is being implemented as planned. This helps to ensure that everything is on track and serves to minimize learning about critical deviations toward the end of the project when it may be too late to remedy the situation. For the current project, the ongoing assessments will include:

- Field Oversight

- Readiness review of the field team prior to starting field efforts,

- Field activity audits, and

- Review of field sampling and measurement activities methodologies and documentation at the end of each event, and

- Laboratory Oversight - evaluation of laboratory data generated for each quarterly sampling event.

Details regarding these assessments are included below.

Field Oversight

Readiness Reviews

Sampling personnel will be properly trained by qualified personnel before any sampling begins and will be given a brief review of sampling procedures and equipment operation by the QVIR Environmental Director/QA Officer before each sampling event. Equipment maintenance records will be checked to ensure all field instruments are in proper working order. Adequate supplies of all preservatives and bottles will be obtained and stored appropriately before heading to the field. Sampling devices will be checked to ensure that they have been properly cleaned (for devices which might be reused) or are available in sufficient quantity (for devices which are disposable). Proper paperwork, logbooks, chain of custody forms, etc. will be assembled by the sampling technician. The QVIR Environmental Director/QA Officer will review all field equipment, instruments, containers, and paperwork to ensure that all is in readiness prior to the first day of each sampling event. Any problems that are noted will be corrected before the sampling team is permitted to depart the Tribe’s facilities.

Field Activity Audits

Once a month, the QVIR Environmental Director/QA Officer will assess the sample collection methodologies, field measurement procedures, and record keeping of the field team to ensure activities are being conducted as planned (and as documented in this QA Project Plan). Any deviations that are noted will be corrected immediately to ensure all subsequent samples and field measurements collected are valid. (Note: If the deviations are associated with technical changes and/or improvements made to the procedures, the QVIR EPD QA Officer will verify that the changes have been documented by the QVIR EPD Water Quality Technicians in the Field Log Book and addressed in an amendment to this QA Project Plan.) The QVIR EPD QA Officer may stop any sampling activity that could potentially compromise data quality.

The QVIR EPD QA Officer will document any noted issues or concerns in a QA Audit Logbook and discuss these items informally and openly with the QVIR EPD Water Quality Technicians while on site. Once back in the office, she will formalize the audit findings (for each event) in a Field Audit Report which will be submitted to the QVIR Environmental Director and the QVIR EPD Water Quality Technicians.

The QVIR EPD Water Quality Technician will prepare a Corrective Action Report to address any audit findings discussed in the Field Audit Report. The Corrective Action Report will be issued as an internal memorandum the QVIR Environmental Director/QA Officer in response to problems noted during on-site audits and will document steps taken to reduce future problems prior to the next sampling event.

Post Sampling Event Review

Following each sampling event, the QVIR EPD Data Manager will complete the Field Activities Review Checklist (Appendix D1). This review of field sampling and field measurement documentation will help ensure that all information is complete and any deviations from planned methodologies are documented. This review will be conducted in the office, not in the field. The results of this review, as well as comments associated with potential impacts on field samples and field measurement integrity will be forwarded to the QVIR Environmental Director to be used in preparing the reports for each event and also to be used as a guide to identify areas requiring improvement prior to the next sampling event.

Laboratory Oversight

Following receipt of the off-site laboratory’s data package for each sampling event, the QVIR EPD QA Officer will review the data package for completeness, as well as to ensure that all planned methodologies were followed and that QA/QC objectives were met. The results of the review will be documented on the Laboratory Data Review Checklist (Appendix D2). (Note: The QVIR Environmental Director/QA Officer has the authority to request re-testing or other corrective measures if the laboratory has not met the project’s QA/QC objectives and/or has not provided a complete data package.)

Due to the scope and objectives of the current project, the Tribe is not planning any laboratory audits at this time. However, the Tribe will check periodically with the state of California certification agency to make sure that the laboratory remains in good standing for those methods that the tribe is requesting.

The laboratories’ QA Manuals describe the policies and procedures for assessment and response in the laboratory. Aquatic Analysts and North Coast Laboratories, Ltd. QA Manuals are included as Appendix C.

Reports to Management

Biannually, the QVIR Environmental Director will prepare and submit a report on that quarter’s sampling activities. Contents of this report have been described previously in Section 1.9. This report will be submitted to the Tribal Council for approval. After approval, the report will be submitted to the US EPA Grants Project Officer.

Once a year a report summarizing the year’s reports will be prepared which will show any data trends that have occurred. The report will also discuss how any actions taken during the year may have affected the trends. This report will also be submitted to the Tribal Council for approval. After approval, the report will be submitted to the US EPA Grants Project Officer.

4.0 DATA REVIEW AND USABILITY

Prior to utilizing data to make project decisions, the quality of the data needs to be reviewed and evaluated to determine whether the data satisfy the project’s objectives. This process involves technical evaluation of the off-site laboratory data, as well as review of the data in conjunction with the information collected during the field sampling and field measurement activities. This latter, more qualitative review provides for a clearer understanding of the overall usability of the project’s data and potential limitations on their use. This section describes the criteria and procedures for conducting these reviews and interpreting the project’s data.

4.1 Data Review, Verification, and Validation Requirements

Setting data review, verification, and validation requirements helps to ensure that project data are evaluated in an objective and consistent manner. For the current project, such requirements have been defined for information gathered and documented as part of field sampling and field measurement activities, as well as for data generated by the off-site laboratory.

Field Sampling and Measurement Data

Any information collected during sample collection and field measurements is considered field “data.” This includes field sampling and measurement information documented in field logbooks (as listed in Section 1.9), photographs, and chain of custody forms.

Once the QVIR EPD Water Quality Technician returns to the office following a sampling  event, she turns in the field data to the QVIR EPD Data Manager who is responsible for conducting a technical review of the field data to ensure that all information is complete and any deviations from the planned methodologies are documented. For the purpose of this project, the review will be documented using the Field Activities Review Checklist provided in Appendix D1. This checklist comprehensively covers the items to be reviewed and leaves room to capture any comments associated with potential impacts on field samples and field measurement integrity based on the items listed.

Laboratory Data

For the data generated by an off-site laboratory, the laboratory is responsible for its own internal data review and verification prior to submitting the associated data results package to the QVIR EPD QA Officer. The details of the review (including checking calculations, reviewing for transcription errors, ensuring the data package is complete, etc.) are discussed in the laboratory’s QA Manual included as Appendix C. Details of the information that will be included in each data package is listed in Section 1.9 of this QA Project Plan.

Once the laboratory data are received by the Tribe, the QVIR EPD QA Officer is responsible for further review and validation of each data package. For the purpose of this project, data review and validation will be conducted using the Data Review Checklist provided in Appendix D2 in conjunction with the QC criteria (i.e., frequency, acceptance limits, and corrective actions) defined in Tables 2.5.1 through 2.5.2. This review will include evaluation of the field and laboratory duplicate results, field and laboratory blank data, matrix spike recovery data, and laboratory control sample data pertinent to each analysis. The review will also include ensuring data are reported in compliance with the project action limits and quantitation limits defined in Table 1.7.1; the sample preparation/analytical procedures were performed by the methods listed in Table 2.2.1; sample container, preservation, and holding times met the requirements listed in Table 2.2.1; the integrity of the sample (ensuring proper chain of custody and correct sample storage temperatures) is documented from sample collection through shipment and ultimate analysis, and the data packages. The Data Review Checklist comprehensively covers the review of all these items. (Note: Calibration data will not be requested for the project at this time.)

The QVIR EPD QA Officer will further evaluate each data package’s narrative report and summary tables to see whether the laboratory “flagged” any sample results based on poor or questionable data quality and to ensure that any exceedances of the laboratory’s QC criteria (as listed in Tables 2.5.1 through 2.5.2) are documented. If a problem was noted by the laboratory, the QVIR EPD QA Officer will evaluate whether the appropriate prescribed corrective action was taken by the laboratory, the action successfully resolved the problem, and the process and its resolution were accurately documented.

An effort will be made to identify whether any data quality problem is the result of laboratory issues and/or if it may be traced to some field sampling activity. If the laboratory is determined to be responsible, the QVIR EPD QA Officer will request information from the laboratory documenting that the problem has been resolved prior to submitting future samples. If some aspect of the field operation (e.g., sample collection, sample containers and/or preservation, chain-of-custody, sample shipment, paperwork, etc.) is identified as the possible problem, efforts will be made to retrain the Tribe’s field staff to minimize the potential of the problem recurring. If the problem is believed to be due to the sample matrix, the QVIR Environmental Director/QA Officer will discuss the use of alternative analytical methods with the laboratory; and, if an alternative method is available that might minimize the problem, the QA Project Plan will be modified and/or amended accordingly.

If any of the QC criteria and/or the project requirements (as discussed above) are exceeded, the associated data will be qualified as estimated and flagged with a “J”. If grossly exceeded, the associated data will be rejected and the need for re-sampling will be considered. However, since the data are being generated for a baseline assessment, it is generally felt that paying special attention to some troublesome sample collection or analytical concern during the next sampling event will be sufficient and re-sampling will not be necessary.

4.2 Verification and Validation Methods

Defining the data verification and validation methods helps to ensure that project data are evaluated in an objective and consistent manner. For the current project, such methods have been described for information gathered and documented as part of the field sampling and field measurement activities, as well as the data generated by the off-site laboratory.

Field Sampling and Measurement Data

The methods associated with verification and validation of the field sampling and measurement data are included within the discussion provided in Section 4.1.

Laboratory Data

The methods associated with verification and validation of the laboratory data are included within the discussion provided in Section 4.1.

4.3 Reconciliation with User Requirements

The purpose of the continued monitoring of the Scott River Watershed is to assess the surface water resources and determine whether analytes of concern exceed national water quality standards. This also provides the Tribe with the opportunity to begin efforts of co-management in the Scott River watershed. Data must fulfill the requirements of this QA Project Plan to be useful for the overall project. Information needed to support decision making under the surface water monitoring program is contained in this QA Project Plan, field documentation, the laboratory “data package” report, the Field Activities Review Checklist, the Laboratory Data Review Checklist, and the Field Audit Report and associated Corrective Action Report. This section describes the steps to be taken to ensure data usability (after all the data have been assembled, reviewed, verified, and validated) prior to summarizing the information in the Biannual and Annual Reports.

Once all the data from the field and laboratory have been evaluated (as described in Sections 4.1 and 4.2), the QVIR Environmental Director/QA Officer will make an overall assessment concerning the final usability of the data (and any limitations on its use) in meeting the project’s needs. The initial steps of this assessment will include, but not necessarily be limited to:

- Discussions with the QVIR EPD Water Quality Technician,

- Review of deviations from the QA Project Plan or associated SOPs to determine whether these deviations may have impacted data quality (and determining whether any impacts are widespread or single incidents, related to a few random samples or a batch of samples, and/or affecting a single or multiple analyses),

- Evaluation of the field and laboratory results and QC information,

- Review of any other external information which might influence the results, such as off-reservation activities up stream, meteorological conditions (such as storm events proceeding sampling that might contribute to high turbidity readings), and data from other sources,

- Evaluation of whether the completeness goals defined in this QA Project Plan have been met,

- Examination of any assumptions made when the study was planned, if those assumptions were met, and, if not, how the project’s conclusions are affected.

After all this information has been reviewed, the QVIR Environmental Director/QA Officer will incorporate her perspective on the critical nature of any problems noted and, ultimately, identify data usability and/or limitations in supporting project objectives and decision making. All usable data will then be compared to the Project Action Limits (as listed in Table 1.7.1) to identify whether these limits have been exceeded. Decisions made regarding exceeding the Project Action Limits will follow the “...if...then...” statements included in Section 1.7.

In addition, the QVIR Environmental Director/QA Officer will assess the effectiveness of the monitoring program and data collection at the end of each calendar year. Sampling locations, frequency, list of analytical parameters, field measurement protocols, choice of the analytical laboratory, etc. will be modified as needed to reflect the changing needs and project objectives of the Quartz Valley Indian Community. This QA Project Plan will be revised and/or amended accordingly.

5.0  References

Barbour, M.T., J. Gerritsen, B.D. Snyder, and J.B. Stribling. 1999. Rapid Bioassessment

Protocols for Use in Streams and Wadeable Rivers: Periphyton, Benthic

Macroinvertebrates and Fish, Second Edition. EPA 841-B99-002. US Environmental

Protection Agency, Office of Water, Washington, D.C.

Berg, L. 1982. The effect of short term pulses of suspended sediment on the behavior of juvenile salmonids. Pages 177-196 in G.F. Hartman et al.editors. Proceedings of the Carnation Creek Workshop: a ten year review. Department of Fisheries and Oceans, Nanaimo, B.C.

Blue Lake Rancheria. Quality Assurance Project Plan for Water Quality Sampling and Analysis.  September 2005

Blue Lake Rancheria. Water Quality Sampling and Analysis Plan.  September 2005

Brown, L.R., Moyle, P.B., and Yoshiyama R.M., 1994, Historical decline and current status of coho salmon in California: North American Journal of Fisheries Management, v. 14, no. 2, p. 237-261.

California Department of Fish and Game (CDFG), 2002, Status review of California coho

salmon north of San Francisco: Report to the California Fish and Game Commission: The

Resources Agency, Sacramento, CA, 232 p. plus appendices.

California Department of Fish and Game: Aquatic Bioassessment Laboratory.

December, 2003. California Stream Bioassessment Procedure:

(Protocol Brief for Biological and Physical/Habitat Assessment in Wadeable Streams). http://www.dfg.ca.gov/cabw/csbp_2003.pdf  11p.

Gibbs, George. 1851. Journal of Redick McKee’s Expedition through Northwestern California in 1851. Robert F. Heizer, ed. (Berkeley, Univ. of California, 1972).

Hardy, T.B., and Addley, R.C., 2001, Evaluation of interim instream flow needs in the Klamath River, phase II, draft final report: report prepared for USDI, Institute for Natural Systems Engineering, Utah Water Research Laboratory, Utah State University, Logan UT, November 2001, 304 p.

Klamath River Basin Fisheries Task Force (KRBFTF), 1991, Long range plan for the Klamath River Basin conservation area Fishery Restoration Program: prepared with assistance from Kier Associates, Yreka, CA, 403 p.

Maria, D. 2002. French Creek Watershed Advisory Group Monitoring Program – Steelhead

Monitoring Results, 1990-2002. California Dept. of Fish and Game, Yreka.

Scott River Watershed Council (SRWC), 2004, Initial phase of the Scott River Watershed

Council Strategic Action Plan, living document, pages indefinite.

Scott River Watershed Council (SRWC), 2005, Initial phase of the Scott River Watershed

Council Strategic Action Plan October 2005 Update, 249 p.

Snyder, J.O. 1931. Salmon of the Klamath River. Calif. Dept. of Fish and Game Fisheries Bulletin No. 34. 130 p.

Stuart, G. 1977. Prospecting for gold from Dogtown to Virginia City, 1852-1864. Edited by P. Phillips. Univ. of Nebraska Press, Lincoln. pp. 87-88.

U.S. Environmental Protection Agency, 1991. Office of Emergency and Remedial Response (OERR) Directive 9345.302, May.

U.S. Environmental Protection Agency, 2000. EPA Guidance for the Data Quality Objectives Process, EPA QA/G-4, EPA/600/R-96/005, August.

U.S. Environmental Protection Agency, 2001. EPA Requirements for Quality Assurance Project Plans, EPA QA/R-5, EPA/240/B-01/003, March.

U.S. Environmental Protection Agency, 2002, EPA Guidance for Quality Assurance Project Plans, EPA QA/G-5, EPA/240/R-02/009, December.

U.S. Environmental Protection Agency, 2002. EPA Guidance on Choosing a Sampling Design for Environmental Data Collection for Use in Developing a Quality Assurance Project Plan, QA/G-5sS, EPA/240/R-02/005, December.

U.S. Environmental Protection Agency, 2002. Guidance on Environmental Data Verification and Data Validation, EPA QA/G-8, EPA/240/R-02/004, November.

U.S. Environmental Protection Agency, 2006.  Guidance on Preparing a QA Project Plan.  EPA Region 9

U.S. Forest Service (USFS), 2000a, Field Guide: Explanations and Instructions for Klamath National Forest road sediment source field inventory form, Revised 2000, 13 p.

U.S. Forest Service (USFS), 2000b, Lower Scott ecosystem analysis: Klamath National Forest, Scott River Ranger District, United States Department of Agriculture, Pacific Southwest Region.

Appendix B1: Prior groundwater test results before ECORP WQ Study on QVIR

Appendix B3: CLS Lab QA Information from ECORP Water Quality Study on QVIR

Appendix B4: Complete copy of lab analytical report for ECORP testing

(See Hard Copy)

Appendix B5: Habitat Unit Characterization from ECORP Study of QVIR Feb 2004

Appendix B6: Flow Measurement Data from ECORP Water Quality Study on QVIR

| Site #1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Stream Width = |  |  |  | 42.5 | ft. |
| Cell Width |  | Cell Depth |  | Velocity | Q |
| (ft) |  | (ft) |  | (ft/s) | (cfs) |
| 13.5 | 16 | 1.00 |  | 0.14 | 0.35 |
| 16 | 18 | 1.00 |  | 0.22 | 0.44 |
| 18 | 20 | 0.80 |  | 0.72 | 1.15 |
| 20 | 22 | 0.80 |  | 0.90 | 1.44 |
| 22 | 24 | 0.75 |  | 0.87 | 1.31 |
| 24 | 26 | 0.60 |  | 1.08 | 1.30 |
| 26 | 28 | 0.70 |  | 1.22 | 1.71 |
| 28 | 30 | 0.85 |  | 0.75 | 1.28 |
| 30 | 32 | 1.10 |  | 0.64 | 1.41 |
| 32 | 34 | 1.15 |  | 0.73 | 1.68 |
| 34 | 36 | 1.35 |  | 1.03 | 2.78 |
| 36 | 38 | 1.30 |  | 0.94 | 2.44 |
| 38 | 40 | 1.40 |  | 1.56 | 4.37 |
| 40 | 42 | 1.40 |  | 1.45 | 4.06 |
| 42 | 44 | 0.90 |  | 1.83 | 3.29 |
| 44 | 46 | 1.10 |  | 1.49 | 3.28 |
| 46 | 48 | 0.95 |  | 1.47 | 2.79 |
| 48 | 50 | 0.85 |  | 1.25 | 2.13 |
| 50 | 52 | 0.60 |  | 1.35 | 1.62 |
| 52 | 54 | 0.40 |  | 0.97 | 0.78 |
| 54 | 56 | 0.15 |  | 0.00 | 0.00 |
| Total Q = |  |  | 39.6 |  |  |

Flow measurement Site #1 was located at the downstream end of Shackleford Creek, approximately 25 feet downstream of the Quartz Valley Road bridge, at the same location as surface water sample site #1: Northing 3072717.15, Easting 6294992.58.

Flow measurement Site #2 was located at the upstream end of Shackleford Creek, approximately 200 feet downstream of the QVIR property boundary, near surface water sample site #3: Northing 3072126.99, Easting 6293632.67.

| Site #2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Stream Width = |  |  |  | 27.0 | ft. |
| Cell Width |  | Cell Depth |  | Velocity | Q |
| (ft) |  | (ft) |  | (ft/s) | (cfs) |
| 3.0 | 4.5 | 0.20 |  | 0.14 | 0.04 |
| 4.5 | 6.0 | 0.45 |  | 0.15 | 0.10 |
| 6.0 | 7.5 | 0.90 |  | 0.45 | 0.61 |
| 7.5 | 9.0 | 1.20 |  | 0.70 | 1.26 |
| 9.0 | 10.5 | 1.45 |  | 1.26 | 2.74 |
| 10.5 | 12.0 | 1.40 |  | 1.60 | 3.36 |
| 12.0 | 13.5 | 1.40 |  | 1.49 | 3.13 |
| 13.5 | 15.0 | 1.30 |  | 2.80 | 5.46 |
| 15.0 | 16.5 | 1.20 |  | 2.12 | 3.82 |
| 16.5 | 18.0 | 1.60 |  | 1.94 | 4.66 |
| 18.0 | 19.5 | 1.45 |  | 1.07 | 2.33 |
| 19.5 | 21.0 | 1.40 |  | 2.15 | 4.52 |
| 21.0 | 22.5 | 1.40 |  | 1.62 | 3.40 |
| 22.5 | 24.0 | 1.20 |  | 1.22 | 2.20 |
| 24.0 | 25.5 | 1.05 |  | 1.13 | 1.78 |
| 25.5 | 27.0 | 0.85 |  | 0.88 | 1.12 |
| 27.0 | 28.5 | 0.60 |  | 0.00 | 0.00 |
| 28.5 | 30.0 | 0.20 |  | 0.00 | 0.00 |
| Total Q = |  |  | 40.5 |  |  |

Appendix C1: Sample Labels from Labs

Appendix C2: Sample Chain of Custody and Custody Seals

Appendix C3-1: Jon Lee Consulting QA Information

Jon Lee Consulting

2337 15th Street

Eureka, CA   95501

707-441-9347

jlee@humboldt1.com

BENTHIC MACROINVERTEBRATE SAMPLE PROCESSING STANDARD OPERATING PROCEDURE FOR CSBP PROTOCOL

Upon benthic macroinvertebrate sample arrival, samples are checked against the chain of custody form to make sure all samples are present and that sample preservative is adequate.  Samples are stored within the laboratory.

Subsampling:

Samples are subsampled to a 300, 500, or higher specimen count - protocol dependent.  Each sample is placed into a 500 micron sieve.  Larger debris is carefully inspected for clinging organisms, thoroughly rinsed, and returned to the original container.  The sample is placed in water for approximately five minutes and strained.  The sample is moved to a gridded pan (25 cm.2 grids) and evenly spread among the grids.  The number of grids covered by the sample is recorded.  At least five grid numbers are randomly selected.  Macroinvertebrates are systematically removed from the selected grids and placed into vials containing 70% ethanol until the targeted number of specimens have been removed.  The number removed from each grid is recorded.  All specimens are removed from the last grid processed and this number is recorded.  The number of specimens removed from each grid provide an estimate of sample relative abundance.  The processed sample debris from each grid is placed into a container labeled ôremnantö.  The remaining sample is returned to the original sample container and labeled ôoriginalö.

Taxonomic Determination and Enumeration:

Macroinvertebrates are generally determined to the genus level.  The California Stream Bioassessment Procedure Taxonomic Level 1, as outlined in the CAMLnet Short List of Taxonomic Effort, is followed.  For more precise taxonomic resolution the CSBP Level 2 (EMAP) Taxonomic Effort is followed at the request of the client.  A conservative approach is taken when specimens cannot be taken to the target taxonomic level due to damaged or very young specimens. These specimens are left at a higher taxonomic level and specimens not considered distinct taxa are labeled ônot distinctö to avoid generation of ôphantom taxaö.  Each individual taxon (genus or species) is placed into a vial containing 70% ethanol and approximately 2% glycerin.  The taxonomic name and number of specimens in each individual taxon is recorded.  Each vial is labeled with the sample date, the initials of the taxonomist, the taxonomic name, and the sample I.D.

Quality Assurance/Quality Control:

A built in check for macroinvertebrate enumeration exists when samples are processed.  If the sample final count is greater than or less than 10% of the subsample count corrective action is taken. At least 10% of the vials containing individual taxa are also randomly selected after each sample has been processed.  These vials are checked for accurate taxonomic determination and enumeration.  If the investigator is not satisfied with the taxonomic determination, or the enumeration is considered substantially inaccurate, corrective action is taken.

The California Department of Fish and Game Aquatic Bioassessment Laboratory  is contacted when problem groups of benthic macroinvertebrates are encountered.

Appendix C3-2: North Coast Laboratories QA Information

Appendix C3-3: Aquatic Analysts QA Information

Appendix D1: QVIR Field Activities Review Checklist

Sampling Location(s): 					Date(s) of Sampling: _________

Mark each topic “Yes,” “No,” or “NA” (not applicable), and comment as appropriate.

______ All required information was entered into field logbooks in ink, and logbook pages were signed & dated. Comment:

______ Deviations from SOPs , along with any pertinent verbal approval authorizations and dates, were documented in field logbooks. Comment:

______ Samples that may be affected by deviations from SOPs were flagged appropriately. Comment:

______ Field measurement calibration standards were not expired and were in the correct concentrations. Comment:

______ Field calibrations were performed and results were within QAPP-specified limits for all parameters (Temperature, pH, Dissolved Oxygen, Conductivity, and Turbidity). Comment:

______ Field measurement QC samples were within the QAPP-specified limits for all parameters. Comment:

______ Field measurement data were recorded in the appropriate logbooks(s). Comment:

______ Samples were collected at the correct sites. Comment:

______ The correct number of samples for each type of analysis and the correct volume was collected. Comment:

______ Certified clean sample containers, appropriate for the intended analysis, were used. Comment:

______ Requested/required field quality control (QC) samples (Field blanks and field duplicates) were collected, and at the correct frequency. Comment:

______ Samples were preserved with the correct chemicals, if required. Comment:

______ Samples were stored and/or shipped at the proper temperature. Comment:

______ Chain-of-custody documents were completed properly. Comment:

______ Custody seals were applied and intact when relinquishing custody of the samples. Comment:

______ Sample holding times were not exceeded during field operations. Comment:

Reviewer’s Name (print):

Reviewer’s Signature: _____________________________________________________________

Reviewer’s Title: _________________________________________________________________

Quartz Valley Indian Reservation Tribal EPD Department

Date of Review: ___/___/_____

Appendix D2: QVIR Laboratory Data Review Checklist

Sampling Project:_____________________________________________________________________

Date of Sampling:_____________________________________________________________________

Analytical Laboratory:_________________________________________________________________

Mark each topic “Yes,” “No,” or “NA” (not applicable), and comment as appropriate.

______ Final data package includes chain-of-custody forms. Comment:

______ Chain-of-custody forms were properly completed and signed by everyone involved in transporting the samples. Comment:

______ Laboratory records indicate sample custody seals were intact upon receipt. Comment:

_____ Samples arrived at the laboratory at the proper temperature. Comment:

_____ All requested analyses were performed and were documented in the analytical report. Comment:

_____ Analyses were performed according to the methods specified in the approved QA Project Plan. Comment:

_____ Holding times for extraction and analysis were not exceeded. Comment:

_____ Method detection and/or quantitation limits were included in the report. Comment:

_____ A Narrative summarizing the analyses and describing any analysis problems was included in the final report. Comment:

_____ Data qualifiers and flags were explained in the analytical report. Comment:

____ Method (laboratory) blank results were included for all analyses, at the appropriate frequency, and showed no laboratory contamination. Comment:

_____ Initial calibration data (if requested from the laboratory) were within QAPP, method, or laboratory SOP defined acceptance criteria for all analyses. Comment:

_____ Continuing calibration data (if requested from the laboratory) were within QAPP, method, or laboratory SOP defined acceptance criteria for all analyses. Comment:

_____ Matrix spike data were included for all pertinent analyses for every 20 samples. Comment:

_____ Laboratory Control Sample data were included for all analyses for every 20 samples. Comment:

_____ Laboratory Duplicate data were included for all analyses for every 20 samples. Comment:

_____ Field blanks do not contain analytes of interest or interfering compounds and included for all pertinent analyses for every 20 samples. Comment:

_____ Field Duplicates are within QAPP-defined acceptance criteria and included for all analyses for every 10 samples. Comment:

_____ Matrix spike results were listed and within QAPP or laboratory defined acceptance criteria. Comment:

_____ Matrix interferences were definitively identified either through a second analysis or use of Laboratory Control Sample Results. Comment:

_____ Laboratory Control Sample results were within QAPP or laboratory defined acceptance criteria.

Comment:

______ Laboratory Duplicate results were within QAPP or laboratory defined acceptance criteria. Comment:

______ Reported results were within method detection or quantitation limits. Comment:

Reviewer’s Name (print):

Reviewer’s Signature: __________________________________________________________________

Reviewer’s Title: ______________________________________________________________________

Quartz Valley Indian Reservation Tribal EPD Department:

Date of Data Review: __/___ /_____

| Appendix D3: QVIR Water Quality Data |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date_________________________ |  |  |  |  |  |  |  |  |  |  |  |
| Crew____________________________________________________________________ |  |  |  |  |  |  |  |  |  |  |  |
| Location | Time | Temp (°C) | Specific Conductance (mS/cm) | Conductivity (us/cm) | Dissolved Oxygen (% Saturation) | Dissolved Oxygen (mg/L) | pH | pHmV | ORP | Barometric Pressure (mmHg) | Comments |

Appendix E1: GPS manual/procedures

Appendix E2:	AquaCalc 5000 Open Channel Flow Computer (*see complete manual in e – file)

Appendix E3: HoboTemp Datalogger

Appendix E4: Van Dorn Sample Bottle Instructions

Appendix E5: YSI MultiProbe System Manual (* see complete manual in e – file)

Appendix E6: YSI 6600 EDS MultiProbe System Manual (*see complete e-file)l

Appendix E7: Model WQ770 Turbidity Meter

Appendix F1: Rapid Bioassessment Protocols (*see complete e-file)

Appendix F2: NCWAP Methods Manual (* see complete e-file)

AppendixG1: Basin Plan MCL Tables

Appendix G2 EPA Surface Water Quality Standards (*see complete e-file)

b

b

b

c

c

All analyses will be performed at an off-site laboratory. There will be no field screening analyses. Field measurements will be performed at each sample collection location.

Samples will be collected at depth of 6-12 inches. If depth of water is less than 12 inches, sample will be collected at mid depth and noted in the field logbook.

Field duplicates will be collected at a frequency of 10% of the samples collected for laboratory analysis. Field duplicates will be collected at a frequency of 10% or one per day, whichever is more frequent, for samples collected for field measurements.

Include number of associated analytical QC samples if collection of additional sample volume and/or bottles is necessary. If the QC samples listed are part of the analysis but no additional sample volume and/or bottles are needed, include “NAS” (for “no additional sample”) in the column. (Note: MS=matrix spike, MSD=matrix spike duplicate, dup=laboratory duplicate/replicate.)

Field blanks will be collected at a frequency of 10% of the of samples collected, or one per day, whichever is less frequent. Field blanks will not be collected, as they were determined not to be critical, to support laboratory analysis of Total Dissolved Solids, alkalinity, total coliform, e. coli or for field measurements.

Temperature blank samples will be submitted with each cooler of samples. These samples were not included in the sample count, as they are not carried though the analyses.

6

6

6

6

6

ba Maximum weekly maximum temperature (MWMT) or “7-day maximum” threshold for coho salmon from Welsh et al. (2001).

b Scott River Water Quality Objectives from the North Coast Regional Water Quality Control Board’s Water Quality Control Plan (2001)

c Action levels adopted from U.S. EPA Ambient Water Quality Criteria Recommendations Rivers and Streams in Nutrient Ecoregion II (U.S. EPA 2000).

b

d

1

21 Include number of associated analytical QC samples if collection of additional sample volume and/or bottles is necessary. If the QC samples listed are part of the analysis but no additional sample volume and/or bottles are needed, include “NAS” (for “no additional sample”) in the column. (Note: MS=matrix spike, MSD=matrix spike duplicate, dup=laboratory duplicate/replicate.)

Methods are provided in Appendix A-2.

Data Quality Indicators may be related to sampling (S) and/or analysis (A) activities.

For field duplicate samples, there are no method-specific QC acceptance limits. (NA - Not applicable.)

The information in this column supports acceptance criteria/measurement performance criteria introduced in Section 1.7 For this study, the field measurement’s QC acceptance limits (as determined from a calibration check sample analyzed half-way through the field day) were reviewed and found acceptable to meet the current data quality needs. As such, the field measurement’s QC acceptance limits and the project’s measurement performance criteria are equivalent.

Accuracy is not ensured through the analysis of a QC check. If the temperature sensor meets the annual calibration procedures and criteria presented in Table XX, the measurements are considered accurate enough to meet the needs of the current project.

Accuracy is ensured through the calibration and calibration check process presented in Table XX. The post calibration check sample(s) will be considered as QC check samples for the field measurements.

ALL SAMPLES ARE SURFACE WATER MATRIX. ALL SAMPLES ARE COLLECTED BY THE SAME PROCEDURE, AS PRESENTED IN APPENDIX XX. NO ADDITIONAL QC CHECKS ARE PLANNED BEYOND THOSE IDENTIFIED ABOVE FOR ACCURACY AND PRECISION.