2018 California State Water Quality Control Board

Integrated Report Data Submittal Information

Redwood Creek Stream and Air Temperature Data

4-27-2017

Introduction

Redwood National Park (RNP) has monitored continuous water and air temperature on Redwood Creek and several tributaries since 1997. The monitoring program was established in collaboration with the U.S. Geological Survey (USGS) with the objective to provide long-term stream temperature monitoring on the mainstem of Redwood Creek and tributaries to detect trends in stream temperature both spatially and temporally, and to attempt to explain any trends detected by changing watershed conditions. Temperature was recorded at one hour intervals from June 1 to September 30.

Monitoring Sites

Five long-term water temperature monitoring sites are established along the mainstem of Redwood Creek which includes the Redwood Creek estuary (Figure 1) and selected tributaries (Table 1). Air temperature was also recorded at three mainstem and six tributary stations. Stream temperature monitoring sites were selected based upon representative channel reaches and available stream access. The location of the long-term Redwood Creek mainstem monitoring sites were validated using thermal infrared data (TIR) of surface water temperatures collected along the main channel of Redwood Creek in 2003 (Madej, et. al. 2006) and are representative of the thermal conditions and reaches on Redwood Creek.

Data Submitted

Data submitted for the integrated report includes statistics for water and air temperature from 2010 to 2016 collected by Redwood National Park in the Redwood Creek watershed. Methodology and analysis is described in the submitted RNSP Quality Assurance Project Plan. The analysis period extended from July 1 to August 30. The highest water temperatures occur on the mainstem in late July. All information is stored in the Temperature Data_Submitted folder.

- Stream Temperature Site Names: Stream temperature naming convention is described in the document “RNSP Stream Temperature File Naming Conventions” (filename: REDW_StrTemp_StationNames.docx). This document outlines our file naming format, station prefix codes, brief location description and brief monitoring notes.

- Stream Temperature Monitoring Locations:

The file REDW_TempSiteCoordinatesLocations.xlsx contains the site code, general location description, Latitude/Longitude, and UTM NAD83 coordinates for RNP stream monitoring sites.

Figure 1. Location of active stream temperature monitoring sites on Redwood Creek. Long-term mainstem monitoring sites are at the Redwood Creek estuary (REST), lower Redwood Creek (RLOW), above the Tall Trees Grove (RTTG), above Lacks Creek in Redwood Valley (RV), and at the USGS O’Kane or Blue Lake gaging station (ROKN). Tributary gaging stations include: Prairie Creek at Wolf Creek bridge (PRW), Little Lost Man Creek (LLM), Lost Man Creek (LMC), Lost Man Lower and Upper Middle Fork (LMLM and LMUM respectively), Lost Man South Fork (LMSF), Tom McDonald Creek (TMC), Emerald Creek (EMR), Bridge Creek (BRI), Lacks Creek (LAC), Minor Creek (MIN). Godwood, May, Streelow and Skunk Cabbage Creek locations are not plotted.

| Table 1. Temperature monitoring sites on Redwood Creek and selected tributaries and period of record. |  |  |  |
| --- | --- | --- | --- |
| Redwood Creek Mainstem |  |  |  |
| Location (Site Code) | Medium | Period of Record |  |
| Estuary (REST) | Water | 1997-Present |  |
| Lower Redwood Creek (RLOW) | Water and Air | 1997-Present |  |
| Upstream Tall Trees Grove (RTTG) | Water | 1997-Present |  |
| Redwood Valley upstream Lacks Creek (RV) | Water and Air | 1997-Present |  |
| O'Kane (USGS Gaging Station) (ROKN) | Water and Air | 1997-Present |  |
| Redwood Creek upstream Minon Creek (RMIN) | Water | 1997-2003 |  |
| Redwood Creek Tributaries |  |  |  |
| Location (Site Code) | Medium | Period of Record |  |
| Tom McDonald Creek (TMC) | Water and Air | 2001-Present |  |
| Bridge Creek (BRI) | Water | 1995-Present |  |
| Lacks Creek (LAC) | Water | 1997-Present |  |
| Minor Creek (MIN) | Water | 1997-Present |  |
| Prairie Creek at Wolf Creek Bridge (PRW) | Water and Air | 1997- Present |  |
| Prairie Creek and Tributaries |  |  |  |
| Little Lost Man Creek (LLM) | Water and Air | 2003-Present |  |
| Lost Man Creek- Hatchery Ladder (LMC) | Water | 2000-Present |  |
| Larry Dam Creek (LDM) | Water | 2000-2002; 2008-Present |  |
| Lost Man Creek South Fork (LMSF) | Water and Air | 2013-Present |  |
| Lost Man Creek Lower Middle Fork (LMLM) | Water and Air | 2013-Present |  |
| Lost Man Creek Upper Middle Fork (LMUM) | Water and Air | 2013-Present |  |
| Godwood Creek (GOD) | Water | 2015-2016 |  |
| May Creek (MAY) | Water | 2015-2016 |  |
| Streelow Creek (STR) | Water | 2015-2016 |  |
| Skunk Cabbage Creek (SKC) | Water | 2015-2016 |  |

- Temperature Probe Summary Reports:  Temperature summary reports are included for each monitoring site. Reports are grouped by water year under the subfolder:

\Temperature Data_Submitted\Annual Summary Reports

The temperature summary monitoring reports contain detailed water and air information, statistics, and temperature plots for each site. The filename structure is

xxxxyym#_sn_Summary.xlsx

where: xxxx = station code, yy = year, m = w (water) or a (air), “#” = probe number when a pair of probes are deployed, sn = serial number of the probe

Main tabs in the Excel spreadsheet include:

- Stat Summary- Summary statistics from July 1 to August 30.

  - outliers (change in stream temp >3C)

  - minimum temperature (C), date of occurrence, total occurrences

  - maximum temperature (C), date of occurrence, total occurrences

  - average temperature (C)

  - maximum diurnal temperature range (C), date of occurrence, total occurrences

  - minimum diurnal temperature range (C), date of occurrence, total occurrences

  - sample interval

  - MWAT(C), date of occurrence, total occurrences

  - MWMT(C), date of occurrence, total occurrences

  - Temperature plot from July 1-August 30.

- Daily Stats

  - minimum

  - maximum

  - mean, range (diurnal change)

  - 18C Points Above (number of hours above 18C)

  - 18C Dur. Above (number of hours above 18C)

  - 15C Points Above (number of hours above 15C)

  - 15C Dur. Above (number of hours above 15C)

- Plots–Three plots from July 1 to August 30 which include: 1) daily maximum, mean and minimum temperatures, 2) diurnal range, and 3) MWMT and MWAT

- MWMT–7-day moving average values for the maximum weekly maximum stream temperature.

- MWAT–7-day moving average values for the maximum weekly average stream temperature.

- Import_Data (ignore)

- Import_ImportNewParameterColumn (ignore)

- Redwood Creek Stream Temperature Statistics

Temperature statistics for all water and air stations are compiled and summarized in the file:

\Temperature Data_Submitted\RNP_StreamTempSummaryStats-2010-2016.xlsx

This file contains:

- Station Code

- Station Name

- Water Year

- Medium (air or water)

- Analysis start date

- Analysis end date

- The remaining columns are the same what is listed in the Stats Summary with the exception of the “outliers” column

Maximum Weekly Maximum Temperature on Redwood Creek

Maximum Weekly Maximum Temperatures (MWMT) 1998 to 2016 for Redwood Creek and selected tributaries are plotted in the Figure 2. Coho (Oncorhynchus kitsuch) thermal tolerance thresholds (presence/absence temperatures) on the graph are from field observations and studies in the nearby Mattole River by Welsh et al. (2001).  Welsh et al. (2001) suggests that streams with MWMT greater than 18.1°C may preclude the presence of juvenile coho salmon. Since Redwood Creek and the Mattole River are grouped into the same Evolutionary Significant Unit, it is likely that the juvenile salmonids would respond to temperatures similarly. Madej et al. (2006) concluded that high water temperatures in the middle basin likely restrict juvenile coho rearing during the summer. She based her conclusions these tolerance thresholds, stream temperature data and the low number of juvenile coho salmon observed on Redwood Creek.

Since 1999, Redwood Creek MWMT temperatures have exceeded the upper 18°C MWMT threshold and indicate that it is unlikely that juvenile coho will rear in mainstem. In Redwood Creek tributaries, MWMTs for Prairie Creek and Tom McDonald indicate temperatures are fully suitable for coho rearing. While Bridge Creek may provide some coho rearing, Lacks and Minor Creek exceed the temperatures where coho would be expected to be present.

A

B

Figure 2. Plots of the Maximum Weekly Maximum Temperature for Redwood Creek (A) and selected tributaries (B).

Redwood Creek Juvenile Fish Kills

Multiple fish kills of juvenile salmonids have been observed in Redwood Creek by California Department of Fish and Wildlife (CDFW) biologists due to high stream temperatures during late July (Sparkman and Holt 2017). Sparkman (2007) documented the first observed fish kill on Redwood Creek during the summer of 2006.

“For the first time in seven consecutive trapping years, we observed numerous dead juvenile salmonids at the trapping site and areas nearby the trapping site from July 23rd –July 25th. Numerous residents in Redwood Valley both above and below the trap site also witnessed dead fish floating by in the stream, or on the stream bottom. Although most of the fish killed were 0+ steelhead trout, several older juvenile steelhead trout age classes and various amphibians were also killed (pacific giant salamander and yellow legged frog)”

He concluded that the dissolved oxygen concentrations in the stream were favorable to juvenile steelhead survival and that lethal stream temperatures during the late afternoon were most likely responsible for the juvenile fish kill.

Juvenile salmonid fish kills have occurred in late July in 2006, 2009, 2014 and a minor fish kill in 2015 (Sparkman and Holt 2017). Summer stream temperatures currently don’t support rearing juvenile coho. The recovery of Redwood Creek to a more suitable stream temperature for summer juvenile salmonid rearing will require the reestablishment of large riparian trees to shade the stream channel and will likely take decades.

Literature Cited:

Madej, M.A., Currens, C., Ozaki, V., Yee, J., and D.Anderson. 2006. Assessing possible thermal rearing restrictions for juvenile coho salmon (Onchorhynchus kisutch) through thermal infrared imaging and in-stream monitoring, Redwood Creek, California. Can. J. fish. Aquat. Sci. 63: 1384-2696.

Sparkman, M. D.  2007.  Upper Redwood Creek juvenile salmonid (smolt) downstream migration study year 2006, CDFG AFRAMP Project 2a5. California Department of Fish and Wildlife, Arcata, California.

Sparkman M.D. and S.C. Holt. 2017. Mid-upper Redwood Creek juvenile salmonid (smolt) abundance project study year 2016. CDFG AFRAMP Project 2a5. California Department of Fish and Wildlife, Arcata, California.

Welsh, H.H., Jr., Hodgson, G.R., Harvey, B.C., and Roche, M.E.  2001. Distribution of juvenile coho salmon in relation to water temperatures in tributaries of the Mattole River, California. N. Am. J. Fish Manag. 21: 464–470.

For questions regarding the temperature data contact:

Vicki Ozaki, Geologist, Redwood National and State Parks

Phone: 707 825-5142, e-mail:  vicki_ozaki@nps.gov