2018 California State Water Quality Control Board

Integrated Report Data Submittal Information

Redwood Creek Stream Discharges and Sediment Loads

4-27-2016

### Introduction

Redwood National Park (RNP) operates a hydrologic monitoring program that collects stage, discharge and suspended sediment data at a network of stream gaging stations within the Redwood Creek basin. As stewards of some of the last remaining pristine redwood watersheds, a unique and important function of RNP’s hydrologic monitoring program is to collect and disseminate data that characterize ‘baseline’ hydrologic conditions in the redwood ecosystem. This is critical for understanding the physical conditions under which dependent aquatic species evolved before major human disturbances began and for providing a basis for comparison against managed watersheds to evaluate land use effects. In addition to ‘in-house’ uses, RNP data provides essential support for researchers from both the educational and governmental sectors, to regulators tasked with protecting water quality (e.g. California State and Regional Water Quality Control Boards, EPA), and to private interests.

### Data Submitted

The data included with this section of RNP’s data submittal are listed below (some of the hydrologic data collected in Redwood Creek are published by the US Geological Survey and are also readily available through their website (http://water.usgs.gov/):

- Annual maximum peak water discharge

- Annual suspended sediment load

Two mainstem gaging stations are located on Redwood Creek (Figure 1)Redwood Creek at Orick (#11282500; referred to as ORK or Orick) and Redwood Creek near Blue Lake (#11481500; referred to as OKN or Blue Lake). The drainage area of the gaging stations are 277 mi2; drainage area 67.7 mi2 respectively.

The USGS operates the Orick gaging station as a long-term stream monitoring station. The park contracts with the USGS to sample suspended sediment samples 2-3 times per year during high flows for both the Orick and Blue Lake gage. Operation of the Blue Lake gage is cooperatively funded by RNP, USGS, and California Department of Water Resources.  The years of operation and data collected are listed for Redwood Creek at Orick in Table 1 and Redwood Creek at Blue Lake in Table 2.

The USGS computed annual sediment loads for the entire water year (October-September) for both stations until 1981. They switched to sampling and estimating loads for the storm season only (October-April) in 1982 and subsequent years. These years with partial data underestimate the total sediment yield for the year. However, most sediment transport occurs during the high winter storm flows. After WY1992, RNP began computing annual suspended sediment loads, using a custom program (HYDRO) through the 1990s and later using the USGS’ “Graphical Constituent Loading Analysis System” software (GCLAS). These two programs operate using essentially the same input data (15-minute discharge data for the full water year and sampled suspended sediment concentrations) and algorithms. A comparison of sediment yield computed from the HYDRO and GCLAS programs show that the computed yields are comparable. The average difference between estimates was 9 percent and the maximum difference in one year was 25 percent. RNP recalculated the annual suspended sediment transport for years where the HYDRO program was used at ORK (WY1993-2000) and for 1996 and 1997 at OKN. This provides more consistency with the USGS analysis for other years. The HYDRO program only operated in Windows 95 and is no longer functional.

In Table 3, annual peak flow and suspended sediment yields are listed for the Orick and Blue Lake gaging stations. Suspended sediment yield data in Table 3 supersedes data provided in the past by Redwood National Park.

Figure 1. Location of gaging stations on Redwood Creek.

Table 1. Years of operation and data collected for discharge, suspended sediment and bedload sampling on Redwood Creek at Orick gaging station.

| Redwood Creek at Orick (#11482500) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Discharge |  | Suspended Sediment |  |  | Bedload |  |  | Sediment Load Analysis |  |  |
| Water Years | Agency | Water Years | Months Sampled | Agency | Water Years | Agency | Months Sampled | Water Years | Sediment Yield* | Agency |
| 1911-1912 | USGS | 1954-1970 | Oct-Sept | RNP | 1974-1981 | USGS | Oct-Sept | 1954-1970 | QSS & QBD** | RNP |
| 1913-1952 | Not in operation | 1971-1981 | Oct-Sept | USGS | 1982-1992 | USGS | Oct-April | 1971-1973 | QSS | USGS |
| 1953-2016 | USGS | 1982-1992 | Oct-Apr | USGS |  |  |  | 1974-1992 | QSS & QBD | USGS |
|  |  | 1993-2000 | Oct-Sept | RNP |  |  |  | 1993-2000 | QSS | RNP |
|  |  | 2001 | Oct-Apr | USGS |  |  |  | 2001 | QSS | USGS |
|  |  | 2002-2016 | Oct-Sept | RNP |  |  |  | 2002-2016 | QSS | RNP |
| *QSS is the suspended sediment and QBD is the bedload **Computed by RNP from a sediment discharge relationship and flow duration table data which was provided by the USGS from John R. Crippen letter to RNP 1981 |  |  |  |  |  |  |  |  |  |  |

Table 2. Years of operation and data collected for discharge, suspended sediment and bedload sampling on Redwood Creek near Blue Lake gaging station.

| Redwood Creek near Blue Lake (#11481500) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Discharge |  | Suspended Sediment |  |  | Bedload |  |  | Sediment Load Analysis |  |  |
| Water Years | Agency | Water Years | Months Sampled | Agency | Water Years | Agency | Months Sampled | Water Years | Sediment Yield** | Agency |
| 1954-1958 | USGS | 1973-1981 | Oct-Sept | USGS | 1974-1981 | USGS | Oct-Sept | 1973 | QSS | USGS |
| 1959-1972 | Not in Operation | 1982-1992 | Oct-Apr | RNP | 1982-1992 | USGS | Oct-April | 1974-1992 | QSS & QBD | USGS |
| 1973-1993 | USGS | 1993-1995* | Oct-Sept | RNP |  |  |  | 1993-1995* | QSS | RNP |
| 1994-1997 | RNP | 1996-1997 | Oct-Sept | RNP |  |  |  | 1996-1997 | QSS | RNP |
| 1998-2015 | USGS | 1998-2000* | Oct-Sept | USGS |  |  |  | 1998-2000* | QSS | RNP |
|  |  | 2001-2002 | Oct-Apr | RNP |  |  |  | 2001-2002 | QSS | USGS |
|  |  | 2003-2015 | Oct-Sept |  |  |  |  | 2003-2015 | QSS | RNP |
| *Suspended sediment data has not been reviewed and is provisional. **QSS is the suspended sediment and QBD is the bedload |  |  |  |  |  |  |  |  |  |  |

Table 3. Peak flow and annual suspended load by water year for Redwood Creek at Orick and Blue Lake gaging stations.

|  | Orick Gaging Station |  | Blue Lake (O'Kane) Gaging Station |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Peak Flow | Annual Suspended Sediment Load | Peak Flow | Annual Suspended Sediment Load |  |
| Water Year | cfs | tons/mi2 | cfs | tons/mi2 |  |
| 1973 | 10000 | 2736 | 2980 | 2733 |  |
| 1974 | 24800 | 8051 | 5720 | 9719 |  |
| 1975 | 50200 | 10000 | 12200 | 10266 |  |
| 1976 | 12100 | 2690 | 4100 | 1199 |  |
| 1977 | 3310 | 82 | 827 | 18 |  |
| 1978 | 21200 | 3426 | 3490 | 1802 |  |
| 1979 | 14100 | 1058 | 4760 | 802 |  |
| 1980 | 19400 | 2545 | 4840 | 2408 |  |
| 1981 | 9030 | 679 | 3410 | 693 |  |
| 1982* | 26500 | 4621 | 5730 | 3767 |  |
| 1983* | 29500 | 4801 | 6750 | 5524 |  |
| 1984* | 17900 | 2260 | 3950 | 2570 |  |
| 1985* | 11400 | 1014 | 4390 | 1202 |  |
| 1986* | 30700 | 3646 | 6470 | 3545 |  |
| 1987* | 5870 | 375 | 2020 | 182 |  |
| 1988* | 15200 | 567 | 3900 | 425 |  |
| 1989* | 21400 | 1675 | 5980 | 1325 |  |
| 1990* | 18100 | 690 | 5980 | 486 |  |
| 1991* | 5280 | 126 | 1430 | 75 |  |
| 1992* | 5000 | 66 | 1370 | 43 |  |
| 1993 | 11800 | 1307 | 5340 | 1654 |  |
| 1994 | 7890 | 243 | 2209 | 186 |  |
| 1995 | 18600 | 2220 | 6205 | 3840 |  |
| 1996 | 31800 | 3682 | 8884 | 5007 |  |
| 1997 | 40300 | 4188 | 7052 | 4225 |  |
| 1998 | 19900 | 2838 | 7000 | 3900 |  |
| 1999 | 31000 | 2740 | 4469 | 1227 |  |
| 2000 | 19900 | 942 | 4390 | 619 |  |
| 2001 | 2320 | 20 | 1020 | 24 |  |
| 2002 | 16500 | 841 | 5200 | 678 |  |
| 2003 | 23700 | 2343 | 6300 | 1297 |  |
| 2004 | 17300 | 1040 | 4440 | 730 |  |
| 2005 | 15300 | 610 | 6350 | 464 |  |
| 2006 | 30100 | 2993 | 9550 | 3235 |  |
| 2007 | 11100 | 888 | 3030 | 334 |  |
| 2008 | 12400 | 625 | 4970 | 465 |  |
| 2009 | 17500 | 480 | 2600 | 291 |  |
| 2010 | 8830 | 277 | 3340 | 285 |  |
| 2011 | 15100 | 599 | 5440 | 526 |  |
| 2012 | 14500 | 661 | 3370 | 572 |  |
| 2013 | 20200 | 437 | 6420 | 387 |  |
| 2014 | 8960 | 150 | 2320 | 89 |  |
| 2015 | 13300 | 328 | 3480 | 394 |  |
| 2016 | 18800 | 1105 | 6680 | a |  |
| *1982-1992 Qss = October through April data only a SSC Not yet analyzed |  |  |  |  |  |

The EPA’s sediment TMDL target was established at 1900 tons/square mile/year for Redwood Creek at Orick. (Redwood Creek TMDL, 1998) and represents a 40 percent decrease in the total sediment loading capacity. This target is based on the total load for Redwood Creek at Orick (suspended and bedload yield). Unfortunately bedload sampling was discontinued since 1992 and only suspended sediment has been monitored since then.

Since no bedload data exists to compute total sediment load for Redwood Creek, we computed the suspended sediment TMDL threshold using the methodology from Section 3.3 Source Analysis in the 1998 Total Maximum Daily Load for Sediment, Redwood Creek, California. The table below follows the 1998 TMDL report Table 6. Summaries of annual average sediment loading per square mile at Orick (TMDL, page 42). Based on our calculations the TMDL suspended sediment threshold is 1640 (tons/mi2/year) and represents 40 percent of the annual suspended sediment load for 1954-1997.

Table 4. Summaries of Annual Average Suspended Sediment Loading Per Square Mile at Orick.

| Time period (Water Years) | Number of Years | Annual Suspeneded Sediment Load (tons) | Annual Average Suspeneded Sediment Load (tons/mi2/year) | Source |
| --- | --- | --- | --- | --- |
| 1954-1980 | 27 | 39960000 | 5343 | Redwood Creek Watershed Analysis, Appendix C. 1954-1970 based on Crippen, 1981; 1971-1980 based on USGS publications |
| 1981-1997 | 17 | 8908400 | 1892 | Calculated from Watershed Analysis and RNP data (*1982-1992 Oct-Apr only) |
| 1954-1997 | 44 | 176420 | 4010 | Calculated from Watershed Analysis and RNP data =[(5343*27) + (1982*17)] 44 |
|  |  | Suspended Sediment TMDL Threshold | 1604 | Suspended sediment loading capacity (tons/mi2/year) = 40% of annual suspended sediment load for 1954-1997 |

The 10-year rolling average suspended sediment load for Redwood Creek at Orick are plotted in Figure 2 with the computed suspended sediment TMDL target. Sediment yields in the Redwood Creek basin have been steadily decreasing since the 1997 storm which was the highest peak flow since 1976 (Figure 3). The moderate flows from 1996-1997 were the first high flows since the 1975 flood, transported more total sediment than the previous decade (Madej et.al. 2006) is reflected in the 10-year rolling average. Madej et.al. (2006) found that trends in the 10-year rolling average are not necessarily indicative of a change in sediment transport but more dependent on magnitude of flows.

Figure 2. Plot of the suspended sediment yield 10-year rolling average at Orick. The black dashed line is the computed suspended sediment TMDL target and the black bars are rolling averages that include the 2010-2016 sediment yield data.

The suspended sediment yield per unit at Redwood Creek gaging stations shows that from 1974-1975 sediment yields were significantly higher (Figure 4). The moderate flows in the mid-1990s show elevated suspended sediment yield for many years. While recently suspended sediment yields have been low, continued monitoring especially during the next large flood is important to help determine if the trends we see presently are an artifact of the flow regime or truly a decrease in suspended sediment.

Figure 3. Redwood Creek annual peak flow at Orick from 1948 to 2016.

Figure 4. The sediment yield per unit.at Redwood Creek gaging stations shows low sediment yields on Redwood Creek from 2007-2015.

Literature Cited

Madej, M.A., Klein, R.K., Ozaki, V. and T.Marquette. 2006. Analyzing sediment yields in the context of TMDLs. In: Proceedings of the Eighth Federal Interagency Sedimentation Conference. April 2-6, 2006. Reno Nevada.

U.S. Environmental Protection Agency ( EPA). 1998. Total maximum daily load sediment Redwood Creek, California. U.S. Environmental Protection Agency Region 9. December 30, 1998.

For questions regarding the sediment data contact:

Vicki Ozaki, Geologist, Redwood National and State Parks

Phone: 707 825-5142, e-mail:  vicki_ozaki@nps.gov