From: 	"Michael Crane" <MCRANE@ci.watsonville.ca.us>

To:	"Mary Adams" <madams@waterboards.ca.gov>

Date: 	5/21/2007 10:01:22 AM

Subject: 	Re: Fwd: Pinto Lake data question

Hi Mary,

Here's the method / probe information you requested. Let me know if there is anything else you need.

Thanks, Mike

Temp. -- Standard Methotds 18th Ed. (S.M.) 2550 B, Hach 44600 Cond./TDS meter

pH -- S.M. 4500-H+, Hach HQ20 Meter with combination electrode

DO -- Hach HQ20 Meter with 45' cord-length LDO optical probe (in situ measurements)

Cond. -- S.M. 2510 B, Hach 44600 Cond./TDS meter

Turb. -- S.M. 2130 B, Hach 2100P Turbidimeter

Chl-a -- EPA 446.0 (Did not do validation with CRM's)

NH3 -- S.M. 4500-NH3 F (ion select electrode)

NO2--N  -- EPA 300.0

NO3--N -- EPA 300.0

Cl- -- EPA 300.0

SO42- -- EPA 300.0

PO43--P -- EPA 300.0

All 300.0 samples were filtered prior to analysis by ion chromatography.

Mike Crane

Water Quality Chemist

City of Watsonville

Phone: (831) 768-3180

Fax: (831) 763-1970

>>> Robert Ketley 5/21/2007 7:44 AM >>>

Mike:

Can you address this question?

Rob

>>> "Mary Adams" <madams@waterboards.ca.gov> 5/18/2007 1:14 PM >>>

Hi Rob, Could you provide the methods (or type of probe) used for the following analyses?  I would like to document that in the database.

Thanks so much

Mary :)

Temp.

pH

DO

Cond.

Turb.

Chl-a

NH3

NO2--N

NO3--N

Cl-

SO42-

PO43--P

CC:	"Robert Ketley" <RKETLEY@ci.watsonville.ca.us>