From: 	Alison Gee <alison@elkhornslough.org>

To:	Mary Adams <madams@waterboards.ca.gov>

Date: 	2/5/2008 12:27:19 PM

Subject: 	Re: Data questions (data pre 2005)

Hey Mary,

i was able to check in with John about this and here are the cut offs he

and i feel are reasonable:

Dissolved Oxygen (mg/l)  reject anything above 35, (probe range is 0-50

mg/L)

Dissolved Oxygen (% sat) reject anything above 350, (probe range is 0-500%)

pH accept full pH range, but we haven't seen anything less than 6 or

above 11 (probe range 0-14)

Conductivity reject anything above 100 mS/cm, (probe range is 0-100 mS/cm)

Salinity reject anything above 70 ppt, (probe range is 0-70 ppt)

Turbidity reject anything above 1000 NTU, (range 0-1000 NTU)

Temperature reject anything above 35 degrees C, (range is -5 to 50 C)

Chlorophyll a reject anything above 200 ug/L, (probe range is 0-400 ug/L)

i hope this helps.  are you using the Chl a data for impairment

assessment?  i beleive this was an add on parameter for us.  this

parameter is probably the one we have the least confidence in and John

was concerned about using it for evaluation.

hope you are well and enjoying the sunshine!

-Alison.

Alison Gee wrote:

> Hey Mary,

> i am sorry i have not responded to you sooner about this.  i was

> wanting to check in with John Haskins about this and he has not yet

> responded to my email nor have i seen him this week.  I know he will

> be around next week as we have field work to do though.

>

> here is what i know for sure:

> let's use the following cut offs for nutrient detection limits for the

> volunteer ESNERR data:

> Ammonia 0.05 mg/L

> Nitrate 0.07 ( 0.015 or 0.02 mg/L as N)

> Orther-Phos 0.03 mg/L

> these are the current detection limits from the County lab.  we have

> significantly better DLs from MLML and UCSC currently.  but the County

> lab DLs haven't changed much and they did most of the historical

> nutrient analyses.

>

> for the physico-chemical parameters, I know what the current

> sensitivities are, but want to check in with John about the historical

> data as we used different instruments and he was around back then. i

> did note that the sensitivities are listed in our QAPP for nutrients

> but not for the field data. i am sending you those two tables from our

> QAPP attached.

>

> So, i will try to get with John first thing Monday about the probe

> sensitivities for the historical data and hopefully have an answer for

> you then!

> Have a great weekend!

> Kind regards,

> -Alison.

>

> Mary Adams wrote:

>> Hi Alison, Thank you so much for your time and dedication to the

>> data!  Much Appreciated!!!!

>>

>> The following is a list of questions I still have for you folks about

>> the data.  If you could send me a message back that

>> 1) Detection Limits for lab analyses prior to 2005?  Could you

>> provide a typical DL for each analyte so tat I can assume everything

>> less than that value is a Non-detect?

>>

>> Ammonia as N (our lab is getting as low as 0.02)

>> Nitrate (our lab typically reports 0.01)

>> Ortho-Phos (our lab typically report 0.02)

>>

>>

>> 2) Detection limits for the probe parameters

>> Dissolved Oxygen (mg/l and % sat)

>> pH

>> Conductivity

>> Salinity

>> Turbidity

>> Temperature

>> Chlorophyll a

>>

>>

>> 3) There are several probe measurements that - at a glance- look like

>> outliers or typos.  Could you either verify that these are true

>> numbers  we should accept or simply provide a cut off value fore each

>> parameter.  For example there are DO % sat number as high as 314% .

>> Is this a number you believe is true or should it be flagged and

>> rejected?  Should I flag and reject all number over 300, over 200 etc.

>>

>> Any guidance you could provide to me  would help me to better use the

>> data.  Im attaching an Excel file with a bunch of probe data that

>> looks like outliers to me.

>>  I arbitrarily chose cut offs like chlorophyll a > 200, green water

>> is usually 25ug/L or higher Conductivity > 100mS/cm (isn't ocean

>> water typically 55 uS/cm?)

>> DO sat < 10% and > 200% (just because I would question anything my

>> probe said in those ranges)

>> Salinity > 70 ppt (ocean water is about 35 ppt and the Salton Sea is

>> less than 50 ppt)

>> Turbidity > 1000 NTU - does the probe measure > 1000?

>>

>>

>> Anyway - take a look at the attached - provide comments there if you

>> like or just use it to define "cut off points" for the range of

>> acceptable data.  Thanks so much and  call me if you like - Im on my

>> cell all next week

>>

>> Thanks :)

>>

>>

>>

>>

>>

>> Mary S. Adams

>> Central Coast Ambient Monitoring Program (CCAMP)

>> Regional Water Quality Control Board

>> 895 Aerovista Place, Suite 101

>> San Luis Obispo, CA 93401

>> (805) 542-4768

>> Fax    788-3502

>> madams@waterboards.ca.gov

>> http://www.ccamp.org

>>

>> If you have a comment you would like to share regarding the customer

>> service of any state employee please visit the following site:

>> http://www.calepa.ca.gov/customer/CSForm.asp

>>

>> * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

>> *  Please note my new email address as of November 5th.

>> madams@waterboards.ca.gov

>> * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

>>