| Stream & site # | Description/Directions | GIS info | Sampled |
| --- | --- | --- | --- |
| Malibu Cr. MC#1 | Impacts upstream: 105 sq miles of drainage 6 man made lakes Tapia Wastewater treatment plant and compost facility, landfill, sheep and cow grazing, horses, septic systems 3000 + urban runoff from Agoura Hills, Westlake Village, part of Calabasas and the 101 fwy. all drain here. From the Malibu Lagoon Parking Lot turn right onto PCH immediately get in the left lane and turn left at Serra Rd (first available left) go past the guard shack until you get to Mariposa De Oro and turn left. At the creek, turn right and follow the road to the end. Park at the State park sign. Follow the trail to the creek take the first fork. Reach is ~Walk downstream until the riffle sequence turns into a glide 50 yards downstream begin sampling within the 5 riffles. | Elevation: 32.79 ft. Northing:3768379.350 Easting:344201.287. Stream order 5 | Sp 00 F 00 |
| Malibu Cr. MC#9 | Downstream of all Tapia Discharges. Impacts upstream: 6 man made lakes Tapia Wastewater treatment plant and compost facility, landfill, sheep and cow grazing, horses, septic systems 3000+, urban runoff from Agoura Hills, Westlake Village, part of Calabasas and the 101 fwy. Exit Tapia Park turn right on Malibu Canyon Rd. .5 miles on your left open gate at LA County stream gauge drive to bottom. Walk upstream between willows. ## Reach begins at riffle just above the gauge 5 riffles. | Elevation: 431.32ft. Northing: 3771973.345 Easting: 343102.468. Reach length: 388 ft. Stream order5 | Sp 00 F 00 |
| Malibu Cr. MC#8b | Riffle below grated discharge 001 Impacts upstream: 6 man made lakes landfill, sheep and cow grazing, horses, septic systems 3000+, urban runoff from Agoura Hills, Westlake Village, part of Calabasas and the 101 fwy. Ground water discharge (can discharge reclaimed water). D.S. Rancho Compost Sludge injection, massive reclaimed irrigation and overflow discharges from compost facility. First riffle below the grated Tapia discharge. 100 yards downstream from MC#8. Approach from stream edge on the left. | Elevation: 448.79ft Northing:3772284.577 Easting: 342699.281. Stream order 4 | ## Sp 00 |
| Malibu Cr. MC#8 | Upstream Tapia grated discharge. Upstream impacts: 1 sheep and cow grazing, horses, septic systems 3000+, urban runoff from Agoura Hills, Westlake Village, part of Calabasas and the 101 fwy. D.S. Rancho Compost Sludge injection, massive reclaimed irrigation and overflow discharges from compost facility. Tapia Park veer left at first street until it ends at a locked gate. Unlock the gate and drive through, be sure to lock gate behind you follow the dirt road to the end and park just in front of the next gate. Take the trail on the right (South) down to the creek. Find a path and head down stream from the picnic area and large vegetated island. Reach begins just above grated discharge 5 riffles and ends at long glide where stream veers. | Elevation: 457.00ft Northing: 3772323.80 Easting: 342631.093. Reach length:228 ft. Stream order4 |  |
| Malibu Cr. MC#13 | ## Downstream Malibu Creek x-ing . Upstream impacts: 95 sq miles of drainage 6 man made lakes, landfill, Agoura Hills, Westlake Village, part of Calabasas and the 101 fwy. drain here. From Malibu Canyon/Las Virgenes Canyon Rd. heading North turn left into the entrance of Malibu Creek State Park. Park in the 3rd parking lot by the soda machines and bathroom. Proceed over the first bridge (Training site) until you reach the second bridge/stream crossing. Reach begins 150 ft downstream just above confluence with Las Virgenes Creek 3-5 riffles ends at pool in front of stream x-ing. | Elevation: 494ft. Northing: 3774146.762 Easting: 341264.361. Reach length: 81 ft. Stream order 4 |  |
| Malibu Cr. MC#12 | ## Upstream Malibu Creek x-ing above Las Virgenes. ## Upstream impacts: 95 sq miles of drainage 6 man made lakes, landfill, Agoura Hills, Westlake Village, part of Calabasas and the 101 fwy. drain here. From Malibu Canyon/Las Virgenes Canyon Rd. heading North turn left into the entrance of Malibu Creek State Park. Park in the 3rd parking lot by the soda machines and bathroom. Proceed over the first bridge (Training site) until you reach the second bridge/stream crossing samples were taken upstream in the slow moving glide/pool by the bridge and in the riffle upstream where the creek narrows. Reach begins glide/pool by the bridge and in the riffle upstream where the creek narrows and ends at the long glide above riffle. | Elevation: 497ft. Northing: 3774160.976 Easting: 341265.417. Reach length:282 ft. Stream order 4 |  |
| Cold Cr. CC#2 | Cold Creek Outlet Upstream impacts: Drains numerous houses 500 on septic systems 100-300 horses along the creek. From Site 1 - 2 (Distance 6.75 miles) From the lagoon parking lot go straight through the light and follow the road, veering left at Civic Center Way. To Malibu Canyon Rd. turn right proceed through the canyon until you reach Piuma Rd and turn right. Drive 1/4 mile until you see road marker .27 and a “slippery when wet” sign. Park on the right just in front of the sign. If you pass Crater Camp Rd on the left-hand side you have just missed the site. You will see a brown trail marker that says Backbone Trail. Proceed 10-20 ft down the trail until you reach Cold Creek. Reach begins 400 ft upstream at the culvert 5-7 riffles and ends at large bend just downstream from the trail. | Elevation: 443.73 ft. Northing: 3771987.907 Easting: 343259.556. Reach length:421 ft Stream order3 |  |
| Cold Cr. CC#11 | Cold Creek Mid Upstream impacts: Minimal drains 20-40 homes (septic) 10-30 horses. Turn left on Cold Canyon Rd. go past Wonderview. You will see a gated area on the left hand side just below mile marker 1.36. Follow trail cross over the creek by 5ft cascade downstream 100 yards creek narrows and canopy increases. Reach begins 5 riffles downstream from where stream narrows and ends at the first riffle. | Elevation: 621ft Northing: 3773105.339 Easting: 344962.349. Reach length:160 ft Stream order3 |  |
| Cold Cr. CC#3 | Cold Creek Reference Upstream impacts: Minimal (road may drain to canyon in spots). From Site 2 - 3 (Distance 5.7 miles) Continue straight on Piuma Rd and turn left on Cold Canyon Rd to Mulhulland Hwy turn right. To Stunt Rd. turn right. As you proceed around a 360 degree turn you will see a gate on the left hand side that says Cold Creek Preserve, park and enter the gate. Follow the trail past the info kiosk and head left on small path until you reach the stream. Reach begins at waterfall downstream from entry point and 5-7 riffles upstream. | Elevation: 1325ft Northing: 3773384.861 Easting: 348087.917. Reach length:139 ft Stream order 2 | Sp 00 F 00 |
| ## Las Virgenes ## LV#5 | Las Virgenes Outlet Upstream impacts: Drains parts of Calabasas, Rancho Compost Facility irrigation, overflows and sludge injection and the landfill. Livestock (cow and sheep) is grazed upstream in this drainage. From Malibu Canyon/Las Virgenes Canyon Rd. heading North turn left into the entrance of Malibu Creek State Park. (Tell the guard that you are with Heal the Bay's Stream Team and show your parking pass!) Park in the 3rd parking lot by the soda machines and bathroom. Proceed over the first bridge (Training site) until you reach the second bridge/stream crossing on the left hand side you will see a dirt path before you hit the creek follow the path to the left about 100 feet until you see an easy access point between the vegetation on your right. Walk to the water and then turn right for about 30 feet reach begins where the creek turns sharply to the left 5 riffles and ends U.S. at split riffle. | Elevation:503 ft. Northing: 3774096.607 Easting: 341305.609. Reach length: 605 ft Stream order 3 |  |
| Las Virgenes LV#5A | Mid Las Virgenes Creek Upstream impacts: Drains part of Calabasas and the 101 fwy. Livestock (cow and sheep) is grazed upstream in this drainage. Possibly one discharge from compost facility upstream. Lost Hills Rd. Stop at trail past RCD re-vegetation project at the apar5tment complex and walk down the trail to the rip-rap. Follow rip-rap down to creek. Reach begins at first riffle upstream and ends 5-7 riffles above. | Elevation: 708 ft. Northing: 3778446.11 Easting: 342898.886. Reach length: 269 ft Stream order 3 |  |
| Las Virgenes LV# 17 | Take Las Virgenes Rd. North of the 101 fwy. to the end. Upstream Impacts: Minimal (Day hiking mountain biking and horses). Walk through the gate take the ridge route until you pass the gas pipelines and the dirt road crosses the creek. Head upstream sample beginning at the first riffle. Reach begins at first riffle upstream and ends 5 riffles above. | Elevation: 944 ft. Northing: 3783286.019 Easting: 342674.993. Reach length: 144 ft Stream order 3 |  |
| Cr. Agoura subshed #6 | Cheseboro Creek Reference Upstream impacts: Minimal (Day hiking and horses). *Could receive groundwater from landfill. From the Lagoon parking lot turn left on Pacific Coast Highway 1/4 mile to Las Virgenes Rd./Malibu Canyon Rd (just before Pepperdine) turn right to the 101Ventura Fwy going North (11 miles through the Malibu Canyon) the on ramp will be on your left hand side after Mcdonalds and Taco Bell. Exit the 101 at Cheseboro Rd. and turn right at the stop sign you will turn right again this is Cheseboro Rd. Drive 1/4- 1/2 mile on your right hand side you will see a small gate (Key needed). Drive through the gate and lock it behind you. Drive along the dirt road until you pass an old torn down house right after you pass over the creek park off the road in a small clearing on the left hand side. Reach begins upstream from the road crossing, just before the first cascade, and ends above bedrock cascade 4 riffles. | Elevation: 976 ft. Northing: 3780494.398 Easting: 340893.917. Reach length: 95 ft Stream order 2 |  |
| Medea Cr. Agoura subshed #7 | Upstream impacts: Receives drainage from Lake Lindero, Lindero Creek, Palo Comado Creek, Cheseboro Creek and most of Agoura Hills including the 101 fwy. Creek upstream is channelized with concrete. From site 6, head back on Cheseboro Rd. to the stop sign and turn left and then a quick right onto the 101 Freeway North. Take the first exit, Kanan Rd., and turn left. Continue into the canyon and turn left onto Cornell Rd. Go past the first street on left and immediately park at the turnout on the right side of the road in front of 45mph sign and a telephone pole. Walk down the path to your left until you get to the stream. Reach begins two riffles downstream past the stormdrain pipe and ends just below two culverts. | Elevation: 812 ft. Northing: 3778810.139 Easting: 337821.028. Reach length: 147 ft Stream order 4 |  |
| Triunfo Cr. ## TR#10 | Upstream impacts: Kanan Rd. heading north turn left on Triunfo Cyn Rd. Park on street just past the Triunfo/Lobo Cyn community center. Walk downstream approximately 500 yds. (where channel migrates to the right). Reach begins at and ends at | Elevation: 778. ft Northing: 3776869.193 Easting: 335036.884. Reach length: 372 ft Stream order 4 |  |
| Cr. Solstice Canyon #14 | Upstream impacts: Minimal (Day hiking 1 small stream x-ing). Corral left Solstice Cyn. Park left 300-ft. from Corral Cyn. Rd left to parking lot take trail by phones to large wooden bridge upstream. Reach begins at first riffle upstream from bridge and ends at 5 riffles up. | Elevation: 163 ft Northing: 3767687.83 Easting: 338275.68. Reach length: 310 ft Stream order 3 |  |
| Cr. Carlysle Canyon #15 | West Carlysle Creek Reference Upstream impacts: Minimal Upstream Lake Sherwood Exit Westlake Blvd. straight though Potrero Rd. stop sign vear right on to West Carlysle Rd. to last house 384. Must receive permission from owner private property. Reach begins above houses where stream becomes step pool habitat 3 riffles and ends at 6 ft. cascade above Sycamore in stream. | Elevation:1081 ft Northing: 3776475.288 Easting: 325788.335. Reach length: 182 ft Stream order 2 |  |
| Cr. Palo Comado Cyn. #16 | Upstream impacts: Minimal (Day hikers, mountain bikers and equestrian.) Cheseboro rd to end go through gate need 4x4 2.1 miles from gate to site tree no. 2050 (4 small riffles.) Reach begins 4 riffles downstream and ends at rusted pipe where we enter. | Elevation: 1233 ft Northing: 3784927.108 Easting: 339157.985. Reach length: 147 ft Stream order 2 |  |