LA Regional Water Quality

Control Board

1996

Water Quality

Assessment &

Documentation

Regional Water Quality Control Board, Los Angeles Region

1996 California Water Quality Assessment - 305(b) Report

Supporting Documentation for Los Angeles Region

Table of Contents

Introduction	  1

Data	  2

Assessment methodology	  2

Criteria used for assessing each Surface Water Beneficial Use	  4

Aquatic Life Use Support	  4

Primary Contact Recreation Use	 13

Secondary Contact Recreation Use	 14

Fish and Shellfish Consumption Use	 14

Drinking water use	 16

Agriculture use	 18

Criteria used for assessing lakes	 19

Criteria used for assessing Nearshore, Open Bays, Estuaries, and Ocean	 19

Criteria used for assessing Ground Water	 19

303(d) list	 20

Additional lists in the assessment	 21

Introduction

The Clean Water Act (Federal Water Pollution Control Act of 1972 as amended) requires that states submit biennial 305(b) reports that describe the status of the nation's waters.  Reports from each state are compiled into the "National Water Quality Inventory Report to Congress."  The nine California Regional Water Quality Control Boards prepare individual Water Quality Assessment databases that the State Water Resources Control Board compiles into the 305(b) Report.

In the past year, the United States Environmental Protection Agency (USEPA) has reviewed the 305(b) reporting process and proposed that the reports be generated on 5 or 7 year cycles that correspond to the watershed approach periods.  Therefore, the 1996 California 305(b) report including the contribution from this region is most likely the last of the biennial reports.

In 1994, the Los Angeles Regional Water Quality Control Board (hereafter referred to as the Regional Board) did not update the region's Water Quality Assessment Report due to staff constraints.  This 1996 report, therefore, involves a more extensive review of data than that of other regions.  This 1996 report, as in previous years, will be used to target areas for water quality control programs and special funding efforts.  Waterbodies listed on the 303(d) list, a subset of the 305(b) waterbodies, composed of the region's impaired waterbodies (the 303(d) list is described below), qualify for certain USEPA funding programs.  In addition, the Regional Board staff plans to use the assessment information as a starting point for work with watershed stakeholder groups developing watershed plans.

In most areas, additional data will be needed in order to link impaired waterbodies with sources of pollutants.  It appears that most of the pollutants causing impairments are of a nonpoint source nature.  The data for the assessment will be used for some Board tasks, as in developing appropriate limits in certain Waste Discharge Requirements.  In general, however, Regional Board staff do not intend to use the assessment or the impairments noted on the 303(d) list as a basis for recommending new, advanced, and/or upgraded treatment technologies for POTWs.  Rather, these issues will be considered in greater depth by stakeholders within the context of individual watersheds.

In this report, six types of waterbodies are assessed:  inland surface waterbodies (including lakes and wetlands), estuaries, nearshore zones, beaches, bays and harbors, and groundwater basins.  In this 1996 assessment cycle, most of the major rivers and creeks in the Los Angeles Region are assessed but many small tributaries, small coastal streams, channel island streams are not assessed.  Waterbodies that are not frequently sampled (but do have some data) are not assessed in detail at this time due to limited staff resources.

Data

For this report, Regional Board staff supplemented Regional Board ambient water quality data with data from a large number of agencies.  These agencies include:  United States Geological Survey, Ventura County Flood Control District, Los Angeles County Department of Public Works, City of Thousand Oaks, San BuenaVentura, City of Pasadena, City of Los Angeles, Metropolitan Water District of Southern California, Water Replenishment District of Southern California, Central and West Basin Municipal Water Districts, and local water purveyors.  Self-monitoring data was used from major dischargers in the Ventura and Calleguas Creek watersheds (these two areas were Regional Board "target watersheds" in 1995-1996).  In general, six years of water quality data (1988 to present) were used for this assessment; for bioaccumulation and sediment toxicity data, up to ten years of data were used.  Most of the water column and sediment data were generated from grab samples.  In addition to data, information from various articles and reports were used for the assessment.  These reports are cited in the attached Data Summary Tables.

The data for the assessment were compiled and organized by watershed.  Watersheds in this assessment include:

Coastal Ventura County

Ventura River

Santa Clara River

Calleguas Creek

Santa Monica Bay

Southern Los Angeles County (includes Dominguez and Los Cerritos Channels)

Los Angeles River

San Gabriel River

Ventura County coastal features

Los Angeles County coastal features

Channel islands (Santa Catalina, Santa Cruz, San Nicholas, Santa Barbara, San Clemente)

Data used for this assessment were limited;  a compete suite of water quality parameters were not available for measurement against appropriate water quality standards for each waterbody (e.g., all of the Safe Drinking Water Act constituents or USEPA priority pollutants).  Therefore, it is not correct to assume that constituents not cited in the assessment are not causing water quality problems.

Assessment methodology

The Regional Board's water quality assessment follows USEPA (1995) guidance as outlined in the Guidelines for Preparation of the 1996 State Water Quality Assessments (305(b) Reports).  The guidance specifies that seven broad beneficial use categories be assessed under the federal guidance; the federal beneficial uses in this assessment report and the corresponding State beneficial uses are shown in Table 1.

Each of these federal beneficial uses is assessed according to the following designations:  fully supporting, fully supporting but threatened, partially supporting, not supporting, and not assessed.   The fully supporting but threatened category relates to waterbodies where a use is supported but may not be in the future (because of anticipated sources or adverse pollution trends) unless pollution prevention or control action is taken .  Waterbodies that are assessed as partially supporting and not supporting are considered "impaired."  In addition, the terms "partially supporting" and "not supporting" are federal terms and roughly equate to "intermediate" and "impaired" terms used by the state in preparing previous water quality assessments for the Los Angeles Region.

Table 1.  Correlation between California Beneficial Uses and USEPA 305(b) Beneficial Uses.

| State Beneficial Use as designated in 1994 Basin Plan | Federal Beneficial Use assessed in this report |
| --- | --- |
| Commercial and sport fishing | Fish consumption |
| Aquaculture |  |
| Shellfish Harvesting | Shellfish harvesting |
| Warm freshwater habitat | Aquatic life use support |
| Cold freshwater habitat |  |
| Inland saline water habitat |  |
| Freshwater replenishment |  |
| Estuarine habitat |  |
| Wetland habitat |  |
| Marine habitat |  |
| Wildlife habitat |  |
| Preservation of biological habitat |  |
| Rare, threatened, or endangered species |  |
| Migration of aquatic organisms |  |
| Spawning, reproduction, and/or early development |  |
| Water contact recreation | Swimming or primary contact recreation use |
| Non-contact recreation | Secondary contact recreation use |
| Municipal and domestic supply | Drinking water supply (raw data) |
| Ground water recharge (where appropriate) |  |
| Agricultural supply | Agriculture |
| Ground water recharge (where appropriate) |  |
| Hydropower Generation | Not assessed in this report |
| Navigation |  |
| Industrial process supply |  |
| Industrial service supply |  |

When comparing data against standards, the "worst case approach" is used.  That is, if one parameter, such as temperature, ammonia, or an organic chemical, indicates impairment for a particular use, the waterbody is designated as impaired for the use affected by this parameter.  For example, a waterbody that is not supporting the aquatic life use due to high ammonia concentrations and is partially supporting the use due to elevated metal concentrations would be given an overall classification of "not supporting."  Exceptions to this are as follows:

1.  For the drinking water use, if constituents (with the exception of  volatile organic chemicals) exceed secondary standards (i.e., secondary MCLs), and thus are not supporting the use, the overall use is classified as "fully supporting but threatened."  If constituents less frequently exceed secondary standards and thus are partially supporting the use, the overall use is classified as "fully supporting."

2.  For the contact recreation use,  if constituents exceed secondary drinking water or taste and odor standards (i.e., pH, turbidity, color, ammonia) and thus are not supporting the use, the overall use is classified as "partially supporting."  If these constituents less frequently exceed the secondary standards and thus are partially supporting the use, the overall use is classified as "fully supporting but threatened."

3.  For the secondary contact recreation use,  if color or turbidity are elevated and are not or are partially supporting the use, the overall use is classified as "fully supporting but threatened" because these parameters may be seasonally controlled and further observations are needed to determine the definitive impairment status.

Each watershed in the region is divided into waterbody reaches (a specified segment of river or creek) and lakes or reservoirs that match those designated in the 1994 Water Quality Control Plan (hereafter referred to as Basin Plan).  For this report, some individual reaches are combined into longer reaches while other reaches are listed as "not assessed" due to lack of data.   Beneficial uses in each reach are assessed and are given a number of miles (or square miles/acreage for lakes and groundwater basins) that are supporting that use (e.g., 3.5 miles of the xx reach fully supports the agriculture use but 2.5 miles only partially support the aquatic life use support).  Data collection stations are located within each reach and cover areas no more than 25 miles but usually no more than a few miles.  In this 1996 report, particular emphasis is placed on the Ventura River and Calleguas Creek watersheds because they are the 1995-1996 Regional Board targeted watersheds.  The main stem of the Los Angeles River is also assessed in detail because it will be a targeted watershed in the 1996-1997 watershed cycle.  Ocean water data are not assessed in this report other than from seafood consumption advisories and review of literature.

In the assessment, potential sources of contamination are identified to the level that is known at this time.  For most waterbodies, data were not sufficient to link specific sources to specific pollutants so this is not done in the 1996 assessment.  In future assessments for this region, linkage may be possible.  Sources are listed for a waterbody that do not contribute necessarily to the listed "causes" of impairment.  These potential sources are listed in order to assist staff performing future assessments; incomplete data for waterbodies precluded making a complete list of causes of impairments at this time (many waterbodies have not been analyzed for metals and/or priority pollutants).

Some beneficial uses, notably agriculture and in some cases aquatic life and contact recreation, are impaired due to constituents that have naturally high concentrations within a watershed or subwatershed.  Examples of these constituents include total dissolved solids, chlorides, boron and sulfate that are leached from rock formations.  In some lakes and estuaries, coliform counts may be high due to a large population of waterfowl.  Not enough information is available at this point to classify any of the affected uses as "unattainable".  Under the sources column (or in the sources filed in the database), "natural source" is listed for these waterbodies.

Ranking of relative contributions of each cause and source to the overall impairment of a waterbody is classified with slight (S), moderate (M) and high (H) magnitude.  For example, contaminant sources for a waterbody that include natural sources, urban runoff and municipal effluent would all be classified as "moderate" because they all contribute to some degree.  If there is only one source listed then it is considered to be a high magnitude source contributing to the impairment.

Assessments in this report are distinguished as either "evaluated" or "monitored".  "Evaluated" assessments are based on information about land uses, location of sources, predictive modeling, best professional judgement, as well as the use of older data.  "Monitored" assessments are based on recent ambient water quality, sediment quality, bioaccumulation and toxicity data that are collected relatively frequently.  Most of the assessments in the 1996 cycle are considered "monitored".

Criteria used for assessing each Surface Water Beneficial Use

The USEPA Guidelines for Preparation of the 1996 State Water Quality Assessments (305(b) Reports) provides formulas for conducting assessment of the six 305(b) beneficial uses.  These guidelines are described below along with the criteria or standards against which the data is compared.  References for standards are included in each section.

Aquatic Life Use Support

Aquatic life use support can be assessed based on biological and habitat factors or on physical and chemical data.

Biological/habitat assessments

Biological/habitat assessments in this region are limited to reported or observed sediment and erosion impacts and personal communication with federal Fish and Wildlife, state Fish and Game biologists and other local experts.  The Regional Board does not have resources at this time to perform detailed field biological/habitat assessments.  USEPA's application of biological/habitat assessments are described in table 2.  In addition, the Regional Board used best professional judgement to indicate a few localized habitat-related problems such as areas of high sedimentation, fish kills, barriers to fish migration, and impairment of benthic communities.  Information from published documents such as the Santa Monica Bay State of the Watershed Report (1993) was also used.

Table 2.  Water Quality Assessment guidelines

| Assessment designation | Assessment Guideline |
| --- | --- |
| Aquatic life use support: biological and habitat factors |  |  |
| Fully supporting | Reliable data indicates functioning, sustainable biological communities (e.g., macro-invertebrates, fish, or algae) none of which has been modified significantly beyond the natural range of the reference condition. |
| Partially supporting | At least one assemblage (e.g., macro-invertebrates, fish, or algae) indicates less than full support with slight to moderate modification of the biological community noted. Other assemblages indicate full support. |
| Not supporting | At least one assemblage indicates nonsupport. Data clearly indicate severe modification of the biological community. |
| Aquatic life use support: water column toxic substances- priority pollutants, chlorine, ammonia |  |  |
| Fully supporting | For any one pollutant, no more than 2 violations of chronic criteria (acute if no chronic criteria available) within a 6 year period based on at least 20 grab or 1-day composite samples. If fewer than 20 samples are available, then best professional judgement is used considering the number of pollutants having violations and the magnitudes of the exceedence(s). |
| Partially supporting | For any one pollutant, criteria exceeded more than twice within a 6-year period, but in  10 percent of samples. |
| Not supporting | For any one pollutant, criteria exceeded in > 10 percent of samples. |
| Aquatic life use support: water column conventional constituents and stressors- dissolved oxygen, temperature, chloride, pH |  |  |
| Fully supporting | For any one pollutant, criteria exceeded in  10 percent of measurements. |
| Partially supporting | For any one pollutant, criteria exceeded in 11 to 25 percent of measurements. |
| Not supporting | For any one pollutant, criteria exceeded in > 25 percent of measurements. |
| Primary contact and non-contact recreation use and Agriculture use: Taste and Odor (includes secondary drinking water MCLs) and aesthetic stressors |  |  |
| Fully supporting | For any one pollutant or stressor, criteria exceeded in  10 percent of measurements or observations. |
| Partially supporting | For any one pollutant, criteria exceeded in 11 to 25 percent of measurements or observations. |
| Not supporting | For any one pollutant, criteria exceeded in > 25 percent of measurements or observations. |
| Coliform bacteria data for Primary and Secondary Contact (inland surface waterbodies) recreation use |  |  |
| Fully supporting | Criterion 1 and/or Criterion 2 met. (see table 7) |
| Partially supporting | For contact recreation: Criterion 1 met but not more than 10 percent of samples exceed 2,000 per 100 ml. For non-contact recreation, not more than 10 percent of the samples exceed 10,000 per 100 ml. |
| Not supporting | Neither criterion met. |
| Contact Recreation: beach coliform data |  |  |
| Fully supporting | For entire data set, wet and dry weather fecal coliform standards are exceeded 15% or less times on average and wet and dry weather total coliform data are exceeded 20% or less times on average. |
| Partially supporting | At least one of the following is exceeded: wet and dry weather fecal coliform greater than 15% and wet and dry weather total coliform greater than 20%. |
| Not supporting | For entire data set, wet and dry weather fecal coliform standards are exceeded more than 15% or wet and dry weather total coliform data are exceeded more than 20%. |
| Primary Recreation Contact Use: Beach and inland bathing area closure |  |  |
| Fully supporting | No bathing area closures or restrictions in effect during past 6 years. |
| Partially supporting | On average, one bathing area closures per year of less than 1 week's duration. |
| Not supporting | On average, one bathing area closure per year of greater than 1 week's duration, or more than one bathing area closure per year. |
| Fish and shellfish consumption use: advisories |  |  |
| Fully supporting | No fish or shellfish restrictions or bans are in effect. |
| Partially supporting | "Restricted consumption" of fish or shellfish in effect. Restricted consumption is defined as limits on the number of meals or size of meals consumed per unit time for one or more fish or shellfish species. |
| Not supporting | "No consumption" of fish or shellfish ban in effect for general population, or a subpopulation that could be at potentially greater risk, for one or more fish or shellfish species; or commercial fishing or shellfishing ban in effect. |
| Primary contact and non-contact recreation: aesthetics stressor-field observations |  |  |
| Fully supporting | Criteria exceeded in less than or equal to 10 % of observations. |
| Partially supporting | Criteria exceeded in 11 to 25 % of observations |
| Not supporting | Criteria exceeded in greater than 25% of observations. |
| Drinking water use: water quality data |  |  |
| Fully supporting | No contaminants where the median concentration exceeds the state water quality standard. No restrictions (i.e., no source water closures or advisories, no waters requiring more than conventional treatment to enable drinking water use) |
| Fully supporting but threatened | No contaminants where the median concentration exceeds the state water quality standards. Increased monitoring imposed on public water supplies supplied by the waterbody (due to previous detections of contaminants that triggered an increased monitoring frequency) or potential for water quality degradation by contaminants that are known to be used or present in the watershed or basin. |
| Partially supporting | No contaminants where the median concentration exceeds the state water quality standards. One or more drinking water source advisories lasting greater than 30 days per year or public water supplies supplied by the waterbody require more than conventional treatment due to contaminants concentrations in source water that may adversely affect treatment costs or the quality of finished water (e.g., due to taste, odor, turbidity, dissolved solids, etc.). |
| Not supporting | One or more contaminants where the median concentration exceeds the state water quality standards. One or more contamination-based closures of a drinking water source. |
| Assessment guideline for cases where there are fewer than 20 data points (all uses) |  |  |
| Not supporting | For constituents where there are <20 and  3 samples, more than 40% of the values exceed the standard. |

Physical and chemical water column data

Most of the aquatic life use support assessments in the Los Angeles Region are based on physical and chemical water, as well as sediment, toxicity and bioaccumulation data (described below).  Physical and chemical data (water column) includes toxic substances (priority pollutants, chlorine and ammonia) and conventional constituents or stressors (dissolved oxygen, pH, and temperature).  The assessment guidelines, based on USEPA's guidance document, are shown in table 2.

Criteria for aquatic life use support are drawn from the region's 1994 Basin Plan and the USEPA Water Quality Criteria for Water (1986 and updates).  Relevant criteria are listed in tables 3 and 4.  Note that the metals data are compared "total recoverable" standards.

Table 3.  Freshwater Aquatic life use support standards (water column)

Note:  The most stringent criteria are selected.  Constituents are only listed if samples used for assessment had values above detection limits.

Reference:  US Environmental Protection Agency National Ambient Water Quality  Criteria 1986 with updates, except where noted.

| Constituent or Stressor | Status | Criteria (ppb unless noted) |
| --- | --- | --- |
| Inorganic Constituent or other stressor |  |  |  |  |
| Aluminum (pH: 6.5 - 9.0) | 4-day average | 87 |
| Ammonia | 4-day average | see table d |
| Antimony (trivalent) | 4-day average | 30 (Proposed) |
| Arsenic | 4-day average | 190 |
| Cadmium* (dissolved) | 4-day average | [e(0.7852[ln{hardness}]-3.490)] x [1.101672-([ln{hardness}] x [0.041838])] |
| Cadmium* (total recov'ble) | 4-day average | [e(0.7852[ln{hardness}]-3.490)] |
| Chloride (as NaCl) | 4-day average | 230 |
| Residual chlorine, total | 4-day average | 11 |
| Chromium VI | 4-day average | 10 |
| Copper* (dissolved) | 4-day average | (e{0.8545[ln(hardness)]-1.465}) x (0.960) |
| Copper* (total recov'ble) | 4-day average | (e{0.8545[ln(hardness)]-1.465}) |
| Cyanide | 4-day average | 5.2 |
| Lead* (dissolved) | 4-day average | (e{1.273[ln(hardness)]-4.705}) x (1.46203-{[ln(hardness)] x [0.145712]}) |
| Lead* (total recoverable) | 4-day average | (e{1.273[ln(hardness)]-4.705}) |
| Mercury (total recoverable) | 4-day average | 0.012 |
| Nickel* (dissolved) | 4-day average | (e{0.8460[ln(hardness)]+1.1645}) x (0.997) |
| Nickel* (total recoverable) | 4-day average | (e{0.8460[ln(hardness)]+1.1645}) |
| Oxygen, dissolved (1994 Basin Plan) |  | At a minimum (see specifics below), the mean annual dissolved oxygen concentration of all waters shall be greater than 7 mg/L, and no single determination shall be less than 5.0 mg/L, except when natural conditions cause lesser concentrations. The dissolved oxygen content of all surface waters designated as WARM shall not be depressed below 5 mg/L as a result of waste discharges. The dissolved oxygen content of all surface waters designated as COLD shall not be depressed below 6 mg/L as a result of waste discharges. The dissolved oxygen content of all surface waters designated as both COLD and SPWN shall not be depressed below 7 mg/L as a result of waste discharges. For that area known as the Outer Harbor area of Los Angeles-Long Beach Harbors, the mean annual dissolved oxygen concentrations shall be 6.0 mg/L or greater, provided that no single determination shall be less than 5.0 mg/L. |
| pH | Instantaneous maximum | 6.5 - 9.0 units |
| Selenium (total recoverable) | 4-day average | 5.0 |
| Silver* (dissolved) | 1-hour average | (e{1.72[ln(hardness)]-6.52}) x (0.85) |
| Silver* (total recoverable) | 1-hour average | (e{1.72[ln(hardness)]-6.52}) |
| Temperature (1994 Basin Plan) |  | The natural receiving water temperature of all regional waters shall not be altered unless it can be demonstrated to the satisfaction of the Regional Board that such alteration in temperature does not adversely affect beneficial uses. Alterations that are allowed must meet the requirements below. For waters designated WARM, water temperature shall not be altered by more than 5 F above the natural temperature. At no time shall these WARM-designated waters be raised above 80 F as a result of waste discharges. For waters designated COLD, water temperature shall not be altered by more than 5 F above the natural temperature. |
| Zinc (dissolved) | 4-day average | e{0.8473[ln(hardness)]+0.7614} x (0.986) |
| Zinc (total recoverable) | 4-day average | e{0.8473[ln(hardness)]+0.7614} |
| Organic Constituents |  |  |  |  |
| Aldrin | Instantaneous maximum | 3 |
| Benzene | Acute toxicity | 5300 |
| Benzene hexachloride (g-BHC; Lindane) | 24-hour average | 0.08 |
| Bromodichloromethane ** | Acute toxicity | 11,000 (sum of halomethanes) |
| Bromoform** | Acute toxicity | 11,000 (sum of halomethanes) |
| Carbon tetrachloride | Acute toxicity | 35,200 |
| Chlordane | 24-hour average | 0.0043 |
| Chloroform | Chronic toxicity | 1240 |
| DDD | Acute toxicity | 0.6 |
| DDE | Acute toxicity | 1050 |
| DDT | 24-hour average | 0.001 |
| Dibromochloromethane** | Chronic toxicity | 11,000 (sum of halomethanes) |
| 1,1-Dichloroethylene (1,1-DCE)† | Acute toxicity | 11,600 (sum of dichlorethylenes) |
| cis-1,2-Dichloroethylene† | Acute toxicity | 11,600 (sum of dichlorethylenes) |
| trans- 1,2-Dichloroethylene† | Acute toxicity | 11,600 (sum of dichlorethylenes) |
| Di(2-ethylhexyl)pthalate | 4-day average | 360 |
| Di(n-octyl) phthalate | Chronic toxicity | 3 |
| Endrin | 24-hour average | 0.0023 |
| Ethylbenzene | Acute toxicity | 32,000 |
| Heptachlor | 24-hour average | 0.0038 |
| Heptachlor epoxide | 24-hour average | 0.0038 |
| Toluene | Acute toxicity | 17,000 |
| 1,1,1-Trichloroethane (1,1,1-TCA) | Acute toxicity | 18,000 |
| Trichloroethylene (TCE) | Acute toxicity | 45,000 |

*Criteria for metals based on hardness use actual hardness at time of sampling or if not available, use average hardness for reach.** Halomethanes  † Dichlorothylenes

Table 4.  Four-day Average Concentration for Ammonia*

Reference:  USEPA:  Revised tables for determining average freshwater ammonia concentrations.  Office of Water Memorandum, July 30, 1992.

| pH | Temperature oC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 5 | 10 | 15 | 20 | 25 | 30 | 0 | 5 | 10 | 15 | 20 | 25 | 30 |
| Un-ionized ammonia (mg/liter NH3) for waters designated as COLD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Un-ionized ammonia (mg/liter NH3) for waters designated as WARM |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.50 | 0.0008 | 0.0011 | 0.0016 | 0.0022 | 0.0022 | 0.0022 | 0.0022 | 0.0008 | 0.0011 | 0.0016 | 0.0022 | 0.0031 | 0.0031 | 0.0031 |
| 6.75 | 0.0014 | 0.0020 | 0.0028 | 0.0039 | 0.0039 | 0.0039 | 0.0039 | 0.0014 | 0.0020 | 0.0028 | 0.0039 | 0.0055 | 0.0055 | 0.0055 |
| 7.00 | 0.0025 | 0.0035 | 0.0049 | 0.0070 | 0.0070 | 0.0070 | 0.0070 | 0.0025 | 0.0035 | 0.0049 | 0.0070 | 0.0099 | 0.0099 | 0.0099 |
| 7.25 | 0.0044 | 0.0062 | 0.0088 | 0.0124 | 0.0124 | 0.0124 | 0.0124 | 0.0044 | 0.0062 | 0.0088 | 0.0124 | 0.0175 | 0.0175 | 0.0175 |
| 7.50 | 0.0078 | 0.0111 | 0.0156 | 0.022 | 0.022 | 0.022 | 0.022 | 0.0078 | 0.0111 | 0.0156 | 0.022 | 0.031 | 0031 | 0.031 |
| 7.75 | 0.0129 | 0.0182 | 0.026 | 0.036 | 0.036 | 0.036 | 0.036 | 0.0129 | 0.0182 | 0.026 | 0.036 | 0.051 | 0.051 | 0.051 |
| 8.00 | 0.0149 | 0.021 | 0.030 | 0.042 | 0.042 | 0.042 | 0.042 | 0.0149 | 0.021 | 0.030 | 0.042 | 0.059 | 0.059 | 0.059 |
| 8.25 | 0.0149 | 0.021 | 0.030 | 0.042 | 0.042 | 0.042 | 0.042 | 0.0149 | 0.021 | 0.030 | 0.042 | 0.059 | 0.059 | 0.059 |
| 8.50 | 0.0149 | 0.021 | 0.030 | 0.042 | 0.042 | 0.042 | 0.042 | 0.0149 | 0.021 | 0.030 | 0.042 | 0.059 | 0.059 | 0.059 |
| 8.75 | 0.0149 | 0.021 | 0.030 | 0.042 | 0.042 | 0.042 | 0.042 | 0.0149 | 0.021 | 0.030 | 0.042 | 0.059 | 0.059 | 0.059 |
| 9.00 | 0.0149 | 0.021 | 0.030 | 0.042 | 0.042 | 0.042 | 0.042 | 0.0149 | 0.021 | 0.030 | 0.042 | 0.059 | 0.059 | 0.059 |
| Total ammonia (mg/liter NH3) for waters designated as COLD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Total ammonia (mg/liter NH3) for waters designated as WARM |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.50 | 3.0 | 2.8 | 2.7 | 2.5 | 1.76 | 1.23 | 0.87 | 3.0 | 2.8 | 2.7 | 2.5 | 2.5 | 1.73 | 1.23 |
| 6.75 | 3.0 | 2.8 | 2.7 | 2.6 | 1.76 | 1.23 | 0.87 | 3.0 | 2.8 | 2.7 | 2.6 | 2.5 | 1.74 | 1.23 |
| 7.00 | 3.0 | 2.8 | 2.7 | 2.6 | 1.76 | 1.23 | 0.87 | 3.0 | 2.8 | 2.7 | 2.6 | 2.5 | 1.74 | 1.23 |
| 7.25 | 3.0 | 2.8 | 2.7 | 2.6 | 1.77 | 1.24 | 0.88 | 3.0 | 2.8 | 2.7 | 2.6 | 2.5 | 1.75 | 1.24 |
| 7.50 | 3.0 | 2.8 | 2.7 | 2.6 | 1.78 | 1.25 | 0.89 | 3.0 | 2.8 | 2.7 | 2.6 | 2.5 | 1.76 | 1.25 |
| 7.75 | 2.8 | 2.6 | 2.5 | 2.4 | 1.66 | 1.17 | 0.84 | 2.8 | 2.6 | 2.5 | 2.4 | 2.3 | 1.65 | 1.18 |
| 8.00 | 1.82 | 1.70 | 1.62 | 1.57 | 1.10 | 0.78 | 0.56 | 1.82 | 1.70 | 1.62 | 1.57 | 1.55 | 1.10 | 0.79 |
| 8.25 | 1.03 | 0.97 | 0.93 | 0.90 | 0.64 | 0.46 | 0.33 | 1.03 | 0.97 | 0.93 | 0.90 | 0.90 | 0.64 | 0.47 |
| 8.50 | 0.58 | 0.55 | 0.53 | 0.53 | 0.38 | 0.28 | 0.21 | 0.58 | 0.55 | 0.53 | 0.53 | 0.53 | 0.39 | 0.29 |
| 8.75 | 0.34 | 0.32 | 0.31 | 0.31 | 0.23 | 0.173 | 0.135 | 0.34 | 0.32 | 0.31 | 0.31 | 0.32 | 0.24 | 0.190 |
| 9.00 | 0.195 | 0.189 | 0.189 | 0.195 | 0.148 | 0.116 | 0.094 | 0.195 | 0.189 | 0.189 | 0.195 | 0.21 | 0.163 | 0.133 |

* To convert these values to mg/liter N, multiply by 0.822.

Sediment chemistry and toxicity, water column toxicity, benthic community and bioaccumulation data

Lacking USEPA guidelines, the Regional Board developed assessment guidelines for sediment chemistry and toxicity, benthic community and bioaccumulation data for purposes of this water quality assessment report.  These general guidelines are described below.

Virtually all of this Region's sediment toxicity data has been generated through the Bay Protection & Toxic Cleanup Program (BPTCP).  The most commonly used sediment toxicity test is the amphipod (a crustacean) survival test.  A review of all the data for the region reveals the number of tests in which less than 60% of the amphipods survive is much less than the number of tests in which at least 60% or more amphipods survive.  Consequently, the "significant toxicity" line is drawn at 60% survival.  Below that number it's more likely that impairment is occurring (especially since existing benthic data at those sites support this).  No statistical analyses or comparison to reference sites were done, however.  This is a qualitative analysis utilizing Best Professional Judgement.  Also, no conclusions are drawn based solely on one-time toxicity testing.

Table 5.  Sediment and Bioaccumulation chemistry probable "background" levels

| Constituent | Sediment Chemistry: Probable "background" levels in the Region | Bioaccumulation: Probable "background" levels in the Region |
| --- | --- | --- |
| PAHs (polycyclic aromatic hydrocarbons) | 1 ppm | ND |
| chlordane | 100 ppb | 100 ppb |
| PCBs | 200 ppb | 300 ppb |
| DDT | 200 ppb | 300 ppb |
| zinc | 200 ppm | 250 ppm |
| lead | 50 ppm | 15 ppm |
| copper | 100 ppm | 50 ppm |
| chromium | 75 ppm | 5 ppm |
| TBT (tributyltin paint) | 1 ppm | ND |

Note:  older TBT numbers in the database are almost certainly obsolete; levels have dropped dramatically since 1988 when TBT use prohibitions started, also, TBT degrades fairly quickly.

Listed in table 5 are the region's probable "background" numbers for the more common sediment chemistry pollutants.  These numbers are approximate and based on pollutant levels found in areas removed from direct point sources where impacts do not appear to be occurring in the benthic community.  Often background concentrations are due to natural sources or are due to persistent organic chemicals that have not yet biodegraded completely.  The background levels were determined by evaluating data from areas that are remote from point sources and significant nonpoint sources.

For bioaccumulation in sediment, "background" numbers are also utilized (table 5).  These may be adjusted  up or down depending on the type of waterbody (commercial port vs. coastal lagoon).

For bioaccumulation in aquatic organisms, data from the State Mussel Watch and Toxic Substances Monitoring program were used.  These two state programs provide information about the occurrence of toxic substances in fresh, estuarine, and marine waters through analysis of fish, mussels and other aquatic life (referred to as "tissue" in this report).  Ten metals and approximately 45 pesticides and other organic chemicals are analyzed from the tissue of these organisms.  Not every sample is analyzed for all metals or organic chemicals.  For this 305(b) assessment, Mussel Watch data were used only to evaluate the aquatic life use.  Toxic Substances Monitoring data were used to evaluate both the fish consumption and the aquatic life use.

Bioaccumulation data collected from tissue are compared to criteria such as Maximum Tissue Residue Levels (MTRLs), U. S. Food and Drug Administration (FDA) action levels, Median International Standards (MIS), and the National Academy of Sciences (NAS) recommended guidelines for predator protection (table 6 ).

Fish tissue Elevated Data Level (EDL) values are an internal state comparative measure that ranks a given concentration of a particular substance with previous data from the state programs.  EDLs are calculated by ranking all of the results for a given chemical from the highest concentration measured down to and including those records where the chemical was not detected.  The 85th percentile (EDL85) was chosen as an indication that a chemical is elevated from the median and the 95th percentile (EDL95) was chosen to indicate values that are highly elevated.  EDLs were used in the assessment as follows:  If no other constituents exceed standards, but if one or two constituents were above the EDL85 or EDL95, then those constituents are listed as "fully supporting but threatened."  If three or more constituents are above the EDL then those constituents are listed as "partially supporting".

For water column toxicity, the Basin Plan (1994) objective of "no less than 70% in a single test and no less than a mean of 90% in any three consecutive tests" is used.

BPTCP benthic community data was collected in Los Angeles and Long Beach Harbor (LA/LB Harbor) and Alamitos Bay.  Extrapolations were made from the data, mostly regarding the presence of pollutant-tolerant species.  Areas

Table 6.  Standards used for tissue data (State Mussel Watch Toxic Substances Monitoring Programs).

| Constituent | Standards-ppb (see text for explanation of abbreviations) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NAS Recommended guideline for freshwater fish | FDA Action level for freshwater and marine fish | MTRLs for inland surface waters | MTRLs for ocean waters | MIS for freshwater fish and marine shellfish (range) |
| Mercury | 500 | 1000 | 1000 | - | 100-1000 |
| DDT (total) | 1000 | 5000 | 32.0 | 9.1 | - |
| PCBs | 500 | 2000 | 2.2 | - | - |
| Aldrin | 100 | 300 | 0.05 | 0.1 | - |
| Dieldrin | 100 | 300 | 0.65 | 0.2 | - |
| Endrin | 100 | 300 | 3000 | - | - |
| Heptachlor | 100 | 300 | 1.8 | 8.1 | - |
| Heptachlor epoxide | 100 | 300 | 0.8 | - | - |
| Chlordane | 100 | 300 | 1.1 | 0.32 | - |
| Lindane | 100 | - | 2.5 | - | - |
| Hexachlorocyclohexane | 100 | - | alpha: 0.5 beta: 1.8 | - | - |
| Endosulfan | 100 | - | 250 | - | - |
| Toxaphene | 100 | 5000 | 8.8 | 2.75 | - |
| Arsenic | - | - | 200 | - | 100-5000 |
| Hexochlorobenzene (HCB) | - | - | 6.0 | 0.6 | - |
| PCBs | - | - | 2.2 | 0.6 | - |
| PAHs | - | - | 0.08 | - | - |
| Pentachlorophenol (PCP) | - | - | 3.1 | - | - |
| Cadmium | - | - | 640 | - | 50-2000 |
| Nickel | - | - | 28000 | - | - |
| Chromium | - | - | - | - | 1000 |
| Copper | - | - | - | - | 10000-100000 |
| Lead | - | - | - | - | 500-10000 |
| Zinc | - | - | - | - | 40000-100000 |

distinctly different in benthic community composition from the "norm" for that kind of waterbody and containing large numbers of pollutant-tolerant species are designated as "not supporting".

BPTCP benthic community data was collected in Los Angeles and Long Beach Harbor (LA/LB Harbor) and Alamitos Bay.  Extrapolations were made from the data, mostly regarding the presence of pollutant-tolerant species.  Areas distinctly different in benthic community composition from the "norm" for that kind of waterbody and containing large numbers of pollutant-tolerant species are designated as "not supporting".

Combining these different data types (sediment chemistry and toxicity, water column toxicity, benthic community and bioaccumulation)  into an overall assessment for each waterbody requires balancing quantity of evidence and type of evidence.  Using Los Angeles/Long Beach Harbor BPTCP data as an example, most areas of the harbor where background levels of contaminants occur exhibited moderate to low sediment toxicity and, at most, a marginally impacted benthic community.  Many areas are contaminated with metals and, as BPTCP work has revealed (and expert advice from the program's Scientific Review & Planning Committee supports), generally metals are not bio-available (they tend to bind with the usually generous amounts of sulfides in the sediments) and don't contribute to sediment toxicity.  Sediments contaminated with even high levels of metals, but not with organic chemicals, will usually not result in adverse effects.  The chief concern is organic chemicals but even here impacts appear minimal at background pollutant levels.

A weight-of-evidence approach, heavily influenced by best professional judgement, is used to judge aquatic life beneficial use support for coastal areas.  Ideally, this approach would utilize field-replicated triad data (benthic, sediment toxicity, and sediment chemistry) collected at the same time at least twice over some fairly short time period and use a reference site for comparison purposes.  Instead, what is mostly available are data collected under different programs, measuring different media, at different sites, over a number of years, without good reference sites for statistical purposes.

At least two data points are desired for any particular type of data.  Thus, one toxicity test result (lab-replicated) is revealing but not necessarily useful without some other kind of information.  For example, high sediment toxicity in one test conducted under the BPTCP combined with low sediment contamination and bioaccumulation found through the SMWP would not necessarily lead to assigning the waterbody a "not supporting" status.  There are natural causes of toxicity that may be transient.  If sampling occurred during rainy weather, other unknown (and often non-anthropogenic) factors may be involved.  Test results (especially effects information) obtained during aberrant conditions really shouldn't be used to define the status of a waterbody.

As another example, past SMWP data may show low to moderate PCBs and PAHs in sediment and tissue and more recent sediment toxicity testing may result in less than 60% survival on two sampling dates.  The SMWP data may not be obviously high but the toxicity data point out a problem.  Benthic data would be helpful, but based on toxicity, bioaccumulation, and sediment chemistry, the waterbody would be designated not fully supporting aquatic life beneficial uses.  "Good" effects data are weighed more heavily than simple measurements such as sediment pollutant levels or tissue bioaccumulation (within the limitations described earlier).  The latter measurement isn't an effect, it's just an indicator of the presence of pollutants.  Unless National Academy of Sciences tissue guidelines are exceeded, effects are not expected.

For SWMP data, "background level guidelines" apply only to transplanted California mussels.  Other organisms used by the program include transplanted freshwater clams, resident California mussels, transplanted bay mussels, and resident bay mussels.  These bioaccumulate at different rates (especially freshwater clams) and are hard to compare to each or to a guideline.

Primary Contact Recreation Use

One of the goals of the Clean Water Act is that all waterbodies of the nation be "swimmable."  Many of the waterbodies of the Los Angeles region are designated as "swimmable" or usable for water-contact recreation.  Some of these designated waterbodies, however, are inaccessible due to gates and fences installed for flood control or drinking water reservoir protection purposes.  In spite of this, residents, homeless individuals and occasionally children often gain access and use these areas.  Therefore, all waterbodies with a water-contact recreation use have been included in this report.  On the 303(d) list table, access restrictions are indicated with a footnote.

Assessment of primary contact recreational uses is based on closure data for bathing areas, coliform bacteria data, hazardous substances and aesthetics.  Bathing closure data was acquired from the Los Angeles and Ventura County Departments of Health Services.  Guidelines for assessments and coliform bacteria standards and assessment guidelines are shown in Table 7.  Inland surface waterbody coliform data is not collected on a frequent basis; only fecal coliform standards are used.  Dry weather beach data are collected frequently, weekly or daily, in the surfzone by major ocean dischargers and by the Los Angeles County Department of Health Services.  Wet weather coliform data is collected during storms.  These data are compared to Ocean Plan standards and include both total and fecal coliform.

Hazardous substances in water and bottom sediment are evaluated on a case by case basis.  Secondary Drinking water MCLs (table 8) related to contact recreation are also assessed.  Additional factors such as persistent scum, oily films, excessive algae growth, significant trash, and persistent observations of non-natural foam and/or odor were also considered (tables 2 and 9).

Table 7.   Indicator Bacteria Criteria for Primary and Secondary contact recreation use and Shellfish Consumption

| Assessment designation | Assessment Guideline |
| --- | --- |
| Criterion | Primary Contact Recreation (inland) |
| Criterion 1 | Fecal coliform concentration shall not exceed a log mean of 200/100 ml (based on a minimum of not less than four samples for any 30-day period) [Note for this Region, we do not have weekly data for most of the waterbodies so this criterion is not used] |
| Criterion 2 | No more than 10 percent of total samples during any 30-day period exceed 400/100 ml. [Note: used entire data set for each reach rather than 30-day period] |
| Criterion | Secondary Contact Recreation(Inland) |
| Criterion 1 | Fecal coliform concentration shall not exceed a log mean of 2000/100 ml (based on a minimum of not less than four samples for any 30-day period) [Note for this Region, we do not have weekly data for most of the waterbodies o this criterion is not used] |
| Criterion 2 | No more than 10 percent of total samples during any 30-day period exceed 4000/100 ml. [Note: used entire data set for each reach rather than 30-day period] |
|  | Primary Contact Recreation (beaches) |
| Total coliform | Samples of water from each sampling station shall have a density of total coliform organisms less than 1000 per 100 ml; provided that not more than 20 percent of the samples at any sampling station, in any 30-day period, may exceed 1000 per 100 ml, and provided further that no single sample when verified by a repeat sample taken within 48 hours shall exceed 10,000 per 100 ml. |
| Fecal coliform | The fecal coliform density based on a minimum of not less than five samples for any 30-day period, shall not exceed a geometric mean 200 per 100 ml nor shall more than 10 percent of the total samples during any 60-day period exceed 400 per 100 ml. |
| Criterion | Shellfish consumption |
| Criterion 1 | In all waters where shellfish can be harvested for human consumption, the median total coliform concentration throughout the water column for any 30-day period shall not exceed 70/100 ml, [Note for this Region, we do not have weekly data so this criterion is not used] |
| Criterion 2 | No more than ten percent of the samples collected during any 30-day period exceed 230/100 ml for a five-tube decimal dilution test or 330/100 ml when a three-tube decimal dilution test is used. |

Secondary Contact Recreation Use

Most of the waterbodies of the region are designated for non-contact recreational use.  This use includes activities where water is not normally ingested.  The assessment for this use includes many of the same factors as for primary contact recreation but to a somewhat less stringent standard for coliform bacteria.  Chemical and physical data  are assessed using guidelines in Table 2 and standards in Table 8.  In addition, field observations at stations along many of the reaches were tabulated for aesthetic factors such as significant excessive algae growth, significant trash, persistent non-natural foam and odor (table 9).  These observations were assessed using guidelines in Table 2.

Fish and Shellfish Consumption Use

Fish and shellfish consumption use is assessed based on status of fishing advisories and bioaccumulation data.  Guidelines for use of advisory data are listed in Table 2.  Bioaccumulation standards are described above under aquatic life use.  Table 7 includes shellfish bacteria limits which were not used in this assessment due to lack of resources.

Table 8.  Aesthetics/taste and odor standards

Note:  most stringent criteria is selected.  Constituents are only listed if samples used for assessment had values above detection limits.

| Constituent | Reference | Status | Criteria (ppm unless noted) | Comments |
| --- | --- | --- | --- | --- |
| Aluminum | CA Department of Health Services | Secondary MCL | 0.02 |  |
| Ammonia | McKee and Wolf, 1978 |  | 0.037 |  |
| Chloride | CA Department of Health Services | Secondary MCL | 250 |  |
| Color | CA Department of Health Services | Secondary MCL | 15 units |  |
| Copper | CA Department of Health Services | Secondary MCL | 1000 ppb |  |
| Fluoride | US Environmental Protection Agency | Secondary MCL | 2 |  |
| Foaming agents (MBAS) | CA Department of Health Services | Secondary MCL | 0.5 |  |
| Iron | CA Department of Health Services | Secondary MCL | 0.3 |  |
| Manganese | CA Department of Health Services | Secondary MCL | 0.05 |  |
| Odor | CA Department of Health Services | Secondary MCL | 3 threshold units |  |
| pH | US Environmental Protection Agency | Secondary MCL | 6.5 to 8.5 units |  |
| Silver | CA Department of Health Services | Secondary MCL | 100 ppb |  |
| Specific conductance | CA Department of Health Services | Secondary MCL | 900 mhos/cm |  |
| Sulfate | CA Department of Health Services | Secondary MCL | 250 |  |
| Total dissolved solids | CA Department of Health Services | Secondary MCL | 500 |  |
| Turbidity | CA Department of Health Services | Secondary MCL | 5 units |  |
| Organic Constituents |  |  |  |  |  |  |  |  |
| Ethylbenzene | US Environmental Protection Agency | Secondary MCL | 30 |  |
| Phenol | CA Department of Health Services | Taste and odor Action level | 5.0 |  |
| Toluene | US Environmental Protection Agency | Secondary MCL | 40 |  |
| Xylene(s) | US Environmental Protection Agency | Secondary MCL | 20 |  |

Table 9.  Observation categories used for assessing field observations of aesthetic stressors

| Stressor | Observation categories |
| --- | --- |
| Trash | None, trash observed, Significant amount of trash observed |
| Unnatural scum/flotsam/foam | None, observed |
| Algae | None, significant amount observed |
| Odors | None, presence |
| Color | Clear, light yellow to green, dark green to brown |
| Turbidity | None, slight to turbid, very turbid |
| Oil and grease | None, Oil sheen, heavy oil |

Drinking water use

Assessment of the use of waterbodies in the region for drinking water is based on concentrations of constituents that are regulated for drinking water.  In this 305(b) report, ambient or raw (untreated) surface and ground waters are assessed (Note that such water would be treated and disinfected, in accordance with requirements from the State Department of Health Services, prior to distribution for potable use).  Contaminants that are generally not source-water related (e.g., corrosion byproducts, lead or copper from distribution system, or TTHMs) are not considered.  Assessment of waterbodies for drinking water use differs from other uses in that median rather than mean of data area considered.  Table 2 lists the guidelines for assessment and Table 10 lists the standards and references.

Assessed under the "federal" drinking water use are two "state" beneficial uses, namely MUN (municipal supply) and GWR (ground water recharge).  Currently, all waterbodies in the region are designated as MUN per the 1988 Sources of Drinking Water Policy.  A large number of waterbodies, however, were footnoted in the 1994 Basin Plan as being eligible for review and possible exemption status during the Triennial Review (1994-1997).  The Regional Board staff is currently reviewing these footnoted MUN waterbodies and later this year (1996), intend to bring forward a revised MUN policy more appropriate for this region as well as criteria specific to this region for possible exemptions from the MUN designation.  Waterbodies that were designated MUN (Municipal Drinking Water Supply) with a footnote in the region's Basin Plan (1994) and did not have a groundwater recharge (GWR) use are not assessed in this 1996 305(b) assessment.  As discussed above, these waterbodies will be considered for exception from MUN in a separate Board action later this year.  After consultation with State Board and USEPA staff, Regional Board staff decided to not assess the drinking water use for those waterbodies that were footnoted MUN in the 1994 Basin Plan and do not have a GWR use.  These waterbodies may be removed from our Basin Plan within this year and, if any of them are determined to have drinking water uses and are impaired, staff will revise the 303(d) accordingly.

Table 10.  Drinking Water standards

Note:  most stringent criteria is selected.  Constituents are only listed if samples used for assessment had values above detection limits.

| Constituent | Reference | Status | Criteria | Comments |
| --- | --- | --- | --- | --- |
| Inorganic constituents (ppm unless noted) |  |  |  |  |  |  |  |  |
| Aluminum | CA Department of Health Services | Primary MCL | 1000 ppb |  |
| Gross alpha particle activity | CA Department of Health Services | Primary MCL | 15 pCi/L |  |
| Antimony | CA Department of Health Services | Primary MCL | 6 ppb |  |
| Arsenic | CA Department of Health Services | Primary MCL | 50 ppb |  |
| Bacteria, coliform | 1994 Basin Plan | groundwater | 1.1/100 ml (any seven day period) |  |
| Barium | CA Department of Health Services | Primary MCL | 1000 ppb |  |
| Gross beta particle activity | CA Department of Health Services | Primary MCL | 50 pci/L |  |
| Beryllium | CA Department of Health Services |  | 4 ppb |  |
| Boron | CA Department of Health Services | Toxicity Action Level | 1 |  |
| Cadmium | CA Department of Health Services | Primary MCL | 5 ppb |  |
| Chlorine (Cl2) | US Environmental Protection Agency | Primary MCL | 4 | Proposed |
| Chromium total | CA Department of Health Services | Primary MCL | 50 ppb |  |
| Copper | US Environmental Protection Agency | Primary MCL | 1300 ppb |  |
| Cyanide | CA Department of Health Services | Primary MCL | 0.200 |  |
| Fluoride | CA Department of Health Services | Primary MCL | * |  |
| Lead | US Environmental Protection Agency | Primary MCL | 15 ppb |  |
| Mercury | CA Department of Health Services | Primary MCL | 2 ppb |  |
| Nickel | CA Department of Health Services | Primary MCL | 100 ppb |  |
| Nitrate | CA Department of Health Services | Primary MCL | 45 |  |
| Nitrite-N | CA Department of Health Services | Primary MCL | 1 |  |
| Nitrate-N + Nitrite-N | CA Department of Health Services | Primary MCL | 10 |  |
| Selenium | CA Department of Health Services | Primary MCL | 50 ppb |  |
| Sulfate | US Environmental Protection Agency | Primary MCL | 400 | Proposed |
| Thallium | CA Department of Health Services | Primary MCL | 2 ppb |  |
| Organic Constituents (ppb unless noted) |  |  |  |  |  |  |  |  |
| Aldrin | CA Department of Health Services | Toxicity Action Level | 0.05 |  |
| Benzene | CA Department of Health Services | Primary MCL | 1 |  |
| Benzene hexachloride (a-BHC) | CA Department of Health Services | Toxicity Action Level | 0.7 |  |
| Benzene hexachloride (b-BHC) | CA Department of Health Services | Toxicity Action Level | 0.3 |  |
| Benzene hexachloride (g-BHC; Lindane) | CA Department of Health Services | Primary MCL | 0.2 |  |
| Bromodichloromethane† | CA Department of Health Services | Primary MCL | 100 | Sum of trihalomethanes |
| Bromoform† | CA Department of Health Services | Primary MCL | 100 | Sum of trihalomethanes |
| n-Butylbenzene | CA Department of Health Services | Toxicity Action Level | 45 |  |
| n-Butyl benzyl phthalate | US Environmental Protection Agency | Primary MCL | 100 | Proposed |
| Carbon tetrachloride | CA Department of Health Services | Primary MCL | 0.5 |  |
| Chlordane | CA Department of Health Services | Primary MCL | 0.1 |  |
| Chloroform† | CA Department of Health Services | Primary MCL | 100 | Sum of trihalomethanes |
| 2,4-D | CA Department of Health Services | Primary MCL | 70 |  |
| Dibromochloromethane† | CA Department of Health Services | Primary MCL | 100 | Sum of trihalomethanes |
| 3,3'-Dichlorobenzidine (DCB) | none |  |  |  |
| 1,1-Dichloroethylene (1,1-DCE) | CA Department of Health Services | Primary MCL | 6 |  |
| cis-1,2-Dichloroethylene | CA Department of Health Services | Primary MCL | 6 |  |
| trans- 1,2-Dichloroethylene | CA Department of Health Services | Primary MCL | 10 |  |
| 1,3-Dichloropropene (cis and trans) | CA Department of Health Services | Primary MCL | 0.5 |  |
| Di(2-ethylhexyl)pthalate | CA Department of Health Services | Primary MCL | 4 |  |
| Di(n-octyl) phthalate | none |  |  |  |
| Endrin | CA Department of Health Services | Primary MCL | 2 |  |
| Ethylbenzene | CA Department of Health Services | Primary MCL | 700 |  |
| Heptachlor | CA Department of Health Services | Primary MCL | 0.01 |  |
| Heptachlor epoxide | CA Department of Health Services | Primary MCL | 0.01 |  |
| Methyl ethyl ketone (MEK) | none |  |  |  |
| Methylene chloride (Dichloromethane) | CA Department of Health Services | Primary MCL | 5 |  |
| Tetrachloroethylene (PCE) | CA Department of Health Services | Primary MCL | 5 |  |
| Toluene | CA Department of Health Services | Primary MCL | 150 |  |
| 1,1,1-Trichloroethane (1,1,1-TCA) | CA Department of Health Services | Primary MCL | 200 |  |
| Trichloroethylene (TCE) | CA Department of Health Services | Primary MCL | 5 |  |
| Vinyl chloride | CA Department of Health Services | Primary MCL | 0.5 |  |
| Xylene(s) | CA Department of Health Services | Primary MCL | 1750 |  |

* Fluoride criteria is based on annual average of maximum daily air temperature:  MCL at 58.4 to 63.8 oF: 2.0 mg/L; 63.9 to 70.6 oF: 1.8 mg/L; and 70.7 to 79.2 oF:  1.6 mg/L.

† 100 ppb is total trihalomethanes (sum of bromoform, bromodichloromethane, chloroform, and dibromochloromethane)

Agriculture use

Water quality standards can vary by area and by crop.  Due to a lack of state or federal standards, assessment of water quality for agricultural use is based upon local guidelines (table 11).

Table 11.  Agriculture  standards

Note:  most stringent criteria is selected.  Constituents are only listed if samples used for assessment had values above detection limits.

| Constituent | Reference | Criteria | Comments |
| --- | --- | --- | --- |
| Aluminum | Ayers and Westcot, 1985 | 5 ppm |  |
| Arsenic | Ayers and Westcot, 1985 | 100 ppb |  |
| Boron | USEPA Quality Criteria for Water, 1986 | 0.75 ppm |  |
| Cadmium | Ayers and Westcot, 1985 | 10 ppb |  |
| Chloride | Ayers and Westcot, 1985 | 106 ppm |  |
| Chromium VI | Ayers and Westcot, 1985 | 100 ppb |  |
| Cobalt | Ayers and Westcot, 1985 | 50 ppb |  |
| Copper | Ayers and Westcot, 1985 | 200 ppb |  |
| Total dissolved solids | Bucy, 1995 | 750 ppm | Ayers and Westcot recommend 450 but locally that is not used |
| Fluoride | Ayers and Westcot, 1985 | 1 ppm |  |
| Iron | Ayers and Westcot, 1985 | 5 ppm |  |
| Lead | Ayers and Westcot, 1985 | 5000 ppb |  |
| Manganese | Ayers and Westcot, 1985 | 0.2 ppm |  |
| Nickel | Ayers and Westcot, 1985 | 200 ppb |  |
| Selenium | Ayers and Westcot, 1985 | 20 ppb |  |
| Specific conductance | Bucy, S. 1995. Fruit Growers Laboratory, Santa Paula, CA. Personal communication, November 14, 1995. | 750 umhos/cm | Ayers and Westcot recommend 700 but locally that is not used |
| Vanadium | Ayers and Westcot, 1985 | 100 ppb |  |
| Zinc | Ayers and Westcot, 1985 | 2000 ppb |  |

Criteria used for assessing lakes

Although the general guidelines for surface waters apply to lakes of the region, special consideration is given to urban lakes.  The Regional Board contracted with University of California, Riverside to assess urban lakes. Twenty three urban and non-urban lakes of the region were studied over a year period and this information and data are used in this Water Quality 305(b) Assessment.

For lakes, trophic status is assessed.  Assessment is additionally based on, where available, the following factors:  total phosphorus, chlorophyll a, secchi transparency, frequency of algal blooms, surface scum and mat, turbidity, reduction of water depth due to sediment, extent of nuisance macrophyte growth, and aesthetics.

Criteria used for assessing Nearshore, Open Bays, Estuaries, and Ocean

Due to lack of staff resources at this time, the assessment of nearshore areas, open bays, estuaries, and ocean areas is mostly limited to the review of published reports.  Fish consumption advisories and some bioaccumulation data are also used.

Criteria used for assessing Ground Water

Neither the US EPA nor State Board has established a set methodology for assessing the quality of ground waters.  Staff at the USEPA  recognize that such assessments are monumental tasks.  Accordingly, the USEPA Guidance document suggests that agencies do what is practicable during the 1996 reporting period, focussing on demand for and vulnerability of ground waters.

Regional Board staff did not compile a database on the quality of ground water, due to limited staff resources.  Nor did Regional Board staff assess the quality of ground water based upon numbers of known contaminated sites.  These data would not accurately reflect water quality, since the data (from monitoring wells) are inherently skewed toward water quality problems.  Regional Board staff also rejected the idea of assessing the quality of ground water based upon data from production wells, as such data might inherently overstate water quality.

Assessments of the quality of ground waters in the Los Angeles Region, therefore, are based upon the extent to which beneficial uses have been, or are threatened to be, impaired.  Beneficial use categories that were assessed include drinking water and industrial uses, which were lumped together (since industrial users in the Region typically require ground water that meets Title 22 standards (Table 10).  The quality of ground waters to support agricultural uses was not assessed in most areas; exceptions were made in certain areas of Ventura County, where agriculture is an important industry.

Table 12.  Assessment guidelines for groundwater basins

| Assessment classification | Guideline |
| --- | --- |
| Fully supporting | No known contamination |
| Fully supporting but threatened | Evidence of contamination, but beneficial use has not been affected. |
| Partially supporting | Contamination has significantly affected production of ground water. For example, over 10% of production capacity in a basin has been shut down, or over 10% of production in a basin requires wellhead treatment/dilution prior to beneficial use. For those basins where production capacity is not easily available (perhaps due to widespread contamination), staff evaluated trends to determine the degree to which water quality had been degraded from background. |
| Not supporting | Contamination has impaired production of ground water. For example, over 25% of production capacity in a basin has been shut down, or over 25% of production in a basin requires wellhead treatment/dilution prior to beneficial use. Again, for those basins where production capacity is not readily available (perhaps due to widespread contamination), staff evaluated trends to determine the degree to which water quality had been degraded from background. |

In order to determine impairments to ground waters, Regional Board staff gathered information on the extent to which production had been curtailed or cut back due to water quality problems.  Ground waters that were pumped in spite of contamination, and then treated at wellhead or blended to meet water quality standards, also were considered impaired.  Assessment classifications are summarized in Table 12.  Production data for the assessments were obtained from many sources, including published reports and communications with water purveyors, wholesalers, and watermasters.

The attached Data Summary Tables present the results of the Regional Board's assessments.  Ground waters in the "partially supporting" or "not supporting categories" are considered impaired with regard to respective beneficial uses; however, these ground waters are not entered on the 303(d) list, as the scope of the 303(d) list is limited to surface waters.

Ground waters in this assessment are generally considered to be single units with the exception of two basins, the Central Basin and the West Coast Basin that were divided into upper and lower, or production, aquifers.  The total area given in square miles is the same for both the upper and lower aquifers, therefore, care needs to be taken to not double count these areas when looking at total square miles of impaired groundwater basins for the Los Angeles Region.

303(d) list

Impaired surface waterbodies included in the 305(b) assessment are also listed on the region's updated 303(d) list.  Under Section 303 of the Clean Water Act,  each state must submit a list of those waters that do not, or are not expected to, attain water quality standards after application of required technology-based controls.  This list, known as the 303(d) list, serves to focus water quality efforts and resources toward the most significant water quality problems.

Waterbodies are placed on the 303(d) list if any uses were either "not" supported or "partially" supported for any beneficial use with the following exceptions:

1.  Waterbodies where the sources of elevated constituents are most likely natural (i.e., chlorides, sulfates, and boron in areas where natural levels of these constituents are high).

2.  Waterbodies for which color and/or turbidity (aesthetic constituents) for contact and non-contact recreation uses are the only elevated constituents.  We did not separate wet weather sampling from dry weather sampling and these two constituents should be sampled on a more frequent and consistent basis in order to be sure that they are problems.  They are noted, however, in the general 305(b) assessment.

3.  Waterbodies that had only trash and/or algae impairing contact or non-contact recreation.  Staff felt that these problems alone, did not make a waterbody eligible for listing given the more serious nature of other water quality problems in the Los Angeles area.  Only a few waterbodies were exempted from the 303(d) list for this reason.  The East Fork San Gabriel River, however, was included on the 303(d) list because removing the trash (and graffiti) problem from this waterbody would significantly enhance this otherwise pristine area.

The Regional Board will use a variety of approaches to address water quality problems affecting waters on the 303(d) list.  In addition to water quality controls in Waste Discharge Requirements, these approaches may include:  new watershed-based management efforts, enhanced stormwater programs for releases from municipal, industrial, and construction sources, and estimates of total maximum daily loads (TMDLs) of pollutants.  TMDLs are a way to quantify pollutants loads from point and nonpoint sources, and can be used to allocate allowable loads in order to meet water quality standards.

TMDL priorities (high, medium, or low) on the draft 303(d) list, sent to the public in December, 1995, were based on a combination of many factors, including the severity of the problems, the value of the resources, the watershed schedule, staff resources and practicality/availability of solutions.  As a result of discussions with staff from dischargers and members of the public, all TMDL priorities were downgraded to low with the exception of the targeted high priority efforts (Los Angeles River-nitrate and Malibu Creek- nutrients) already underway on two watersheds.  The priority of all future TMDLs will be discussed and evaluated by stakeholder groups under the Watershed Approach.  Staff and stakeholder resources will be key factors in determining the number of TMDLs we can undertake in the future.

As mentioned above,TMDL efforts are already underway for the Los Angeles River and Malibu Creek watersheds.  Other efforts to estimate and reduce pollutant loadings will be initiated as the Watershed Initiative in the Los Angeles Region proceeds and as Regional Board resources allow.

Additional lists in the assessment

The 305(b) report includes a number of state lists that are or have been mandated by the Clean Water Act.  As part of the assessment for 1996, the 303(d) list is updated but previous lists were not updated.  The lists in this assessment include:

§303(d)	This list is updated in this assessment and is described above.

§314:		This is a list from 1992 of publicly owned lakes that were nominated for restoration.  This list is not updated in this report.

§319:		This is a one time state-wide list that was compiled in the 1980's of nonpoint source pollution problems due to both toxic and nontoxic pollutants.  This list is not updated in this report.

Ventura River Watershed

| Reach/location (Ventura River Watershed)/Lists |  |  | Hydro Unit | Size (miles / acres) | Code | Previous WQA | WQA | Assessed Uses |  |  | Not supporting |  |  | Partially supporting |  |  | Sources |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temp | pH | DO | TDS | SC | Hard | B | Chl | SO4 | Tissue, Sediment and Toxicity Data | NH3 | N+N | Fec Col | metals | Org Chem |
| Ventura River Estuary 303(d)-low 319 |  |  | 402.10 | 0.35 mi | CA402.0000E | 10 acres impaired | N P P | Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | AMM*,TIS(DDT), EUT AMM*,COLOR, ALGAE COLOR, ALGAE TIS(DDT) |  |  | pH,TRASH TRASH |  |  | Urban runoff (m) Oil fields (s) Municipal Effluent (m) Transient encampments (s) Natural sources (m) Recreational use (s) |  |  |
| 56 10-23 16 ± 4 | 53 7.2-9.6 8.3 ± 0.4 | 52 5.4-22 8.9 ± 2.7 | 1 29900 | no data | no data | no data | no data | no data | Tissue('93): DDT(MTRLs) 3 Sed chem: low pesticide levels 1 Sed tox: good survival rates 2 Water tox: 100% normal 2 | 4 ND-1.7 0.8 ± 0.7 | 5 0.5-2.5 1.6 ± 0.7 | 3 220-1600 | 1-2 Ba 90 | H2O(1): All ND Sed(1): All ND |
| Ventura River R1 (Estuary to Main St) and R2 (Main St. to Weldon Cyn) 303(d)-low 319 |  |  | 402.10 | 0.18 and 4.64 | CA402.0010R and CA402.0020R | 6 impaired | N FT N N N | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,CHL*,B* TDS*,SC*,CHL*,SO4*,AMM CHL*,AMM,DO AMM,ALGAE, COLOR ALGAE, COLOR |  |  | TIS(Cu,Se,Ag,Zn) Turb Turb |  |  | Urban runoff (m) Oil fields (s) Municipal Effluent (m) Transient encampments (s) Natural sources (m) Agriculture (s) Recreational use (s) Industrial sources (s) Spills (s) |  |  |
| 262 9-26 17 ± 4 | 268 7.1-8.8 7.9 ± 0.3 | 258 2.0-14.8 7.6 ± 2.6 | 13 660-1480 1067 ± 338 | 7 927-1800 1396 ± 325 | 9 377-624 523 ± 80 | 8 0.3-1.3 0.8 ± 0.4 | 10 24-304 202 ± 104 | 10 248-430 358 ± 58 | Tissue('90): no organic chemicals at elevated levels 3 Tissue('91): Zn(EDL85) 3 Tissue('93): Cu(EDL85), Se(EDL85), Ag(EDL85), Zn(EDL85) 3 | 489 ND-5.2 1.4 ± 0.8 | 13 1.6-27.0 9.1 ± 8.4 | 1 130 | 7 Ba 1700 Cd 16 Cr 96 Cu 191 Pb 140 Zn 893 | H20(9) i,p,q,r,u,v,z,be,bf Sed(1) ND |
| Canada del Diablo |  |  | 402.10 | 9.5 | CA402.0030R |  |  |  |  |  | Unassessed |  |  |  |  |  | Spills Oil field operations |  |  |
| Canada Larga |  |  | 402.10 | 8.01 | CA402.0080R | 5 unknown | N N F N N | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,CHL*,B* SO4*,B* TDS*,SC*,SO4* ALGAE ALGAE TDS*,SO4*,B* |  |  | TRASH TRASH |  |  | Natural sources (m) Oil field operations (s) Agriculture (m) Grazing (m) |  |  |
| 5 10-29 20 ± 8 | 8 6.9-8.9 8.1 ± 0.5 | no data | 8 84-2920 2225 ± 861 | 7 147-3500 2545 ± 1010 | 4 794-1134 999 ± 136 | 7 0.1-1.7 1.2 ± 0.5 | 8 8-216 138 ± 67 | 8 8-1560 1000 ± 593 | no data | 1 ND | 7 0-2.6 0.8# | no data | 1 Ba 40 Se 9 | no data |
| Ventura River R3 (Weldon Cyn to confl with Coyote Creek [Casitas Vista/Santa Ana Rd]) 303(d)-low 319 |  |  | 402.10 | 0.78 | CA402.0160R | 6 impaired | N FT N P F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SC* TDS*,SC*,SO4*AMM Water diversion and pumping AMM |  |  | pH |  |  | Natural sources (m) Urban runoff (m) Diversions/flow alteration (m) Agriculture (s) Recreational use (s) Grazing (s) |  |  |
| 130 9-23 16 ± 4 | 109 7.2-8.9 8.2 ± 0.3 | 107-3.3-14.4 10.4 ± 1.7 | 9 575-812 689 ± 72 | 9 844-1098 963 ± 93 | 8 359-510 449 ± 52 | 2 0.2-0.3 0.26 ± 0.06 | 8 15-62 44 ± 13 | 9 239-268 253 ± 9 | no data | 478 0.1-4.5 0.4 ± 0.3 | 2 1.2-1.4 1.3 ± 0.09 | no data | no data | no data |
| Weldon Cyn |  |  | 402.10 | 6.3 | CA402.0170R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Ventura River R4 (Coyote Creek to Camino Cielo Rd) 303(d)-low |  |  | 402.20 | 14.94 | CA402.0180R | 9 good | N FT N F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SC TDS,SC Water diversion and pumping |  |  |  |  |  | Natural sources (m) Urban runoff (m) Diversions/flow alteration (m) Agriculture (s) Recreational use (s) Groundwater withdrawal (m) |  |  |
| 85 13-28 21 ± 3 | 41 7.3-8.5 8.2 ± 0.2 | no data | 41 540-964 669 ± 87 | 41 720-1100 950 ± 81 | 1 338 | 41 0.2-0.6 0.4 ± 0.1 | 19 8-56 41 ± 13 | 19 215-291 247 ± 20 | no data Robles Diversion by-pass flows 13 | 2 0.2-0.3 | 19 0.09-2.3 1.2 ± 0.6 | no data | 1 Ba 62 | 1 ND |
| Coyote Creek R1 (confl Ventura to Lake Casitas Dam) |  |  | 402.20 | 2.93 | CA402.0190R | 3 good | FT F F F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,SO4* TDS*,CHL*,SO4* |  |  |  |  |  | Natural sources (m) Recreational use (s) Urban runoff (s) Flow alterations (m) Agriculture (s) Animal operations (s) |  |  |
| 3 10-22 17 ± 5 | 5 7.4-8.5 7.7 ± 0.4 | no data | 5 410-1320 987 ± 307 | 4 595-1460 1193 ± 348 | 3 305-828 603 ± 220 | 4 ND-0.7 0.5 ± 0.3 | 5 17-126 76 ± 35 | 5 64-445 328 ± 139 | no data | 2 ND-0.4 0.2 ± 0.2 | 5 0.5-0.9 0.7 ± 0.2 | no data | 1 Ba 110 Hg 0.4 | 1 ND |
| Lake Casitas 303(d) low 314 319 |  |  | 402.20 | 2702 ac | CA402.0220L | 2720 interm | F F FT F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  |  |  |  | TIS(As): Fully but threatened |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) |  |  |
| 113 10-26 18 ± 5 | 113 7.2-8.5 7.6 ± 0.4 | 113 0.2-10.9 5.9 ± 3.5 | 42 282-479 388 ± 35 | 42 557-608 591 ± 15 | no data | no data | 42 146-190 169 ± 11 | 47 165-261 219 ± 24 | Tissue ('92): As(EDL85) 3 | 42 ND | 42 ND-0.27 0.1# | no data | no data | 24+ a,m,s,v,w,y,ae,aj,al,ao,ap,b,av,au,ay,ba,bo |
| Coyote Creek R2 (above lake) |  |  | 402.20 | 7.68 | CA402.0290R |  |  |  |  |  | Unassessed |  |  |  |  |  | Grazing Groundwater pumping |  |  |
| Santa Ana Creek † |  |  | 402.20 | 3.68 | CA402.0340R | 10 good | N FT F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SC* TDS* |  |  |  |  |  | Natural sources (m) Agriculture (s) Grazing (s) Diversions/flow alteration (s) |  |  |
| 3 12-19 14 ± 4 | 4 7.9-8.5 8.2 ± 0.3 | no data | 4 420-1078 682 ± 242 | 4 630-1327 894 ± 260 | 1 430 | 4 ND-0.7 0.3 ± 0.3 | 4 9-124 41# | 4 129-363 199 ± 95 | no data | 1 ND | 1 0.2 | no data | no data | no data |
| San Antonio Creek R1 (confl. Ventura to Ojai Valley Country Club) and San Antonio Creek R2 (abv Ojai Valley Country Club) |  |  | 402.20 402.32 | 5.42 and 9.02 | CA402.0410R and CA402.0510R | 16 good | N FT F P P | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,SC TDS,SC,SO4 TDS,CHL,SO4 |  |  | ALGAE, Turb ALGAE, Turb |  |  | Natural sources (m) Urban runoff (m) Agriculture (m) Grazing (s) Diversions/flow alteration (s) |  |  |
| 23 12-26 19 ± 4 | 27 7.7-8.4 8.1 ± 0.2 | no data | 27 652-1000 847 ± 67 | 26 973-1560 1227 ± 147 | 3 455-529 485 ± 32 | 26 nd-0.5 0.3 ± 0.1 | 8 36-120 67 ± 29 | 8 220-328 309 ± 17 | no data | 2 ND-0.3 | 8 1.2-5.9 3.2 ± 1.4 | no data | 1 Ba 70 Se 6 | 1 ND |
| Lion Canyon R1 (confl SA to Reservoir) and Lion Canyon R2 (abv reservoir) |  |  | 402.20 402.31 | 4.65 and 4.84 | CA402.0430R and CA402.0470R | 9 good | N F P P | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC* TDS*,SC*,SO4* COLOR COLOR TDS*,SO4* |  |  | Turb Turb |  |  | Natural sources (m) Grazing (s) Agriculture (s) Urban runoff (s) |  |  |
| 2 12-22 | 4 7.4-8.2 7.9 ± 0.3 | no data | 4 706-974 866 ± 101 | 3 1060-1270 1180 ± 88 | 3 519-551 538 ± 14 | 3 0.2-0.5 0.4 ± 0.1 | 4 45-80 59 ± 13 | 4 200-338 276 ± 53 | no data | 1 0.8 | 5 ND-5.3 2.3 ± 2.1 | no data | 1 Ba 60 Se 16 | 1 ND |
| Stewart Cyn |  |  | 402.32 | 2.85 | CA402.0530R |  |  |  |  | Unassessed |  |  |  |  |  |
| Fox Cyn |  |  | 402.32 | 9.3 | CA402.0540R |  | N FT F U U | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,CHL* TDS*,SC*,SO4* TDS*,CHL* |  |  |  |  |  | Natural sources (m) Urban runoff (m) |  |  |
| 1 12 | 3 6.8-8.5 7.9 ± 0.8 | no data | 3 88-920 629 ± 383 | 3 134-1450 981 ± 600 | 2 623-633 | 3 ND-0.1 0.07 ± 0.04 | 3 7-116 79 ± 51 | 3 7-299 198 ± 135 | no data | no data | 3 0.2-5.9 3.6 ± 2.4 | no data | 1-2 ND | no data |
| Ventura River R5 (Camino Cielo Rd to Jct Matilija) |  |  | 402.20 | 0.23 | CA402.0790R | 9 good | N FT U P F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | B* TDS*,AMM* AMM* CHL* |  |  |  |  |  | Natural sources (m) Mine drainage (s) Recreational use (s) Agriculture (s) Grazing (s) |  |  |
| 3 11-19 16 ± 4 | 4 7.8-8.3 8.0 ± 0.2 | no data | 5 534-751 629 ± 81 | 4 780-849 819 ± 26 | 5 355-433 391 ± 34 | 5 ND-1.2 0.5# | 5 9-150 56 ± 52 | 5 205-275 246 ± 24 | no data | 3 ND-0.7 0.3# | 5 ND | no data | 2-4 Ba 181 Cd 1 | 3 ND |
| Matilija Creek R1 (jct with north fork to reservoir) and R2 (abv reservoir) 303(d)-low |  |  | 402.20 402.20 | 1.6 and 16.8 | CA402.0800R and CA402.0820R | 7 good | N FT N F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SC,B TDS Fish barrier B |  |  | TDS,CHL |  |  | Natural sources (m) Diversion/flow alteration (m) Agriculture (s) Grazing (s) Recreational use (s) |  |  |
| 25 9-26 16 ± 5 | 29 7.7-8.6 8.1 ± 0.2 | 21 5.6-13.2 9.5 ± 1.5 | 29 356-776 607 ± 76 | 28 545-2015 939 ± 317 | 19 184-488 366 ± 60 | 27 ND-1.8 0.7 ± 0.5 | 29 4-83 36 ± 24 | 29 83-294 240 ± 37 | Matilija Dam removal issues 13 | 1 0.6 | 27 ND-0.7 0.1# | no data | 1 Ba 90 | 1 ND |
| Matilija Reservoir 303(d)-low 319 |  |  | 402.20 | 198 acres | CA402.0810L | 124 interm | N FT N F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SC,B TDS Fish barrier B |  |  | TDS,CHL |  |  | Natural sources (m) Flow modification (m) Agriculture (s) Recreational use (s) Grazing (s) |  |  |
| Matilija Creek Upper North Fork |  |  | 402.20 | 7.02 | CA402.0840R | 9 unknown |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Matilija Creek North Fork and Bear Creek |  |  | 402.20 402.20 | 7.71 and 7.75 | CA402.0860R and CA402.0870R | 5 good | N FT U F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SC* TDS*,SO4* |  |  |  |  |  | Natural sources (m) Recreational use (s) Grazing (s) |  |  |
| 3 12-20 16 ± 3 | 8 7.8-8.4 8.1 ± 0.2 | no data | 8 530-818 639 ± 100 | 7 750-1189 896 ± 135 | 4 400-457 415 ± 25 | 7 ND-1 0.4# | 8 6-46 20 ± 15 | 8 215-319 260 ± 34 | no data | no data | 7 0-1.4 0.3# | no data | 1 Ba 50 | no data |

Santa Clara River Watershed

| Reach/location (Santa Clara River Watershed) |  |  | Hydro Unit | Size | Code | Previous WQA | WQA | Assessed Uses |  |  | Not supporting |  |  | Partially supporting |  |  | Potential Sources |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temp | pH | DO | TDS | SC | Hard | B | Chl | SO4 | Tissue, Sediment and Toxicity Data | NH3 | N+N | Fec Col | metals | VOCs |
| Santa Clara River Estuary 303(d)-low |  |  | 403.11 | 2.07 | CA403.0000E | 60 ac unknown | F N U | Aquatic life Recreation: cont. Recreation: non-c |  |  | COLIFORM |  |  |  |  |  | Urban runoff (m) Agricultural (s) Oil spills (s) Municipal effluent (m) Transient encampments (m) Bird droppings (m) |  |  |
|  |  |  |  |  |  |  |  |  | Sed tox: good survival rates 2 Sed chem: low pesticide levels 1 Wat tox: 100% normal 2 |  |  | 78-93% exceedences |  |  |
| Santa Clara River R1 (Estuary to Hwy 101 bridge) and R2 (Hwy 101 bridge to Freeman Diversion) |  |  | 403.11 and 403.11 | 5.79 and 8.96 | CA403.0020R and CA403.0030R | 79 interm | N N F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC* SO4* TDS*,SO4* (Exotic vegetation) |  |  |  |  |  | Natural sources (m) Diversion/flow alteration (m) Agriculture (m) Urban runoff (m) Resource extract'n (m) Municipal effluent (m) |  |  |
| 19 9-28 16 ± 6 | 6 7.7-8.3 8.0 ± 0.2 | no data | 5 710-1363 977 ± 226 | 5 790-1700 1344 ± 317 | 2 420-750 585 ± 165 | 5 0.1-0.7 0.5 ± 0.2 | 5 24-74 49 ± 17 | 5 290-620 434 ± 114 | no data | no data | 2 0.8-0.9 0.9 ± 0.1 | no data | 1-2 Ba 85 Cd 2 Se 6 | H20(4) ND Sed (1) ND |
| Brown Barranca/Long Canyon † |  |  | 403.11 | 3.79 | CA403.0080R |  | N N F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,CHL* SO4* TDS*,SC*,SO4* TDS,SO4* |  |  |  |  |  | Natural sources (m) Agriculture (m) Urban runoff (m) |  |  |
| 3 15-17 16 ± 1 | 6 7.4-8.4 7.8 ± 0.3 | no data | 6 572-5772 2387 ± 1657 | 5 826-3480 2164 ± 883 | 5 213-1910 833 ± 604 | 5 0.14-1.4 0.6 ± 0.5 | 6 32-255 118 ± 69 | 6 101-3300 1112 ± 1081 | no data | no data | 6 2.5-9.9 4.8 ± 2.7 | no data | 1 Se 5 | H2O(1) af Sed(2) sb,sc,sd Sed |
| Ellsworth Barranca/Aliso Cyn † |  |  | 403.21 | 7.1 | CA403.0110R | 9 unknown | N N F U U | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,CHL*,B* SO4* TDS*,SC*,SO4* TDS,SO4*, CHL*,B* |  |  |  |  |  | Natural sources (m) Urban runoff (m) Agriculture (s) |  |  |
| 6 2-26 13 ± 7 | no data | no data | 10 580-2571 1665 ± 749 | 9 755-3712 2198 ± 1051 | 9 243-1165 614 ± 304 | 9 0.3-2.2 1.0 ± 0.7 | 11 39-580 221 ± 188 | 10 231-1039 692 ± 311 | no data | no data | no data | no data | no data | H2O (1) ND Sed(2) sb,sc,sd |
| Wheeler Cyn/Todd Barranca † |  |  | 403.21 | 4.17 | CA403.0140R | 6 interm | N N F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,CHL*,B* SO4* TDS*,SO4* TDS,SO4*, CHL* |  |  |  |  |  | Natural source (m) Urban runoff (m) Petroleum operations (s) Agriculture (m) |  |  |
| 7 3-31 19 ± 9 | 12 7.3-8.1 7.7 ± 0.2 | no data | 13 599-2990 1991 ± 613 | 11 876-3382 2311 ± 671 | 11 237-1358 908 ± 297 | 12 0.08-1.7 0.8 ± 0.4 | 14 40-215 144 ± 51 | 12 238-1412 875 ± 309 | no data | no data | 12 0.8-25.8 5.6# | no data | 0-1 Zn 70 | H2O (1 ) ae,af Sed (2) sc |
| Santa Clara River R3 (Diversion to abv SP Creek/blw Timber Cyn) |  |  | 403.21 | 13.24 | CA403.0160R | 79 interm | N N FT P F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,B* SO4* TDS*,SC*,SO4*,AMM* (Exotic vegetation) AMM* SO4*,CHL* |  |  | CHL* TOX: Fully but threatened |  |  | Natural sources (m) Urban runoff (m) Resource extraction (m) Municipal effluent (s) Industrial point source (s) |  |  |
| 20 13-28 19 ± 3 | 3 8.2-8.3 8.2 ± 0.05 | no data | 5 832-1390 1048 ± 241 | 23 1120-2150 1484 ± 304 | 5 410-790 586 ± 131 | 5 0.6-0.9 0.7 ± 0.2 | 27 34-110 79 ± 21 | 26 320-790 585 ± 162 | Tissue('91): no organic chemicals at elevated levels 3 Tissue('92): no organic chemicals at elevated levels 3 Wat Bio Toxicity: reproduction affected, low level chronic toxicity found 5 | 5 0.02-0.45 0.25 ± 0.19 | 5 1.6-3.2 2.5 ± 0.7 | no data | no data | no data |
| Fagan Cyn † |  |  | 403.21 | 4.66 | CA403.0260R |  | N FT F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC* TDS*,Fe*,Mn* |  |  |  |  |  | Natural sources (m) Agricultural (s) Urban runoff (m) |  |  |
| 2 11 | 5 6.8-8.4 7.3 ± 0.6 | no data | 4 132-1070 625 ± 354 | 5 199-1390 731 ± 471 | 1 257 | 4 0.1-0.8 0.4 ± 0.3 | 5 9-79 38 ± 26 | 5 ND-518 208 ± 192 | no data | no data | 5 1.1-3.6 2.1 ± 0.9 | no data | no data | no data |
| Santa Paula Creek R1 (confluence with SC to Diversion Dam) |  |  | 403.21 | 2.0 | CA403.0280R | 14 good | N FT F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,SC TDS,SC |  |  | CHL |  |  | Natural sources (m) Resource extraction (m) Urban runoff (s) Agriculture (s) |  |  |
| 22 8-23 15 ± 4 | 21 7.8-8.6 8.1 ± 0.2 | 17 7.1-14.0 10.2 ± 1.7 | 21 410-912 675 ± 153 | 21 540-1414 977 ± 257 | 19 154-414 315 ± 61 | 4 0.1-0.6 0.3 ± 0.2 | 21 11-158 57 ± 40 | 21 86-354 226 ± 60 | no data | 2 0.03-0.05 | 5 0-0.5 0.2 ± 0.2 | no data | no data | 1 ND |
| Santa Paula Creek R2 (Abv Diversion Dam) † |  |  | 403.21 | 12.41 | CA403.0310R | 14 good | F FT FT N F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | Turb* Turb*,oil |  |  | TOX: Fully but threatened COLOR |  |  | Natural sources (m) Urban runoff (s) Agriculture (s) |  |  |
| 10 10-22 15 ± 4 | 17 7.2-8.4 7.9 ± 0.4 | 1 10.6 | 15 188-791 417 ± 145 | 16 214-1000 576 ± 145 | 13 110-550 267 ± 113 | 16 ND-0.2 0.05# | 16 4-34 9 ± 7 | 16 36-300 153 ± 65 | no data | no data | 9 0-1.1 0.4 ± 0.3 | no data | 1 Ba 59 | 2 be |
| Sisar Creek R1 and R2† |  |  | 403.21 403.22 | 1.22 and 6.65 | CA403.0340R and CA403.0350R | 2 impaired and 5 good | N FT F N F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SC* Turb*, TDS* Turb* SO4* |  |  |  |  |  | Natural sources (m) Urban runoff (s) Agriculture (s) |  |  |
| 8 9-16 12 ± 2 | 12 6.9-8.2 7.9 ± 0.4 | 1 8.2 | 12 326-936 550 ± 198 | 11 464-1275 845 ± 285 | 11 196-527 379 ± 122 | 12 ND-0.5 0.1# | 12 3-110 32 ± 29 | 12 115-307 205 ± 68 | no data | no data | 9 0-1.5 0.6 ± 0.5 | no data | 1 Ba 60 Cd 2 | 1 ND |
| Santa Clara River R4 (below Timber Cyn to abv Grimes Cyn) |  |  | 403.31 | 9.83 | CA403.0430R | 79 interm | N N F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,B* SO4 TDS*,SC (Exotic vegetation) TDS*,SO4 |  |  | CHL |  |  | Natural sources (m) Oil spills (s) Industrial point sources (s) POTW effluent (m) Agriculture (m) Urban runoff (m) |  |  |
| 21 7-29 17 ± 6 | 17 7.8-8.4 8.0 ± 0.2 | 16 7.0-10.7 9.1 ± 1.1 | 17 558-1615 1179 ± 309 | 22 737-1950 1461 ± 351 | 11 316-776 684 ± 126 | 16 0.1-1.1 0.7 ± 0.3 | 21 25-110 65 ± 22 | 21 232-741 530 ± 176 | no data | no data | 17 0.6-3.5 2.2 ± 1.0 | no data | no data | 3 ND |
| Timber Cyn |  |  | 403.31 | 5.12 | CA403.0440R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |
| Balcom Cyn |  |  | 403.31 | 2.88 | CA403.0480R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |
| Sespe Creek R1 (confl. w/SC to 500' d/s Little Sespe) |  |  | 403.31 | 7.6 | CA403.0540R | 45 good | N N FT P P | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SC*,F*,B* B* TDS*,SO4* COLOR B* |  |  | TOX: Fully but threatened ALGAE ALGAE |  |  | Natural sources (m) Agriculture (m) Urban runoff (m) |  |  |
| 4 18-25 23 ± 3 | 6 8.0-8.6 8.2 ± 0.2 | 1 10.8 | 5 518-759 648 ± 101 | 5 720-1000 801 ± 105 | 3 345-390 365 ± 19 | 4 ND-2.1 1.4 ± 0.8 | 5 10-88 51 ± 33 | 5 220-325 263 ± 40 | Wat Bio Toxicity: reproduction affected, low level chronic toxicity found 5 | no data | 4 ND-1.3 0.4# | no data | 1 Ba 78 Cd 2 | 1 ND |
| Sespe Creek R2 (abv 500' d/s Little Sespe) |  |  | 403.32 | 33.80 | CA403.0620R | 45 good | N FT F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,SC,B TDS,SC,SO4 TDS,CHL,SO4,B |  |  |  |  |  | Natural sources (h) |  |  |
| 22 8-27 17 ± 6 | 26 7.8-8.8 8.2 ± 0.2 | 19 8.8-13.1 10.5 ± 1.2 | 26 494-1021 761 ± 136 | 26 674-1468 1078 ± 218 | 14 320-463 389 ± 47 | 22 0.1-4.2 1.4 ± 1.3 | 26 12-198 83 ± 64 | 26 190-432 304 ± 52 | no data | 2 0.01-0.05 0.03 ± 0.02 | 18 ND-0.4 0.1# | no data | Ba(2) Ba 51 | 1 ND |
| Pole Cyn † |  |  | 403.31 | 5.5 | CA403.0940R | 7 interm | N N F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC* SO4* TDS*,SC*,SO4* TDS*,SO4* |  |  |  |  |  | Natural sources (m) Industrial point source (s) Agriculture (m) |  |  |
| 4 10-13 11 ± 1 | 8 7.5-8.3 7.9 ± 0.2 | no data | 8 860-1794 1372 ± 277 | 8 1027-2035 1643 ± 292 | 5 559-1018 815 ± 162 | 8 0.1-0.4 0.3 ± 0.1 | 9 9-32 19 ± 6 | 8 452-951 717 ± 149 | no data | no data | 5 0.3-1.3 0.7 ± 0.4 | no data | 1 Ba 40 Se 16 | 2 ND |
| Santa Clara River R5 (abv Grimes Cyn to Propane Rd) and R6 (Propane Rd to Blue Cut) |  |  | 403.31 403.41 | 2.10 and 11.18 | CA403.0930R and CA403.0990R | 79 interm | N FT FT N F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC* TDS*,SC*,SO4*,AMM* (Exotic vegetation) AMM*, Turb,COLOR |  |  | TOX and Sed Chem(DDT): Both fully but threatened |  |  | Natural sources (m) Agriculture (m) Urban runoff (s) |  |  |
| 15 17-29 22 ± 3 | 11 7.5-8.6 8.1 ± 0.3 | no data | 8 653-1560 974 ± 267 | 19 966-1690 1162 ± 202 | 6 310-540 436 ± 73 | 7 0.5-0.8 0.6 ± 0.1 | 14 15-120 91 ± 27 | 14 64-690 312 ± 131 | Wat Bio Toxicity: reproduction affected, low level chronic toxicity found 5 Sed chem: DDE(300ppb), DDT(150 ppb) [RWQCB data] | 3 0.11-0.8 0.5 ± 0.3 | 9 0.6-22.6 5.5# | no data | 2-4 Ba 133 Cu 10 | H2O(5) ND Sed(2) sc,sd,sf |
| Hopper Cyn † |  |  | 403.41 | 13.65 | CA403.1010R | 10 interm | N N F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,SC SO4 Turb*,TDS,SC,SO4 SO4 TDS,SO4 |  |  | CHL,B CHL |  |  | Natural sources (m) Agriculture (m) Urban runoff (s) Municipal effluent (m) |  |  |
| 33 9-31 18 ± 7 | 65 6.9-9.5 7.9 ± 0.4 | 1 9.5 | 64 274-8740 1590 ± 1407 | 66 370-8600 1918 ± 1418 | 54 153-1370 563 ± 282 | 64 ND-8.3 0.5# | 68 2-543 58# | 66 6-1440 576 ± 327 | no data | no data | 43 ND-2.9 0.6# | no data | 1 Ba 40 Se 10 | 1 ND |
| Torrey Cyn † |  |  | 403.41 | 1.7 | CA403.1060R |  | N N F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,B* SO4* TDS*,SC*,SO4* TDS*,SO4* |  |  |  |  |  | Natural sources (m) Agriculture (m) |  |  |
| 2 12-14 | 4 7.1-8.2 7.6 ± 0.5 | no data | 4 2056-5160 3460 ± 1151 | 4 2203-5260 3421 ± 1185 | 3 1194-2810 1993 ± 660 | 4 0.4-1.8 1.0 ± 0.6 | 4 1-103 42 ± 40 | 4 1222-3020 2020 ± 645 | no data | no data | 3 1.2-17.7 7.0# | no data | 0-1 ND | no data |
| Real Wash/Canyon/Drain |  |  | 403.41 | 2.42 | CA403.1070R |  | N N F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC* SO4* TDS*,SC*,SO4* TDS*,SO4* |  |  |  |  |  | Natural sources (m) Urban runoff (s) |  |  |
| 2 10-11 | 3 7.1-8.5 7.6 ± 0.6 | no data | 3 165-4680 2259 ± 1858 | 3 233-4910 2500 ± 1912 | 2 111-2336 | 3 0.2-1.6 0.7 ± 0.6 | 3 3-44 27 ± 17 | 3 55-2970 1450 ± 1193 | no data | no data | 3 ND-7.2 3.0# | no data | 1 Se 12 | no data |
| Smith Cyn |  |  | 403.41 | 2.27 | CA403.1090R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| 1 12 | 2 8.0-8.1 | no data | 2 3510-4973 | 2 3800-6120 | 1 2108 | 2 1.4-13.3 | 2 119-490 | 2 1210-2110 | no data | no data | 1 0.36 | no data | 1 ND (Cu and Zn only metals scanned) | 1 ND |
| Piru Creek R1 (confl. Santa Clara to abv gaging station below Santa Felica Dam) |  |  | 403.41 | 6.08 | CA403.1120R | 55 good | N FT FT N N | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC* TDS*,SC*,SO4* ODOR ODOR |  |  | TOX: Fully but threatened Turb |  |  | Imported water (m) Natural sources (m) Agriculture (m) Urban runoff (s) |  |  |
| 6 16-23 19 ± 2 | 7 7.7-8.6 8.1 ± 0.3 | 1 11.7 | 6 590-1057 794 ± 151 | 8 600-1400 999 ± 208 | 4 305-558 417 ± 91 | 6 0.3-0.8 0.5 ± 0.1 | 7 47-114 76 ± 23 | 7 210-442 316 ± 68 | Wat Bio Toxicity: reproduction affected, low level chronic toxicity found 5 | no data | 5 ND-0.5 0.2 ± 0.1 | no data | 0-3 Ba 54 Cd 22 | 1 ND |
| Piru Creek R2 (gage to Santa Fe. Dam) |  |  | 403.41 | STILL NEED | CA403.1180R | 55 good | N FT F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SC* TDS*,SC*,SO4* CHL* |  |  |  |  |  | Natural sources (m) Imported water (m) |  |  |
| 19 7-27 15 ± 4 | 16 7.9-8.9 8.3 ± 0.3 | 16 7.8-14.7 11.0 ± 2.0 | 18 530-965 720 ± 116 | 19 822-2205 1098 ± 294 | 13 273-434 371 ± 51 | 17 0.14-2.4 0.7 ± 0.5 | 18 61-113 89 ± 16 | 18 200-370 273 ± 42 | no data | 2 0.02-0.04 0.03 ± 0.01 | 17 ND-0.7 0.3 ± 0.2 | no data | 2 Ba 34 Ba only metal scanned. | no data |
| Lake Piru (lower and upper areas) |  |  | 403.41 403.42 | 1439 ac | CA403.1190L and CA403.1260L | 1240 good | N FT N F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | SC TDS DO |  |  |  |  |  | Natural sources (m) Aerial deposition (s) Imported water (m) |  |  |
| 105 10-26 18 ± 5 | 105 7.1-8.2 7.8 ± 0.3 | 105 0.3-10.1 6.0 ± 3.3 | 47 785-940 878 ± 43 | 47 785-940 878 ± 43 | no data | no data | 47 40-74 58 ± 9 | 42 80-159 112 ± 29 | no data | 47 ND-0.1 0.007# | 105 0.3-10.1 6.0 ±3.3 | no data | no data | no data |
| Piru Creek R3 (Lake Piru to Pyramid Lake) |  |  | 403.42 | 8.70 | CA403.1290R | 55 good | F F F P F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | CHL* |  |  | ALGAE |  |  | Imported water (m) Natural sources (m) Recreational uses (s) |  |  |
| 2 11-21 16 ± 5 | 6 6.7-8.7 7.5 ± 0.8 | 6 10.3-13.0 11.3 ± 0.9 | 6 346-537 407 ± 66 | 6 622-800 668 ± 65 | 6 136-248 165 ± 39 | 6 0.2-0.6 0.3 ± 0.1 | 6 100-130 114 ± 10 | 6 59-134 79 ± 26 | no data | no data | 6 ND-0.9 0.6 ± 0.3 | 6 100-130 | no data | no data |
| Pyramid Lake |  |  | 403.42 | 1928 ac | CA403.1430L | 1360 good | F F F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  |  |  |  |  |  |  | Natural sources (m) Aerial deposition (s) Imported water (m) |  |  |
| 90 6-24 17 ± 6 | 90 7.2-8.8 8.0 ± 0.5 | 90 4.0-13.1 8.6 ± 2.6 | 42 662-718 681 ± 18 | 42 662-718 681 ± 18 | no data | no data | 42 56-111 90 ± 18 | 43 89-156 122 ± 17 | no data | 42 ND-0.2 0.01# | 90 4.0-13.1 8.6 ± 2.6 | no data | no data | no data |
| Canada de los Alamos † |  |  | 403.43 | 7.84 | CA403.1480R | 6 unknown | F FT F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | Turb* CHL* |  |  |  |  |  | Natural sources (m) Grazing (s) |  |  |
| 12 6-27 16 ± 7 | 10 7.4-9.2 8.3 ± 0.3 | 11 7.8-11.8 9.3 ± 1.2 | 13 162-1262 567 ± 392 | 13 305-1761 872 ± 533 | 13 83-480 217 ± 122 | 13 0.13-2.5 0.8 ± 0.7 | 13 16-184 78 ± 40 | 13 22-529 183 ± 180 | no data | no data | 12 0.07-0.9 0.3 ± 0.2 | no data | no data | no data |
| Gorman Creek |  |  | 403.43 | 12.21 | CA403.1510R | 12 interm |  |  |  |  | Unassessed |  |  |  |  |  |  |  |
| Piru Creek Reach 4 (abv Pyramid Lake) † |  |  | 403.42 | 34.23 | CA403.1580R |  | N N F N F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SC*,B* B* Turb*,TDS*,SC*,SO4* Turb*,SO4* B* |  |  |  |  |  | Natural sources (m) Grazing (s) Agriculture (s) Recreation (s) |  |  |
| 13 6-28 17 ± 7 | 8 8.2-9.2 8.6 ± 0.3 | 10 7.9-11.8 9.6 ± 1.1 | 15 404-1195 754 ± 249 | 15 619-1381 960 ± 236 | 13 267-800 405 ± 143 | 15 0.2-3.7 1.5 ± 0.96 | 15 5-26 11 ± 6 | 15 182-672 386 ± 156 | no data | no data | 9 ND-1.3 0.2# | no data | no data | 1 ND |
| Lockwood Creek R1 and R2 † |  |  | 403.42 403.44 | 4.16 and 7.68 | CA403.1710R and CA403.1720R |  | N N F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SC*,B* B* TDS*,SO4* B* |  |  |  |  |  | Natural sources (m) Grazing (s) Agriculture (s) Recreation (s) |  |  |
| 4 15-18 16 ± 1 | 7 7.2-8.4 8.0 ± 0.4 | 1 10.6 | 7 420-7135 1526# | 7 594-901 769 ± 103 | 3 311-390 358 ± 34 | 7 1.6-9.1 5.3 ± 2.9 | 7 6-11 9 ± 2 | 7 175-273 240 ± 38 | no data | no data | 4 0.4-2.7 1.3 ± 0.9 | no data | 1 As 7 Ba 50 | no data |
| Seymour Creek † |  |  | 403.44 | 7.2 | CA403.1730R |  | N N F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | B* B* B* |  |  |  |  |  | Natural sources (m) Grazing (s) Agriculture (s) Recreation (s) |  |  |
| 1 16 | 3 7.8-8.4 8.2 ± 0.3 | no data | 3 320-545 435 ± 92 | 3 430-780 623 ± 145 | no data | 3 0.5-3.7 1.8 ± 1.4 | 3 5-10 7 ± 2 | 3 59-192 123 ± 54 | no data | no data | 1 ND | no data | no data | no data |
| Armagosa Creek † |  |  | 403.44 | 4.92 | CA403.1740R |  | N N F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | B* B* B* |  |  |  |  |  | Natural sources (m) Grazing (s) Agriculture (s) Recreation (s) |  |  |
| 1 11 | 4 7.5-8.4 8.0 ± 0.4 | no data | 4 373-612 454 ± 93 | 4 530-817 628 ± 111 | no data | 4 5.1-13.4 8.5 ± 3.0 | 4 4-6 5.5 ± 1 | 4 156-258 186 ± 42 | no data | no data | 3 ND-0.4 0.2# | no data | no data | no data |
| Lockwood Creek Middle Fork † |  |  | 403.44 | 1.2 | CA403.1760R |  | N N F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | B* B* Fe*,Mn* B* |  |  |  |  |  | Natural sources (m) Grazing (s) Agriculture (s) Recreation (s) |  |  |
| 5 10-19 15 ± 4 | 7 8.0-8.4 8.3 ± 0.2 | no data | 7 195-820 437 ± 186 | 7 438-1010 603 ± 207 | 1 300 | 7 0.1-2.4 1.4 ± 0.9 | 7 4-9 7 ± 2 | 7 ND-463 171 ± 138 | no data | no data | 2 ND | no data | no data | no data |
| Tapo Cyn |  |  | 403.41 | 1.02 | CA403.1830R | 10 interm |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Santa Clara River R7 (Blue cut [west of Salt] to West Pier Hwy 99) |  |  | 403.51 | 9.21 | CA403.1840R | 79 interm | N FT FT N F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,CHL* TDS*,SC*,SO4*,AMM* (Exotic vegetation) Turb* CHL* |  |  | TIS(Ag,Zn)-Fully supporting but threatened |  |  | Grazing - riparian (m) Agriculture (m) Natural sources (m) Urban runoff (m) Industrial point source (s) |  |  |
| 14 21-27 23 ± 2 | 13 7.3-8.5 8.2 ± 0.4 | 8 8.1-8.9 8.2 ± 0.4 | 8 680-936 876 ± 80 | 15 984-1492 1335 ± 107 | 5 390-470 425 ± 27 | 5 0.4-0.7 0.6 ± 0.1 | 9 54-138 105 ± 22 | 9 198-370 306 ± 50 | Tissue('91): no organic chemicals at elevated levels 3 Tissue('92): Ag(EDL85), Zn(EDL85) 3 | 4 0.07-0.44 0.26 ± 0.13 | 8 1.3-7.5 4.5 ± 1.9 | 3 28-880 | no data | 3 ND |
| Castaic Creek R1 (confl SC to Castaic Lagoon) |  |  | 403.51 | 4.3 | CA403.1990R | 8 good |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Castaic Lagoon 314 319 |  |  | 403.51 | 190 ac | CA403.2240L | 197 interm | N F F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | SC, CHL |  |  |  |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Recreational use (m) Bird droppings (m) |  |  |
| 116 9-28 19 ± 6 | 106 7.1-8.7 8.2 ± 0.4 | 116 0.2-13.7 8.1 ± 3.2 | 43 744-798 759 ± 13 | 43 744-798 759 ± 13 | no data | no data | 43 88-114 103 ± 10 | 62 80-124 113 ± 7 | no data | 43 ND-0.13 0.02# | 116 0.2-13.7 8.1 ± 3.2 | no data (closures reported due to coliform) | no data | no data |
| Castaic Creek R2 (lagoon to lake) |  |  | 403.51 | 0.71 | CA403.2270R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Castaic Lake |  |  | 403.51 | 4109 ac | CA403.2280L | 2630 good | F F F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  |  |  |  |  |  |  | Imported water (m) Natural sources (m) Oil spills (s) Aerial deposition (s) |  |  |
| 141 7.2-8.7 7.9 ± 0.4 | 141 0.5-14.6 8.8 ± 3.3 | 62 689-728 703 ± 7 | 62 689-728 703 ± 7 | no data | no data | 62 79-105 93 ± 7 | 15 24-82 50 ± 20 | no data | 62 ND-0.1 0.003# | 141 0.15-14.6 8.8 ± 3.3 | no data | no data | no data |  |
| Elderberry Forebay |  |  | 403.51 | 380 ac | CA403.2300L |  |  |  |  | Unassessed |  |  |  |  |  |
| Castaic Creek R3 (abv Castaic Lake) † |  |  | 403.51 | 11.2 | CA403.2320R |  | N FT N F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SC* TDS* |  |  |  |  |  | Natural sources (m) |  |  |
| 1 27 | 4 8.0-8.4 8.2 ± 0.1 | 1 8.8 | 7 460-716 619 ± 94 | 4 550-1100 815 ± 200 | 4 260-380 348 ± 51 | no data | 4 13-43 30 ± 11 | 4 140-320 233 ± 66 | no data | no data | 4 0.03-0.8 0.4 ± 0.3 | 2 2-5 | 0-4 Ba 37 | no data |
| Elizabeth Lake Cyn † |  |  | 403.51 | 19.09 | CA403.2470R |  | N FT F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SC*,F* TDS* |  |  |  |  |  | Natural sources (m) Urban runoff (m) |  |  |
| 2 26 | 9 8.0-8.5 8.3 ± 0.2 | 2 8.2-8.8 | 12 380-950 557 ± 158 | 9 450-1500 893 ± 300 | 9 210-440 320 ± 75 | no data | 9 18-190 73 ± 48 | 9 62-250 152 ± 55 | no data | no data | 9 ND-0.8 0.2# | 2 ND-3 | 1 Ba 65 Cu 50 Zn 250 | 1 ND |
| Lake Hughes 303(d) - low 314 319 |  |  | 403.51 | 34 ac | CA403.2610L | 40 impaired | N FT N N N | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS*,SC* TDS*,SC*,SO4* EUT, Fish kills TRASH,ODORS, ALGAE TRASH,ODORS, ALGAE |  |  |  |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Septic systems (m) Recreational use (m) |  |  |
| 15 10-21 16 ± 5 | 15 7.7-8.1 8.0 ± 0.1 | 15 4.5-11.0 8.8 ± 2.1 | 10 231-1100 768 ± 222 | 10 1105-6230 1732 ± 1506 | no data | no data | 9 78-262 103 ± 57 | 10 217-2914 544# |  | 9 ND | 9 ND | no data | no data | no data |
| Munz Lake 303(d)-low |  |  | 403.51 | 15 ac | CA403.2620L |  | N F N N N | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | SC* EUT TRASH TRASH |  |  |  |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Nutrient-rich sediments (m) Septic systems (m) Recreational use (m) |  |  |
| 22 6-24 15 ± 7 | 22 6.5-8.1 7.5 ± 0.4 | 22 6.5-10.4 8.3 ± 1.3 | 15 392-822 638 ± 174 | 15 392-822 638 ± 174 | no data | no data | 15 32-73 55 ± 17 | 15 24-82 50 ± 20 | no data | 15 ND-0.23 0.04# | 22 6.5-10.4 8.3 ± 1.3 | no data | no data | no data |
| Elizabeth Lake 303(d)-low 314 319 |  |  | 403.51 | 194 ac | CA403.2640L | 90 impaired | N N N N N | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS,SC,CHL SO4* pH,TDS,SC,CHL,SO4,AMM CHL,pH,EUT AMM,pH,TRASH TRASH |  |  | DO |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Nutrient-rich sediments (m) Septic systems (m) Recreational use (m) |  |  |
| 38 6-22 14 ± 6 | 38 7.3-9.6 8.5 ± 0.7 | 38 0.8-11.0 7.7 ± 2.5 | 20 722-4689 2985 ± 1774 | 20 1152-6330 4141 ± 2323 | no data | no data | 20 128-1056 653 ± 412 | 20 153-1789 956 ± 636 |  | 20 ND-0.45 0.13# | 26 ND-0.58 0.2# | no data | no data | no data |
| Santa Clara River R8 (W Pier Hwy 99 to Bouquet Cyn Rd Bridge) 303(d)-low |  |  | 403.51 | 3.42 | CA403.2690R | 79 interm | N FT N N F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,SC,CHL,B TDS,SC,AMM AMM AMM,COLIFORM CHL |  |  | TOX-Fully but threatened COLOR |  |  | Natural sources (m) Oil spills (s) POTW effluent (m) Urban runoff (m) Industrial point source s(s) |  |  |
| 88 10-27 18 ± 4 | 91 6.8-8.4 7.8 ± 0.3 | 20 4.2-10.8 7.4 ± 2.0 | 90 78-1136 754 ± 109 | 91 120-1412 1131 ± 146 | 82 32-605 355 ± 67 | 89 ND-1.4 0.7 ± 0.2 | 89 10-138 105 ± 21 | 90 20-513 201 ± 60 | Wat Bio Toxicity: reproduction affected, low level chronic toxicity found 5 | 69 ND-4.9 1.4 ± 1.3 | 89 0.3-15.4 5.7 ± 2.4 | 66 20-24000 | 69 As 12 Ba 280 Cd 11 Cr 10 CrVI 29 Cu 60 Pb 100 Se 19 Zn 180 Ni 60 | 3-4 ND be scanned 67 times and detected above detection limit |
| San Fransisquito Cyn |  |  | 403.51 | 25.42 | CA403.2710R |  | F F FT F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  |  |  |  | TOX: Fully but threatened COLOR |  |  | Natural sources (m) Urban runoff (m) Agriculture (s) |  |  |
| 2 18-21 | 9 7.8-8.3 8.2 ± 0.2 | 1 8.2 | 9 217-787 446 ± 192 | 8 350-916 585 ± 230 | 7 92-440 249 ± 144 | 3 0.1-0.7 0.3 ± 0.26 | 9 14-44 28 ± 8 | 9 19-290 122 ± 95 | Wat Bio Toxicity: reproduction affected, low level chronic toxicity found 5 | no data | 9 ND-1.1 0.3# | 3 ND | 0-4 Ag 160 As 35 Ba 52 Cu 30 Pb 4 | 1 ND |
| Santa Clara River South Fork |  |  | 403.51 | 6.08 | CA403.2790R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |
| Newhall Creek |  |  | 403.51 | 4.2 | CA403.2800R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |
| Santa Clara River R9 (Bouquet Cyn Rd to abv Lang gaging station) and R10 (abv Lang gaging station) † |  |  | 403.51 and 403.55 | 12.69 and 3.0 | CA403.2960R and CA403.3380R | 79 interm | F F F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  |  |  |  |  |  |  | Resource extraction |  |  |
| 3 18-30 25 ± 5 | 15 7.9-8.6 8.1 ± 0.2 | 6 5.7-9.8 7.6 ± 1.2 | 18 310-877 440 ± 117 | 18 390-1500 701 ± 216 | 14 190-296 251 ± 29 | 1 ND | 15 8-47 32 ± 10 | 15 20-100 69 ± 20 | no data | no data | 15 ND-4.5 0.5# | 10 2-62 | 0-2 Ba 82 Pb 63 Cu & Zn(14) Zn 20 | 4 ND |
| Bouquet Cyn R1 |  |  | 403.51 | 11.16 | CA403.2970R | 17 good | F F FT F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  |  |  |  | TOX: Fully but threatened |  |  | Toxicity may be from copper sulfate treatments in reservoir (m) Natural sources (m) Imported water (m) Recreational use (m) |  |  |
| 4 11-13 12 ± 1 | 12 7.6-8.6 8.2 ± 0.3 | 1 12 | 11 229-414 286 ± 45 | 11 341-654 440 ± 84 | 9 86-275 147 ± 51 | 5 0.5-0.6 0.56 ± 0.05 | 11 19-33 24 ± 4 | 11 28-77 38 ± 13 | Wat Bio Toxicity: reproduction affected, low level chronic toxicity found 5 | no data | 10 ND-1.2 0.4# | 3 2-13 | 0-4 As 10 Ba 40 Cr 15 Zn 50 | 1 ND |
| Bouquet Reservoir |  |  | 403.52 | 1206 ac | CA403.3090L | 628 good | F F F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  |  |  |  |  |  |  |  |  |  |
| Mint Cyn R1 (confl to Rowler Cyn) |  |  | 403.51 | 8.16 | CA403.3180R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Mint Cyn Creek R2 (abv Rowler Cyn) |  |  | 403.53 | 7.35 | CA403.3230R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |
| Agua Dulce Cyn R1 (confl SC to Hwy 14) |  |  | 403.55 | 2.88 | CA403.3410R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Agua Dulce Cyn R2 (abv Hwy 14) |  |  | 403.54 | 6.61 | CA403.3430R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Solidad Cyn |  |  | 403.55 | 2.89 | CA403.3760R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Aliso Cyn |  |  | 403.55 | 7.05 | CA403.3780R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |

Calleguas Creek Watershed

| Reach/location (Calleguas Creek Watershed) |  |  |  |  | Hydro Unit | Size | Code | Previous WQA | WQA | Assessed Uses |  |  | Not supporting |  |  | Partially supporting |  |  | Potential Sources |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temp |  | pH |  | DO | TDS | SC | Hard | B | Chl | SO4 | Tissue, Sediment and Toxicity Data | NH3 | N+N | Fec Col | metals | Org Chem |
| Mugu Lagoon 303(d)-low 319 |  |  |  |  | 403.11 | 505 ac | CA404.0000E | 1500 impaired | N U U N | Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | Cu, Hg,Ni,Zn, Bird reproductivity(DDT), TIS(Chlordane, DDT, endosulfan, dacthal, toxaphene, PCBs); Sed(DDT,toxaphene), SED TOX,Nitrogen, Excessive sediment TIS(DDT, PCBs, toxaphene) |  |  | TIS(As,Cd,Ag) |  |  | Natural sources (m) Agriculture runoff (m) Naval base activities (m) Construction (s) Landfill runoff (s) Industrial sites (m) Urban runoff (m) |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Wat Chem: Cu, Hg, Ni, Zn exceedences [US Navy data] Bird reproductivity : DDT14 Tissue('90): no organic chemicals at elevated levels 3 Tissue('91): As(EDL95), Cd(EDL95), Ag(EDL85) 3 Tissue('92): DDT(MTRLs), PCBs(MTRLs), As(EDL85), Cd(EDL85), Ag(EDL95) 3 Tissue('93): DDT(MTRLs), Ag(EDL95) 3 Sed Chem('88,'89,'91,'92): DDE(200 ppb), DDD,toxaphene(1900ppb),DDT(30ppb) 1 Sed Toxicity('93): poor survival rates 2 Tissue('88,93): toxaphene (NAS), DDT(NAS),chlordane,endosulfan,dacthal 2 |  |  |  |  |  |  |
| Rio de Santa Clara/Oxnard Drain #3 303(d)-low |  |  |  |  | 403.11 | 2.48 | CA404.0005R |  | N U U N | Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | TIS(chlordane, DDT, toxaphene, ChemA, PCBs), SED TOX,Nitrogen TIS(chlordane, DDT, toxaphene, ChemA, PCBs) |  |  |  |  |  | Natural sources (m) Agriculture runoff (m) Urban runoff (m) |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Tissue('90): chlordane(NAS), DDT(NAS), toxaphene(NAS), ChemA(NAS) 3 Tissue('91): chlordane(NAS), DDT(NAS), PCBs(NAS), toxaphene(NAS), ChemA(NAS)3 Tissue('93): no organic chemicals at elevated levels 3 Sed toxicity ('93,'94): poor survival rates 2 Tissue('90-'91): toxaphene (NAS), PCBs(NAS), chlordane(NAS) 1 |  |  |  |  |  |  |
| Duck Pond Ag Drain /Mugu Drain/Oxnard Dr #2 303(d)-low |  |  |  |  | 403.11 | 13.5 | CA404.0010R |  | N F F N | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | TDS*, CHL* SO4*, N+N* TDS* TOX, SED TOX, Sed(DDT),TIS(Chlordane, toxaphene, DDT, ChemA), Nitrogen TIS(Chlordane, toxaphene, DDT, ChemA) |  |  | TIS(Dacthal): Fully supporting bu threatened COLOR |  |  | Natural sources (m) Agriculture runoff (m) Urban runoff (m) Municipal effluent (m) |  |  |
| 2 20-22 21 ± 1 |  | 4 7.2-7.6 7.4 ± 0.1 |  | no data | 3 3386-5360 4065 ± 916 | 2 3120-3700 3410 ± 290 | 2 1515-1850 1683 ± 168 | 2 1.9-2.2 2.1 ± 0.1 | 3 176-580 324 ± 182 | 3 1538-2300 1818 ± 393 | Tissue('93): chlordane(MTRLs,FDA), DDT(MTRLs,FDA), toxaphene(MTRLs, NAS), dacthal(EDL85), ChemA(NAS) 3 Sed Chem('88,'89,'95): DDT (500ppb) 1 Wat toxicity: poor survival rates 5 Sed toxicity: poor survival rates 1 | no data | 3 54-118 80 ± 28 | no data | 1 ND | H2O (2): a,s,y,ae,af Sed (5): sb,sc,sd |
| Calleguas Creek Estuary 303(d)-low |  |  |  |  | 403.11 | 0.8 | CA404.0060E |  | N F F N | Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | AMM, TOX, SED TOX, Sed(DDT), TIS(chlordane, endosulfan,DDT,toxaphane), Nitrogen TIS(chlordane, DDT,toxaphane), |  |  | COLOR |  |  | Natural sources (m) Agriculture runoff (m) Urban runoff (m) |  |  |
| 9 10-29 19 ± 5 |  | 26 6.8-8.6 7.7 ± 0.5 |  | 3 5.7-8.3 7.3 ± 1.1 | 26 636-7072 1719 ± 1391 | 21 1085-9000 2704 ± 1736 | 20 305-1360 705 ± 235 | 1 0.8 | 19 105-2820 551# | 20 142-857 439 ± 183 | Wat toxicity: poor survival rates 5 Sed toxicity (93): poor survival rates 2 Sed chem: DDT 1 Tissue: chlordane (263 ppb), DDT (4284 ppb), endosulfan (880 ppb), toxaphene(3600 ppb) 1 | 28 0.1-20.2 2.7 ± 3.6 | 9 1.12-26.9 14.8 ± 6.0 | no data | Cu and Zn (19) Cu ND Zn 40 no other metals scanned | H2O no data Sed(1) ND |
| Revolon Slough main branch: Mugu Lagoon to Central Avenue 303(d)-low 319 |  |  |  |  | 403.11 | 8.90 | CA404.0080R | 9 impaired | N N N N N N | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | TDS,SC,CHL,B SO4,B,N+N TDS,SC,SO4,AMM Se*,TOX,Sed(toxaphene,chlordane, DDT, endosulfan, dacthal),TIS(endosulfan, chlordane, toxaphene, DDT,ChemA,dieldrin,PCBs), Nitrogen AMM*, ALGAE, COLOR, TRASH, Turb ALGAE,TRASH TIS(endosulfan, chlordane, toxaphene, DDT,ChemA,dieldrin,PCBs) |  |  | [GROSS ALPHA (N=3) ABOVE STND] TIS(Chlorpyrifos, hexochlorobenzene,dacthal) |  |  | Natural sources (m) Agriculture runoff (m) Urban runoff (m) |  |  |
| 15 14-24 19 ± 3 |  | 23 5.2-8.6 7.9 ± 0.6 |  | 1 7 | 22 2160-4900 3663 ± 694 | 20 2600-6160 4353 ± 989 | 15 1012-2140 1682 ± 305 | 20 0.6-2.6 1.8 ± 0.5 | 22 131-276 185 ± 35 | 22 1083-2450 1840 ± 354 | Tissue('90): dacthal(EDL95), DDT(NAS), endosulfan(NAS), toxaphene(NAS), ChemA(NAS) 3 Tissue('91): chlordane(MTRLs), DDT(MTRLs), dieldrin(MTRLs), toxaphene(MTRLs,NAS), dacthal(EDL85), ChemA(NAS) 3 Tissue('93): chlordane(NAS), chloropyrifo(EDL95), dacthal(EDL95), DDT(NAS)< hexochlorobenzene(EDL85), toxaphene(NAS), ChemA(NAS) 3 Wat toxicity: poor survival rates 5 Sed chem(88-90): DDT(1648 ppb), toxaphene, chlordane, endosulfan, dacthal 1 Tissue(86-88): DDT(NAS), PCBs(NAS) 1 | 12 ND-1.2 0.5 ± 0.4 | 22 11.9-70.0 48.5 ± 13 | no data | 2-4 Ba 82 Cu 11 Pb 9 Se 11 | H2O (8): b,c,d,e, ad,be Sed (13): sb,sc,sd,sg |
| Beardsley Channel (abv Central Ave) 303(d)-low 319 |  |  |  |  | 403.61 | 6.16 | CA404.0160R | 5 impaired | N P P | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | SO4*, N+N* TDS*, SC*,SO4* TOX, Sed(DDT) |  |  | Sed pest. are very high ALGAE, COLOR ALGAE |  |  | Natural sources (m) Agriculture runoff (m) Urban runoff (m) Illegal dumping (s) Excessive sediment (m) |  |  |
| 5 17-24 21 ± 2 |  | 6 7.3-8.8 7.9 ± 0.5 |  | no data | 5 1170-3300 1988 ± 741 | 5 1320-4990 2315 ± 1370 | 2 660-940 800 ± 140 | 5 0.5-2 1.0 ± 0.6 | 5 75-177 124 ± 32 | 5 470-1790 965 ± 467 | Wat toxicity: poor survival rates 5 Sed chem: DDD(1466ppb), DDE(173ppb), DDT(18ppb) [RWQCB] | no data | 5 3.8-38.8 16.1 ± 13.0 | no data | no data | H20(1) ND Sed(4) sb,sc,sd,sh |
| Calleguas Creek R1 (estuary to 0.5 mi s of Broome Rd) and R2 (0.5 mi s of Broome Rd to Potrero Rd) 303(d)-low 319 |  |  |  |  | 403.11 403.12 | 2.2 and 2.3 | CA404.0200R and CA404.0220R | 11 impaired | N N N FT F N | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | TDS, SC,CHL N+N TURB*, TDS,SC,SO4,AMM AMM, TIS(DDT,toxaphene, ChemA, chlordane)Nitrogen TURB*,AMM TIS(DDT,toxaphene, ChemA, chlordane) |  |  | TIS(dacthal): Fully but threatened |  |  | Natural sources (m) Agriculture runoff (m) Urban runoff (m) Excessive sediment (m) POTW effluent (m) |  |  |
| 3 10-31 20 ± 8.6 |  | 22 7-8.6 7.7 ± 0.5 |  | no data | 26 464-2772 918 ± 411 | 21 909-4995 1650 ± 795 | 19 230-1972 568 ± 353 | 1 0.5 | 19 92-400 204 ± 65 | 20 114-1286 347 ± 231 | Tissue('90): dacthal(EDL85), DDT(NAS), toxaphene(NAS), ChemA(NAS) 3 Tissue('91): chlordane(MTRLs), DDT(MTRLs,NAS), toxaphene(NAS), ChemA(NAS), dacthal(EDL85), ChemA(NAS) 3 Tissue('92): DDT(NAS), toxaphene(NAS), ChemA(NAS) 3 Tissue('93): chlordane(NAS), DDT(NAS), toxaphene(NAS), ChemA(NAS) 3 | 25 0.25-21.4 3.54 ± 4.2 | 26 4.5-30.1 14.1 ± 6.5 | no data | Cu and Zn(19) Cu 30 Zn 50 no other metals scanned | no data |
| Calleguas Creek R3 (Potrero to Somis Rd) 303(d)-low |  |  |  |  | NEED | 7.7 | CA404.0260R |  | N FT N FT F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,SC,CHL TURB*,TDS,SC,SO4,AMM AMM, TOX, Sed(toxaphene, DDT) TURB,AMM TDS,CHL,SO4,N+N |  |  |  |  |  | Natural sources (m) Agriculture runoff (m) Urban runoff (m) Excessive sediment (m) POTW effluent (m) |  |  |
| 18 11-30 21 ± 5 |  | 60 6.7-8.5 7.6 ± 0.4 |  | no data | 63 544-1340 860 ± 159 | 53 943-1950 1451 ± 226 | 44 240-652 464 ± 112 | 12 0.3-0.6 0.4 ± 0.1 | 49 101-264 185 ± 29 | 51 100-550 272 ± 85 | Sed chem: toxaphene, DDT 1 Wat toxicity: poor survival rates 5 | 59 ND-20 4.6 ± 4.0 | 67 1.9-36 12.1 ± 6.8 | no data | 2-41 Ba 86 Cu 70 Zn 30 Al 80 | H2O(4): d,l Sed(5): sc,sd,sg |
| Conejo Creek R1 (confl Call to Santa Rosa Rd) and R2 (Santa Rosa Rd to ~Thousand Oaks city limit) and R3 (~Thousand Oaks city limit to Lynn Rd.) and R4 (abv Lynn Rd) 303(d)-low |  |  |  |  | 403.12 403.63 403.64 403.68 | 5.8 and 2.67 and 5.6 and 4.98 | CA404.0270R and CA404.0340R and CA404.0400R and CA404.0500R | 10 interm | N FT N N N N | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish consumption Basin Plan |  |  | TDS,SC,CHL TURB*,TDS,SC,SO4,AMM DO, AMM, TOX, TIS(endosulfan, toxaphane, DDT, ChemA),Sed(toxaphene) TURB,AMM, ALGAE, COLOR ALGAE TIS(endosulfan, toxaphane, DDT, ChemA) TDS,CHL,SO4,N+N |  |  | TIS(Dacthal, Ag, Cd, Cr, Ni) |  |  | Natural sources (m) Agriculture runoff (m) Urban runoff (m) Sediment (m) POTW effluent (m) Grazing (s) |  |  |
| R1: 13 13-29 21 ± 5 R2: 5 13-25 19 ± 5 R3: 187 12-26 19 ± 3 |  | 53 6.8-8.7 7.5 ± 0.4 27 7.0-8.1 7.5 ± 0.3 223 6.8-8.6 7.6 ± 0.3 |  | no data no data 173 2.6-10.9 7.0 ± 1.8 | 60 560-1210 825 ± 129 31 408-1210 819 ± 154 60 536-1240 888 ± 69 | 49 980-1825 1400 ± 195 20 250-608 482 ± 93 43 960-2080 1527 ± 273 | 44 260-608 456 ± 102 20 250-608 482 ± 93 39 240-792 545 ± 143 | 9 0.2-0.5 0.4 ± 10.1 6 0.2-0.5 0.4 ± 0.1 3 0.4-0.5 0.5 ± 0.04 | 47 109-236 181 ± 125 25 78-230 173 ± 31 41 70-242 172 ± 36 | 48 86-414 261 ± 176 25 99-386 264 ± 78 41 143-571 286 ± 95 | Wat toxicity: poor survival rates 5 Sed chem: toxaphene 1 Tissue('91): Ag(EDL95), dacthal(EDL85), DDT(NAS), endosulfan(NAS), toxaphene(NAS), ChemA(NAS) 3 Tissue('92): Cd(EDL95), Cr(EDL95), Ni(EDL85), Ag(EDL85), DDT(NAS), toxaphene(NAS), ChemA(NAS) 3 | 60 0.75-28.0 6.89 ± 5.01 26 0.5-28.1 9.2 ± 7.0 58 ND-34.4 6.49# | 60 2.6-33.6 11.0 ± 16.8 31 1.6-25.0 9.4 ± 6.1 60 0.2-22.1 3.7# | no data no data no data | 2-77 Ba 166 Cd 1 Zn 100 | H2O (8): b,d Sed(2): sb,sc,sd |
| Arroyo Santa Rosa R1 (confl Conejo to Knob Lane) and R2 (abv Knob Lane) |  |  |  |  | 403.63 403.65 | 4.09 and 4.55 | CA404.0370R and CA404.0380R | 8 impaired |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Trib from Olsen Rd POTW |  |  |  |  | 403.65 | 2.74 | CA404.0390R |  | U F F F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  |  |  |  |  |  |  |  |  |  |
| 350 7-24 18 ± 3 |  | 350 7.0-8.4 7.9 ± 0.3 |  | 348 3.2-11.5 7.5 ± 1.2 | no data | no data | no data | no data | no data | no data | no data | no data | no data | no data | no data | no data |
| Conejo Creek/Arroyo Conejo North Fork 303(d)-low |  |  |  |  | 403.64 | 6.51 | CA404.0420R |  | N FT N F F N | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish consumption Basin Plan |  |  | TDS,SC,CHL TDS,SC,SO4,AMM CHL, AMM,TIS(chlordane,DDT) TIS(chlordane,DDT) TDS,CHL,SO4 |  |  | TIS(Cd, Ag): Fully but threatened |  |  | Natural sources (m) Agriculture runoff (m) Urban runoff (m) Sediment (m) POTW effluent (m) Grazing (s) Sewage spills (s) |  |  |
| 282 6-26 17 ± 5 |  | 301 6.6-8.8 7.9 ± 0.6 |  | 267 4.5-12.9 8.9 ± 1.7 | 42 460-1470 1023 ± 246 | 23 789-2240 1766 ± 316 | 19 235-884 649 ± 152 | 3 0.2-0.5 0.4 ± 0.1 | 22 75-324 231 ± 14 | 22 43-455 299 ± 105 | Tissue('90) no organic chemicals at elevated levels 3 Tissue('91): Cd(EDL85) 3 Tissue('93): chlordane(MTRLs), DDT(MTRLs), Ag(EDL85) 3 | 40 ND-24.8 4.2 ± 7.5 | 35 0.2-4.5 2.1 ± 1.2 | no data | Cu and Zn (19) Zn 20 no other metals scanned | 1 ND |
| Arroyo Conejo South Branch |  |  |  |  | 403.64 | 6.25 | CA404.0450R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| 2 19-19 |  | no data |  | no data | 2 1720-2200 | 2 2820-3310 | no data | 2 0.2 | 2 187-208 | 2 669-982 | no data | no data | 2 1.5-2.5 | no data | no data | no data |
| Arroyo Las Posas R1 (Lewis-Somis Rd to Fox Barranca) and R2 (Fox Barranca to ~Moorpark Frwy (23)) 303(d)-low 319 |  |  |  |  | 403.12 403.62 | 1.99 and 9.62 | CA404.0590R and CA404.0600R | 8 interm | N N N FT F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,CHL*,B* SO4* TDS*,SC*,SO4*,AMM* AMM*, Sed(DDT) AMM* TDS*,CHL*,SO4*,N+N |  |  |  |  |  | Natural sources (m) Agriculture runoff (m) Urban runoff (m) Sediment (m) POTW effluent (m) Grazing (s) Fuel spills (s) |  |  |
| 4 18.9-28 22 ± 3 |  | 4 7.5-8.0 7.9 ± 0.2 |  | no data | 5 1120-1280 1194 ± 52 | 4 1630-1800 1728 ± 62 | 3 460-580 513 ± 50 | 4 0.7-0.9 0.8 ± 0.1 | 4 142-190 171 ± 18 | 4 401-500 438 ± 39 | Sed Chem('88): DDD(204ppb), DDE(1084ppb), DDT(2339ppb)[RWQCB data] | 4 2.3-4.6 3.95 ± 0.96 | 5 9.0-12.3 11.3 ± 1.2 | no data | Al 30 no other metals scanned | H2O (1): ND Sed(1): sb,sc,sd |
| Fox Barranca |  |  |  |  | 403.62 | 3.03 | CA404.0610R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Mahan Barranca |  |  |  |  | 403.62 | 4.4 | CA404.0660R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Arroyo Simi R1 (~Moorpark Frwy (23) to Brea cyn) 303(d)-low |  |  |  |  | 403.62 | 7.58 | CA404.0710R | 15 interm | N P N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SO4*,B* TDS*,SO4*,AMM* SO4*,AMM*,ALGAE ALGAE TDS*,CHL*,SO4*,B* |  |  | TIS(Cr, Ni, Ag, Se,Zn); AMM*, Temp, TOX (3 are FT) COLOR |  |  | Natural sources (m) Agriculture runoff (m) Urban runoff (m) POTW effluent (m) Grazing (s) Spills (s) |  |  |
| 218 9-32 19 ± 4 |  | 213 6.9-8.9 8.0 ± 0.3 |  | 208 5.2-12.4 8.5 ± 1.2 | 7 1120-2600 1751 ± 470 | 10 1620-4550 2402 ± 827 | 3 535-1000 838 ± 215 | 10 0.4-1.4 1.1 ± 0.3 | 10 133-1190 277# | 7 185-1000 642 ± 278 | Tissue('91) Cr(EDL95), Ni(EDL85), Se(EDL95), Ag(EDL85), Zn(EDL85) 3 Wat toxicity: poor survival rates 7 Temperature 7 | 7 0.03-16.1 6.11# | 10 1.4-14.8 4.7 ± 3.8 | no data | no data | no data |
| Arroyo Simi R2 (abv Brea Cyn) |  |  |  |  | 403.67 | 11.12 | CA404.0770R | 15 interm | N F F F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SO4*,B* TDS*,SC*,SO4*, TDS*,CHL*,SO4*,B* |  |  |  |  |  | Ground water de-watering (m) Natural sources (m) Agriculture runoff (m) Urban runoff (m) Grazing (s) |  |  |
| 4 12-29 22 ± 7 |  | 7 7.7-9.0 8.3 ± 0.4 |  | no data See Simi Report | 7 492-2380 1654 ± 619 | 7 825-2700 2121 ± 670 | 2 1000-1030 1015# | 7 0.2-1.5 0.9 ± 0.5 | 7 53-180 130 ± 50 | 7 222-1040 800 ± 307 | no data | 2 0.08-0.1 0.09 ± 1.5 | 7 0.85-5.2 3.3 ± 1.5 | no data | 0-1 As 2 Ba 100 | 1 ND |
| Tapo Cyn R1 |  |  |  |  | 403.67 | 5.23 | CA404.0880R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |

Los Angeles River watershed

| Reach/location (Los Angeles River) |  |  | Hydro Unit | Size | Code | Prev WQA | WQA | Assessed Uses |  |  | Not supporting |  |  | Partially supporting |  |  | Potential Sources |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temp | pH | DO | TDS | SC | Hard | B | Chl | SO4 | Tissue, Sediment and Toxicity Data | AMM | N+N | Fec Col | metals | VOCs |
| Los Angeles River Estuary (Queensway Bay) |  |  | 405.12 | 3.71 | CA409.0000E |  | FT U U | Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  |  |  |  | TIS(Chlordane,DDT),Sed(PAHs,Chlordane): Fully but threatened |  |  | Urban runoff (m) Dredging activities (m) POTW effluent (m) Spills (s) Industrial point sources (m) Illegal dumping (m) |  |  |
|  |  |  |  |  |  |  |  |  | Sed tox: good survival rates 2 Sed Chem: PAH (2ppm), chlordane (15 ppb) 2 Benthic community: slightly impaired 2 Tissue ('86): chlordane (150ppb), DDT(250ppb) 1 |  |  |  |  |  |
| Los Angeles River R1 (Estuary to upstream Carson St) 303(d) low |  |  | 405.12 | 2.01 | CA409.0010R |  | FT N N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | (NO2) pH,TDS,SC,AMM pH,AMM,Pb pH,TURB*,AMM,COLIFORM,TRASH,SCUM,ALGAE,COLOR COLIFORM, TRASH, SCUM, ALGAE |  |  | CHL |  |  | Natural sources (m) POTW effluent (m) Illegal dumping (s) Transient encampments (s) Oil/waste spills (m) Urban runoff (m) Horse corrals (s) |  |  |
| 145 7-34 22±6 | 148 7-10.6 9.2±0.9 | 80 9.1-24 19.2±2.4 | 95 399-926 683±85 | 141 624-1310 1061±99 | 87 153-464 275±55 | 68 ND-0.93 0.5±0.2 | 94 44-205 131±28 | 93 115-307 188±42 | no data | 94 ND-8.2 2.7±2.4 | 94 0.01-13.16 4.6±2.4 | 91 ND-93000 | 68-87 As 37 Ba 630 Cd 42 Cr 20 CrVI 16 Cu 40 Pb 120 Se 3 Zn 340 Be 0.5 Ni 86 | H2O(11): s,af (68)be |
| Los Angeles River R2 (Upstream Carson St. to Figueroa St.) 303(d) low |  |  | 405.15 | 19.37 | CA409.0020R | 28 interm | FT N N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,SC,AMM AMM,Pb AMM,COLIFORM,TRASH, SCUM,ALGAE,COLOR COLIFORM,TRASH, SCUM,ALGAE |  |  | pH,ODOR,OIL,Turb ODOR, OIL CHL,N+N |  |  | Natural sources (m) POTW effluent (m) Illegal dumping (s) Transient encampments (s) Spills (s) Urban runoff (m) Horse corrals (s) |  |  |
| 156 7-32 19±5 | 163 6.7-9.7 8.1±0.6 | 21 9-18 13.2±2.5 | 152 300-972 672±92 | 142 427-1318 1056±127 | 142 150-466 271±52 | 142 ND-1.8 0.5±0.2 | 163 37-226 125±30 | 160 88-370 190±49 | no data | 162 ND-29.8 6±4.5 | 160 0-19.2 6.2±3.8 | 156 ND-230000 | 54-142 As 190 Ba 290 Cd 2 Cr 70 CrVI 17 Cu 210 Pb 140 Se 15 Zn 2300 Hg 1 Ni 70 | H2O(11): d,n,p,s,u,y,av, be(138) |
| Compton Creek 303(d) low |  |  | 405.15 | 8.52 | CA409.0040R | 8 interm | FT N N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS, AMM Cu,Pb AMM,COLIFORM COLIFORM |  |  | pH pH |  |  | Natural sources (m) POTW effluent (m) Illegal dumping (s) Fuel spills (s) Urban runoff (m) Horse stables (s) |  |  |
| 58 8-28 20±5 | 59 6.9-9.9 8.1±0.6 | 8 1.8-7 5.5±1.7 | 58 180-1386 531±152 | 52 270-2230 851±234 | 52 85-348 229±50 | 52 ND-1.1 0.3±0.2 | 59 19-520 99±62 | 58 28-283 120±44 | no data | 58 ND-12.1 0.7±1.7 | 57 ND-7.6 0.4±1.1 | 57 ND-130000 | 51 As 28 Ba 312 Cd 10 Cr 40 CrVI 15 Cu 230 Pb 200 Se 7 Zn 1000 Ni 50 | H2O (1) ND (51)be |
| Rio Hondo R1 (Confluence LA River to Santa Ana Frwy) 303(d) -low |  |  | 405.15 | 4.19 | CA409.0080R |  | FT N N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,SC,AMM AMM,Cu,Pb,Zn AMM,COLIFORM,TRASH TRASH CHL |  |  | CHL,pH COLIFORM SO4 |  |  | Natural sources (m) POTW effluent (m) Illegal dumping (s) Urban runoff (m) |  |  |
| 56 8-38 21±6 | 57 7.3-9.9 8.3±0.6 | 1 12 | 57 138-2020 786±331 | 56 200-2850 1194±464 | 56 60-653 282±102 | 56 ND-0.8 0.3±0.2 | 57 12-559 164±107 | 57 22-561 209±106 | no data | 57 ND-2.6 0.34±0.47 | 57 ND-5 0.7±1.1 | 60 ND-28000 | 56 As 67 Ba 333 Cd 10 Cr 30 CrVI 42 Cu 63 Pb 110 Se 14 Zn 1340 Hg 1 Ni 30 | H2O(2): c,e be scanned 55 times and detected above detection limit |
| Rio Hondo R2 (at Spreading Grounds) and R3 (Spreading Grounds to Whittier Narrows Dam) and R4: at Whittier Narrows Flood Control Basin 303(d) -low |  |  | 405.15 405.15 405.41 | 2.71 and 1.36 and 3.10 | CA409.0090R and CA409.0100R and CA409.0110R | 20 interm | FT N N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,AMM AMM COLIFORM, AMM |  |  | DO: fully but threatened Turb COLIFORM N+N |  |  | Natural sources (m) POTW effluent (m) Illegal dumping (s) Urban runoff (m) |  |  |
| R2: 60 8-26 18±4 R3: 28 10-27 18±4 R4: 116 8-29 19±4 | 65 6.7-10.7 7.8±0.6 29 7-8.9 8±0.3 124 6.6-8.6 7.8±0.4 | no data 28 4.9-19.5 10.7±3.1 59 1.8-19 7.9±3.3 | 65 104-669 506±134 28 283-730 538±117 124 ND-653 486±128 | 64 129-1084 804±215 28 394-1080 842±160 124 273-1085 789±189 | 65 48-324 220±51 27 152-348 227±41 124 12-369 221±56 | 64 ND-0.5 0.2±0.1 no data 65 ND-0.5 0.2±0.1 | 65 11-178 85±33 28 5-128 79±35 124 1-52 71±28 | 65 17-297 142±63 28 50-229 142±43 124 3-281 137±61 | no data no data no data | 65 ND-18.2 4.4±4.6 no data 65 ND-22.3 6.7±5.9 | 64 0.2-14.5 2.7±3.2 1 5.73 65 0.2-21 3.4±4.2 | 61 ND-90000 no data 65 ND->160000 | 61-130 Ag 30 As 44 Ba 430 Cd 20 Cr 20 CrVI 49 Cu 57 Pb 80 Se 13 Zn 240 Ni 45 | H2O(7): b,d,e,f,g,n,t,ad (65)be |
| Garvey Reservoir |  |  | 405.41 | 33 ac | CA409.0150L | 1610 unkn |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Rio Hondo R5 (Whittier Narrows Flood Control Basin to Peck Road Park Lake) and R6 (Peck Road Park Lake to Santa Fe Flood Control Basin) |  |  | 405.41 405.33 | 5.16 and 2.82 | CA409.0190R and CA409.0670R | 20 interm |  |  |  |  | Unassessed |  |  |  |  |  | Urban runoff (m) |  |  |
| no data | no data | no data | 2 241-518 380±139 | no data | 1 204 | 1 ND | 2 32-136 84±53 | 2 26-39 33±7 | no data | no data | 1 3.5 | no data | 3 Ba 48 Cd 2 Cr 47 Cu 27 Pb 12 Zn 92 | H2O(4): y |
| Eaton Wash R1 (confluence Rio Hondo to Huntington Drive) and R2 (Huntington Drive to Eaton Canyon Reservoir Dam) |  |  | 405.41 405.31 | 4.98 and 2.84 | CA409.0220R and CA409.0240R |  |  |  |  |  | Unassessed |  |  |  |  |  | Urban runoff (m) |  |  |
| no data | 1 9.1 | no data | 2 213-271 | no data | 1 104 | 1 ND | 2 26-38 | 2 38-55 | no data | no data | 1 0.6 | no data | no data | H2O(1)ND |
| Eaton Canyon Reservoir |  |  | 405.31 | 0.066 | CA409.0320L |  |  |  |  |  |  |  |  |  |  |
| Eaton Canyon Creek R3 (upstream of Reservoir) |  |  | 405.31 | 6.95 | CA409.0330R |  |  |  |  |  | Unassessed |  |  |  |  |  | Urban runoff (m) |  |  |
| no data | 1 7.8 | no data | 2 212-213 212±1 | no data | 1 166 | 1 0.2 | 2 7-9 8±1 | 2 18-20 19±1 | no data | no data | no data | no data | no data | no data |
| Peck Road Park Lake 303(d)-low |  |  | 405.41 | 166 ac | CA409.0490L | 80 interm | F N N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | DO,Pb,TIS(DDT,chlordane) AMM,ODORS,TRASH ODORS,TRASH TIS(DDT,chlordane) |  |  |  |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Recreational use (m) |  |  |
| 195 10-26 17 ± 5 | 181 6.9-8.8 7.7 ± 0.5 | 195 0.2-15.2 6.0 ± 4.0 | 90 116-271 175 ± 32 | 90 169-427 276 ± 52 | no data (220) | no data | 90 5-36 9.2 ± 8 | 90 8-44 20 ± 8 | Tissue ('91): chlordane(MTRLs), DDT(MTRLs) 3 Tissue ('92): No organic chemicals at elevated levels 3 | 90 ND-1.2 0.14# | 90 ND-1.1 0.5 ± 0.4 | no data | 90 Ag 1.1 As 6 Cu 69 Pb 73 Se 2 Zn 47 Ni 23 | 41+ a,m,s,t,v,w,y,z,ag,aj,al,aa,ad,au,av,aw,ax,az,bo,bp |
| Santa Anita Wash R1 (Peck Road Park Lake to Foothill Blvd) and R2 (Foothill Blvd to Dam at Carolwood Dr) and R3 (Debris Basin at Carolwood Dr. to Santa Anita Reservoir) |  |  | 405.41 405.33 405.33 | 1.33 and 3.5 and 1.06 | CA409.0500R and CA409.0510R and CA409.0550R |  | F F FT F | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | COLOR |  |  |  |  |  | Natural sources (m) Urban runoff (m) |  |  |
| no data | 2 8.4 8.4±0 | no data | 3 230-381 283±70 | no data | 1 220 | 2 ND-0.1 0.05±0.05 | 3 12-32 19±9 | 3 16-41 25±11 | no data | no data | 1 4.4 | no data | no data | no data |
| Santa Anita Reservoir |  |  | 405.33 | 0.021 ac | CA409.0580L | 17 unknown |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Santa Anita Canyon Creek R4 (upstream of Reservoir) |  |  | 405.33 | 4.84 | CA409.0590R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| 3 15-25 18±4 | 15 7.8-8.5 8.1±0.2 | 3 8.1-8.3 8.2±0.1 | 18 130-320 205±46 | 15 170-530 348±79 | 14 100-206 158±27 | no data | 15 4-10 7±1 | 15 7-150 23±34 | no data | no data | 15 0.16-1.37 0.7±0.4 | 5 ND-300 | 1-15 Ba 32 Cu and Zn (15) Cu 40 Zn 11 | no data |
| Sawpit Wash R1 (Confluence Rio Hondo to Dam at Spanish Canyon) and R2 (Debris Basin at Spanish Canyon to Sawpit Dam) |  |  | 405.41 405.33 | 5.45 and 0.36 | CA409.0690R and CA409.0730R |  | F F F F | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  |  |  |  |  |  |  | Urban runoff (m) |  |  |
| no data | 1 9 | no data | 3 83-231 170±63 | 1 256 | 2 107-150 129±22 | 2 ND | 3 10-136 54±58 | 3 11-44 29±14 | 0 | 1 0.8 | 2 ND-0.8 0.4±0.4 | no data | 2 Ba 58 | H2O(1)ND |
| Monrovia Canyon Creek 303(d) - low |  |  | 405.33 | 2.90 | CA409.0740R |  | F N FT F | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | Pb |  |  | AMM |  |  | Natural sources (m) Urban runoff (m) Unknown sources (s) |  |  |
| 65 6-24 16±4 | 68 7.4-8.9 8.2±0.3 | no data | 68 200-638 278±57 | 68 331-882 450±74 | 68 160-260 213±23 | 68 ND-0.5 0.1±0.1 | 68 4-8720 138±1049 | 68 14-209 30±23 | no data | 68 ND-4.3 0.1±0.5 | 68 0.18-9.54 1.5±1.6 | 67 ND-300 | 68 As 11 Ba 182 Cd 18 Cr 10 Cu 40 Pb 40 Se 6 Zn 230 Hg 2 Ni 85 | H20 no data (68)be |
| Sawpit Reservoir |  |  | 405.33 | 10 ac | CA409.0760L |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Sawpit Canyon Creek R3 (upstream of Reservoir) |  |  | 405.33 | 3.51 | CA409.0770R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Hollenbeck Park Lake |  |  | 405.15 | 4.75 ac | CA409.0870L | 5 unknown |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Belvedere Park Lake 314 |  |  | 405.15 | 4 ac | CA409.0880L | 2 intermediate | F F F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | Lake was restored to Fish and Game specifications in 1995. pH pH,AMM,DO,Cu,Pb,EUT, TIS(chlordane, ChemA) AMM,pH,ODORS,TRASH,ALGAE ODORS,TRASH,ALGAE |  |  | TIS(Cr,Cu,Pb,Zn) |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Nutrient-rich sediments (m) Septic systems (m) Recreational use (m) |  |  |
| 34 12-28 21 ± 5 | 34 7.0-10.1 8.8 ± 0.6 | 34 0.5-18.5 8.2 ± 5.0 | 34 19-722 417 ± 216 | 34 100-1115 662 ± 327 | no data (300) | no data | 34 8-163 83 ± 53 | 34 10-155 102 ± 53 | Tissue ('91): Cr(EDL85), Cu(EDL95), Pb(EDL95), Zn(EDL95), chlordane(NAS), ChemA(NAS) 3 Tissue ('92): Pb(EDL85), Zn(EDL95), chlordane(NAS), ChemA(NAS) 3 | 34 ND-1.5 0.20# | 34 ND-0.18 0.04# | no data | 34 Cd 1 Cu 81 Pb 51 Se 3 Zn 31 Ni 16 | no data |
| Lincoln Park Lake 303(d)-low 314 |  |  | 405.15 | 7 ac | CA409.0890L | 5 intermediate | N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS,SC,AMM AMM,Pb,EUT AMM,ODORS,TRASH ODORS,TRASH |  |  | DO;TIS(Cd)Fully but threatened pH |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Park runoff (m) Ducks (m) Recreational use (m) |  |  |
| 78 11-30 20 ± 5 | 68 7.2-9.1 8.1 ± 0.4 | 78 0.1-13.7 6.9 ± 3.3 | 28 419-701 619 ± 65 | 28 771-1045 962 ± 74 | no data (300) | no data | 28 57-93 79 ± 12 | 28 183-280 220 ± 29 | Tissue ('91): No organic chemicals at elevated levels 3 Tissue ('92): Cd(EDL85) 3 | 28 ND-1.14 0.34 ± 0.32 | 28 ND-0.33 0.09# | no data | 28 As 5.5 Cd 1.6 Cu 61 Pb 94 Zn 13 | no data |
| Macarthur Park Lake |  |  | 405.15 | 23 ac | CA409.0900L |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Echo Park Lake 303(d)-low 314 |  |  | 405.15 | 2 ac | CA409.0910L | 15 intermediate | N N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | AMM,Cu,Pb,EUT,TIS(PCBs) AMM,pH,ODORS,TRASH,ALGAE ODORS,TRASH,ALGAE TIS(PCBs) |  |  | pH |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Algae treatment (m) Recreational use (m) |  |  |
| 80 10-29 21 ± 6 | 69 7.0-9.4 8.5 ± 0.5 | 80 1.9-15.6 9.1 ± 2.6 | 31 249-453 345 ± 58 | 31 417-795 580 ± 105 | no data (300) | no data | 31 36-94 62 ± 16 | 31 51-144 89 ± 25 | Tissue ('91): No organic chemicals at elevated levels 3 Tissue ('92): PCBs(MTRLs) 3 | 31 ND-0.71 0.11# | 31 ND-0.2 0.04# | no data | 31 Cu 105 Pb 105 Se 3 Zn 14 | 16+ a,p,s,m,v,w,y,z,ag,ay,ba,aj,al,aa,ab,au,av,ax,bo,bp |
| Elysian Park Lake |  |  | 405.15 | 0.002 | CA409.0920L |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Arroyo Seco R1 (LA River to West Holly Ave) and R2 (West Holly Ave. to Devils Gate Dam) 303(d) -low 319 |  |  | 405.15 405.31 | 7.02 and 2.53 | CA409.0930R and CA409.0970R | 1 impaired 7 intermediate 10 unknown | F N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,AMM* AMM*,COLIFORM*, TRASH,ALGAE,COLOR TRASH, ALGAE |  |  |  |  |  | Natural sources (m) Urban runoff (m) Transient encampments (m) Illegal dumping (s) |  |  |
| 9 11-29 19±6 | 11 6.8-8.8 8.1±0.5 | 7 7.9-13 9.7±1.6 | 11 301-788 562±151 | 4 552-1200 899±241 | 4 184-453 317±95 | 4 ND-0.1 0.03±0.04 | 11 32-114 80±28 | 10 55-270 175±69 | no data | 10 ND-3 .5±0.9 | 10 1.8-6.5 3.7±1.5 | 7 ND-30000 | 4 Ba 176 Cd 3 Cu 20 Pb 43 Zn 133 | H2O(4)ND (2)be |
| Eagle Rock Reservoir |  |  | 405.25 | 0.012 | CA409.0960L |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Devils Gate Reservoir (empty) |  |  | 405.32 | 0.183 | CA409.0990L |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Arroyo Seco R3 (upstream of Devils Gate Reservoir) |  |  | 405.32 | 13.67 | CA409.1080R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| 3 16-24 18±3 | 16 7.9-8.8 8.2±0.2 | 3 8.6-9.3 8.9±0.3 | 17 170-490 277±65 | 19 260-625 454±89 | 13 130-234 194±27 | no data | 14 5-18 13±4 | 14 1-160 35±36 | no data | no data | 15 0.1-2 0.8±0.6 | 6 ND-500 | 0-2 Ba 47 Cu and Zn (14-15) Cu 30 Zn 30 | no data |
| Silverlake Reservoir |  |  | 405.15 | 0.115 | CA409.1340L | 78 good |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Ivanhoe Reservoir |  |  | 405.15 | 0.011 | CA409.1350L |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Los Angeles River R3 (Figueroa St. to Riverside Dr.) 303(d) -low 319 |  |  | 405.21 | 7.24 | CA409.1360R | 18 interm 7 impaired | FT N N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,SC*,AMM AMM AMM,TRASH,ALGAE,ODOR, COLOR, TRASH,ALGAE,ODOR |  |  | TIS(Ag) Fully but threatened SCUM SCUM |  |  | Natural sources (m) POTW effluent (m) Illegal dumping (s) Urban runoff (m) |  |  |
| 20 12-28 19±4 | 22 7.1-8.1 7.5±0.2 | 14 7.8-11 8.9±1 | 30 83-844 631±185 | 7 700-1238 1017±202 | 7 216-295 257±26 | 8 0.4-0.7 0.5±0.1 | 27 5-172 114±33 | 25 27-280 189±52 | Tissue('95): Ag(EDL95) 3 | 20 2.2-14.9 9.1±2.7 | 20 3.1-7.6 4.5±1.1 | 12 110-800 | 7-17 As 7 Ba 65 Cd 3 Cr 28 CrVI 20 Cu 10 Pb 22 Zn 74 Hg 0.7 | H2O(24-28): b,c,d,e,g,h,i,j,l,n,p,r,s,t,u,v,x,be,bg,bh,bi,bj,bk |
| Sycamore Canyon (South Branch) |  |  | 405.21 | 7.75 | CA409.1370R |  | FT F N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,AMM* AMM*,COLIFORM* COLIFORM* |  |  |  |  |  | Natural sources (m) Illegal dumping (s) Urban runoff (m) |  |  |
| 8 7-23 14±5 | 8 7.3-8.8 7.8±0.4 | no data | 8 548-730 605±52 | 8 867-1023 938±46 | 8 237-358 297±33 | 8 0.04-0.13 0.1±0.04 | 8 72-99 84±8 | 8 145-220 184±22 | no data | 8 ND-0.3 0.12±0.09 | 8 1.2-4.5 2.4±1.1 | 8 280-9E5 | 8 As 3 Ba 80 Cu 20 Pb 40 Zn 250 | H2O(8):a,be |
| Verdugo Wash R1 (LA River to Verdugo Rd) and R2 (upstream of Verdugo Rd) 303(d) -low |  |  | 405.21 405.24 | 3.41 and 5.55 | CA409.1390R and CA409.1420R |  | FT F N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,AMM* AMM*,COLIFORM*,TRASH,ALGAE,COLOR COLIFORM*,TRASH,ALGAE |  |  |  |  |  | Natural sources (m) Illegal dumping (s) Urban runoff (m) |  |  |
| 6 10-24 18±5 | 9 7.3-9.2 8.2±0.6 | 7 8.6-11 9.4±0.8 | 7 468-726 600±85 | 2 900-1003 952±52 | 2 310-356 333±23 | 2 ND | 9 60-133 91±22 | 8 101-191 149±31 | no data | 8 ND-1.3 0.3±0.4 | 8 1.1-3.8 2.6±0.8 | 6 ND-30000 | 2 Ba 224 | H2O(2-ND) |
| Los Angeles River R4 (Riverside Dr. to Sepulveda Dam) 303(d) -low 319 |  |  | 405.21 | 11.84 | CA409.1620R |  | FT N N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,SC,AMM AMM,Pb AMM,COLIFORM,TRASH,SCUM,ALGAE,ODOR,COLOR COLIFORM,TRASH,SCUM,ALGAE,ODOR |  |  | CHL,SO4,N+N |  |  | Natural sources (m) POTW effluent (m) Illegal dumping (s) Urban runoff (m) Spills (s) |  |  |
| 89 4-28 18±5 | 97 6.7-8.6 7.7±0.4 | 20 8-40 12.2±6.7 | 117 56-1146 632±195 | 74 604-1580 1078±163 | 74 116-550 260±90 | 77 ND-1 0.6±0.2 | 112 6-221 124±38 | 109 12-484 207±90 | no data | 95 ND-34.9 10.7±4.8 | 92 0.03-20.42 4.7±3.9 | 86 ND-90000 | 69-103 As 71 Ba 225 Cd 10 Cr 30 CrVI 14 Cu 70 Pb 120 Se 10 Zn 250 Hg 1 Ni 60 | H2O(42): a,b,c,d,e,g,k,m,n,p,q,r,s,t,u,v,w,y,z,bi,av,bl,bm (71) be |
| Burbank Western Channel 303(d) -low |  |  | 405.21 | 6.35 | CA409.1630R | 6 interm | N N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,AMM* AMM*,Cd* AMM*,TRASH,SCUM,ALGAE TRASH,SCUM,ALGAE |  |  | ODOR,COLOR ODOR |  |  | Natural sources (m) POTW effluent (m) Illegal dumping (s) Urban runoff (m) Spills (s) |  |  |
| 10 10-26 19±4 | 11 7.2-7.8 7.5±0.2 | 7 9-13 10.2±1.3 | 10 515-987 692±126 | 4 900-1191 1033±134 | 4 184-340 272±56 | 4 0.3-0.6 0.5±0.1 | 12 101-182 133±24 | 11 82-270 188± | no data | 11 8.3-16.3 12±2.7 | 11 0.4-11.7 3.9±3.0 | 6 ND-10 | 3 Ag 3 Ba 125 Cd 3 Cr 15 Cu 28 Pb 31 Zn 80 | H2O(5): b,c,d,e,f,n,s,be |
| Tujunga Wash (LA River to Hansen Dam) 303(d) -low |  |  | 405.21 | 9.68 | CA409.1920R | 10 interm | FT N N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | pH*,TDS*,AMM* AMM*,Cu* AMM*,COLIFORM*,TRASH,COLOR TRASH |  |  | SCUM,ODOR SCUM,ODOR |  |  | Natural sources (m) Illegal dumping (s) Urban runoff (m) |  |  |
| 6 11-30 18±6 | 8 7.3-8.9 8.3±0.5 | 6 7-11 9.2±1.4 | 7 224-1358 663±322 | 2 1000-1766 1383±383 | 2 206-275 241±35 | 2 0.3-1.9 1.1±0.8 | 9 11-339 135±110 | 8 31-280 112±74 | no data | 7 ND-2.4 0.6±0.8 | 7 ND-0.22 0.09±0.1 | 5 ND-2300 | 3 As 21 Ba 136 Cd 6 Cu 101 Zn 97 | H2O(3) be |
| Pacoima Wash South Branch (downstream of Pacoima Spreading Grounds) |  |  | 405.21 | 4.02 | CA409.1930R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| 1 17 | no data | no data | no data | no data | no data | no data | no data | no data | no data | no data | no data | no data | 2 As 19 Ba 65 Cu 22 Zn 88 | H2O(3) d |
| Pacoima Wash R1 (Pacoima Spreading Grounds to Lopez Flood Control Basin) and R2 (Lopez Flood Control Basin to Pacoima Dam) |  |  | 405.21 405.21 | 3.41 and 2.85 | CA409.2070R and CA409.2110R | 14 interm | F F F F | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  |  |  |  |  |  |  |  |  |  |
| 2 27 27 | 4 8.1-8.2 8.1±0.1 | 1 8.1 | 13 220-590 345±96 | 10 300-690 512±103 | 10 160-320 236±45 | no data | 11 4-18 10±4 | 11 29-110 75±24 | no data | no data | 11 ND-0.5 0.18±0.19 | 5 ND-9 | 2 Ba 98 Se 8 Cu and Zn(11) ND | no data |
| Pacoima Reservoir |  |  | 405.22 | 0.092 | CA409.2180L | 68 unknown |  |  |  |  | Unassessed |  |  |  |  |  | Urban runoff (m) |  |  |
| Pacoima Canyon Creek R3 (upstream of Pacoima Reservoir) |  |  | 405.22 | 19.02 | CA409.2210R |  |  |  |  |  | Unassessed |  |  |  |  |  | Urban runoff (m) |  |  |
| Hansen Lake |  |  | 405.21 | 0.184 | CA409.2440L |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Middle Lake |  |  | 405.21 | 0.005 | CA409.2450L |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Little Tujunga Canyon Creek |  |  | 405.23 | 7.16 | CA409.2490R |  | F F F F | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,CHL*,SO4* |  |  |  |  |  |  |  |  |
| 2 27-29 28±1 | no data | no data | 10 410-623 497±76 | 7 500-943 763±147 | 7 253-408 328±59 | no data | 7 19-56 37±13 | 7 109-207 155±33 | no data | no data | 7 ND-1.13 0.39±0.37 | 2 11-105 | Cu and Zn(7) Zn 11 | no data |
| Big Tujunga Canyon Creek R1 (Hansen Flood Control Basin to Big Tujunga Reservoir) |  |  | 405.23 | 15.13 | CA409.2680R |  | F F F F | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  |  |  |  |  |  |  | Resource extraction (s) |  |  |
| 3 16-29 23±5 | 9 8-9.1 8.4±0.3 | 3 7.6-9.8 8.7±0.9 | 15 208-590 315±96 | 11 270-650 467±105 | 10 130-254 194±38 | 1 ND | 12 6-14 10±2 | 12 19-67 38±14 | no data | no data | 11 ND-1.28 0.39±0.37 | 5 ND-52 | 2-3 Ba 82 Pb 9 Cu and Zn(11) ND | no data |
| Big Tujunga Reservoir |  |  | 405.23 | 0.141 | CA409.3060L | 84 unknown |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Big Tujunga Canyon Creek R2 (upstream of Big Tujunga Reservoir) |  |  | 405.23 | 7.58 | CA409.3140R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Lucas Creek |  |  | 405.23 | 2.12 | CA409.3180R |  | F F F F | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  |  |  |  |  |  |  |  |  |  |
| 2 19-20 20±1 | 12 7.4-8.7 8.3±0.3 | 2 8.3-9.1 8.7±0.4 | 15 200-420 262±51 | 12 270-470 397±50 | 12 125-219 171±22 | no data | 12 7-18 11±3 | 12 15-33 23±5 | no data | no data | 12 ND-2 0.27±0.54 | 3 2-8 | 1 Ba 82 Pb 4 Cu and Zn (12) Zn 10 | no data |
| Upper Big Tujunga Canyon Creek |  |  | 405.23 | 6.35 | CA409.3600R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Los Angeles River R5 (within Sepulveda Basin) 303(d) - low 319 |  |  | 405.21 | 1.93 | CA409.3650R | 18 interm 7 impaired | FT N N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*,SC*,AMM*,SO4* AMM, TIS(ChemA) TRASH,SCUM,ALGAE,ODOR,COLOR TRASH,SCUM,ALGAE,ODOR SO4* |  |  | TIS(Ag,Chloropyritos) OIL OIL |  |  | Natural sources (m) POTW effluent (m) Illegal dumping (s) Urban runoff (m) Recreational use (s) |  |  |
| 8 7-24 18±5 | 11 6.5-7.8 7.4±0.5 | 7 7-9.4 8±1 | 8 611-1260 822±224 | 4 890-1169 1011±101 | 4 172-301 224±48 | 4 0.5-1.2 0.8±0.3 | 11 93-176 132±23 | 11 142-632 336±177 | Tissue('91): no elevated levels of organic chemicals 3 Tissue('92): Ag(EDL95), chloropyritos(EDL85), ChemA(NAS) 3 | 10 2.2-20.1 8.8±6 | 10 0.5-15.7 3.8±4.1 | 6 300-1700 | 4-5 As 16 Ba 72 Cd 2 Cu 10 Se 7 Zn 61 | H2O(8): b,c,d,e,f,g,n,r,t,u Sed(1)ND |
| Encino Reservoir |  |  | 405.21 | 0.209 | CA409.3700L |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Bull Creek |  |  | 405.21 | 8.78 | CA409.3730R | 10 interm | FT F F F | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS* |  |  |  |  |  | Natural sources (m) Urban runoff (m) |  |  |
| 1 14 | 3 7.6-8.5 8.1±0.4 | no data | 4 500-953 660±181 | 3 796-1047 881±117 | 3 253-484 342±101 | 3 0.2-0.9 0.5±0.3 | 4 76-116 97±18 | 4 134-370 209±95 | no data | 1 1.1 | 3 ND-2 1±0.8 | no data | 4 As 14 Ba 117 Cd 2 Cu 15 | H2O(1-5) n,y,z,be Sed(1) ba |
| Los Angeles Reservoir |  |  | 405.21 | 0.257 | CA409.3790L | 448 good |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Los Angeles River R6 (upstream of Sepulveda Flood Control Basin) 303(d) -low 319 |  |  | 405.21 | 6.17 | CA409.3890R | 18 interm 7 impaired | N F N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SO4 TDS,SC*,SO4,AMM*, 1,1-DCE,PCE,TCE AMM*,COLIFORM* COLIFORM* TDS,CHL,SO4 |  |  |  |  |  | Rising contaminated groundwater (m) Natural sources (m) POTW effluent (m) Illegal dumping (s) Urban runoff (m) Transient encampments (m) |  |  |
| 17 10-33 21±6 | 21 7-8.7 8.1±0.4 | 14 9-18 13±3 | 28 123-1960 1213±458 | 4 1268-1700 1407±172 | 6 355-775 607±132 | 5 0.63-0.8 0.5±0.2 | 28 12-208 145±44 | 28 67-1030 596±226 | no data | 17 ND-2 0.3±0.5 | 19 0.8-8.6 3.4±2 | 13 170-9000 | 6-16 As 6 Ba 115 Cd 16 Cu 22 Pb 13 Se 14 Zn 11 Hg 0.65 | H2O(2-32): a,b,c,d,e,f,g,h,i,j,k,l,m,n,p,r,t,u,v,w,be,bf,bn |
| Aliso Canyon Wash 303(d) -low |  |  | 405.21 | 10.13 | CA409.3930R | 15 interm | FT N N N | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS* Se* SO4* |  |  | COLOR |  |  | Natural sources (m) Unknown sources (m) Urban runoff (m) |  |  |
| 2 13-25 19±6 | 4 7.9-8.6 8.3±0.2 | no data | 5 592-978 768±155 | 3 878-1114 964±106 | 4 308-465 378±61 | 3 0.3-0.8 0.5±0.2 | 5 76-196 128±42 | 5 183-324 245±61 | no data | 2 0.2-1.2 0.7±0.5 | 4 2.1-4.2 3.1±0.8 | 1 1600 | 4 As 10 Ba 113 Cd 2 Cu 12 Se 9.3 Zn 65 | H2O(2-3) be |
| Browns Canyon Wash R1 (Confluence LA River to Reservoir) and R2 (upstream of Reservoir) |  |  | 405.21 405.21 | 7.56 and 2.44 | CA409.4060R and CA409.4190R | 2 interm | FT F F F | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS* |  |  |  |  |  | Natural sources (m) Urban runoff (m) |  |  |
| 2 14-24 19±5 | 3 7.5-8.8 8.2±0.5 | 0 | 4 305-1339 769±370 | 2 1100-1350 1225±125 | 3 135-830 421±294 | 2 0.3-0.4 0.4±0.1 | 4 52-200 114±55 | 4 72-500 248±166 | no data | no data | 3 ND-1.8 0.7±0.8 | 1 ND | 3 Ba 96 Cd 2 Cu 16 Se 6 | H2O(4) r,s |
| Arroyo Calabasas |  |  | 405.21 | 3.30 | CA409.4230R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| 2 16-25 20±5 | 2 8.2-8.3 8.3±0.1 | 1 17 | 2 1960-2490 2225±265 | no data | 1 1010 | no data | 2 217-240 229±12 | 2 947-1270 1109±162 | no data | 2 0.1-0.3 0.2±0.1 | 2 0.2-1 0.6±0.4 | 2 500-1700 | no data | H2O(1-ND) |
| Lake Calabasas 303(d)-low |  |  | 405.21 | 28 ac | CA409.4240L | 21 interm | FT N N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | pH AMM,TIS(DDT),EUT AMM,ODORS ODORS TIS(DDT), |  |  | DO,pH,TIS(Cd,Cu,Zn) |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Algae treatment (m) Ducks (m) |  |  |
| 92 9-32 21 ± 6 | 85 7.4-9.3 8.6 ± 0.4 | 92 0.2-15.7 8.7 ± 3.3 | 28 322-561 433 ± 79 | 28 565-956 759 ± 139 | no data (1010) | no data | 28 78-167 126 ± 32 | 28 51-116 83 ± 20 | Tissue ('91): Cu(EDL85) 3 Tissue ('92): DDT(MTRLs), Cd(EDL85), Cu(EDL95), Zn(EDL95) 3 | 28 ND-0.45 0.06# | 28 ND-0.1 0# | no data | 28 Cu 92 Pb 67 Se 2 Zn 11 | no data |
| Bell Creek 303(d) -low |  |  | 405.21 | 9.81 | CA409.4300R |  | N F N F | Drinking: Primary Drinking: Secondary Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SO4* TDS*,SO4* COLIFORM* TDS*,CHL*,SO4* |  |  | COLIFORM* |  |  | Industrial point source (m) Natural sources (m) Urban runoff (m) |  |  |
| 3 16-25 21±4 | 3 8.1-8.3 8.2±0.1 | 1 21 | 3 1320-1470 1410±65 | no data | 2 665-780 723±58 | no data | 3 141-238 193±40 | 3 326-640 526±142 | no data | 2 0.1-0.2 0.2±0.1 | 3 0-3.5 2.1±1.5 | 3 ND-8000 | no data | H2O(1-ND) |
| Chatsworth Creek R1 (downstream of Hidden Lake) |  |  | 405.21 | 1.76 | CA409.4310R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Chatsworth Reservoir |  |  | 405.21 | 849 ac | CA409.4360L |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |

San Gabriel River Watershed

| Reach/location (San Gabriel) |  |  | Hydro Unit | Size | Code | Prev WQA | WQA | Assessed Uses |  |  | Not supporting |  |  | Partially supporting |  |  | Potential Sources |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temp | pH | DO | TDS | SC | Hard | B | Chl | SO4 | Tissue, Sediment and Toxicity Data | AMM | N+N | Fec Col | metals | VOCs |
| San Gabriel River Estuary 303(d) low 319 |  |  | 405.15 | 2.95 | CA410.0000E |  | N N F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  | TIS(As) COLOR TIS(As) |  |  | TOX, TIS(Cu, Ag,Cr),HIST Turb |  |  | POTW effluent (m) Urban runoff (m) Spills (s) Illegal dumping (s) |  |  |
|  |  |  |  |  |  |  |  |  | Tissue ('90): Cu(EDL95), Ag(EDL95)3 Tissue ('91): Cr(EDL85), Ag(EDL85) 3 Tissue ('92): As(MTRLs), Cu (EDL85), Ag(EDL95) 3 Tissue ('93): Cu(EDL85), As(MTRLs) 3 Wat Bio Toxicity: poor survival rates 5 |  |  |  |  |  |
| Coyote Creek 303(d) |  |  | 405.15 | 13.45 | CA410.0010R | 17 inter | N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,SC,AMM AMM,Pb,TOX, HIST AMM,COLIFORM,ALGAE,COLOR COLIFORM, ALGAE NA |  |  | CHL,TIS(Cr,Cu,Ag) Turb |  |  | Natural sources (m) POTW effluent (m) Urban runoff (m) Spills (s) Illegal dumping (s) |  |  |
| 134 7-31 19 ± 6 | 144 7.1-9.9 8.0 ± 0.4 | 1 13.8 | 139 320-1922 925 ± 240 | 138 482-2600 1428 ± 298 | 139 134-632 331 ± 110 | 139 ND-3.7 0.4# | 139 53-405 182 ± 58 | 138 91-583 262 ± 100 | Tissue ('92): Cr(EDL85), Cu (EDL85), Ag(EDL95) 3 Wat Bio Toxicity: poor survival rates 5 | 143 ND-32 5.1+6 | 140 ND-17.8 3.5 ± 3.0 | 71 ND-240000 | 139-143 Ag 30 As 74 Ba 800 Cd 10 Cr 90 CrVI 20 Cu 90 Pb 310 Se 14 Zn 770 Ni 30 | 16 d,g,ad, be scanned 138 times and detected above detection limit |
| Fullerton Drain/Creek |  |  |  | 12.88 | CA410.0050R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |
| Coyote Creek North Fork |  |  |  | 3.75 | CA410.0080R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |
| San Gabriel River R1 (Estuary to Firestone) 303(D) low |  |  | 405.15 | 8.73 | CA410.0230R | 9 impaired | N N P | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,SC,AMM AMM,Pb,TOX,HIST AMM,COLIFORM,COLOR |  |  | ALGAE,Turb COLIFORM, ALGAE, Turb |  |  | Natural sources (m) POTW effluent (m) Urban runoff (m) Spills (s) Illegal dumping (s) |  |  |
| 72 13-30 22 ± 4 | 76 6.6-8.9 7.9 ± 0.5 | no data | 70 204-1456 763 ± 134 | 70 351-2290 1238 ± 217 | 70 132-410 243 ± 36 | 70 ND-1.2 0.5 ± 0.2 | 70 22-291 167 ± 37 | 70 39-450 171 ± 47 | Wat Bio Toxicity: poor survival rates 5 | 77 ND-21.1 10.1 ± 4.1 | 69 0.79-26.5 6.9 ± 5.8 | 68 ND-50000 | 70-75 Ag 130 As 22 BA 219 Cd 20 Cr 20 CrVI 24 Cu 100 Pb 100 Se 17 Zn 340 Ni 30 | 17 b,d,f,g,k,n,r,t,v,y,z be scanned 69 times and detected above detection limit |
| El Dorado Lakes 303(d)-low 314 |  |  | 405.15 | 220 ac | CA410.0230L | 220 interm | N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | pH,AMM AMM,Cu,Pb,EUT,TIS(Hg) AMM,pH,ALGAE ALGAE |  |  | pH,TIS(Cr) |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Nutrient-rich sediments (m) Algae treatment (m) |  |  |
| 126 12-30 22 ± 6 | 116 6.9-9.4 8.5 ± 0.6 | 126 0.2-25.6 7.0 ± 4.6 | 45 380-496 449 ± 26 | 45 673-832 753 ± 44 | no data (240) | no data | 45 65-83 75 ± 6 | 45 ND-70 40 ± 17 |  | 45 ND-1.92 0.30# | 45 ND-0.3 0.03# | no data | 45 As 28 Cu 99 Pb 108 Zn 21 Ni 11 | 29+ a,m,p,s,t,v,w,y,z,ag,aj,al,an,ao,aa,ac,ad,au,av,aw,ax,ay,az,ba,bb,bo,bp |
| San Gabriel River R2 (Firestone to Whittier Narrows Dam) 303(d) low |  |  | 405.15 | 9.99 | CA410.0250R | 9 interm | FT N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS, SC,AMM AMM,Pb AMM,COLIFORM COLIFORM |  |  | N+N |  |  | Natural sources (m) POTW effluent (m) Urban runoff (m) Spills (s) Illegal dumping (s) |  |  |
| 46 9-27 19 ± 5 | 50 6.2-8.8 7.8 ± 0.5 | no data | 50 164-760 584 ± 120 | 49 269-1196 918 ± 191 | 50 116-330 240 ± 50 | 49 ND-17 5 ± 4.8 | 50 7-150 102 ± 29 | 50 20-335 150 ± 52 |  | 49 ND-16.5 5.2 ± 4.8 | 50 0.38-12.6 4.2 ± 3.6 | 48 ND-13000 | 49-50 As 22 Ba 198 Cd 10 Cr 10 CrVI 24 Cu 140 Pb 110 Se 20 Zn 120 Ni 103 | 2-9 f,g,n,r,af be scanned 47 times and detected above detection limit |
| Legg Lake 303(d)-low |  |  | 405.41 | 40 ac | CA410.0310L | 70 good | FT N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | pH Cu,Pb ODORS,TRASH ODORS,TRASH |  |  | AMM,pH |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Algae treatment (m) Ducks (m) Recreational use (m) |  |  |
| 91 11-29 21 ± 6 | 84 7.6-8.9 8.3 ± 0.3 | 91 5.8-14.6 11.1 ± 1.7 | 43 179-248 207 ± 15 | 43 295-386 327 ± 29 | no data (240) | no data | 43 18-26 22 ± 2 | 43 39-55 46 ± 4 | Tissue ('91): No organic chemicals at elevated levels 3 Tissue ('92): No organic chemicals at elevated levels 3 | 43 ND-0.35 0.05# | 43 ND-0.2 0.02# | no data | 43 Cu 97 Pb 70 Zn 134 | 13+ a,m,s,v,w,y,z,ac,av,ax,ay,az,ba,bb,bp |
| San Gabriel River R3 (Whittier Narrows to Ramona) 303(d) low |  |  | 405.41 | 3.52 | CA410.0320R | (42)interm | FT N P F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS, SC TOX |  |  | TURB |  |  | Natural sources (m) Urban runoff (m) Spills (s) Illegal dumping (s) Municipal effluent (m) Poultry farm (s) |  |  |
| 27 9-26 16 ± 4 | 28 7.3-8.8 8.0 ± 0.4 | 26 3.8-12.9 9.0 ± 2.1 | 27 201-888 572 ± 156 | 27 300-1240 875 ± 235 | 27 120-470 247 ± 74 | 1 0.2 | 27 4-224 95 ± 45 | 27 24-265 157 ± 66 | Wat Bio Toxicity: reduced survival rates 5 | 2 1.8-3.2 2.5# | 1 0.9 | no data | 2 Ba 128 Cd 2 | 2 ND |
| San Jose Creek R1 (SG confl. to ~Temple Street) and R2 (~Temple to ~I-10 at White Ave) 303(d) low |  |  | 405.41 and 405.51 | 13.12 and 4.93 | CA410.0330R and CA410.0460R | (16) interm | FT N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS,SC,SO4,AMM AMM,Pb AMM,COLIFORM,COLOR COLIFORM TDS,SO4 |  |  | TOX pH, ALGAE ALGAE CHL |  |  | Natural sources (m) POTW effluent (m) Urban runoff (m) Fuel/oil Spills (s) Illegal dumping (s) |  |  |
| 68 9-27 17 ± 4 | 73 6.9-9.1 8.1 ± 0.4 | 1 8.4 | 73 591-1120 840 ± 129 | 71 780-1810 1235 ± 182 | 72 286-610 436 ± 91 | 73 ND-1.2 0.25 ± 0.19 | 73 65-292 130 ± 38 | 73 180-435 282 ± 52 | Wat Bio Toxicity: reduced survival rates 5 | 74 ND-10.3 2.0 ± 3 | 72 ND-12.1 2.97# | 68 ND-16000 | 70-72 As 11 Ba 282 Cd 30 Cr 150 CrVI 28 Cu 90 Pb 170 Se 13 Zn 190 Hg 2 Ni 120 | 4 s be scanned 69 times and detected above detection limit |
| Puente Creek |  |  | 405.41 | 5.76 | CA410.0340R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Walnut Creek Wash (goes to Puddingstone res) 303(d) low |  |  | 405.41 | 13.9 | CA410.0590R | 13 interm | FT N N F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TDS*, AMM* pH*,TOX AMM*,pH* |  |  |  |  |  | Natural sources (m) POTW effluent (m) Urban runoff (m) Fuel/oil Spills (s) Illegal dumping (s) |  |  |
| 3 14-29 22 ± 6 | 4 8.0-10.6 9.2 ± 1.0 | 1 10.7 | 4 471-629 581 ± 64 | 2 756-933 844# | 3 136-270 224 ± 62 | 4 ND-0.2 0.13 ± 0.08 | 4 82-106 94 ± 9 | 4 112-230 189 ± 46 | Wat Bio Toxicity: poor survival rates 5 | 3 0.2-1.6 1.03 ± 0.6 | 3 0.3-7 3.1 ± 2.9 | no data | 1-3 Ba 145 Cu 14 | 3 ND |
| Big Dalton Wash R1 (to lower reservoir) |  |  | 405.41 | 10.42 | CA410.0600R |  |  |  |  |  | Unassessed |  |  |  |  |  | Urban runoff (m) |  |  |
| Little Dalton Wash Creek R1 (below res) |  |  | 405.41 | 6.96 | CA410.0630R |  |  |  |  |  | Unassessed |  |  |  |  |  | Urban runoff (m) |  |  |
| Little Dalton Reservoir |  |  | 405.41 | 8.6 | CA410.0670L |  |  |  |  |  | Unassessed |  |  |  |  |  | Urban runoff (m) |  |  |
| Little Dalton Cyn Creek R2 (abv res) |  |  | 405.41 | 3.96 | CA410.0680R |  |  |  |  |  | Unassessed |  |  |  |  |  | Urban runoff (m) |  |  |
| San Dimas Wash R1 (Big Dalton to lower San Dimas Reservoir at ext of Oak Knoll) |  |  | 405.41 | 7.93 | CA410.0690R |  |  |  |  |  | Unassessed |  |  |  |  |  | Urban runoff (m) |  |  |
| San Dimas Creek R2 (Ham Cyn to San Dimas Reservoir) |  |  | 405.44 | 3.30 | CA410.0720R |  |  |  |  |  | Unassessed |  |  |  |  |  | Urban runoff (m) |  |  |
| San Dimas Reservoir |  |  | 405.44 | 550 ac | CA410.0760L | 500 good |  |  |  |  | Unassessed |  |  |  |  |  | Urban runoff (m) |  |  |
| San Dimas Wash R3 (abv res) |  |  | 405.44 | 6.67 | CA410.0770R |  | F F F F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | FULLY SUPPORTING |  |  |  |  |  |  |
| 2 22-25 241 | 8 8.0-8.6 | 2 8.2-9.3 | 11 260-2710 | 8 390-610 | 8 200-265 | no data | 8 11-21 | 8 21-60 | 3 5-51 | no data | 8 0.3-1.0 | no data | 1 Ba 51 Cu and Zn (7) ND | no data |
| Big Dalton Wash R2 (between reservoirs) |  |  | 405.41 | 1.85 | CA410.0980R |  |  |  |  |  | Unassessed |  |  |  |  |  | Urban runoff (m) |  |  |
| Big Dalton Reservoir |  |  | 405.41 | 21.9 ac | CA410.1040L |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Big Dalton Wash R3 (abv reservoir) |  |  | 405.41 | 2.99 | CA410.1050R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Puddingstone Reservoir 303(d)-low 314 319 |  |  | 405.52 | 382 ac | CA410.1180L | 490 impaired | F F N N F N | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | DO,TIS(PCBs,chlordane, Hg,DDT) AMM TIS(PCBs,chlordane, Hg,DDT) |  |  | TIS(dacthal, oxadiazon, As) pH |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Recreational use (m) |  |  |
| 187 10-28 18 ± 6 | 159 6.6-8.7 7.5 ± 0.5 | 187 0.1-14.9 4.3 ± 3.5 | 75 177-245 214 ± 18 | 75 266-451 375 ± 46 | no data | no data | 75 10-44 27 ± 13 | 75 15-49 34 ± 10 | Tissue ('91): chlordane(MTRLs), PCBs(MTRLs), Dacthal(EDL85), oxadiazon(EDL95), As(EDL85), Hg(MIS) 3 Tissue ('92): Chlordane(MTRLs), DDT(MTRLs), PCBs(MTRLs), As(EDL85), oxadiazon(EDL95) 3 | 75 ND-5 0.29# | 75 ND-2 0.6# | no data | 75 Cu 130 (pb not scanned) | 43+ a,m,p,r,s,t,v,w,y,z,ag,ai,al,am,ap,as,aa,ad,ae,au,av,aw,ax,ay,az,ba,bb,bo,bp |
| Live Oak Channel/Wash R1 (Puddingstone to ~Foothill) |  |  | 405.52 | 4.45 | CA410.1190R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Santa Fe Dam Park Lake 303(d)-low |  |  | 405.41 | 70 ac | CA410.1370L |  | FT N N F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | pH Pb, Cu AMM,pH |  |  | pH |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Recreational use (m) |  |  |
| 102 12-29 21 ± 6 | 95 7.5-9.6 8.7 ± 0.3 | 102 0.5-12.6 10.6 ± 2.0 | 37 181-346 259 ± 38 | 37 370-484 426 ± 35 | no data (150) | no data | 37 52-84 66 ± 10 | 37 30-54 45 ± 6 | Tissue ('92): No organic chemicals at elevated levels 3 | 37 ND-0.4 0.7# | 37 ND-0.2 0.02# | no data | 37 Cu 56 Pb 51 Zn 65 | no data |
| San Gabriel River R4 (Ramona to Santa Fe Spreading Basin Dam) and R5 (Santa Fe Spreading basin Dam to ~ext Millbrae St) and R6 (~ext Millbrae St to Royal Oaks Drive Ext.) and R7 (Ext Royal Oaks Drive to Morris Dam) |  |  | 405.41 405.41 405.42 405.43 | 3.67 and 2.0 and 1.29 and 5.57 | CA410.1330R and CA410.1350R and CA410.1410R and CA410.1430R | 42 interm | F F F FT F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | (Cd (n=1) IS ABV STND) Turb |  |  | COLOR |  | Natural runoff (m) Urban runoff(m) Resource extraction (m) |  |  |
| 3 11-17 14 ± 3 | 5 7.7-9.1 8.3 ± 0.5 | 1 10.3 | 4 194-410 287 ± 86 | 2 335-600 468 ± 133 | 4 132-170 149 ± 15 | 2 ND | 4 9-105 54 ± 43 | 4 19-78 42 ± 20 |  | 2 0.8-1.1 0.95 ± 0.15 | 3 0.6-1.6 1.0 ± 0.4 | no data | 2 Ba 67 Cd 2 Cu 13 | 2 ND |
| Morris Reservoir |  |  | 405.43 | 442 | CA410.1500L | 420 unknown | F F F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  |  |  |  | B |  | Former military testing |  |  |
| 36 4-23 12 ± 4 | 46 7.0-19.0 8.4 ± 2.0 | 39 4.3-12.2 9.9 ± 1.7 | 41 140-438 215 ± 60 | 40 258-822 361 ± 96 | 31 140-246 176 ± 29 | 42 ND-1.6 0.2# | 46 ND-95 7# | 46 0.4-161 29 ± 25 |  | 1 0.7 | need | no data | 1 Ba 100 | 1 ND |
| San Gabriel River R8 (Morris reservoir to San Gabriel Reservoir) |  |  | 405.43 | 0.95 | CA410.1560R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| San Gabriel Reservoir |  |  | 405.43 | 820 ac | CA410.1590L | 500 good |  |  |  |  |  |  |  |  |  |
| San Gabriel River West Fork R1 (to Cogswell res) |  |  | 405.43 | 8.18 | CA410.1660R | 18 good | F F FT FT | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | SO4* |  |  | TRASH: fully but threatened TRASH: fully but threatened |  |  | Recreational use (m) Illegal dumping (m) |  |  |
| 4 7-18 12 ± 5 | 11 7.4-8.9 8.1 ± 0.4 | 1 9.0 | 9 45-300 182 ± 67 | 6 186-460 326 ± 95 | 7 90-183 148 ± 31 | 3 ND | 9 3-8 5 ± 2 | 9 11-150 41 ± 42 |  | no data | 8 ND-0.45 0.16# | 3 ND-4 | 2-4 Ba 69 | no data |
| San Gabriel River North Fork |  |  | 405.43 | 10.0 | CA410.1680R |  | F F FT FT | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  |  |  |  | COLOR; TRASH: fully but threatened TRASH: fully but threatened |  |  | Recreational use (h) Illegal dumping (m) |  |  |
| 4 7-15 11 ± 3 | need | 1 9.6 | 10 180-245 219 ± 19 | 7 273-390 358 ± 39 | 8 145-208 179 ± 16 | 3 ND-0.1 0.03# | 10 3-8 5 ± 1 | 10 8-150 28# |  | 1 0.7 | 9 ND-2.8 0.62# | 3 ND-4 | 1-6 Ba 204 Zn 80 | 1 ND |
| Crystal Lake 303(d)-low 314 319 |  |  | 405.43 | 5.8 | CA410.1770L | 5 impaired | N FT F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | DO AMM |  |  |  |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Recreational use (m) |  |  |
| Cedar Creek |  |  | 405.43 | 2.37 | CA410.1790R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| 100 7-26 12 ± 6 | 100 6.3-8.5 7.1 ± 0.5 | 100 0.1-13.0 3.9# | 40 83-179 125 ± 22 | 40 173-287 213 ± 27 | no data | no data | 40 1-5 3 ± 1 | 40 3-11 7 ± 1 |  | 40 ND-1.54 0.14# | 40 ND | no data | no data | 20+ a,s,v,w,y,z,ag,ak,ad,au,av,aw,ay,bc,bo |
| 1 7 | 2 7.9-8.2 | no data | 2 178-208 | 1 299 | 2 178-180 | 1 0.2 | 2 3-4 | 2 ND-10 |  | no data | 1 0.68 | no data | no data | no data |
| San Gabriel River West Fork R2 (Cogswell res to Red Box Gap) |  |  | 405.43 | 8.14 | CA410.1920R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| San Gabriel East Fork 303(d)-low |  |  | 405.43 | 12 | CA410.1980R | 15 good | F F N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | TRASH TRASH SO4* |  |  |  |  |  | Recreational use (h) Illegal dumping (m) Septic systems (s) |  |  |
| 6 7-16 11 ± 4 | 14 7.8-8.9 8.2 ± 0.3 | 2 9.6-9.8 9.7# | 12 166-252 211 ± 26 | 7 274-380 330 ± 36 | 9 140-216 176 ± 24 | 5 ND | 12 2-8 4 ± 2 | 12 14-210 41# |  | 1 0.6 | 11 ND-1.3 0.43# | 2 <2 | 2-4 As 3 Ba 88 Pb 4 Zn 40 | no data |
| Cattle Cyn Creek |  |  | 405.43 | 8.98 | CA410.2060R | 16 good | F F F F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Basin Plan |  |  | FULLY SUPPORTING |  |  |  |  |  |  |  |
| 1 8 | 3 7.9-8.3 8.0 ± 0.2 | no data | 3 220-263 242 ± 18 | 1 322 | 2 188-220 204# | 2 ND | 3 5-8 6 ± 1 | 3 14-30 21 ± 7 |  | 1 0.7 | 2 0.7-1.2 0.95 ± 0.25 | no data | no data | no data |

Islands

| Reach/location (Islands) | Hydro Unit | Size (acres) | Code | Previous WQA acres status | WQA | Assessed Use | Not Supporting | Partially Supporting | Potential Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fecal Coliform (Wet and Dry Weather) | Total Coliform (Wet and Dry weather) |  |  |  | Beach Closures |  |  | Fish Consumption | Tissue, Sediment, and Toxicity Data |  |  |  |  |
| Anacapa Island: nearshore (Area of Special Biological Significance) | 406.10 | 21280 | CA414.0100O | 21280 good | F F F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  |  |
| San Nicolas Island and Begg Rock nearshore (Area of Special Biological Significance) | 406.20 | 102528 | CA414.0200O | 102528 good | F F F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  | Threat of military impacts |
| Santa Barbara Island: nearshore (Area of Special Biological Significance) | 406.30 | 14000 | CA414.0400O | 14000 good | F F F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  |  |
| Santa Catalina Island: nearshore (Areas of Special Biological Significance 1-4) | 406.40 | 17936 | CA414.0500O | 17936 good | F F F F F F | Drinking Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  |  |
|  |  |  |  |  |  |  |  |  | West side and at Ribbon Rock: Tissue-low to nondetectable levels of metals and pesticides 1 |  |  |  |  |
| San Clemente Island: nearshore (Area of Special Biological Significance) | 406.50 | 80512 | CA414.0600O | 80512 good | F F F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  | Threat of military impacts |

Ventura County Coastal streams, lakes and estuaries

| Reach/location (Ventura County Coastal streams, lakes and estuaries) |  |  | Hydro Unit | Size (miles or acres) | Code | Previous WQA | WQA | Assessed Uses |  |  | Not Supporting |  |  | Partially Supporting |  |  | Potential Sources |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temp | pH | DO | TDS | SC | Hard | B | Chl | SO4 | Tissue, Sediment and Toxicity Data | AMM | N+N | Fec Col | metals | Org Chem |
| Los Sauces Creek |  |  | 401.00 | 4.7 | CA401.0030R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| no data | 2 7.9-8.0 | no data | 2 | 2 1780-1950 | no data | no data | 2 78-125 | 2 541-683 |  | no data | no data | no data | no data | 2 be |
| Javon Cyn |  |  | 401.00 | 2.8 | CA401.0060R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Padre Juan Cyn |  |  | 401.00 | 3.6 | CA401.0070R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| A Lease Cyn (Dulah West) |  |  | 401.00 | 1.9 | CA401.0080R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Prince Barranca |  |  | 402.10 | 1.4 | CA401.0120R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Hall Canyon |  |  | 402.10 | 7.1 | CA401.0130R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Arrundell Barranca † |  |  | 403.11 | 4.36 | CA401.0160R |  | N N U U | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS*,SC*,CHL*,B* SO4* TDS*,SC*,SO4* CHL*, |  |  |  |  |  | Natural sources (m) Agricultural runoff (m) Urban runoff (m) |  |  |
| 7 15-26 19 ± 3 | 11 6.8-8.4 7.7 ± 0.5 | no data | 10 160-5200 2736 ± 1710 | 11 253-7870 3405 ± 2089 | 6 90-1605 747 ± 535 | no data | 11 13-1800 468 ± 520 | 11 42-2460 1092 ± 819 |  | 11 0.4-38.7 9.1# | 11 0.2-1.2 0.8 ± 0.4 | no data | 1 ND | 1 ND |
| McGrath Lake Ag Drain |  |  | 403.11 | 0.5 | CA401.0200R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| McGrath Lake (Estuary) † 303(d)-low |  |  | 403.11 | 18.7 ac | CA401.0210E | 40 ac intermed | N F F F | Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | Sed(DDT,chlordane, total pesticides),SED TOX |  |  |  |  |  | Natural sources (m) Agricultural runoff (m) Oil spills(m) [Significant oil spill in December 1993] |  |  |
| 2 20-26 | 4 7.4-8.7 8.1 ± 0.5 | no data | 4 1810-8340 | 4 2292-12900 6197 ± 4198 | 2 845-1220 | 4 0.6-1.6 0.9 ± 0.4 | 4 236-3830 1498 ± 1434 | 4 640-1150 906 ± 181 | Sed Chem: DDT (3ppm), chlordane (160ppb), other pesticides (3ppm) 2 Sed tox: low survival rates 2 | no data | 3 1.8-11.8 7.8 ± 4.3 | no data | no data | no data |
| West 5th Street Drain |  |  | 403.11 | 2.14 | CA401.0250R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Oxnard Drain #1 |  |  | 403.11 | NEED | CA401.0270R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Oxnard West Drain † |  |  | 403.11 | 3.3 | CA401.0280R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| 1 18 | 3 7.0-7.9 7.5 ± 0.4 | no data | 3 584-11200 4864 ± 4571 | 3 846-15800 6545# | no data | 3 0.4-2.5 1.5 ± 0.9 | 3 60-4680 1640# | 3 223-1720 1128 ± 650 | no data | no data | 3 1.8-4.3 2.7 ± 1.1 | no data | no data | no data |
| Hueneme Drain |  |  | 403.11 | 1.17 | CA401.0300R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| J Street Drain |  |  | 403.11 | 2.41 | CA401.0310R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Oxnard Industrial Drain |  |  | 403.11 | 4.31 | CA401.0320R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Rice Pond Drain |  |  | 403.11 | 3.56 | CA401.0330R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| La Jolla cyn |  |  | 404.48 | 2.7 | CA401.0340R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Big Sycamore Cyn † |  |  | 404.47 | 10.3 | CA401.0350R |  | F F F F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  |  |  |  |  |  |  |  |  |  |
| 1 19 | 3 7.1-8.2 7.6 ± 0.4 | no data | 3 650-708 688 ± 27 | 2 980-1020 | no data | 3 0.1-0.3 0.2 ± 0.1 | 3 62-85 75 ± 10 | 3 138-223 180 ± 35 | no data | 1 0.2 | 3 0.4-1.1 0.8 ± 0.3 | no data | no data | no data |
| Deer Cyn † |  |  | 404.46 | 2.2 | CA401.0390R |  | N F F F | Agriculture Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS*,SC*,CHL* TDS*,SC*,SO4* |  |  |  |  |  | Natural sources (h) |  |  |
| 2 17 | 3 7.0-8.3 7.7 ± 0.5 | no data | 3 950-1150 1038 ± 83 | 3 1301-1450 1374 ± 61 | no data | 3 0.1-0.2 0.17 ± 0.05 | 3 114-130 122 ± 7 | 3 256-284 272 ± 12 | no data | no data | 3 ND-0.4 0.22 ± 0.18 | no data | no data | no data |
| Little Sycamore Cyn † |  |  | 404.45 | 4.8 | CA401.0400R |  | F F F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS*,SC*,SO4* |  |  |  |  |  |  |  |  |
| 1 17 | 3 7.0-8.3 7.7 ± 0.5 | no data | 3 700-1000 844 ± 123 | 3 1050-1140 1090 ± 37 | 1 655 | 3 ND-0.1 0.07 ± 0.05 | 3 47-80 63 ± 13 | 3 210-316 263 ± 43 | no data | no data | 2 0.5-2.1 | 1 170 | no data | no data |

Los Angeles County Coastal streams, estuaries and lakes

| Reach/location (LA Co. Coastal watersheds) |  |  | Hydro Unit | Size (miles / acres) | Code | Previous WQA | WQA | Assessed Uses |  |  | Not Supporting |  |  | Partially Supporting |  |  | Potential Sources |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temp | pH | DO | TDS | SC | Hard | B | Chl | SO4 | Tissue, Sediment and Toxicity Data | AMM | N+N | Fec Col | metals | Org Chem |
| Lachusa Cyn |  |  | 404.42 | 2.9 | CA407.0070R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Zuma Cyn |  |  | 404.36 | 6.9 | CA407.0140R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Corral Cyn |  |  | 404.31 | 4.1 | CA407.0220R |  | F FT F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | SO4 TDS,SC,SO4 |  |  | AMM |  |  | Natural sources (m) Unknown sources (m) Septic systems (s) Horse corals (m) Urban runoff (m) |  |  |
| 35 10-24 17 ± 4 | 34 7.7-8.5 8.1 ± 0.2 | no data | 36 120-2518 1110 ± 446 | 34 189-4385 1553 ± 696 | 36 75-1528 689 ± 243 | 36 ND-0.5 0.2 ± 0.1 | 36 7-143 74 ± 25 | 34 46-1280 492 ± 210 |  | 35 ND-2.02 0.07# | 36 ND-5.5 0.3# | 35 ND-790 | 2 Ba 382 Cu 41 | 34 be [be only chemical scanned] |
| Malibu Lagoon 303(d)-low 319 |  |  | 404.21 | 32.5 ac | CA405.0000E | 29 ac impaired | N N F N N | Aquatic life Recreation: cont. Recreation: non-c Fish consumption Shellfish Harvesting |  |  | Benthic community, TIS(As,Ni,Se,Pb) COLIFORM, Enteric viruses, swimming restriction TIS(As, Se, Pb) Advisory |  |  | TIS(Ag,Cr), EUT; Excessive freshwater -Fully but threatened |  |  | Natural sources (m) Unknown sources (m) Septic systems (s) Horse corals (m) Urban runoff (m) POTW effluent (m) Transient encampments (m) Artificial breaching of lagoon sandbar (m) Recreational use (m) Fires (m) |  |  |
|  |  |  |  |  |  |  |  |  | Tissue ('93): no elevated constituents 3 Tissue: no elevated constituents 1 Sed tox: good survival 2 Wat tox: possible impairment during storms 2 Benthic community 12 Tissue ('93): As(MFS,MIS), Ni(MFS), Se(MFS,MIS), Pb(MFS,MIS), Ag(EDL95), Cr(EDL95) 12 Enteric viruses 6 |  |  | Coliform 12 |  |  |
| Malibu Creek (lagoon to Malibu Lake) 303(d)-low except targeted for nutrients) |  |  | 404.21 | 9.5 | CA405.0010R | 6 interm 3 unknown | N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | SO4 TDS,SC,SO4 Fish barrier, TIS(As,Se,Cr,Ni) AMM,TRASH,ALGAE TRASH,ALGAE |  |  | TIS(Cd,Cu,Pb,,Ag) COLIFORM,SCUM,COLOR SCUM |  |  | Natural sources (m) Unknown sources (m) Septic systems (s) Horse corals (m) Urban runoff (m) POTW effluent (m) Transient encampments (m) Artificial breaching of lagoon sandbar (m) Recreational use (m) Fires (m) Fish barrier (m) |  |  |
| 101 8-28 18 ± 4 | 106 7.2-8.9 8.1 ± 0.3 | 19 6.2-12.8 8.5 ± 1.8 | 95 600-2314 1358 ± 351 | 81 903-3340 1839 ± 454 | 93 7-1300 751 ± 209 | 94 ND-1 0.4 ± 0.2 | 95 47-225 143 ± 32 | 95 288-1245 623 ± 216 | Tissue ('92): Cd(EDL95), Se(EDL85), Ag(EDL85) 3 Tissue('93): As(MFS,MIS), Se(MFS,MIS), Cr(MIS), Ni (MFS), Ag(EDL95), Cu(EDL95), Pb(EDL95) 12 | 108 ND-1.1 0.1# | 113 ND-11.0 3.6 ± 3.0 | 83 ND-14000 | 66-78 As 32 Ba 55 Cd 20 Cr 50 CrVI 20 Cu 240 Pb 300 Se 65 Zn 270 Hg 1 Ni 90 | H2O (6) d,be Sed (1): sc |
| Cold Creek |  |  | 404.21 | 5.16 | CA405.0030R |  | F FT F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS*,SC* AMM*,COLOR |  |  |  |  |  | Natural sources (m) Septic systems (s) Horse corals (m) Urban runoff (m) Grazing (s) |  |  |
| 18 11-20 17 ± 2 | 18 7.7-8.4 8.0 ± 0.2 | 8 8-10.2 8.9 ± 0.7 | 10 425-1060 759 ± 169 | 5 1070-1330 1139 ± 96 | 10 315-620 481 ± 89 | 10 0.3-3.6 1.1# | 10 32-110 74 ± 22 | 10 62-734 257 ± 177 | no data | 19 ND-1 0.1# | 22 ND-2.3 0.6# | 7 ND-90000 | 0-2 As 3 Ba 770 Cd 10 Cr 380 Cu 180 Pb 100 Se 1 Zn 730 Hg 0.1 Ni 90 | H2O no data Sed (1) sc |
| Las Virgenes Creek 303(d)-low except targeted for nutrients) |  |  | 404.22 | 11.47 | CA405.0070R |  | N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | SO4* TDS*,SC*,SO4* Se*, DO AMM,COLIFORM*,SCUM,ALGAE,TRASH,COLOR,Turb SCUM,ALGAE,TRASH |  |  |  |  |  | Natural sources (m) Septic systems (s) Horse corals (m) Urban runoff (m) Grazing (s) POTW effluent (m) Aerial deposition (m) |  |  |
| 22 10-25 18 ± 4 | 24 7.5-8.1 7.9 ± 0.2 | 12 3.1-15.6 7.9 ± 3.8 | 16 2560-3520 3042 ± 274 | 10 2680-4690 3776 ± 720 | 16 1200-1812 1570 ± 148 | 16 ND-0.8 0.5 ± 0.2 | 16 114-197 161 ± 27 | 16 1290-1919 1595 ± 191 | no data | 23 ND-1.4 0.1# | 28 ND-4.7 2.0 ± 1.6 | 10 40-17000 | 4-5 Ag 3 As 3 Ba 60 Cd 10 Cr 2 Cu 10 Pb 100 Se 38 Zn 50 Hg 0.1 | no data |
| Stokes Creek 303(d)-low except targeted for nutrients) |  |  | 404.22 | 5.33 | CA405.0080R |  | F P F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | SO4* |  |  | COLIFORM* |  |  | Natural sources (m) Septic systems (s) Horse corals (m) Urban runoff (m) Aerial deposition (s) |  |  |
| 8 12-23 17 ± 3 | 7 7.5-7.9 7.7 ± 0.1 | 5 7.8-16.8 11.1 ± 3.8 | 4 234-1150 891 ± 382 | 2 1860 | 4 103-690 478 ± 227 | 4 ND-0.5 0.3 ± 0.2 | 4 15-115 79 ± 40 | 4 148-321 269 ± 71 | no data | 12 ND-0.16 0.03# | 12 0.7-8.8 3.7 ± 2.8 | 4 80-14000 | 1 Ag 3 Zn 140 Hg 0.5 | no data |
| Malibu Lake 303(d)-low 314 |  |  | 404.24 | 69 ac | CA405.0180L | 55 interm | N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | SO4 TDS,SC,SO4 DO,EUT,TIS(PCBs,chlordane) AMM,ALGAE ALGAE |  |  | TIS(Cd,Cu,Zn) |  |  | Algae treatment (m) Natural sources (m) Urban runoff (m) Aerial deposition (s) Septic systems (m) |  |  |
| 112 10-30 19 ± 6 | 101 7.0-9.0 7.7 ± 0.3 | 112 0.1-20.4 6.9 ± 5.1 | 42 347-2287 1631 ± 451 | 42 534-3650 2105 ± 582 | no data (1080) | no data | 42 28-274 165 ± 64 | 42 115-1823 767 ± 364 | Tissue ('91): PCBs (MTRLs), Cd(EDL85), Cu(EDL95), Zn(EDL85) 3 Tissue ('92): Chlordane(MTRLs), Cd(EDL85), Cu(EDL95), Zn(EDL85) 3 | 42 ND-0.66 0.1# | 42 ND-2.2 0.44# | no data | 42 Cu 62 Pb 89 Se 42 Zn 31 Ag 2.4 Ni 46 | 27+ a,m,p,s,t,u,w,y,z,ag,aj,al,aa,ab,ac,ad,au,av,ax,bp,ay,ba,bc |
| Medea Creek R1 (lake to confl with Lindero) and R2 (abv confl with Lindero) 303(d)-low (except targeted for nutrients) 319 |  |  | 404.24 404.23 | 3.01 and 5.44 | CA405.0190R and CA405.0200R | 7 interm | N N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | SO4* TDS*,SC*,SO4*,AMM* Se* AMM, COLIFORM,COLOR COLIFORM* |  |  | ALGAE,TRASH,Turb ALGAE,TRASH |  |  | Natural sources (m) Horse corals (m) Urban runoff (m) Grazing (s) Aerial deposition (s) |  |  |
| R1: 14 12-27 19 ± 4 R2: 4 11-21 17 ± 4 | 15 7.4-8.2 7.9 ± 0.2 8 7.0-8.1 7.6 ± 0.4 | 2 7.6-7.7 no data | 15 507-2908 2236 ± 668 8 295-2930 1979 ± 775 | 8 2390-4200 3353 ± 670 7 1900-4350 3029 ± 871 | 14 298-1600 1217 ± 392 8 172-1820 1266 ± 484 | 15 0.04-0.71 0.37 ± 0.21 8 ND-265 33# | 15 54-298 209 ± 61 8 37-950 258 ± 271 | 15 235-1522 1130 ± 360 8 134-1339 890 ± 376 | no data no data | 11 ND-2.0 0.36# 5 ND-0.50 0.22 ± 0.20 | 16 0.3-3.8 1.3 ± 0.9 8 ND-1.1 0.6 ± 0.4 | 8 23-50000 4 300-90000 | 0-8 As 4.2 Ba 68 Cd 10 Cr 3 Cu 20 Pb 100 Se 8 Zn 187 Hg 0.5 | H2O: no data Sed(1): sc |
| Lindero Creek R1 (Medea to Lake Lindero) and R2 (abv lake) 303(d)-low |  |  | 404.23 404.23 | 2.2 and 4.8 | CA405.0210R and CA405.0230R |  | N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | SO4* TDS*, SC*, CHL*, SO4*, AMM* CHL*, Se* AMM*, COLIFORM, TRASH, ALGAE, COLOR COLIFORM, TRASH, ALGAE |  |  | SCUM SCUM |  |  | Natural sources (m) Horse corals (m) Urban runoff (m) Grazing (s) Aerial deposition (s) |  |  |
| 7 11-21 16 ± 4 | 12 7.5-8.2 7.8 ± 0.2 | no data | 12 326-3460 2549 ± 1029 | 9 2830-4730 3884 ± 682 | 12 158-2140 1501 ± 653 | 12 ND-0.5 0.17 ± 0.16 | 12 61-310 233 ± 81 | 12 79-1967 1295 ± 589 | no data | 7 ND-1.80 0.36# | 12 0.3-4.2 1.0# | 9 1700-90000 | 0-5 As 4 Ba 77 Cd 4 Cr 10 Cu 27 Se 10 Zn 131 Hg 0.6 | no data |
| Lake Lindero 303(d)-low |  |  | 404.23 | 13.56 | CA405.0220L | 77 unknown | N N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | SO4 TDS,SC,CHL,SO4 CHL,SC,Se,EUT,TIS(Se) AMM,ODORS,TRASH,ALGAE ODORS,TRASH,ALGAE TIS(Se) |  |  | TIS(oxadiazon) |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Algae treatment (m) Nutrient-rich sediments (m) Recreational use (m) |  |  |
| 81 10-31 21 ± 6 | 76 7.5-8.4 8.4 ± 0.2 | 81 1.1-18.2 11.0 ± 3.5 | 30 929-3448 2477 ± 726 | 30 1325-3530 2937 ± 747 | no data (1490) | no data | 30 89-330 244 ± 76 | 30 374-1610 1146 ± 368 | Tissue('91): Se(MIS), Oxadiazon(EDL95) 3 Tissue('92): Se(MIS) 3 | 30 ND-0.49 0.10# | 30 ND-1.3 0.4# | no data | 30 Cu 86 Pb 69 Se 19 Ni 14 | no data |
| Palo Comado 303(d)-low |  |  | 404.23 | 7.78 | CA405.0240R |  | N N N P | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | SO4* TDS*,SC*,SO4*,AMM* CHL* AMM*,COLIFORM* |  |  | COLIFORM* |  |  | Natural sources (m) Horse corals (m) Urban runoff (m) Grazing (s) Aerial deposition (s) |  |  |
| 5 13-22 18 ± 3 | 5 7.2-8.0 7.8 ± 0.3 | no data | 5 486-3502 2300 ± 1070 | 4 2390-4900 3608 ± 1153 | 5 168-1830 1270 ± 578 | 5 ND-1.0 0.4# | 5 26-283 164 ± 94 | 5 174-1932 1196 ± 634 | no data | 13 ND-1.8 0.7 ± 0.6 | 15 0.6-9.4 4.3 ± 3.0 | 4 220-30000 | 0-1 Ba 150 Cr 40 Pb 50 Zn 210 Hg 0.7 | no data |
| Triunfo Cyn Creek R1 and R2 303(d)-low 319 (for 404.24 only) |  |  | 404.24 404.25 | 4.06 and 1.98 | CA405.0260R and CA405.0320R | 7 interm | FT N FT F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS*,SC*,SO4* Hg*,Pb* |  |  | COLOR |  |  | Natural sources (m) Horse corals (m) Urban runoff (m) Grazing (s) Aerial deposition (s) |  |  |
| 6 13-24 19 ± 4 | 8 7.3-8.6 7.7 ± 0.4 | 1 4.5 | 8 430-1014 778 ± 173 | 6 982-1660 1269 ± 282 | 7 382-640 481 ± 81 | 8 ND-0.2 0.04# | 8 40-130 94 ± 28 | 8 115-361 257 ± 72 | no data | 3 ND-0.17 0.06# | 6 ND-1.4 0.4# | 4 ND-2300 | 0-3 As 3.7 Ba 230 Cd 10 Cr 190 Cu 60 Pb 100 Zn 190 Hg 1 | no data |
| Westlake Lake 303(d)-low 314 319 |  |  | 404.25 | 186 ac | CA405.0330L | 156 interm | N N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | TDS,SC,SO4,AMM AMM,DO,Cu,Pb,EUT,TIS(chlordane) AMM,ALGAE ALGAE TIS(chlordane) |  |  | TIS(Cd,Cu,Zn) |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Algae treatment (m) Nutrient-rich sediments (m) |  |  |
| 135 10-30 20 ± 6 | 119 6.7-8.7 7.7 ± 0.4 | 135 0.1-19.3 4.9 ± 4.5 | 52 250-1479 814 ± 275 | 52 378-1490 1151 ± 344 | no data (300) | no data | 52 22-150 104 ± 41 | 52 66-381 251 ± 93 | Tissue ('91): Chlordane(MTRLs), Cd(EDL85) 3 Tissue ('92): Cu(EDL85), Zn(EDL85) 3 | 52 ND-1.34 0.35 ± 0.33 | 52 ND-1.7 0.3# | no data | 52 Cu 56 Pb 91 Se 15 Zn 12 | 31+ a,m,s,v,w,y,z,ag,aj,al,ap,aa,ab,ad,au,av,aw,ae,bb,bc,ay,az |
| Potrero Cyn |  |  | 404.25 | 2.59 | CA405.0380R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Eleanor Trib R1 (blw Lake) |  |  | 404.25 | 1.66 | CA405.0390R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Lake Eleanor 314 319 |  |  | 404.25 | 8.17 ac | CA405.0400L | 8 interm | U U U U | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  |  |  |  |  |  |  | Natural sources (m) Aerial deposition (s) |  |  |
| Lake Sherwood 314 319 |  |  | 404.26 | 213 ac | CA405.0430L | 184 interm | FT N N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | AMM AMM,DO,EUT,TIS(Hg) AMM,ALGAE ALGAE TIS(Hg) |  |  |  |  |  | Natural sources (m) Urban runoff (m) Aerial deposition (s) Golf course runoff (m) |  |  |
| 170 9-32 17 ± 5 | 156 6.6-9.0 7.5 ± 0.6 | 170 0.1-16.3 4.0# | 77 203-484 349 ± 58 | 77 290-685 541 ± 96 | no data | no data | 77 15-46 32 ± 9 | 77 6-140 93 ± 35 | Tissue ('91): Hg(MIS) 3 Tissue ('92): Hg(MTRLs,FDA) 3 | 77 ND-5.97 0.81# | 77 ND-1.4 0.4# | no data | 77 As 6 Cu 179 Pb 88 Se 3 Zn 20 Ni 15 | 37+ a,m,s,v,w,y,z,ag,an,ap,at,aa,ab,ac,ad,au,av,aw,ax,bp,ay,ae,bb,ba,bc |
| Hidden Valley Creek |  |  | 404.26 | 4.75 | CA405.0480R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Tuna Cyn |  |  | 404.12 | 2.4 | CA407.0300R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Topanga Cyn 303(d)-low |  |  | 404.11 | 8.6 | CA407.0310R |  | N FT F | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS,SC,SO4 Pb |  |  | AMM |  |  | Natural sources (m) Horse corals (m) Urban runoff (m) Grazing (s) Aerial deposition (s) |  |  |
| 68 6-24 17 ± 4 | 68 7.4-8.7 | 1 7 | 69 680-1800 1039 ± 169 | 68 967-2005 1476 ± 193 | 68 456-792 600 ± 87 | 69 ND-1 0.6 ± 0.2 | 68 12-175 108 ± 24 | 68 272-697 411 ± 104 | no data | 67 ND-0.49 0.03# | 65 ND-4.3 0.5# | 69 ND-1300 | 67-68 As 23 Ba 246 Cd 20 Cr 20 CrVI 13 Cu 40 Pb 230 Se 7 Zn 100 Hg 0.1 Ni 80 | H2O(1) All ND be scanned 67 times and detected above detection limit Sed(1) sc |
| Santa Monica Cyn 303(d)-low |  |  | 405.13 | 2.9 | CA407.0490R |  | N N P | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS,SC,SO4 Pb AMM,COLIFORM |  |  | pH COLIFORM |  |  | Natural sources (m) Horse corals (m) Urban runoff (m) |  |  |
| 65 3-27 14 ± 5 | 68 7.7-8.9 8.2 ± 0.2 | no data | 68 562-1254 911 ± 110 | 68 810-1636 1324 ± 149 | 68 380-723 560 ± 58 | 68 ND-0.8 0.2 ± 0.1 | 68 45-169 108 ± 25 | 68 10-512 304 ± 77 | no data | 68 ND-1.22 0.09# | 67 0.7-9.1 3.2 ± 1.5 | 68 ND-16000 | 68 As 40 Ba 286 Cd 20 Cr 20 CrVI 12 Cu 40 Pb 240 Se 3 Zn 320 Hg 5 Ni 80 | 1 All ND TPH scanned 68 times and detected above detection limit |
| Pico Kenter Drain 303(d)-low |  |  | 405.13 | 4.77 | CA407.0560R |  | N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS,SC,AMM CHL,AMM,Cu,Pb,TOX,PAHs AMM,COLIFORM,Enteric viruses COLIFORM |  |  | COLOR,TRASH TRASH |  |  | Natural sources (m) Urban runoff (m) |  |  |
| 60 9-34 16 ± 5 | 64 7.4-8.9 8.0 ± 0.3 | no data | 64 352-20024 2541# | 63 60-38200 4019# | 63 104-3853 638 ± 789 | 64 ND-3.2 0.4# | 64 51-10403 1130# | 64 67-1744 290# | Enteric viruses, PAHs, water toxicity 6 | 63 ND-4.57 0.65# | 64 ND-13.3 2.0 ± 1.7 | 63 ND-160000 | 63-64 Ag 100 As 112 Ba 420 Cd 110 Cr 90 CrVI 20 Cu 293 Pb 460 Se 7 Zn 370 Ni 110 | 1-63 b,d,be |
| Ashland Avenue Drain 303(d)-low |  |  | 405.13 | 0.57 | CA407.0570R |  | N N U | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS DO,TOX COLIFORM |  |  |  |  |  | Urban runoff (m) Natural sources (m) |  |  |
|  |  |  |  |  |  |  |  |  | TDS, DO, coliform, and toxicity data 6 |  |  |  |  |  |
| Ballona Creek Estuary 303(d)-low |  |  | 405.13 | 2.5 | CA406.0000E |  | N N N N | Aquatic life Recreation: cont. Recreation: non-c Shellfish |  |  | SED TOX,Sed(Pb,Zn, DDT, aroclor, PCBs,PAHs,chlordane),TIS(chlordane,PCBs) COLIFORM Advisory |  |  |  |  |  | Urban runoff (m) Natural sources (m) Spills (m) POTW effluent (m) Industrial point sources (m) Combined sewer overflow (m) |  |  |
|  |  |  |  |  |  |  |  |  | Tissue ('88): chlordane (~200 ppb), PCBs (550 ppb) 1 Sed tox: low survival 2 Sed chem: PCBs (200 ppb), PAHs (5 ppb), chlordane (~100 ppb) 2 Sed Chem ('91-92): Pb, Zn, chlordane, DDT,DDE,DDT,aroclor) 9 Sed chem('95): PAHs (6509ppb), PCBs, Pb(306ppb) 10 |  |  |  |  |  |
| Ballona Wetland 303(d)-low 319 |  |  | 405.13 | 86 degrade 151 total | CA406.0040T |  | N(86) N N N | Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | TIS(As),habitat alteration, exotic vegetation, reduced tidal flushing, hydromodification TRASH TRASH TIS(As) |  |  | TIS(Pb, Cr) |  |  | Urban runoff (m) Natural sources (m) Flow modification (m) Habitat alteration (m) Construction runoff (m) Hydromodification (m) Periodic stagnation(m) Recreation (s) |  |  |
|  |  |  |  |  |  |  |  |  | Tissue ('93): As (MTRLs), Cr (EDL85), Pb (EDL95) 3 Habitat alteration, exotic vegetation, reduced tidal flushing, trash 11 |  |  |  |  |  |
| Ballona Creek 303(d)-low |  |  | 405.13 | 4.3 | CA406.0050R | 150 ac impaired | N N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c fish consumption |  |  | TDS,AMM Pb,SED TOX,TOX,TIS(As,chlordane,DDT,dieldrin,PCBs,ChemA) Sed(Cd,TBT, Cu,Pb,Ag) AMM,COLIFORM COLIFORM TIS(chlordane, DDt, dieldrin, PCBs, ChemA) |  |  | CHL,TIS(Cr,Cu,Pb,Ag,Zn) pH,TRASH TRASH |  |  | Urban runoff (m) Natural sources (m) Fuel spills (m) POTW effluent (m) Industrial point sources (m) Sewage overflow(m) RCRA sites (m) |  |  |
| 210 4-29 16 ± 5 | 217 7.1-9.6 8.2 ± 0.4 |  | 216 176-5810 824 ± 429 | 216 420-9270 1236 ± 660 | 216 67-1280 382 ± 141 | 216 ND-1.1 0.4 ± 0.2 | 217 13-3020 161# | 217 1-467 210 ± 102 | Tissue ('93): As(MTRLs), Cr(EDL85), Cu(EDL95), Pb(EDL95), Ag(EDL95), Zn(EDL95), chlordane(MTRLs, NAS), DDT(MTRLs), dieldrin(MTRLs), PCBs(MTRLs,NAS), ChemA(NAS) 3 Sed chem ('95): Cd,TBT, Cu,Pb,Ag 10 | 214 ND-4.48 0.20# | 215 ND-10.9 | 214 20-160000 | 213-216 Ag 10 As 80 Ba 1200 Cd 30 Cr 60 CrVI 26 Cu 117 Pb 260 Se 27 Zn 1310 Hg 1 Ni 110 | 2-213 be,af |
| Sepulveda Canyon/Channel 303(d)-low |  |  | 405.13 | 6.8 | CA406.0070R |  | N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | CHL,AMM,Pb AMM,COLIFORM COLIFORM |  |  | pH,COLOR |  |  | Urban runoff (m) Natural sources (m) Industrial point sources (m) |  |  |
| 66 8-26 17 ± 4 | 71 7.4-9.8 8.3 ± 0.4 | no data | 71 714-7800 2966 ± 1844 | 70 1100-10800 4740 ± 2939 | 70 360-3400 1376 ± 815 | 71 0.1-1.2 0.6 ± 0.3 | 71 1-4775 1311 ± 1040 | 71 114-754 357 ± 140 | no data | 69 ND-4.3 0.3# | 70 ND-13.7 3.4 ± 2.1 | 69 20-50000 | 69-70 Ag 10 As 52 Ba 449 Cd 70 Cr 64 Cu 160 Pb 260 Se 2500 Zn 480 Hg 2 Ni 240 | 3-4 All ND TPH scanned 68 times and detected above detection limit |
| Polliwog Park Lake |  |  | 405.12 | 3 | CA407.0650L | 2 ac unknown |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Dominguez Channel estuary (to Vermont) and Dominguez Channel above Vermont 303(d)-low 319 |  |  | 405.12 405.12 | 8.4 and 9 | CA408.0120E and CA408.0190R | 7 interm. | N N N N | Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | AMM, Sed(Cr,Zn,DDT,PAHs), Benthic community, CHL, Cu, Pb, TIS(aldrin,chlordane,DDT,dieldrin,PCBs) AMM,pH,COLIFORM,COLOR COLIFORM TIS(chlordane, DDT, dieldrin, PCBs) |  |  | pH,TIS(ChemA,Pb); Sed Chem(Pb, Zn)-Fully but threatened |  |  | Urban runoff (m) Natural sources (m) Oil/waste spills (m) POTW effluent (m) Industrial point sources (m) Historic DDT releases (m) Refinery discharges (m) |  |  |
| 73 6-27 19 ± 5 | 74 6.8-9.7 8.4 ± 0.6 | no data | 74 170-20758 1579# | 74 273-28600 2394# | 74 67-4050 431# | 74 ND-2.0 0.4 ± 0.3 | 74 50-13674 669# | 74 27-1922 203# | NPDES monitoring-Sed chem: Cr(120ppm), Zn(290 ppm), DDT(1.5ppm), PAHs(2ppm) See also LA Harbor- Consolidated Slip below. Tissue ('92): aldrin(MTRLs), chlordane(MTRLs,EDL95), DDT(MTRLS,EDL95), dieldrin(MTRLs), PCBs(MTRLs,EDL95), ChemA(EDL95), Pb (EDL95) 3 Sed chem ('91): Pb(122 ppm), Cu(50 ppm), Zn(746 ppm) [RWQCB data} | 71 ND-10.0 0.3# | 73 ND-6.2 0.9# | 70 2-49000 | 75 Ag 20 As 6 Ba 274 Cd 20 Cr 106 Cu 80 Pb 290 Se 29 Zn 2370 Ni 101 Hg 2 | H2O (7) i,n,r,s be scanned 72 times and detected above detection limit Sed(1-2): sa,se,sc,si,sj,sk |
| Harbor Park Lake 303(d)-low 314 319 |  |  | 405.12 | 45.2 ac | CA408.0140L | 50 impaired | N N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c Fish Consumption |  |  | TDS AMM,EUT,TIS(chlordane, ChemA,DDT,dieldrin, PCBs) AMM,ODORS,TRASH,ALGAE ODORS,TRASH,ALGAE ADVISORY(DDT,chlordane), TIS(chlordane, ChemA,DDT,dieldrin, PCBs) |  |  |  |  |  | Urban runoff (m) Natural sources (m) Aerial deposition (s) Golf course runoff (m) Nutrient-rich sediments (m) Recreational use (m) |  |  |
| 49 11-29 22 ± 6 | 49 6.7-9.1 7.8 ± 0.5 | 49 4.9-15.0 8.2 ± 2.4 | 37 125-1059 476 ± 268 | 37 204-1393 752 ± 420 | no data (440) | no data | 37 15-173 82 ± 55 | 37 30-195 117 ± 58 | Tissue('90): chlordane(NAS), ChemA(MAS) 3 Tissue ('910: chlordane(MTRLs, NAS), DDT(MTRLs), dieldrin(MTRLs), PCBs(MTRLs,NAS), oxadiazon(EDL95), ChemA(NAS) 3 Tissue ('92): chlordane(MTRLs), DDT(MTRLs) 3 Tissue ('93): chlordane (MTRLs, FDA), DDT(MTRLs), dieldrin(MTRLs), PCBs(MTRLs, NAS), oxadiazon (EDL95), ChemA(NAS) 3 Fish consumption advisory: DDT,chlordane-Goldfish or carp 4 | 37 ND-0.6 0.1# | 37 ND-0.5 0.1# | no data | 37 As 7 Cu 61 Pb 158 Zn 17 | 23+ a,m,s,r,v,w,y,z,aa,ab,ac,ad,ag,au,av,aw,ax,bo |
| Wilmington Drain 303(d)-low |  |  | 405.12 | 4.9 | CA408.0150R |  | N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS,SC,SO4,AMM AMM,Cu,Pb AMM,COLIFORM COLIFORM |  |  | CHL |  |  | Urban runoff (m) Natural sources (m) Oil/fuel/waste spills (m) Industrial point sources (m) |  |  |
| 67 7-29 18 ± 5 | 66 6.7-8.8 | no data | 66 106-1750 883 ± 409 | 66 158-2850 1310 ± 598 | 66 48-900 438 ± 223 | 66 ND-3.8 0.8 ± 0.7 | 66 16-398 169 ± 93 | 66 39-887 293 ± 197 | no data | 66 ND-18 1.0# | 63 ND-7.6 | 67 33-160000 | 67 As 21 Ba 450 Cd 13 Cr 60 CrVI 12 Cu 140 Pb 290 Se 67 Zn 980 Hg 1.3 Ni 197 | 1 be |
| Torrance Carson Channel 303(d)-low |  |  | 405.12 | 12.6 | CA408.0160R |  | N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS,SC CHL,Cu,Pb AMM,COLIFORM COLIFORM |  |  | pH |  |  | Urban runoff (m) Natural sources (m) Fuel/oil spills (m) Industrial point sources (m) |  |  |
| 68 6-28 19 ± 5 | 69 6.7-9.3 8.2 ± 0.5 | no data | 69 326-1174 799 ± 160 | 69 467-1799 1260 ± 252 | 69 96-520 354 ± 89 | 69 ND-1.3 0.4 ± 0.2 | 69 68-418 228 ± 79 | 69 33-321 101 ± 55 | no data | 69 ND-6.03 0.24# | 68 ND-4.2 0.3# | 69 130-930000 | 69 As 35 Ba 300 Cd 10 Cr 210 CrVI 74 Cu 60 Pb 90 Se 62 Zn 370 Hg 1.2 Ni 40 | 69 be [be only chem scanned] |
| Colorado Lagoon 303(d)-low 319 |  |  | 405.12 | 13.6 ac | CA408.0260T | 13 impaired | N U U N | Aquatic life Recreation: cont. Recreation: non-c Fish consumption |  |  | Sed(Pb,Zn,chlordane,PAHs),TIS(Cu, ChemA,chlordane,DDT,dieldrin,PCBs), SED TOX TIS(chlordane,DDT,dieldrin,PCBs) |  |  | TIS(dacthal,Chem A, Cu, Pb,Cd,Cr,Ni,Ag) |  |  | Urban runoff (m) Natural sources (m) |  |  |
|  |  |  |  |  |  |  |  |  | Tissue ('92): chlordane(MTRLs,EDL95), DDT(MTRLs), dieldrin(MTRLS, EDL95), PCBs(MTRLs,EDL85), ChemA(EDL95), Cu(EDL85), Pb(EDL85), Ag(EDL85) 3 Tissue('86): chlordane (320ppb) 1 Sed chem: Pb(510 ppm), Zn(690 ppm), chlordane(140 ppb), PAHs(10ppm) 2 Sed toxicity: poor survival rates 2 Water toxicity: impairment unlikely 2 |  |  |  |  |  |
| Sims Pond |  |  | 405.15 | 3.2 | CA408.0265T |  | FT U U | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  |  |  |  | TIS(oxadiazon):Fully but threatened |  |  | Urban runoff (m) Natural sources (m) Spills (s) |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Tissue: chlordane (320 ppb) 1 Sed chem: Pb(510ppm), Zn(690ppm), chlordane(140 ppb), PAHs(10ppm)2 Sed tox: low survival2 Wat tox: slight toxicity2 Tissue('93): Oxadazon(EDL95)3 |  |  |  |  |  |  |  |
| Los Cerritos Estuary |  |  | 405.15 | 54 ac | CA408.0270E |  | F U U | Aquatic life Recreation: cont. Recreation: non-c |  |  |  |  |  |  |  |  | Urban runoff (m) Natural sources (m) Spills (s) Industrial point sources (m) |  |  |
| Los Cerritos Channel 303(d)-low |  |  |  | 16 | CA408.0290R |  | N N N | Drinking: Primary Drinking: Second. Aquatic life Recreation: cont. Recreation: non-c |  |  | TDS,SC,AMM AMM,Cu,Pb,Zn AMM,COLIFORM COLIFORM |  |  | pH |  |  | Urban runoff (m) Natural sources (m) Fuel spills (s) Industrial point sources (m) |  |  |
| 65 7-30 18 ± 5 | 69 3.3-9.1 8.2 ± 0.7 | no data | 69 300-2458 684 ± 275 | 69 408-3540 1040 ± 394 | 69 110-555 240 ± 66 | 69 ND-0.9 0.3 ± 0.2 | 69 43-1278 154# | 69 40-327 136 ± 61 | no data | 69 ND-2.19 0.34 ± 0.41 | 67 ND-16.0 0.6# | 69 2-170000 | 70 | 2-3 all nd be scanned 69 times and detected above detection limit |
| Palos Verdes Channel |  |  | 405.15 | 10.9 | CA408.0310R |  |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |

Ventura County Coastal Features and Bays

| Reach/location (Ventura County Coastal Features and Bays) | Hydro Unit | Size (miles or acres) | Code | Previous WQA | WQA | Assessed Uses |  | Not Supporting | Partially Supporting |  | Potential Sources |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fecal Coliform (Wet and Dry) | Total Coliform (Wet and Dry) |  |  |  | Beach Closures |  |  | Fish Consumption |  | Tissue, sediment and toxicity data |  |  |  |  |  |
| Ventura County Nearshore zone: Pt Mugu to Latigo Point |  | 11410 | CA412.0000O | 11410 ac good | F F F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  |  |  |  |  |
| Nearshore: other areas |  | ask Deb | CA412.0010O |  |  |  |  | Unassessed |  |  |  |  |
| Ventura County offshore zone | 401.00 | ask Deb | CA412.0020O |  |  |  |  | Unassessed |  |  |  |  |
| Rincon Beach (county line to Pitas Point) | 401.00 | 7.73 mi | CA412.0030C |  |  |  |  | Unassessed |  |  |  |  |
| Solimar Beach | 401.00 | 3.86 | CA412.0040C |  |  |  |  | Unassessed |  |  |  |  |
| Emma Wood Beach | 401.00 | 1.59 | CA412.0050C |  |  |  |  | Unassessed |  |  |  |  |
| Seaside Wilderness Park Beach | 401.00 | 0.79 | CA412.0060C |  |  |  |  | Unassessed |  |  |  |  |
| Seaside Park Beach | 402.10 | 0.57 | CA412.0070C |  |  |  |  | Unassessed |  |  |  |  |
| San BuenaVentura Beach | 402.10 | 2.03 | CA412.0080C |  |  |  |  | Unassessed |  |  |  |  |
| Marina Park Beach | 403.11 | 1.25 | CA412.0090C |  |  |  |  | Unassessed |  |  |  |  |
| Ventura Harbor: main harbor area (except Keyes area) | 403.11 | 125 ac | CA412.0100B | 223 good | F F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption |  |  |  |  | Urban runoff (m) Recreational use (m) |  |
| no data | no data |  |  |  | none |  |  | no advisories |  | Tissue ('88): DDT(800ppb),TBT(8700 ppb), Cu(120 ppb), Zn(520ppb) 1 Sed tox('93): good survival 2 |  |  |  |  |  |
| Ventura Harbor beaches | 403.11 | 0.82 | CA412.0110C |  |  |  |  | Unassessed |  |  |  |  |
| Ventura Marina | 403.11 | 33.3 ac | CA412.0120B |  |  |  |  | Unassessed |  |  |  |  |
| Ventura Harbor: Ventura Keyes at Arrundell Barranca 303(d)-low 319 | 403.11 | 40 ac | CA412.0130B | 195 interm | F N F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption |  | COLIFORM |  |  | Urban runoff (m) Agricultural runoff (m) Marina activities (s) |  |
| 75% | 96% |  |  |  | none |  |  | no advisories |  |  |  |  |  |  |  |
| Marina Cove Beach | 403.11 | 0.07 | CA412.0140C | 5 impaired |  |  |  | Unassessed |  |  |  |  |
| Santa Clara River Estuary beach/Surfers Knoll 303(d)-low | 403.11 | 0.56 | CA412.0160C |  | F N F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | COLIFORM |  |  | Urban runoff (m) Agricultural runoff (m) |  |
|  | 32% |  |  |  | none |  |  | no advisories |  |  |  |  |  |  |  |
| McGrath Beach 303(d)-low | 403.11 | 1.35 | CA412.0170C |  | F N F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | COLIFORM | beach closures |  | Oil spills (m) Urban runoff (m) Agricultural runoff (m) |  |
|  | 21-46% |  |  |  | 1993(14 days) 1994(10 days) 1994(37 days) |  |  | no advisories |  | no data |  |  |  |  |  |
| Edison Canal area beach | 403.11 | 0.37 | CA412.0180C |  |  |  |  | Unassessed |  |  |  |  |
| Mandalay Beach 303(d)-low | 403.11 | 1.55 | CA412.0190C |  | F P F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  | beach closures |  | Sewage spills (m) Urban runoff (m) Agricultural runoff (m) |  |
|  |  |  |  |  | 1995(3 days) |  |  | no advisories |  | no data |  |  |  |  |  |
| Oxnard Beach | 403.11 | 0.65 | CA412.0120C |  |  |  |  | Unassessed |  |  |  |  |
| Hollywood Beach | 403.11 | 1.10 | CA412.0210O |  |  |  |  | Unassessed |  |  |  |  |
| Hollywood by the Sea beach | 403.11 | 0.30 | CA412.0120C |  |  |  |  | Unassessed |  |  |  |  |
| Channel Islands Harbor 303(d)-low | 403.11 | 220 ac | CA412.0230B | 220 ac interm | FT U U F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption |  |  | Sed(Pb,Zn): FT |  | Agricultural drains (m) Urban runoff (m) Harbor activities (m) |  |
| no data | no data |  |  |  | none |  |  | no advisories |  | Tissue ('86): DDT(2ppm), Zn(320 ppb) 1 Sed tox('93): high toxicity during winter rainy season but low toxicity in re-samples later2 Sed Chem: Pb(180ppm), Zn(380ppm) 2 |  |  |  |  |  |
| Channel Islands Harbor beaches | 403.11 | 0.64 | CA412.0240C |  |  |  |  | Unassessed |  |  |  |  |
| Mandalay Bay marina | 403.11 | 75 ac | CA412.0250B |  |  |  |  |  |  |  |  |  |
| Silver Strand Beach | 403.11 | 1.00 | CA412.0260C |  |  |  |  | Unassessed |  |  |  |  |
| Port Hueneme Harbor (back basins) 303(d)-low 319 | 403.11 | 50 | CA412.0270B | 121 ac impaired | N U U F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption |  | Sed(PAHs),TIS(DDT,PCBs,TBT,Zn) |  |  | Naval activities (m) Spills (m) Urban runoff (m) Agricultural runoff (m) |  |
| no data | no data |  |  |  | none |  |  | no advisories |  | Tissue ('88): DDT(700ppb),PCBs(2000ppb), TBT(7000 ppb), Zn(400ppm) 1 Sed tox: good survival 2 US Navy-Sed Chem ('88): PAHs(10ppm) |  |  |  |  |  |
| Port Hueneme Harbor (main harbor) | 403.11 | 70 | CA412.0275B | 121 ac impaired | F U U F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption |  |  |  |  | Naval activities (m) Spills (m) Urban runoff (m) Agricultural runoff (m) |  |
| Port Hueneme Harbor Beaches | 403.11 | 0.86 | CA412.0280C |  |  |  |  | Unassessed |  |  |  |  |
| Port Hueneme Beach Park Beach | 403.11 | 0.68 | CA412.0290C |  |  |  |  | Unassessed |  |  |  |  |
| Ormand Beach | 403.11 | 2.5 | CA412.0300C |  |  |  |  | Unassessed |  |  |  |  |
| Ormand Beach Wetland | 403.11 | 9.8 ac | CA412.0310T |  |  |  |  | Unassessed |  |  |  |  |
| Beach between Ormand Beach and Mugu Lagoon | 403.11 | 4.37 | CA412.0320C |  |  |  |  | Unassessed |  |  |  |  |
| Mugu Lagoon Beach | 403.11 | 8.22 | CA412.0330C |  |  |  |  | Unassessed |  |  |  |  |
| Leo Carrillo Beach (to county line) | 404.45 | 0.47 | CA412.0370C |  |  |  |  | Unassessed |  |  |  |  |

LA County Coastal Features and Bays

| Reach/location (LA County Coastal Features and Bays) |  |  | Hydro Unit | Size | Code | Previous WQA | WQA | Assessed Uses |  | Not Supporting |  |  | Partially Supporting |  |  | Potential Sources |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fecal C. dry | Fecal C. wet | Total C. dry | Total C. wet |  | Beach Closures |  |  | Fish Consumption |  | Tissue, sediment and toxicity data |  |  |  |  |  |  |  |  |  |
| SANTA MONICA BAY NEARSHORE ZONE AND OFFSHORE ZONE: Hyperion 5 mile and 7 mile outfall area Joint Water Pollution Control Plant outfall area Palos Verdes shelf Marina del Rey area Santa Monica Pier area Manhattan Beach area Redondo Pier area Malibu Pier area Short Bank Point Dume area Malibu area Point Vicente area Palos Verdes-NW White's Point 310(d) - low 319 |  |  | 1995: 226 square miles total 30 square miles impaired |  | CA413.0000B | 1992: 239500 ac intermediate 16500 impaired | N (26) F F N (4) N (4) | Aquatic life Recreation: contact Recreation: non-c Fish Consumption Shellfish Harvesting |  | SED TOX,TIS(Ag,DDT,Cr,Pb,PCBs), Sed(Cd,Cu,Pb,Hg,Ni,Zn,DDT,PCBs,chlordane, PAHs),debris (see beach closures, COLIFORM below for impairment) Advisories: (DDT,PCBs) |  |  |  |  |  | Natural sources (m) POTWs (m) Dredge spoils dumping (s) Urban runoff (m) Spills (m) Refinery discharges (m) Historic DDT discharges (m) Generating stations (m) Marina activities (m) Aerial fallout (m) Natural oil seeps (s) Fires (s) Illegal dumping (s) |  |  |
| See beach information below | See beach information below | See beach information below | See beach information below |  |  |  |  | Due to DDT and PCBs4: Redondo Pier: Corbina Malibu Pier: Queenfish Short Bank: White Croaker Malibu: White Croaker Point Dume: White Croaker Point Vicente: White Croaker Palos Verdes NW: White Croaker White's Point : White Croaker, sculpin, rock fishes, kelp bass |  | Malibu, Santa Monica, Manhattan Beach, Redondo Piers-Tissue: Ag(5 ppm), DDT(400ppb), PCBs(1.5ppm)1 Tissue (85, and 86-88): DDT, PCBs in ridgeback rock shrimp 6 Tissue (90): DDT, PCBs in yellow rock crab 6 |  |  |  |  |  |  |  |  |  |
| Leo Carrillo Beach (south of County line) 303(d)-low |  |  | 404.44 | 1.15 mi | CA413.0020C |  | F P F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  |  |  | COLIFORM, beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| <10 | 12 | <10 | 29 |  | 1991(5 days) 1992(9 days) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nicholas Cyn Beach 303(d)-low |  |  | 404.43 | 1.94 | CA413.0030C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1991(5 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Robert H Meyer Memorial Beach 303(d)-low |  |  | 404.42 | 1.23 | CA413.0040C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1991(5 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Sea Level Beach 303(d)-low |  |  | 404.41 | 0.67 | CA413.0050C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| <10 | <10 | <10 | <10 |  | 1991(5 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Trancas Beach (Broad Beach) 303(d)-low |  |  | 404.37 | 2.02 | CA413.0060C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | COLIFORM, beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| <10 | 21 | <10 | 21 |  | 1991(5 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Zuma (Westward) Beach 303(d)-low |  |  | 404.36 | 1.65 | CA413.0070C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| <10 | 11 | <10 | <10 |  | 1991(5 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |
| Point Dume Beach 303(d)-low |  |  | 404.36 | 0.95 | CA413.0080C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1991(5 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Paradise Cove Beach 303(d)-low |  |  | 404.35 | 1.33 | CA413.0090C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | COLIFORM, beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| <10 | 27 | <10 | 27 |  | 1991(5 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Escondido Beach 303(d)-low |  |  | 404.34 | 2.05 | CA413.0100C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1991(5 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Dan Blocker Memorial (Corral) Beach 303(d)-low |  |  | 404.31 | 1.04 | CA413.0110C |  | F P F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  |  |  | COLIFORM |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| <10 | 21 | <10 | 14 |  | 1991(5 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Puerco Beach 303(d)-low |  |  | 404.31 | 1.68 | CA413.0120C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| <10 | 14 | <10 | <10 |  | 1991(5 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Amarillo Beach 303(d)-low |  |  | 404.21 | 0.30 | CA413.0130C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1991(5 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Malibu Beach 303(d)-low |  |  | 404.21 | 0.53 | CA413.0140C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1991(5 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Malibu Lagoon Beach (Surfrider) 303(d)-low |  |  | 404.21 | 0.66 | CA413.0150C |  | F N F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | COLIFORM Advisory (DDT, PCBs) |  |  | beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) Fuel spills (m) |  |  |
| 25 | 53 | 11 | 41 |  | 1991(13 days) 1991(1 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Carbon Beach 303(d)-low |  |  | 404.16 | 1.48 | CA413.0160C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1991(13 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| La Costa Beach 303(d)-low |  |  | 404.16 | 0.74 | CA413.0170C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1991(13 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Las Flores Beach 303(d)-low |  |  | 404.15 | 0.76 | CA413.0180C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | COLIFORM, beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| <10 | 19 | <10 | 46 |  | 1991(13 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Big Rock Beach 303(d)-low |  |  | 404.16 | 1.09 | CA413.0190C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | COLIFORM, beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| 14 | 20 | <10 | 10 |  | 1991(13 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Las Tunas Beach 303(d)-low |  |  | 404.12 | 1.25 | CA413.0200C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1991(13 days) 1992(9 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Topanga Beach 303(d)-low |  |  | 404.11 | 1.01 | CA413.0210C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | beach closures Advisory (DDT, PCBs) |  |  | COLIFORM |  |  | Septic system washouts (m) Sewage spills (m) Urban runoff (m) |  |  |
| <10 | 26 | <10 | 25 |  | 1990(4 days) 1991(6 days) 1991(13 days) 1992(9 days) 1993(4 days) 1993(6 days) 1993(3 days) 1993(4 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Castlerock Beach 303(d)-low |  |  | 405.13 | 0.81 | CA413.0220C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  | Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1991(6 days) 1992(9 days) 1993(4 days) 1993(3 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Will Rogers Beach 303(d)-low |  |  | 405.13 | 2.20 | CA413.0230C |  | F N F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | beach closures |  |  | COLIFORM |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| 11 | 32 | <10 | 35 |  | 1988(1 day) 1988(3 days) 1989(2 days) 1990(4 days) 1991(6 days) 1992(9 days) 1993(4 days) 1993(6 days) 1993(3 days) 1993(4 days) 1993(11 days) 1995(4 days) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Santa Monica Beach 303(d)-low |  |  | 405.13 | 2.95 | CA413.0240C |  | F P F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | beach closures |  |  | COLIFORM |  |  | Sewage spills (m) Urban runoff (m) Construction debris (s) |  |  |
| 10 | 36 | 11 | 39 |  | 1988(2 days) 1988(1 day) 1989(3 days) 1989(3 days) 1990(4 days) 1990(9 days) 1991(6 days) 1992(9 days) 1992(1 day) 1993(4 days) 1993(6 days) 1993(3 days) 1993(4 days) 1993(3 days) 1994(4 days) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Venice Beach 303(d)-low |  |  | 405.13 | 1.50 | CA413.0250C |  | F N F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | beach closures |  |  | COLIFORM |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| <10 | 13 | <10 | 27 |  | 1988(3 days) 1990(4 days) 1991(6 days) 1992(9 days) 1992(3 days) 1993(4 days) 1993(6 days) 1993(4 days) 1993(4 days) 1995(1 day) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Marina del Rey Harbor-back basins 303(d)-low |  |  | 405.13 | 413 ac | CA413.0260B | 121 impaired | N N F N N | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | TIS(chlordane, dieldrin, DDT, PCBs, TBT, Zn, Cu),Sed(Zn,Cu,Pb, Cr,PCBs,chlordane, DDT),SED TOX, Benthic community COLIFORM TIS(chlordane,dieldrin, DDT,PCBs,TBT,Zn,Cu) Advisory (DDT, PCBs) |  |  | TIS(ChemA,Pb) |  |  | Sewage spills (m) Marina activities (m) Anti-fouling paints (m) Urban runoff (m) Runoff from dumps (s) RCRA site (m) |  |  |
|  |  |  |  |  |  |  |  |  |  | Tissue ('93): chlordane(MTRLs, EDL95), DDT(MTRLs, EDL85), dieldrin(MTRLs, EDL95), PCBs(MTRLs, EDL95), ChemA(EDL95), Cu(EDL95), Pb(EDL95) 3 Tissue: chlordane (300ppb), DDT(700ppb), PCBs(1000ppb), TBT(6000ppb), Zn(500ppm), Cu(100ppm) 1 Sed Chem: Zn(500ppm), Cu(400ppm), Pb(100ppm) 2 Sed tox: poor survival 2 Benthic community effects 9 Sed chem('92): Cu, Pb, Zn, chlordane, DDT 9 Tissue: DDT, PCBs 9 Sed chem('95): Pb, Cr, PCBs 11 |  |  |  |  |  |  |  |  |  |
| Marina del Rey Harbor-Public beach areas 303(d)-low |  |  | 405.13 | 0.65 | CA413.0290C |  | F N F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | COLIFORM, beach closures |  |  |  |  |  | Sewage spills (m) RCRA site(s) Urban runoff (m)\ Marina activities (m) |  |  |
| 17 | 38 | <10 | 45 |  | 1988(3 days) 1990(4 days) 1991(15 days) 1992(11 days) 1992(3 days) 1992(4 days) 1993(4 days) 1993(6 days) 1993(8 days) 1993(4 days) 1993(4 days) 1003(1 day) 1994(2 days) 1995 (4 days) 1995 (1 day) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dockweiler Beach 303(d)-low |  |  | 405.12 | 5.40 | CA413.0310C |  | F N F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | beach closures |  |  | COLIFORM |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| <10 | 24 | 12 | 39 |  | 1988(1 day) 1988(3 days) 1989(2 days) 1990(4 days) 1991(15 days) 1991(4 days) 1992(11 days) 1992(3 days) 1992(3 days) 1992(1 day) 1993(4 days) 1993(6 days) 1993(3 days) 1993(4 days) 1993(1 day) 1994(2 days) 1995(4 days) 1995(1 day) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Manhattan Beach 303(d)-low |  |  | 405.12 | 2.08 | CA413.0320C |  | F N F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | beach closures |  |  |  |  |  | Sewage spills (m) Urban runoff (m) Fuel spills (m) |  |  |
| <10 | <10 | <10 | <10 |  | 1990(4 days) 1991(6 days) 1991(2 days) 1992 (11 days) 1993(4 days) 1993(6 days) 1993(3 days) 1995 (4 days) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hermosa Beach 303(d)-low |  |  | 405.12 | 1.88 | CA413.0330C |  | F N F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | beach closures |  |  |  |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| <10 | 12 | <10 | 18 |  | 1990(4 days) 1991(6 days) 1992 (11 days) 1993(4 days) 1993(6 days) 1993(3 days) 1995 (4 days) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| King Harbor 319 |  |  | 405.12 | 110 ac | CA413.0340B | 40 good 50 interm | F F F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | none |  | Tissue ('88): TBT(6 ppm), Cu(170ppm), Zn(680 ppm) 1 Sed Chem ('88): Cu(100ppm) [Regional Board data] |  |  |  |  |  |  |  |  |  |
| Redondo Beach 303(d)-low |  |  | 405.12 | 1.37 | CA413.0380C |  | F N F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | beach closures Advisory (DDT, PCBs) |  |  | COLIFORM |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| 12 | 32 | <10 | 18 |  | 1989(3 days) 1990(4 days) 1990(4 days) 1991(6 days) 1992 (11 days) 1992(1 day) 1992(1 day) 1992(3 days) 1992(3 days) 1993(4 days) 1993(6 days) 1993(3 days) 1995 (4 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Torrance Beach 303(d)-low |  |  | 405.12 | 0.58 | CA413.0390C |  | F N F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | beach closures |  |  | COLIFORM |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| <10 | 15 | <10 | 25 |  | 1990(4 days) 1991(6 days) 1992 (11 days) 1993(4 days) 1993(6 days) 1993(3 days) 1995 (4 days) 1995(3 days) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Malaga Cove Beach 303(d)-low |  |  | 405.11 | 1.13 | CA413.0410C |  | F N F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | beach closures Advisory (DDT, PCBs) |  |  |  |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| <10 | 10 | <10 | <10 |  | 1990(4 days) 1991(6 days) 1992 (11 days) 1993(4 days) 1993(6 days) 1993(3 days) 1995 (4 days) 1995(3 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Flat Rock Point beach area 303(d)-low |  |  | 405.11 | 0.30 | CA413.0420C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1990(4 days) 1991(6 days) 1992 (11 days) 1995(3 days) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bluff Cove beach 303(d)-low |  |  | 405.11 | 0.61 | CA413.0440C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| 14 | <10 | <10 | <10 |  | 1990(4 days) 1991(6 days) 1992 (11 days) 1995(3 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Rocky Point beach 303(d)-low |  |  | 405.11 | 0.52 | CA413.0450C |  | F P F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1992 (11 days) 1995(3 days) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Luneda Bay beach 303(d)-low |  |  | 405.11 | 0.35 | CA413.0470C |  | F P F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1992 (11 days) 1995(3 days) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Resort Point beach 303(d)-low |  |  | 405.11 | 0.49 | CA413.0480C |  | F P F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1992 (11 days) 1995(3 days) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Point Vicente beach 303(d)-low |  |  | 405.11 | 2.13 | CA413.0490C |  | F P F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1992 (11 days) 1995(3 days) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Long Point Beach 303(d)-low |  |  | 405.11 | 0.45 | CA413.0500C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| <10 | <10 | <10 | <10 |  | 1992 (11 days) 1995(3 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| San Pedro Bay nearshore and offshore zone (Cabrillo Pier area) 303(d)-low 319 |  |  |  | 500 | CA413.0510B | 10700 ac 500 impaired | N F F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | TIS(DDT),SED TOX,Sed(PAHs,DDT,Zn,Cu,Cr) Advisory (DDT, PCBs) |  |  |  |  |  | Sewage spills (m) POTW effluent (m) Fuel spills (m) Urban runoff (m) Historic DDT discharges (m) |  |  |
|  |  |  |  |  |  |  |  | See Santa Monica Bay (above) |  | Tissue: DDT(1200ppb) 1 Sed tox: variable survival rates 2 Sed Chem: PAHs(3.4ppm), DDT(250ppb), Zn(250ppm), Cu(270 ppm), Cr(95ppm)2 |  |  |  |  |  |  |  |  |  |
| San Pedro Bay nearshore and offshore zone except Cabrillo Pier area |  |  |  | 10200 ac | CA413.0515 B | 10200 ac intermed | F F F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Sed tox: good survival rates 2 Sed chem: Pb(50 ppm) 2 Benthic community-no evidence of degradation 2 |  |  |  |  |  |  |  |  |  |
| Abalone Cove beach 303(d)-low |  |  | 405.11 | 0.94 | CA413.0540C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| <10 | <10 | <10 | <10 |  | 1992(11 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Portuguese Bend beach 303(d)-low |  |  | 405.11 | 2.20 | CA413.0550C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| <10 | <10 | <10 | <10 |  | 1992(11 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Inspiration Point beach 303(d)-low |  |  | 405.11 | 0.30 | CA413.0560C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1992 (11 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Palos Verdes Shoreline Park beach 303(d)-low |  |  | 405.11 | 0.12 | CA413.0570C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1992 (11 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Royal Palms Beach 303(d)-low |  |  | 405.11 | 1.06 | CA413.0580C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1992 (11 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| White's Point Beach 303(d)-low |  |  | 405.11 | 0.70 | CA413.0590C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| <10 | <10 | <10 | <10 |  | 1992(11 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |
| Pt Fermon Park Beach 303(d)-low |  |  | 405.11 | 1.5 | CA413.0600C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| no data | no data | no data | no data |  | 1992 (11 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |  |  |  |  |
| Cabrillo Beach-outer 303(d)-low |  |  | 405.12 | 0.51 | CA413.0610C |  | F P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | Advisory (DDT, PCBs) |  |  | beach closures |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| <10 | <10 | <10 | <10 |  | 1992(11 days) |  |  | See Santa Monica Bay (above) |  |  |  |  |  |  |  |
| Cabrillo Beach (inner)-LA Harbor area 303(d)-low |  |  | 405.12 | 0.79 | CA413.0620C |  | F N F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | COLIFORM, beach closures Advisory (DDT, PCBs) |  |  |  |  |  | Sewage spills (m) Urban runoff (m) |  |  |
| 36 | 19 | <10 | 35 |  | 1991(3 days) 1991(2 days) 1992(11 days) 1992(7 days) 1994(2 days) 1995(4 days) |  |  | See LA Harbor (below) |  |  |  |  |  |  |  |
| Los Angeles Harbor, especially Main Channel, Fish Harbor, Cabrillo Pier, and breakwater 303(d)-low 319 |  |  | 405.12 | 3785 ac | CA413.0640B | inner 1260 impaired | N P F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | TIS(DDT,PCB,Zn,Cu,PAHs),Sed(DDT,PCBs,Cu, Zn,PAHs,TBT),SED TOX Advisory (DDT, PCBs) |  |  | Beach closures |  |  | Oil/other spills (m) Refinery discharges (m) Anti-fouling paints (m) Boatyard practices (m) Urban runoff (m) Historic DDT loading (m) Industrial point sources (m) |  |  |
|  |  |  |  |  | 1992(11 days) |  |  | PCBs and DDT: LA Harbor, especially Cabrillo Pier-White croaker, Queenfish, Black croaker, Surfperches 4 |  | Main Channel and Fish Harbor: Tissue: DDT(1ppm), PCBs(1 ppm), Zn(400ppm), Cu(100ppm), PAHs(5 ppm) 1 Sed Chem: DDT(0.5ppm), PCBs(0.5 ppm), Cu(200ppm), Zn(550ppm), PAHs(15ppm), TBT( 5 ppb) 2 |  |  |  |  |  |  |  |  |  |
| LA Harbor-Consolidated Slip 303(d)-low 319 |  |  | 405.12 | 37.13 ac | CA413.0740B |  | N F F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | TIS(DDT,chlordane,PCBs,TBT,Zn),SED TOX, Benthic community effects, Sed(PAHs,Zn,Cr,Pb,DDT, chlordane, PCBs) Advisory (DDT, PCBs) |  |  |  |  |  | Oil/other spills (m) Refinery discharges (m) Anti-fouling paints (m) Boatyard practices (m) Urban runoff (m) Historic DDT loading (m) Industrial point sources (m) |  |  |
|  |  |  |  |  |  |  |  | see LA Harbor |  | Sed Chem: PAHs(10ppm),Zn(500ppm), Cu(140ppm), Cr(100ppm), Pb(120ppm), DDT(600ppb),chlordane(100ppb),PCBs(1000ppb) 2 Sed tox: poor survival rates 2 Benthic community: degraded 2 Tissue: DDT(1ppm), chlordane(110ppm), PCBs(600ppb), TBT(2000ppb),Zn(400ppm) 1 |  |  |  |  |  |  |  |  |  |
| LA Harbor-Southwest Slip 303(d)-low 319 |  |  | 405.12 | 30 ac | CA413.0745H |  | N F F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | SED TOX Advisory (DDT, PCBs) |  |  |  |  |  | Oil/other spills (m) Refinery discharges (m) Anti-fouling paints (m) Boatyard practices (m) Urban runoff (m) Historic DDT loading (m) |  |  |
|  |  |  |  |  |  |  |  | see LA Harbor |  | Sed tox: poor survival rates 2 |  |  |  |  |  |  |  |  |  |
| Long Beach Harbor , especially Main Channel, Southeast Basin, West Basin, Pier J, and breakwater 303(d)-low 319 |  |  | 405.12 | 3594 ac | CA413.0760B | LB inner 840 ac impaired | N F F N F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  | TIS(DDT, PCBs), Benthic community effects, Sed(PAHs), SED TOX Advisory (DDT, PCBs) |  |  |  |  |  | Oil/other spills (m) Refinery discharges (m) Anti-fouling paints (m) Boatyard practices (m) Urban runoff (m) Historic DDT loading (m) Naval activities (m) |  |  |
|  |  |  |  |  |  |  |  | DDT and PCBs: Pier J: Surfperches LB Harbor and LB breakwater: White croaker, Queenfish, Black croaker, Surfperches |  | Main channel, SE basin, W Basin (Naval Base)-Tissue: DDT(500ppb),PCBs(800ppb) 1 DOP-West Basin: Benthic community impacts SE and West Basins-Sed chem: PAHs (4000ppb) 2 SE and West Basins-Sed tox: poor survival rates 2 |  |  |  |  |  |  |  |  |  |
| Long Beach shore aquatic park beach |  |  | 405.12 | 0.80 | CA413.0880C |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Bluff Park beach |  |  | 405.12 | 0.90 | CA413.0910C |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Belmont Shore beach |  |  | 405.12 | 1.50 | CA413.0920C |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Alamitos Bay beaches |  |  | 405.12 | 1.50 | CA413.0930C |  |  |  |  | Unassessed |  |  |  |  |  |  |  |  |
| Alamitos Bay 319 |  |  | 405.12 | 350 ac | CA413.0940B | 285 ac interm | FT F F F F | Aquatic life Recreation: cont. Recreation: non-c Fish Consumption Shellfish Harvesting |  |  |  |  | TIS(PCBs, ChemA) Fully supported but threatened |  |  | Urban runoff (m) |  |  |
|  |  |  |  |  |  |  |  | none |  | Tissue('91): PCB(EDL95), ChemA(EDL85) 3 Sed chem: PAHs(900ppb) 2 Sed tox: good survival rates 2 |  |  |  |  |  |  |  |  |  |

Groundwater Basins

| Basin | Hydro Unit | Area | Aquifers | Use | Status | Evidence | Possible Sources |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Upper Ojai Valley (CA415.0001G) | 402.31 403.22 | 6 sq mi | all | drinking and industrial | F | -- | -- |
|  |  |  |  | agriculture | F | -- |  |
| Lower Ojai Valley (CA415.0002G) | 402.32 | 23 sq mi | all | drinking and industrial | FT | Nitrate concentrations in municipal supply wells show increasing trend. | Domestic septic systems Infiltration of contaminated animal waste runoff |
|  |  |  |  | agriculture | F | -- |  |
| Upper Ventura (CA415.0003aG) | 402.2 | 11 sq mi | all | drinking and industrial | F | -- | -- |
|  |  |  |  | agriculture | U | Unassessed |  |
| San Antonio (CA415.0003bG) | 402.2 | 4 sq mi | all | drinking and industrial | P | Elevated nitrate concentrations in sampled ground water wells. | Domestic septic systems Infiltration of contaminated animal waste runoff Infiltration of contaminated urban runoff |
|  |  |  |  | agriculture | U | Unassessed |  |
| Lower Ventura (CA415.0003cG) | 402.1 | 8 sq. mi. | all | drinking and industrial | N | Elevated nitrate concentrations in sampled ground water wells as well as detectable hydrocarbons. | Domestic septic systems Infiltration of contaminated animal waste runoff Oil field activities. |
|  |  |  |  | agriculture | U | Unassessed |  |
| Santa Clara - Piru (CA415.0004aG) | 403.41 | 14 sq mi | all | drinking and industrial | P | Elevated nitrate concentrations in sampled ground water wells. | Domestic septic systems Infiltration of contaminated agricultural runoff Infiltration of contaminated animal waste runoff Infiltration of contaminated urban runoff |
|  |  |  |  | agriculture | F | -- |  |
| Santa Clara - Sespe (CA415.0004bG) | 403.31 | 31 sq mi | all | drinking and industrial | P | Elevated nitrate concentrations in sampled ground water wells. | Domestic septic systems Infiltration of contaminated agricultural runoff |
|  |  |  |  | agriculture | F | -- |  |
| Santa Clara - Santa Paula (CA415.0004cG) | 403.12 | 42 sq mi | all | drinking and industrial | F | -- | -- |
|  |  |  |  | agriculture | F | -- |  |
| Oxnard Plain (CA415.0004dG) | 403.12 | 111 sq mi | all | drinking and industrial | P | Elevated nitrate concentrations in sampled ground water wells in some areas. Seawater intrusion within 2 miles of coast. | Domestic septic systems Infiltration of contaminated agricultural runoff Abandoned wells acting as conduits Seawater intrusion |
|  |  |  |  | agriculture | U | Unassessed |  |
| Pleasant Valley (CA415.0006G) | 403.21 | 36 sq mi | all | drinking and industrial | FT | Localized areas of high TDS and chloride. | Infiltration of irrigation waters past the root zone Abandoned wells acting as conduits |
|  |  |  |  | agriculture | F | -- |  |
| Arroyo Santa Rosa (CA415.0007G) | 403.63 | 5 sq mi | all | drinking and industrial | N | Blending required for nitrate contamination (more than 50% of production). Elevated ethylene dibromide and dibromo-chloropropane concentrations in sampled ground water wells. | Domestic septic systems Infiltration of contaminated agricultural runoff |
|  |  |  |  | agriculture | F | -- |  |
| Las Posas - South (CA415.0008aG) | 403.62 | 22 sq mi | all | drinking and industrial | U | Unassessed | -- |
|  |  |  |  | agriculture | F | -- |  |
| Las Posas -North (CA415.0008bG) | 403.62 | 41 sq mi | all | drinking and industrial | F | -- | -- |
|  |  |  |  | agriculture | F | -- |  |
| Acton Valley (CA415.0005aG) | 403.55 | 26 sq mi | all | drinking and industrial | F | -- | -- |
|  |  |  |  | agriculture | U | Unassessed |  |
| Sierra Pelona Valley (CA415.0005bG) | 403.55 | 11 sq mi | all | drinking and industrial | P | Elevated nitrate concentrations in sampled ground water wells (affects approximately 30% of production). | Domestic septic systems Infiltration of contaminated animal waste runoff |
|  |  |  |  | agriculture | U | Unassessed |  |
| Upper Mint Cyn (CA415.0005cG) | 403.53 | 5.9 sq mi | all | -- | U | Unassessed | -- |
| Upper Bouquet Canyon (CA415.0005dG) | 403.52 | 3.2 sq mi | all | -- | U | Unassessed | -- |
| Green Valley (CA415.0005gG) | 403.51 | 1.8 sq mi | all | -- | U | Unassessed | -- |
| L. Elizabeth & L. Hughes (CA415.0005fG) | 403.51 | 0.23 sq mi | all | -- | U | Unassessed | -- |
| Eastern Santa Clara Basin (CA415.000407G) | 403.51 | 41 sq mi | all | drinking and industrial | F | -- | -- |
|  |  |  |  | agriculture | U | Unassessed |  |
| Simi Valley (CA415.0009aG) | 403.67 | 19 sq mi | all | -- | U | Unassessed | -- |
| Gillibrand (CA415.0009bG) | 403.66 | 7 sq mi | all | drinking and industrial | F | -- | -- |
|  |  |  |  | agriculture | F | -- |  |
| Conejo Valley (CA415.0010G) | 403.64 | 9 sq mi | all | drinking and industrial | F | -- | -- |
|  |  |  |  | agriculture | F | -- |  |
| West Coast Basin-upper (CA415.0011aUG) | 405.12 | 141 sq mi | shallow and semi-perched zones | drinking and industrial  | N | Over 800 cases of known leaking underground storage tanks that have contaminated soil and/or ground water with petroleum hydrocarbons (including BTEX) and other VOCs. Eleven cases of refineries/tank farms that have contaminated soil and/or ground water with petroleum hydrocarbons (including BTEX). Seawater intrusion (chloride) into the Gaspur aquifer through Dominguez Gap. | Leaks from refineries, pipelines, and underground storage tanks Infiltration of contaminated storm water and urban runoff Other illegal discharges Seawater intrusion through the Dominguez Gap |
|  |  |  |  | agriculture | U | Unassessed |  |
| West Coast Basin-lower (CA415.0011aLG) | 405.12 | 141 sq. mi. | production (deeper) zones | drinking and industrial  | P | Approximately 250,000 AF of ground water contaminated by chloride, necessitating shutdown or wellhead treatment/blending for 17% of production capacity. | Plume of saline water trapped behind seawater intrusion barrier Seawater intrusion (underflow) beneath parts of the West Coast Basin Barrier Mmigration of contaminants from shallow and semi-perched aquifers |
|  |  |  |  | agriculture | U | Unassessed |  |
| Central Basin-upper (CA415.0011bUG) | 405.15 | 277 sq mi. | shallow and semi-perched aquifers | drinking and industrial 1 | N | Over 1,100 cases of known leaking underground storage tanks that have contaminated soil and/or ground water with petroleum hydrocarbons (including BTEX) and other VOCs. Six cases of refineries/tank farms that have contaminated soil and/or ground water with petroleum products. | Leaks from refineries and underground storage tanks Infiltration of contaminated storm water and urban runoff Other illegal discharges |
|  |  |  |  | agriculture | U | Unassessed |  |
| Central Basin-lower (CA415.0011bLG) | 405.15 | 277 sq. mi. | production zones | drinking and industrial | FT | Wellhead treatment/blending required for VOC contamination at several wells (4% of production). | Migration of contaminants from shallow and semi- perched aquifers Threat of contaminant (VOCs) migration from the San Gabriel Basin into the Central Basin Leaks from an old landfill |
|  |  |  |  | agriculture | U | Unassessed |  |
| Santa Monica (CA415.0011cG) | 405.13 | 40 sq. mi. | all | drinking and industrial | P | Wellhead treatment required for VOC contamination in ground water from several production wells. MTBE contamination in Charnock wellfield. | Industrial discharge of solvents |
|  |  |  |  | agriculture | U | Unassessed |  |
| Hollywood (CA415.0011dG) | 405.14 | 20 sq. mi. | all | -- | U | Unassessed | -- |
| Sylmar (CA415.0012aG) | 405.22 | 10 sq. mi. | all | drinking and industrial | F | -- | -- |
|  |  |  |  | agriculture | U | Unassessed |  |
| Verdugo (CA415.0012bG) | 405.24 | 11 sq mi | all | drinking and industrial | N | All production shut down due to nitrate contamination. | Domestic septic systems |
|  |  |  |  | agriculture | U | Unassessed |  |
| San Fernando (CA415.0012cG) | 405.21 | 190 sq mi | all | drinking and industrial | P | Some production wells shut down due to nitrate, PCE, and/or TCE contamination. | Domestic septic systems Infiltration of contaminated urban runoff Industrial discharge of solvents Old landfills |
|  |  |  |  | agriculture | U | Unassessed |  |
| Eagle Rock (CA415.0012dG) | 405.25 | 2 sq mi | all | -- | U | Unassessed | -- |
| Raymond Basin (CA415.0013aG) | 405.31 | 37 sq mi | all | drinking and industrial | P | Wellhead treatment required for PCE contamination (approximately 30% of production). | Industrial discharge of solvents Leaking landfills |
|  |  |  |  | agriculture | U | Unassessed |  |
| Main San Gabriel Basin CA415.0013bG) | 405.42 405.41 | 141 sq. mi. | all | drinking and industrial | P | Wellhead treatment, blending, and/or shut down for PCE and/or TCE contamination. | Industrial discharge of solvents Leaking landfills |
|  |  |  |  | agriculture | U | Unassessed |  |
| Puente Basin CA415.0013cG) | 405.41 | 16 sq. mi. | all | drinking and industrial | N | All drinking water production shut down for PCE, TCE, and/or TDS contamination. | Industrial discharge of solvents Leaking landfills Historical agriculture and cattlefeed operations |
|  |  |  |  | agriculture | P | Blending required for elevated TDS concentrations. |  |
| Chino Area (CA415.0014cG) | 481.21 | 10 sq. mi. | all | drinking and industrial | N | Wellhead treatment required for nitrate contamination. | Domestic septic systems Agriculture and animal waste runoff |
|  |  |  |  | agriculture | U | Unassessed |  |
| Live Oak Area (CA415.0014 aG) | 405.41 405.44 | 5.1 sq. mi. | all | -- | U | Unassessed | -- |
| Claremont Hts (CA415.0014b) | 405.53 | 6.7 sq. mi. | all | -- | U | Unassessed | -- |
| Pomona Area (CA415.0014dG) | 405.52 | 9.0 sq. mi. | all | -- | U | Unassessed | -- |
| Spadra Area (CA415.0014eG) | 405.51 | 6.0 sq. mi. | all | -- | U | Unassessed | -- |
| Terra Rejada (CA415.0015G) | 403.65 | 2 sq. mi. | all | -- | U | Unassessed | -- |
| Hidden Valley - Santa Monica Mountains (CA415.0016) | 404.26 | 3.8 sq. mi. | all |  | U | Unassessed | -- |
| Thousand Oaks - Santa Monica Mountains (CA415.0019) | 403.64 403.68 | 11.5 sq. mi. | all |  | U | Unassessed | -- |
| Russell Valley- Santa Monica Mountains (CA415.0020) | 404.23 404.25 | 9.1 sq. mi. | all |  | U | Unassessed | -- |
| Conejo - Santa Monica Mountains (CA415.0021) | 403.64 | 2.4 sq. mi. | all |  | U | Unassessed | -- |
| Southern Slopes - Santa Monica Mountains (CA415.0022) | -- | 1.3 sq mi | all | -- | U | Unassessed | -- |
| Lockwood Valley (CA415.0017) | 403.44 | 13 sq mi | all | drinking and industrial | F | -- | -- |
|  |  |  |  | agriculture | F | -- |  |
| Peace Valley & Hungry Valley (CA415.0018) | 403.43 | 18 sq mi | all | -- | U | Unassessed | -- |

Regional Water Quality Control Board - Los Angeles Region

1996 Water Quality Assessment Data Summaries

Explanatory notes for interpreting the tables

Units are in ppm unless noted below.

Most of the data in this table are water column data.   To conserve space, the descriptive terms have been eliminated.  For each constituent, the top row is number of samples, second row is range of values (minimum to maximum), and third row is mean plus or minus standard deviation.  Means are not given for pH and fecal coliform.  For some constituents a "#" next to the mean indicates that no standard deviation has been calculated because there is not a normal distribution or there are less than 3 samples.   Water Column data:  top row is number of samples, second row is range of values (minimum to maximum) and third row is mean plus or minus standard deviation.

*    Between 3 and 20 samples and greater than 40% of the values are higher than the standard

†   Older data was combined with recent data for conventional constituents such as TDS, B, and SO4.  This older data was only used for tributaries in relatively undeveloped areas.

   Although ground water is semi-perched and shallow aquifers may not be directly used for drinking, industrial, or agriculture purposes, it can migrate to and partially recharge deeper aquifers

References:

1.  Tissue data:  California State Water Resources Board.  State Mussel Watch Program.  Elevated metals or organic chemicals are listed.  Maximum concentrations are shown in parenthesis.

2.  California State Water Resources Board.  Bay Protection and Toxic Cleanup Program.  For sediment chemistry, elevated metals or organic chemicals are listed.  Maximum concentrations are shown in parenthesis.  For sediment or water toxicity tests, survival rates of test organisms are described as "low", "poor", or "good".

3.  California State Water Resources Board. Toxic Substance Monitoring Program.  Elevated constituents are shown with standard exceeded in parenthesis.

4.  California Department of Fish and Game, 1994-1996.  California Sport Fishing Regulations.

5.  Regional Water Quality Control Board. 1995.  Draft Final Report:  Toxicity Study of the Santa Clara River, San Gabriel River, and Calleguas Creek.    For water toxicity tests, survival rates of test organisms are described as "poor", or "good".

6.  Santa Monica Bay Restoration Project.  1994. State of the Bay, 1993:  Characterization study of the Santa Monica Bay Restoration Plan.   Includes Gold, et. al., 1990, 1991,1992; Thompson et. al., 1987a; SCCWRP et. al., 1992;  Greene, 1992.

7.  Simi Valley County Sanitation District.  1995 (September). Report on Arroyo Simi Characterization.

8.  Regional Water Quality Control Board.  1994 (December).  Evaluation of Water Quality for Selected Lakes in the Los Angeles Hydrologic Basin.  Final Report.

9.  Los Angeles, County of.  Department of Beaches and Harbors.  1992.  Marine Studies of San Pedro Bay, California Part 20H.  The Marine Environment of Marina Del Rey:  October 1991-June 1992.

10.  Los Angeles, County of.  Department of Beaches and Harbors.  1995.  Results of chemical and physical testing of sediments from Marina del Rey - October 1991-1992.

11.  United States Army Corps of Engineers.  1995.  Marina del Rey and Ballona Creek:  Final reconnaissance report - Marina del Rey sediment chemistry.

12.  Las Virgenes Municipal Water District.  1995.  Enhanced Environmental Monitoring Program at Malibu Lagoon and Malibu Creek.  Used STSM standards from reference 3 above as well as Mearne Fish Tissue Standards (MFS)

13.  California Department of Fish and Game.  1995.  Draft Steelhead restoration and management plan for California.

14.  California Department of Fish and Game.  1990.  Preliminary report on the ecology of the Light-Footed Clapper Rail at Mugu Lagoon, Ventura County, California.

ABBREVIATIONS and Units in Main Header (1st Row of header):

Reach/location	Segment of waterbody that is assessed.  Some waterbodies are combined.  Waterbodies that are on lists are noted.

Hydro. Unit	Hydrologic unit as designated by the state.

Size		Length or area of waterbody (miles or acreage respectively).

Code		Code that has been assigned to the reach by the Regional Board.  CA (California) 4 (Region 4) xx (watersheds 01 to 14). xxxx (segment number) waterbody type (O:  oceans, nearshore, and offshore areas; C:  beaches; H:  bays and harbors; R:  rivers and streams; L:  lakes and reservoirs, E estuaries; and T tidal wetlands)

Previous WQA	Status of waterbody in 1992 and 1994 Water Quality Assessment Reports.  Length or acreage is also shown.

WQA		1996 cycle status of each beneficial use for each assessed waterbody.  Abbreviations for this column are:  Fully supporting (F), Fully supporting but threatened (FT), Partially supporting (P), and Unassessed (U).  Contact and non-contact recreational use was considered "Unassessed" if there was no   water quality data (i.e., secondary MCL data) or no field observations (i.e., trash, odors, etc.); if there is an assessment noted then field observations were made but are not included in any columns on this table due to lack of space (see files).

Use		As appropriate, USEPA's beneficial uses are listed for each waterbody.  These uses are assessed in this report.  Some uses are struck-out ; these are uses that are listed in the Basin Plan but are not assessed in this phase  (see responsiveness summary).  For uses listed as "Basin Plan", minerals constituents were compared to Basin Plan objectives.  These Basin Plan comparisons are not included in the Waterbody System database.

Not Supporting	For each beneficial use (read across for each use), constituents that are "not supporting" that particular use.  Number of exceedences was determined for each use; for many uses, >25% exceedences was used as the trigger to place a constituent or stressor in this column.

Partially supporting	For each beneficial use (read across for each use), constituents that are "partially supporting" that particular use.  Number of exceedences was determined for each use; for many uses, >10% exceedences was the trigger used to place a constituent or stressor in this column.  In some cases, constituents or stressors that are "fully supporting but threatened" are listed in this column.

Potential Sources	Potential sources for constituents or stressors .  These sources are not linked specifically to any constituent.  In parentheses, magnitude of sources are indicsated:  H (high), M (moderate) and S (slight).

ABBREVIATIONS and units in Water Column Header row (2nd row of header):

Temp		Temperature values in table are in Centigrade.

pH		Hydrogen ion activity (stnd units)

DO		Dissolved oxygen (depleted levels) (parts per million:   ppm).

TDS		Total dissolved solids (ppm).

SC		Specific conductance (ppm).

Hard		Hardness (ppm).

B		Boron (ppm).

CHL		Chloride (ppm).

SO4		Sulfate (ppm).

Tissue, sediment

and toxicity	See references and descriptions #1, 2 ,3, etc. above.

AMM		Ammonia-N (ppm).

N+N		Nitrate-N + Nitrite-N (ppm).

Fec Col.		Fecal coliform for inland surface waterbodies.

Fecal Col.	(Coastal Features) Fecal Coliform: dry weather and wet weather:  the numbers in these columns are average number of exceedences for surfzone sampling events.  Most of these areas are sampled weekly.

Total Col.	(Coastal features) see Fecal coliform above.

Metals		Ag (Silver), Ba (Barium), Cd (Cadmium), Cr (Chromium), CrVI (Chromium VI), Cu (Copper), Pb (Lead), Se (Selenium), TBT (Tributyl Tin), Zn (Zinc), and Hg (Mercury).  Be (Beryllium) was scanned by only a few agencies.  The number of sampling events or scans for metals is at the top of column.  Individual metals are listed if they were detected at levels above the detection limit .  Maximum concentration found for each metal is shown to the right of each metal.   In some cases only one or two metals were scanned (usually a large number of times) and these are indicated in the column (e.g., all 11 metals were scanned 1 or 2 times but copper and zinc were scanned 67 times) (ppb).

Org Chem	See Organic chemicals on next page.

Beach Closures	Bathing areas closures issued by county departments of health.

Fish Consumption	Fish consumption advisories.

ADDITIONAL ABBREVIATIONS in Not Supporting and Partially Supporting columns:

ND		Non-detect.  The constituent was not detected at or above the equipment's detection limitation.

Temp		Temperature of effluent is greater than 5oF above ambient temperature in waterbody.

COLIFORM	Coliform bacteria:  fecal coliform for inland surface waterbodies; total and fecal coliform for beaches.

PAHs		Polycyclic aromatic hydrocarbons.

TRASH		Significant trash and debris observed.

ODORS		Significant odors noted.

COLOR		Green, yellow or brownish water color observed.

ALGAE		Significant algal growth observed.

OIL		Oily film observed on surface of water.

SCUM		Nonnatural foam, scum material or other flotsam observed.

Turb		Turbidity.

EUT		Eutrophic.

Sed(x)		Level of constituent in sediment is elevated.

HIST		Abnormal fish histology observed.

TIS(x) 		Fish or mussel data from State Mussel Watch and State Toxic Substances Monitoring programs.  The constituent in parenthesis exceeds the standard(s).

TOX		Tests or studies of chronic or acute toxicity of species (water column).

SED TOX	Tests or studies of chronic or acute toxicity of species (sediment).

ChemA	Toxic Substances Monitoring Program combination of pesticides:  Aldrin, dieldrin, chlordane, endrin, heptachlor, heptachlor epoxide, HCH (including lindane), endosulfan, and toxaphene.

Org Chem	Organic chemicals.  These chemicals are usually analyzed in water samples (in general, the number of sampling events is shown at top of column).  If sediment samples are analyzed, the terms H2O and Sed are used to distinguish the analyses (number of samples are shown in parenthesis).  Organic chemicals that are detected above detection limits are indicated by the following abbreviations:

Water Column organic chemicals

a  benzene

b  bromodichloromethane

c  bromoform

d  chloroform

e  dibromochloromethane

f  o-DCB (1,2-Dichlorobenzene)

g  p-DCB (1,4-Dichlorobenzene)

h  1,1-DCE (1,1-Dichloroethylene)

i  cis-1,2-dichloroethylene

j  trans-1,2-dichloroethylene

k  Carbon tetrachloride

l  cis and trans-1,3-dichloroprop

m  ethyl benzene

n  methylene chloride

o  methyl ethyl ketone

p  naphthalene

q  n-propylbenzene

r  PCE (Tetrachloroethylene)

s  toluene

t  1,1,1-TCA (1,1,1-Trichloroethane)

u  TCE (Trichloroethylene)

v  1,2,4-trimethylbenzene

w  1,3,5-trimethylbenzene

x  vinyl chloride

y  m,p-xylenes

z  o-xylenes

aa  alpha-BHC

ab  beta-BHC

ac  delta-BHC

ad  gamma-BHC (Lindane)

ae  phenol (ppm)

af  cyanide (ppm)

ag  p-IPT

ah  2,4-Dichlorophenol

ai  4-Nitrophenol

aj  2,6-Dichlorophenol

ak  2,3,4,6-Tetrachlorophenol

al  DNBP

am  4-Chloro-3-methylphenol

an  2,4,6-Trichlorophenol

ao  Pentachlorophenol

ap  2-Chlorophenol

aq  2-Nitrophenol

ar  2,4-Dinitrophenol

as  2,4-Dimethylphenol

at  4-Methylphenol

au  Heptachlor

av  Aldrin

aw  Heptachlor epoxide

ax  Dieldrin

ay  Endrin Aldehyde

az  Endosulfan I

ba  Endosulfan II

bb  4,4'-DDD

bc  4,4'-DDE

bd  4,4'-DDT

be  TPH

bf   1,1-DCA

bg   1,1-dichloroethylene

bh   1,2-DCA

bi   1,2,3-Trichlorobenzene

bj   1,2,3-Trichloropropane

bk   1,3,5-Trochlorobenzene

bl   Dibromethane

bm  Methyl-t-butyl ether

bn   Trichlorotrifluoroethane (F113)

bo   2,4,5-Trich

bp   Endrin

Sediment organic chemicals:

sa  Lead

sb  4,4'-DDD

sc  4,4'-DDE

sd  4,4'-DDT

se  Copper

sf   Dieldrin

sg  Mercury

sh   Gamma-BHC (Lindane)

si   Cadmium

sj   Barium

sk  Zinc