Central Valley Regional Water Quality Control Board

San Joaquin River TMDL Unit

STANDARD OPERATING PROCEDURE

Title: Permethrin ELISA SOP 					SOP No: OP 08

References:

1.0	PURPOSE

###### Laboratory Guide for Permethrin ELISA

General:

- The Enzyme-Linked Immunosorbent Assay (ELISA) is a technique that can be used to quantify an analyte. The assay uses a 96 well high binding plate. This is an calorimetric test; ie., the color developed during the test is inversely proportional to the concentration of analyte in the sample.

- The summary provide below is very basic and far from being in final form. As more research is done this guide will be updated and developed. Before using this guide make sure you have the most up to date copy (available from Dan Carlton).

Before starting the test:

- Before starting the ELISA procedure the technician must read the MSDS on all chemicals being used. Copies of all MSDS are available in the back of the lab manual.

- The following supplies will be needed:

Table 1. Chemical-Reagents

| Sodium Carbonate Na2CO3 | Potassium phosphate KH2PO4 | Antibody Rabbit 549 Provided UCD | Hydrogen peroxide 30% H2O2 |
| --- | --- | --- | --- |
| Sodium BicarbonatE NaHCO3 | Sodium phosphate dibasic Na2HPO4 | Goat Anti-Rabbit lgG-Horseradish Peroxidase | # 4N Sulfuric Acid H2SO4 |
| Sodium Hydroxide NaOh | ## Potassium chloride KCl | ## Sodium citrate NaO2CCH2C(OH) (CO2Na)CH2 CO2Na*2H2O | # Hydrochloric Acid HCl |
| Permethrin Hapten 4 Conjugated to Bovine Serum Albumin (4-BSA) | Bovine Serum Albumin Blocking Protein | ## Acetic Acid CH3COOH |  |
| ## Tween 20 C58H114O26 | Analytical Grade Permethrin C21H20Cl2O3 | 3,3’, 5,5’ Tetramethylbenzidine C16H20N2 |  |
| # Sodium Chloride NaCl | # Methanol CH3OH | ## Dimethylsulfoxide (CH3)2SO |  |

# Table 2. Lab Supplies

| Eppendorf Multi Channel Pipette | 1000mL Polyethylene Bottles | Orbital Shaker | pH Meter |
| --- | --- | --- | --- |
| Eppendorf .1-2.5uL Adjustable Pipette | 500mL Polyethylene Bottles | 2uL Pipette Tips | Analytical Balance |
| Eppendorf 10-100uL Adjustable Pipette | 50mL Polyethylene Dropper Bottles | 200uL Pipette Tips | Versamax Plate Reader |
| Eppendorf 500-5000uL Adjustable Pipette | 96 Well High Binding ELISA Plates | 5mL Pipette Tips | Nunc plate washer and tray |
| 1000mL Graduated Cylinder | 96 Well Untreated ELISA Plates | 2 Gallon Carboy | 4mL Glass Vial |
| 50mL Beakers | Acetate Plate Sealer | Disposable Multichannel Pipetter Basins | Weigh Paper |
| 500mL Beakers | Weigh Instruments: Spoonula/Scoopula | Disposable Latex Gloves | Parafilm |
| 10mL Graduated Cylinder | Disposable Weigh Boats | Safety Glasses | 96 Well Mixing Block |
| 250mL Graduated Cylinder |  |  |  |

# A.  Coating the Microtiter Plate with Coating Antigen

- Make the Coating buffer - .1 M Carbonate/Bicarbonate buffer

0.795 g Na2CO3

1.465 g NaHCO3

Dilute to almost 500 mL with distilled water.  Adjust pH to 9.6.  Use NaOH for pH adjustment.  Bring final volume to 500 ml.  Store refrigerated.  Check pH periodically.

- Make the coating solution:

4-BSA diluted to 0.4 ug/ml in 1 M carbonate/bicarbonate buffer

Make 10 mls

Formula for 0.4ug/mL solution

A *  B = C     C / D = E

A = Desired concentration

B = Desired over all volume in mL

C = Result of concentration x Volume

D = Concentration of Permethrin Hapten 4-BSA

E = Amount of uL of D to be spiked in B mL

#### B.  Add 100 ul to each well of microtiter plate.

- Use the Eppendorf Multichannel pipette to add solution. Make sure pipette is set to 100uL

C.  Cover and incubate overnight at 4oC.

- The following incubation times have been tested by Shirley Gee and although 4o  is standard the other temps have also been used with success.

Table 3. Incubation Times

| Temperature Co | ### Time |
| --- | --- |
| 37o | 30 Minutes |
| 25o | 2 Hours |
| 4o | 16 Hours |

# D.  Wash Coated Plate after incubation

- Make wash buffer (0.05% Tween 20) can be made ahead of time:

2.0 ml Tween 20, 4 L distilled water, stored at room temp

- Wash 5 times with wash buffer

- Dry by tapping onto a paper towel

# E.  Block unoccupied sites on surface of microtiter plate

- Make phosphate buffered saline (PBS)

8 g NaCl

0.2 g KH2PO4

1.15 g Na2HPO4

0.2 g KCl

Dissolve in approx 1 L distilled water, pH to 7.4, and then bring volume to 1 L

- Make blocking solution (0.2% BSA in PBS)

0.2 g bovine serum albumin dissolved in 100 mls PBS or 0.1 g into 50 mls (*make at least 50 mls since we’ll need 10 ml to block for this step and more to block antibody in later step)

*blocking solution must be made fresh when needed

- Add 100 uls to each well.

- Incubate for 30 minutes at room temperature.

# F.  Wash coated plate after incubation

- Wash 5 times with wash buffer

- Dry by tapping onto a paper towel

# G.  Preparation of standards, etc.

- Make the primary stock solution

5 mg analytical grade permethrin dissolved in 1 mL MeOH

- Make x mls 40% MeOH in PBS, make standards, and make solution of diluted antibody rabbit 549

- Make standards double-check all concentrations

Table 4. Dilution Table

| Corresponding Cell | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Corresponding Concentration | Reagent blank/no antibody blank | 0 | 0.005 | 0.01 | 0.1 | 0.5 | 1 | 10 | 50 | 500 | 5000 | 10000 |
| ul standard solution | 0 | 0 | 500 ul of 0.01 ppb | 100 ul of 0.1 ppb | 200 ul ml of 0.5 | 500 ul of 1 | 100 ul of 10 | 200 ul 50 ppb | 100 ul of 500 | 100 ul of 5000 | 500 ul of 10,000 | 2 uL of stock solution |
| ul 40% methanol in PBS | 500 | 500 | 500 | 900 | 800 | 500 | 900 | 800 | 900 | 900 | 500 | 1000 |

H.  Conjugate Preparation

- Make 1/3000 diluted antibody rabbit 549:

50ul antibody Rabbit 549 diluted 1/3000 in PBSB.

#### I. Sample Preparation

- For this protocol samples are analyzed following solid phase extraction (SPE) with a C18 column.

- Before loading the column the column must be a preconditioned with methanol (3.5mL) followed by deionized water (3.5mL) before sample application.

- Water samples (200mL) are loaded on the column and eluted with 3-5 mL/min flows. The containers were washed with deionized water, and all washes were applied to the column.

- After the water had passed through, the column was dried under vacuum for 15 minutes and then eluted with 100% methanol (3.5mL). A small amount of methanol is used to rinse the sample container. This methanol is added to the methanol elute from the column.

- The combined methanol is evaporated to 1ml. Four mL of PBSB is then added. The sample is now ready for ELISA analysis.

# J.  Addition of standards, samples and conjugate

- Use multichannel pipette to add 50 ul of standard to each cell.  If two replicates, A and B.  If 4 reps. A through D.

Table 5. ELISA Plate Demo.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | *no AB | 0 | .005 | .01 | 0.1 | 0.5 | 1 | 10 | 50 | 500 | 5000 | 10000 |
| B |  |  |  |  |  |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |  |  |  |  |  |
| F |  |  |  |  |  |  |  |  |  |  |  |  |
| G |  |  |  |  |  |  |  |  |  |  |  |  |
| H |  |  |  |  |  |  |  |  |  |  |  |  |

- Replace tips and use multichannel to add 50 ul of 1/3000 diluted antibody rabbit 549—DO NOT ADD TO A1 AND ITS REPS; to these wells add 50 ul PBSB

- Cover and incubate for 1 hour at room temperature

# K.  Wash plate after incubation

- Wash 5 times with wash buffer

- Dry by tapping onto a paper towel

# L.  Addition of substrate

- Make substrate working solution (DO NOT MAKE SUBSTRATE UNTIL READY TO ADD TO PLATE—LIGHT SENSITIVE)**make solutions that are ingredients of this beforehand and store properly

25 ml citrate-acetate buffer

0.4 ml tetramethylbenzidine solution

  - ml hydrogen peroxide solution

- Add 100 ul substrate working solution to each well.

- Cover and incubate at room temperature for 15 minutes

# M.  Addition of Stop Solution

- Add 50 ul of 4N sulfuric acid to each well.  Color will turn from blue to yellow.

N.  Additional Recipes

- Substrate Solution

Citrate Acetate Buffer

13.61g sodium citrate (100mM)

Bring to approximately 1L with D.I

Adjust pH to 5.5 with acetic acid

Tetramethylbenzidine (TMB) (0.6% TMB in dimethyl sulfoxide)

60mg of 3,3’5, 5’-tetramethylbenzidine

Dissolved in 10mL dimethyl sulfoxide

Store @ room temperature in the dark

Hydrogen Peroxide (1% in water)

1mL of 30% hydrogen peroxide

29mL of D.I

Store in plastic container in the refrigerator