Summary of Total Mercury Concentrations in Fillets of Selected Sport Fishes Collected During 2000–03 from Lake Natoma, Sacramento County, California

Michael K. Saiki1, Darell G. Slotton2, Thomas W. May3, Shaun Ayers2, and Charles N. Alpers4

1U.S. Geological Survey, Western Fisheries Research Center, Dixon Duty Station, 6924 Tremont Road, Dixon, California  95620

2University of California at Davis, Department of Environmental Science and Policy, One Shields Avenue, Davis, California  95619

3U.S. Geological Survey, Columbia Environmental Research Center, 4200 New Haven Road, Columbia, Missouri  65201

4U.S. Geological Survey, Placer Hall, 6000 J Street, Sacramento, California  95819

Abstract.  This report summarizes results of total mercury measurements in skinless fillets of sport fishes collected during August 2000, September- 2002, and July 2003 from Lake Natoma, a small (162-ha) afterbay for Folsom Dam on the lower American River.  The primary objective of the study was to determine if mercury concentrations in fillets approached or exceeded guidelines for human consumption.  Wet weight concentrations of total mercury in skinless fillets were as high as 0.19 µg/g in bluegill (Lepomis macrochirus), 0.39 µg/g in redear sunfish (L. microlophus), 1.02 µg/g in largemouth bass (Micropterus salmoides), and 1.88 µg/g in channel catfish (Ictalurus punctatus).  Maximum concentrations of mercury in other fish species varied from 0.10 µg/g in rainbow trout (Oncorhynchus mykiss) to 0.56 µg/g in white catfish (Ameiurus catus).  Altogether, 1 of 86 largemouth bass and 11 of 11 channel catfish exceeded 1.00 µg/g, the FDA action level for methylmercury in fish.  In addition, 1 of 20 redear sunfish, 26 of 86 largemouth bass, 2 of 3 spotted bass (M. punctulatus), 1 of 1 brown bullhead (A. nebulosus), and 1 of 1 white catfish exceeded 0.30 µg/g, the EPA fish tissue methylmercury residue criterion.  These results indicate that some fish species inhabiting Lake Natoma are contaminated with undesirably high concentrations of mercury in their skinless fillets.

_____________________________________________________________________________

Introduction

Mercury contamination from historic gold mining operations is widespread in many rivers, lakes, and reservoirs on the western slopes of the Sierra Nevada (Alpers and Hunerlach 2000).  Miners used mercury (quicksilver) to recover gold from placer (alluvial) mines, which used hydraulic, drift, and dredging methods, and from hardrock (lode) mines.  At hydraulic mining operations, placer ores were eroded with monitors (water cannons) and the resulting slurry was directed through sluices and drainage tunnels where gold particles were combined with liquid mercury to form gold-mercury amalgam.  Bowie (1905) estimated that 10-30% of mercury used in this process was lost each season, resulting in highly contaminated sediments downstream from mines.  According to Alpers and Hunerlach (2000), annual loss of mercury from a typical sluice was likely several hundred kilograms during the operating season or roughly 1.4-3.6 million kg statewide since 1850.  Although mercury was also used in drift mining operations, in dredging operations, and at hardrock mines, the amounts lost to the environment by these sources have not been estimated.

Several ongoing studies by the U.S. Geological Survey (USGS) are assessing the status and dynamics of mercury contamination associated with past hydraulic mining operations in the Sierra Nevada (C.N. Alpers, USGS, Sacramento, personal communication, http://water.ca.usgs.gov/mercury).  By comparison, little work is being conducted on mercury contamination in historically dredged environments in urbanized areas (for example, the cities of Folsom, Gold River, and Rancho Cordova) within the American River watershed.  Lake Natoma, a 162-ha reservoir on the American River, is managed by the U.S. Bureau of Reclamation and the California Department of Parks and Recreation for multiple uses that include fishing and other water-based recreation.

This study was part of a broader investigation of mercury dynamics and bioaccumulation in urbanized areas within the American River watershed that were historically subjected to gold dredging activities.  The specific objective was to determine if total mercury concentrations in skinless fillets of selected sport fishes approach or exceed criteria for human health concerns.  The FDA action level for methylmercury in fish is 1.00 mg/kg wet weight, which is intended to limit exposure by humans to levels 10 times lower than the lowest levels associated with adverse health effects (USFDA 1994).  This action level is used to regulate the sale of commercially caught fish.  In addition, EPA recently established 0.30 mg methylmercury/kg wet weight as the fish tissue residue criterion that should not be exceeded to avoid undesirable neurological abnormalities in human infants exposed in utero when pregnant women consume mercury-contaminated foods (USEPA 2001).

# Study Area and Methods

Fish were collected from Lake Natoma on the American River during August 2000, September-October 2002, and July 2003 from as many as six general locations as follows (roughly from upstream to downstream):  the Negro Bar vicinity, Natomas Slough, the mouth of Willow Creek, the Mississippi Bar vicinity, the mouth of Alder Creek, and the Nimbus Dam vicinity (Figure 1).  Lake Natoma is located about 19 km east of Sacramento, near the City of Folsom.  Water in Lake Natoma originates primarily from upstream releases at Folsom Dam, with small inflows from Willow and Alder creeks (Figure 1) and other sources.

In 2000, the California Department of Fish and Game (CDFG), with assistance from the State Water Resources Control Board, used a boat-mounted electroshocker to collect a few largemouth bass (Micropterus salmoides) and channel catfish (Ictalurus punctatus) from Lake Natoma.  In 2002, the USGS used a boat-mounted electroshocker to obtain large numbers of bluegill (Lepomis macrochirus) and largemouth bass.  Small numbers of the following species were also retained: green sunfish (L. cyanellus), redear sunfish (L. microlophus), smallmouth bass (M. dolomieu), spotted bass (M. punctulatus), white catfish (Ameiurus catus), black bullhead (A. melas), brown bullhead (A. nebulosus), and rainbow trout (Oncorhynchus mykiss).  In 2003, the University of California-Davis (UCD) used a boat-mounted electroshocker and gill nets to capture a few largemouth bass and channel catfish.

Both catchable and subcatchable sizes of fish were collected as part of this study.  Although the designation of a minimum size for catchable fish is often arbitrary, this study adopted either the minimum legal size limit established by the State of California (CDFG 2002) or the minimum sizes for human consumption suggested by various authors.  In Lake Natoma, the largemouth bass fishery is regulated by a minimum size restriction of 305 mm whereas other fishes have no legal size restrictions (CDFG 2002).  However, Bennett (1971) indicated that anglers generally keep fish of the following minimum sizes:  for bluegill and other sunfishes, 152 mm; for bullheads (close relatives of the white catfish), 178 mm; and for channel catfish, 305 mm.  According to Leitritz (1970, cited by Edmondson 2002), a "harvestable trout" is a fish between 178 and 254 mm or about 182 g each.

Immediately after capture, fish retained for mercury determinations were measured for total length, then placed into clean plastic ziplock bags (along with unique code identifiers) and chilled on wet ice.  Within 24 hrs, fish were weighed, then skinless fillets from both sides of each fish were obtained by dissection.  The two fillets were weighed and wrapped in clean plastic sheets, double-bagged in clean plastic ziplock bags, and frozen (-10C).  The remaining carcass of each fish was then archived by wrapping and bagging in plastic before freezing.

Determinations of moisture content and total mercury were made either by the USGS Columbia Environmental Research Center (CERC) in Columbia, MO (fish samples collected in 2002), or UCD in Davis, CA (fish samples collected in 2000 and 2003).  Total mercury was measured because approximately 95-99% of the mercury in fish muscle tissues occurs as methylmercury, resulting in essentially identical results using either analysis (Grieb et al. 1990, Bloom 1992, Weiner and Spry 1996).

Frozen fish fillets collected in 2002 were shipped overnight on dry ice to CERC.  Methods used to homogenize fillets varied according to biomass of the sample.  Large fillets (>500 g) were processed through a Hobart band saw whereas intermediate-sized fillets (100-500 g) were homogenized with a Kitchen Aid blender and attached meat processor unit.  Small fillets (20-100 g) were minced with a titanium knife.  After homogenization, the samples were lyophilized with a Virtis Genesis 35EL freeze dryer for moisture determination.  Once dried, samples weighing more than 3 g were further homogenized with a Bamix mixer/blender whereas smaller samples were mechanically ground with a glass rod.  Dried samples were stored in glass vials in a desiccator.  Total mercury was determined with a direct mercury analyzer.  A dried fish sample (50-100 mg) was first combusted in a stream of oxygen, then the volatilized sample was trapped by amalgamation on a gold substrate and thermally desorbed for quantification by atomic absorption spectrophotometry.  The entire process was conducted with a Milestone DMA-80 analyzer equipped with an automated sample carousel.

For quality control purposes, the CERC-analyzed samples of fish fillets were placed into 10 groups or blocks for mercury determinations.  Quality control procedures included analysis of blanks, replicate samples, pre-combustion spikes, and tissue reference materials.  During instrumental runs, additional quality control procedures consisted of independent calibration verification checks.  An independent calibration verification standard was analyzed at the beginning and end of each instrumental run to confirm calibration status of the DMA-80 system.  Percentage errors varied from –6.3 to 7.8.  Results from analysis of three reference fish tissues--CERC wholebody striped bass, 2.29±0.05 µg Hg/g dry weight; National Institute of Standards and Technology (NIST) Research Material 50 albacore tuna fillet, 0.92±0.02 µg Hg/g dry weight; and National Research Council Canada (NRCC) DORM-2 CRM dogfish muscle, 4.77±0.09 µg Hg/g dry weight--were within certified or recommended ranges.  Method precision, determined from analyses of 18 duplicate pairs of fish tissue samples, did not exceed 5.8% relative standard deviation (RSD); most were less than 3.0% RSD.  Recovery of methylmercury from pre-combustion tissue spikes of methylmercury chloride varied from 102% to 111% (mean, 108%).  Blank equivalent concentrations were consistently less than the method detection limits (0.0007-0.0059 µg Hg/g dry weight), whereas the limits of quantitation were 0.0023-0.0194 µg Hg/g dry weight.  The instrument detection limit was 0.066 ng Hg.Overall, these quality control results were within acceptable limits as specified by CERC.

Fish fillets processed by UCD were analyzed either fresh (wet) or dry.  Fish collected in 2000 were analyzed as dried material because “freezer burn” had occurred in some samples after being stored frozen for more than a year prior to transfer to UCD.  Fish collected in 2003 were analyzed as fresh material within 24 hours of collection and, in the case of channel catfish, also as dried material.  Moisture percentage in the 2003 samples was determined by multiple weighings before and after drying at 55°C, allowing conversion of dry results to a fresh weight basis.  For 2000 samples that experienced variable freezer dessication, conversion of dry weight concentrations to wet weight concentrations was accomplished by using a size-versus-moisture-percentage relationship obtained from the same species in the 2002 USGS samples.  The UCD fish samples were ground to a fine powder with a modified coffee grinder.  Both fresh and dried samples were digested under pressure at 90°C in a mixture of concentrated nitric and sulfuric acids with potassium permanganate, then analyzed for total mercury with standard cold vapor atomic absorption spectrophotometry using a Perkin Elmer Flow Injection Mercury System with AS-90 autosampler.  Sufficient tissue biomass from each sample was archived to allow for reanalysis.

The UCD mercury analyses were conducted with extensive quality control similar to that used by CERC.  Quality control procedures for each analytical run included analysis of blanks, seven aqueous mercury standards, laboratory replicate samples, pre-digestion matrix spikes, matrix spike duplicates, and tissue reference materials with certified concentrations of Hg.  During instrumental runs, additional quality control procedures consisted of independent aqueous-based calibration verification checks using a standard different from the one used to prepare the basic aqueous calibration series.  Recoveries varied from 102.5% to 104.9% relative to a control range of 75-125%.   Additionally, continuous within-run calibration was tested with repeat analysis of tissue-based samples.  Recoveries from the within-run calibrations varied from 98.2% to 104.7% relative to a control range of 75-125%.  Results from multiple analyses of four certified reference tissues—National Institute of Standards and Technology (NIST) Research Material 2976 CRM mussel tissue, 0.061±0.004 µg Hg/g dry weight; National Research Council Canada (NRCC) TORT-2 CRM lobster tissue, 0.27±0.02 µg Hg/g dry weight; National Research Council Canada (NRCC) DOLT-3 CRM dogfish liver, 3.37±0.14 µg Hg/g dry weight; and National Research Council Canada (NRCC) DORM-2 CRM dogfish muscle, 4.64±0.26 µg Hg/g dry weight—were within certified or recommended means and ranges.  Method precision, determined from analysis of 20 duplicate pairs of fish tissue samples, varied from 0.1% to 5.4% RPD and averaged 2.6% RFD.  Recovery of mercury from pre-digestion tissue spikes varied from 86% to 98% (mean, 91%).  Blank equivalent concentrations were consistently less than the method detection limits (0.0001-0.0022 µg Hg/g dry weight), whereas the limits of quantitation were 0.0019-0.0056 µg Hg/g dry weight.  The instrument detection limit was 0.004 µg Hg/g dry weight.  Overall, these quality control results were well within acceptable limits as specified by UCD.

Mercury measurements by CERC and UCD were compared by using 20 samples of dried fish tissue over the range of mercury concentrations encountered during this study.  Dried splits of 10 channel catfish samples that were initially collected and processed by UCD were sent to CERC, and dried splits of 10 largemouth bass samples initially collected by USGS and processed by CERC were sent to UCD.  Results of the intercomparison were excellent; with RPD values for all 20 samples measuring less than 8% and with 16 samples measuring less than 5%.  Detailed results of the laboratory intercomparison, including tabulated data and a correlation plot of the results, are given in Appendix Table A2 and Appendix Figure A1.

Computerized databases were created as Excel spreadsheets.  Raw data were summarized by using SAS software and Lotus Freelance Graphics for Windows.  Many variables measured during this study were not normally distributed.  Even when these variables were subjected to standard transformations (angular transformation for moisture content; logarithmic transformation for length, weight, and mercury concentration), normality of the data was not always achieved.  Consequently, with two exceptions, nonparametric techniques (for example, Spearman rank correlation) were used to describe relations among variables such as total length or weight of fish, and moisture content and mercury concentrations in their fillets.  The exceptions were computations of predictive equations for length-moisture relationships and length-mercury relationships.  Unless specified otherwise, the level of significance for all statistical tests was P=0.05.

# Results

A total of 228 fish-fillet samples were analyzed for moisture content and mercury concentrations during this study (Table 1).  Moisture content in various species ranged from 74.5% to 83.6%, whereas total mercury concentrations ranged from 0.02 µg/g to 1.88 µg/g wet weight  (henceforth, all references to mercury concentrations will refer to total mercury reported in terms of wet weight unless specified otherwise).  The species with highest maximum concentrations of mercury included brown bullhead (0.35 µg/g), redear sunfish (0.39 µg/g), spotted bass (0.49 µg/g), white catfish (0.56 µg/g), largemouth bass (1.02 µg/g), and channel catfish (1.88 µg/g).

Sufficient sample sizes were available for bluegill, redear sunfish, largemouth bass, and channel catfish to determine if moisture content and mercury concentrations in fillets varied with total length and weight.  Moisture content was inversely correlated with total length and weight in redear sunfish and largemouth bass whereas similar relationships were not observed in bluegill and channel catfish (Table 2).  On the other hand, mercury concentrations were directly correlated with total length, weight, and age in bluegill and largemouth bass but not in redear sunfish and channel catfish.

Discussion

A number of field investigations involving a variety of fish species indicate that mercury concentrations typically increase with size of fish (Lange et al. 1993, 1994; Wiener and Spry 1996).  In Lake Natoma, the mercury concentrations in fillets of bluegill and largemouth bass, but not redear sunfish and channel catfish, also increased as fish increased in total length and weight (Table 2).

Although fish are exposed to mercury from both water (Olsen et al. 1973) and food (Phillips and Buhler 1978, Phillips and Gregory 1979, Rogers et al. 1987, Turner and Swick 1983), bioaccumulation of mercury through the food chain plays a more important role in determining fish mercury burdens (Spry and Wiener 1991, Watras and Bloom 1992, Wiener and Spry 1996).  Fish assimilate 15-20% of the methylmercury present in their forage (Phillips and Gregory 1979) and eliminate methylmercury slowly relative to the rate of uptake (Laarman et al. 1976, McKim et al. 1976, Weiner and Spry 1996), resulting in a net increase in mercury burdens.  MacCrimmon et al. (1983) observed increased rates of mercury accumulation in lake trout (Salvelinus namaycush) when the young switched from a diet of invertebrates to forage fish.  Wren and MacCrimmon (1986) also observed that piscivorous fish had higher concentrations of mercury than prey fish of comparable age.  In Lake Natoma, top-level predators such as largemouth bass, spotted bass, white catfish, and channel catfish had higher concentrations of mercury than did lower trophic level insectivores or planktivores such as bluegill, redear sunfish, and rainbow trout (Table 1).  Moreover, smaller bass, which generally feed on zooplankton and small insects, contained lower concentrations of mercury than did larger bass that feed primarily on large-bodied invertebrates (for example, crayfish) and forage fish (Moyle 2002).  Although size-related increases in mercury body burdens were anticipated in channel catfish, our failure to detect significant correlations may have been due to small sample size (only 11 individuals were captured during our study).  Mercury concentrations were weakly correlated with size of bluegill and not correlated with size of redear sunfish possibly because juveniles and adults of these species tend to forage on similar foods (zooplankton, immature aquatic insects, and other benthic invertebrates, although mollusks are usually less conspicuous in diets of bluegill than redear sunfish; Moyle 2002).  Unlike adult largemouth bass and channel catfish, bluegill and redear sunfish are seldom piscivorous.

One of 86 largemouth bass and 11 of 11 channel catfish from Lake Natoma exceeded the FDA action level of 1.00 mg methylmercury/kg (Figures 2-6).  In addition, 1 of 20 redear sunfish, 26 of 86 largemouth bass, 1 of 1 brown bullhead, 2 of 3 spotted bass, and 1 of 1 white catfish exceeded the EPA fish tissue residue criterion of 0.30 mg methylmercury/kg.  These results confirmed that some fish species inhabiting Lake Natoma are contaminated with undesirably high concentrations of mercury in their skinless fillets.

Acknowledgments.  We thank Rick Humphreys of the California State Water Resources Control Board and Janna Herren of the California Department of Fish and Game for helping to initiate this study by collecting fish during 2000.  We also thank Melissa Farhina, Paul Gerrity, and Michael Healey for assisting with fish collections; Melissa Farhina, Paul Gerrity, Jason May, Francine Mejia, and Amy Story for processing fish samples or assisting with data entry and other technical support; and William Brumbaugh for assisting with mercury and moisture determinations of fish samples.  We especially thank the U.S. Bureau of Reclamation for funding this work through Interagency Agreement No. 5-07-20-X0338, Work Request No. 45, with the U.S. Geological Survey, Water Resources Discipline (California District).  In addition, we thank the California Department of Water Resources for funding a portion of this work through a grant from its Watershed Program.  Matching funds from the USGS Cooperative Water Program also contributed to this study.

References

Alpers, C.N., and Hunerlach, M.P., 2000, Mercury contamination from historic gold mining in California:  U.S. Geological Survey Fact Sheet FS-061-00, 6 p.

Bennett, G.W., 1971, Management of lakes and ponds, second edition:  New York, Van Nostrand Reinhold Company, 375 p.

Bloom, N.S., 1992, On the chemical form of mercury in edible fish and marine invertebrate tissue:  Canadian Journal of Fisheries and Aquatic Sciences, v. 49, p. 1010–1017.

Bowie, A.J., 1905, A practical treatise on hydraulic mining in California:  New York, Van Nostrand, 313 p.

CDFG (California Department of Fish and Game), 2002, Freshwater sport fishing regulations booklet:  Sacramento, California Fish and Game Commission.

Edmondson, J., 2002, Setting priorities--the benefits of revamping the California Department of Fish and Game catchable trout hatchery project:  San Francisco, California Trout, Inc. (Unpublished report).

Grieb, T.M., Driscoll, C.T., Gloss, S.P., Schofield, C.L., Bowie, G.L., and Porcella, D.B., 1990, Factors affecting mercury accumulation in fish in the upper Michigan peninsula:  Environmental Toxicology and Chemistry, v. 9, p. 919–930.

Lange, T.R., Royals, H.E., and Connor, L.L., 1993, Influence of water chemistry on mercury concentration in largemouth bass from Florida lakes:  Transactions of the American Fisheries Society, v. 122, p. 74-84.

Lange T.R.,  Royals, H.E., and Connor, L.L., 1994, Mercury accumulation in largemouth bass (Micropterus salmoides) in a Florida lake:  Archives of Environmental Contamination and Toxicology, v. 27, p. 466-471.

Laarman, P.W., Willford, W.A., and Olson, J.R., 1976, Retention of mercury in the muscle of yellow perch (Perca flavescens) and rock bass (Ambloplities rupestris):  Transactions of the American Fisheries Society, v. 105, p. 296-300.

Leitritz, E., 1970, A history of California's fish hatcheries:  1860-1970:  California Fish and Game Bulletin No. 150, Sacramento.

MacCrimmon, H.R., Wren, C.D., and Gots, B.L., 1983, Mercury uptake by lake trout, Salvelinus namaycush, relative to age, growth, and diet in Tadenac Lake with comparative data from other Precambrian shield lakes:  Canadian Journal of Fisheries and Aquatic Sciences, v. 40, p. 114-120.

McKim, J.M., Olson, G.F., Holcombe, G.W., and Hunt, E.P., 1976, Long-term effects of methylmercuric chloride on three generations of brook trout (Salvelinus fontinalis):  toxicity, accumulation, distribution, and elimination:  Journal of the Fisheries Research Board of Canada, v. 33, p. 2726-2739.

Moyle, P.B., 2002, Inland fishes of California, revised and expanded:  Berkeley, University of California Press, 502 p.

Olsen, K.R., Bergmann, H.L., and Fromm, P.O., 1973, Uptake of methyl mercuric chloride and mercuric chloride by trout:  a study of uptake pathways into the whole animals and uptake by erythrocytes in vitro:  Journal of the Fisheries Research Board of Canada, v. 30, p. 1293-1299.

Phillips, G.R., and Buhler, D.R., 1978, The relative contributions of methylmercury from food or water to rainbow trout (Salmo gairdneri) in a controlled laboratory environment:  Transactions of the American Fisheries Society, v. 107, p. 853-861.

Phillips, G.R., and Gregory, R.W., 1979, Assimilation efficiency of dietary methylmercury by northern pike (Esox lucius):  Journal of the Fisheries Research Board of Canada, v. 36, p. 1516-1519.

Rogers, D.W., Watson, T.A., and Langan, J.S., 1987, Effects of pH and feeding regime on methyl mercury accumulation within aquatic microcosms:  Environmental Pollution, v. 45, p. 261-274.

Spry, D.J., and Wiener, J.G., 1991, Metal bioavailability and toxicity to fish in low-alkalinity lakes:  a critical review:  Environmental Pollution, v. 71, p. 243-304.

Turner, M.A., and Swick, A.L., 1983, The English-Wabigoon river system.  IV.  Interactions between mercury and selenium accumulation from water borne and dietary sources by northern pike (Esox lucius):  Canadian Journal of Fisheries and Aquatic Sciences, v. 40, p. 2241-2250.

USEPA (U.S. Environmental Protection Agency), 2001, Water quality criterion for the protection of human health:  methyl mercury:  EPA-823-R-01-001, Washington, DC, accessed April 27, 2004, at URL http://www.epa.gov/waterscience/criteria/methylmercury/

USFDA (U.S. Food and Drug Administration), 1994, Mercury in fish:  cause for concern?:  FDA Consumer v. 28, no. 7, unnumbered pages, accessed April 27, 2004, at URL http://www.fda.gov/fdac/reprints/mercury.html

Watras, C.J., and Bloom, N.S., 1992, Mercury and methylmercury in individual zooplankton:  implications for bioaccumulation:  Limnology and Oceanography v. 37, no. 6, p. 1313-1318.

Wiener, J.G., and Spry, D.J., 1996, Toxicological significance of mercury in freshwater fish in  Beyer, W.N., Heinz, G.H., and Redmon, A.W., eds., Environmental Contaminants in Wildlife—Interpreting Tissue Concentrations:  Boca Raton, Lewis Publishers, p 297-339.

Wren, C.D., and MacCrimmon, H.R., 1986, Comparative bioaccumulation of mercury in two adjacent freshwater ecosystems:  Water Research, v. 20, p. 763-769.

TABLES AND APPENDIX

Table 1. Vital statistics of fish species sampled from Lake Natoma during 2000-2003, and moisture content and total mercury concentrations in their fillets.  Except for moisture, values are geometric means (ranges in parentheses).  For moisture, values are back-transformed angular means (ranges in parentheses)

[g, gram; mm, millimeter; N, number of samples; %, percent, µg/g, microgram per gram (equivalent to part per million)]

| Fish species | Year | N | Total length (mm) | Weight (g) | Moisture (%) | Mercury (µg/g wet weight) |
| --- | --- | --- | --- | --- | --- | --- |
| Green sunfish | 2002 | 4 | 118 (90-151) | 30.8 (12.5-75.5) | 81.4 (80.8-81.8) | 0.11 (0.06-0.20) |
| Bluegill | 2002 | 97 | 116 (72-174) | 25.2 (5.0-91.5) | 81.8 (80.7-83.2) | 0.08 (0.04-0.18) |
| Redear sunfish | 2002 | 20 | 127 (80-187) | 33.7 (7.5-117.0) | 80.8 (80.1-81.8) | 0.07 (0.03-0.39) |
| Smallmouth bass | 2002 | 2 | 159 (145-174) | 46.0 (35.0-60.5) | 79.8 (79.0-80.6) | 0.13 (0.11-0.16) |
| Spotted bass | 2002 | 3 | 228 (118-335) | 141.9 (15.5-476.0) | 79.3 (77.1-81.9) | 0.27 (0.10-0.49) |
| Largemouth bass | 2000 | 21 | 251 (128-480) | 230.3 (24.0-1655.0) | 79.2 (77.5-81.0) | 0.36 (0.14-1.02) |
|  | 2002 | 61 | 212 (88-490) | 126.3 (7.0-1967.5) | 79.7 (77.6-83.0) | 0.21 (0.06-0.86) |
|  | 2003 | 4 | 225 (174-255) | 167.4 (68-255) | 79.5 (79.2-80.2) | 0.30 (0.25-0.36) |
| White catfish | 2002 | 1 | 249 | 229.5 | 81.8 | 0.56 |
| Black bullhead | 2002 | 1 | 214 | 134.0 | 83.6 | 0.15 |
| Brown bullhead | 2002 | 1 | 317 | 554.0 | 82.1 | 0.35 |
| Channel catfish | 2000 | 1 | 540 | 1867.0 | 77.9 | 1.02 |
|  | 2003 | 10 | 641 (505-750) | 3366.5 (1270.0-5200.0) | 77.9 (76.3-79.6) | 1.50 (1.10-1.89) |
| Rainbow trout | 2002 | 2 | 239 (177-324) | 145.6 (48.0-441.5) | 78.9 (75.8-81.9) | 0.04 (0.02-0.10) |

Table 2.  Spearman rank correlation coefficients between total length and weight of bluegill, redear sunfish, largemouth bass, and channel catfish, and moisture content and total mercury concentrations in their filletsa

[N, number of samples; %, percent, µg/g, microgram per gram (equivalent to part per million)]

| Common name | N | Vital statistic | Moisture (%) | Mercury (µg/g dry weight) |
| --- | --- | --- | --- | --- |
| Bluegill | 97 | Total length | -0.168 | 0.232* |
|  |  | Weight | -0.181 | 0.236* |
| Redear sunfish | 20 | Total length | -0.630** | -0.155 |
|  |  | Weight | -0.595** | -0.190 |
| Largemouth bass | 86 | Total length | -0.934*** | 0.889*** |
|  |  | Weight | -0.933*** | 0.898*** |
| Channel catfish | 11 | Total length | -0.132 | 0.487 |
|  |  | Weight | -0.227 | 0.573 |

aCode:  *, P<0.05; **, P<0.01; ***, P<0.001.

Appendix Table A1.  Raw data for total mercury in fillets of fish collected from Lake Natoma.  Unless indicated otherwise, wet-weight concentrations of mercury were calculated from dry-weight concentrations and moisture values

[Aug., August; Jul., July; Oct., October; Sep., September; Hg, mercury; Moist., moisture content, SL, standard length; TL, total length; wt., weight; µg/g, microgram per gram (equivalent to part per million); %, percent. Site abbreviations (see locations in Figure 1):  AC, Alder Creek arm; DAM, Nimbus Dam; MB, Mississippi Bar; NB, Negro Bar; NS, Natomas Slough; WC, Willow Creek arm. Species abbreviations:  BB, brown bullhead; BG, blugill; BLB, black bullhead; CCF, channel catfish; GS, green sunfish; LMB, largemouth bass, RBT, rainbow trout; RE, redear sunfish; SMB, smallmouth bass; SPB, spotted bass; WCF, white catfish.]

| Sample ID | Site | Capture date | Species | SL (mm) | TL (mm) | Wt. (g) | Moist. (%) | Hg (µg/g dry wt.) | Hg (µg/g wet wt.) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LKNAT-NS-CHCAT-1867g/540TL-0800 | NS | Aug 2000 | CCF | . | 540 | 1867 | 77.2a | 4.600 | 1.049 |
| LKNAT-AC-LMB-24g/128TL-0800 | AC | Aug 2000 | LMB | . | 128 | 24 | 81.0b | 0.830 | 0.158 |
| LKNAT-AC-LMB-56g/165TL-0800 | AC | Aug 2000 | LMB | . | 165 | 56 | 80.3b | 0.910 | 0.179 |
| LKNAT-AC-LMB-422g/300TL-0800 | AC | Aug 2000 | LMB | . | 300 | 422 | 78.7b | 1.280 | 0.273 |
| LKNAT-AC-LMB-504g/315TL-0800 | AC | Aug 2000 | LMB | . | 315 | 504 | 78.6b | 1.580 | 0.338 |
| LKNAT-AC-LMB-709g/360TL-0800 | AC | Aug 2000 | LMB | . | 360 | 709 | 78.3b | 1.650 | 0.358 |
| LKNAT-AC-LMB-1043g/385TL-0800 | AC | Aug 2000 | LMB | . | 385 | 1043 | 78.1b | 2.850 | 0.624 |
| LKNAT-AC-LMB-1655g/480TL-0800 | AC | Aug 2000 | LMB | . | 480 | 1655 | 77.5b | 4.550 | 1.024 |
| LKNAT-NS-LMB-31g/135TL-0800 | NS | Aug 2000 | LMB | . | 135 | 31 | 80.8b | 0.740 | 0.142 |
| LKNAT-NS-LMB-266g/270TL-0800 | NS | Aug 2000 | LMB | . | 270 | 266 | 79.0b | 1.580 | 0.332 |
| LKNAT-NS-LMB-923g/390TL-0800 | NS | Aug 2000 | LMB | . | 390 | 923 | 78.1b | 2.850 | 0.624 |
| LKNAT-NS-LMB-1243g/410TL-0800 | NS | Aug 2000 | LMB | . | 410 | 1243 | 77.9b | 3.570 | 0.789 |
| LKNAT-WC-LMB-35g/140TL-0800 | WC | Aug 2000 | LMB | . | 140 | 35 | 80.8b | 0.990 | 0.190 |
| LKNAT-WC-LMB-42g/145TL-0800 | WC | Aug 2000 | LMB | . | 145 | 42 | 80.7b | 1.310 | 0.253 |
| LKNAT-WC-LMB-72g/175TL-0800 | WC | Aug 2000 | LMB | . | 175 | 72 | 80.2b | 1.190 | 0.236 |
| LKNAT-WC-LMB-91g/190TL-0800 | WC | Aug 2000 | LMB | . | 190 | 91 | 79.9b | 1.390 | 0.279 |
| LKNAT-WC-LMB-92g/185TL-0800 | WC | Aug 2000 | LMB | . | 185 | 92 | 80.0b | 1.310 | 0.262 |
| LKNAT-WC-LMB-94g/185TL-0800 | WC | Aug 2000 | LMB | . | 185 | 94 | 80.0b | 1.260 | 0.252 |
| LKNAT-WC-LMB-407g/300TL-0800 | WC | Aug 2000 | LMB | . | 300 | 407 | 78.7b | 3.290 | 0.701 |
| LKNAT-WC-LMB-490g/320TL-0800 | WC | Aug 2000 | LMB | . | 320 | 490 | 78.6b | 3.170 | 0.678 |
| LKNAT-WC-LMB-590g/330TL-0800 | WC | Aug 2000 | LMB | . | 330 | 590 | 78.5b | 2.950 | 0.634 |
| LKNAT-WC-LMB-1247g/425TL-0800 | WC | Aug 2000 | LMB | . | 425 | 1247 | 77.8b | 2.710 | 0.602 |
| LN-205-F | NB | Sep-Oct 2002 | BB | 275 | 317 | 554 | 82.1 | 1.966 | 0.353 |
| LN-006-F | AC | Sep-Oct 2002 | BG | 75 | 95 | 12.5 | 81.8 | 0.420 | 0.076 |
| LN-007-F | AC | Sep-Oct 2002 | BG | 76 | 97 | 13 | 81.9 | 0.261 | 0.047 |
| LN-021-F | AC | Sep-Oct 2002 | BG | 74 | 95 | 12 | 81.2 | 0.505 | 0.095 |
| LN-066-F | AC | Sep-Oct 2002 | BG | 88 | 108 | 20 | 82.0 | 0.463 | 0.083 |
| LN-068-F | AC | Sep-Oct 2002 | BG | 77 | 90 | 10 | 81.5 | 0.529 | 0.098 |
| LN-084-F | AC | Sep-Oct 2002 | BG | 82 | 106 | 17.5 | 81.3 | 0.348 | 0.065 |
| LN-085-F | AC | Sep-Oct 2002 | BG | 111 | 144 | 50 | 81.3 | 0.455 | 0.085 |
| LN-124-F | AC | Sep-Oct 2002 | BG | 110 | 140 | 44.5 | 81.5 | 0.505 | 0.093 |
| LN-128-F | AC | Sep-Oct 2002 | BG | 115 | 135 | 41.5 | 81.4 | 0.336 | 0.062 |
| LN-139-F | AC | Sep-Oct 2002 | BG | 96 | 115 | 33 | 80.9 | 0.438 | 0.083 |
| LN-206-F | AC | Sep-Oct 2002 | BG | 80 | 116 | 18.5 | 82.1 | 0.366 | 0.066 |
| LN-231-F | AC | Sep-Oct 2002 | BG | 96 | 120 | 28.5 | 80.9 | 0.319 | 0.061 |
| LN-247-F | AC | Sep-Oct 2002 | BG | 90 | 112 | 22 | 82.7 | 0.287 | 0.050 |
| LN-248-F | AC | Sep-Oct 2002 | BG | 86 | 107 | 19 | 82.0 | 0.374 | 0.067 |
| LN-257-F | AC | Sep-Oct 2002 | BG | 89 | 111 | 20.5 | 81.8 | 0.296 | 0.054 |
| LN-271-F | AC | Sep-Oct 2002 | BG | 80 | 104 | 17 | 82.3 | 0.285 | 0.050 |
| LN-278-F | AC | Sep-Oct 2002 | BG | 110 | 140 | 41 | 81.7 | 0.454 | 0.083 |
| LN-285-F | AC | Sep-Oct 2002 | BG | 84 | 105 | 16 | 82.7 | 0.596 | 0.103 |
| LN-287-F | AC | Sep-Oct 2002 | BG | 95 | 119 | 29 | 82.2 | 0.369 | 0.066 |
| LN-026-F | DAM | Sep-Oct 2002 | BG | 84 | 105 | 16.5 | 82.0 | 0.338 | 0.061 |
| LN-044-F | DAM | Sep-Oct 2002 | BG | 80 | 104 | 16.5 | 81.6 | 0.551 | 0.101 |
| LN-047-F | DAM | Sep-Oct 2002 | BG | 83 | 104 | 17 | 81.7 | 0.480 | 0.088 |
| LN-051-F | DAM | Sep-Oct 2002 | BG | 83 | 108 | 19 | 82.3 | 0.330 | 0.058 |
| LN-057-F | DAM | Sep-Oct 2002 | BG | 93 | 115 | 24.5 | 81.3 | 0.425 | 0.080 |
| LN-073-F | DAM | Sep-Oct 2002 | BG | 79 | 101 | 17.5 | 82.3 | 0.367 | 0.065 |
| LN-092-F | DAM | Sep-Oct 2002 | BG | 73 | 85 | 12.5 | 81.9 | 0.323 | 0.058 |
| LN-126-F | DAM | Sep-Oct 2002 | BG | 100 | 123 | 34.5 | 82.7 | 0.485 | 0.084 |
| LN-127-F | DAM | Sep-Oct 2002 | BG | 73 | 93 | 11 | 81.2 | 0.436 | 0.082 |
| LN-148-F | DAM | Sep-Oct 2002 | BG | 84 | 104 | 17 | 81.8 | 0.336 | 0.061 |
| LN-171-F | DAM | Sep-Oct 2002 | BG | 85 | 105 | 18 | 81.9 | 0.357 | 0.065 |
| LN-177-F | DAM | Sep-Oct 2002 | BG | 112 | 133 | 36 | 82.2 | 0.286 | 0.051 |
| LN-182-F | DAM | Sep-Oct 2002 | BG | 77 | 99 | 17 | 81.4 | 0.504 | 0.094 |
| LN-190-F | DAM | Sep-Oct 2002 | BG | 88 | 109 | 19 | 81.3 | 0.261 | 0.049 |
| LN-193-F | DAM | Sep-Oct 2002 | BG | 90 | 114 | 22.5 | 81.9 | 0.377 | 0.068 |
| LN-195-F | DAM | Sep-Oct 2002 | BG | 90 | 114 | 22.5 | 82.4 | 0.365 | 0.064 |
| LN-228-F | DAM | Sep-Oct 2002 | BG | 102 | 131 | 34 | 81.9 | 0.830 | 0.150 |
| LN-232-F | DAM | Sep-Oct 2002 | BG | 92 | 115 | 23.5 | 81.6 | 0.509 | 0.094 |
| LN-252-F | DAM | Sep-Oct 2002 | BG | 105 | 131 | 35 | 81.2 | 0.306 | 0.058 |
| LN-258-F | DAM | Sep-Oct 2002 | BG | 125 | 157 | 69.5 | 81.9 | 0.708 | 0.128 |
| LN-275-F | DAM | Sep-Oct 2002 | BG | 84 | 107 | 21 | 81.6 | 0.341 | 0.063 |
| LN-276-F | DAM | Sep-Oct 2002 | BG | 71 | 93 | 12.5 | 83.2 | 0.453 | 0.076 |
| LN-005-F | MB | Sep-Oct 2002 | BG | 111 | 129 | 33.5 | 81.7 | 0.372 | 0.068 |
| LN-008-F | MB | Sep-Oct 2002 | BG | 98 | 121 | 28.5 | 81.9 | 0.408 | 0.074 |
| LN-009-F | MB | Sep-Oct 2002 | BG | 70 | 92 | 12 | 81.9 | 0.465 | 0.084 |
| LN-012-F | MB | Sep-Oct 2002 | BG | 95 | 120 | 25 | 82.0 | 0.510 | 0.092 |
| LN-013-F | MB | Sep-Oct 2002 | BG | 84 | 106 | 19.5 | 81.3 | 0.522 | 0.098 |
| LN-014-F | MB | Sep-Oct 2002 | BG | 98 | 120 | 29.5 | 82.4 | 0.722 | 0.127 |
| LN-015-F | MB | Sep-Oct 2002 | BG | 108 | 139 | 42 | 83.0 | 0.345 | 0.059 |
| LN-020-F | MB | Sep-Oct 2002 | BG | 120 | 150 | 45.8 | 81.6 | 0.613 | 0.113 |
| LN-030-F | MB | Sep-Oct 2002 | BG | 84 | 106 | 18 | 82.5 | 0.304 | 0.053 |
| LN-054-F | MB | Sep-Oct 2002 | BG | 89 | 112 | 22.5 | 80.9 | 0.545 | 0.104 |
| LN-056-F | MB | Sep-Oct 2002 | BG | 107 | 135 | 37 | 81.8 | 0.585 | 0.107 |
| LN-060-F | MB | Sep-Oct 2002 | BG | 111 | 136 | 45 | 81.4 | 0.570 | 0.106 |
| LN-082-F | MB | Sep-Oct 2002 | BG | 75 | 95 | 12 | 82.1 | 0.467 | 0.083 |
| LN-097-F | MB | Sep-Oct 2002 | BG | 104 | 129 | 35 | 81.1 | 0.350 | 0.066 |
| LN-164-F | MB | Sep-Oct 2002 | BG | 126 | 156 | 61.5 | 81.9 | 0.532 | 0.096 |
| LN-189-F | MB | Sep-Oct 2002 | BG | 111 | 133 | 36.5 | 81.6 | 0.728 | 0.134 |
| LN-199-F | MB | Sep-Oct 2002 | BG | 124 | 152 | 59 | 81.6 | 0.783 | 0.144 |
| LN-215-F | MB | Sep-Oct 2002 | BG | 82 | 103 | 17.5 | 81.2 | 0.495 | 0.093 |
| LN-218-F | MB | Sep-Oct 2002 | BG | 83 | 105 | 18.5 | 82.4 | 0.329 | 0.058 |
| LN-237-F | MB | Sep-Oct 2002 | BG | 95 | 120 | 24.5 | 80.9 | 0.383 | 0.073 |
| LN-241-F | MB | Sep-Oct 2002 | BG | 124 | 152 | 60 | 81.2 | 0.295 | 0.055 |
| LN-264-F | MB | Sep-Oct 2002 | BG | 72 | 90 | 11 | 81.7 | 0.772 | 0.141 |
| LN-270-F | MB | Sep-Oct 2002 | BG | 85 | 113 | 20.5 | 82.3 | 0.499 | 0.088 |
| LN-289-F | MB | Sep-Oct 2002 | BG | 116 | 149 | 53.5 | 81.5 | 0.444 | 0.082 |
| LN-291-F | MB | Sep-Oct 2002 | BG | 78 | 100 | 14.5 | 82.7 | 0.605 | 0.105 |
| LN-011-F | NB | Sep-Oct 2002 | BG | 137 | 174 | 91.5 | 82.6 | 1.064 | 0.185 |
| LN-027-F | NB | Sep-Oct 2002 | BG | 119 | 152 | 67 | 81.7 | 0.399 | 0.073 |
| LN-031-F | NB | Sep-Oct 2002 | BG | 73 | 95 | 15 | 82.1 | 0.313 | 0.056 |
| LN-039-F | NB | Sep-Oct 2002 | BG | 111 | 143 | 48 | 81.3 | 0.782 | 0.146 |
| LN-045-F | NB | Sep-Oct 2002 | BG | 127 | 152 | 68.5 | 81.5 | 0.626 | 0.116 |
| LN-075-F | NB | Sep-Oct 2002 | BG | 74 | 98 | 15.5 | 83.1 | 0.614 | 0.104 |
| LN-083-F | NB | Sep-Oct 2002 | BG | 78 | 102 | 17.5 | 82.0 | 0.276 | 0.050 |
| LN-103-F | NB | Sep-Oct 2002 | BG | 124 | 155 | 68.5 | 81.0 | 0.529 | 0.100 |
| LN-104-F | NB | Sep-Oct 2002 | BG | 131 | 159 | 81 | 82.0 | 0.669 | 0.121 |
| LN-106-F | NB | Sep-Oct 2002 | BG | 128 | 161 | 69.5 | 81.7 | 0.676 | 0.124 |
| LN-141-F | NB | Sep-Oct 2002 | BG | 83 | 108 | 21 | 82.1 | 0.870 | 0.155 |
| LN-181-F | NB | Sep-Oct 2002 | BG | 112 | 143 | 52.5 | 81.9 | 0.566 | 0.102 |
| LN-194-F | NB | Sep-Oct 2002 | BG | 119 | 151 | 67 | 81.7 | 0.883 | 0.162 |
| LN-211-F | NB | Sep-Oct 2002 | BG | 120 | 154 | 69.5 | 81.8 | 0.499 | 0.091 |
| LN-221-F | NB | Sep-Oct 2002 | BG | 122 | 151 | 67.5 | 81.1 | 0.349 | 0.066 |
| LN-233-F | NB | Sep-Oct 2002 | BG | 80 | 102 | 16.5 | 81.7 | 0.412 | 0.075 |
| LN-259-F | NB | Sep-Oct 2002 | BG | 74 | 97 | 14.5 | 82.8 | 0.461 | 0.079 |
| LN-261-F | NB | Sep-Oct 2002 | BG | 121 | 150 | 65.5 | 82.8 | 0.669 | 0.115 |
| LN-268-F | NB | Sep-Oct 2002 | BG | 115 | 148 | 59.5 | 82.2 | 0.356 | 0.063 |
| LN-283-F | NB | Sep-Oct 2002 | BG | 93 | 117 | 30 | 81.2 | 0.437 | 0.082 |
| LN-292-F | NB | Sep-Oct 2002 | BG | 69 | 91 | 11.5 | 82.2 | 0.511 | 0.091 |
| LN-294-F | NB | Sep-Oct 2002 | BG | 112 | 136 | 50 | 80.7 | 0.573 | 0.110 |
| LN-022-F | WC | Sep-Oct 2002 | BG | 56 | 73 | 5 | 81.5 | 0.455 | 0.084 |
| LN-049-F | WC | Sep-Oct 2002 | BG | 58 | 72 | 5 | 80.8 | 0.593 | 0.114 |
| LN-088-F | WC | Sep-Oct 2002 | BG | 85 | 110 | 19 | 81.2 | 0.362 | 0.068 |
| LN-111-F | WC | Sep-Oct 2002 | BG | 61 | 81 | 8 | 82.5 | 0.900 | 0.158 |
| LN-114-F | WC | Sep-Oct 2002 | BG | 80 | 103 | 16.5 | 81.6 | 0.268 | 0.049 |
| LN-135-F | WC | Sep-Oct 2002 | BG | 132 | 160 | 75.5 | 81.4 | 0.752 | 0.140 |
| LN-154-F | WC | Sep-Oct 2002 | BG | 78 | 102 | 16 | 81.4 | 0.223 | 0.041 |
| LN-160-F | WC | Sep-Oct 2002 | BG | 53 | 73 | 5 | 82.0 | 0.674 | 0.121 |
| LN-169-F | WC | Sep-Oct 2002 | BG | 92 | 123 | 23.5 | 81.9 | 0.311 | 0.056 |
| LN-210-F | WC | Sep-Oct 2002 | BLB | 189 | 214 | 134 | 83.6 | 0.881 | 0.145 |
| LN-152-F | DAM | Sep-Oct 2002 | GS | 75 | 90 | 12.5 | 81.4 | 0.331 | 0.061 |
| LN-334-F | MB | Sep-Oct 2002 | GS | 108 | 126 | 36.5 | 81.8 | 0.611 | 0.111 |
| LN-390-F | MB | Sep-Oct 2002 | GS | 97 | 115 | 26 | 81.5 | 0.529 | 0.098 |
| LN-227-F | NB | Sep-Oct 2002 | GS | 130 | 151 | 75.5 | 80.8 | 1.024 | 0.196 |
| LN-010-F | AC | Sep-Oct 2002 | LMB | 260 | 301 | 399 | 77.6 | 1.639 | 0.367 |
| LN-029-F | AC | Sep-Oct 2002 | LMB | 291 | 340 | 562 | 79.4 | 2.697 | 0.557 |
| LN-071-F | AC | Sep-Oct 2002 | LMB | 333 | 395 | 941 | 77.9 | 2.603 | 0.576 |
| LN-129-F | AC | Sep-Oct 2002 | LMB | 200 | 238 | 188.5 | 79.0 | 1.019 | 0.214 |
| LN-142-F | AC | Sep-Oct 2002 | LMB | 305 | 353 | 658 | 78.3 | 1.232 | 0.268 |
| LN-143-F | AC | Sep-Oct 2002 | LMB | 187 | 220 | 146 | 79.5 | 1.039 | 0.213 |
| LN-153-F | AC | Sep-Oct 2002 | LMB | 152 | 178 | 72 | 79.5 | 0.648 | 0.133 |
| LN-157-F | AC | Sep-Oct 2002 | LMB | 327 | 378 | 939 | 79.0 | 2.309 | 0.485 |
| LN-180-F | AC | Sep-Oct 2002 | LMB | 173 | 201 | 118 | 79.1 | 0.769 | 0.161 |
| LN-191-F | AC | Sep-Oct 2002 | LMB | 97 | 117 | 15.5 | 81.1 | 0.507 | 0.096 |
| LN-208-F | AC | Sep-Oct 2002 | LMB | 130 | 158 | 38.5 | 80.7 | 0.624 | 0.121 |
| LN-219-F | AC | Sep-Oct 2002 | LMB | 130 | 151 | 43.5 | 81.2 | 0.674 | 0.127 |
| LN-245-F | AC | Sep-Oct 2002 | LMB | 354 | 407 | 1141 | 78.1 | 2.762 | 0.604 |
| LN-246-F | AC | Sep-Oct 2002 | LMB | 157 | 190 | 92 | 80.3 | 0.584 | 0.115 |
| LN-272-F | AC | Sep-Oct 2002 | LMB | 360 | 425 | 1285 | 77.9 | 2.605 | 0.577 |
| LN-312-F | AC | Sep-Oct 2002 | LMB | 154 | 188 | 86.5 | 79.5 | 1.050 | 0.215 |
| LN-042-F | DAM | Sep-Oct 2002 | LMB | 114 | 139 | 25 | 81.4 | 0.659 | 0.122 |
| LN-070-F | DAM | Sep-Oct 2002 | LMB | 253 | 292 | 365 | 78.3 | 1.104 | 0.240 |
| LN-121-F | DAM | Sep-Oct 2002 | LMB | 90 | 110 | 14 | 82.9 | 0.602 | 0.103 |
| LN-168-F | DAM | Sep-Oct 2002 | LMB | 70 | 88 | 7 | 81.2 | 0.345 | 0.065 |
| LN-202-F | DAM | Sep-Oct 2002 | LMB | 129 | 153 | 40.5 | 79.7 | 0.453 | 0.092 |
| LN-204-F | DAM | Sep-Oct 2002 | LMB | 73 | 90 | 7 | 81.7 | 0.339 | 0.062 |
| LN-255-F | DAM | Sep-Oct 2002 | LMB | 76 | 95 | 8 | 83.0 | 0.453 | 0.077 |
| LN-002-F | MB | Sep-Oct 2002 | LMB | 335 | 390 | 837 | 78.4 | 1.308 | 0.282 |
| LN-023-F | MB | Sep-Oct 2002 | LMB | 432 | 490 | 1967.5 | 78.0 | 3.661 | 0.807 |
| LN-134-F | MB | Sep-Oct 2002 | LMB | 132 | 154 | 44 | 79.5 | 0.996 | 0.204 |
| LN-136-F | MB | Sep-Oct 2002 | LMB | 87 | 105 | 12 | 81.8 | 0.538 | 0.098 |
| LN-159-F | MB | Sep-Oct 2002 | LMB | 124 | 159 | 39 | 81.0 | 0.530 | 0.101 |
| LN-240-F | MB | Sep-Oct 2002 | LMB | 90 | 110 | 13.5 | 82.1 | 0.387 | 0.069 |
| LN-040-F | NB | Sep-Oct 2002 | LMB | 179 | 208 | 130 | 79.0 | 0.738 | 0.155 |
| LN-091-F | NB | Sep-Oct 2002 | LMB | 209 | 244 | 260 | 79.1 | 1.228 | 0.256 |
| LN-098-F | NB | Sep-Oct 2002 | LMB | 198 | 232 | 161 | 79.0 | 0.930 | 0.196 |
| LN-099-F | NB | Sep-Oct 2002 | LMB | 169 | 201 | 110.5 | 79.9 | 1.148 | 0.231 |
| LN-100-F | NB | Sep-Oct 2002 | LMB | 110 | 132 | 30 | 82.5 | 0.632 | 0.111 |
| LN-117-F | NB | Sep-Oct 2002 | LMB | 146 | 181 | 81.5 | 79.5 | 0.934 | 0.192 |
| LN-144-F | NB | Sep-Oct 2002 | LMB | 135 | 164 | 52 | 80.1 | 0.495 | 0.099 |
| LN-150-F | NB | Sep-Oct 2002 | LMB | 172 | 206 | 133.5 | 79.4 | 1.125 | 0.232 |
| LN-165-F | NB | Sep-Oct 2002 | LMB | 124 | 149 | 40.5 | 80.6 | 0.689 | 0.133 |
| LN-172-F | NB | Sep-Oct 2002 | LMB | 158 | 181 | 98 | 80.6 | 0.610 | 0.118 |
| LN-220-F | NB | Sep-Oct 2002 | LMB | 135 | 159 | 58 | 80.4 | 0.663 | 0.130 |
| LN-224-F | NB | Sep-Oct 2002 | LMB | 354 | 415 | 1197.5 | 78.6 | 3.241 | 0.695 |
| LN-253-F | NB | Sep-Oct 2002 | LMB | 139 | 169 | 67.5 | 80.3 | 0.755 | 0.149 |
| LN-254-F | NB | Sep-Oct 2002 | LMB | 146 | 179 | 64.5 | 79.4 | 0.809 | 0.167 |
| LN-281-F | NB | Sep-Oct 2002 | LMB | 168 | 281 | 121.5 | 79.5 | 0.894 | 0.183 |
| LN-282-F | NB | Sep-Oct 2002 | LMB | 244 | 290 | 370 | 78.9 | 1.998 | 0.421 |
| LN-048-F | WC | Sep-Oct 2002 | LMB | 166 | 193 | 96.5 | 79.4 | 1.178 | 0.243 |
| LN-052-F | WC | Sep-Oct 2002 | LMB | 145 | 173 | 66 | 80.4 | 0.865 | 0.169 |
| LN-076-F | WC | Sep-Oct 2002 | LMB | 289 | 341 | 572 | 78.7 | 2.607 | 0.555 |
| LN-078-F | WC | Sep-Oct 2002 | LMB | 175 | 204 | 110 | 79.9 | 0.824 | 0.165 |
| LN-086-F | WC | Sep-Oct 2002 | LMB | 316 | 361 | 913.5 | 78.2 | 1.751 | 0.383 |
| LN-101-F | WC | Sep-Oct 2002 | LMB | 125 | 150 | 38.5 | 80.0 | 0.673 | 0.135 |
| LN-112-F | WC | Sep-Oct 2002 | LMB | 386 | 446 | 1448.5 | 78.3 | 3.186 | 0.692 |
| LN-185-F | WC | Sep-Oct 2002 | LMB | 239 | 274 | 299 | 78.6 | 1.172 | 0.251 |
| LN-213-F | WC | Sep-Oct 2002 | LMB | 184 | 216 | 129.5 | 79.7 | 1.130 | 0.230 |
| LN-222-F | WC | Sep-Oct 2002 | LMB | 215 | 245 | 206 | 78.8 | 0.880 | 0.186 |
| LN-229-F | WC | Sep-Oct 2002 | LMB | 317 | 369 | 858 | 77.7 | 3.853 | 0.859 |
| LN-235-F | WC | Sep-Oct 2002 | LMB | 323 | 375 | 921 | 78.4 | 2.392 | 0.516 |
| LN-250-F | WC | Sep-Oct 2002 | LMB | 236 | 275 | 355.5 | 77.6 | 1.265 | 0.283 |
| LN-273-F | WC | Sep-Oct 2002 | LMB | 140 | 169 | 60.5 | 79.4 | 1.191 | 0.246 |
| LN-280-F | WC | Sep-Oct 2002 | LMB | 139 | 164 | 61 | 79.9 | 0.756 | 0.152 |
| LN-284-F | WC | Sep-Oct 2002 | LMB | 204 | 239 | 196.5 | 79.3 | 0.932 | 0.193 |
| LN-132-F | DAM | Sep-Oct 2002 | RBT | 282 | 324 | 441.5 | 75.8 | 0.082 | 0.020 |
| LN-300-F | NB | Sep-Oct 2002 | RBT | 146 | 177 | 48 | 81.9 | 0.537 | 0.097 |
| LN-368-F | MB | Sep-Oct 2002 | RE | 71 | 87 | 10 | 80.9 | 0.553 | 0.106 |
| LN-380-F | MB | Sep-Oct 2002 | RE | 110 | 134 | 38 | 80.7 | 0.146 | 0.028 |
| LN-034-F | NB | Sep-Oct 2002 | RE | 94 | 116 | 25 | 80.7 | 0.211 | 0.041 |
| LN-225-F | NB | Sep-Oct 2002 | RE | 115 | 141 | 44.5 | 80.5 | 0.235 | 0.046 |
| LN-315-F | NB | Sep-Oct 2002 | RE | 119 | 145 | 51.5 | 80.1 | 0.602 | 0.120 |
| LN-328-F | NB | Sep-Oct 2002 | RE | 116 | 142 | 47 | 80.6 | 1.995 | 0.388 |
| LN-381-F | NB | Sep-Oct 2002 | RE | 110 | 137 | 43.5 | 80.3 | 0.371 | 0.073 |
| LN-087-F | WC | Sep-Oct 2002 | RE | . | 80 | 7.5 | 81.8 | 0.360 | 0.065 |
| LN-302-F | WC | Sep-Oct 2002 | RE | 132 | 161 | 69.5 | 80.6 | 0.159 | 0.031 |
| LN-306-F | WC | Sep-Oct 2002 | RE | 103 | 126 | 31.5 | 81.2 | 0.378 | 0.071 |
| LN-308-F | WC | Sep-Oct 2002 | RE | 96 | 116 | 29.5 | 80.7 | 0.478 | 0.092 |
| LN-318-F | WC | Sep-Oct 2002 | RE | 96 | 122 | 29.5 | 80.6 | 0.538 | 0.104 |
| LN-323-F | WC | Sep-Oct 2002 | RE | 113 | 129 | 35.5 | 80.6 | 0.294 | 0.057 |
| LN-324-F | WC | Sep-Oct 2002 | RE | 110 | 136 | 40.5 | 81.0 | 0.886 | 0.168 |
| LN-329-F | WC | Sep-Oct 2002 | RE | 117 | 143 | 49.5 | 80.8 | 0.319 | 0.061 |
| LN-338-F | WC | Sep-Oct 2002 | RE | 96 | 117 | 26 | 80.4 | 0.685 | 0.135 |
| LN-344-F | WC | Sep-Oct 2002 | RE | 115 | 140 | 51.5 | 80.7 | 0.268 | 0.052 |
| LN-355-F | WC | Sep-Oct 2002 | RE | . | 187 | 117 | 80.4 | 0.368 | 0.072 |
| LN-358-F | WC | Sep-Oct 2002 | RE | 87 | 112 | 21.5 | 81.3 | 0.417 | 0.078 |
| LN-382-F | WC | Sep-Oct 2002 | RE | . | 114 | 23.5 | 81.2 | 0.538 | 0.101 |
| LN-295-F | AC | Sep-Oct 2002 | SMB | 142 | 174 | 60.5 | 79.0 | 0.511 | 0.107 |
| LN-077-F | MB | Sep-Oct 2002 | SMB | 117 | 145 | 35 | 80.6 | 0.842 | 0.163 |
| LN-032-F | AC | Sep-Oct 2002 | SPB | 249 | 300 | 387.5 | 78.7 | 2.291 | 0.488 |
| LN-335-F | DAM | Sep-Oct 2002 | SPB | 281 | 335 | 476 | 77.1 | 1.778 | 0.407 |
| LN-036-F | MB | Sep-Oct 2002 | SPB | 95 | 118 | 15.5 | 81.9 | 0.527 | 0.096 |
| LN-019-F | NB | Sep-Oct 2002 | WCF | 221 | 249 | 229.5 | 81.8 | 3.069 | 0.560 |
| LKNATWILLCHCAT-3450g/630TL-070103 | WC | Jul 2003 | CCF | . | 630 | 3450 | 77.4 | 8.339 | 1.785c |
| LKNATWILLCHCAT-1270g/505TL-070103 | WC | Jul 2003 | CCF | . | 505 | 1270 | 80.1 | 7.880 | 1.610c |
| LKNATWILLCHCAT-4950g/682TL-070103 | WC | Jul 2003 | CCF | . | 682 | 4950 | 76.4 | 7.714 | 1.716c |
| LKNATWILLCHCAT-2250g/555TL-070103 | WC | Jul 2003 | CCF | . | 555 | 2250 | 74.5 | 4.882 | 1.098c |
| LKNATWILLCHCAT-4120g/682TL-070103 | WC | Jul 2003 | CCF | . | 682 | 4120 | 75.6 | 6.224 | 1.434c |
| LKNATWILLCHCAT-2790g/615TL-070103 | WC | Jul 2003 | CCF | . | 615 | 2790 | 77.7 | 4.846 | 1.103c |
| LKNATWILLCHCAT-5200g/750TL-070103 | WC | Jul 2003 | CCF | . | 750 | 5200 | 77.6 | 7.977 | 1.887c |
| LKNATWILLCHCAT-4110g/685TL-070103 | WC | Jul 2003 | CCF | . | 685 | 4110 | 77.0 | 7.146 | 1.601c |
| LKNATWILLCHCAT-4560g/690TL-070103 | WC | Jul 2003 | CCF | . | 690 | 4560 | 77.5 | 7.234 | 1.576c |
| LKNATWILLCHCAT-3420g/649TL-070103 | WC | Jul 2003 | CCF | . | 649 | 3420 | 77.8 | 6.837 | 1.444c |
| WILLKNAT-LMB-68g/174TL-070203 | WC | Jul 2003 | LMB | . | 174 | 68 | 80.2c | 1.278 | 0.253 |
| WILLKNAT-LMB-195g/228TL-070203 | WC | Jul 2003 | LMB | . | 228 | 195 | 79.5c | 1.585 | 0.325 |
| WILLKNAT-LMB-232g/252TL-070203 | WC | Jul 2003 | LMB | . | 252 | 232 | 79.2c | 1.736 | 0.361 |
| WILLKNAT-LMB-255g/255TL-070203 | WC | Jul 2003 | LMB | . | 255 | 255 | 79.2c | 1.312 | 0.273 |

aMoisture content (estimated as the arcsine-transformed mean for the 2003 CCF data) was used to convert Hg dry-weight concentration to wet-weight concentration.

bMoisture content (estimated from the total length-moisture relation for 2002 LMB data, %Moist=95.1*TL-0.0331, R2=0.780, N=61) was used either to convert Hg dry-weight concentrations to wet-weight concentrations (samples collected in 2000) or to convert Hg wet-weight concentrations to dry-weight concentrations (samples collected in 2003).

cWet-weight concentrations were measured directly from fresh (wet) samples and not estimated from dry-weight concentrations and moisture values.

Appendix Table A2.  Results of laboratory intercomparison for total mercury in fish tissue.

[CERC, Columbia Environmental Research Center; UC Davis, University of California, Davis; Hg, mercury; µg/g, microgram per gram (equivalent to part per million); Relative percent difference (RPD) computed as RPD = (v1-v2)/[(v1+v2)/2], where v1 and v2 are values under comparison]

|  |  |  |  | CERC | UC Davis | Relative Percent |
| --- | --- | --- | --- | --- | --- | --- |
|  | USGS |  |  | Hg | Hg | Difference |
| CERC # | Field/Lab ID | UCDavis ID | Species | µg/g dry | µg/g dry | (%) |
| 28612 | LN-245-F | CERC, Columbia, MO-A | Largemouth Bass | 2.76 | 2.67 | 3.48 |
| 28610 | LN-157-F | CERC, Columbia, MO-B | Largemouth Bass | 2.31 | 2.35 | -1.92 |
| 28594 | LN-086-F | CERC, Columbia, MO-C | Largemouth Bass | 1.75 | 1.63 | 7.03 |
| 28592 | LN-250-F | CERC, Columbia, MO-D | Largemouth Bass | 1.27 | 1.19 | 6.90 |
| 28619 | LN-023-F | CERC, Columbia, MO-E | Largemouth Bass | 3.66 | 3.54 | 3.33 |
| 28590 | LN-222-F | CERC, Columbia, MO-F | Largemouth Bass | 0.88 | 0.91 | -2.85 |
| 28581 | LN-070-F | CERC, Columbia, MO-G | Largemouth Bass | 1.10 | 1.04 | 5.21 |
| 28561 | LN-220-F | CERC, Columbia, MO-H | Largemouth Bass | 0.66 | 0.68 | -3.34 |
| 28573 | LN-282-F | CERC, Columbia, MO-I | Largemouth Bass | 2.00 | 2.05 | -2.59 |
| 28574 | LN-224-F | CERC, Columbia, MO-J | Largemouth Bass | 3.24 | 3.13 | 3.57 |
| 31531 | . | LKNATWILLCHCAT-1270G/505TL-070103 | Channel Catfish | 8.21 | 7.88 | 4.07 |
| 31533 | . | LKNATWILLCHCAT-2250G/555TL-070103 | Channel Catfish | 5.05 | 4.88 | 3.35 |
| 31535 | . | LKNATWILLCHCAT-2790G/615TL-070103 | Channel Catfish | 5.03 | 4.85 | 3.74 |
| 31539 | . | LKNATWILLCHCAT-3420G/649TL-070103 | Channel Catfish | 7.21 | 6.84 | 5.34 |
| 31530 | . | LKNATWILLCHCAT-3450G/630TL-070103 | Channel Catfish | 8.57 | 8.34 | 2.71 |
| 31537 | . | LKNATWILLCHCAT-4110G/685TL-070103 | Channel Catfish | 7.49 | 7.15 | 4.65 |
| 31534 | . | LKNATWILLCHCAT-4120G/682TL-070103 | Channel Catfish | 6.43 | 6.22 | 3.28 |
| 31538 | . | LKNATWILLCHCAT-4560G/690TL-070103 | Channel Catfish | 7.45 | 7.23 | 2.98 |
| 31532 | . | LKNATWILLCHCAT-4950G/682TL-070103 | Channel Catfish | 7.95 | 7.71 | 3.08 |
| 31536 | . | LKNATWILLCHCAT-5200G/750TL-070103 | Channel Catfish | 8.32 | 7.98 | 4.16 |
|  |  |  |  |  | average | 2.81 |
|  |  |  |  |  | s.d. | 3.06 |

FIGURES

Figure 1.  Map of the study area showing general locations of sampling sites.

Figure 2.  Relation between total length (TL) of 97 bluegill and total mercury (Hg) concentrations in their skinless fillets.  The relation between TL and Hg was described by a “best fit” linear equation as follows:  Hg=0.0415 + 0.000388, R2=0.0874.

Figure 3.  Relation between total length (TL) of 20 redear sunfish and total mercury (Hg) concentrations in their skinless fillets.  The relation between TL and Hg was not statistically significant.

Figure 4.  Relation between total length (TL) of 86 largemouth bass and total mercury (Hg) concentrations in their skinless fillets.  The relation between TL and Hg was described by a “best fit” power-curve equation as follows:  Hg=0.000112*TL1.42, R2=0.819.

Figure 5.  Relation between total length (TL) of 11 channel catfish and total mercury (Hg) concentrations in their skinless fillets.  The relation between TL and Hg was not statistically significant.

Figure 6.  Relation between total length (TL) of miscellaneous fishes and total mercury (Hg) concentrations in their skinless fillets.

Appendix Figure A1.  Results of laboratory intercomparison for mercury in fish tissue.

Squares represent largemouth bass samples and circles denote channel catfish samples.

Dotted line is 1:1 correspondence and solid line is linear least-squares regression, for which

equation and R2 value are shown.