Mercury Contamination from Historical Gold Mining in the Greenhorn Creek Drainage,

Bear River Watershed, California, 1999–2001

Charles N. Alpers, Michael P. Hunerlach1, Jason T. May1, Roger L. Hothem2, Howard E. Taylor3, and John F. DeWild4

U.S. Geological Survey

Data Series XXXX-XXXX

Prepared in cooperation with:

Bureau of Land Management

California State Water Resources Control Board

Nevada County Resource Conservation District

U.S. Department of Agriculture – Forest Service

### U.S. DEPARTMENT OF THE INTERIOR

### GALE A. NORTON, Secretary

### U.S. GEOLOGICAL SURVEY

### CHARLES G. GROAT, Director

### The use of firm, trade, and brand names in this report is for identification purposes only and does not constitute endorsement by the U.S. Geological Survey.

For additional information write to:

District Chief

U.S. Geological Survey

Water Resources Division

Placer Hall

6000 J Street

Sacramento, CA  95819-6129

Copies of this report can be purchased from:

U.S. Geological Survey

Branch of Information Services

Box 25286

Denver, CO  80225-0286

### CONTENTS

Abstract

Introduction

Mining History

Previous Work on Mercury Contamination

Purpose and Scope

Acknowledgments

Study Design

Sampling Locations

Constituents Analyzed

Methods

Quality Assurance and Quality Control

Results

Water Quality

Sediment

Biota

Discussion

Greenhorn Creek—Headwaters

Sailor Flat Area

Buckeye Area

Boston Mine Area

South Fork Greenhorn Creek

Poore Mine Area

Starr Mine Area

Missouri Canyon Area

Greenhorn Creek—Mainstem

Greenhorn Creek at You Beat Road—Integrator Site

Summary and Conclusions

References

Appendix 1.  Locations and Characteristics of Sampling Stations

- Appendix 2.  Field Methods of Sample Collection and Preservation

- Appendix 3.  Laboratory Methods of Chemical Analysis

Appendix 4.  Quality Assurance and Quality Control

### LIST OF FIGURES

- Map of Bear River watershed, California, showing location of Greenhorn Creek drainage

- Map of Greenhorn Creek drainage showing sampling points

- Map of northern part of Greenhorn Creek drainage showing sampling points

- Map of central part of Greenhorn Creek drainage showing sampling points

- Map of southern part of Greenhorn Creek drainage showing “integrator” sampling point

- Probability box plots showing distribution of total mercury concentration and methylmercury concentration in unfiltered and filtered water samples

- Probability box plots showing distribution of total mercury concentration in unfiltered water samples distinguished by type of water body

- Probability box plots showing distribution of total mercury concentration in filtered water samples distinguished by type of water body

- Probability box plots showing distribution of methylmercury concentration in unfiltered water samples distinguished by type of water body

- Probability box plots showing distribution of the ratio of methylmercury to total mercury in unfiltered water samples distinguished by type of water body

- Plot showing relation between total mercury concentration in unfiltered and filtered water samples

- Plot showing relation between methylmercury concentration in unfiltered and filtered water samples

- Plot showing relation between total mercury and methylmercury concentration in unfiltered water samples

- Plot showing relation between total mercury and methylmercury concentrations in filtered water samples

- Plot showing relation between pH in unfiltered water and sulfate concentration in filtered water

- Plot showing relation between pH in unfiltered water and total mercury concentration in unfiltered water

- Plot showing relation between pH in unfiltered water and total mercury concentration in filtered water

- Plot showing relation between pH in unfiltered water and methylmercury concentration in unfiltered water

- Plot showing relation between sulfate concentration in filtered water and total mercury concentration in unfiltered water

- Plot showing relation between sulfate concentration in filtered water and total mercury concentration in filtered water

- Plot showing relation between sulfate concentration in filtered water and methylmercury concentration in unfiltered water

- Plot showing relation between total mercury concentration in unfiltered water and total suspended sediment concentration

- Plot showing relation between total mercury concentration in unfiltered water and suspended fines (silt and clay) concentration

- Plot showing relation between particulate total mercury concentration in unfiltered water and total suspended sediment concentration

- Plot showing relation between particulate total mercury concentration in unfiltered water and suspended fines (silt and clay) concentration

- Probability box plots showing distribution of total mercury and methylmercury concentration in sediment samples

- Plot showing relation between total mercury and methylmercury concentration in sediment samples

- Probability box plots showing distribution of methylmercury concentration in biological samples (invertebrates and amphibians)

- (a) Scatter plots showing relation between total mercury and methylmercury concentration in invertebrate samples, and (b) Box plots showing ratio of methylmercury to total mercury for individual invertebrate taxa

- Plot showing relation between methylmercury concentration and mass of individual banana slugs

- Plot showing relation between methylmercury concentration and mass of individual dragonflies

- Plot showing relation between methylmercury concentration and mass of individual hellgrammites

- Plot showing relation between methylmercury concentration and mass of individual predaceous diving beetles

- Plot showing relation between methylmercury concentration and average mass of predaceous stoneflies

- Plot showing relation between methylmercury concentration and average mass of water striders

- Plot showing relation between total mercury concentration and sample mass of all frog species

- Plot showing relation between total mercury concentration and mass of individual bullfrogs

- Plot showing relation between total mercury concentration and mass of individual foothill yellow-legged frogs

- Plot showing relation between total mercury concentration and average mass of pacific treefrogs

Appendix 1.  Locations and Characteristics of Sampling Stations

A1-1. 	Map showing locations of mine tunnels, hydraulic open-pit mines, and lands managed by federal land management agencies.

A1-2.	Photograph of site BY105, Sailor Flat main drainage gulch

A1-3.	Photograph of site BY106, Sailor Flat main drainage to Greenhorn Creek

A1-4.	Photograph of site BY130, Tom and Jerry mine tunnel outlet

A1-5.	Photograph of site BY58, Buckeye main drain entering Greenhorn Creek

A1-6.	Photograph of site BY20, Boston mine tunnel outlet, mercury on bedrock

A1-7. 	Historical photograph of site BY20, Boston mine

A1-8. 	Photograph of site BY20, Boston mine tunnel outlet discharge

A1-9. 	Photograph of site BY88, Poore mine, ground sluice, with iron precipitate

A1-10.	Photograph of site BY89, Poore Mine, pit lake

A1-11.	Photograph of site BY91, Poore mine tunnel outlet

A1-12.	Photograph of site BY122, Starr mine tunnel inlet

A1-13. 	Photograph of site BY123, Starr mine tunnel midway

A1-14.	Photograph of site BY124, Starr mine tunnel outlet

A1-15.	Photograph of site BY148, Starr pit pond

A1-16.	Photograph of site BY146, Missouri Canyon ground sluice

A1-17.	Photograph of site BY75, Missouri Canyon ground sluice above creek

A1-18.	Photograph of site BY59, Greenhorn Creek below You Bet Road

A1-19.	Photograph of site BY59, Confluence of Little Greenhorn Creek with Greenhorn Creek upstream of You Bet Road

Appendix 4.  Quality Assurance and Quality Control

A4-1. 	Correlation plot of values versus reported values for erbium, europium, holmium, lutetium, terbium, thulium, and ytterbium determined in standard reference materials

A4-2.	Correlation plot of observed values versus reported values for cerium, dysprosium, gadolinium, lanthanum, neodymium, praseodymium, samarium, tellurium, and yttrium determined in standard reference materials

A4-3.	Correlation plot of observed values versus reported values for bismuth, rubidium, selenium, and uranium determined in standard reference materials

A4-4.	Correlation plot of observed values versus reported values for arsenic, beryllium, cadmium, cobalt, lead, thallium, and vanadium determined in standard reference materials

A4-5.	Correlation plot of observed values versus reported values for aluminum, antimony, chromium, copper, lithium, nickel, and zinc determined in standard reference materials

A4-6.	Correlation plot of observed values versus reported values for barium, boron, manganese, molybdenum, and strontium determined in standard reference materials

A4-7.	Correlation plot of observed values versus reported values for mercury determined in standard reference materials [cannot locate this figure]

A4-8.	Bar graph plotting recovering (in percent) for arsenic, beryllium, copper, and zinc spiked in blank samples for selected analysis dates

A4-9.	Bar graph plotting recovering (in percent) for mercury spiked in blanks for selected analysis dates

A4-10.	Bar graph plotting recovering (in percent) for beryllium, cadmium, copper, and zinc spiked in blank samples for selected analysis dates

A4-11.	Bar graph plotting recovering (in percent) for mercury spiked in samples for selected analysis dates

A4-12.	Correlation plots of six typical elements A) arsenic, B) barium, C) beryllium, D) calcium, E) cobalt, and f) copper, determined on field duplicate samples

A4-13.	Correlation plots of six typical elements A) mercury, B) holmium, C) manganese, D) niobium, E) lead, and f) uranium determined on field duplicate samples

A4-14.	Plots of percent relative standard deviation versus concentration for six selected elements A) cadmium, B) copper, C) gadolinium, D) mercury, E) magnesium, and F) lead

A4-15.	Plot of total mercury concentrations in unfiltered water showing relation of replicate 1 of 2 and replicate 2 of 2

A4-16.	Plot of total mercury concentrations in filtered water showing relation of replicate 1 of 2 and replicate 2 of 2

A4-17.	Plot showing relation of specific conductance with sum of anion concentrations (expressed as microequivalents per liter)

LIST OF TABLES

- Data for water-quality parameters measured in the field

- Concentrations of total mercury in water samples

- Concentrations of methylmercury in water samples

- Concentrations of trace metals and major cations in water samples

  - Unfiltered water

  - Filtered water

- Concentrations of major anions and alkalinity in filtered water samples

- Concentrations of nutrients and organic carbon in water samples

- Data for suspended sediments in water samples

- Data for stable isotopes of sulfur and oxygen in aqueous sulfate and of oxygen and hydrogen in water

- Concentrations of mercury and methylmercury in sediment samples determined by laboratory methods

- Concentrations of total mercury in sediment determined by field observation, including panning methods

- Mercury and methylmercury concentration data for invertebrates

- Mercury concentration data for frog samples

- Summary of data on mercury in water, sediment, and biota

Appendix 1.  Locations and Characteristics of Sampling Stations

A1-1. Location, descriptions, and characteristic of sampling stations

A1-2. Characteristics of tunnels at hydraulic mine sites in the Greenhorn Creek drainages.

A1-3. Descriptions of photographs of sampling sites

Appendix 4.  Quality Assurance and Quality Control

A4-1. Trace elements analyzed by ICP-MS, with corresponding median detection limits

A4-2.  Listing of selected trace elements with regression correlation coefficients (R2) for correlation plots of observed versus reported values of analysis of standard reference materials.

A4-3. Data for total mercury in blanks

A4-4. Data for methylmercury in blanks

A4-5. Data for trace elements in unfiltered blanks

A4-6. Data for trace elements in filtered blanks

Study Design

- Reconnaissance sampling of water, sediment, and biota by the U.S. Geological Survey (USGS) began in 1999 in the Greenhorn Creek drainage. During the first field season, sampling was done at several different mine sites and adjacent areas affected by historical mining activity.  Based on preliminary data from the initial sampling, follow-up sampling took place during 2000 and 2001 at a number of the locations to confirm anomalously high concentrations of water, sediment, and biota; also, some mine sites in the study area were visited for the first time during 2000 and 2001.  Results of additional follow-up sampling that took place during 2002 will be published separately from this report.

- Sample Locations

- The locations of sampling sites in the Greenhorn Creek drainage are displayed in Figure 2.  Descriptions of each sampling site are given in Appendix 1 (Table A1-1).  More detailed maps of the Greenhorn Creek drainage (Figs. 3–5) include “Map ID” numbers (such as “BY20”) which can be used to track information in data tables. Symbol colors in Figures 2–5 are keyed to specific areas within the Greenhorn Creek drainage, on the basis of sub-watersheds, stream segments, and mining districts.  Symbol shapes in Figures 2–5 indicate the type of water body or mining feature that was sampled, including ground sluice, tunnel sluice, pit lake or wetland, and stream.  The same definitions of symbol color and shape are used in many of the plots described in the Results section.

- [PLEASE NOTE: The map in Fig. 2 will be interactive in the final version of this report (.pdf format). Pointing the cursor on a part of the map will link to more detailed maps (Figs. 3, 4, and 5).  On Figs. 3, 4, and 5, pointing and clicking on a sampling site will bring up a draw-down menu with links to tables with data for that site.]

- Samples were collected from a total of 40 sampling locations for this study during 1999–2001.  A total of 49 water-quality samples were taken at 27 of the sampling stations.  Ten sediment samples for laboratory analysis were taken at eight different locations.  In addition, visible elemental mercury was assessed at 14 sites using a field panning method.   A total of 194 invertebrate samples were taken at 31 different sites, and 69 frog samples were taken at 19 different sites in the study area.

- Constituents Analyzed

- Constituents analyzed in water samples included mercury in four forms:  (a) total mercury in unfiltered water, (b) total mercury in filtered water, (c) monomethylmercury in unfiltered water, and (d) monomethylmercury in filtered water. [For the remainder of this report the term “methylmercury” (abbreviated as “methyl Hg” and “MeHg”) is used instead of “monomethylmercury,” with recognition that other methylated forms of mercury such as dimethylmercury may be present but are not included in reported “methylmercury” analyses.]  Water filtration was done with a Gelman capsule filter (0.45 micrometers nominal pore diameter).  (Trade names are used for identification purposes only and do not constitute endorsement by the U.S. Geological Survey.)  Mercury forms (a) through (c) were analyzed in nearly all of the 49 water-quality samples, however form (d), filtered methylmercury, was analyzed in only 12 of the samples.  Unstable water-quality parameters measured in the field included pH, temperature, specific conductance, and dissolved oxygen.  Alkalinity (a proxy for dissolved bicarbonate) was analyzed by titration on a filtered subsample, usually within 48 hours of collection (stored chilled until analysis).  Major cations and trace elements were analyzed both in filtered and unfiltered samples. Major anions (sulfate and chloride) were analyzed in filtered samples only. Nutrients (five forms of nitrogen and three forms of phosphorous) were analyzed in a combination of filtered and unfiltered samples.  Organic carbon was analyzed in filtered and particulate forms. Stable isotopes of hydrogen and oxygen were analyzed in unfiltered water and stable isotopes of sulfur and oxygen were analyzed in dissolved sulfate. Concentration of total suspended sediment was determined in the water samples along with the percentage of sand in the suspended sediment, which was used to compute the concentration of suspended silt and clay (< 0.063 millimeter).

- Sediment samples submitted to the laboratory were analyzed for concentrations of total mercury and methylmercury.  In nearly all cases, the concentrations are reported on a dry basis. However, the total mercury concentration in one sediment sample is reported on a wet basis because the moisture content was not determined.  The sediment samples analyzed in the field for visible mercury by panning methods were analyzed on a wet basis.

- Invertebrate taxa collected included Hemiptera (Gerridae, Belostomatidae), Coleoptera (Dytiscidae), Plecoptera (Perlidae), Megaloptera (Corydalidae), Odonata (Aeshnidae, Cordulegastridae, Gomphidae, Libellulidae) and Gastropoda (Arionidae).  Insects were identified using taxonomic keys of Merritt and Cummins (1996) and McCaffrey (1981).  All invertebrates were analyzed for MeHg; some of the invertebrate samples were analyzed for THg as well, so that the ratio MeHg/THg could be assessed. Previous studies have shown that values of MeHg/THg in invertebrates can vary from 20–90%, depending on functional feeding strategies, with predatory invertebrates generally having higher values (Slotton and others, 1997;  Tremblay and Lucotte, 1997; Hall and others 1998; Mason and others 2000; Schwarzbach and others 2002).

- Three species of postmeamorphic frogs were collected in this study:  bullfrogs (Rana catesbeiana), foothill yellow-legged frogs (Rana boylii), and Pacific treefrogs (Hyla regilla).  Frogs were identified using descriptions by Stebbins (1985).  Frog samples were analyzed for total mercury after cleaning and removal of gut contents. It has been shown in other studies that the mercury in frog tissue is largely in the form of MeHg (R. Hothem, unpublished data).

- Methods

- Methods of field sampling and sample preservation are described in Appendix 2.  Laboratory methods of chemical analysis are documented in Appendix 3.

- Quality Assurance and Quality Control

- Quality assurance and quality control information is provided in Appendix 4.

References 

Alpers, C.N., and Hunerlach, M.P., 2000, Mercury contamination from historic gold mining in California. U.S. Geological Survey Fact Sheet 061-00, 6 p.
http://ca.water.usgs.gov/mercury/fs06100.html

Alpers, C.N., Taylor, H.E., and Domagalski, J.L., eds., 2000, Metals transport in the Sacramento River, California, 1996-1997: Volume 1. Methods and Data. U.S. Geological Survey Water-Resources Investigations Report 99-4286.
http://water.usgs.gov/pubs/wri/wrir_994286

Cain, D. J., Luoma, S.N., Carter, J.L., and S. V. Fend, 1992 Aquatic insects as bioindicators of trace element contamination in cobble-bottom rivers and streams. Canadian Journal of Fisheries and Aquatic Sciences v. 49, p. 2141-2154.

Cain, D. J., Luoma, S.N., and Axtmann, E.V., 1995, Influence of gut content in immature aquatic insects on assessments of environmental metal  contamination. Canadian Journal of Fisheries and Aquatic Sciences, v. 52, p. 2736-2746.

Clark, W.B., 1970, Gold districts of California: California Division of Mines and Geology Bulletin193, 186 p.

De Wild, J.F., Olson, M.L., and Olund, S.D., 2002, Determination of methylmercury by aqueous phase ethylation, followed by gas chromatographic separation with cold vapor atomic fluorescence detection. U.S. Geological Survey Open-File Report 01-445.
http://wi.water.usgs.gov/pubs/ofr-01-445/ofr-01-445.pdf

Hall, B. D., Rosenberg, D. M. and Wiens, A.P., 1998, Methylmercury  in aquatic insects from an experimental reservervoir. Can. J. Fish. Aquat. Sci. 55: 2036-2047.

Hare, L., 1992, Aquatic insects and trace metals: bioavailability, bioaccumulation, and toxicity. Critical Reviews in Toxicology, v.  22, No. 5/6, p. 327-369.

Hobson, E.M., and Wiltsee, E.M., 1893, Eleventh Report of the State Mineralogist: California State Mining Bureau, 612 p.

Hunerlach, M.P, and Alpers, C.N., 2003, Mercury contamination from hydraulic gold mining in the Sierra Nevada, California, in Gray, J.E., ed., Geologic Studies of Mercury by the U.S. Geological Survey, U.S. Geological Survey Circular 1248, 41 p.

Hunerlach, M.P. Rytuba, J.J., and Alpers, C.N., 1999, Mercury contamination from hydraulic placer-gold mining in the Dutch Flat mining district, California, in Morganwalp, D.W., and Buxton, H.T., eds., U.S. Geological Survey Toxic Substances Hydrology Program -- Proceedings of the Technical Meeting, Charleston, South Carolina, March 8-12, 1999: U.S. Geological Survey Water-Resources Investigations Report 99-4018B, p. 179-189.
http://ca.water.usgs.gov/mercury/dutch/wrir994018b.pdf

Jarmin, A., 1927, An investigation of “The feasibility of any plan or plans whereby hydraulic mining operations can be resumed in the state” in Report 23 of the State Mineralogist: California State Mining Bureau Vol. 23, No. 1, 130 p.

Kelley, R.L., 1959, Gold vs. Grain: The Hydraulic Mining Controversy in California’s Sacramento Valley: The Arthur Clark Company, Glendale, California, 309 p.

Klasing, S., and Brodberg, R., 2003, Evaluation of potential health effects of eating fish from selected water bodies in the northern Sierra Nevada Foothills (Nevada, Placer, and Yuba Counties): Guidelines for Sport Fish Consumption, December 2003, 46 p.

http://www.oehha.ca.gov/fish/so_cal/nosierra.html

Lindgren, W., 1911, The Tertiary gravels of the Sierra Nevada of California: U.S. Geological Survey Professional Paper 73, 226 p.

May, J.T., Hothem, R.L., Alpers, C.N., and Law, M.A., 2000, Mercury bioaccumulation in fish in a regional affected by historic gold mining: The South Yuba River, Deer Creek, and Bear River watersheds, California, 1999. U.S. Geological Survey Open-File Report 00-367, 30 p.

http://ca.water.usgs.gov/archive/reports/ofr00367/index.html

May, P.R. 1970, Origins of hydraulic mining: The Holmes Book Company, Oakland, 88 p.

Marvin-Dipasquale, M., Agee, J.,  McGowan, C., Oremland, R.S., Thomas, M.,  Krabbenhoft, D., and Gilmour, C., 2000, Methyl-mercury degradation pathways: A comparison among three mercury-impacted ecosystems. Environmental Science and Technology, v. 34, p. 4908-4916.

Marshack, J.B., 2003, A compilation of water-quality goals. California Environmental Protection Agency, Regional Water Quality Control Board – Central Valley Region. August 2003, 186 p.

Mason, R. P., Laporte, J.M., Andres, S., 2000, Factors controlling the bioaccumulation of mercury, methylmercury, arsenic, selenium, and cadmium by freshwater invertebrates and fish. Archives of Environmental Contamination and Toxicology, v. 38, p. 283-297.

Merritt, R. W. and Cummins, K.W., 1996, An introduction to the aquatic insects of North America. 3rd ed. Kandall/Hunt Publishing Co. Dubuque, Iowa.

McCafferty, W.P., 1981, Aquatic entomology : the fishermen's and ecologists' illustrated guide to insects and their relatives. Science Books International, Boston, Mass. 448 p.

Olson, M.L., and De Wild, J.F., 1999, Techniques for the collection and species-specific analysis of low levels of mercury in water, sediment, and biota, in Morganwalp, D.W., and Buxton, H.T., editors, U.S. Geological Survey Toxic Substances Hydrology Program -- Proceedings of the Technical Meeting, Charleston, South Carolina, March 8-12, 1999: U.S. Geological Survey Water-Resources Investigations Report 99-4018B.
http://wi.water.usgs.gov/pubs/WRIR-99-4018-B/index.html

Rudd, J.W.M., 1995, Sources of methylmercury to freshwater ecosystems, a review.  Water Air Soil Pollution, 80, 697-713.

Slotton, D.G., Ayers, S.M., Reuter, J.E., and Goldman, C.R., 1997, Gold mining impacts on food chain mercury in northwestern Sierra Nevada streams. Appendix B in Larry Walker Associate, 1997, Sacramento River Watershed mercury control planning project – report for the Sacramento Regional County Sanitation District, 74 p.

Stebbins, R.C. 1985. A field guide to western reptiles and amphibians: field marks of all

species in western North America, including Baja California. 2nd ed. rev., Boston: Houghton Mifflin, 533 p.

Schwarzbach, S., Thompson, L. and Adelsbach, T., 2001, Cache Creek mercury investigations USFWS Final Report. Off Refuge Investigations Report. FFS #11301F22, 55 p.

Tremblay, A., Lucotte, M., and Rheault, I., 1996, Methylmercury in a benthic food web of two hydroelectric reservoirs and a natural lake of Northern Quebec (Canada). Water, Air, and Soil Pollution, v. 91, p. 255-269.

Tremblay, A. and Lucotte, M., 1997, Accumulation of total mercury and methylmercury in insect larvae of hydroelectric reservoirs: Canadian  Journal of Fisheries and Aquatic. Sciences. 54:832-841.

U.S. Environmental Protection Agency, 1990, Guidelines establishing test procedures for the analysis of pollutants (Appendix B, Part 136, Definition of procedures for the determination of the method detection limit—Revision 1.11): Revised as of July 1, 1999, p. 537–539.

U.S. Fish and Wildlife Service, 2003, Evaluation of the Clean Water Act section 304(a) human health criterion: protectiveness for threaten and endangered wildlife in California. U. S. Fish and Wildlife Service, Sacramento Fish and Wildlife Office, Environmental Contaminants Division. Sacramento, California. 96 p. + appendix.

USDA Forest Service, 2002, Preliminary Assessment of the Buckeye Diggings and Sailor Flat Hydraulic Mines, Prepared by CDM Federal, Walnut Creek, CA for the Tahoe National Forest, Nevada City, CA, April 2002.

Wolfe, M.F., Schwarzbach, S., and Sulaiman, R.A., 1998, Effects of mercury on wildlife–A comprehensive review: Environmental Toxicology and Chemistry, v. 1, no. 2., p. 146-160.

Yeend, W.E., 1974, Gold-bearing gravels of the ancestral Yuba River, Sierra Nevada, California: U.S. Geological Survey Professional Paper 772, 44 p.

Appendix 1. Locations and Characteristics of Sampling Stations, Greenhorn Creek Drainage

Descriptions of the 40 sampling stations for this report can be found in Table A1-1. The table lists the spatial coordinates (latitude, longitude, and elevation) as well as a narrative description of each site location. In addition, information is provided on site ownership, alternate names for the mines, and the mining district.

During this investigation a number of tunnels were located, some of which are not mapped on USGS 7.5-minute topographic quadrangles.  Most (if not all) of these tunnels were constructed during the period of hydraulic mining activity for the purpose of draining water and tailings from the hydraulic mine pits. In many cases, gold recovery was practiced within the tunnels, in which case the tunnels are referred to as “tunnel sluices.”  Table A1-2 lists the locations and characteristics of the tunnels that were encountered in the Greenhorn Creek drainage during this study.

- The map in Figure A1-1 shows the locations of the mine tunnels described in Table A1-2.  Also shown on the map are the locations of hydraulic mine pits, as mapped by Yeend (1974), and the distribution of lands managed by the U.S.D.A. Forest Service and the Bureau of Land Management (from CAGIS database, Bureau of Reclamation)

- Figures A1-2 through A1-19 are photographs of some of the sampling stations in the Greenhorn Creek drainage. All of the photographs were taken during 1999–2003 with the exception of Figure A1-7 which is a historical photograph of the Boston mine operation (circa 1880?).

- [Please note: The final version of this report will be in .pdf format. An additional map will be included that will have clickable links to the photographs in figures A1-2 through A1-20.]

- Appendix 2. Field Methods of Sample Collection and Preservation

Special cleaning and sampling techniques were used during all stages of sample collection to prevent sample contamination. All containers and equipment used for water, sediment, were extensively cleaned in 5 percent hydrochloric acid and rinsed in ultrapure, deionized water (18 megaohm-cm) prior to use.  Specific sampling and preservation methods for each type of sampling are described below.

Water 
	Water samples were collected in Teflon or fluorinated plastic containers and then transferred to a Teflon-lined churn for splitting. (Use of commercial product names is for identification purposes only and does not constitute endorsement by the U.S. Geological Survey.) Filtration was done in the field using Gelman capsule filters of 0.45 micrometer pore size. Filtered and unfiltered water samples for analysis of total Hg were stored in acid-washed glass bottles and preserved using a potassium dichromate–nitric acid solution. Filtered and unfiltered water samples for analysis of monomethylmercury (hereinafter referred to as MeHg) were stored in Teflon bottles and preserved either by freezing on dry ice or by a hydrochloric acid solution.

Sediment

Sediment samples were colleted with polycarbonate, Teflon, or stainless steel instruments, and transferred to Teflon or acid-washed glass containers using an acid-cleaned stainless steel spatula. Sediment samples for analysis of total Hg, major elements, and trace metals were immediately stored on wet ice and chilled until analysis. Sediment samples for analysis of MeHg were immediately stored on dry ice and frozen until analysis.

Biota

Invertebrates and amphibians were collected using dip nets and (or) by hand. Captured invertebrates and frogs were temporarily stored with native water in Zip-Lock bags. Invertebrates were typically analyzed as composite of multiple individuals to obtain a minimum of one gram of biomass. In some cases, however, samples weighing less than one gram were analyzed. Prior to initial processing, samples were held on dry ice or wet ice. Initial processing of invertebrate samples proceeded within 24 hours of collection. Samples were thoroughly rinsed with deionized water and patted dry using a disposable paper towel. Invertebrates were passively depurated 4-24 hours. Composites of similar size/age individuals (visually estimated) were placed in chemically cleaned, certified jars.

Frogs were held on dry ice or wet ice. The samples were stored in freezers for 1 to 3 months before dissecting out the gastrointestinal (G.I.) tract. After excising the G.I. tract, frog carcasses were placed in certified, chemically cleaned jars and held in freezers until sent to the analytical laboratory. Usually individual frog samples were sent for analysis, but, in a few cases, composites of frogs were sent.

Invertebrate samples were analyzed primarily for MeHg; in some cases, however, samples were analyzed for both Hg and MeHg. Frogs were analyzed only for Hg.

- Appendix 3. Laboratory Methods of Chemical Analysis 
Water and Sediment

- Analysis of total mercury in water and sediment was done by the USGS research laboratory in Boulder, Colorado, under the direction of Dr. Howard E. Taylor, using cold-vapor atomic fluorescence spectroscopy (CVAFS). Details regarding the method and related quality-assurance procedures are described by Alpers and others (2000). Results of quality-assurance and quality-control are described in Appendix 1 of Alpers and others (2000).

Analysis of methylmercury in water and sediment and some analyses of total merecury were done at the USGS Wisconsin District Mercury Laboratory (WDML) in Middleton, Wisconsin, under the direction of Dr. David P. Krabbenhoft.  Methymercury in water was analyzed using ethylation-distillation CVAFS methods described by Olson and DeWild (1999) and by DeWild and others (2002).  Analysis of methylmercury in sediment was done using ethylation-extraction CVAFS methods.

Analysis of major elements and trace metals in water and sediment were done by inductively coupled plasma (ICP) analysis using atomic emission spectroscopy (ICP-AES) and mass spectrometry (ICP-MS).  Major anions in filtered water (chloride and sulfate) were analyzed by ion chromatography at the Boulder NRP laboratory.  Dissolved and particulate organic carbon and nutrients were analyzed by the USGS National Water Quality Laboratory (NWQL) in Denver, Colorado.  The specifics of the nutrient analyses are described by Alpers and others (2000).

Biota

Analysis of Hg and MeHg in biological samples was done at the Trace Element Research Laboratory (TERL) at Texas A&M University in College Station, Texas, under the direction of Dr. Robert Taylor. Before samples are analyzed for Hg by cold-vapor atomic absorption spectroscopy (CVAAS), the Hg is converted to the Hg 2+ form. Mercury was digested by a modified version of EPA methods 245.5 and 245.6. Tissue samples were analyzed either freeze dried or on a wet basis. Tissue samples were homogenized in the original sample containers, either after freeze drying or by using a Tekmar Tissumizer, and then subsampled. Samples were digested with nitric acid, sulfuric acid, potassium permanganate, and potassium persulfate in polypropylene tubes in a water bath at 90–95 ºC. Before analysis, hydroxylamine hydrochloride was added to reduce excess permanganate, and the samples were brought to volume using distilled-deionized water.

In the CVAFS procedure for determination of Hg, divalent mercury (Hg2+) in aqueous samples (digests of water, tissue or sediment samples) is reduced to the elemental state (Hg0) by a strong reducing agent (stannous chloride). Gaseous Hg0 enters the sweep gas and is introduced into an atomic absorption cell, where light produced by a Hg vapor lamp is absorbed by the free Hg atoms. Mercury in the sample is determined by comparing light absorption of the sample with that of external calibration standards.

The procedure for determination of MeHg measures the sum of all organo-mercury species extracted into the solvent. Samples are analyzed either wet or after freeze-drying. Homogenized aliquots are extracted into an organic solvent with potassium bromide and copper sulfate added to improve partitioning between phases. The organic phase is digested in combusted glass vials, using nitric and sulfuric acids and potassium permanganate, to convert all mercury species to ionic mercury and to remove traces of organic solvent that would otherwise affect the measurement. Analysis is based upon the cold vapor atomic absorption spectroscopy (CVAAS) method, although CVAFS can be used when lower detection limits are required.

Moisture content was determined by weight loss upon freeze drying, and is expressed as weight percentage of the original wet sample. Depending upon sample size, either the whole sample or a representative aliquot was frozen and then dried under vacuum until a constant weight is attained. Samples were prepared and dried using plastic materials to minimize potential contamination artifacts.

- Appendix 4.  Quality Assurance and Quality Control 

Water and Sediment

- A variety of measurements and analyses were used to determine the quality of the data associated with this study. Quality assurance and quality control (QA-QC) measures include field and laboratory blank samples, standard reference materials (where available), spike recoveries, and replicate samples. Alpers and others (2000) described the application of these QA-QC measures in a project assessing metals transport in the Sacramento River Basin, in which similar methods of sample collection, preparation, and analysis were used.

Trace metals and major cations were analyzed by inductively coupled plasma (ICP) methods with each analysis consisting of a minimum of four replicate instrumental measurements. Each filtered and unfiltered sample was collected in duplicate and analyzed in triplicate. Statistical analysis of the triplicate analyses was performed and the analyses were accepted if the standard deviation was within standard tolerances of precision (generally less than 15 percent of the amount present). The error tolerance is increased for analyses close to the detection limit for a given analyte.  Median detection limits for trace elements analyzed by ICP-MS are given in table A4-1.

With each batch of samples run by ICP for trace element and major cations, several standard reference materials are run as unknowns as part of the quality assurance/quality control program.  Plots of reported (published) values in relation to observed values for certified standard reference materials and USGS standard reference water samples (SRWS) are shown in figures A4-1 through A1-6.  The number of times each standard was analyzed during the period 1999–2001 is indicated on each figure; in most cases the standards were run several hundred times during the study period.  In all cases there is excellent agreement between reported and observed values for the standards.  The regression coefficients from correlation plots of observed versus reported values for the analysis of standard reference materials for 21 selected trace elements range from 0.9881 for manganese to 1.000 for beryllium and antimony (Table A4-2).

Data for spikes on blanks samples are shown in figure A4-8 for the elements arsenic, beryllium, copper, and zinc for analyses performed during the period July 2000 through August 2001. In nearly all cases, the spike recovery was between 80 % and 120% of the expected amount.  Data for spikes of mercury (5 ng/L) into blanks are given in figure A4-9.  Nearly all results fall in the range of 80 to 120% recovery.  Spikes for trace elements onto unknown samples are shown in Figure A4-10 for four trace elements and in Figure A4-11 for mercury.  Again, in nearly all cases, the spike recoveries are in an acceptable range of 80-120 percent of the expected values.

Because replicate split samples are routinely analyzed as separate unknowns, and each replicate is analyzed in triplicate, plots of replicate 1 versus replicate 2 give a good idea of analytical variability as well as possible contamination issues regarding bottles and sample splitting equipment.  The plots in figures A4-12 through A4-14 demonstrate that there are very few outliers for replicate analyses of the 18 elements shown for all analyses done by the Taylor laboratory during the study period.  Similar plots of replicate analyses for the specific samples considered in this report are shown in figure A4-15 for unfiltered mercury and figure A4-16 for mercury in filtered water.  A single outlier is evident on each of these plots.  As discussed in the text, the suspect values identified by these plots were not used in calculating average values for each site or the site median values for each analyte.

Quantitative analysis was done to determine total Hg in both filtered and unfiltered samples. Precision and accuracy criteria were evaluated by analyzing field and laboratory blank samples, standard reference materials (at least 30 % frequency of total samples analyzed), spike addition recovery analysis, and replicate sample analysis. All sample analyses were done at least in triplicate.  In most cases, each sample was sent to the laboratory in two replicate bottles designated as replicates “1 of 2” and “2 of 2”.  The method detection limit (MDL) for the total mercury analyses is 0.4 nanograms per liter (ng/L).

Data for total mercury in field blanks are given in Table A4-3.  Source water used for blanks taken in late November 1999 had total mercury concentrations of 2.1 to 3.3 ng/L, which are anomalously high. This makes it somewhat difficult to evaluate the results of 3.8 and <0.6 ng/L total mercury from replicate equipment blanks taken December 1, 1999 from a Teflon-lined churn.  Nine subsequent equipment and process blanks taken during January and February, 2000 had a median total mercury concentration of 0.5 ng/L, just above the method detection limit, and a range from < 0.4 ng/L to 1.6 ng/L.  For comparison, the medians of site averaged values for unfiltered and filtered Hg-T in this study were 14.6 ng/L and 2.8 ng/L, respectively.  Several values for filtered Hg-T in field samples are reported as less than the method detection limit of 0.4 ng/L (Table 2). Given that some of the total mercury blank values are in the same concentration range as some of the filtered samples, these lower-levels values must be interpreted with caution. It is possible that the reported values of total mercury in the range of < 0.4 ng/L to 3.0 ng/L (especially in filtered samples) were affected by contamination of equipment or the deionized water used to clean the equipment. However, the extensive rinsing that is done with native water, both of the equipment and the bottles prior to sample collection, is likely to make the influence of any such contamination extremely minor.

Methods of QA-QC during analysis of methylmercury in water were described by De Wild and others (2002). Analyses of water samples were done in batches of 11 samples plus three laboratory blanks, a matrix spike, and a matrix spike duplicate. The reported values for MeHg concentration are blank-corrected as described by De Wild and others (2002). The method detection limit for MeHg in water, defined using standard protocol (U.S. Environmental Protection Agency, 1990), is 0.04 ng/L. Daily detection limits were computed for each batch of samples. In cases where the daily detection limit is greater than the method detection limit, and the measured sample value is greater than the method detection limit but less than the daily detection limit, the measured value is considered an estimate and is flagged with data quality code “E” in the data tables.

Concentrations of methylmercury in blanks analyzed during this study are given in Table A4-4.  In all six equipment and process blanks taken during the study period, mthyl mercury wss below the method detection limit of 0.04 ng/L.  The single filtered blank run for MeHg have a value of 0.04 ng/L, right at the detection limit.   This result raises the possibility that the filtering process may cause an extremely low level of methylmercury contamination of the filtrate.  As with total mercury, because of the extensive rinsing of equipment and sampling bottles that is done as part of the routine sampling protocol, the effect of such possible contamination is thought to be minor.  Because of the reconnaissance nature of the present study, the levels of total mercury and methylmercury detected in the various blanks do not have any bearing on the outcome of the present study.

Figure A4-17 displays the relation between specific conductance and the sum of equivalent change based on analyses of major cations (Cl – and SO4 2-) plus bicarbonate alkalinity (as HCO3-).  Theoretically, the relation in Figure A4-17 should be a straight line.  Five of the samples in Figure A4-17 appear to be outliers, which indicates that one of the analyses or measurements involved is likely to be incorrect.   Additional analysis such as aqueous speciation and charge balance could help to resolve which specific data are most suspect for these five samples.

Blank values for trace metals and major cations are given in Table A4-3 (unfiltered blanks) and Table A4-4 (filtered blanks). Blank values for most elements are in a concentration range considerably lower than the range of values of the unknown samples (Tables 4A and 4B).

Biota 
	As part of QA-QC procedures, a standard number of replicates, blank, and spikes were analyzed with each instrument run. All QA-QC objectives were achieved during the instrument runs by the Trace Element Research Laboratory (TERL) that are reported here. See the Patuxent Analytical Control Facility (PACF) web site for details on acceptance criteria for quality assurance of samples. (http://www.pwrc.usgs.gov/pacf/)

U.S. Geological Survey, 6000 J Street, Placer Hall, Sacramento, CA 95819-6129

2 U.S. Geological Survey, Dixon Field Station, Dixon, CA

3 U.S. Geological Survey, 3215 Marine Street, Boulder, CA  80303

4 U.S. Geological Survey, Middleton, WI