NAWQA Sacramento Study Unit files description

By Gene Davis

5/23/07

# Associating long parameter names with records

The first files placed in this directory were a copy of the “Sacramento_Study_Unit.xls” from the “Raw data” directory and “pmcodes” [no extension] from http://nwis.waterdata.usgs.gov/nwis/pmcodes/

The extenionless file “pmcodes” was opened in Excel, cleaned-up (by removal of extra rows at the top of the file), and saved as “pmcodes.xls”. This file was then imported into the Access file “pmcodes.mdb”. The Excel file of data, “Sacramento_Study_Unit.xls”, was also imported into Access as table “Sacramento_Study_Unit”.

In Access, using queries, the shorter “Parameter Short Name” values were replaced with the longer “parameter_nm” values, which are newer and more descriptive.  A list of unique parameter groups was created and added to another query.  By querying the Sacramento_Study_Unit data, it appears that data extends through 9/25/2005-the same as for NAWQA pesticide data in the CDPR SWDB.

Conclusion: The SJTBasin files could be imported into (a series of?) Access databases for easier querying/data review, if needed, although the pesticide data presumably is also in the CDPR SWDB.

# Identifying unique parameters and parameter group names

Ten parameter group names are represented by detectable “Values”: biological, information, major organics, minor and trace organics, nutrients, organics, other, physical property, radiochemicals, and sediment were identified using the “Sacramento_Study_Unit” table in Access, as follows. 406 unique parameter names that were identified in this process.

The “Sacramento_Study_Unit” table was queried for relevant columns and by specifying “Remarks” as “--“, making a new table “SVU_detects_multiple” from the query results.  This file was copied and pasted as a “Structure Only” table “SVU_detects_unique”.  The properties of this file were changed such that the column “parameter_nm” was indexed, with no duplicates allowed. Table “SVU_detects_multiple” was appended, using Append Query, into the table “SVU_detects_unique”.  This created a table of unique parameter names (in the long parameter name format).

Two versions of these tables were exported as Excel tables – “parameter_nm_cd_unique.xls”, which includes the short and long parameter names, parameter groups, and parameter codes, and “parameter_nm_unique.xls”, which includes only the unique long parameter names.

The “parameter_group_nm” (parameter group name) column in the table “SVU_detects_unique” was appended into the “unique_parameter_group” table with the “parameter_group_nm” column properties indexed and set to no duplicates allowed.  This resulted in the table of ten unique parameter groups for which there are “detected” (“Remarks” = “--“) values.

Conclusions: Only the 10 unique parameter groups need to be evaluated further for exceedances. Potentially all 406 unique parameters will have to be screened, once criteria are identified for each parameter.  These parameters include water and bed sediment samples.

Data from 1/1/2000 through 9/30/2005 includes:

Parameter_group_nm		Primary Schedule Name		Parameter_nm

Physical property			Field parameters

Sediment				Field parameters

Major inorganics			Major anions and cations

Organics				Pesticides by GCMS

Organics				Pesticides by HPLC

Organics				Pesticides by HPLC/MS-SPE

Organics				Volatile organic compounds

# Identifying detected pesticides in the NAWQA data

The “organics” parameter group included pesticides data.  All of the detected pesticides in the “Sacramento_Study_Unit” table (in Access), except bromoxynil, were already identified for criteria identification from the CDPR Surface Water Database (SWDB).

# Associating calculated hardness values with metals data

The file “SacRivBasin_hardness.xls” was derived from the “SacRivBasin_anions_cations.xls” raw data file.  All extra columns were removed.  All anions and cations were removed except for calcium and magnesium.  The data was sorted by “Result Datetime” ascending, and “Parameter Short Name” ascending. This resulted in paired calcium and magnesium records for each “Result Datetime”.

Hardness was calculated in the cell (“Hardness_calc”) following each magnesium “Value” by using the formula: Hardness (mg/L as Ca CO3) = 2.497Ca + 4.118Mg (APHA, 1992).  In Excel, this step created artificial (misgrouped by “Result Datetime”) hardness in the cell following each calcium “Value”.  The data were resorted by “Parameter Short Name”, thus grouping all of the calcium records together and all of the magnesium records together.  The artificial hardnesses were removed from the cells following each calcium “Value”.

“Hardness_calc” values were copied and pasted (“Paste Special”, “Values) in a new column (“Hardness”), preserving the calculation formulae in the “Hardness_calc” column.

The data was resorted by “Result Datetime” ascending, and “Parameter Short Name” ascending, as before, resulting in paired calcium and magnesium values and their associated hardness values. “Result Datetime” ranged from 1996 to 2001, with most of the data collected in 1996-1998.

Extra columns in the metals data (from the raw data file “SacRivBasin_traceelements_water_2703.xls”) were removed, so that the resulting file (“SacRivBasin_metals.xls”) had the same eight columns as the “SacRivBasin_hardness.xls” file (but without the “Hardness_calc” and “Hardness” columns. “Result Datetime” ranged from 1996 to 2001, with most of the data collected in 1996-1998. The metals data was then copied and pasted into the “SacRivBasin_hardness.xls” file, which was saved as “SacRivBasin_hardness_metals.xls”.  The data was resorted by “Result Datetime” to pair the hardness and metals data by sample date.  Hardness values ranged from 9 to 263.

Conclusion: Since the Sacramento Study Unit metals data is only through 1998, do not include Sacramento Study Unit NAWQA metals in the 2008 303(d) list update.

Sacramento Study Unit “Organics” files

By Gene Davis

6/11/2007

The Sacramento Study Unit DB file was queried, with “Result Datetime” = “>#1/1/2000#” (i.e., all records since 1 January 2000) for “parameter_gp_nm” = “organics”, yielding 10315 records.  “Sample Medium Desc” was “surface*”.

The query was revised to specify “Primary Schedule Name” = “volatile*, yielding   1305 records. The query was again revised to specify “Primary Schedule Name” = “pesticide*”, yielding the balance of “organics” records (= 9010 records).