# SJTBasin_chlorpesticides_PCBs_sediment.xls

The entire dataset (on the “serial” worksheet) was copied to a new (“working”) worksheet.  Fields that are not necessary for data screening were “hidden”, and rows that had “Report Units” of either “grams” (for sample weight) or “percent” (for surrogate samples) were deleted.  The remaining 591 records/rows were sorted by “Parameter Short Name” (ascending), “Place Name” (ascending), and “Value” (descending).

Only 50 records contained “Value” values associated with a “Remark” = “--“.  The remaining records contained “Value” values associated with a “Remark” = “<”.  Thus, the “Value” values for the former were compared to Table 1 (Sediment Quality Guidelines for Freshwater Sediments) provided by the SWRCB, for which there are Probable Effect Concentrations.

Cis-chlordane, cis-nonachlor, cis-permethrin, DCPA, dieldrin, toxaphene, trans (tr)-chlordane, tr-nonachlor, and tr-permethrin were all reported, but Probable Effect Concentration (PEC) values are not included in Table 1.  Waterbodies associated with these reportable levels include Del Puerto Creek, Orestimba Creek, Mokelumne River, and Spanish Grant combined drain.

One sample collected from Orestimba Creek on 16 October 1992 contained pp’-DDD, pp’-DDE, and pp’-DDT at levels above their respective PECs (28.0, 31.3, and 62.9 ug/kg, respectively).  One sample collected from Orestimba Creek on 6 November 1992 contained pp’-DDE at a level above the PEC for “Sum DDE.”

One sample collected from Del Puerto Creek on 16 October 1992 contained pp’-DDE at a level above the PEC for “Sum DDE.”

One sample collected from Spanish Grant combined drain on 15 October 1992 contained pp’-DDE above the PEC.

# SJTBasin_pesticides_HPLC.xls

The entire dataset of surface water samples (on the “serial” worksheet) was copied to a new (“working”) worksheet.  Fields that are not necessary for data screening were “hidden”, and rows that had “Report Units” of either “mL” (for sample volume), “number (for set number), or “percent” (for surrogate samples) were deleted.

242 records contained “Value” values associated with a “Remark” = “--“.  The remaining records contained “Value” values associated with either a “Remark” = “<” or a “Remark” = “E”.  These 242 records were copied to another new worksheet (“screening”) and sorted by “Parameter Short Name (ascending), “Value (descending), and “Place Name” (ascending).  The “Value” values for these (242) records were compared to pesticide screening values to determine which had exceedances, as summarized in the table below.

| Waterbody (from Place Name) | Parameter | Screening Value (ug/L) | Number of Exceedances | Date(s) |
| --- | --- | --- | --- | --- |
| Harding Drain | Carbaryl | 0.02 | 1 | 22 June 1994 |
| Salt Slough | Carbaryl | 0.02 | 1 | 29 June 1993 |
| San Joaquin River | Carbaryl | 0.02 | 1 | 23 June 1994 |
| Orestimba Creek | Carbofuran | 0.4 | 1 | 4 March 1997 |
| Salt Slough | Diuron | 1.3 | 1 | 4 May 1993 |
| Salt Slough | Methomyl | 0.4 | 1 | 25 August 1993 |

# SJTBasin_pesticides_HPLC_MSSPE.xls

The entire dataset of surface water samples (on the “serial” worksheet) was copied to a new (“working”) worksheet.  Fields that are not necessary for data screening were “hidden”, and rows that had “Report Units” of either “mL” (for sample volume), “number (for set number), or “percent” (for surrogate samples) were deleted.

1,564 records contained “Value” values associated with a “Report Units” = “ug/L“.  These 1,564 records were copied to another new worksheet (“screening”) and sorted by “Remark” (ascending).  Only 19 records contained “Remark” values of “--.”  These 19 records were copied to another new worksheet (“screening”) and sorted by “Parameter Short Name (ascending), “Value (descending), and “Place Name” (ascending).  The “Value” values for these 19 records were compared to pesticide screening values (as available).  There are no screening value exceedances.

# SJTBasin_GCMS.mdb

The table “SJTBasin_GCMS” was queried with “Report Units” Like “ug/L” and “Remark” Like “—“ using a Make Table query, to create the table “SJTBasin_GCMS_screening” (containing 8,200 records), which was exported as an Excel file as “SJTBasin_GCMS_screening.xls”.  These 8,200 records were sorted by “Parameter Short Name (ascending), “Value (descending), and “Place Name” (ascending).  The “Value” values for these (8,200) records were compared to pesticide screening values to determine which had exceedances, as summarized in the table below.

| Waterbody (from Place Name) | Parameter | Screening Value (ug/L) | Number of Exceedances | Date(s) |
| --- | --- | --- | --- | --- |
| Orestimba Creek | Alachlor | 0.4 | 1 | 7/2/1992 |
| Del Puerto Creek | Alachlor | 0.4 | 1 | 6/23/1994 |
| Harding Drain | Alpha-HCH | 0.0026 | 23 | 4/22/1992 – 6/21/2001 |
| Orestimba Creek | Alpha-HCH | 0.0026 | 1 | 7/24/1992 |
| San Joaquin River | Alpha-HCH | 0.0026 | 1 | 6/10/1992 |
| Salt Slough | Chlorpyrifos | 0.015 | 14 | 1/29/1993 - 8/21/2001 |
| Farabuindo stormdrain | Chlorpyrifos | 0.015 | 1 | 2/13/1995 |
| Mustang Creek | Chlorpyrifos | 0.015 | 29 | 7/17/2003 – 3/2/2004 |
| Orestimba Creek | Chlorpyrifos | 0.015 | 86 | 4/22/1992 – 9/7/2005 |
| Merced River | Chlorpyrifos | 0.015 | 41 | 1/22/1993 - 5/14/2003 |
| Sonoma stormdrain | Chlorpyrifos | 0.015 | 1 |  |
| Highline Canal spillway | Chlorpyrifos | 0.015 | 5 | 2/8/1994 – 2/27/2004 |
| Del Puerto Creek | Chlorpyrifos | 0.015 | 6 | 6/23/1994 – 8/21/2001 |
| Stanislaus River | Chlorpyrifos | 0.015 | 2 | 1/4/2001 – 6/21/2001 |
| Olive Ave drain | Chlorpyrifos | 0.015 | 1 | 6/23/1994 |
| Dry Creek | Chlorpyrifos | 0.015 | 8 | 2/11/1995 – 1/12/2000 |
| Bear Creek | Chlorpyrifos | 0.015 | 1 | 6/20/2001 |
| Culvert discharges to Mustang Ck | Chlorpyrifos | 0.015 | 12 | 3/15/2003 – 2/25/2004 |
| Harding Drain | Chlorpyrifos | 0.015 | 16 | 4/22/1992 – 1/6/1993 |
| Hospital Creek | Chlorpyrifos | 0.015 | 1 | 6/23/1994 |
| Ingram Creek | Chlorpyrifos | 0.015 | 1 | 6/21/2001 |
| Livingston Canal | Chlorpyrifos | 0.015 | 2 | 2/8/1994 |
| McHenry stormdrain | Chlorpyrifos | 0.015 | 14 | 2/13/1995 – 1/26/2001 |
| Mud Slough | Chlorpyrifos | 0.015 | 2 | 9/21/1999 & 8/21/2001 |
| Newman Wasteway | Chlorpyrifos | 0.015 | 3 (4?) | 2/4/2000 – 2/14/2000 |
| Ninth St stormdrain | Chlorpyrifos | 0.015 | 1 | 2/13/1995 |
| San Joaquin River | Chlorpyrifos | 0.015 | 26 | 6/23/1994 – 4/27/2004 |
| Spanish Grant combined drain | Chlorpyrifos | 0.015 | 1 | 6/22/1994 |
| Storm drain inlet | Chlorpyrifos | 0.015 | 2 | 2/15/2005 |
| Tuolumne River | Chlorpyrifos | 0.015 | 7 | 1/25/1994 – 8/7/2001 |
| Turlock ID Ceres | Chlorpyrifos | 0.015 | 1 | 2/14/1995 |
| West Side stormdrain | Chlorpyrifos | 0.015 | 1 | 2/13/1995 |
| Westport drain | Chlorpyrifos | 0.015 | 1 | 8/2/2001 |
| Culvert discharge to Mustang Creek | Cis-Permethrin | 0.0033 | 7? | 2/2/2004 & 2/18/2004 |
| Mustang Creek | Cis-Permethrin | 0.0033 | 10? | 2/2/2004 |
| San Joaquin River | Cis-Permethrin | 0.0033 | 1 | 6/29/1993 |
| Dry Creek | DCPA | 0.008 | 4 (6?) | 2/13/1995 – 3/10/1995 |
| Farabuindo stormdrain | DCPA | 0.008 | 1 | 2/13/1995 |
| Harding Drain | DCPA | 0.008 |  | 1/13/1995 – 1/20/1995 |
| Livingston Canal | DCPA | 0.008 | 1 (2?) | 2/8/1994 |
| McHenry stormdrain | DCPA | 0.008 | 3 (9?) | 2/13/1995 – 1/25/2001 |
| Merced River | DCPA | 0.008 | 2 | 2/8/1993 – 2/11/1993 |
| Ninth St stormdrain | DCPA | 0.008 | 1 | 2/13/1995 |
| Orestimba Creek | DCPA | 0.008 | 56 | 4/17/1992 – 10/9/2001 |
| Salt Slough | DCPA | 0.008 | 8 | 1/20/1993 – 4/1/1993 |
| San Joaquin River | DCPA | 0.008 | 27 | 8/12/1992 - 1/11/2001 |
| Sonoma stormdrain | DCPA | 0.008 | 1 | 2/13/1995 |
| Stanislaus River | DCPA | 0.008 | 1 | 2/8/1994 |
| Storm drain inlet | DCPA | 0.008 | 1 | 2/15/2005 |
| Tuolumne River | DCPA | 0.008 | 2 (4?) | 2/8/1994 & 3/10/1995 |
| Turlock ID Ceres | DCPA | 0.008 | 1 | 2/14/1995 |
| West Side stormdrain | DCPA | 0.008 | 1 | 2/13/1995 |
| Culvert discharge to Mustang Creek | Diazinon | 0.1 | 3 (8?) | 2/16/2003 – 2/2/2004 |
| Del Puerto Creek | Diazinon | 0.1 | 4 (5?) | 1/6/2000 – 2/12/2000 |
| Dry Creek | Diazinon | 0.1 | 6 (9?) | 2/13/1995 - 2/11/2000 |
| Farabuindo drain | Diazinon | 0.1 | 1 | 2/13/1995 |
| Harding Drain | Diazinon | 0.1 | 3 | 12/22/1992 – 1/20/1993 |
| Highline Canal spillway | Diazinon | 0.1 | 2 (5?) | 2/8/1994 & 2/14/2000 |
| Hospital Creek | Diazinon | 0.1 | 1 | 6/21/2001 |
| Livingston Canal | Diazinon | 0.1 | 1 (2?) | 2/8/1994 |
| McHenry stormdrain | Diazinon | 0.1 | 5 (17?) | 2/13/1995 – 1/26/2001 |
| Merced River | Diazinon | 0.1 | 11 (24?) | 2/8/1993 – 1/27/2001 |
| Mud Slough | Diazinon | 0.1 | 1 | 5/16/2001 |
| Mustang Creek | Diazinon | 0.1 | 3 (7?) | 2/2/2004 – 2/26/2004 |
| Newman Wasteway | Diazinon | 0.1 | 1 (2?) | 2/14/2000 |
| Ninth St stormdrain | Diazinon | 0.1 | 1 | 2/13/1995 |
| Oakdale ID drainage | Diazinon | 0.1 | 1 | 2/14/1995 |
| Orestimba Creek | Diazinon | 0.1 | 35 | 4/15/1992 - 7/24/2003 |
| Salt Slough | Diazinon | 0.1 | 5 | 2/11/1993 – 7/31/2001 |
| San Joaquin River | Diazinon | 0.1 | 23 (46?) | 1/13/1993 – 2/26/2001 |
| Sonoma stormdrain | Diazinon | 0.1 | 1 | 2/13/1995 |
| Stevinson Lower Lateral | Diazinon | 0.1 | 1 (2?) | 2/8/1994 |
| Tuolumne River | Diazinon | 0.1 | 7 (18?) | 1/25/1994 – 1/26/2001 |
| West Side stormdrain | Diazinon | 0.1 | 1 | 2/13/1995 |
| Anderson drain at Orestimba Creek | Dieldrin | 0.000052 | 1 | 1/10/1995 |
| Del Puerto Creek | Dieldrin | 0.000052 | 1 | 2/12/2000 |
| Hospital Creek | Dieldrin | 0.000052 | 3 | 6/23/1994 & 6/21/2001 |
| Ingram Creek | Dieldrin | 0.000052 | 2 | 6/24/1994 & 6/21/2001 |
| Livingston Canal | Dieldrin | 0.000052 | 2 | 2/11/2000 & 2/12/2000 |
| Newman Wasteway | Dieldrin | 0.000052 | 1 | 8/1/2001 |
| Orestimba Creek | Dieldrin | 0.000052 | 101 | 4/17/1992 – 8/3/2004 |
| River Rd drain | Dieldrin | 0.000052 | 1 | 1/10/1995 |
| San Joaquin River | Dieldrin | 0.000052 | 2 | 8/12/1992 & 9/7/1993 |
| Spanish Grant combined drain | Dieldrin | 0.000052 | 2 | 6/20/2001 & 8/1/2001 |
| Farabuindo stormdrain | Disulfoton | 0.05 | 1 | 2/13/1995 |
| Orestimba Creek | Ethoprop | 15 | 1 | 3/26/2004 |
| Orestimba Creek | Fonofos | 15 | 14 | 4/17/1992 – 5/6/1997 |
| Harding Drain | Lindane | 0.019 | 22 | 5/6/1992 – 8/2/2001 |
| San Joaquin River | Lindane | 0.019 | 1 | 7/17/2001 |
| Newman Wasteway | Malathion | 0.1 | 1 | 2/12/2000 |
| Salt Slough | Malathion | 0.1 | 1 | 3/18/1993 |
| Dry Creek | Malathion | 0.1 | 1 | 2/14/1995 & 3/10/1995 |
| Orestimba Creek | Malathion | 0.1 | 3 | 3/18/1995 – 8/16/2001 |
| San Joaquin River | Malathion | 0.1 | 2 | 1/26/1994 – 5/23/2001 |
| Tuolumne River | Malathion | 0.1 | 1 | 2/8/1994 |
| McHenry stormdrain | Malathion | 0.1 | 1 (2?) | 2/13/1995 |
| Farabuindo stormdrain | Malathion | 0.1 | 1 | 2/13/1995 |
| West Side stormdrain | Malathion | 0.1 | 1 | 2/13/1995 |
| Del Puerto Creek | Methyl parathion | 0.08 | 2 (3?) | 5/23/2001 & 6/19/2001 |
| Orestimba Creek | Methyl parathion | 0.08 | 2 | 7/19/1999 – 6/22/2005 |
| Spanish Grant combined drain | Methyl parathion | 0.08 | 1 | 6/20/2001 |
| San Joaquin River | Methyl parathion | 0.08 | 1 | 7/10/2001 |
| Mud Slough | Molinate | 1 | 1 | 5/23/2001 |
| Salt Slough | Molinate | 1 | 1 | 6/14/1993 |
| Del Puerto Creek | p_p’-DDE | 0.00022 | 12 | 2/12/2000 – 8/14/2001 |
| Harding Drain | p_p’-DDE | 0.00022 | 1 | 5/13/1992 |
| Highline Canal spillway | p_p’-DDE | 0.00022 | 1 | 2/14/2000 |
| Hospital Creek | p_p’-DDE | 0.00022 | 3 | 6/23/1994 – 8/2/2001 |
| Ingram Creek | p_p’-DDE | 0.00022 | 3 | 6/24/1994 – 8/2/2001 |
| Newman Wasteway | p_p’-DDE | 0.00022 | 2 | 2/14/2000 & 6/20/2001 |
| Olive Ave drain | p_p’-DDE | 0.00022 | 1 | 6/23/1994 |
| Orestimba Creek | p_p’-DDE | 0.00022 | 139 | 4/15/1992 – 4/20/2005 |
| San Joaquin River | p_p’-DDE | 0.00022 | 4 | 1/13/1993 – 5/20/2002 |
| Spanish Grant combined drain | p_p’-DDE | 0.00022 | 2 | 6/22/1994 & 6/20/2001 |
| Highline Canal | Simazine | 3.5 | 5 (8?) | 2/8/1994 – 2/27/2004 |
| Merced River | Simazine | 3.5 |  | 1/26/1994 – 12/21/2002 |
| Mustang Creek | Simazine | 3.5 | (34?) | 12/16/2002 – 3/2/2004 |
| Newman Wasteway | Simazine | 3.5 | 2 (3?) | 2/4/2000 & 2/14/2000 |
| Stevinson Lower Lateral | Simazine | 3.5 | 1 (2?) | 2/8/1994 |
| Harding Drain | Terbufos | 0.017 | 1 | 1/13/1993 |
| San Joaquin River | Terbufos | 0.017 | 1 | 7/2/1992 |
| Del Puerto Creek | Trifluralin | 0.7 | 2 | 4/25/2001 – 7/17/2001 |
| Hospital Creek | Trifluralin | 0.7 | 1 | 6/23/1994 & 6/21/2001 |
| Ingram Creek | Trifluralin | 0.7 | 2 | 6/24/1994 & 6/21/2001 |
| Olive Ave drain | Trifluralin | 0.7 | 2 | 6/23/1994 & 6/21/2001 |
| Orestimba Creek | Trifluralin | 0.7 | 1 | 6/11/1999 |
| Salt Slough | Trifluralin | 0.7 | 2 | 4/11/2001 & 5/16/2001 |
| Spanish Grant combined drain | Trifluralin | 0.7 | 1 | 6/20/2001 |