APPENDIX A: Sampling Locations

Table 1) Deer Creek Mercury Load Sampling Sites 2005-2006

Total mercury in water and total suspended solids samples were collected at the following sites:

DCL1. Scotts Flat: Deer Creek between Upper and Lower Scotts Flat Reservoirs via Scotts Flat Dam Rd.  First trail on right above NID gate.

DCL2. Willow Valley: Deer Creek 20 m below Boulder St. ford on Willow Valley Rd.

DCL3. Little Deer Creek: Culvert entrance above Stonehouse parking lot.

DCL4. Gold Run Creek: Gold Run Creek above confluence via trail on fwy easement.

DCL5. Champion Mine: Deer Creek 1 mile below Champion mine on Rothert property.

DCL6. Lake Wildwood: Deer Creek inlet at Lake Wildwood Dr. bridge.

DCL7. Squirrel Creek: Pleasant Valley Rd bridge over Squirrel Creek.

DCL8. Mooney Flat: Mooney Flat Rd. bridge over Deer Creek near Smartville.

DCL9.  Woods Ravine: Woods Ravine 200 m above confluence (off Champion Rd).

DCL10. Slack’s Ravine: Slack’s Ravine 200 m above confluence, from Mooney Flat Rd.

DCL11.  Nevada St:  Nevada St. bridge crossing Deer Creek.

DCL12. Little Deer Ln: Little Deer Creek Ln bridge over Deer Creek (off Slate Creek Rd)

DCL13. Bitney: Bitney Springs Rd crossing Deer Creek.

DCL14. Banner Mtn Trail: Little Deer Creek, upstream side of Banner Mtn Trail crossing.

DCL15.  Northern Queen: Gold Run Creek at Northern Queen Inn, just before it goes under fwy.

DCL16. GR Railroad Ave fork:  Gold Run river right tributary off Railroad Ave.

DCL17. GR Mowhawk fork:  Gold Run river left fork below marsh off Railroad Ave.

DCL18. Eagle Ravine: Eagle Ravine crossing Willow Valley Rd, just past HEW.

DCL19. Manzanita drain: Drainage from Manzanita Diggins discharging below Deer Creek Inn.

2)  Deer Creek Sediment Sampling Sites, 2005-2006

1. (no sample)

- S. Fork Deer Creek- below Deer Creek Powerhouse, pool at Cascade Canal diversion

- FODC Site 1- Confluence of S. and N. Fork Deer Creek

- Casci creek- at confluence with Scotts Flat Lake

4Q. Casci creek- at confluence with Scotts Flat Lake, duplicate

- Cascade Shores- Scotts Flat upper reservoir, below hydraulic cliffs

- Lower Scotts Flat- above reservoir/DCL1

- Burjer- Tailings eroding into Deer Creek, BLM off Burjer Rd.

- (not sampled)

- Murchie- Mill foundations from Murchie mine on Creek

- Mosquito- Mosquito creek above confluence w/ Deer Creek

- Manzanita- outlet below Deer Creek Inn

- Banner Mtn Trail- Little Deer Creek at Banner Mtn. trail crossing

- Little Deer Creek: DCL3, above Stonehouse parking lot

- GR-Gracie Canal- Gold Run off Gracie below canal crossing

- Gold Run confluence- DCL4, above confluence with Deer Creek

15A. Flume’s End- Gold Run at lower tailings dam, Flume’s End inn

- Providence- Tailings eroding into creek and in-stream fines

- Woods Ravine- 100m upstream of confluence

- Champion/DCL5- Deer Creek below Champion Mine

18Q. Champion/DCL5- Deer Creek below Champion Mine duplicate

- Bitney- Deer Creek 100m upstream of Bitney Springs bridge

- LWW- DCL6, Lake Wildwood Dr. bridge crossing Deer Creek inlet

- Squirrel Creek- Squirrel Creek confluence with Deer Creek

- Squirrel/DCL7- Pleasant Valley bridge crossing Squirrel Creek

- Mooney Flat- Mooney Flat pit lake

24. Big Blue- Little Deer Creek, Big Blue Rd tailings below NID flume

- No. Banner- Little Deer Creek, Banner Mtn. trail crossing, North Banner fork

- Canada Hill- Little Deer Creek, Banner Mtn. trail crossing Canada Hill fork

26Q. Canada Hill- Little Deer Creek, above Banner Mtn. trail, Canada Hill duplicate

26A. Canada Hill right- Little Deer Creek, small trib river left Banner Trail Crossing

- Stillwater- Little Deer Creek, fork near Stillwater Creek Rd on Banner Trail.

- Rocker- Little Deer Creek main (S.) fork at Rocker Rd. Crossing

- Eagle Ravine- Eagle Ravine crossing Willow Valley Rd

- Shooting Range- Drainage of shooting range to Little Deer Creek off Rocker Rd.

- Caledonia- Little Deer Creek at end of Caledonia Wy.

- Pioneer- Little Deer Creek at Pioneer Park, above picnic area

32Q. Pioneer- Little Deer Creek at Pioneer Park, near picnic parking lot

- GR Gracie- Gold Run crossing Gracie Rd near Banner Mtn. trail turnoff

- GR Sneath- Gold Run upper Sneath fork off Sneath Clay Rd, in-stream fines

34Q. GR Sneath- Gold Run upper Sneath fork off Sneath Clay Rd, bank

- GR Palisade- Gold Run, Palisade Rd fork below BLM parcel

- GR Pittsburgh- Gold Run, Pittsburgh fork below Gold Flat Rd

- GR Sulphuret- Gold Run, Sulphuret works fork, in-stream fines

37Q. GR Sulphuret- Gold Run, Sulphuret works fork, bank

- GR Mowhawk- Gold Run at New Mowhawk crossing, in-stream fines

38Q. GR Mowhawk- Gold Run at New Mowhawk crossing, bank

- GR N. Queen- Gold Run forks confluence off Railroad Ave, in-stream fines pool

39Q. GR N. Queen- Gold Run forks confluence off Railroad Ave, bank below confl.

39Q1. GR N. Queen- Gold Run forks confluence off RR Ave, bank at turn/old tailings

39A. GR N. Queen- Gold Run forks confluence off RR Ave, Railroad Ave fork in-stream

39B. GR N. Queen- Gold Run forks confluence off RR Ave, Mowhawk fork in-stream

- GR school- Gold Run school fork off Clark St.

- GR Clark- Gold Run off Clark St./fwy right of way, pool below concrete channel

- GR Flume’s- Gold Run at Flume’s End dam

42A. GR DCL4- Gold Run at DC confluence/DCL4, in-stream

42AQ. GR DCL4- Gold Run at DC confluence/DCL4, dry fines on bank

- Woods Ravine- Woods Ravine upstream side of Champion Rd crossing

- Stocking Flat- Deer Creek at Stocking Flat on BLM, beach of corner pool at large hydraulic terrace

- Una’s- off Slate Cr Rd, eroding bank

45Q. Una’s- off Slate Cr Rd, graded tailings river left

45Q1. Una’s- off Slate Cr Rd, in-stream

45Q2. Una’s- off Slate Cr Rd, eroding bank river right

- Little Deer Ln- Little Deer Creek Ln crossing, bank deposit in tree roots

46Q. Little Deer Ln- Little Deer Creek Ln crossing, in-stream fines

46Q1. Little Deer Ln- Little Deer Creek Ln crossing, bank fines from Rick’s 2nd hole

46Q2. Little Deer Ln- Little Deer Creek Ln crossing, Rick’s panned sands from 46Q1

- Slate Creek- Slate creek confluence below Kubich mill via Lowhills Dr.

- Bitney- Bitney Springs crossing, in-stream fines below new bridge

48Q. Bitney- Bitney Springs crossing, bank fines below new bridge

- Empress Cr- Empress Creek off Newtown rd above Bitney crossing

- (no sample)

- DC Falls- Deer Creek falls,  side channel top fines (high flow)

51Q. DC Falls- Deer Creek falls, fines from crack

51Q1. DC Falls- Deer Creek falls, Rick’s panned sample from crack

- Below DC Falls- Below Deer Creek falls, depositional area, in-stream

52Q. Below DC Falls- Below Deer Creek falls, depositional area, fines in tree roots

- Prospector- Tributary creek crossing Prospector Rd

- Herrlingers’- Deer Creek at Herrlingers’, in-stream beach

54Q. Herrlingers’- Deer Creek at Herrlingers’, channel bar/bank

- Lower Paddy- Lower Paddy flat, in-stream backwater eddy

55Q. Lower Paddy- Lower Paddy flat, bank

55A. Middle Paddy- Middle Paddy flat, main bar near Pioneer Rd, Paddy Flat “T”

- LWW inlet- Lake Wildwood Dr./ Deer Creek inlet (DCL6), in-stream under bridge

56Q. LWW inlet- Lake Wildwood Dr./ Deer Creek inlet (DCL6), sandbar under bridge

- Above Squirrel- Deer Creek above Squirrel Creek confluence/FODC Site 8

- Below Squirrel- Deer Creek below Squirrel Creek confluence

58Q. Below Squirrel- Deer Creek below Squirrel Creek confluence, duplicate

- Laurel’s- Deer Creek above Mooney Flat bridge, FODC Site 10

59Q. Laurel’s- Deer Creek above Mooney Flat bridge, FODC Site 10 duplicate

- Manzanita- Manzanita Diggins drainage between Nevada St. bridge and Deer Creek Inn

APPENDIX B: Sampling Protocols

Included for reference are FODC’s mercury sampling protocols, adapted from CALFED Mercury Studies Program Standard Operating Procedures and trainings with USGS and Regional Water Quality Control Board scientists:

- Total Mercury in Water, TSS

- Total Mercury in Sediment

- Methyl Mercury in Macroinvertebrates

- Flow Profile Sampling for Rating Curves

1) Field Sampling Protocols for Total Mercury Water Samples and TSS

Friends of Deer Creek, 1/12/06

Pre-sampling preparations:

1.) Visit the sampling site to determine any special equipment that will be needed. Have the pre-cleaned 250 ml glass sample jars used in clean sampling procedures double bagged in plastic and placed in appropriate containers for transport to the field site. Fill out as much information on the data sheet as possible to save time in the field, especially during wet conditions: Date, Samplers, Site ID, Sample IDs.

2.) Print labels for all water and TSS sample bottles with site identifier, unique sample number, date and type of sample. Write in sample ID and date if using pre-printed labels, and place on sample outer bag, and on TSS bottles.  Example:

Site: DCL1		Analysis: THg Water

Sample ID# DCHgW25 	Friends of Deer Creek

Date: 10/20/05	Time:		Initials:

3.) Check and assemble equipment, as needed:

- Field data sheets and/or write-in-rain notebooks

- Vinyl or polyethylene powderless gloves

- De-ionized water

- Sampling pole (all flows) and/or sampler bucket to lower from bridge (high flows only, bucket rope needs to be washed between samples)

- 250 ml glass sample jars (# of sample sites + 10% QA + 1extra per team in case of breakage or contamination)

- Pre-rinsed, labeled plastic TSS bottles

- Sample labels and chain of custody forms

- Coolers with ice

- Waste containers

- Turbidity meter

- Conductivity meter or thermometer

- QA known concentration/blank samples

- Flashlight, first aid and safety equipment

- Boots or waders

General ultra-clean sampling procedures

Friends of Deer Creek’s samples will be unfiltered, and can be collected by dipping the bottle into the stream or river by hand or on a sampling pole or bucket using Frontier Geoscience's “Clean-Hands Dirty-Hands” technique:  Two people are required for sample collection; determine which person is designated as clean-hands and which person is designated as dirty-hands for the duration of the sample collection. Only the clean-hands person touches the sampling bottles, while the dirty-hands person assists with other tasks to help keep the clean-hands person’s gloves clean.

Mercury in Water and TSS sampling:

- Gloves: Personnel involved in sample processing don vinyl gloves. Gloves can be discarded and replaced during sampling if anything happens which may compromise the cleanliness of the gloves.

- Clean-hands handling: Mercury samples are taken before TSS.  The sample bags are labeled with the time. The dirty-hands person opens the first bag and squeezes the inner bag up like toothpaste without touching the inner bag.  The clean-hands person opens the inner bag, removes the bottle from the inner bag, and stuffs the inner bag back inside of the outer bag.   The dirty-hands person places the outer bag on top of sampling pack or in a place where nothing could touch or contaminate the inner bag.  The clean-hands person empties the 250 ml bottle of acid.

- Low flow:  In shallow streams/low flow conditions, the 250 ml bottles may be directly filled on the end of a sampling pole or by wading into the middle of the current, facing upward, and filling the sample bottle after all turbidity caused by walking has dispersed, taking care not to hit the bottom.

- High-flow:  Water samples should be collected from the fastest moving, well-mixed water.  High-flow samples are taken from shore on the end of a pole or from the downstream side of a bridge, where possible, in a specialized bucket.

  - Rinse pole/bucket: The dirty-hands person extends the sampling pole or ties the end of the bucket rope onto the bridge and lowers the bucket to rinse it with ambient water.  The dirty-hands person holds the bucket or pole while the clean-hands person places the 250 ml bottle in it.  The dirty-hands person tightens the pole strap around the sample bottle, or the clean-hands person secures the tethers on the bottle in the bucket.

  - Triple rinse sample bottle and cap:  The dirty-hands person extends the pole or lowers the sample bucket into the water and rinses the bottle 3 times with ambient water, pouring the rinse water through the cap as well to remove any residual acid.

  - Collect sample:  Dirty-hands re-lowers or extends bucket/pole and collects sample.  Sample bottle should be filled to the brim if possible, which may require several dips.

- Clean-hands handling: The clean hands person caps the filled 250 ml bottle and reseals it in the inner bag while the dirty-hands person holds the outer bag.  The dirty-hand person reseals the outer bag.  Samples are transported on ice, taking care that ice packs do not directly touch bottles and cause freezing.

- Duplicates: Duplicate mercury in water samples for QA are also collected at suspected high mercury sites to assess variability.  Collect duplicate mercury samples by repeating steps 1-5.

- Blanks: MilliQ water is transported into the field and removed from bags using clean-hands dirty-hands, uncapped, lowered over creek or allowed to sit for 1 minute, used to rinse the 250 ml bottles 3 times and then decanted into 250ml sample bottles, labeled and shipped on ice).

- Collect TSS samples: Clean-hands precautions are not required for collecting TSS.  Mark the time on the pre-rinsed, labeled TSS bottles.  If using 1L bottles, the red ring is removed.  Low-flow samples are either collected directly by wading, facing upstream and collecting a subsurface grab in the fastest flowing water after turbidity caused by walking has dispersed, or by dipping the plastic bottles on the end of the sampling pole in the fastest-moving water (being careful not to touch the bottom).  At high flow the sample may be collected on the end of a pole or a bucket lowered from a bridge.  TSS duplicates should be collected for QA.

- Field Forms/Ancillary measurements: Field forms are filled out, including notes on location or conditions that might affect results.  In heavy rain, write-in-rain notebooks are used, and data transposed to field data sheets in the car. Ancillary measurements (and GPS first time at site) are collected:

- Flow or staff gauge height

- Specific conductance and Temperature

- Turbidity

Labeling Bottles

Each 250 ml sample bottle must have a label on the outer Ziploc with site identifier, date of collection, time of collection, unique sample number and sample type.  Waterproof labels can be placed directly on the plastic TSS bottles before sampling.

Sample tracking

Sample tracking: Field samples are checked for proper labeling before shipping, including site, sample ID, analysis type, date, and time. Field forms are filled out with information on sample collectors and processing and other relevant information. A chain of custody form is prepared for each batch of samples shipped to another lab.

Preservation and Analysis

Samples are stored and shipped on ice to California Department of Fish and Game’s Moss Landing Marine Laboratory (MLML) via Fed Ex overnight for preservation within 48 hours of collection. Samples are packed in their original cardboard box in a hard plastic cooler with bubble wrap preventing jostling and keeping them from directly touching the blue ice packs or they may freeze and break.  The cooler is labeled and duct-taped closed.  Raw total mercury is analyzed at MLML using EPA method 1631 (revision B).  Moss Landing is notified when samples are shipped.

Total Suspended Solids (TSS) analysis is performed at the Friends of Deer Creek Laboratory using Standard Methods 2540D:

American Public Health Association, American Water Works Association, and Water Pollution Control Federation. 1992. Standard Methods for the Examination of Water and Wastewater, 18th edition, American Public Health Association, Washington, DC, p 2-56.

.

Quality Assurance/Quality Control Program

Friends of Deer Creek’s mercury survey is a collaboration with the Regional Water Quality Control Board/Central Valley Region and their CALFED mercury QA program.  The program has both a field and a laboratory component.  The field component is composed of the collection of blanks and field duplicates. At least ten percent of FoDC’s samples will be designated as QA samples and distributed over the sampling sites and dates.  Blank samples are most important at headwaters and low mercury sites, and duplicates most important at most contaminated sites. The field blank procedure consists of randomly selecting one site and carrying MilliQ water of known low mercury concentration into the field and processing it thereafter in a manner identical to the field sample. The results are compared with laboratory detection limits for total and methyl mercury to ascertain whether field contamination occurred. Duplicate field samples will be taken simultaneously at selected sites. The difference in TSS and mercury concentration between the paired samples will be used to assess field variability. The laboratory component (except for TSS) will be carried out by Moss Landing Marine Lab.

2) Field Sampling Protocols for Sediment Total Mercury Samples

Friends of Deer Creek, 10/10/05

Friends of Deer Creek’s mercury survey is a collaboration with the Regional Water Quality Control Board/Central Valley Region and their CALFED mercury studies program.  The pilot study will consist of sampling for total mercury in water, total suspended solids (TSS), and total mercury in sediment.

Pre-sampling preparations:

1.) Visit the sampling site to determine the type of equipment that will be needed. Have the pre-cleaned sample scoop and stainless knives used in clean sampling procedures double bagged in plastic and placed in appropriate containers for transport to the field site.

2.) Print labels for all environmental sample bottles with site identifier, unique sample number, date and type of sample. Example:

Site: DC1M		Analysis: THg Sediment

Sample ID# FODCS26 	Friends of Deer Creek

Date: 10/20/05	Time:		Initials:

3.) Check and assemble equipment:

- Field data sheets

- Polyethylene or polycarbonate sampling scoop (Cole Parmer P66600-00 or P66600-30)

- Scoop-cleaning brush

- Sieve with both coarse mesh and 62 micron nylon mesh

- De-ionized water

- Heavy-duty Ziplocs

- Sample labels and chain of custody forms

- Cooler with ice

- Waste containers

- Thermometers

- QA known concentration samples

- First aid and safety equipment

- Boots or waders

General ultra-clean sampling procedures

Total mercury sediment samples will not require ultra-clean sampling procedures, but water and any methylmercury samples will be collected using Frontier Geoscience's “Clean-Hands Dirty-Hands” technique:  Two people are required if performing ultra-clean sample collection: Only the clean-hands person touches the sample jars and scoop, while the dirty-hands person assists with other tasks to help keep the clean-hands person’s gloves clean.

Sediment sample collection:

- Sediment sampling is conducted using a polycarbonate scoop and isolating only the top 2 cm of fines.  If sampling area is mostly large pebbles and cobble, some of these may be moved aside to sample fines underneath, or deeper scoops can be taken.

- Label a medium Ziploc bag with a sharpie: Sample ID, Site ID, date, time, initials, sample type.

- The sampler scoops the sediment from areas with fine material on the bottom or from exposed erosive material below the high water line (sandbars and banks).  Note whether sample material is in-stream fines or bank/floodplain sediments.  5-10 sub-samples are collected from a 10 m2 area and sieved through a coarse 2 mm screen into a Ziploc sample bag.  The Ziploc should be at least 1/3 full to ensure that sufficient fines are included.  If wet sediment is used, do not decant off dirty water, as this is will include suspended fines.

- Take duplicate samples if possible, since sediment samples are highly variable (3 samples per site if possible).  Blank samples should also be processed using Standard Reference Material PAC of 4ppm total mercury.

- GPS, photos and ancillary measurements are collected at the site.  Field data sheets are filled out.

- Processing through the fine mesh may be done either in the field or the lab.  First, label the large Ziploc.

- The coarse sieve is replaced with the 62 micron mesh and the sample is re-sieved into a larger Ziploc, stirring to ensure fines get through and adding small amounts of ambient or DI water as necessary.

- The fine mesh and coarse material are placed in the first Ziploc bag and saved in order to be checked at the lab for mercury residues under the microscope.

- Sediment samples for total mercury do not need to be chilled.  However, water and methylmercury samples must be chilled immediately on blue ice and stored at 5°C or less until analysis.

- In between samples, the scoop and coarse screen are cleaned by rinsing off with ambient water and wiping with a paper towel. The fine mesh should not be reused.  Sometimes it is also necessary to brush the sediment off of the scoop while rinsing with the ambient water. The brush is always kept in a Ziploc bag and is rinsed with ambient water to keep it clean. If the sample scoop appears to be difficult to clean or looks like an oily film is on the plastic, it is scrubbed with Micro cleaning detergent and rinsed with ambient water to insure cleanliness.

- At the lab, sediment bags are allowed to settle for 24 hours and the top water siphoned off.  The sediment is placed in labeled glass sample jars, allowed to evaporate until it is lightly moist, then sealed and shipped to the Regional Board with chain of custody forms.

Labeling Bottles

Each sample jar must be labeled with the site identifier, date of collection, time of collection, unique sample number and analysis type.

Preservation and Analysis

Sediment samples for total mercury are shipped to the Regional Board to be analyzed at California Laboratory Services in Rancho Cordova. Raw total mercury is analyzed at the lab using EPA method 7471.

Sample tracking

Sample tracking: Field samples are checked for proper labeling including site, sample ID, type, date, and time. Field forms are filled out with information on sample collectors and processing and other relevant information. Chain of custody forms are prepared for each shipment of samples transferred to another lab.

Quality Assurance/Quality Control Program

Friends of Deer Creek’s mercury survey is a collaboration with the Regional Water Quality Control Board/Central Valley Region and their CALFED mercury QA program.  The program has both a field and a laboratory component.  The field component is composed primarily of the collection of field duplicates. At least ten percent of FoDC’s samples will be designated as QA samples and distributed over the sampling sites and dates. Duplicate field samples will be simultaneously taken at randomly selected sites. The difference in mercury concentration between the paired samples will be used to assess field variability.  The field blank procedure consists of randomly selecting a site and carrying laboratory reference material of known mercury concentration into the field and processing it thereafter in a manner identical to the field sample.  The site #s associated with blank samples will be DC100. The results are compared with the known mercury concentration to ascertain whether field contamination occurred. The laboratory component (except for TSS) will be carried out by California Laboratory Services.

3) Field Sampling Protocol for Methyl Mercury in Macroinvertebrates

Friends of Deer Creek, June 2006

Field Sample Collection:

Materials:

Kick nets

Cooler

Wet ice (in cooler), dry ice if storing samples in cooler more than 1 day

Ziploc bags (qt and gal sizes)

Sharpie pens

Boots/waders

Plastic containers for sorting, Forceps

Camera

Loppers for brush

Procedure:

1.)    Stream collection methods:  kick or rub on rocks while holding net so that the kicked material flows into the net.  Pick needed bugs from net and place directly in specified Ziploc bags (one taxa per bag).  All bags should be filled to about 1/3 with ambient water, except for waterstriders, which go in a dry bag (slightly moist is good).  Be consistent in the stage/size collected for a sample.  See table below for number of each kind of bug needed.  For dragonflies (Gomphidae, Aeschnidae, etc), seek out calmer/near shore sandy or muddy pools.  Aggressively scrape the bottom/vegetation areas with net and pour sample into plastic container to sort.  For waterstriders, also seek calmer areas, and scoop with net from above or below.  In riffles, turn over rocks individually to look for Hydropsychidae and other bugs, then pick off with fingers or forceps.

2.)    Use sharpie to label each ziploc with:  Site Name, Date, Time of collection, Taxa name.

3.)    Place Ziploc bags with bugs on wet ice in cooler for transport.  Store on dry ice if not processing/freezing same day.

Count Requirements:  Need 1 gram of each sample type.

| Macro Type | Approx. # of individuals in 1 gram |
| --- | --- |
| Gomphidae (dragonflies) | 10-20 |
| Hydripsychidae (caddisflies) | 30-40 |
| Megaloptera (dobson flies) | 1-2 |
| Cordologastridae (dragonflies) | 3-4 |
| Gerridae (waterstriders, 5th instar) | 25-40 |
| Plecoptera Perlidae (stoneflies) | 25-40 |
| Aeschnidae (dragonflies) | 3-4 |

Sample Laboratory Preparation for Shipping:

Materials:

Macro samples in Ziplocs chilled on ice

Powder-free latex gloves

DI water in squirt bottles

Clean forceps (Teflon preferred)

Paper towels

Disposable sorting trays (paper or plastic)

Certified chemically cleaned sample jars

Parafilm

Sample Labels (preprinted)

Sharpies

Data Sheets and pens

Micro-90 cleaner

Procedure:

- Keep field samples on dry ice 4 to 24 hours to depurate before processing in lab.  Taxa must be chilled or they will crawl out of sorting dishes.  Organize taxa collected to be processed separately, site by site, to avoid confusion.

- Personnel involved in sample washing and sorting don powder-free latex gloves

- Place one taxa from one site in a clean disposable sorting tray and rinse thoroughly with DI water.  Sometimes animals will need to be held with clean forceps and agitated in the water or turned over to clean all surfaces.  Water striders or other taxa may also be rinsed 2-3 times in their Ziploc.

- Dry individual(s) from that sample by patting between clean paper towels

- Place unlabeled jar on scale, and push tare.

- Place bugs of that taxa and site in jar until you have at least 1 gram of material

- Record family, number of individuals and weight on data label.  Cap jar.  Add sample label, fill out additional label fields (date, unique sample #), and place on jar.

- Copy unique sample number, family and # individuals/weight (g) with sharpie on jar lid.

- Seal jar lid seam with parafilm.

- Record date, sample #, etc. on data sheet.

- Put samples in freezer.  Frozen samples can be held for several weeks before sending on dry ice to analytical laboratory for analysis.  Send samples overnight Mon-Wed to ensure lab receives them before the weekend.

- Change gloves, sorting trays and paper towels between each sample and wash forceps with micro-90 soap to avoid cross-contamination of samples.

4) Flow Profile Protocol - Friends of Deer Creek, 5/4/06

Flow is calculated by measuring velocity and depth at regular intervals on a cross section of the creek or stream with the flow probe.  For each interval, the flow is calculated by multiplying the length of the Interval (ie 1 ft) by Depth by Velocity (Flow(Q)=IxDxV).  Each interval’s flow is summed to determine the total flow (Total flow = ∑Q1, Q2, Q3…).  See attached data sheet for example. Calculations can be done on a computer.

Supplies:

Flow probe

Long tape measure

Waders, boots or sandals

Flow data sheet or Write-in-Rain notebook and pencil

Measurements:

- Clean magnetic dots on propeller before using.

- Fill in site information- location, samplers, date, time, channel width, sampling interval and staff height.

- Set up measuring tape across stream so you can read ft. on your cross-section.

- Starting from one bank, take depth and velocity measurements at each interval.

- Take a wide stance in the water, facing upstream, with the flow probe held out in front of you and lined up with the current length you are measuring on the tape measure.  The probe should be far enough away from your legs that the water pillowing off you legs does not affect flow moving through the probe.

- Place the bottom of the flow probe on the creek bottom and measure the depth on the side of the probe.  Look at the surface of the water moving past the probe and measure its depth on the probe, not the top of the pile of water that builds up on the upstream side of the probe in fast-moving water.  Record the depth.  If your interval lands on a rock, do not move the probe to the side of the rock, but record the depth at that point, even if it is 0 (emergent rock).

- Move the propeller up to (.6) x the depth you just recorded (ie, if the depth you recorded was 1.0ft, move the propeller up to a height of .6ft in the water column- the side of your probe should say .4ft, indicating that the propeller is .4ft from the surface of the water, or .6ft from the bottom).  Make sure that the red arrows on the side of the propeller are pointing in the direction of flow.  In eddies, flow may be moving upstream or 0.

- Press the bottom button until AVG SPEED appears

- Hold down top button until the bottom number flashes to 0, then release.  The top number is instantaneous velocity, measured in .5 cfs, not nearly as accurate as the average velocity (bottom number), measured in .01 cfs.  Wait 5-30 seconds until the average velocity number stabilizes, then record that value as the velocity.

- If you do not register a velocity and the water is moving, check to see if propeller is clear of debris/magnetic dots are relatively clean.

- Carefully reposition at the next interval and repeat steps 5-9.

- Disregard mph on display- the manufacturer says it is measuring cfs.

- Flow probe does not turn off.

APPENDIX C: Quality Assurance

I. Field Quality Assurance

Water QA Samples:

Seven Field Duplicate and four Field Blank samples were collected.  The mean difference in field duplicate water samples collected was 6.75%.  None of the Field Blanks indicated contamination from sampling procedures.

All field blank water samples analyzed were below the Method Detection Limit of 0.20 ng/L.

Sediment QA Samples:

Eleven duplicate sediment samples of the same type of material (in-stream fines) were collected.  Some replicate samples collected at a given site were of different source material (bank sediments vs in-stream fines), and are not included in this comparison.  Some of the duplicates were collected in September 2005 and then in June 2006 from the same site and material, with the results suggesting high seasonal variation in the mercury content of the material deposited, generally higher [Hg] in the higher spring flows (June 2006 samples), and lower [Hg] in the lower summer flows (Sept 2005 samples).