# Post Event Summary (SRWP Event 063)

Sampling Crew:	Quang Do and Lance Haines, Pacific EcoRisk

Sampling Dates: 	May 15-May17, 2007.

Sampling Type:

| pH | Dissolved Ortho-Phosphate | Triazine Pesticides |
| --- | --- | --- |
| Conductivity | Sulfate | Carbamate Pesticides |
| Dissolved Oxygen | UVA 254 | Urea Pesticides |
| Water Temperature | NO3+NO2, TKN and Total P | Chronic Toxicity (Three Species) |
| Total and Dissolved Hg | TOC, DOC |  |
| Total and Dissolved MeHg | E. coli |  |
| TDS, TSS, Turbidity | OP Pesticides |  |

Sites Sampled:

| Site ID | Site Description | Sample Date |
| --- | --- | --- |
| SRBKR | Sacramento River below Keswick | 5/15/2007 |
| CHKNT | Churn Creek at Knighton Road | 5/15/2007 |
| SRABB | Sacramento River above Bend Bridge | 5/15/2007 |
| SRHAM | Sacramento River near Hamilton City | 5/15/2007 |
| SRCOL | Sacramento River at Colusa | 5/15/2007 |
| YRMRY | Yuba River at Marysville | 5/16/2007 |
| COLDR | Colusa Basin Drain above Knights Landing | 5/16/2007 |
| FRNIC | Feather River near Nicolaus | 5/16/2007 |
| SACSL | Sacramento Slough | 5/16/2007 |
| SRVET | Sacramento River at Veterans Bridge | 5/16/2007 |
| ARDPK | American River at Discovery Park | 5/17/2007 |
| SRFPT | Sacramento River at Freeport | 5/17/2007 |
| SRRMF | Sacramento River at River Mile 44 | 5/17/2007 |

Sites Not Sampled

## ENVIRONMENTAL CONDITIONS

Weather conditions were sunny and warm during the sampling event.  Ambient light was available between 0615 and 2000.  Water level throughout the Sacramento River watershed was slightly lower compared to the last event.  Flow was moderate to strong.

## DEVIATIONS FROM SCOPE OF WORK

## Observed Exceedances of Water Quality Objectives

Data were evaluated for exceedances of relevant Basin Plan numeric objectives for pH, dissolved oxygen. Toxicity results were evaluated for exceedances of the narrative objective for toxicity. Exceedances observed during this event are summarized in the following table. Note that the objective for dissolved oxygen saturation is intended to be compared to a long-term median value, not a single result from a single event.

Exceedances of Basin Plan Objectives

| Site ID | Site Description | Sample Date | Parameter | Objective | Result | Units |
| --- | --- | --- | --- | --- | --- | --- |

## ADDITIONAL INFORMATION

Foam was observed on water surface at Colusa Basin Drain above Knights Landing and American River at Discovery Park sites.

| Prepared by: | Quang Do, Pacific EcoRisk | Submittal Date: | May 17, 2007 |
| --- | --- | --- | --- |
| Approved by: |  | Submittal Date: |  |