# Post Event Summary (SRWP Event 054 & 054 resample)

Sampling Crew:	Quang Do and Lance Haines, Pacific EcoRisk

Sampling Dates: 	August 22 – August 24, 2006, and August 30, 2006.

Sampling Type:

| pH | Dissolved Ortho-Phosphate | Triazine Pesticides |
| --- | --- | --- |
| Conductivity | Sulfate | Organochlorine Pesticides |
| Dissolved Oxygen | UVA 254 | Carbamate Pesticides |
| Water Temperature | NO3+NO2, TKN and Total P | Urea Pesticides |
| Total and Dissolved Hg | TOC, DOC | E. coli |
| Total and Dissolved MeHg | OP Pesticides | Chronic Toxicity (Three Species) |
| TDS, TSS, Turbidity | Pyrethroid Pesticides |  |

Sites Sampled:

| Site ID | Site Description | Sample Date |
| --- | --- | --- |
| CHKNT | Churn Creek at Knighton Road | 8/22/2006 |
| SRBKR | Sacramento River below Keswick | 8/22/2006 |
| SRABB | Sacramento River above Bend Bridge | 8/22/2006 |
| SRHAM | Sacramento River near Hamilton City | 8/22/2006 |
| SRCOL | Sacramento River at Colusa | 8/23/2006 |
| YRMRY | Yuba River at Marysville | 8/23/2006 |
| COLDR | Colusa Basin Drain above Knights Landing | 8/23/2006 |
| SRVET | Sacramento River at Veterans Bridge | 8/23/2006 |
| ARDPK | American River at Discovery Park | 8/23/2006 |
| FRNIC | Feather River near Nicolaus | 8/24/2006 |
| SACSL | Sacramento Slough | 8/24/2006 |
| SRFPT | Sacramento River at Freeport | 8/24/2006 |
| SRRMF | Sacramento River at River Mile 44 | 8/24/2006 |
| SRFPT | Sacramento River at Freeport (re-sample event for toxicity) | 8/30/2006 |
| SACSL | Sacramento Slough (re-sample event for toxicity) | 8/30/2006 |
| RSAER | Reclamation Slough at Ensley Road (re-sample event for toxicity) | 8/30/2006 |

Sites Not Sampled

## ENVIRONMENTAL CONDITIONS

Weather conditions were sunny and warm during the three days of sampling.   Flows were moderate at all sites, and water was predominantly clear.

## DEVIATIONS FROM SCOPE OF WORK

There is no deviation from scope of work during the initial sampling event. During the resample event, steep banks at Reclamation Slough at Ensley Road site prevented collection of sample using peristaltic pump; samples were collected as grabs.

## Observed Exceedances of Water Quality Objectives

Data were evaluated for exceedances of relevant Basin Plan numeric objectives for pH, dissolved oxygen. Toxicity results were evaluated for exceedances of the narrative objective for toxicity. Exceedances observed during this event are summarized in the following table. Note that the objective for dissolved oxygen saturation is intended to be compared to a long-term median value, not a single result from a single event.

Exceedances of Basin Plan Objectives

| Site ID | Site Description | Sample Date | Parameter | Objective | Result | Units |
| --- | --- | --- | --- | --- | --- | --- |

## ADDITIONAL INFORMATION

Sampling for aquatic toxicity at Sacramento River near Hamilton City (SRHAM) was amended to original sampling plan on August 22, 2006.

| Prepared by: | Quang Do, Pacific EcoRisk | Submittal Date: | September 8, 2006 |
| --- | --- | --- | --- |
| Approved by: |  | Submittal Date: |  |