# Post Event Summary (SRWP Event 058)

Sampling Crew:	Quang Do and Lance Haines, Pacific EcoRisk

Sampling Dates: 	December 11 – December 13 , 2006.

Sampling Type:

| pH | Dissolved Ortho-Phosphate | Triazine Pesticides |
| --- | --- | --- |
| Conductivity | Sulfate | Organochlorine Pesticides |
| Dissolved Oxygen | UVA 254 | Carbamate Pesticides |
| Water Temperature | NO3+NO2, TKN and Total P | Urea Pesticides |
| Total and Dissolved Hg | TOC, DOC | E. coli |
| Total and Dissolved MeHg | OP Pesticides | Chronic Toxicity (Three Species) |
| TDS, TSS, Turbidity | Pyrethroid Pesticides |  |

Sites Sampled:

| Site ID | Site Description | Sample Date |
| --- | --- | --- |
| CHKNT | Churn Creek at Knighton Road | 12/11/2006 |
| SRBKR | Sacramento River below Keswick | 12/11/2006 |
| SRABB | Sacramento River above Bend Bridge | 12/11/2006 |
| SRHAM | Sacramento River near Hamilton City | 12/11/2006 |
| SRCOL | Sacramento River at Colusa | 12/11/2006 |
| YRMRY | Yuba River at Marysville | 12/12/2006 |
| COLDR | Colusa Basin Drain above Knights Landing | 12/12/2006 |
| SRVET | Sacramento River at Veterans Bridge | 12/12/2006 |
| SRRMF | Sacramento River at River Mile 44 | 12/12/2006 |
| SRFPT | Sacramento River at Freeport | 12/12/2006 |
| FRNIC | Feather River near Nicolaus | 12/13/2006 |
| SACSL | Sacramento Slough | 12/13/2006 |
| ARDPK | American River at Discovery Park | 12/13/2006 |

Sites Not Sampled

## ENVIRONMENTAL CONDITIONS

Weather conditions were rainy and windy during the sampling event.  Ambient light was available between 7:20 and 16:45.  Water level has risen throughout the Sacramento River watershed compared to the last event.  Flow was moderate to strong.

## DEVIATIONS FROM SCOPE OF WORK

Due to a dry launch channel, boat launch was not possible at Sacramento River at Colusa and sampling was conducted from shore.

## Observed Exceedances of Water Quality Objectives

Data were evaluated for exceedances of relevant Basin Plan numeric objectives for pH, dissolved oxygen. Toxicity results were evaluated for exceedances of the narrative objective for toxicity. Exceedances observed during this event are summarized in the following table. Note that the objective for dissolved oxygen saturation is intended to be compared to a long-term median value, not a single result from a single event.

Exceedances of Basin Plan Objectives

| Site ID | Site Description | Sample Date | Parameter | Objective | Result | Units |
| --- | --- | --- | --- | --- | --- | --- |

## ADDITIONAL INFORMATION

For Colusa Basin Drain above Knight Landing site, additional analyses for nitrate, nitrite, TKN and Total P were subsequently amended due to omission from field log.

| Prepared by: | Quang Do, Pacific EcoRisk | Submittal Date: | December 14, 2006 |
| --- | --- | --- | --- |
| Approved by: |  | Submittal Date: |  |