# Post Event Summary (SRWP Event 050)

Sampling Crew:	Drew Gantner and Quang Do, Pacific EcoRisk

Sampling Dates: 	April 19 - April 21, 2006.

Sampling Type:	 Late Wet Season Event

| pH | Dissolved Ortho-Phosphate | Triazine Pesticides |
| --- | --- | --- |
| Conductivity | Sulfate | Organochlorine Pesticides |
| Dissolved Oxygen | UVA 254 | Carbamate Pesticides |
| Water Temperature | NO3+NO2, TKN and Total P | Urea Pesticides |
| Total and Dissolved Hg | TOC, DOC | E. coli |
| Total and Dissolved MeHg | OP Pesticides | Chronic Toxicity (three Species) |
| TDS, TSS, Turbidity | Pyrethroid Pesticides |  |

Sites Sampled:

| Site ID | Site Description | Sample Date |
| --- | --- | --- |
| CHKNT | Churn Creek at Knighton Road | 4/19/06 |
| SRBKR | Sacramento River below Keswick | 4/19/06 |
| SRABB | Sacramento River above Bend Bridge | 4/19/06 |
| SRHAM | Sacramento River near Hamilton City | 4/19/06 |
| SRCOL | Sacramento River at Colusa | 4/20/06 |
| COLDR | Colusa Basin Drain above Knights Landing | 4/20/06 |
| SACSL | Sacramento Slough | 4/20/06 |
| YRMRY | Yuba River at Marysville | 4/20/06 |
| FRNIC | Feather River near Nicolaus | 4/20/06 |
| SRVET | Sacramento River at Veterans Bridge | 4/21/06 |
| ARDPK | American River at Discovery Park | 4/21/06 |
| SRFPT | Sacramento River at Freeport | 4/21/06 |
| SRRMF | Sacramento River at River Mile 44 | 4/21/06 |

Sites Not Sampled

## ENVIRONMENTAL CONDITIONS

Weather conditions were sunny and cool for the three days of sampling.   Flows were high at all sites, and water was predominantly clear.

## DEVIATIONS FROM SCOPE OF WORK

Due to heavy siltation, closure of boat ramps or impending darkness at boat launches, launching the boat was not possible throughout the event.  All samples had to be collected from shore using a peristaltic pump.

At the Sacramento River at River Mile 44, a large yacht passed by, generating a huge wake and stirring up the water just after having collected the sulfate, TSS sample, and before collecting the mercury samples.  Although we waited several minutes, the water did not settle and was more turbid than it was prior to the boat wake.

The digital camera seemed to have malfunctioned due to a bad memory card, and some photos were not stored.  Once this was realized, we began taking photos using a disposable camera.

## Observed Exceedances of Water Quality Objectives

Data were evaluated for exceedances of relevant Basin Plan numeric objectives for pH, dissolved oxygen. Toxicity results were evaluated for exceedances of the narrative objective for toxicity. Exceedances observed during this event are summarized in the following table. Note that the objective for dissolved oxygen saturation is intended to be compared to a long-term median value, not a single result from a single event.

Exceedances of Basin Plan Objectives

| Site ID | Site Description | Sample Date | Parameter | Objective | Result | Units |
| --- | --- | --- | --- | --- | --- | --- |

## ADDITIONAL INFORMATION

| Prepared by: | Drew Gantner, Pacific EcoRisk | Submittal Date: | July 1, 2006 |
| --- | --- | --- | --- |
| Approved by: |  | Submittal Date: |  |