# Post Event Summary (SRWP Event 067)

Sampling Crew:	Quang Do and Lance Haines, Pacific EcoRisk

Sampling Dates: 	August 7-August 9, 2007.

Sampling Type:

| pH | Dissolved Ortho-Phosphate | Triazine Pesticides |
| --- | --- | --- |
| Conductivity | Sulfate | Carbamate Pesticides |
| Dissolved Oxygen | UVA 254 | Urea Pesticides |
| Water Temperature | NO3+NO2, TKN and Total P | Chronic Toxicity (Three Species) |
| Total and Dissolved Hg | TOC, DOC |  |
| Total and Dissolved MeHg | E. coli |  |
| TDS, TSS, Turbidity | OP Pesticides |  |

Sites Sampled:

| Site ID | Site Description | Sample Date |
| --- | --- | --- |
| SRBKR | Sacramento River below Keswick | 8/7/2007 |
| CHKNT | Churn Creek at Knighton Road | 8/7/2007 |
| SRABB | Sacramento River above Bend Bridge | 8/7/2007 |
| SRHAM | Sacramento River near Hamilton City | 8/7/2007 |
| SRCOL | Sacramento River at Colusa | 8/7/2007 |
| YRMRY | Yuba River at Marysville | 8/8/2007 |
| COLDR | Colusa Basin Drain above Knights Landing | 8/8/2007 |
| FRNIC | Feather River near Nicolaus | 8/8/2007 |
| SACSL | Sacramento Slough | 8/8/2007 |
| SRVET | Sacramento River at Veterans Bridge | 8/8/2007 |
| ARDPK | American River at Discovery Park | 8/9/2007 |
| SRFPT | Sacramento River at Freeport | 8/9/2007 |
| SRRMF | Sacramento River at River Mile 44 | 8/9/2007 |

Sites Not Sampled

## ENVIRONMENTAL CONDITIONS

Weather conditions were sunny and warm during the sampling event.  Ambient light was available between 0615 and 2015.  Water level throughout the Sacramento River watershed was lower compared to the last event.  Flow was slow to moderate.

## DEVIATIONS FROM SCOPE OF WORK

Due to an essentially dry streambed and interrupted flow at Churn Creek at Knighton Road site, flow measurements were replaced by one flow estimation.

## Observed Exceedances of Water Quality Objectives

Data were evaluated for exceedances of relevant Basin Plan numeric objectives for pH, dissolved oxygen. Toxicity results were evaluated for exceedances of the narrative objective for toxicity. Exceedances observed during this event are summarized in the following table. Note that the objective for dissolved oxygen saturation is intended to be compared to a long-term median value, not a single result from a single event.

Exceedances of Basin Plan Objectives

| Site ID | Site Description | Sample Date | Parameter | Objective | Result | Units |
| --- | --- | --- | --- | --- | --- | --- |

## ADDITIONAL INFORMATION

Sample for field duplicate analysis of carbamate and urea pesticides was collected at Yuba River at Marysville site instead of Sacramento River at Veterans Bridge site.

| Prepared by: | Quang Do, Pacific EcoRisk | Submittal Date: | October 3, 2007 |
| --- | --- | --- | --- |
| Approved by: |  | Submittal Date: |  |