A Summary of the 2004 TMDL Monitoring for Selected Pesticides in the Sacramento-San Joaquin Delta

January - February 2004

Henry J. Calanchini

Michael L. Johnson

John Muir Institute of the Environment

University of California, Davis

October 2005

Contents

Introduction	3

Objective	3

Monitoring Overview	3

Hydrologic Conditions During The Study	5

Sample Collection Methods	7

Discharge Methods	7

Loading Rate Calculations	8

Laboratory Analysis Methods	8

Quality Assurance Objectives	9

Results	10

Sources Cited	19

Acknowledgements	19

Figures

Figure 1. The twelve sampling sites in the Sacramento-San Joaquin delta monitored for pesticides during January and February 2004.	4

Figure 2.  Average daily rainfall in the Sacramento-San Joaquin delta, California, during the 2003-04 winter storm sampling.	6

Tables

Table 1. Sample sites, collection methods and sampling dates	5

Table 2.  CDFA Laboratory limits of detection and practical quantitation limits for select pesticides	9

Table 3.  Summary of environmental data collected on diazinon and chlorpyrifos concentrations and instantaneous loading rates for sites in the Sacramento-San Joaquin River Delta, California.  February 2004.	12

Table 4. Summary of diazinon and chlorpyrifos concentrations quality-control data for sites in the Sacramento-San Joaquin River Delta, February 2004.	17

Appendix A.  Pesticide results (excluding diazinon and chlorpyrifos).	21

Appendix B.  Lab blank data	26

Appendix C.  Recovery rates in lab control matrix spikes	26

## Introduction

This report describes the results of pesticide monitoring at twelve locations in California’s Sacramento-San Joaquin Delta during January and February 2004. Monitoring was conducted by staff of the Aquatic Ecosystems Analysis Laboratory (AEAL) of the John Muir Institute of the Environment, University of California, Davis, as authorized under Contract No. 02-210-150 from the Central Valley Regional Water Quality Control Board (CVRWQCB).

## Objective

The primary objective of this project was to monitor twelve sites in the Sacramento-San Joaquin River Delta during the 2003-04 winter storm season to characterize the sources of diazinon, chlorpyrifos and other pesticides that can cause surface water contamination and toxic conditions to aquatic life. The results of this study will be used to support the development of diazinon and chlorpyrifos TMDLs in the Sacramento-San Joaquin delta.

## Monitoring Overview

Two sites were monitored once every two weeks from 8 January to 19 February 2004 independent of weather and runoff conditions.  Ten additional sites were monitored once per day for five consecutive days during and following two separate storm events in January and February, with two of the sites monitored for a sixth day during each storm event  (Figure 1, Table 1).  The trigger for initiating a sampling event was a cumulative total of 0.5” of rainfall within the sampling area in a 24-hour period.

The measured field parameters included pH, water temperature and electrical conductivity (EC). Stream discharge was measured at three sites (Delt01, Delt09 and Delt10) using standard USGS methods and a Swoffer Model 2100 current meter.  Discharge estimates for one site (Delt06) were obtained from the California Department of Water Resources (CDPR).

Water samples were delivered to the California Department of Food and Agriculture (CDFA) laboratory in Sacramento, California for chemical analysis using gas chromatography (GC) and mass spectrometry (MS). The CDFA laboratory analyzed 17 chemical compounds for each water sample. The list of compounds is provided in Table 2.  The detection frequencies, concentrations and calculated instantaneous loading rates for diazinon and chlorpyrifos are presented in Table 3.  The detection frequencies and concentrations of the other 15 compounds are listed in Appendix A.  The analytical results for all tested compounds, and the physical parameters measured in the field are presented in tabular format on a compact disc appended to this report.

Figure 1. The twelve sampling sites in the Sacramento-San Joaquin delta monitored for pesticides during January and February 2004.

| Table 1. Sample sites, collection methods and sampling dates |  |  |  |
| --- | --- | --- | --- |
| Site # | Site Name | Sample collection Method | Sampling Dates |
| Delt01 | Mokelumne River at New Hope Road | Integrated grab from bridge | February 2-6 & 16-20, 2004 |
| Delt02 | Mosher Slough at Mariners Drive | Grab from bank | February 2-6 & 16-20, 2004 |
| Delt03 | Five Mile Slough at Plymouth Road | Grab from bank | February 2-6 & 16-20, 2004 |
| Delt04 | Calaveras River at Ijams Road | Grab from bank | February 2-6 & 16-20, 2004 |
| Delt05 | Mid Roberts Island Drain | Grab from bank | January 8, 22, February 2, 19 |
| Delt06 | French Camp Slough at Airport Way | Grab from bank | February 2-6 & 16-20, 2004 |
| Delt08 | Old River at Tracy Road | Grab from bank | January 8, 22, February 2, 19 |
| Delt09 | Marsh Creek at Cypress Road | Grab from bank | February 2-6 & 16-20, 2004 |
| Delt10 | Ulatis Creek at Brown Road | Integrated grab from bridge/Grab from bank | February 2-6 & 16-20, 2004 |
| Delt11 | Duck Slough at Five Points Marina | Grab from bank | February 2-6 & 16-20, 2004 |
| Delt13 | Cache Slough at Real McCoy Ferry | Grab from bank | February 2-7 & 16-21, 2004 |
| Delt14 | Sacramento River at Rio Vista | Integrated grab from pier | February 2-7 & 16-21, 2004 |

## Hydrologic Conditions During The Study

The rainfall amounts listed in this summary are averages of the combined rainfall accumulations at five rain gages throughout the delta region: Stockton Airport, Stockton Fire Station (south of Lodi), Lodi, Fairfield and Livermore.

There were no storms in the delta in January 2004. Delta rainfall for January was 1.61 inches; significantly lower than the historical average of 3.63 inches.  Most of the delta precipitation in January came from isolated showers, the largest of which dropped 0.138 inches in 24 hours. February rainfall total in the delta of 4.45 inches was significantly higher than the historical average of 2.80 inches.

The first major storm event of 2004 fell on February 2nd and 3rd.  The week preceding the first storm was relatively calm with winds less than 10 mph.  Sampling began on February 2nd and continued through February 8th; during this sampling period 0.97 inches of rain fell in the delta, almost 35% of normal rainfall for the entire month.  Within this six-day sampling period the most rain fell on February 2nd, a total of 0.68 inches in 24 hours.  February 3rd also had substantial rainfall, with 0.20 inches of rain falling in 24 hours.  The remaining four days experienced showers accumulating another 0.09 inches.

The second storm event occurred between February 16th and February 19th; sampling began on the 16th and continued through the 21st.  This storm was preceded by 9 days during which the winds were below 10 mph and rainfall totaled 0.04 inches.  During this sampling period 1.77 inches of rain fell, accounting for 63% of normal February rainfall.  Within this six-day sampling period, the most rain fell on the 16th, with 0.72 inches of rain in 24 hours.  The 17th and 18th had substantial rainfall as well, with 0.41 and 0.57 inches, respectively.  Smaller showers for the remaining 3 days totaled 0.07 inches.

Figure 2.  Average daily rainfall in the Sacramento-San Joaquin delta, California, during the 2003-04 winter storm sampling.

## Sample Collection Methods

All samples were collected by either grab or integrated grab methods (Table 1).

Grab samples were collected by harnessing a 1-liter amber glass bottle to a pole sampler and dipping the bottle into the stream as close to the center of the channel as possible.

Integrated grab samples were collected by lowering a 3-liter PTFE (polytetrafluoroethylene) bottle, strapped in a weighted cage, from a bridge at three equally spaced verticals.  At each vertical the bottle was filled approximately ¼ full.  The composite sample was then thoroughly agitated and poured into a 1-liter amber glass sample bottle.

## Discharge Methods

At Marsh Creek and Ulatis Creek discharge was measured using a Swoffer Model 2100 current meter while wading.  At the Mokelumne River, and at Ulatis Creek during high flows, discharge was measured from a bridge using a bridgeboard, sounding reel and Swoffer Model 2100 current meter.  All measurements were made using standard USGS current-meter methods (Nolan, et al. 2001) with the following exception: due to current meter failure, discharge at the Mokelumne River on Feb 18 was measured using the float-method (Nolan, et al. 2001).

The float method is a crude means of determining stream velocity by timing the travel of a floating object, such as an orange peel or a stick, along a known length of stream, repeating the process at varying distances across the channel width, averaging the measured velocities then applying the average velocity to an area-velocity equation.  The discharge we calculated using this method fit nicely into a normal bell-shaped hydrograph when plotting it with the discharge measurements made on the preceding three days and the following two days.

Discharge estimates for French Camp Slough at Airport Way were provided courtesy of John Tingle of the California Department of Water Resources (CDPR) from the CDPR gage located on site.

No discharge measurements were made at any of the other sites due to tidal influences, safety considerations and site logistics.

## Loading Rate Calculations

Instantaneous loading rates of diazinon and chlorpyrifos were calculated by multiplying the stream discharge at the time of sample collection by the measured concentrations of each pesticide by the number of seconds (86,400) in one day.  Loading rates were only calculated when the pesticide concentration was above the limit of detection and a discharge estimate was available.  The loading rate was assumed to be zero for all samples where pesticide concentrations were below the limit of detection.

## Laboratory Analysis Methods

Upon arrival at the CDFA laboratory, the environmental samples were weighed then spiked with 500µL of 1.0 µg/ml chlorpyrifos methyl (0.5µg/mL) surrogate spiking solution. Each sample was emptied into a 2-liter separatory funnel and approximately 10-15g of granular sodium chloride was added.  Sixty ml of methylene chloride were added and the sample was then mixed for three minutes.  The organic fraction was filtered through a bed of granular anhydrous sodium sulfate (approx. 20g).  The extraction process was repeated three times and the resultant sample was evaporated to 5-7 ml at 40° C and then evaporated to dryness with an N-evaporator.   One ml of methylene chloride and 10µL of a 5.0µg/mL internal standard solution were added to each sample. Samples were stored in a –5ºC freezer until analysis.  Samples were analyzed with an Agilent Model 5973 GC-MSD using a HP-5MS or equivalent GC column.  Analysis was performed in the selective ion-monitoring mode.

Each sample was analyzed for seventeen compounds.  The compounds and their respective limits of quantitation (LOQ) and limits of detection (LOD) are listed in Table 2. The lab reported estimated values when the values were below the LOQ but above the LOD. To ensure the accuracy and precision of the sample analysis, lab spikes, blanks, and a surrogate standard (chlorpyrifos methyl) were used. If the recovery of a spike sample was out of the control range, the water sample was re-analyzed.

Table 2.  CDFA Laboratory limits of detection and practical quantitation limits for select pesticides

| Compound | Limit of Detection (LOD in µg/L) | Limit of Quantitation (LOQ in µg/L) |
| --- | --- | --- |
| Azinphos methyl | 0.007 | 0.050 |
| Bifenthrin | 0.007 | 0.050 |
| Carbaryl | 0.007 | 0.020 |
| Chlorpyrifos | 0.004 | 0.010 |
| Cyanazine | 0.007 | 0.050 |
| Cyfluthrins | 0.070 | 0.200 |
| Cypermethrins | 0.070 | 0.200 |
| Dacthal (DCPA) | 0.007 | 0.050 |
| Diazinon | 0.007 | 0.020 |
| Disulfoton | 0.007 | 0.020 |
| EPTC (Eptam) | 0.020 | 0.050 |
| Esfenvalerate | 0.007 | 0.050 |
| l-Cyhalothrin | 0.030 | 0.100 |
| Methidathion | 0.010 | 0.030 |
| Metolachlor | 0.007 | 0.020 |
| Propargite | 0.150 | 0.500 |
| Simazine | 0.005 | 0.200 |

## Quality Assurance Objectives

Sampling during the 2003-04 winter storm season was conducted under the guidance of a draft Quality Assurance Project Plan (QAPP) (San Joaquin River TMDL Quality Assurance Project Plan Azimi-Gaylon and Reyes, 2002).

Sampling precision and variability were measured through the use of field duplicates.  The draft QAPP stated the Quality Assurance Objective (QAO) for precision was a relative percent difference (RPD) of less than 50%.  No QAO was stated for accuracy in the draft QAPP.  Accuracy was measured by determining the percent recovery of known concentrations of analytes spiked into environmental samples or reagent water prior to extraction.  A 70-130% recovery rate is commonly viewed as acceptable  (D. McClure, pers. comm.) and, for the purpose of this report, was used as the QAO for accuracy in laboratory analytical measurements.

When reporting analytical results it is customary to flag those results that fall outside of the acceptable level of recovery as stated in the QAOs.   For the purpose of this report all results outside of the 70-130% recovery range are flagged as follows: BL = results should be viewed as biased low due to low surrogate recovery in sample.  BH = results should be viewed as biased high due to high surrogate recovery in sample.

## Results

A total of 112 environmental samples (Table 3) and 23 quality control (QC) samples (Table 4) were collected and analyzed.

Environmental samples

Concentrations of diazinon and chlorpyrifos ranged from below detection to 0.520 parts per billion (ppb) of diazinon in the Calaveras River on 2 February and 0.510 ppb chlorpyrifos in Duck Slough on 18 February (Table 3).

The highest and lowest calculated instantaneous loading rates for diazinon were in Ulatis Creek and Marsh Creek, respectively.  The highest and lowest calculated instantaneous loading rates for chlorpyrifos were also in Ulatis Creek and Marsh Creek, respectively (Table 3).

Other pesticides detected in the environmental samples were Bifenthrin, Carbaryl, Cyanazine, Dacthal (DCPA), Eptam (EPTC), Esfenvalerate, Methidathion, Metolachlor and Simazine (Appendix A).

Four environmental samples had surrogate recoveries outside of the QAO acceptance limits – see footnotes in Table 3 and Appendix A.

Environmental Quality Control Samples

Sample quality control was measured through collection of sequential and split duplicates (n=9), field blanks (n=7) and matrix spikes (n=7).  Duplicate samples provided a measure of analytical precision; field blanks were used to evaluate possible introduction of contaminants during sample collection, handling and transport to the lab; and matrix spikes were used to evaluate the relative percent recovery of spiked chemicals through extraction from the sample matrix.

The procedures used for collecting the QA/QC samples were based on the draft San Joaquin River TMDL Quality Assurance Project Plan (Azimi-Gaylon and Reyes, 2002)

The QAO for duplicate samples was a relative percent difference (RPD) of < 50% between the duplicate and the corresponding environmental sample concentrations.  Eight of nine sample pairs met the QAO for precision and variability. The RPDs for diazinon and chlorpyrifos ranged from 0 – 105.26% and 0 – 114.96%, respectively (Table 4).  The single highest RPD values for both diazinon and chlorpyrifos were likely due to low recoveries of those compounds in the environmental sample collected at Mosher Slough on 2 February; the surrogate recovery for the environmental sample was 29% while the surrogate recovery for the corresponding duplicate sample was 121%.  A duplicate sample from Five Mile Slough on 17 February had a surrogate recovery of 135%, exceeding the QAO of 70-130% for recoveries of spiked compounds.  The corresponding environmental sample had a surrogate recovery of 130%.  The RPD between the two samples was 18.18% for chlorpyrifos and 0% for diazinon.  The similar rates of surrogate recovery and the low RDP values indicate an acceptable level of precision in the analytical method.

The QAO acceptance limit for field blanks was “less than the reporting limit”. All seven field blanks met the acceptance limits with no detections of any pesticides in any blank (Table 4).

The QAO acceptance limit for matrix spikes was a 70-130% recovery rate for both chlorpyrifos and diazinon.  Each of the seven matrix spikes met the QAO objective for recovery. Recoveries ranged from 88-119% for chlorpyrifos and 88-115% for diazinon (Table 4).

Laboratory Quality Control Samples

Seventeen lab blanks and 17 lab control matrix spikes were analyzed with the environmental samples.  The QAO acceptance limits for lab blanks and lab control matrix spikes were recoveries of 70-130% each for bifenthrin, chlorpyrifos, diazinon and the surrogate (chlorpyrifos methyl).  All laboratory quality control samples met the quality assurance objectives.

Recoveries in lab blanks ranged from 79-122% (Appendix B).  Recoveries in lab control matrix spikes ranged from 85-128%, 82-109%, 87-119% and 92-124% for bifenthrin, chlorpyrifos, diazinon and chlorpyrifos methyl, respectively (Appendix C).

| Table 3. Summary of environmental data collected on diazinon and chlorpyrifos concentrations and instantaneous loading rates for sites in the Sacramento-San Joaquin River Delta, California. February 2004. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream flow is in cubic feet per second. J: the reported concentrations were below the quantitative limit and are considered estimates; NA: not available; ND: Not detected; g a.i./d: grams active ingredient per day; μg/L: micrograms per liter; BL: result should be viewed as biased low due to low surrogate recovery in sample; BH: result should be viewed as biased high due to high surrogate recovery in sample. |  |  |  |  |  |  |  |  |
| Site number | Site name | Date (month/day/year) | Time (24 hr) | Stream flow (cfs) | Chlorpyrifos concentration (μg/L) | Chlorpyrifos instantaneous loading rate (g a.i./d) | Diazinon concentration (μg/L) | Diazinon instantaneous loading rate (g a.i./d) |
| Delt01 | Mokelumne River at New Hope Road | 2/2/2004 | 17:10 | NA | ND | NA | (0.012 J) | NA |
|  |  | 2/3/2004 | 8:00 | 292.76 | 0.010 | 7.16 | 0.073 | 52.29 |
|  |  | 2/4/2004 | 8:10 | 274.75 | ND | NA | (0.014 J) | 9.41 |
|  |  | 2/5/2004 | 8:00 | 238.37 | ND | NA | ND | NA |
|  |  | 2/6/2004 | 8:10 | 238.37 | ND | NA | ND | NA |
|  |  | 2/16/2004 | 12:00 | 308.30 | 0.043 | 32.43 | 0.059 | 44.50 |
|  |  | 2/17/2004 | 15:10 | NA | ND | NA | (0.009 J) | NA |
|  |  | 2/18/2004 | 16:40 | 284.99 | (0.006 J) | 4.18 | (0.017 J) | 11.85 |
|  |  | 2/19/2004 | 8:20 | 232.02 | ND | NA | (0.012 J) | 6.81 |
|  |  | 2/20/2004 | 8:10 | 196.35 | ND | NA | ND | NA |
| Delt02 | Mosher Slough at Mariners Drive | 2/2/2004 | 17:002 | NA | BL (0.027) | NA | BL (0.180) | NA |
|  |  | 2/3/2004 | 9:50 | NA | 0.100 | NA | 0.480 | NA |
|  |  | 2/4/2004 | 10:10 | NA | 0.038 | NA | 0.400 | NA |
|  |  | 2/5/2004 | 10:00 | NA | 0.023 | NA | 0.410 | NA |
|  |  | 2/6/2004 | 10:30 | NA | 0.048 | NA | 0.410 | NA |
|  |  | 2/16/2004 | 16:30 | NA | 0.037 | NA | 0.180 | NA |
|  |  | 2/17/2004 | 9:203 | NA | BH (0.020) | NA | BH (0.160) | NA |
|  |  | 2/18/2004 | 10:00 | NA | 0.011 | NA | 0.140 | NA |
|  |  | 2/19/2004 | 10:30 | NA | 0.041 | NA | 0.130 | NA |
|  |  | 2/20/2004 | 11:30 | NA | 0.020 | NA | 0.130 | NA |
| Delt03 | Five Mile Slough at Plymouth Road | 2/2/2004 | 16:50 | NA | (0.007 J) | NA | 0.330 | NA |
|  |  | 2/3/2004 | 10:10 | NA | 0.044 | NA | 0.300 | NA |
|  |  | 2/4/2004 | 10:20 | NA | ND | NA | 0.290 | NA |
|  |  | 2/5/2004 | 10:20 | NA | ND | NA | 0.300 | NA |
|  |  | 2/6/2004 | 10:50 | NA | 0.043 | NA | 0.320 | NA |
|  |  | 2/16/2004 | 16:00 | NA | 0.021 | NA | 0.160 | NA |
|  |  | 2/17/2004 | 9:40 | NA | (0.006 J) | NA | 0.160 | NA |
|  |  | 2/18/2004 | 10:20 | NA | ND | NA | 0.120 | NA |
|  |  | 2/19/2004 | 10:40 | NA | ND | NA | 0.130 | NA |
|  |  | 2/20/2004 | 11:40 | NA | ND | NA | 0.110 | NA |
| Delt04 | Calaveras River at Ijams Road | 2/2/2004 | 16:10 | NA | 0.068 | NA | 0.520 | NA |
|  |  | 2/3/2004 | 10:40 | NA | 0.027 | NA | 0.240 | NA |
|  |  | 2/4/2004 | 11:00 | NA | (0.006 J) | NA | 0.043 | NA |
|  |  | 2/5/2004 | 10:50 | NA | ND | NA | 0.022 | NA |
|  |  | 2/6/2004 | 11:20 | NA | 0.011 | NA | 0.022 | NA |
|  |  | 2/16/2004 | 15:40 | NA | 0.014 | NA | 0.170 | NA |
|  |  | 2/17/20044 | 10:00 | NA | BH (0.018) | NA | BH (0.170) | NA |
|  |  | 2/18/2004 | 10:50 | NA | 0.015 | NA | 0.110 | NA |
|  |  | 2/19/2004 | 11:10 | NA | (0.009 J) | NA | (0.016 J) | NA |
|  |  | 2/20/2004 | 12:00 | NA | (0.005 J) | NA | (0.007 J) | NA |
| Delt05 | Mid Roberts Island Drain | 1/8/2004 | 12:20 | NA | ND | NA | ND | NA |
|  |  | 1/22/2004 | 11:20 | NA | ND | NA | (0.019 J) | NA |
|  |  | 2/5/2004 | 12:30 | NA | (0.009 J) | NA | 0.062 | NA |
|  |  | 2/19/2004 | 12:40 | NA | ND | NA | 0.026 | NA |
| Delt06 | French Camp Slough at Airport Way | 2/2/2004 | 15:20 | 9.99 | 0.015 | 0.37 | 0.210 | 5.13 |
|  |  | 2/3/2004 | 11:30 | 9.99 | 0.012 | 0.29 | 0.160 | 3.91 |
|  |  | 2/4/2004 | 14:10 | 9.65 | 0.016 | 0.38 | 0.280 | 6.61 |
|  |  | 2/5/2004 | 11:20 | 105.00 | 0.012 | 3.08 | 0.440 | 113.03 |
|  |  | 2/6/2004 | 12:30 | 73.40 | (0.005 J) | 0.90 | 0.085 | 15.26 |
|  |  | 2/16/2004 | 15:10 | 10.70 | (0.004 J) | 0.10 | 0.043 | 1.13 |
|  |  | 2/17/2004 | 10:50 | 10.70 | (0.006 J) | 0.16 | 0.043 | 1.13 |
|  |  | 2/18/2004 | 11:20 | 10.70 | (0.006 J) | 0.16 | 0.130 | 3.40 |
|  |  | 2/19/2004 | 11:40 | 526.00 | 0.011 | 14.16 | 0.055 | 70.78 |
|  |  | 2/20/2004 | 12:40 | 214.00 | (0.004 J) | 2.09 | 0.026 | 13.61 |
| Delt08 | Old River at Tracy Boulevard | 1/8/2004 | 11:30 | NA | ND | NA | ND | NA |
|  |  | 1/22/2004 | 10:40 | NA | ND | NA | 0.028 | NA |
|  |  | 2/5/2004 | 11:50 | NA | (0.006 J) | NA | 0.027 | NA |
|  |  | 2/19/2004 | 12:10 | NA | ND | NA | (0.014 J) | NA |
| Delt09 | Marsh Creek | 2/2/2004 | 15:20 | 19.07 | ND | NA | 0.086 | 4.01 |
|  |  | 2/3/2004 | 11:00 | 14.13 | (0.008 J) | 0.28 | 0.087 | 3.01 |
|  |  | 2/4/2004 | 10:40 | 13.77 | (0.006 J) | 0.20 | 0.054 | 1.82 |
|  |  | 2/5/2004 | 10:30 | 7.42 | ND | NA | 0.044 | 0.80 |
|  |  | 2/6/2004 | 10:40 | 4.94 | (0.006 J) | 0.07 | 0.028 | 0.34 |
|  |  | 2/16/2004 | 14:40 | 41.67 | ND | NA | 0.077 | 7.85 |
|  |  | 2/17/2004 | 12:40 | 3.53 | ND | NA | 0.065 | 0.56 |
|  |  | 2/18/2004 | 12:30 | 373.98 | 0.012 | 10.98 | 0.047 | 43.00 |
|  |  | 2/19/2004 | 10:30 | 104.18 | ND | NA | (0.011 J) | 2.80 |
|  |  | 2/20/2004 | 10:40 | 41.32 | ND | NA | (0.007 J) | 0.71 |
| Delt10 | Ulatis Creek at Brown Rd | 2/2/2004 | 14:10 | NA | 0.030 | NA | 0.025 | NA |
|  |  | 2/3/2004 | 13:20 | 604.59 | 0.091 | 134.60 | 0.035 | 51.77 |
|  |  | 2/4/2004 | 13:20 | 321.72 | 0.085 | 66.90 | 0.024 | 18.89 |
|  |  | 2/5/2004 | 13:00 | 228.84 | 0.095 | 53.19 | 0.021 | 11.76 |
|  |  | 2/6/2004 | 13:00 | 176.93 | 0.091 | 39.39 | 0.021 | 9.09 |
|  |  | 2/16/2004 | 16:30 | 1687.33 | ND | NA | ND | NA |
|  |  | 2/17/2004 | 10:20 | 897.70 | 0.120 | 263.55 | 0.100 | 219.62 |
|  |  | 2/18/2004 | 10:10 | 3092.51 | 0.039 | 295.07 | 0.052 | 393.42 |
|  |  | 2/19/2004 | 12:50 | 398.70 | 0.110 | 107.30 | 0.030 | 29.26 |
|  |  | 2/20/2004 | 12:50 | 205.88 | 0.086 | 43.32 | (0.020 J) | 10.07 |
| Delt11 | Duck Slough at Five Points Marina | 2/2/2004 | 13:10 | NA | 0.020 | NA | 0.027 | NA |
|  |  | 2/3/2004 | 16:10 | NA | 0.230 | NA | 0.024 | NA |
|  |  | 2/4/2004 | 16:40 | NA | 0.360 | NA | 0.032 | NA |
|  |  | 2/5/2004 | 16:50 | NA | 0.120 | NA | 0.028 | NA |
|  |  | 2/6/2004 | 8:50 | NA | 0.100 | NA | 0.024 | NA |
|  |  | 2/16/2004 | 12:50 | NA | (0.009 J) | NA | (0.007 J) | NA |
|  |  | 2/17/2004 | 15:30 | NA | 0.025 | NA | (0.015 J) | NA |
|  |  | 2/18/2004 | 16:10 | NA | 0.510 | NA | (0.013 J) | NA |
|  |  | 2/19/2004 | 8:50 | NA | 0.400 | NA | (0.016 J) | NA |
|  |  | 2/20/2004 | 10:30 | NA | 0.190 | NA | (0.017 J) | NA |
| Delt13 | Cache Slough at Real McCoy Ferry | 2/2/2004 | 13:40 | NA | (0.004 J) | NA | 0.055 | NA |
|  |  | 2/3/2004 | 15:30 | NA | ND | NA | 0.029 | NA |
|  |  | 2/4/2004 | 16:10 | NA | ND | NA | (0.020 J) | NA |
|  |  | 2/5/2004 | 16:20 | NA | ND | NA | 0.023 | NA |
|  |  | 2/6/2004 | 9:30 | NA | ND | NA | 0.096 | NA |
| Delt13 | Cache Slough at Real McCoy Ferry | 2/7/2004 | 9:50 | NA | (0.004 J) | NA | 0.074 | NA |
|  | continued | 2/16/2004 | 13:20 | NA | ND | NA | 0.021 | NA |
|  |  | 2/17/2004 | 15:10 | NA | ND | NA | (0.018 J) | NA |
|  |  | 2/18/2004 | 15:40 | NA | (0.004 J) | NA | (0.017 J) | NA |
|  |  | 2/19/2004 | 9:20 | NA | ND | NA | 0.037 | NA |
|  |  | 2/20/2004 | 10:10 | NA | (0.010 J) | NA | 0.038 | NA |
|  |  | 2/21/2004 | 10:50 | NA | (0.006 J) | NA | 0.039 | NA |
| Delt14 | Sacramento River at Rio Vista | 2/2/2004 | 14:10 | NA | ND | NA | 0.035 | NA |
|  |  | 2/3/2004 | 14:40 | NA | (0.006 J) | NA | 0.037 | NA |
|  |  | 2/4/2004 | 15:30 | NA | (0.006 J) | NA | 0.027 | NA |
|  |  | 2/5/2004 | 15:50 | NA | ND | NA | 0.026 | NA |
|  |  | 2/6/2004 | 9:40 | NA | ND | NA | 0.086 | NA |
|  |  | 2/7/2004 | 9:20 | NA | ND | NA | 0.066 | NA |
|  |  | 2/16/2004 | 13:50 | NA | (0.005 J) | NA | 0.024 | NA |
|  |  | 2/17/20045 | 14:30 | NA | ND | NA | 0.025 | NA |
|  |  | 2/18/2004 | 15:10 | NA | ND | NA | 0.027 | NA |
|  |  | 2/19/2004 | 9:40 | NA | (0.009 J) | NA | 0.036 | NA |
|  |  | 2/20/2004 | 9:40 | NA | (0.009 J) | NA | 0.038 | NA |
|  |  | 2/21/2004 | 10:20 | NA | (0.005 J) | NA | 0.029 | NA |

| Table 4. Summary of diazinon and chlorpyrifos concentrations quality-control data for sites in the Sacramento-San Joaquin River Delta, February 2004. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NA: not applicable - cannot be calculated because of "less than" concentration; μg/L: micrograms per liter; J: the reported concentrations were below the quantitative limit and are considered estimates; <: less than; BL: result should be viewed as biased low due to low surrogate recovery in sample; BH: result should be viewed as biased high due to high surrogate recovery in sample. |  |  |  |  |  |  |
| Site identification number | Site name | Date and time (month/day/year 24-hour time) | Chlorpyrifos (ug/L) | Relative percent difference OR percent recovery (chlorpyrifos) | Diazinon (ug/L) | Relative percent difference OR percent recovery (diazinon) |
| DUPLICATES |  |  |  |  |  |  |
| Delt02 | Mosher Slough at Mariners Drive | 02/02/2004 17:001 | BL (0.027) |  | BL (0.180) |  |
|  |  | 02/02/2004 17:05 | 0.100 | 114.96% | 0.580 | 105.26% |
| Delt03 | Five Mile Slough at Plymouth Road | 02/17/2004 09:40 | (0.006 J) |  | 0.160 |  |
|  |  | 02/17/2004 09:452 | BH (0.005 J) | 18.18% | BH (0.160) | 0.00% |
| Delt04 | Calaveras River at Ijams Road | 02/18/2004 10:50 | 0.015 |  | 0.110 |  |
|  |  | 02/18/2004 10:55 | 0.013 | 14.29% | 0.100 | 9.52% |
| Delt05 | Mid Roberts Island Drain | 02/05/2004 12:30 | (0.009 J) |  | 0.062 |  |
|  |  | 02/05/2004 12:35 | (0.007 J) | 25.00% | 0.065 | 4.72% |
| Delt06 | French Camp Slough at Airport Way | 02/05/2004 11:20 | 0.012 |  | 0.440 |  |
|  |  | 02/05/2004 11:25 | (0.010 J) | 18.18% | 0.420 | 4.65% |
| Delt08 | Old River at Tracy Boulevard | 01/22/2004 10:40 | <0.004 |  | 0.028 |  |
|  |  | 01/22/2004 10:45 | <0.004 | NA | 0.026 | 7.41% |
| Delt09 | Marsh Creek at Cypress Road | 02/04/2004 10:40 | (0.006 J) |  | 0.054 |  |
|  |  | 02/04/2004 10:45 | (0.006 J) | 0.00% | 0.059 | 8.85% |
| Delt10 | Mokelumne River at New Hope Road | 02/17/2004 15:10 | ND |  | (0.009 J) |  |
|  |  | 02/17/2004 15:15 | ND | NA | (0.011 J) | 20.00% |
| Delt14 | Sacramento River at Rio Vista | 02/21/2004 10:20 | (0.005 J) |  | 0.029 |  |
|  |  | 02/21/2004 10:25 | (0.004 J) | 22.22% | 0.036 | 21.54% |
| BLANKS |  |  |  |  |  |  |
| Delt03 | Five Mile Slough at Plymouth Road | 02/03/2004 10:15 | <0.004 |  | <0.007 |  |
| Delt06 | French Camp Slough at Airport Way | 02/19/2004 11:45 | <0.004 |  | <0.007 |  |
| Delt08 | Old River at Tracy Boulevard | 02/19/2004 12:15 | <0.004 |  | <0.007 |  |
| Delt09 | Marsh Creek at Cypress Road | 02/19/2004 10:35 | <0.004 |  | <0.007 |  |
| Delt10 | Ulatis Creek at Brown Road | 02/06/2004 13:05 | <0.004 |  | <0.007 |  |
| Delt11 | Duck Slough at Five Points Marina | 02/06/2004 08:55 | <0.004 |  | <0.007 |  |
| Delt13 | Cache Slough at Real McCoy Ferry | 02/16/2004 13:25 | <0.004 |  | <0.007 |  |
| SPIKES 3,4 |  |  |  |  |  |  |
| Delt01 | Mokelumne River at New Hope Road | 02/02/2004 17:10 | <0.004 |  | (0.012 J) |  |
|  |  | 02/02/2004 17:10 |  | 109% |  | 103% |
| Delt02 | Mosher Slough at Mariners Drive | 02/16/2004 16:30 | 0.037 |  | 0.180 |  |
|  |  | 02/16/2004 16:30 |  | 119% |  | 88% |
| Delt04 | Calaveras River at Ijams Road | 02/04/2004 11:00 | (0.006 J) |  | 0.043 |  |
|  |  | 02/04/2004 11:00 |  | 104% |  | 93% |
| Delt05 | Mid Roberts Island Drain | 01/08/2004 12:20 | <0.004 |  | <0.007 |  |
|  |  | 01/08/2004 12:20 |  | 112% |  | 104% |
| SPIKES continued |  |  |  |  |  |  |
| Delt11 | Duck Slough at Five Points Marina | 02/20/2004 10:30 | 0.190 |  | (0.017 J) |  |
|  |  | 02/20/2004 10:30 |  | 88% |  | 104% |
| Delt13 | Cache Slough at Real McCoy Ferry | 02/07/2004 09:50 | (0.004 J) |  | 0.074 |  |
|  |  | 02/07/2004 09:50 |  | 93% |  | 115% |
| Delt14 | Sacramento River at Rio Vista | 02/02/2004 14:10 | <0.004 |  | 0.035 |  |
|  |  | 02/02/2004 14:10 |  | 105% |  | 88% |
| 1 Sample had extremely low surrogate recovery of 29%. Results should be viewed as biased low. |  |  |  |  |  |  |
| 2 Sample had high surrogate recovery (135%). Results should be viewed as biased high. |  |  |  |  |  |  |
| 3 Spiked samples were injected with 0.05 ug/L of chlorpyrifos; 0.10 ug/L of diazinon. |  |  |  |  |  |  |
| 4 First sample in each pair is the environmental sample; second sample is the spike. |  |  |  |  |  |  |

## Sources Cited

Azimi-Gaylon, S., and E. Reyes. 2002. Quality Assurance Project Plan for Monitoring

Organophosphorous Pesticides in the Lower San Joaquin Basin. CVRWQCB-Sacramento, California.

Nolan, M., Frey, C. &  Jacobson, J. 2001. Surface-Water Field Techniques Training Class (Version 1.0). Water Resources Investigations Report 98-4252. U.S.G.S. Training Class – SW 4230. http://wwwrcamnl.wr.usgs.gov/sws/fieldmethods/index.html

## Acknowledgements

Monitoring water quality during the 2003-04 winter storm season required working long hours, often in adverse weather conditions.  Field staff were: Jonathan Katz, Meghan Gilbart, Brian Strand and Jeremy Ramsey from the University of California, Davis.  Their hard work and commitment was vital to collecting the data used in this report.

We would also like to thank Jaime Lu of the Central Valley Regional Water Quality Control Board for providing training, equipment and consultation throughout the project.

Thanks to Stephen Siegel and staff from the California Department of Food and Agriculture Lab for their unwavering enthusiasm and cheerfulness in processing dozens of water quality samples.

Thanks to John Tingle of the California Department of Water Resources for supplying the discharge data for French Camp Slough at Airport Way.

We would like to offer a special thank you to Jennifer Nickell of the John Muir Institute at UC Davis for her tireless efforts in processing numerous purchases, and handling all personnel matters.

Appendices

| Appendix A. Pesticide results (excluding diazinon and chlorpyrifos). (Concentrations are in units of μg/L. ND: Not detected; J: the reported concentrations were below the quantitative limit and are considered estimates; BL: result should be viewed as biased low due to low surrogate recovery in sample; BH: result should be viewed as biased high due to high surrogate recovery in sample. Each sample was also analyzed for Azinphos methyl, Cyfluthrins, Cypermethrins, Disulfoton, Propargite and l-Cyhalothrin which were not present at detectable levels). |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Site | Date | Time | Bifenthrin | Carbaryl | Cyanazine | Dacthal (DCPA) | Eptam (EPTC) | Esfen-valerate | Methidathion | Metolachlor | Simazine |
| Mokelumne River at New Hope Road | 2/2/2004 | 17:10 | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Mokelumne River at New Hope Road | 2/3/2004 | 8:00 | ND | ND | ND | ND | ND | ND | ND | ND | (0.057 J) |
| Mokelumne River at New Hope Road | 2/4/2004 | 8:10 | ND | ND | ND | ND | ND | ND | ND | ND | (0.011 J) |
| Mokelumne River at New Hope Road | 2/5/2004 | 8:00 | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Mokelumne River at New Hope Road | 2/6/2004 | 8:10 | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Mokelumne River at New Hope Road | 2/16/2004 | 12:00 | ND | ND | ND | ND | ND | ND | ND | ND | 0.520 |
| Mokelumne River at New Hope Road | 2/17/2004 | 15:10 | ND | ND | ND | ND | ND | ND | ND | ND | (0.021 J) |
| Mokelumne River at New Hope Road | 2/18/2004 | 16:40 | ND | ND | ND | ND | ND | ND | ND | ND | (0.057 J) |
| Mokelumne River at New Hope Road | 2/19/2004 | 8:20 | ND | ND | ND | ND | ND | ND | ND | ND | (0.051 J) |
| Mokelumne River at New Hope Road | 2/20/2004 | 8:10 | ND | ND | ND | ND | ND | ND | ND | ND | (0.090 J) |
| Mosher Slough at Mariners Drive | 2/2/2004 | 17:001 | ND | BL (0.053) | ND | ND | ND | ND | ND | ND | BL (0.029 J) |
| Mosher Slough at Mariners Drive | 2/3/2004 | 9:50 | ND | 0.140 | ND | (0.008 J) | ND | ND | ND | ND | 3.900 |
| Mosher Slough at Mariners Drive | 2/4/2004 | 10:10 | ND | ND | ND | (0.007 J) | ND | ND | ND | (0.007 J) | 2.600 |
| Mosher Slough at Mariners Drive | 2/5/2004 | 10:00 | ND | ND | ND | ND | ND | ND | ND | ND | 2.300 |
| Mosher Slough at Mariners Drive | 2/6/2004 | 10:30 | ND | ND | ND | ND | ND | ND | ND | ND | 2.000 |
| Mosher Slough at Mariners Drive | 2/16/2004 | 16:30 | ND | ND | ND | (0.008 J) | ND | ND | ND | ND | 0.850 |
| Mosher Slough at Mariners Drive | 2/17/2004 | 9:20 | ND | ND | ND | BH (0.010 J) | ND | ND | ND | ND | BH (4.800) |
| Mosher Slough at Mariners Drive | 2/18/2004 | 10:00 | ND | ND | ND | (0.010 J) | ND | ND | ND | ND | 0.960 |
| Mosher Slough at Mariners Drive | 2/19/2004 | 10:30 | ND | 0.054 | ND | (0.009 J) | ND | ND | ND | (0.008 J) | 0.600 |
| Mosher Slough at Mariners Drive | 2/20/2004 | 11:30 | ND | ND | ND | (0.008 J) | ND | ND | ND | (0.007 J) | 0.820 |
| Five Mile Slough at Plymouth Road | 2/2/2004 | 16:50 | ND | ND | ND | ND | ND | ND | ND | ND | (0.034 J) |
| Five Mile Slough at Plymouth Road | 2/3/2004 | 10:10 | ND | 0.032 | ND | ND | ND | ND | ND | ND | (0.047 J) |
| Five Mile Slough at Plymouth Road | 2/4/2004 | 10:20 | ND | ND | ND | ND | ND | ND | 0.053 | ND | (0.051 J) |
| Five Mile Slough at Plymouth Road | 2/5/2004 | 10:20 | ND | ND | ND | ND | ND | (0.033 J) | ND | ND | (0.052 J) |
| Five Mile Slough at Plymouth Road | 2/6/2004 | 10:50 | ND | ND | ND | ND | ND | ND | ND | ND | (0.060 J) |
| Five Mile Slough at Plymouth Road | 2/16/2004 | 16:00 | ND | ND | ND | ND | ND | ND | ND | ND | (0.079 J) |
| Five Mile Slough at Plymouth Road | 2/17/2004 | 9:40 | ND | ND | ND | ND | ND | ND | ND | ND | BH (0.077 J) |
| Five Mile Slough at Plymouth Road | 2/18/2004 | 10:20 | ND | ND | ND | (0.008 J) | ND | ND | ND | ND | (0.12 J) |
| Five Mile Slough at Plymouth Road | 2/19/2004 | 10:40 | ND | 0.097 | ND | ND | ND | ND | ND | ND | (0.13 J) |
| Five Mile Slough at Plymouth Road | 2/20/2004 | 11:40 | ND | ND | ND | ND | ND | ND | (0.016 J) | ND | (0.13 J) |
| Calaveras River at Ijams Road | 2/2/2004 | 16:10 | ND | 0.490 | ND | (0.012 J) | ND | ND | ND | ND | 0.200 |
| Calaveras River at Ijams Road | 2/3/2004 | 10:40 | ND | 0.380 | ND | (0.007 J) | ND | ND | ND | 0.027 | (0.179 J) |
| Calaveras River at Ijams Road | 2/4/2004 | 11:00 | ND | ND | ND | ND | ND | ND | ND | (0.008 J) | 0.920 |
| Calaveras River at Ijams Road | 2/5/2004 | 10:50 | ND | ND | ND | ND | ND | ND | ND | ND | 0.990 |
| Calaveras River at Ijams Road | 2/6/2004 | 11:20 | ND | ND | ND | ND | ND | ND | ND | ND | 1.000 |
| Calaveras River at Ijams Road | 2/16/2004 | 15:40 | ND | ND | ND | (0.013 J) | ND | ND | (0.011 J) | 0.026 | 0.470 |
| Calaveras River at Ijams Road | 2/17/2004 | 10:00 | ND | BH (1.300) | ND | BH (0.016 J) | ND | ND | ND | BH (0.051) | BH (0.390) |
| Calaveras River at Ijams Road | 2/18/2004 | 10:50 | ND | ND | ND | (0.018 J) | ND | ND | ND | 0.051 | 0.490 |
| Calaveras River at Ijams Road | 2/19/2004 | 11:10 | ND | ND | ND | ND | ND | ND | ND | (0.007 J) | 0.750 |
| Calaveras River at Ijams Road | 2/20/2004 | 12:00 | ND | ND | ND | ND | ND | ND | ND | ND | 0.400 |
| Mid Roberts Island Drain | 1/8/2004 | 12:20 | ND | ND | ND | ND | ND | ND | ND | 0.140 | 0.310 |
| Mid Roberts Island Drain | 1/22/2004 | 11:20 | ND | ND | ND | ND | ND | ND | ND | 0.069 | (0.091 J) |
| Mid Roberts Island Drain | 2/5/2004 | 12:30 | ND | ND | ND | ND | ND | ND | ND | 0.073 | (0.17 J) |
| Mid Roberts Island Drain | 2/19/2004 | 12:40 | ND | ND | ND | ND | ND | ND | ND | 0.180 | (0.11 J) |
| French Camp Slough at Airport Way | 2/2/2004 | 15:20 | ND | ND | ND | (0.014 J) | ND | ND | ND | ND | 0.210 |
| French Camp Slough at Airport Way | 2/3/2004 | 11:30 | ND | ND | ND | (0.015 J) | ND | ND | ND | ND | (0.160 J) |
| French Camp Slough at Airport Way | 2/4/2004 | 14:10 | ND | ND | ND | (0.011 J) | ND | ND | 0.070 | ND | 0.210 |
| French Camp Slough at Airport Way | 2/5/2004 | 11:20 | ND | ND | ND | ND | ND | ND | ND | ND | 0.350 |
| French Camp Slough at Airport Way | 2/6/2004 | 12:30 | ND | ND | ND | ND | ND | ND | ND | ND | 1.800 |
| French Camp Slough at Airport Way | 2/16/2004 | 15:10 | ND | ND | ND | (0.013 J) | ND | ND | ND | ND | 0.490 |
| French Camp Slough at Airport Way | 2/17/2004 | 10:50 | ND | ND | ND | (0.017 J) | ND | ND | ND | ND | 0.460 |
| French Camp Slough at Airport Way | 2/18/2004 | 11:20 | ND | ND | ND | (0.014 J) | ND | ND | ND | (0.007 J) | 1.200 |
| French Camp Slough at Airport Way | 2/19/2004 | 11:40 | ND | ND | ND | ND | ND | ND | ND | (0.009 J) | 1.800 |
| French Camp Slough at Airport Way | 2/20/2004 | 12:40 | ND | ND | ND | ND | ND | ND | ND | ND | 1.300 |
| Old River at Tracy Boulevard | 1/8/2004 | 11:30 | ND | ND | ND | ND | ND | ND | ND | ND | (0.095 J) |
| Old River at Tracy Boulevard | 1/22/2004 | 10:40 | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Old River at Tracy Boulevard | 2/5/2004 | 11:50 | ND | ND | ND | ND | ND | ND | ND | ND | (0.070 J) |
| Old River at Tracy Boulevard | 2/19/2004 | 12:10 | ND | ND | ND | ND | ND | ND | ND | (0.011 J) | (0.066 J) |
| Marsh Creek at Cypress Road | 2/2/2004 | 15:20 | (0.044 J) | 0.130 | ND | ND | ND | ND | ND | ND | 0.230 |
| Marsh Creek at Cypress Road | 2/3/2004 | 11:00 | (0.021 J) | 0.920 | ND | ND | ND | ND | ND | ND | (0.043 J) |
| Marsh Creek at Cypress Road | 2/4/2004 | 10:40 | (0.046 J) | 0.660 | ND | ND | ND | ND | ND | ND | (0.100 J) |
| Marsh Creek at Cypress Road | 2/5/2004 | 10:30 | ND | 2.300 | ND | ND | ND | ND | ND | ND | (0.072 J) |
| Marsh Creek at Cypress Road | 2/6/2004 | 10:40 | ND | 1.400 | ND | ND | ND | ND | 0.099 | ND | 0.300 |
| Marsh Creek at Cypress Road | 2/16/2004 | 14:40 | (0.026 J) | 1.800 | ND | ND | ND | ND | 0.085 | 0.030 | (0.130 J) |
| Marsh Creek at Cypress Road | 2/17/2004 | 12:40 | ND | 1.200 | ND | ND | ND | ND | ND | (0.009 J) | (0.032 J) |
| Marsh Creek at Cypress Road | 2/18/2004 | 12:30 | ND | 0.140 | ND | ND | ND | ND | ND | 0.037 | (0.130 J) |
| Marsh Creek at Cypress Road | 2/19/2004 | 10:30 | ND | (0.019 J) | ND | ND | ND | ND | ND | ND | 0.230 |
| Marsh Creek at Cypress Road | 2/20/2004 | 10:40 | ND | ND | ND | ND | ND | ND | ND | ND | (0.110 J) |
| Ulatis Creek at Brown Road | 2/2/2004 | 14:10 | ND | ND | ND | ND | ND | ND | ND | ND | 0.250 |
| Ulatis Creek at Brown Road | 2/3/2004 | 13:20 | ND | ND | ND | ND | ND | ND | ND | 0.021 | 4.200 |
| Ulatis Creek at Brown Road | 2/4/2004 | 13:20 | ND | ND | ND | ND | ND | ND | ND | 0.025 | 3.800 |
| Ulatis Creek at Brown Road | 2/5/2004 | 13:00 | ND | ND | ND | ND | ND | ND | ND | (0.014 J) | 1.800 |
| Ulatis Creek at Brown Road | 2/6/2004 | 13:00 | ND | ND | ND | ND | ND | ND | ND | (0.010 J) | 1.300 |
| Ulatis Creek at Brown Road | 2/16/2004 | 16:30 | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Ulatis Creek at Brown Road | 2/17/2004 | 10:20 | ND | ND | ND | ND | ND | ND | ND | (0.015 J) | 1.700 |
| Ulatis Creek at Brown Road | 2/18/2004 | 10:10 | ND | ND | ND | ND | ND | ND | ND | 0.029 | 2.500 |
| Ulatis Creek at Brown Road | 2/19/2004 | 12:50 | ND | ND | ND | ND | ND | ND | ND | 0.037 | 1.100 |
| Ulatis Creek at Brown Road | 2/20/2004 | 12:50 | ND | ND | ND | ND | ND | ND | ND | 0.021 | 0.590 |
| Duck Slough at Five Points Marina | 2/2/2004 | 13:10 | ND | ND | ND | ND | ND | ND | ND | ND | 2.100 |
| Duck Slough at Five Points Marina | 2/3/2004 | 16:10 | ND | ND | ND | ND | ND | ND | ND | ND | 0.890 |
| Duck Slough at Five Points Marina | 2/4/2004 | 16:40 | ND | ND | ND | ND | ND | ND | ND | ND | 0.290 |
| Duck Slough at Five Points Marina | 2/5/2004 | 16:50 | ND | ND | ND | ND | ND | ND | ND | 0.036 | 0.360 |
| Duck Slough at Five Points Marina | 2/6/2004 | 8:50 | ND | ND | ND | ND | ND | ND | ND | ND | 0.220 |
| Duck Slough at Five Points Marina | 2/16/2004 | 12:50 | ND | ND | ND | ND | ND | ND | ND | ND | 0.320 |
| Duck Slough at Five Points Marina | 2/17/2004 | 15:30 | ND | ND | ND | ND | ND | ND | ND | ND | (0.150 J) |
| Duck Slough at Five Points Marina | 2/18/2004 | 16:10 | ND | ND | ND | ND | ND | ND | ND | 0.033 | 2.800 |
| Duck Slough at Five Points Marina | 2/19/2004 | 8:50 | ND | ND | ND | ND | ND | ND | ND | ND | 2.600 |
| Duck Slough at Five Points Marina | 2/20/2004 | 10:30 | ND | ND | ND | ND | (0.037 J) | ND | ND | ND | 0.700 |
| Cache Slough at Real McCoy Ferry | 2/2/2004 | 13:40 | ND | ND | ND | ND | ND | ND | ND | ND | (0.045 J) |
| Cache Slough at Real McCoy Ferry | 2/3/2004 | 15:30 | ND | ND | ND | ND | ND | ND | ND | ND | (0.036 J) |
| Cache Slough at Real McCoy Ferry | 2/4/2004 | 16:10 | ND | ND | ND | ND | ND | ND | ND | ND | (0.051 J) |
| Cache Slough at Real McCoy Ferry | 2/5/2004 | 16:20 | ND | ND | ND | ND | ND | ND | ND | ND | (0.057 J) |
| Cache Slough at Real McCoy Ferry | 2/6/2004 | 9:30 | ND | ND | ND | ND | ND | ND | ND | ND | (0.072 J) |
| Cache Slough at Real McCoy Ferry | 2/7/2004 | 9:50 | ND | ND | ND | ND | ND | ND | ND | ND | (0.039 J) |
| Cache Slough at Real McCoy Ferry | 2/16/2004 | 13:20 | ND | ND | ND | ND | ND | ND | ND | ND | (0.028 J) |
| Cache Slough at Real McCoy Ferry | 2/17/2004 | 15:10 | ND | ND | ND | ND | ND | ND | ND | ND | (0.027 J) |
| Cache Slough at Real McCoy Ferry | 2/18/2004 | 15:40 | ND | ND | ND | ND | ND | ND | ND | ND | (0.039 J) |
| Cache Slough at Real McCoy Ferry | 2/19/2004 | 9:20 | ND | ND | ND | ND | ND | ND | ND | ND | 0.310 |
| Cache Slough at Real McCoy Ferry | 2/20/2004 | 10:10 | ND | ND | ND | ND | ND | ND | ND | ND | 0.260 |
| Cache Slough at Real McCoy Ferry | 2/21/2004 | 10:50 | ND | ND | ND | ND | ND | ND | ND | ND | (0.190 J) |
| Sacramento River at Rio Vista | 2/2/2004 | 14:10 | ND | ND | ND | ND | ND | ND | ND | ND | (0.045 J) |
| Sacramento River at Rio Vista | 2/3/2004 | 14:40 | ND | ND | ND | ND | ND | ND | ND | ND | (0.017 J) |
| Sacramento River at Rio Vista | 2/4/2004 | 15:30 | ND | ND | ND | ND | ND | ND | ND | ND | (0.051 J) |
| Sacramento River at Rio Vista | 2/5/2004 | 15:50 | ND | ND | ND | ND | ND | ND | ND | ND | (0.065 J) |
| Sacramento River at Rio Vista | 2/6/2004 | 9:40 | ND | ND | ND | ND | ND | ND | ND | ND | (0.076 J) |
| Sacramento River at Rio Vista | 2/7/2004 | 9:20 | ND | ND | ND | ND | ND | ND | ND | ND | (0.038 J) |
| Sacramento River at Rio Vista | 2/16/2004 | 13:50 | ND | ND | ND | ND | ND | ND | ND | ND | (0.037 J) |
| Sacramento River at Rio Vista | 2/17/2004 | 14:30 | ND | ND | ND | ND | ND | ND | ND | ND | BH (0.041 J) |
| Sacramento River at Rio Vista | 2/18/2004 | 15:10 | ND | ND | ND | ND | ND | ND | ND | ND | (0.085 J) |
| Sacramento River at Rio Vista | 2/19/2004 | 9:40 | ND | ND | ND | ND | ND | ND | ND | ND | 0.310 |
| Sacramento River at Rio Vista | 2/20/2004 | 9:40 | ND | ND | ND | ND | ND | ND | ND | ND | 0.280 |
| Sacramento River at Rio Vista | 2/21/2004 | 10:20 | ND | ND | ND | ND | ND | ND | ND | ND | (0.180 J) |

Appendix B.  Lab blank data

(No pesticides were present at detectable levels. The pesticides include azinphos methyl, bifenthrin, carbaryl, chlorpyrifos, cyanazine, cyfluthrins, cypermethrins, dacthal (DCPA), diazinon, disulfoton, EPTC (Eptam), esfenvalerate, methidathion, metolachlor, propargite, l-Cyhalothrin and simazine)

| Date Extracted | Chlorpyrifos Methyl (Surrogate) Recovery |
| --- | --- |
| 1/9/2004 | 108% |
| 1/23/2004 | 113% |
| 2/4/2004 | 103% |
| 2/4/2004 | 106% |
| 2/5/2004 | 98% |
| 2/6/2004 | 79% |
| 2/9/2004 | 94% |
| 2/10/2004 | 107% |
| 2/10/2004 | 122% |
| 2/17/2004 | 101% |
| 2/18/2004 | 84% |
| 2/19/2004 | 100% |
| 2/20/2004 | 97% |
| 2/23/2004 | 118% |
| 2/24/2004 | 104% |
| 2/25/2004 | 118% |
| 2/26/2004 | 97% |

Appendix C.  Recovery rates in lab control matrix spikes

| Date Extracted | Bifenthrin | Chlorpyrifos | Diazinon | Surrogate |
| --- | --- | --- | --- | --- |
| 1/9/2004 | 109% | 103% | 101% | 114% |
| 1/23/2004 | 91% | 106% | 98% | 112% |
| 2/4/2004 | 99% | 109% | 89% | 107% |
| 2/4/2004 | 93% | 106% | 100% | 113% |
| 2/5/2004 | 85% | 82% | 90% | 103% |
| 2/6/2004 | 104% | 84% | 90% | 98% |
| 2/9/2004 | 97% | 103% | 95% | 100% |
| 2/10/2004 | 117% | 92% | 110% | 118% |
| 2/10/2004 | 128% | 102% | 119% | 124% |
| 2/17/2004 | 116% | 102% | 97% | 108% |
| 2/18/2004 | 122% | 86% | 87% | 92% |
| 2/19/2004 | 102% | 89% | 91% | 106% |
| 2/20/2004 | 101% | 106% | 93% | 114% |
| 2/23/2004 | 118% | 101% | 106% | 108% |
| 2/24/2004 | 120% | 102% | 100% | 112% |
| 2/25/2004 | 111% | 99% | 98% | 114% |
| 2/26/2004 | 122% | 105% | 100% | 114% |

Float-method was used to estimate discharge.  See explanation in Discharge Methods section.

2 Surrogate recovery (29%) was outside of QAPP acceptance limits. Results should be viewed as biased low.

3 Surrogate recovery (133%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

4 Surrogate recovery (136%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

5 Surrogate recovery (133%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

1 Surrogate recovery (29%) was outside of QAPP acceptance limits. Results should be viewed as biased low.

Surrogate recovery (133%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

Surrogate recovery (136%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

Surrogate recovery (133%) was outside of QAPP acceptance limits. Results should be viewed as biased high.