A Summary of the 2004 TMDL Monitoring for Selected Pesticides in the Sacramento-San Joaquin Delta, California

March - April 2004

Henry J. Calanchini

Michael L. Johnson

John Muir Institute of the Environment

University of California, Davis

October 2005

Contents

Introduction	3

Objective	3

Monitoring Overview	3

Sample Collection Methods	5

Discharge Methods	5

Loading Rate Calculations	6

Laboratory Analysis Methods	6

Quality Assurance Objectives	7

Results	8

Sources Cited	16

Acknowledgements	16

Figures

Figure 1. The twelve sampling sites in the Sacramento-San Joaquin delta monitored for pesticides                                                  during the 2004 spring irrigation season.	4

Tables

Table 1. Sample sites, collection methods and sampling dates	5

Table 2.  CDFA Laboratory limits of detection and practical quantitation limits for select pesticides	7

Table 3.  Summary of environmental data collected on diazinon and chlorpyrifos concentrations and instantaneous loading rates for sites in the Sacramento-San Joaquin River Delta, California.  March - April 2004.	10

Table 4. Summary of diazinon and chlorpyrifos concentrations quality-control data for sites in the Sacramento-San Joaquin River Delta, California, March - April 2004.	15

Appendix A.  Pesticide results (excluding diazinon and chlorpyrifos).	18

Appendix B.  Lab blank data	23

Appendix C.  Recovery rates in lab control matrix spikes	23

## Introduction

This report describes the results of pesticide monitoring at twelve locations in California’s Sacramento-San Joaquin Delta during March and April 2004. Monitoring was conducted by staff of the Aquatic Ecosystems Analysis Laboratory (AEAL) of the John Muir Institute of the Environment, University of California, Davis, as authorized under Contract No. 02-210-150 from the Central Valley Regional Water Quality Control Board (CVRWQCB).

## Objective

The primary objective of this project was to monitor twelve sites in the Sacramento-San Joaquin River Delta during the 2004 spring irrigation season to characterize the sources of diazinon, chlorpyrifos and other pesticides that can cause surface water contamination and toxic conditions to aquatic life. The results of this study will be used to support the development of diazinon and chlorpyrifos TMDLs in the Sacramento-San Joaquin delta.

## Monitoring Overview

Five sites (Figure 1, Table 1) were monitored once per week from 3 March to 28 April 2004 for a total of nine times each.  Seven sites were monitored once per week from 3 March to 7 April 2004 for a total of six times each.

The measured field parameters included pH, water temperature and electrical conductivity (EC). Stream discharge was measured at three sites (Delt01, Delt09 and Delt10) using standard USGS methods and a Swoffer Model 2100 current meter.  Discharge estimates at one site (Delt06) were obtained from the California Department of Water Resources (CDPR).

Water samples were delivered to the California Department of Food and Agriculture (CDFA) laboratory in Sacramento, California for chemical analysis using gas chromatography (GC) and mass spectrometry (MS). The CDFA laboratory analyzed 17 chemical compounds for each water sample. The list of compounds is provided in Table 2.  The detection frequencies, concentrations and calculated instantaneous loading rates for diazinon and chlorpyrifos are presented in Table 3.  The detection frequencies and concentrations of the other 15 compounds are listed in Appendix A.  The analytical results for all tested compounds, and the physical parameters measured in the field are presented in tabular format on a compact disc appended to this report.

Figure 1. The twelve sampling sites in the Sacramento-San Joaquin delta monitored for pesticides                                                  during the 2004 spring irrigation season.

| Table 1. Sample sites, collection methods and sampling dates |  |  |  |
| --- | --- | --- | --- |
| Site # | Site Name | Sample collection Method | Sampling Dates (every 7 days) |
| Delt01 | Mokelumne River at New Hope Road | Integrated grab from bridge | 3 March through 7 April 2004 |
| Delt02 | Mosher Slough at Mariners Drive | Grab from bank | 3 March through 28 April 2004 |
| Delt03 | Five Mile Slough at Plymouth Road | Grab from bank | 3 March through 28 April 2004 |
| Delt04 | Calaveras River at Ijams Road | Grab from bank | 3 March through 7 April 2004 |
| Delt05 | Mid Roberts Island Drain | Grab from bank | 3 March through 28 April 2004 |
| Delt06 | French Camp Slough at Airport Way | Grab from bank | 3 March through 28 April 2004 |
| Delt08 | Old River at Tracy Road | Grab from bank | 3 March through 7 April 2004 |
| Delt09 | Marsh Creek at Cypress Road | Grab from bank | 3 March through 7 April 2004 |
| Delt10 | Ulatis Creek at Brown Road | Integrated grab from bridge/Grab from bank | 3 March through 28 April 2004 |
| Delt11 | Duck Slough at Five Points Marina | Grab from bank | 3 March through 7 April 2004 |
| Delt13 | Cache Slough at Real McCoy Ferry | Grab from bank | 3 March through 7 April 2004 |
| Delt14 | Sacramento River at Rio Vista | Integrated grab from pier | 3 March through 7 April 2004 |

## Sample Collection Methods

All samples were collected by either grab or integrated grab methods (Table 1).

Grab samples were collected by harnessing a 1-liter amber glass bottle to a pole sampler and dipping the bottle into the stream as close to the center of the channel as possible.

Integrated grab samples were collected by lowering a 3-liter PTFE (polytetrafluoroethylene) bottle, strapped in a weighted cage, from a bridge at three equally spaced verticals.  At each vertical the bottle was filled approximately ¼ full.  The composite sample was then thoroughly agitated and poured into a 1-liter amber glass sample bottle.

## Discharge Methods

At Marsh Creek and Ulatis Creek discharge was measured using a Swoffer Model 2100 current meter while wading.  At the Mokelumne River, and at Ulatis Creek during high flows, discharge was measured from a bridge using a bridgeboard, sounding reel and Swoffer Model 2100 current meter.  All measurements were made using standard USGS current-meter methods (Nolan, et al. 2001).  Discharge estimates for French Camp Slough at Airport Way were provided courtesy of John Tingle of the California Department of Water Resources (CDPR) from the CDPR gage located on site. No discharge measurements were made at any of the other sites due to tidal influences, safety considerations and site logistics.

## Loading Rate Calculations

Instantaneous loading rates of diazinon and chlorpyrifos were calculated by multiplying the stream discharge at the time of sample collection by the measured concentrations of each pesticide by the number of seconds (86,400) in one day.  Loading rates were only calculated when the pesticide concentration was above the limit of detection and a discharge estimate was available.  The loading rate was assumed to be zero for all samples where pesticide concentrations were below the limit of detection.

## Laboratory Analysis Methods

Upon arrival at the CDFA laboratory, the environmental samples were weighed then spiked with 500µL of 1.0 µg/ml chlorpyrifos methyl (0.5µg/mL) surrogate spiking solution. Each sample was emptied into a 2-liter separatory funnel and approximately 10-15g of granular sodium chloride was added.  Sixty ml of methylene chloride were added and the sample was then mixed for three minutes.  The organic fraction was filtered through a bed of granular anhydrous sodium sulfate (approx. 20g).  The extraction process was repeated three times and the resultant sample was evaporated to 5-7 ml at 40° C and then evaporated to dryness with an N-evaporator.   One ml of methylene chloride and 10µL of a 5.0µg/mL internal standard solution were added to each sample. Samples were stored in a –5ºC freezer until analysis.  Samples were analyzed with an Agilent Model 5973 GC-MSD using a HP-5MS or equivalent GC column.  Analysis was performed in the selective ion-monitoring mode.

Each sample was analyzed for seventeen compounds.  The compounds and their respective limits of quantitation (LOQ) and limits of detection (LOD) are listed in Table 2. The lab reported estimated values when the values were below the LOQ but above the LOD. To ensure the accuracy and precision of the sample analysis, lab spikes, lab blanks, and a surrogate standard (chlorpyrifos methyl) were used. If the recovery of a spike sample was out of the control range, the water sample was re-analyzed.

Table 2.  CDFA Laboratory limits of detection and practical quantitation limits for select pesticides

| Compound | Limit of Detection (LOD in µg/L) | Limit of Quantitation (LOQ in µg/L) |
| --- | --- | --- |
| Azinphos methyl | 0.007 | 0.050 |
| Bifenthrin | 0.007 | 0.050 |
| Carbaryl | 0.007 | 0.020 |
| Chlorpyrifos | 0.004 | 0.010 |
| Cyanazine | 0.007 | 0.050 |
| Cyfluthrins | 0.070 | 0.200 |
| Cypermethrins | 0.070 | 0.200 |
| Dacthal (DCPA) | 0.007 | 0.050 |
| Diazinon | 0.007 | 0.020 |
| Disulfoton | 0.007 | 0.020 |
| EPTC (Eptam) | 0.020 | 0.050 |
| Esfenvalerate | 0.007 | 0.050 |
| l-Cyhalothrin | 0.030 | 0.100 |
| Methidathion | 0.010 | 0.030 |
| Metolachlor | 0.007 | 0.020 |
| Propargite | 0.150 | 0.500 |
| Simazine | 0.005 | 0.200 |

## Quality Assurance Objectives

Sampling during the 2003-04 winter storm season was conducted under the guidance of a draft Quality Assurance Project Plan (QAPP) (San Joaquin River TMDL Quality Assurance Project Plan Azimi-Gaylon and Reyes, 2002).

Sampling precision and variability were measured through the use of field duplicates.  The draft QAPP stated the Quality Assurance Objective (QAO) for precision was a relative percent difference (RPD) of less than 50%.  No QAO was stated for accuracy in the draft QAPP.  Accuracy was measured by determining the percent recovery of known concentrations of analytes spiked into environmental samples or reagent water prior to extraction.  A 70-130% recovery rate is commonly viewed as acceptable  (D. McClure, pers. comm.) and, for the purpose of this report, was used as the QAO for accuracy in laboratory analytical measurements.

When reporting analytical results it is customary to flag those results that fall outside of the acceptable level of recovery as stated in the QAOs.   For the purpose of this report all results outside of the 70-130% recovery range are flagged as follows:  BH = results should be viewed as biased high due to high surrogate recovery in sample.  No samples had recoveries below the acceptable range.

## Results

A total of 87 environmental samples (Table 3) and 11 quality control (QC) samples (Table 4) were collected and analyzed.

Environmental samples

Concentrations of diazinon and chlorpyrifos ranged from below detection to 0.087 parts per billion (ppb) of diazinon in Mosher Slough on 3 March and 0.24 parts per billion (ppb) of chlorpyrifos in Mid Roberts Island Drain on 17 March  (Table 3).

The highest and lowest calculated instantaneous loading rates for diazinon were in Ulatis Creek and Marsh Creek, respectively.  The highest and lowest calculated instantaneous loading rates for chlorpyrifos were in Ulatis Creek (Table 3).

Other pesticides detected in the environmental samples were Carbaryl, Cyanazine, Dacthal (DCPA), Eptam (EPTC), Metolachlor and Simazine (Appendix A).

Seventeen environmental samples in which at least one pesticide was detected had surrogate recoveries outside of the QAO acceptance limits – see footnotes in Table 3 and Appendix A.

Environmental Quality Control Samples

Sample quality control was measured through collection of sequential and split duplicates (n=1), field blanks (n=6) and matrix spikes (n=4).  Duplicate samples provided a measure of analytical precision; field blanks were used to evaluate possible introduction of contaminants during sample collection, handling and transport to the lab; and matrix spikes were used to evaluate the relative percent recovery of spiked chemicals through extraction from the sample matrix.

The procedures used for collecting the QA/QC samples were based on the draft San Joaquin River TMDL Quality Assurance Project Plan (Azimi-Gaylon and Reyes, 2002).

The QAO for duplicate samples was a relative percent difference (RPD) of < 50% between the duplicate and the corresponding environmental sample concentrations.  No pesticides were detected in the single duplicate sample and corresponding environmental sample (Table 4).

The QAO acceptance limit for field blanks was “less than the reporting limit”. One field blank poured at the Duck Slough site on 17 March had a detected concentration of 0.18 parts per billion (ppb) of the herbicide Simazine.  The other five field blanks met the acceptance limits with no detections of any pesticides (Table 4).

The QAO acceptance limit for matrix spikes was a 70-130% recovery rate for both chlorpyrifos and diazinon.  Each of the four matrix spikes met the QAO objective for recovery. Recoveries ranged from 87-118% for chlorpyrifos and 91-107% for diazinon (Table 4).

No environmental quality control samples in which at least one pesticide was detected had surrogate recoveries outside of the QAO acceptance limits.

Laboratory Quality Control Samples

Fifteen lab blanks and 15 lab control matrix spikes were analyzed with the environmental samples.  The QAO acceptance limits for lab blanks and lab control matrix spikes were recoveries of 70-130% each for bifenthrin, chlorpyrifos, diazinon and the surrogate (chlorpyrifos methyl).  All laboratory quality control samples met the quality assurance objectives.

Recoveries in lab blanks ranged from 79-122% (Appendix B).  Recoveries in lab control matrix spikes ranged from 72-123%, 86-113%, 86-110% and 80-130% for bifenthrin, chlorpyrifos, diazinon and chlorpyrifos methyl, respectively (Appendix C).

| Table 3. Summary of environmental data collected on diazinon and chlorpyrifos concentrations and instantaneous loading rates for sites in the Sacramento-San Joaquin River Delta, California. March - April 2004. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream flow is in cubic feet per second. J: the reported concentrations were below the quantitative limit and are considered estimates; NA: not available; ND: Not detected; g a.i./d: grams active ingredient per day; μg/L: micrograms per liter; BH: result should be viewed as biased high due to high surrogate recovery in sample. |  |  |  |  |  |  |  |  |
| Site number | Site name | Date (month/day/year) | Time (24 hr) | Stream flow (cfs) | Chlorpyrifos concentration (μg/L) | Chlorpyrifos instantaneous loading rate (g a.i./d) | Diazinon concentration (μg/L) | Diazinon instantaneous loading rate (g a.i./d) |
| Delt01 | Mokelumne River at New Hope Road | 3/04/2004 | 08:30 | 220.36 | ND | NA | ND | NA |
|  |  | 3/11/2004 | 09:20 | 215.77 | ND | NA | ND | NA |
|  |  | 3/18/2004 | 09:30 | 264.15 | ND | NA | ND | NA |
|  |  | 3/25/2004 | 09:30 | 206.24 | ND | NA | ND | NA |
|  |  | 4/01/2004 | 09:20 | 236.26 | ND | NA | ND | NA |
|  |  | 4/08/2004 | 10:20 | 205.88 | ND | NA | ND | NA |
| Delt02 | Mosher Slough at Mariners Drive | 3/03/2004 | 09:40 | NA | ND | NA | 0.087 | NA |
|  |  | 3/10/2004 | 11:30 | NA | ND | NA | 0.039 | NA |
|  |  | 3/17/2004 | 07:50 | NA | 0.025 | NA | 0.029 | NA |
|  |  | 3/24/2004 | 17:30 | NA | ND | NA | (0.018 J) | NA |
|  |  | 3/31/2004 | 15:20 | NA | BH (0.011) | NA | BH (0.037) | NA |
|  |  | 4/07/2004 | 10:00 | NA | (0.008 J) | NA | (0.018 J) | NA |
|  |  | 4/14/2004 | 08:20 | NA | BH (0.007 J) | NA | BH (0.020 J) | NA |
|  |  | 4/21/2004 | 09:10 | NA | (0.006 J) | NA | 0.046 | NA |
|  |  | 4/28/2004 | 08:00 | NA | ND | NA | (0.013 J) | NA |
| Delt03 | Five Mile Slough at Plymouth Road | 3/03/2004 | 10:00 | NA | ND | NA | 0.021 | NA |
|  |  | 3/10/2004 | 11:40 | NA | ND | NA | 0.036 | NA |
|  |  | 3/17/2004 | 08:20 | NA | 0.022 | NA | 0.027 | NA |
|  |  | 3/24/2004 | 17:10 | NA | ND | NA | (0.017 J) | NA |
|  |  | 3/31/2004 | 15:30 | NA | ND | NA | BH (0.014 J) | NA |
| Delt03 | Five Mile Slough at Plymouth Road | 4/07/2004 | 10:20 | NA | ND | NA | (0.011 J) | NA |
|  | continued | 4/14/2004 | 08:50 | NA | ND | NA | ND | NA |
|  |  | 4/21/2004 | 09:40 | NA | ND | NA | ND | NA |
|  |  | 4/28/2004 | 08:20 | NA | ND | NA | ND | NA |
| Delt04 | Calaveras River at Ijams Road | 3/03/2004 | 10:20 | NA | ND | NA | ND | NA |
|  |  | 3/10/2004 | 12:10 | NA | 0.017 | NA | 0.080 | NA |
|  |  | 3/17/2004 | 09:00 | NA | 0.013 | NA | (0.017 J) | NA |
|  |  | 3/24/2004 | 16:40 | NA | ND | NA | ND | NA |
|  |  | 3/31/2004 | 16:00 | NA | ND | NA | BH (0.074) | NA |
|  |  | 4/07/2004 | 10:50 | NA | NA | NA | NA | NA |
| Delt05 | Mid Roberts Island Drain | 3/03/2004 | 12:10 | NA | ND | NA | ND | NA |
|  |  | 3/10/2004 | 14:40 | NA | ND | NA | ND | NA |
|  |  | 3/17/2004 | 09:40 | NA | 0.240 | NA | ND | NA |
|  |  | 3/24/2004 | 15:40 | NA | 0.039 | NA | ND | NA |
|  |  | 3/31/2004 | 18:20 | NA | BH (0.023) | NA | ND | NA |
|  |  | 4/07/2004 | 11:50 | NA | (0.009 J) | NA | ND | NA |
|  |  | 4/14/2004 | 09:20 | NA | BH (0.180) | NA | BH (0.082) | NA |
|  |  | 4/21/2004 | 10:10 | NA | 0.017 | NA | ND | NA |
|  |  | 4/28/2004 | 09:30 | NA | 0.030 | NA | ND | NA |
| Delt06 | French Camp Slough at Airport Way | 3/03/2004 | 11:10 | 575 | ND | NA | (0.008 J) | 11.25 |
| Delt06 | French Camp Slough at Airport Way | 3/10/2004 | 13:40 | 40.30 | ND | NA | ND | NA |
|  | continued | 3/17/2004 | 11:20 | 27.00 | 0.230 | 15.19 | (0.015 J) | 0.99 |
|  |  | 3/24/2004 | 15:10 | 43.00 | 0.022 | 2.31 | ND | NA |
|  |  | 3/31/2004 | 17:10 | 33.80 | ND | NA | (0.013 J) | 1.07 |
|  |  | 4/07/2004 | 13:00 | 60.00 | (0.006 J) | 0.88 | ND | NA |
|  |  | 4/14/2004 | 10:00 | 43.00 | BH (0.014) | 1.47 | ND | NA |
|  |  | 4/21/2004 | 10:50 | 14.70 | 0.170 | 6.11 | ND | NA |
|  |  | 4/28/2004 | 10:00 | 41.40 | 0.016 | 1.62 | ND | NA |
| Delt08 | Old River at Tracy Boulevard | 3/03/2004 | 11:40 | NA | (0.006 J) | NA | (0.016 J) | NA |
|  |  | 3/10/2004 | 14:10 | NA | (0.008 J) | NA | (0.010 J) | NA |
|  |  | 3/17/2004 | 10:50 | NA | 0.023 | NA | ND | NA |
|  |  | 3/24/2004 | 14:30 | NA | ND | NA | ND | NA |
|  |  | 3/31/2004 | 17:40 | NA | ND | NA | ND | NA |
|  |  | 4/07/2004 | 12:20 | NA | ND | NA | ND | NA |
| Delt09 | Marsh Creek | 3/04/2004 | 11:20 | 21.90 | ND | NA | ND | NA |
|  |  | 3/11/2004 | 12:20 | 8.83 | ND | NA | ND | NA |
|  |  | 3/18/2004 | 12:50 | 7.42 | ND | NA | (0.018 J) | 0.33 |
|  |  | 3/25/2004 | 12:50 | 5.30 | ND | NA | (0.008 J) | 0.10 |
|  |  | 4/01/2004 | 11:40 | 9.18 | ND | NA | BH (0.011 J) | 0.25 |
|  |  | 4/08/2004 | 12:50 | 9.89 | ND | NA | (0.011 J) | 0.27 |
| Delt10 | Ulatis Creek at Brown Rd | 3/04/2004 | 13:50 | 441.08 | 0.042 | 45.32 | (0.012 J) | 12.95 |
|  |  | 3/11/2004 | 14:10 | 66.74 | 0.011 | 1.80 | ND | NA |
|  |  | 3/18/2004 | 14:50 | 54.38 | (0.006 J) | 0.80 | ND | NA |
|  |  | 3/25/2004 | 14:40 | 59.68 | ND | NA | BH (0.009 J) | 1.31 |
|  |  | 4/01/2004 | 13:50 | 62.15 | ND | NA | ND | NA |
|  |  | 4/08/2004 | 14:40 | 8.48 | ND | NA | (0.009 J) | NA |
|  |  | 4/14/2004 | 12:40 | 18.01 | BH (0.020) | 0.88 | ND | NA |
|  |  | 4/21/2004 | 13:30 | 37.08 | BH (0.018) | 1.63 | ND | NA |
|  |  | 4/28/2004 | 11:40 | 26.49 | ND | NA | ND | NA |
|  |  |  |  |  |  | NA |  |  |
| Delt11 | Duck Slough at Five Points Marina | 3/03/2004 | 15:30 | NA | 0.086 | NA | ND | NA |
|  |  | 3/10/2004 | 09:40 | NA | 0.030 | NA | ND | NA |
|  |  | 3/17/2004 | 15:40 | NA | 0.025 | NA | ND | NA |
|  |  | 3/24/2004 | 20:10 | NA | ND | NA | ND | NA |
|  |  | 3/31/2004 | 12:30 | NA | ND | NA | ND | NA |
|  |  | 4/07/2004 | 08:30 | NA | ND | NA | ND | NA |
| Delt13 | Cache Slough at Real McCoy Ferry | 3/03/2004 | 15:00 | NA | ND | NA | (0.008 J) | NA |
|  |  | 3/10/2004 | 10:10 | NA | ND | NA | ND | NA |
|  |  | 3/17/2004 | 15:10 | NA | ND | NA | ND | NA |
|  |  | 3/24/2004 | 18:20 | NA | ND | NA | ND | NA |
|  |  | 3/31/2004 | 13:00 | NA | ND | NA | ND | NA |
|  |  | 4/07/2004 | 08:50 | NA | ND | NA | ND | NA |
| Delt14 | Sacramento River at Rio Vista | 3/03/2004 | 14:30 | NA | ND | NA | ND | NA |
|  |  | 3/10/2004 | 10:30 | NA | ND | NA | ND | NA |
|  |  | 3/17/2004 | 14:30 | NA | ND | NA | (0.008 J) | NA |
|  |  | 3/24/2004 | 18:40 | NA | ND | NA | ND | NA |
|  |  | 3/31/2004 | 13:20 | NA | ND | NA | ND | NA |
|  |  | 4/07/2004 | 09:20 | NA | ND | NA | ND | NA |

| Table 4. Summary of diazinon and chlorpyrifos concentrations quality-control data for sites in the Sacramento-San Joaquin River Delta, California, March - April 2004. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NA: not applicable - cannot be calculated because of "less than" concentration; μg/L: micrograms per liter; J: the reported concentrations were below the quantitative limit and are considered estimates; <: less than |  |  |  |  |  |  |
| Site identification number | Site name | Date and time (month/day/year 24-hour time) | Chlorpyrifos (ug/L) | Relative percent difference OR percent recovery (chlorpyrifos) | Diazinon (ug/L) | Relative percent difference OR percent recovery (diazinon) |
| DUPLICATES |  |  |  |  |  |  |
| Delt13 | Cache Slough at Real McCoy Ferry | 3/24/2004 18:20 | <0.004 |  | <0.007 |  |
|  |  | 3/24/2004 18:25 | <0.004 | NA | <0.007 | NA |
| BLANKS |  |  |  |  |  |  |
| Delt01 | Mokelumne River at New Hope Road | 3/18/2004 09:35 | <0.004 |  | <0.007 |  |
| Delt04 | Calaveras River at Ijams Road | 3/03/2004 10:25 | <0.004 |  | <0.007 |  |
| Delt05 | Mid Roberts Island Drain | 4/07/2004 11:55 | <0.004 |  | <0.007 |  |
| Delt06 | French Camp Slough at Airport Way | 4/28/2004 10:05 | <0.004 |  | <0.007 |  |
| Delt11 | Duck Slough at Five Points Marina | 3/17/2004 15:45 | <0.004 |  | <0.007 |  |
| Delt14 | Sacramento River at Rio Vista | 3/31/2004 13:25 | <0.004 |  | <0.007 |  |
| SPIKES 2,3 |  |  |  |  |  |  |
| Delt03 | Five-mile slough at Plymouth Rd | 4/14/2004 08:50 | <0.004 |  | <0.007 |  |
|  |  | 4/14/2004 08:50 |  | 87% |  | 91% |
| Delt06 | French Camp Slough at Airport Way | 3/10/2004 13:40 | <0.004 |  | <0.007 |  |
|  |  | 3/10/2004 13:40 |  | 101% |  | 98% |
| Delt09 | Marsh Creek | 4/01/2004 11:40 | <0.004 |  | (0.011 J) |  |
|  |  | 4/01/2004 11:40 |  | 118% |  | 107% |
| Delt10 | Ulatis Creek at Brown Rd | 3/04/2004 13:50 | 0.042 |  | (0.012 J) |  |
|  |  | 3/04/2004 13:50 |  | 102% |  | 104% |

## Sources Cited

Azimi-Gaylon, S., and E. Reyes. 2002. Quality Assurance Project Plan for Monitoring

Organophosphorous Pesticides in the Lower San Joaquin Basin. CVRWQCB-Sacramento, California.

Nolan, M., Frey, C. &  Jacobson, J. 2001. Surface-Water Field Techniques Training Class (Version 1.0). Water Resources Investigations Report 98-4252. U.S.G.S. Training Class – SW 4230. http://wwwrcamnl.wr.usgs.gov/sws/fieldmethods/index.html

## Acknowledgements

Monitoring water quality during the 2004 spring irrigation season was performed by Jonathan Katz, Meghan Gilbart, Brian Strand and Jeremy Ramsey from the University of California, Davis.  Their hard work and commitment was vital to collecting the data used in this report.

We would also like to thank Jaime Lu of the Central Valley Regional Water Quality Control Board for providing training, equipment and consultation throughout the project.

Thanks to Stephen Siegel and staff from the California Department of Food and Agriculture Lab for their unwavering enthusiasm and cheerfulness in processing dozens of water quality samples.

Thanks to John Tingle of the California Department of Water Resources for supplying the discharge data for French Camp Slough at Airport Way.

We would like to offer a special thank you to Jennifer Nickell of the John Muir Institute at UC Davis for her tireless efforts in processing numerous purchases, and handling all personnel matters.

Appendix A

| Appendix A. Pesticide results (excluding diazinon and chlorpyrifos). (Concentrations are in units of μg/L. NA: Not available; ND: Not detected; J: the reported concentrations were below the quantitative limit and are considered estimates; BH: result should be viewed as biased high due to high surrogate recovery in sample. Each sample was also analyzed for Azinphos methyl, Bifenthrin, Cyfluthrins, Cypermethrins, Disulfoton, Esfenvalerate, Methidathion, Propargite and l-Cyhalothrin which were not present at detectable levels). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Site | Date | Time | Carbaryl | Cyanazine | Dacthal (DCPA) | Eptam (EPTC) | Metolachlor | Simazine |
| Mokelumne River at New Hope Road | 3/04/2004 | 08:30 | ND | ND | ND | ND | ND | (0.120 J) |
| Mokelumne River at New Hope Road | 3/11/2004 | 09:20 | ND | ND | ND | ND | ND | (0.044 J) |
| Mokelumne River at New Hope Road | 3/18/2004 | 09:30 | ND | ND | ND | ND | ND | (0.038 J) |
| Mokelumne River at New Hope Road | 3/25/2004 | 09:30 | ND | ND | ND | ND | ND | (0.031 J) |
| Mokelumne River at New Hope Road | 4/01/2004 | 09:20 | ND | ND | ND | ND | ND | (0.037 J) |
| Mokelumne River at New Hope Road | 4/08/2004 | 10:20 | ND | ND | ND | ND | ND | (0.030 J) |
| Mosher Slough at Mariners Drive | 3/03/2004 | 09:40 | ND | ND | (0.008 J) | ND | ND | 1.000 |
| Mosher Slough at Mariners Drive | 3/10/2004 | 11:30 | 0.069 | ND | ND | ND | ND | 0.810 |
| Mosher Slough at Mariners Drive | 3/17/2004 | 07:50 | 0.056 | ND | ND | ND | (0.007 J) | 0.760 |
| Mosher Slough at Mariners Drive | 3/24/2004 | 17:30 | 0.570 | ND | ND | ND | ND | 0.490 |
| Mosher Slough at Mariners Drive | 3/31/20041 | 15:20 | BH (0.053) | ND | ND | ND | BH (0.010 J) | BH (0.360) |
| Mosher Slough at Mariners Drive | 4/07/2004 | 10:00 | (0.017 J) | ND | ND | ND | ND | 0.280 |
| Mosher Slough at Mariners Drive | 4/14/20042 | 08:20 | BH (0.067) | ND | ND | ND | BH (0.019 J) | BH (0.210) |
| Mosher Slough at Mariners Drive | 4/21/2004 | 09:10 | 0.069 | ND | ND | ND | ND | (0.065 J) |
| Mosher Slough at Mariners Drive | 4/28/2004 | 08:00 | 0.074 | ND | ND | ND | (0.012 J) | (0.077 J) |
| Five Mile Slough at Plymouth Road | 3/03/2004 | 10:00 | ND | ND | ND | ND | ND | (0.180 J) |
| Five Mile Slough at Plymouth Road | 3/10/2004 | 11:40 | ND | ND | ND | ND | (0.008 J) | 0.540 |
| Five Mile Slough at Plymouth Road | 3/17/2004 | 08:20 | ND | ND | ND | ND | (0.008 J) | 0.610 |
| Five Mile Slough at Plymouth Road | 3/24/2004 | 17:10 | ND | ND | ND | ND | ND | 0.530 |
| Five Mile Slough at Plymouth Road | 3/31/20043 | 15:30 | ND | ND | ND | ND | BH (0.015 J) | BH (0.500) |
| Five Mile Slough at Plymouth Road | 4/07/2004 | 10:20 | ND | ND | ND | ND | (0.012 J) | 0.560 |
| Five Mile Slough at Plymouth Road | 4/14/20044 | 08:50 | ND | ND | ND | ND | BH (0.021) | BH (0.340) |
| Five Mile Slough at Plymouth Road | 4/21/2004 | 09:40 | ND | ND | ND | ND | (0.018 J) | 0.220 |
| Five Mile Slough at Plymouth Road | 4/28/2004 | 08:20 | ND | ND | ND | ND | 0.032 | (0.130 J) |
| Calaveras River at Ijams Road | 3/03/2004 | 10:20 | ND | ND | ND | ND | ND | 0.270 |
| Calaveras River at Ijams Road | 3/10/2004 | 12:10 | 0.034 | ND | ND | ND | ND | 0.370 |
| Calaveras River at Ijams Road | 3/17/2004 | 09:00 | ND | ND | ND | ND | ND | 0.710 |
| Calaveras River at Ijams Road | 3/24/2004 | 16:40 | ND | ND | ND | ND | ND | 0.660 |
| Calaveras River at Ijams Road | 3/31/20045 | 16:00 | ND | ND | BH (0.008 J) | ND | BH (0.045) | BH (0.590) |
| Calaveras River at Ijams Road | 4/07/20046 | 10:50 | NA | NA | NA | NA | NA | NA |
| Mid Roberts Island Drain | 3/03/2004 | 12:10 | ND | ND | ND | ND | 0.120 | 0.270 |
| Mid Roberts Island Drain | 3/10/2004 | 14:40 | ND | ND | ND | ND | 0.066 | (0.120 J) |
| Mid Roberts Island Drain | 3/17/2004 | 09:40 | ND | ND | ND | ND | 0.063 | (0.140 J) |
| Mid Roberts Island Drain | 3/24/2004 | 15:40 | ND | ND | ND | ND | 0.360 | 0.340 |
| Mid Roberts Island Drain | 3/31/20047 | 18:20 | ND | ND | ND | ND | BH (0.170) | BH (0.210) |
| Mid Roberts Island Drain | 4/07/2004 | 11:50 | (0.007 J) | ND | ND | ND | 0.083 | (0.100 J) |
| Mid Roberts Island Drain | 4/14/20048 | 09:20 | ND | ND | ND | ND | BH (0.820) | BH (0.090 J) |
| Mid Roberts Island Drain | 4/21/2004 | 10:10 | 12.000 | ND | ND | 0.200 | 3.100 | (0.100 J) |
| Mid Roberts Island Drain | 4/28/2004 | 09:30 | 0.059 | ND | ND | 0.180 | 1.600 | (0.072 J) |
| French Camp Slough at Airport Way | 3/03/2004 | 11:10 | ND | ND | ND | ND | (0.011 J) | 0.610 |
| French Camp Slough at Airport Way | 3/10/2004 | 13:40 | ND | ND | ND | ND | (0.009 J) | (0.150 J) |
| French Camp Slough at Airport Way | 3/17/2004 | 11:20 | ND | ND | ND | ND | ND | 0.220 |
| French Camp Slough at Airport Way | 3/24/2004 | 15:10 | ND | ND | ND | ND | ND | (0.180 J) |
| French Camp Slough at Airport Way | 3/31/2004 | 17:10 | ND | ND | ND | ND | ND | 0.620 |
| French Camp Slough at Airport Way | 4/07/2004 | 13:00 | (0.014 J) | ND | ND | ND | (0.016 J) | (0.089 J) |
| French Camp Slough at Airport Way | 4/14/20049 | 10:00 | BH (0.007 J) | ND | BH (0.016 J) | ND | BH (0.018 J) | BH (0.190 J) |
| French Camp Slough at Airport Way | 4/21/2004 | 10:50 | (0.019 J) | ND | ND | ND | (0.013 J) | (0.190 J) |
| French Camp Slough at Airport Way | 4/28/2004 | 10:00 | 0.096 | 0.330 | ND | ND | 0.023 | 0.510 |
| Old River at Tracy Boulevard | 3/03/2004 | 11:40 | ND | ND | ND | ND | (0.017 J) | 0.460 |
| Old River at Tracy Boulevard | 3/10/2004 | 14:10 | ND | ND | ND | ND | (0.015 J) | (0.110 J) |
| Old River at Tracy Boulevard | 3/17/2004 | 10:50 | ND | ND | ND | ND | (0.009 J) | (0.076 J) |
| Old River at Tracy Boulevard | 3/24/2004 | 14:30 | ND | ND | ND | ND | (0.011 J) | (0.047 J) |
| Old River at Tracy Boulevard | 3/31/200410 | 17:40 | BH (0.014 J) | ND | ND | ND | BH (0.011 J) | BH (0.070 J) |
| Old River at Tracy Boulevard | 4/07/2004 | 12:20 | ND | ND | ND | 2.400 | 0.034 | (0.051 J) |
| Marsh Creek at Cypress Road | 3/04/2004 | 11:20 | ND | ND | ND | ND | ND | (0.063 J) |
| Marsh Creek at Cypress Road | 3/11/2004 | 12:20 | ND | ND | ND | ND | ND | (0.059 J) |
| Marsh Creek at Cypress Road | 3/18/2004 | 12:50 | 0.160 | ND | ND | ND | ND | (0.053 J) |
| Marsh Creek at Cypress Road | 3/25/2004 | 12:50 | ND | ND | ND | ND | 0.054 | (0.052 J) |
| Marsh Creek at Cypress Road | 4/01/200411 | 11:40 | ND | ND | ND | ND | BH (0.034) | BH (0.063 J) |
| Marsh Creek at Cypress Road | 4/08/2004 | 12:50 | (0.007 J) | ND | ND | ND | 0.360 | (0.019 J) |
| Ulatis Creek at Brown Road | 3/04/2004 | 13:50 | ND | ND | ND | ND | (0.007 J) | 0.290 |
| Ulatis Creek at Brown Road | 3/11/2004 | 14:10 | ND | ND | ND | ND | ND | (0.120 J) |
| Ulatis Creek at Brown Road | 3/18/2004 | 14:50 | ND | ND | ND | ND | ND | (0.058 J) |
| Ulatis Creek at Brown Road | 3/25/200412 | 14:40 | ND | ND | ND | ND | BH (0.008 J) | BH (0.110 J) |
| Ulatis Creek at Brown Road | 4/01/2004 | 13:50 | ND | ND | ND | ND | 0.039 | (0.180 J) |
| Ulatis Creek at Brown Road | 4/08/2004 | 14:40 | ND | ND | ND | ND | (0.014 J) | (0.140 J) |
| Ulatis Creek at Brown Road | 4/14/200413 | 12:40 | ND | ND | ND | ND | BH (0.027) | BH (0.140 J) |
| Ulatis Creek at Brown Road | 4/21/200414 | 13:30 | ND | ND | ND | ND | BH (2.000) | BH (0.062 J) |
| Ulatis Creek at Brown Road | 4/28/2004 | 11:40 | ND | ND | ND | (0.030 J) | 2.800 | (0.082 J) |
| Duck Slough at Five Points Marina | 3/03/2004 | 15:30 | ND | ND | ND | ND | ND | 0.430 |
| Duck Slough at Five Points Marina | 3/10/2004 | 09:40 | ND | ND | ND | ND | (0.007 J) | (0.190 J) |
| Duck Slough at Five Points Marina | 3/17/2004 | 15:40 | ND | ND | ND | ND | ND | (0.180 J) |
| Duck Slough at Five Points Marina | 3/24/2004 | 20:10 | ND | ND | ND | ND | 0.040 | (0.051 J) |
| Duck Slough at Five Points Marina | 3/31/2004 | 12:30 | ND | ND | ND | ND | ND | (0.048 J) |
| Duck Slough at Five Points Marina | 4/07/2004 | 08:30 | ND | ND | ND | ND | (0.009 J) | (0.066 J) |
| Cache Slough at Real McCoy Ferry | 3/03/2004 | 15:00 | ND | ND | ND | ND | ND | (0.050 J) |
| Cache Slough at Real McCoy Ferry | 3/10/2004 | 10:10 | ND | ND | ND | ND | ND | (0.036 J) |
| Cache Slough at Real McCoy Ferry | 3/17/2004 | 15:10 | ND | ND | ND | ND | ND | (0.027 J) |
| Cache Slough at Real McCoy Ferry | 3/24/2004 | 18:20 | ND | ND | ND | ND | ND | (0.026 J) |
| Cache Slough at Real McCoy Ferry | 3/31/200415 | 13:00 | ND | ND | ND | ND | ND | BH (0.036 J) |
| Cache Slough at Real McCoy Ferry | 4/07/2004 | 08:50 | ND | ND | ND | ND | ND | (0.022 J) |
| Sacramento River at Rio Vista | 3/03/2004 | 14:30 | ND | ND | ND | ND | ND | (0.057 J) |
| Sacramento River at Rio Vista | 3/10/2004 | 10:30 | ND | ND | ND | ND | ND | (0.026 J) |
| Sacramento River at Rio Vista | 3/17/2004 | 14:30 | ND | ND | ND | ND | ND | (0.028 J) |
| Sacramento River at Rio Vista | 3/24/200416 | 18:40 | ND | ND | ND | ND | BH (0.008 J) | BH (0.039 J) |
| Sacramento River at Rio Vista | 3/31/200417 | 13:20 | ND | ND | ND | ND | ND | BH (0.032 J) |
| Sacramento River at Rio Vista | 4/07/2004 | 09:20 | ND | ND | ND | ND | ND | (0.024 J) |

| Date Extracted | Chlorpyrifos Methyl (Surrogate) Recovery |
| --- | --- |
| 3/4/2004 | 95% |
| 3/8/2004 | 81% |
| 3/11/2004 | 84% |
| 3/12/2004 | 92% |
| 3/18/2004 | 109% |
| 3/22/2004 | 107% |
| 3/25/2004 | 109% |
| 3/26/2004 | 98% |
| 4/1/2004 | 107% |
| 4/2/2004 | 104% |
| 4/8/2004 | 119% |
| 4/13/2004 | 93% |
| 4/15/2004 | 126% |
| 4/22/2004 | 102% |
| 4/29/2004 | 104% |

Appendix B.  Lab blank data

(No pesticides were present at detectable levels. The pesticides include azinphos methyl, bifenthrin, carbaryl, chlorpyrifos, cyanazine, cyfluthrins, cypermethrins, dacthal (DCPA), diazinon, disulfoton, EPTC (Eptam), esfenvalerate, methidathion, metolachlor, propargite, l-Cyhalothrin and simazine)

Appendix C.  Recovery rates in lab control matrix spikes

| Date Extracted | Bifenthrin | Chlorpyrifos | Diazinon | Chlorpyrifos methyl (surrogate) |
| --- | --- | --- | --- | --- |
| 3/4/2004 | 120% | 103% | 98% | 80% |
| 3/8/2004 | 114% | 86% | 92% | 98% |
| 3/11/2004 | 115% | 102% | 92% | 98% |
| 3/12/2004 | 99% | 102% | 86% | 82% |
| 3/18/2004 | 96% | 99% | 100% | 104% |
| 3/22/2004 | 115% | 99% | 91% | 107% |
| 3/25/2004 | 94% | 95% | 93% | 112% |
| 3/26/2004 | 119% | 101% | 91% | 103% |
| 4/1/2004 | 72% | 110% | 108% | 120% |
| 4/2/2004 | 107% | 105% | 101% | 112% |
| 4/8/2004 | 123% | 109% | 105% | 116% |
| 4/13/2004 | 98% | 104% | 91% | 104% |
| 4/15/2004 | 116% | 109% | 110% | 124% |
| 4/22/2004 | 102% | 113% | 102% | 130% |
| 4/29/2004 | 93% | 100% | 98% | 112% |

 In April the monitoring schedule was abbreviated due to budgetary reasons.  The quality control sampling schedule was not modified at that time resulting in a disproportionate ratio of duplicate samples to field blanks and matrix spikes.  A duplicate sample scheduled for March 17 was mistakenly collected as a field blank further compounding the discrepancy in sample ratios.

Surrogate recovery (135%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

Surrogate recovery (143%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

Surrogate recovery (136%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

Surrogate recovery (139%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

Surrogate recovery (132%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

Surrogate recovery (142%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

Surrogate recovery (148%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

Surrogate recovery (132%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

Surrogate recovery (138%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

Surrogate recovery (131%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

Surrogate recovery (132%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

1 Surrogate recovery (135%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

2 Surrogate recovery (143%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

3 Surrogate recovery (136%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

4 Surrogate recovery (138%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

5 Surrogate recovery (139%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

6 There was no water in the Calaveras River at Ijams Road on 7 April 2004.

7 Surrogate recovery (132%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

8 Surrogate recovery (142%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

9 Surrogate recovery (148%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

10 Surrogate recovery (137%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

11 Surrogate recovery (132%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

12 Surrogate recovery (138%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

13 Surrogate recovery (131%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

14 Surrogate recovery (132%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

15 Surrogate recovery (143%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

16 Surrogate recovery (132%) was outside of QAPP acceptance limits. Results should be viewed as biased high.

17 Surrogate recovery (140%) was outside of QAPP acceptance limits. Results should be viewed as biased high.