Pesticide Concentrations in Water and Sediment and Associated Invertebrate Toxicity in Del Puerto and Orestimba Creeks, California

Michael Ensminger, Rick Bergin, and Frank Spurlock

CALIFORNIA ENVIRONMENTAL PROTECTION AGENCY

California Department of Pesticide Regulation

Environmental Monitoring Branch

Surface Water Protection Program

1001 I Street, Sacramento, CA 95812

November 19, 2009

Report 243

# ABSTRACT

The San Joaquin Valley is an important agricultural production area in California, where more than 1.5 million pounds of organophosphorus (OP) and pyrethroid insecticides are applied annually. The major river flowing through the valley, the San Joaquin River (SJR), is listed on the 2006 Clean Water Act §303(d) list for pesticide impairment. Several SJR tributaries are also listed, including Orestimba (ORC) and Del Puerto (DPC) Creeks. From December 2007 through June 2008, water and sediment samples were collected from ORC and DPC in Stanislaus County to determine concentrations of OP and pyrethroid insecticides, and to identify related toxicity to Ceriodaphnia dubia and Hyalella azteca. OPs were detected in almost half (10 of 21) of the water samples, at concentrations from 0.005 to 0.912 μg L-1. Diazinon was the most frequently detected OP, followed by chlorpyrifos and dimethoate. Two water samples were toxic to C. dubia; based on LC50s, chlorpyrifos was likely the cause of this toxicity. Pyrethroids were detected more frequently in sediment samples (18 detections) then in water samples (three detections). Pyrethroid concentrations in water samples ranged from 0.005 to 0.021 μg L-1. These concentrations were well below reported C. dubia LC50s and toxicity was not observed in laboratory bioassays. Cyfluthrin, bifenthrin, esfenvalerate, and λ-cyhalothrin were detected in sediment samples at concentrations ranging from 1.0 to 74.4 ng g-1, dry weight. At DPC, all but one sample caused 100% toxicity to H. azteca. Based on estimated toxicity units (TUs) calculated from measured pyrethroid concentrations in sediment, bifenthrin was likely responsible for this toxicity; λ-cyhalothrin also probably contributed. At ORC, survival of H. azteca was significantly reduced in four of the 11 sediment samples. However, pyrethroids were detected in only two of these samples. Based on TUs, bifenthrin and λ-cyhalothrin likely contributed to toxicity in these two samples.

# ACKNOWLEDGEMENTS

Many people have generously given their time to help this study succeed. I would like to thank Sheryl Gill for working with CURES and securing the contracts that were needed before this project could be initiated. I would like to thank the staff at DPR, Environmental Monitoring Branch for assisting in the field sampling. I would like to thank Carissa Ganapathy for sample coordination and organization between DPR and CDFG and to Jessie Ybarra for his help in maintaining DPR’s West Sacramento’s facility. I extend gratitude to the staff at CDFG for sample analysis and to the staff at UCD ATL for conducting the toxicity studies. Finally, I thank the California State Water Resources Control Board for funding this project through a Proposition 40 and Proposition 50 Grant.

# TABLE OF CONTENTS

ABSTRACT	i

ACKNOWLEDGEMENTS	i

TABLE OF CONTENTS	ii

LIST OF TABLES	iii

LIST OF FIGURES	iii

I. INTRODUCTION	1

II. MATERIALS AND METHODS	2

Study Area	2

Field Sampling.	2

Water Quality Measurements	3

Analytical Chemistry	3

Toxicity Testing.	3

QA/QC for Water, Sediment, and Toxicity Samples	4

III. RESULTS AND DISCUSSION	4

Quality Control	4

Water Samples	5

Water Quality Measurements	5

Total Suspended Sediments.	5

Pesticide Detections.	5

C. dubia Toxicity.	6

Sediment Samples	7

Grain Size and Total Organic Carbon (TOC).	7

Pesticide Detections.	7

H. azteca Toxicity	7

IV. CONCLUSIONS	8

V.  LITERATURE CITED	9

Appendix 1. QC Data for Water Samples	23

Appendix 2. QC Data for Sediment Samples	31

Appendix 3. UCD ATL QC Data	38

Appendix 4. Field Measurements and Non-chemical Laboratory Analysis Results.	41

# LIST OF TABLES

Table 1. Organophosphorus (OP) insecticide use (lb a.i.) in Stanislaus County, California, for the 2008 water year	13

Table 2.  Pyrethroid insecticide (lb a.i.) use in Stanislaus County, California, for the 2008 water year	14

Table 3.  Insecticides analyzed by the California Department of Fish and Game in water or sediment, with their method detection and reporting limits	15

Table 4. OP and pyrethroid detections and survivability of C. dubia in water samples collected at Del Puerto Creek, California	16

Table 5. OP and pyrethroid detections and survivability of C. dubia in water samples collected at Orestimba Creek, California	17

Table 6.  Grain size, percent organic carbon, pyrethroid detections, and survivability of H. azteca in sediments collected at Del Puerto Creek and Orestimba Creeks, California	18

# LIST OF FIGURES

Figure 1. Sampling sites at Orestimba Creek and at Del Puerto Creek, California	20

Figure 2. Box plots of water quality measurements at Del Puerto Creek (DPC) and at Orestimba Creek (ORC)	214

Figure 3. Toxicity units (TUs) of pyrethroids from sediment samples taken at Del Puerto Creek	215

Figure 4. Toxicity units (TUs) of pyrethroids from sediment samples collected at Orestimba Creek	22

Figure 5. Relationship between the sum of pyrethroid toxicity units (TUs) in sediments and the toxicity to H. azteca	22

# I. INTRODUCTION

The San Joaquin River (SJR) watershed is an important agricultural production area in the Central Valley of California. The SJR drains about 32,000 square miles through the San Joaquin Valley. In the SJR watershed, applicators apply greater than 22 million lb active ingredient (a.i.) of pesticides yearly (CDPR, 2009a). Pesticides are frequently detected in the SJR and observed to cause toxicity to indicator species. As a result, the Central Valley Regional Water Quality Control Board has placed the SJR on the 2006 Clean Water Act §303(d) list for pesticide impairment (CA EPA, 2009). Orestimba (ORC) and Del Puerto (DPC) Creeks are tributaries of the SJR; these two creeks are also on the §303(d) list due to the presence of organophosphorus (OP) insecticides. In addition, ORC and DPC are listed for sediment toxicity (unknown source) and for pyrethroid insecticides, respectively (Cal/EPA, 2009).

Researchers have routinely detected OPs and pyrethroids in water or sediment samples taken from DPC and ORC. An analysis by Spurlock (2002) showed that between 1991 – 2001 diazinon and chlorpyrifos were routinely monitored and detected in rivers and tributaries in the Central Valley, including ORC. Eleven percent of the water samples from this creek during this timeframe were toxic to Ceriodaphnia dubia, although neither diazinon nor chlorpyrifos were likely the sole toxicant. In more recent work, researchers detected chlorpyrifos, dimethoate, diazinon, azinphos-methyl, dichlorvos, methyl parathion, and disulfuton at ORC or DPC; however, no toxicity testing was conducted in these studies (Starner et al., 2003; Kelley and Starner, 2004). Bacey et al. (2004) noted that OP concentrations of diazinon and chlorpyrifos were sufficient to account for the observed toxicity to C. dubia at DPC. In other studies, OPs have routinely been detected and linked to toxicity of aquatic invertebrates (Hunt, et al., 2003; Werner, et al., 2000).

In the past several years, OP agricultural use has decreased. In place of these insecticides, growers are using more pyrethroids. With this increased use, pyrethroids have been detected in creek water and sediments. Bacey et al. (2004) detected the pyrethroid esfenvalerate in the water column and in sediment at DPC. In other studies at DPC, additional pyrethroids (permethrin, esfenvalerate, bifenthrin, λ-cyahalothrin, cyfluthrin) were frequently detected in sediments at concentrations sufficiently high to have caused toxicity to Hyalella azteca (Weston et al., 2004; Weston et al., 2008a).

Currently, both OPs and pyrethroids are used to control a myriad of pests. In the 2008 water year (WY), 158,057 and 31,379 lb a.i., respectively, were applied in Stanislaus County (CDPR, 2009a). OPs were primarily used in production agriculture, but pyrethroids were intensively used in both production and non-production agriculture (Tables 1 and 2). For both classes of insecticides, the heaviest agriculture use is May through September. Chlorpyrifos, dimethoate, and phosmet have the highest OP production agricultural use; bifenthrin, esfenvalerate, λ-cyhalothrin, and permethrin have the highest pyrethroid use. However, over half of all pyrethroid applications are for non-production agricultural use. Permethrin and cypermethrin have the largest non-production agriculture use, with permethrin by far having the most use.

Although there have been numerous monitoring studies at both ORC and DPC, there is a lack of recent published monitoring data collected over consecutive months, especially with concurrent toxicity testing. This study had two objectives:

      - Determine the occurrence and concentrations of OPs in water and of pyrethroids in water and sediment from DPC and ORC during both the rainy and irrigation seasons; and

- Determine the toxicity of creek water and sediments to the representative aquatic invertebrate organisms, C. dubia and H. azteca, respectively.

# II. MATERIALS AND METHODS

Study Area. DPC and ORC run through the west side of Stanislaus County. At DPC, we sampled at Vineyard Avenue near the town of Patterson and at ORC on River Road near the town of Crows Landing (Fig. 1).

Field Sampling. Water and sediment samples were collected monthly from December 2007 through June 2008. In December, January, and February, one additional sample per month was collected during a rain event. Sediment samples could not be collected at every storm event due to high and rapid water flow.

Water samples were collected near midstream directly into 1-L glass amber bottles using an extendable pole. Prior to water collection, bottles were given a native rinse. Bottles were sealed with Teflon®-lined lids. Sediments were collected using a stainless steel trowel or small shovel. Soil sediments (up to a 2 cm depth) were put into clear glass Mason® jars (for chemical analysis) or into pre-cleaned 1-L polypropylene containers (for toxicity studies). Sediments were generally taken from the same areas at both ORC and DPC due to limited sediment accumulation at the selected sampling sites. Both creeks are often scoured clean by rapid water flow; thus the creekbeds are mainly gravel with a few selected spots where sediments build up.

Immediately after sampling, water and sediment samples were stored on wet ice at 4°C for transport. Upon arrival at the laboratory, water samples and sediments for grain size and toxicity testing were refrigerated (4°C) whereas sediments for pyrethroid chemical analysis and for TOC (total organic carbon) analysis were frozen (–20°C). Water samples were analyzed for OPs and pyrethroids and sediment samples were analyzed for pyrethroids. Several other parameters were also analyzed:

- Total suspended solids (TSS) in water samples, following US EPA method 160.2 (US EPA, 1971) and as described in Kelley and Starner (2004);

- TOC in sediment samples, following method of Gunasekara, 2006;

- Sediment grain size (analyzed by Applied Marine Science, Inc.).

Water Quality Measurements.  Dissolved oxygen (DO), electrical conductivity (EC), salinity, pH, and temperature were measured in situ. Measurements were taken with a YSI 85 meter, YSI 60 meter, or YSI 6920 V2 meter (YSI Incorporated, Yellow Springs, Ohio). Prior to use, all instruments were calibrated according to manufacturer’s recommendations (http://www.ysi.com) or CDPR SOPs (http://www.cdpr.ca.gov/docs/emon/pubs/sop.htm).

Analytical Chemistry.  Chemical analyses were conducted by California Department of Fish and Game (CDFG), Fish and Wildlife Water Pollution Control Laboratory. CDFG employed the following methods (CDPR, 2009b):

- OPs:  liquid-liquid extraction and high resolution gas chromatography with Flame Photometric Detector and Thermionic Bead Specific Detector;

- Pyrethroids:  liquid-liquid extraction and high resolution gas chromatography with electron capture detector (GC/ECD).

For both procedures, CDFG used gas chromatography/mass spectrometry and ion trap detection (GC/MS-ITD) to confirm detections. Table 3 lists the method detection limits (MDL) and reporting limits (RL) for each chemical analyzed. The MDL is the lowest concentration of an analyte that can be measured and reported with 99% confidence that the analyte concentration is greater than zero. The reporting limit is usually one to five times the MDL, dependent on analytical method and matrix (Segawa, 1995). We report the results as:

- nd (not detected, concentrations below the MDL);

- trace (trace detection, where in the chemist’s best professional judgment the analyte does exist between the RL and the MDL);

- a numerical concentration in µg L-1 (water samples) or ng g -1 (dry weight; sediment samples).

Toxicity Testing. In addition to the physical and chemical analysis, UCD Aquatic Toxicity Laboratory (ATL) conducted aquatic toxicity tests with C. dubia (water samples) and with H. azteca (sediment samples). Within 24 h of collection, water and sediment samples were transported to UCD ATL for toxicity testing. UCD ATL uses standard EPA methods for these toxicity tests (University of California, Davis, 2009). Briefly, for water toxicity tests, five C. dubia neonates per replicate (four replicates per treatment) were placed into glass scintillation vials containing 18 mL of the water sample. Scintillation vials were housed in an environmental chamber at 25°C with a 16:8 h light:dark photoperiod. Every 24 h, water was renewed and C. dubia was fed daily. After 96 h, the surviving organisms were counted and the percent survival was compared to an untreated control. For sediment studies, 10 - 14 day old H. azteca were placed into 300-mL glass beakers containing control water and sampled creek sediments. Beakers were placed in an environmental chamber or water bath maintained at 23ºC with a 16:8 hour light:dark photoperiod. Water was renewed twice daily and H. azteca was fed after the second daily water change. After 10 days, H. azteca was removed from the beakers, dried at 86ºC for 16 h, cooled and weighed. Percent survival was also recorded.

QA/QC for Water, Sediment, and Toxicity Samples. Quality control for this study followed the CDPR SOP guidelines on Chemistry Laboratory Quality Control (Segawa, 1995) and the Quality Assurance Project Plan (QAPP) written for this study (CDPR, 2009c). For OP and pyrethroid water analysis, quality control consisted of surrogate analytes (“surrogates”), method blanks, laboratory control samples, and laboratory control sample duplicates. In addition, there were two blind spikes, two field duplicates, and two field blanks (5% of the field samples for each). For pyrethroids in sediments, quality control consisted of two surrogates, method blanks, laboratory control samples, laboratory control sample duplicates, matrix spikes, and matrix spike duplicates. In addition, 5% of the field samples were field duplicates. QA/QC for toxicity testing comprised of field duplicates (10%) and field blanks (5%).

# III. RESULTS AND DISCUSSION

# Quality Control

Generally, QC was within control limits; exceptions are discussed below (Appendixes 1 – 3)

Holding times were within control limits for all water samples, sediment samples (including grain size and TOC analysis), and samples send to UCD ATL except:

- Three of 50 water samples exceeded the 40 day holding time by one day (analyzed on day 41; Table A1, Appendix 1);

- Holding times for some of the TSS samples were exceeded (Table A6, Appendix 1).

In water and sediment samples, all 184 surrogates were within control limits. The RPD (relative percent differences) between surrogates added to laboratory duplicate samples were within the RPD control limit (of < 25% differences as defined in the QAPP written for this study) in 29 of 30 samples. The one sample outside the control limit occurred with the first water analysis and thereafter the RPD were within the control limit (Table A2, Appendix 1; Table A10, Appendix 2).

The recovery of spiked laboratory control samples from OP-analyzed water samples was outside of the control limits in 4% of the analyses. Low recovery of phosmet was the most common of the exceedances; five times it was below the lower control limit (LCU). Also, dimethoate and azinphos-methyl exceeded the upper control limit (UCL) in one analysis. In addition, in the OP-analyzed water samples, 8% of the laboratory duplicates were outside of the RPD control limit. However, because OPs were within analytical control limits in the extraction sets where there were detections or during months of OP use in the watersheds, we assume that the variability in spike recoveries do not reflect on the analytical results (Table A3, Appendix 1).

The recovery of spiked laboratory control samples from pyrethroid-analyzed water samples was outside of the control limits in 7% of the analyses. In most cases, exceedances were due to low recoveries of the pyrethroids, which could have led to underreporting of the pyrethroids in water samples. In addition, 9% of the laboratory duplicates were outside of the RPD control limit (Table A4, Appendix 1).

In pyrethroid sediment samples, the recovery of spiked laboratory control samples or matrix samples was outside of the control limits in 5% of the analysis, with no exceedances of the RPD criterion (Table A11, Appendix 2). However, in one lab duplicate analysis, the RPD of bifenthrin was 34.5% (Table A12, Appendix 2). Other than this incident with bifenthrin, QC problems in the sediment samples were mostly due to low recovery of deltamethrin (Table A11, Appendix 2). Deltamethrin had virtually no use in the Stanislaus County during the course of this study; therefore, it is unlikely that this pyrethroid would have been present in the samples (Table 2).

The final QC problem occurred in one of the field blanks, where diazinon was reported below its RL but above its MDL (Table A5, Appendix 1). Unfortunately, these results were received after the field sampling was complete and additional field blanks could not be collected. The analytical results from the field sample taken at the same time (March at DPC) also reported a trace detection of diazinon; this detection may be an artifact.

# Water Samples

Water Quality Measurements. Water temperature, pH, and EC (conductivity and salinity) were significantly higher at DPC than at ORC based on paired t-tests (p<0.05). In addition, DPC water quality parameters were generally more variable than ORC. DPC is a much smaller creek than ORC, with about one-fifth of the flow. Thus, the lower baseline flow at DPC may allow rain or irrigation runoff to have a greater relative impact on DPC water quality. More variable or adverse water quality could potentially have a negative effect on sensitive aquatic species. All of the water quality data can be found in Table A17, Appendix IV.

Total Suspended Sediments. TSS in the water samples ranged from 4.8 mg L-1 to 287 mg L-1 at DPC and from 7.2 mg L-1 to 98.1 mg L-1 at ORC, except for the January storm sample at ORC where the TSS concentration was 1766 mg L-1. The TSS concentrations were not significantly different between the two creeks (Appendix IV, Table A18).

Pesticide Detections. We collected 21 water samples for OP and pyrethroid analysis. In these 21 water samples, we detected 12 OPs (Tables 4 and 5). Diazinon was the most frequently detected OP (six detections) and was detected in the highest concentration of any pesticide in this study (0.912 μg L-1). Diazinon was detected during the dormant season (four detections) and during the irrigation season (two detections). In the dormant season, diazinon was detected during rain and regular monthly (non-storm) sampling events.

Chlorpyrifos was the second most commonly detected OP (four detections). Dimethoate was detected twice, but no other OPs were detected. Both chlorpyrifos and dimethoate were only detected during the irrigation season, which is the highest use season for these two OPs.

In both the DPC and ORC watersheds, there was sufficient reported production agriculture use around the time and location of sampling to account for the dimethoate and chlorpyrifos detections. However, the source of the diazinon detections is unknown. Currently, there is no reported diazinon use around the time of the diazinon detections (CDPR, 2009a).

Only three pyrethroids were detected out of the 21 water samples collected, all at ORC (Tables 4 and 5). Cyfluthrin was detected in January’s storm sample; bifenthrin and λ-cyhalothrin were detected in March. As discussed in the Quality Control section, pyrethroid water concentrations may be low-biased due to low recoveries (Table A4, Appendix 1). Only the λ-cyhalothrin detection was consistent with reported production agriculture use (CDPR, 2009a).

C. dubia Toxicity. UCD ATL tested 21 water samples for toxicity to C. dubia. Only two of these samples (May and June 26) from ORC were toxic to C. dubia (Tables 4 and 5). Based on the acute LC50 of chlorpyrifos to aquatic invertebrates (0.05 μg L-1;US EPA, 2009a), chlorpyrifos concentrations in these two water samples were high enough to cause toxicity to C. dubia. Chlorpyrifos was also detected in the June 10 water sample at ORC (trace detection) and in the June 26 water sample at DPC (0.035 μg L-1), concentrations not likely to be toxic.

Although no other water samples from either creek caused toxicity to C. dubia (Tables 4 and 5), based on the acute LC50 of diazinon to aquatic invertebrates (0.105 μg L-1; US EPA, 2009a), the monthly February ORC sample (0.912 μg L-1) exceeded the C. dubia acute LC50 and both of the January DPC samples were essentially equal to diazinon’s acute LC50. In these water samples, the lack of toxicity to C. dubia is unexplained. Diazinon was also detected in three other samples from DPC, but at concentrations much lower than its LC50. We also detected dimethoate in two water samples, at 0.074 and 0.190 μg L-1 . However, at these concentrations it would not likely be toxic to C. dubia. The LC50 of dimethoate to aquatic invertebrate organisms is more than two orders of magnitude greater than these detections (21.5 μ L-1; US EPA, 2009a).

The three ORC water samples with detectable levels of pyrethroids did not cause any toxicity to C. dubia.  Although there are no water aquatic benchmarks for pyrethroids (US EPA, 2009a), the US EPA does list LC50 values for the pyrethroids in its Ecotox database (US EPA, 2009b). The pyrethroids were detected at an order of magnitude less then their LC50s, thus the lack of toxicity is consistent with these values. Pyrethroids are much more toxic to H. azteca than they are to C. dubia (Haver et al., 2008). H. azteca may be a better choice for toxicity testing in pyrethroid-containing aquatic systems.

# Sediment Samples

Grain Size and Total Organic Carbon (TOC). At DPC and ORC, fine grain particles (“fines”: silt and clay particles less than 62.5 μm) ranged from 15.3 to 86.2% and TOC ranged from 0.28 to 1.39% (Table A19, Appendix IV). Sediments with these properties are likely to contain pyrethroids which could be potentially toxic to H. azteca. Other work has shown that sediments with a similar composition contained pyrethroids and were toxic to H. azteca (Holmes et al., 2008; Weston et al., 2004; You et al., 2008).

Pesticide Detections. Nineteen sediment samples were collected for pyrethroid analysis. Twelve of the 19 samples contained pyrethroids, with some samples containing up to three different pyrethroids. Most of the pyrethroid detections occurred at DPC, where all of the nine sediment samples contained at least one pyrethroid. In all, we detected 14 pyrethroids at DPC (Table 6). ORC sediment was much less contaminated with pyrethroids; only three of the 10 sediment samples contained pyrethroids.

Bifenthrin was the most frequently detected pyrethroid (10 detections) and detected at the highest concentration (74.4 ng g-1). It was detected in all but one of the samples at DPC but was only detected twice at ORC. Bifenthrin was detected in both the dormant and irrigation season with about equal frequency, and was detected at higher concentrations in the irrigation season.

Three other pyrethroids were detected: λ-cyhalothrin (four detections), cyfluthrin (twice) and esfenvalerate (twice) (Table 6). Esfenvalerate was detected in December at both creeks whereas cyfluthrin was detected in the irrigation season at DPC. Lambda-cyhalothrin was detected twice in the dormant season and twice in the irrigation season. There was sufficient reported use around the time and location of sampling to account for the λ-cyhalothrin and esfenvalerate detections. However, this is not the case for the bifenthrin and cyfluthrin detections (CDPR, 2009a).

H. azteca Toxicity. Due to differing toxicities of pyrethroids, toxicity units (TUs) are commonly used to identify pyrethroids that may contribute to sediment toxicity (Amweg et al., 2005). To calculate TUs, pyrethroid concentration, normalized to organic carbon, is  divided by 10-day sediment LC50 values (also normalized to organic carbon). As common practice, we used previously published LC50 values (Amweg, 2005; Maund, 2002) to calculate TUs. We expect 50% H. azteca toxicity with 1 TU, and 0.5 TU can be used as an approximate concentration where toxicity would likely begin to appear (Weston et al., 2008b). TUs for the different pyrethroids are assumed to be additive due to a similar mode of action.

Every sediment sample from DPC caused 100% mortality to H. azteca in the UCD ATL toxicity screen, except for the sediment from the December storm sample which caused almost 80% mortality (Table 6). From calculated TUs, bifenthrin was the major contributor of toxicity at DPC. Except for the December sample, bifenthrin was detected in every sediment sample at this creek, containing between 1.5 and 15 TUs. Cyfluthrin,                λ-cyhalothrin and esfenvalerate were also detected at DPC, but only λ-cyhalothrin would have contributed to toxicity, with between 0.6 to 1.8 TUs. Cyfluthrin and esfenvalerate contributed a minor amount, less than 0.5 TUs (Fig. 3).

At ORC, the sediment was not as toxic. Four of the 11 sediment samples caused significant toxicity to H. azteca. In these four samples, survivability ranged from 55 to 72% of the control levels (Table 6). Pyrethroids were only detected in December’s two sampling dates and in the first June sampling date. In December’s storm sample,            λ-cyhalothrin had 0.97 TUs and likely caused the toxicity to H. azteca (esfenvalerate also had 0.1 TU). The December monthly sample contained 0.44 TU, all due to bifenthrin (Fig. 4). The 0.44 TU could cause some toxicity to H. azteca. Amweg et al. (2005) found that a majority of sediment samples with TUs greater than 0.4 resulted in 40% or higher H. azteca toxicity.

The June 10 sample from ORC contained almost 6 TUs, all due to bifenthrin. However, there was no toxicity associated with this sediment sample. This occurs occasionally; in other research, scientists have reported high TUs with little or no toxicity (Amweg et al., 2005; Amweg et al., 2006; Weston et al., 2008b). This is generally attributed to lack of bioavailability due to sediment factors other than TOC, which has already been factored into the TU analysis. Sediments with medium sands or coarser grain sizes tend to give artificially high TUs with corresponding low toxicity (Amweg et al., 2006; Weston et al., 2008b). However, the June 10 sample contained greater than 50% fines and thus would not explain reduced toxicity. Undetermined factors likely reduced bioavailability of this sediment.

At ORC, no other pyrethroids were detected in any of the other sediment samples, yet significant toxicity to H. azteca occurred in February (monthly) and in April. The source of the toxicity is unknown, likely due to some other contaminant or stressor other than pyrethroids.

At ORC, TUs were not predictive of H. azteca toxicity (Fig. 5). Approximately 40 to 50% H. azteca mortality was associated with anywhere from none to almost 1 TU, and bifenthrin’s almost 6 TUs had no H. azteca toxicity. However, at DPC, TUs greater than 1 TU were always associated with 100% toxicity of H. azteca. In one instance where there was only 0.13 TU, higher than expected toxicity was observed (Fig. 5, point a). Some other contaminant or stressor (e.g., chlorpyrifos or metals), or perhaps a synergist (as PBO), may have caused this higher than expected toxicity. We did not analyze the sediment samples for any of these other stressors.

# IV. CONCLUSIONS

The main conclusions were concise but clear. OPs were detected in water; pyrethroids were mainly detected in sediments (although a few were detected in water samples at ORC).

Diazinon and chlorpyrifos were the most frequently detected OPs. Both were detected at concentrations greater or equal to their respective acute aquatic invertebrate LC50s. However, of the 21 collected water samples, only two samples were toxic to C. dubia. These two samples only contained chlorpyrifos at high enough concentrations to cause the toxicity. Diazinon was not associated with toxicity in the C. dubia toxicity tests. Dimethoate was also detected, but at concentrations much lower then its reported acute aquatic invertebrate toxicity. Chlorpyrifos and dimethoate were detected during the irrigation season and were associated with reported agricultural production use. Diazinon was detected in the dormant (both storm and non-storm sampling) and during the irrigation season, but in current PUR database records, diazinon had little to no reported production agriculture use in the DPC or ORC watersheds around the time of the detections.

Bifenthrin and λ-cyhalothrin were the most commonly detected pyrethroids in sediment, and both (based on TUs) were highly associated with toxicity to H. azteca. However, bifenthrin by far was the most commonly detected pyrethroid in sediment. It was detected every month, at the highest concentration of any pyrethroid, had the highest TUs, and likely contributed the most to H. azteca toxicity. Cyfluthrin and esfenvalerate were also detected in sediments, albeit less frequently and were not associated with toxicity to H. azteca.

When bifenthrin was detected, it was not associated with reported production agriculture use during the sampling period. In the DPC and ORC watersheds, bifenthrin has reported production agriculture use May through September, with no use the remainder of the year. Bifenthrin detections were attributed to the long persistence of this pyrethroid. Esfenvalerate and λ-cyhalothrin detections were associated with reported production agriculture use but, as in the water detections, cyfluthrin detections were not associated with reported use.

Comparing the two creeks, Sediments from DPC were highly contaminated with pyrethroids, whereas sediments from ORC were less contaminated. Thus, at DPC, overall TUs were predictive of toxicity to H. azteca but at ORC TUs were not.

# V.  LITERATURE CITED

Amweg, E.L., D.P. Weston, and N.M. Ureda. 2005. Use and toxicity of pyrethroid pesticides in the Central Valley, California, U.S.A. Environ. Toxicol. Chem. 24:966-972 (erratum 24, 1300-1301).

Amweg, E.L., D.P. Weston, J.You, and M.J. Lydy. 2006. Pyrethroid insecticides and sediment toxicity in urban creeks from California and Tennessee. Environ. Sci. Technol. 40:1700-1706.

Bacey, J., K. Starner, and F. Spurlock. 2004. The occurrence and concentration of esfenvalerate and permethrin in water and sediment in the Sacramento and San Joaquin Watersheds.  CDPR Report EH 04-01. Accessed at http://www.cdpr.ca.gov/docs/emon/surfwtr/swemreps.htm on July 31, 2008.

Cal/EPA. 2009. Central Valley Regional Water Quality Control Board.  Accessed at http://www.swrcb.ca.gov/centralvalley/water_issues/tmdl/impaired_waters_list/index.shtml on June 1, 2009.

CIMIS, 2008. Patterson, CA weather data. Accessed at http://wwwcimis.water.ca.gov/cimis/welcome.jsp on June 1, 2009.

CDPR. 2009a. California Department of Pesticide Regulation’s Pesticide Information Portal, Pesticide Use Report (PUR) data.  Accessed at http://www.cdpr.ca.gov/docs/pur/purmain.htm on September 22, 2008.

CDPR. 2009b. Insecticide Analytical Methods. Accessed at http://www.cdpr.ca.gov/docs/emon/pubs/em_methd_insect.htm on November 17, 2008.

CDPR. 2009c. Surface Water Protection Publications: QUALITY ASSURANCE PROJECT PLAN (QAPP) for CDPR Studies 243 and 244. Accessed at http://www.cdpr.ca.gov/docs/emon/surfwtr/pubs.htm on November 19, 2009.

Gunasekara, A. 2006. California Department of Pesticide Regulation SOP number METH005.00: Instructions for calibration and use of total organic carbon (CD-85A) instrument Accessed at http://www.cdpr.ca.gov/docs/empm/pubs/sops/meth005_00.pdf on Oct 31, 2007.

Haver, D.L., L. Oki, S. Bondarenko, R. Holmes, R. Mazalewski, T. Majcherek, J. Kabashima, I. Werner, and J. Gan. 2008. Detection of pesticides in residential runoff. Abstracts, 5th Biennial CALFED Science Conference, Sacramento,CA. p. 41.

Holmes, R.W., B.S. Anderson, B.M. Phillips, J.W. Hunt, D.B. Crane, A. Mekebri, and V. Connor. 2008. Statewide investigation of the role of pyrethroid pesticides in sediment toxicity in California’s urban waterways. Environ. Sci. Technol. 42:7003-7009.

Hunt, J.W., B.S. Anderson, B.M. Phillips, P.N. Nicely, R.S. Tjeerdema, H.M. Puckett, M. Stephenson, K. Worcester, and V. De Vlaming. 2003. Ambient toxicity due to chlorpyrifos in a central California coastal watershed. Environmental Monitoring and Assessment 82:83-112.

Kelley, K. and K. Starner. 2004. Preliminary results for Study 219: Monitoring surface waters and sediments of the Salinas and San Joaquin River Basins for organophosphate and pyrethroid pesticides. Accessed at http://www.cdpr.ca.gov/docs/emon/surfwtr/swmemos.htm on 18 September 2008.

Maund, S. J., M.J. Hammer, M.C.G. Lane, E. Farrelly, J.H. Rapley, and U.M. Goggin, W. E. Gentle. 2002. Partitioning, bioavailability, and toxicity of the pyrethroid insecticide cypermethrin in sediments. Environ. Toxicol. Chem. 21:9-15.

Segawa, R. 1995. California Department of Pesticide Regulation SOP number QAQC001.00:  Chemistry Laboratory Quality Control. Accessed at http://www.cdpr.ca.gov/docs/emon/pubs/sops/qaqc001.pdf on July 29, 2008.

Spurlock, F. Analysis of diazinon and chlorpyrifos surface water monitoring  and acute toxicity bioassay data, 1991 – 2001. CDPR Report EH 01-01. Accessed at http://www.cdpr.ca.gov/docs/emon/surfwtr/swemreps.htm March 4, 2009.

Starner, K., F. Spurlock, S. Gill, K. Goh, H. Feng, J. Hsu, P. Lee, D. Tran, and J.White. 2003. Monitoring surface waters of the San Joaquin River Basin for selected summer-use pesticides, 2002. CDPR Report EH 03-03. Accessed at http://www.cdpr.ca.gov/docs/emon/surfwtr/swemreps.htm on March 4, 2009.

University of California, Davis.  2009. Aquatic Toxicity Lab. Accessed at http://www.vetmed.ucdavis.edu/apc/atl/default.html on July 29, 2009.

U.S. Environmental Protection Agency. 1971. National Exposure Research Laboratory (NERL), Microbiological and Chemical Exposure Assessment Research Division (MCEARD).  Method 160.2, Residue, Non-Filterable (Gravimetric, Dried at 103 – 105° C).  Accessed at http://web1.er.usgs.gov/nemi/method_summary.jsp?param_method_id=5213 on June 21, 2007.

U.S. Environmental Protection Agency. 2009a. Office of Pesticide Programs. Aquatic Life Benchmarks. Accessed at http://www.epa.gov/oppefed1/ecorisk_ders/aquatic_life_benchmark.htm on August 6, 2009

U.S. Environmental Protection Agency. 2009b. Ecotox database. Accessed at http://www.epa.gov/ecotox/ on October 28, 2008.

Werner, I., L.A. Deanovic, V. Connor, V. de Vlaming, H.C. Bailey, and D.E. Hinton. 2000. Insecticide-caused toxicity to Ceriodaphnia dubia (Cladocera) in the Sacramento-San Joaquin River Delta, California, USA. Environ. Toxicol. Chem. 19:215-227.

Weston, D.P., J. You, and M.J. Lydy. 2004. Distribution and toxicity of sediment-associated pesticides in agriculture-dominated water bodies of California’s Central Valley. Environ. Sci. Technol. 38:2752-2759.

Weston, D.P., M. Zhang, and M.J. Lydy. 2008a. Identifying the cause and source of sediment toxicity in an agriculture-influenced creek. Environ. Toxicol. Chem. 27:953-962.

Weston, D.P., J. You, E.L. Amweg, and M.J. Lydy. 2008b. Sediment toxicity in agricultural areas of California and the role of hydrophobic pesticides. In Synthetic Pyrethroids, Occurrence and Behavior in Aquatic Environments; Gan, J., F.Spurlock, D. Weston, Eds., American Chemical Society, Washington, D.C., pp. 26-54.

You, J., S. Pehkonen, D.P. Weston, and M.J. Lydy. 2008. Chemical availability and sediment toxicity of pyrethroid insecticides to Hyalella azteca: application to field sediment with unexpectedly low toxicity. Environ. Toxicol. Chem. 27:2124-2130.

Zhang, X., M.Zhang, and X.Liu. 2008. Identification of hotspots for potential pyrethroid runoff: a GIS modeling study in San Joaquin Watershed of California, USA. Environ Geol. 55:1195-1206.

Table 1. Organophosphorus (OP) insecticide use (lb a.i.) in Stanislaus County, California, for the 2008 water year.

| OP | Oct | Nov | Dec | Jan | Feb | Mar | Apr | May | June | July | Aug | Sep | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OP Production Agriculture Use in Stanislaus County |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Azinphos-Methyl | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 236 | 119 | 415 | 35 | 0 | 804 |
| Chlorpyrifos | 3,946 | 329 | 419 | 917 | 147 | 950 | 1,880 | 18,003 | 6,369 | 25,979 | 8,879 | 3,319 | 71,137 |
| Diazinon | 208 | 13 | 677 | 1,603 | 21 | 1 | 54 | 61 | 0 | 139 | 68 | 14 | 2,859 |
| Dimethoate | 361 | 44 | 0 | 94 | 0 | 1,678 | 147 | 240 | 2,159 | 12,081 | 37,125 | 1,418 | 55,348 |
| Ethoprop | 0 | 0 | 0 | 0 | 0 | 0 | 1,072 | 4,012 | 0 | 0 | 0 | 0 | 5,084 |
| Malathion | 0 | 0 | 0 | 0 | 0 | 1,540 | 59 | 199 | 172 | 401 | 1,301 | 738 | 4,411 |
| Methidathion | 0 | 14 | 119 | 49 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 181 |
| Methyl Parathion | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1,020 | 871 | 868 | 0 | 0 | 2,759 |
| Naled | 310 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 105 | 350 | 3 | 768 |
| Phosmet | 0 | 0 | 0 | 0 | 0 | 0 | 759 | 3,529 | 3,495 | 4,360 | 1,238 | 694 | 14,076 |
| Agriculture Total | 4,824 | 401 | 1,215 | 2,662 | 168 | 4,170 | 3,971 | 27,300 | 13,185 | 44,349 | 48,997 | 6,187 | 157,428 |
| OP Non-Production Agriculture Use in Stanislaus County |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chlorpyrifos | 3 | 0 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 2 | 1 | 4 | 13 |
| Diazinon | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 8 | 9 |
| Dimethoate | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| Malathion | 51 | 6 | 3 | 39 | 34 | 40 | 71 | 96 | 48 | 64 | 6 | 4 | 462 |
| Naled | 64 | 12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 25 | 24 | 19 | 145 |
| Non-Agriculture Total | 118 | 19 | 3 | 39 | 34 | 42 | 72 | 97 | 48 | 91 | 32 | 36 | 629 |
| Grand Total | 4,942 | 419 | 1,218 | 2,701 | 201 | 4,212 | 4,043 | 27,397 | 13,233 | 44,440 | 49,028 | 6,223 | 158,057 |

Table 2.  Pyrethroid insecticide (lb a.i.) use in Stanislaus County, California, for the 2008 water year.

| Pyrethroid | Oct | Nov | Dec | Jan | Feb | Mar | Apr | May | June | July | Aug | Sep | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pyrethroid Production Agriculture Use in Stanislaus County |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bifenthrin | 0 | 1 | 0 | 24 | 2 | 6 | 1 | 35 | 449 | 2,533 | 497 | 90 | 3,639 |
| Cyfluthrin1 | 0 | 9 | 0 | 0 | 0 | 4 | 4 | 14 | 21 | 55 | 29 | 8 | 144 |
| Cypermethrin2 | 2 | 4 | 0 | 0 | 0 | 20 | 15 | 31 | 4 | 14 | 5 | 0 | 94 |
| Deltamethrin | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 4 | 0 | 0 | 6 |
| Esfenvalerate | 27 | 0 | 90 | 270 | 70 | 12 | 19 | 630 | 297 | 1,201 | 74 | 35 | 2,726 |
| Fenpropathrin | 0 | 0 | 0 | 0 | 0 | 0 | 81 | 7 | 28 | 29 | 81 | 21 | 247 |
| Gamma-Cyhalothrin | 0 | 0 | 0 | 2 | 0 | 0 | 4 | 6 | 5 | 5 | 0 | 0 | 23 |
| Lambda-Cyhalothrin | 30 | 3 | 2 | 60 | 7 | 605 | 47 | 723 | 364 | 935 | 525 | 89 | 3,390 |
| Permethrin | 19 | 3 | 0 | 189 | 0 | 31 | 61 | 744 | 190 | 596 | 33 | 335 | 2,202 |
| Agriculture Total | 79 | 20 | 92 | 545 | 78 | 679 | 231 | 2,191 | 1,361 | 5,372 | 1,245 | 578 | 12,471 |
| Pyrethroid Non-Production Agriculture Use in Stanislaus County |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bifenthrin | 90 | 38 | 24 | 27 | 33 | 38 | 37 | 37 | 52 | 33 | 24 | 119 | 554 |
| Cyfluthrin1 | 11 | 8 | 5 | 4 | 4 | 13 | 55 | 102 | 86 | 96 | 115 | 101 | 599 |
| Cypermethrin2 | 70 | 131 | 48 | 35 | 175 | 119 | 1,041 | 399 | 276 | 501 | 190 | 207 | 3,192 |
| Deltamethrin | 2 | 1 | 1 | 2 | 1 | 2 | 2 | 2 | 2 | 3 | 4 | 2 | 23 |
| Esfenvalerate | 0 | 0 | 1 | 2 | 55 | 3 | 4 | 0 | 0 | 0 | 0 | 0 | 66 |
| Lambda-Cyhalothrin | 5 | 1 | 2 | 2 | 1 | 1 | 2 | 2 | 5 | 8 | 7 | 6 | 41 |
| Permethrin | 269 | 107 | 64 | 70 | 119 | 87 | 836 | 163 | 218 | 2,909 | 4,547 | 5,042 | 14,432 |
| Non-Agriculture Total | 446 | 287 | 144 | 142 | 389 | 264 | 1,978 | 704 | 639 | 3,550 | 4,887 | 5,477 | 18,908 |
| Grand Total | 525 | 307 | 236 | 687 | 468 | 943 | 2,209 | 2,896 | 1,999 | 8,922 | 6,132 | 6,055 | 31,379 |
| 1includes β-cyfluthrin and cyfluthrin 2includes (S)-cypermethrin and cypermethrin |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3.  Insecticides analyzed by the California Department of Fish and Game in water or sediment, with their method detection and reporting limits.

| Compound | Method Detection Limit (µg L-1) |  | Reporting Limit (µg L-1) |  |
| --- | --- | --- | --- | --- |
| Organophosphorus in Water (by GC/FPD) |  |  |  |  |
| Azinphos methyl | 0.030 |  | 0.050 |  |
| Chlorpyrifos | 0.010 |  | 0.020 |  |
| Diazinon | 0.005 |  | 0.020 |  |
| Dimethoate | 0.030 |  | 0.050 |  |
| Disulfoton | 0.010 |  | 0.050 |  |
| Malathion | 0.030 |  | 0.050 |  |
| Methidathion | 0.030 |  | 0.050 |  |
| Methyl Parathion | 0.010 |  | 0.050 |  |
| Phorate | 0.030 |  | 0.050 |  |
| Phosmet | 0.030 |  | 0.050 |  |
| Pyrethoids in Water (by GC/ECD) |  |  |  |  |
| Bifenthrin |  | 0.001 |  | 0.002 |
| Cyfluthrin, total1 |  | 0.002 |  | 0.004 |
| Cypermethrin, total2 |  | 0.002 |  | 0.004 |
| Deltamethrin |  | 0.002 |  | 0.004 |
| Esfenvalerate, total3 |  | 0.001 |  | 0.002 |
| Fenpropathrin |  | 0.002 |  | 0.004 |
| λ-Cyhalothrin |  | 0.001 |  | 0.002 |
| Permethrin (cis, trans isomers or mixed isomers) |  | 0.003 |  | 0.005 |
| Pyrethroids in Sediment (by GC/ECD) |  | Method Detection Limit (ng g-1) |  | Reporting Limit (ng g-1) |
| Bifenthrin |  | 0.500 |  | 1.00 |
| Cyfluthrin, total1 |  | 2.00 |  | 4.00 |
| Cypermethrin, total2 |  | 2.00 |  | 4.00 |
| Deltamethrin |  | 2.00 |  | 4.00 |
| Esfenvalerate, total3 |  | 1.00 |  | 2.00 |
| Fenpropathrin |  | 2.00 |  | 4.00 |
| Permethrin, (cis, trans isomers or mixed isomers) |  | 2.00 |  | 5.00 |
| λ-Cyhalothrin, total |  | 2.00 |  | 4.00 |
| 1contains both cyfluthrin and β-cyfluthrin 2contains both cypermethrin and (S)-cypermethrin 3contains both esfenvalerate and fenvalerate |  |  |  |  |

| Table 4. OP and pyrethroid detections and survivability of C. dubia in water samples collected at Del Puerto Creek, California. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Month | Type |  | Azinphos methyl | Chlorpyrifos |  | Diazinon | Dimethoate | Disulfoton | Malathion | Methidathion | Parathion, methyl | Phorate | Bifenthrin | Cyfluthrin | Cypermethrin | Deltamethrin | Esfenvalerate | Fenpropathrin | λ-Cyhalothrin | Permethrin, cis | Permethrin, trans | C. dubia survival |
|  | Storm | Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | % of control |
|  |  |  | -----------------------------------------------------------Concentration (μg L-1) ---------------------------------------------------------- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec | X |  | nd1 |  | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| Dec |  | X | Missing values (MV), no water in creek to sample |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MV |
| Jan | X |  | nd | nd |  | 0.100 | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| Jan |  | X | nd | nd |  | 0.101 | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| Feb | X |  | nd | nd |  | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| Feb |  | X | nd | nd |  | trace2 | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| Mar |  | X | nd | nd |  | trace3 | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| Apr |  | X | nd | nd |  | nd | 0.074 | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| May |  | X | nd | nd |  | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| June 10 |  | X | nd | nd |  | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| June 26 |  | X | nd | 0.035 |  | trace | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| 1nd, not detected (below the minimum detection limit) 2trace detection (below the reporting limit but above the minimum detection limit) 3A field blank sample also was analyzed as a trace detection of diazinon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Table 5. OP and pyrethroid detections and survivability of C. dubia in water samples collected at Orestimba Creek, California. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Month | Type |  | Azinphos methyl | Chlorpyrifos |  | Diazinon | Dimethoate | Disulfoton | Malathion | Methidathion | Parathion, methyl | Phorate | Bifenthrin | Cyfluthrin | Cypermethrin | Deltamethrin | Esfenvalerate | Fenpropathrin | λ-Cyhalothrin | Permethrin, cis | Permethrin, trans | C. dubia survival |
|  | Storm | Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | % of control‡ |
|  |  |  | -----------------------------------------------------------Concentration (μg L-1) ---------------------------------------------------------- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec | X |  | nd1 |  | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| Dec |  | X | nd |  | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 105 |
| Jan | X |  | nd |  | nd | nd | nd | nd | nd | nd | nd | nd | nd | 0.021 | nd | nd | nd | nd | nd | nd | nd | 95 |
| Jan |  | X | nd |  | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| Feb | X |  | nd |  | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| Feb |  | X | nd |  | nd | 0.912 | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| Mar |  | X | nd |  | nd | nd | nd | nd | nd | nd | nd | nd | 0.005 | nd | nd | nd | nd | nd | 0.016 | nd | nd | 100 |
| Apr |  | X | nd |  | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 100 |
| May |  | X | nd |  | 0.079 | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 0 |
| June 10 |  | X | nd |  | trace2 | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 96 |
| June 26 |  | X | nd |  | 0.046 | nd | 0.190 | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | nd | 65 |
| 1nd, not detected (below the minimum detection limit) 2trace detection (below the reporting limit but above the minimum detection limit) ‡Bolded and highlighted cells are significantly different from the control water |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Table 6. Grain size, percent organic carbon, pyrethroid detections, and survivability of H. azteca in sediments collected at Del Puerto Creek and Orestimba Creeks, California. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Month collected | Collection type |  | Grain Size (% fines1) | Total Organic Carbon (%) | Bifenthrin | Cyfluthrin | Cypermethrin | Deltamethrin | Esfenvalerate | Fenpropathrin | Permethrin, cis | Permethrin, trans | λ-Cyhalothrin | H. azteca survival |
|  | Storm | Non-storm Monthly |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ------------------------- Concentration (ng g-1, dry weight) ------------------------- |  |  |  |  |  |  |  |  | % of control‡ |
| DPC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec | X |  | 28.8. | 0.50 | nd2 | nd | nd | nd | trace3 | nd | nd | nd | nd | 18 / 354 |
| Dec |  | X | Missing values, no water in creek to sample |  |  |  |  |  |  |  |  |  |  |  |
| Jan | X |  | Missing values, current too swift to collect sediment samples |  |  |  |  |  |  |  |  |  |  |  |
| Jan |  | X | 64.6 | 1.11 | 8.57 | nd | nd | nd | nd | nd | nd | nd | nd | 0 |
| Feb | X |  | 82.0 | 1.18 | 24.4 | nd | nd | nd | nd | nd | nd | nd | nd | 0 |
| Feb |  | X | 85.9 | 0.78 | 17.4 | nd | nd | nd | nd | nd | nd | nd | trace | 0 |
| Mar |  | X | 86.2 | 0.89 | 69.06 | 4.19 | nd | nd | nd | nd | nd | nd | 3.58 | 0 |
| Apr |  | X | 74.6 | 1.11 | 62.68 | trace | nd | nd | nd | nd | nd | nd | 8.98 | 0 |
| May |  | X | 81.6 | 0.96 | 74.4 | nd | nd | nd | nd | nd | nd | nd | nd | 0 |
| June 10 |  | X | 66.1 | 1.20 | 28.2 | nd | nd | nd | nd | nd | nd | nd | nd | 0 |
| June 26 |  | X | 79.9 | 1.39 | 43.6 | nd | nd | nd | nd | nd | nd | nd | nd | 0 |

| Table 6 continued. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Month collected | Collection type |  | Grain Size (% fines1) | % Total Organic Carbon | Bifenthrin | Cyfluthrin | Cypermethrin |  | Deltamethrin | Esfenvalerate | Fenpropathrin | Permethrin, cis | Permethrin, trans | λ-Cyhalothrin | H. azteca survival |  |
|  | Storm | Non-storm Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ------------------------- Concentration (ng g-1, dry weight) ------------------------- |  |  |  |  |  |  |  |  |  | % of control‡ |  |
| ORC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec | X |  | 75.1 | 1.14 | nd2 | nd | nd |  | nd | trace | nd | nd | nd | 55 | 82 / | 564 |
| Dec |  | X | 81.1 | 1.28 | 2.9 | nd | nd |  | nd | nd | nd | nd | nd | nd | 72 |  |
| Jan | X |  | Missing values, current too swift to collect sediment samples |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Jan |  | X | 58.0 | 0.95 | nd | nd | nd |  | nd | nd | nd | nd | nd | nd | 1576 |  |
| Feb | X |  | 24.8 | 0.28 | nd | nd | nd |  | nd | nd | nd | nd | nd | nd | 84 |  |
| Feb |  | X | 15.3 | 0.31 | nd | nd | nd |  | nd | nd | nd | nd | nd | nd | 56 |  |
| Mar |  | X | 55.7 | 0.78 | nd | nd | nd |  | nd | nd | nd | nd | nd | nd | 91 |  |
| Apr |  | X | 61.1 | 1.29 | nd | nd | nd |  | nd | nd | nd | nd | nd | nd | 55 |  |
| May |  | X | 65.9 | 1.16 | nd | nd | nd |  | nd | nd | nd | nd | nd | nd | 91 |  |
| June 10 |  | X | 53.8 | 1.17 | 36.3 | nd | nd |  | nd | nd | nd | nd | nd | nd | 94 |  |
| June 26 |  | X | 72.4 | 1.31 | nd | nd | nd |  | nd | nd | nd | nd | nd | nd | 78 |  |
| 1Fines are silt and clay particles < 62.5 μm in size 2nd, not detected (below the minimum detection limit) 3trace detection (below the reporting limit but above the minimum detection limit) 4Field sample and field duplicate, respectively |  |  |  |  |  |  |  | 5Average of two lab analyses. 6Test did not meet USEPA criteria for control survival ‡Bolded and highlighted cells are significantly different from the control sediment |  |  |  |  |  |  |  |  |

# Figure 1. Sampling sites at Orestimba Creek and at Del Puerto Creek, California. GPS Coordinates (NAD83): ORC, N37.41395 W121.01495; DPC, N37.52145 W121.14863. ORC and DPC watersheds have previously been described by Zhang et al. (2008).

Figure 2. Box plots of water quality measurements at Del Puerto Creek (DPC) and at Orestimba Creek (ORC). P values indicate significant differences at the 5% level in paired t-tests of the mean differences between the two creeks (DPC greater than ORC; ns, not significant).

ns

p = 0.015

p = 0.006

p = 0.016

p = 0.021

# Figure 3. Toxicity units (TUs) of pyrethroids from sediment samples taken at Del Puerto Creek. TUs for trace detections were calculated using the MDL values.

# Figure 4. Toxicity units (TUs) of pyrethroids from sediment samples collected at Orestimba Creek. TUs for trace detections were calculated using the MDL values.

# Figure 5. Relationship between the sum of pyrethroid toxicity units (TUs) in sediments and the toxicity to H. azteca. Non-detects were arbitrarily given a value of 0.001 TU and trace detections were calculated using MDL values.

# Appendix 1. QC Data for Water Samples

| Table A1. Holding times of the water samples for chemical analysis collected from Del Puerto and Orestimba Creeks. Red highlighted cells indicate exceedances. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample No, | Sampling Date | Date Received at Lab | Preservation Date | Sampling to Preservation (Days) | Extraction Date | Sampling to Extraction (Days) | Analysis Date | Sampling to Analysis (Days) |
| Chemistry – OP1 |  |  |  |  |  |  |  |  |
| 101 | 18 Dec 07 | 19 Dec 07 | None | NA2 | 22 Dec 07 | 4 | 15 Jan 08 | 28 |
| 105 | 18 Dec 07 | 19 Dec 07 | None | NA | 22 Dec 07 | 4 | 15 Jan 08 | 28 |
| 110 | 26 Dec 07 | 27 Dec 07 | None | NA | 27 Dec 07 | 1 | 15 Jan 08 | 20 |
| 114 | 5 Jan 08 | 7 Jan 08 | None | NA | 8 Jan 08 | 3 | 15 Jan 08 | 10 |
| 119 | 5 Jan 08 | 7 Jan 08 | None | NA | 8 Jan 08 | 3 | 15 Jan 08 | 10 |
| 124 | 5 Jan 08 | 7 Jan 08 | None | NA | 8 Jan 08 | 3 | 15 Jan 08 | 10 |
| 125 | 14 Jan 08 | 14 Jan 08 | 14 Jan 08 | 0 | 19 Jan 08 | 5 | 13 Feb 08 | 30 |
| 129 | 14 Jan 08 | 14 Jan 08 | 14 Jan 08 | 0 | 19 Jan 08 | 5 | 13 Feb 08 | 30 |
| 133 | 20 Feb 08 | 20 Feb 08 | 20 Feb 08 | 0 | 22 Feb 08 | 2 | 7 Mar 08 | 16 |
| 137 | 20 Feb 08 | 20 Feb 08 | 20 Feb 08 | 0 | 22 Feb 08 | 2 | 7 Mar 08 | 16 |
| 141 | 28 Feb 08 | 28 Feb 08 | None | NA | 2 Mar 08 | 3 | 7 Mar 08 | 8 |
| 145 | 28 Feb 08 | 28 Feb 08 | None | NA | 2 Mar 08 | 3 | 7 Mar 08 | 8 |
| 151 | 18 Mar 08 | 18 Mar 08 | None | NA | 22 Mar 08 | 4 | 2 Apr 08 | 15 |
| 156 | 18 Mar 08 | 18 Mar 08 | None | NA | 22 Mar 08 | 4 | 2 Apr 08 | 15 |
| 160 | 18 Mar 08 | 18 Mar 08 | None | NA | 22 Mar 08 | 4 | 2 Apr 08 | 15 |
| 161 | 8 Apr 08 | 8 Apr 08 | None | NA | 10 Apr 08 | 2 | 21 Apr 08 | 13 |
| 166 | 8 Apr 08 | 8 Apr 08 | None | NA | 10 Apr 08 | 2 | 21 Apr 08 | 13 |
| 171 | 19 May 08 | 20 May 08 | None | NA | 22 May 08 | 3 | 6 June 08 | 18 |
| 177 | 19 May 08 | 20 May 08 | None | NA | 22 May 08 | 3 | 6 June 08 | 18 |
| 201 | 19 May 08 | 20 May 08 | None | NA | 22 May 08 | 3 | 6 June 08 | 18 |
| 204 | 19 May 08 | 20 May 08 | None | NA | 22 May 08 | 3 | 6 June 08 | 18 |
| 185 | 10 June 08 | 11 June 08 | None | NA | 11 June 08 | 2 | 2 July 08 | 22 |
| 190 | 10 June 08 | 11 June 08 | None | NA | 11 June 08 | 2 | 2 July 08 | 22 |
| 181 | 26 June 08 | 27 June 08 | None | NA | 1 July 08 | 5 | 1 Aug 08 | 36 |
| 211 | 26 June 08 | 27 June 08 | None | NA | 1 July 08 | 5 | 1 Aug 08 | 36 |
| Chemistry – pyrethroid3 |  |  |  |  |  |  |  |  |
| 102 | 18 Dec 07 | 19 Dec 07 | None | NA | 22 Dec 07 | 4 | 17 Jan 08 | 30 |
| 106 | 18 Dec 07 | 19 Dec 07 | None | NA | 22 Dec 07 | 4 | 17 Jan 08 | 30 |
| 111 | 26 Dec 07 | 27 Dec 07 | None | NA | 27 Dec 07 | 1 | 17 Jan 08 | 22 |
| 115 | 5 Jan 08 | 7 Jan 08 | None | NA | 8 Jan 08 | 3 | 17 Jan 08 | 12 |
| 120 | 5 Jan 08 | 7 Jan 08 | None | NA | 8 Jan 08 | 3 | 17 Jan 08 | 12 |
| 126 | 14 Jan 08 | 14 Jan 08 | 14 Jan 08 | 0 | 19 Jan 08 | 5 | 6 Feb 08 | 23 |
| 130 | 14 Jan 08 | 14 Jan 08 | 14 Jan 08 | 0 | 19 Jan 08 | 5 | 6 Feb 08 | 23 |
| 134 | 20 Feb 08 | 20 Feb 08 | 22 Feb 08 | 2 | 22 Feb 08 | 2 | 19 Mar 08 | 28 |
| 138 | 20 Feb 08 | 20 Feb 08 | 22 Feb 08 | 2 | 22 Feb 08 | 2 | 19 Mar 08 | 28 |
| 142 | 28 Feb 08 | 28 Feb 08 | 29 Feb 08 | 1 | 2 Mar 08 | 3 | 19 Mar 08 | 20 |
| 146 | 28 Feb 08 | 28 Feb 08 | 29 Feb 08 | 1 | 2 Mar 08 | 3 | 19 Mar 08 | 20 |
| 149 | 28 Feb 08 | 28 Feb 08 | 29 Feb 08 | 1 | 2 Mar 08 | 3 | 19 Mar 08 | 20 |
| 152 | 18 Mar 08 | 18 Mar 08 | None | NA | 20 Mar 08 | 2 | 26 Mar 08 | 8 |
| 157 | 18 Mar 08 | 18 Mar 08 | None | NA | 20 Mar 08 | 2 | 26 Mar 08 | 8 |

| Table A1, continued. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample No, | Sampling Date |  | Date Received at Lab |  | Preservation Date |  | Sampling to Preservation (Days) |  | Extraction Date | Sampling to Extraction (Days) |  | Analysis Date |  | Sampling to Analysis (Days) |
| 162 | 8 Apr 08 |  | 8 Apr 08 |  | None |  | NA |  | 10 Apr 08 | 2 |  | 19 May08 |  | 41 |
| 167 | 8 Apr 08 |  | 8 Apr 08 |  | None |  | NA |  | 10 Apr 08 | 2 |  | 19 May08 |  | 41 |
| 170 | 8 Apr 08 |  | 8 Apr 08 |  | None |  | NA |  | 10 Apr 08 | 2 |  | 19 May08 |  | 41 |
| 174 | 19 May 08 |  | 20 May 08 |  | None |  | NA |  | 23 May 08 | 4 |  | 12 June 08 |  | 24 |
| 178 | 19 May 08 |  | 20 May 08 |  | None |  | NA |  | 23 May 08 | 4 |  | 12 June 08 |  | 24 |
| 202 | 19 May 08 |  | 20 May 08 |  | None |  | NA |  | 23 May 08 | 4 |  | 12 June 08 |  | 24 |
| 205 | 19 May 08 |  | 20 May 08 |  | None |  | NA |  | 23 May 08 | 4 |  | 12 June 08 |  | 24 |
| 186 | 10 June 08 |  | 11 June 08 |  | None |  | NA |  | 12 June 08 | 2 |  | 27 June 08 |  | 17 |
| 191 | 10 June 08 |  | 11 June 08 |  | None |  | NA |  | 12 June 08 | 2 |  | 27 June 08 |  | 17 |
| 182 | 26 June 08 |  | 27 June 08 |  | None |  | NA |  | 30 June 08 | 4 |  | 5 Aug 08 |  | 40 |
| 212 | 26 June 08 |  | 27 June 08 |  | None |  | NA |  | 30 June 08 | 4 |  | 5 Aug 08 |  | 40 |
| 1OPs can be held 7 days prior to extraction without chemical preservation. Analysis within 40 days. 2Not applicable, chemical preservation was not needed prior to extraction. 3Pyrethroids can be held up to 4 days prior to extraction without chemical preservation. Analysis within 40 days. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Table A2. Percent recovery and RPD1 of surrogate samples in water and in QC samples. Surrogates were triphenyl phosphate for OPs and dibromooctafluorobiphenyl for pyrethroids. Acceptable range of surrogate recovery was 50 – 150%. Yellow highlighted consecutive rows indicate samples that were also analyzed for RPD among the surrogates. Red highlighted cells indicate exceedances. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OP Water Sample No. |  | Percent Recovery of Triphenyl Phosphate (OPs) |  | Percent RPD among LCS/LCSD |  | Pyrethroid Water Sample No. |  | Percent Recovery of Dibromo- octafluorobiphenyl (Pyrethroids) |  |  | Percent RPD among LCS/LCSD |  |  |  |
| 101 |  | 90.0 |  |  |  | 102 |  | 90.4 |  |  |  |  |  |  |
| 105 |  | 81.5 |  |  |  | 106 |  | 98.6 |  |  |  |  |  |  |
| 110 |  | 72.5 |  |  |  | 111 |  | 105.0 |  |  |  |  |  |  |
| MB2 |  | 81.5 |  |  |  | MB |  | 95.5 |  |  |  |  |  |  |
| LCS |  | 127 |  | 43.1 |  | LCS |  | 95.6 |  |  | 3.9 |  |  |  |
| LCSD |  | 82.0 |  |  |  | LCSD |  | 99.4 |  |  |  |  |  |  |
| 114 |  | 78.0 |  |  |  | 115 |  | 95.6 |  |  |  |  |  |  |
| 119 |  | 83.5 |  |  |  | 120 |  | 85.5 |  |  |  |  |  |  |
| 124 |  | 83.0 |  |  |  | MB |  | 87 |  |  |  |  |  |  |
| MB |  | 70.5 |  |  |  | LCS |  | 82.6 |  |  | 4.0 |  |  |  |
| LCS |  | 126.0 |  | 21.1 |  | LCSD |  | 86 |  |  |  |  |  |  |
| LCSD |  | 102 |  |  |  | 126 |  | 80.4 |  |  |  |  |  |  |
| 125 |  | 108 |  |  |  | 130 |  | 97.4 |  |  |  |  |  |  |
| 129 |  | 94.8 |  |  |  | MB |  | 87.2 |  |  |  |  |  |  |
| MB |  | 98.2 |  |  |  | LCS |  | 79.7 |  |  | 5.6 |  |  |  |
|  |  |  |  |  |  | LCSD |  | 84.3 |  |  |  |  |  |  |
| Table A2, continued. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OP Water Sample No. |  | Percent Recovery of Triphenyl Phosphate (OPs) |  | Percent RPD among LCS/LCSD |  | Pyrethroid Water Sample No. |  | Percent Recovery of Dibromo- octafluorobiphenyl (pyrethroids) |  |  | Percent RPD among LCS/LCSD |  |  |  |
| LCS |  | 95.7 |  | 2.3 |  |  |  |  |  |  |  |  |  |  |
| LCSD |  | 93.5 |  |  |  | 134 |  | 80.6 |  |  |  |  |  |  |
| 133 |  | 96.0 |  |  |  | 138 |  | 75.5 |  |  |  |  |  |  |
| 137 |  | 97.6 |  |  |  | MB |  | 82.2 |  |  |  |  |  |  |
| MB |  | 107 |  |  |  | LCS |  | 78.2 |  |  | 4.7 |  |  |  |
| LCS |  | 99.2 |  | 2.8 |  | LCSD |  | 74.6 |  |  |  |  |  |  |
| LCSD |  | 102 |  |  |  | 146 |  | 82.1 |  |  |  |  |  |  |
| 141 |  | 108 |  |  |  | 142 |  | 90.8 |  |  |  |  |  |  |
| 145 |  | 98 |  |  |  | 149 |  | 88.9 |  |  |  |  |  |  |
| MB |  | 104 |  |  |  | MB |  | 78.6 |  |  |  |  |  |  |
| LCS |  | 98.2 |  | 1.8 |  | LCS |  | 75.1 |  |  | 10.0 |  |  |  |
| LCSD |  | 100 |  |  |  | LCSD |  | 83 |  |  |  |  |  |  |
| 151 |  | 79.8 |  |  |  | 152 |  | 94.0 |  |  |  |  |  |  |
| 156 |  | 108 |  |  |  | 157 |  | 118 |  |  |  |  |  |  |
| 160 |  | 110 |  |  |  | MB |  | 89.6 |  |  |  |  |  |  |
| MB |  | 118 |  |  |  | LCS |  | 87.6 |  |  | 16.2 |  |  |  |
| LCS |  | 106 |  | 17.2 |  | LCSD |  | 103 |  |  |  |  |  |  |
| LCSD |  | 126 |  |  |  | 162 |  | 108 |  |  |  |  |  |  |
| 161 |  | 72.2 |  |  |  | 167 |  | 110 |  |  |  |  |  |  |
| 166 |  | 86.0 |  |  |  | 170 |  | 117 |  |  |  |  |  |  |
| MB |  | 104 |  |  |  | MB |  | 109 |  |  |  |  |  |  |
| LCS |  | 77.8 |  | 4.2 |  | LCS |  | 113 |  |  | 3.6 |  |  |  |
| LCSD |  | 74.6 |  |  |  | LCSD |  | 109 |  |  |  |  |  |  |
| 171 |  | 92.8 |  |  |  | 174 |  | 68.9 |  |  |  |  |  |  |
| 177 |  | 94.8 |  |  |  | 205 |  | 77.5 |  |  |  |  |  |  |
| 201 |  | 93.1 |  |  |  | 202 |  | 64.8 |  |  |  |  |  |  |
| 204 |  | 87.4 |  |  |  | 178 |  | 74.5 |  |  |  |  |  |  |
| MB |  | 83.7 |  |  |  | MB |  | 73.8 |  |  |  |  |  |  |
| LCS |  | 105 |  | 17.7 |  | LCS |  | 80.3 |  |  | 4.7 |  |  |  |
| LCSD |  | 87.9 |  |  |  | LCSD |  | 84.2 |  |  |  |  |  |  |
| 185 |  | 103 |  |  |  | 186 |  | 93.9 |  |  |  |  |  |  |
| 190 |  | 109 |  |  |  | 191 |  | 86.8 |  |  |  |  |  |  |
| MB |  | 97.1 |  |  |  | MB |  | 74.3 |  |  |  |  |  |  |
| LCS |  | 99.4 |  | 9.2 |  | LCS |  | 76.9 |  |  | 2.9 |  |  |  |
| LCSD |  | 109 |  |  |  | LCSD |  | 74.7 |  |  |  |  |  |  |
| 181 |  | 110 |  |  |  | 182 |  | 97.7 |  |  |  |  |  |  |
| 211 |  | 89.1 |  |  |  | 212 |  | 96.5 |  |  |  |  |  |  |
| MB |  | 96.5 |  |  |  | MB |  | 87.4 |  |  |  |  |  |  |
| LCS |  | 107 |  | 16.7 |  | LCS |  | 90.2 |  |  | 6.4 |  |  |  |
| LCSD |  | 90.5 |  |  |  | LCSD |  | 84.6 |  |  |  |  |  |  |
| 1Relative percent differences; acceptable RPDs are < 25% 2MB, method blank; LCS, laboratory control sample; LCSD, LCS duplicate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Table A3. Percent recoveries of spiked OP QC water samples. RPD1 were calculated between the LCS and LCSD and are highlighted in yellow below the LCSD. Acceptable range for RPD were < 25%. Exceedances or areas of QC concern are bolded and highlighted in red |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QC Sample |  | Sampling Date | Azinphos methyl | Chlorpyrifos | Diazinon | Dimethoate | Disulfoton | Malathion | Methidathion | Methyl Parathion | Phorate | Phosmet |
|  |  |  | ------------- Percent Recovery or Percent RPD of QC Water Samples ------------- |  |  |  |  |  |  |  |  |  |
| LCS |  | 18/26 Dec 07 | 107 | 108 | 99.4 | 108 | 106 | 99.8 | 110 | 96.7 | 102 | 128 |
| LCSD |  | 18/26 Dec 07 | 101 | 94.3 | 97.9 | 108 | 90.7 | 110 | 108 | 101 | 82.5 | 110 |
| Percent RPD |  |  | 5.8 | 13.5 | 1.5 | 0 | 15.6 | 9.7 | 1.8 | 4.4 | 21.1 | 15.1 |
| LCS |  | 5 Jan 08 | 127 | 106 | 96.4 | 116 | 96.1 | 103 | 102 | 99.1 | 106 | 99.5 |
| LCSD |  | 5 Jan 08 | 108 | 93.3 | 85.9 | 79.9 | 96.1 | 105 | 104 | 93.4 | 94.9 | 114 |
| Percent RPD |  |  | 16.2 | 12.7 | 11.5 | 36.9 | 0 | 1.9 | 1.9 | 5.9 | 11.1 | 13.6 |
| LCS |  | 14 Jan 08 | 124 | 89.9 | 91.7 | 94.0 | 100 | 87.5 | 85.6 | 90.4 | 87 | 75.6 |
| LCSD |  | 14 Jan 08 | 120 | 101.0 | 94.0 | 86.4 | 88.9 | 101.0 | 91.0 | 86.5 | 89 | 99.3 |
| Percent RPD |  |  | 3.3 | 11.6 | 2.5 | 8.4 | 11.8 | 14.3 | 6.1 | 4.4 | 3.0 | 27.1 |
| LCS | 20 Feb 08 |  | 87.1 | 104 | 107 | 201 | 99.8 | 106 | 105 | 99.6 | 91.4 | 57.3 |
| LCSD | 20 Feb 08 |  | 106 | 93.9 | 105 | 179 | 86.9 | 97.1 | 102 | 93.4 | 94.0 | 44.3 |
| Percent RPD |  |  | 19.6 | 10.2 | 1.9 | 11.6 | 13.8 | 8.8 | 2.9 | 6.4 | 2.8 | 25.6 |
| LCS | 28 Feb 08 |  | 115 | 94.4 | 107 | 94.9 | 99.0 | 95.8 | 103 | 94.1 | 99.6 | 65.2 |
| LCSD | 28 Feb 08 |  | 137 | 105 | 112 | 90.0 | 101 | 107 | 109 | 98.3 | 96.8 | 59.2 |
| Percent RPD |  |  | 17.5 | 10.6 | 4.6 | 5.3 | 2.0 | 11.0 | 5.7 | 4.4 | 2.9 | 9.6 |
| LCS | 18 Mar 08 |  | 94.9 | 106 | 98.0 | 75.5 | 90.7 | 112 | 71.0 | 91.4 | 95.1 | 53.8 |
| LCSD | 18 Mar 08 |  | 110 | 118 | 121 | 89.7 | 105 | 78.4 | 89.5 | 123 | 111 | 84.0 |
| Percent RPD |  |  | 14.7 | 10.7 | 21.0 | 17.2 | 14.6 | 35.3 | 23.1 | 29.5 | 15.4 | 43.8 |
| LCS | 8 Apr 08 |  | 101 | 73.7 | 90.3 | 98.1 | 97.6 | 112 | 72.9 | 94.8 | 106 | 75.6 |
| LCSD | 8 Apr 08 |  | 100 | 122 | 82.6 | 98.5 | 112 | 70.4 | 89.4 | 112 | 114 | 73.9 |
| Percent RPD |  |  | 1.0 | 49.4 | 8.9 | 0.4 | 13.7 | 45.6 | 20.3 | 16.6 | 7.3 | 2.3 |
| LCS | 19 May 08 |  | 90.1 | 105 | 87.0 | 99.1 | 87.4 | 86.8 | 91.8 | 79.0 | 90.0 | 116 |
| LCSD | 19 May 08 |  | 82.6 | 112 | 104 | 92.6 | 89.5 | 84.6 | 77.0 | 81.0 | 70.2 | 100 |
| Percent RPD |  |  | 8.7 | 6.5 | 17.8 | 6.8 | 2.4 | 2.6 | 17.5 | 2.5 | 24.7 | 14.8 |
| LCS | 10 June 08 |  | 96.6 | 97.9 | 99.8 | 93.0 | 80.8 | 87.7 | 85.4 | 94.1 | 90.8 | 90.9 |
| LCSD | 10 June 08 |  | 108 | 97.6 | 103 | 109 | 101 | 96.8 | 99.2 | 100 | 90.1 | 116 |
| Percent RPD |  |  | 11.1 | 0.3 | 3.2 | 15.8 | 22.2 | 9.9 | 15.0 | 6.1 | 0.8 | 24.3 |
| LCS | 26 June 08 |  | 95.6 | 102 | 93.1 | 93.8 | 92.6 | 98.7 | 94.1 | 95.3 | 94.4 | 95.3 |
| LCSD | 26 June 08 |  | 82.7 | 93.6 | 109 | 81.3 | 79.8 | 89.5 | 87.9 | 82.0 | 83.5 | 98.1 |
| Percent RPD |  |  | 14.5 | 8.6 | 15.7 | 14.3 | 14.8 | 9.8 | 6.8 | 15.0 | 12.3 | 2.9 |
| 1RPD, relative percent differences; LCS, laboratory control sample; LCSD, LCS duplicate |  |  |  |  |  |  |  |  |  |  |  |  |

| Table A4. Percent recoveries of spiked pyrethroid QC water samples. RPD1 were calculated between the LCS and LCSD and are highlighted in yellow below the LCSD. Acceptable range for RPD were < 25%. Exceedances or areas of QC concern are bolded and highlighted in red. |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QC Sample | Sampling Date | Bifenthrin | Cyfluthrin | Cypermethrin | Deltamethrin | Esfenvalerate | Fenpropathrin | λ-Cyhalothrin | Permethrin | Permethrin, cis | Permethrin, trans |
|  |  | ----------------- Percent Recovery or Percent RPD of QC Water Samples ------------------ |  |  |  |  |  |  |  |  |  |
| LCS | 18/26 Dec 07 | 82.9 | 84.6 | 75.5 | 85.9 | 102 | 86.6 | 95.8 | 76.8 | NR | NR |
| LCSD | 18/26 Dec 07 | 81 | 85.2 | 73 | 83 | 95.7 | 84.4 | 91.3 | 73.9 | NR | NR |
| Percent RPD |  | 2.3 | 0.7 | 3.4 | 3.4 | 6.4 | 2.6 | 4.8 | 3.8 |  |  |
| LCS | 5 Jan 08 | 75.4 | 86.7 | 75.8 | 83.6 | 77.8 | 77.2 | 89.3 | 74.4 | NR | NR |
| LCSD | 5 Jan 08 | 70.7 | 84.6 | 72.1 | 85.8 | 87.6 | 87.8 | 83.3 | 75.9 | NR | NR |
| Percent RPD |  | 6.4 | 2.5 | 5.0 | 2.6 | 11.9 | 12.8 | 7.0 | 2.0 |  |  |
| LCS | 14 Jan 08 | 68 | 100.0 | 83.4 | 76.4 | 77 | 93.7 | 77.3 | 101.0 | NR | NR |
| LCSD | 14 Jan 08 | 57 | 81.3 | 65.6 | NR | 58.1 | 79.3 | 61.6 | 77.6 | NR | NR |
| Percent RPD |  | 17.6 | 20.6 | 23.9 |  | 28.1 | 16.6 | 22.6 | 26.2 |  |  |
| LCS | 20 Feb 08 | 74.1 | 96.2 | 79.1 | 70.7 | 89.4 | 82.8 | 90.8 | 90.5 | NR | NR |
| LCSD | 20 Feb 08 | 74.2 | 91.1 | 74.7 | 85.5 | 78.8 | 82.0 | 85.2 | 82.9 | NR | NR |
| Percent RPD |  | 0.1 | 5.4 | 5.7 | 19.0 | 12.6 | 1.0 | 6.4 | 8.8 |  |  |
| LCS | 28 Feb 08 | 79.1 | 76.6 | 80.9 | 74.7 | 97.0 | 82.4 | 99.0 | 72.8 | NR | NR |
| LCSD | 28 Feb 08 | 58.4 | 76.5 | 71.5 | 60.8 | 80.0 | 81.2 | 79.3 | 63.3 | NR | NR |
| Percent RPD |  | 30.1 | 0.1 | 12.3 | 20.5 | 19.2 | 1.5 | 22.1 | 14.0 |  |  |
| LCS | 18 Mar 08 | 94.2 | 104 | 122 | 70.0 | 103 | 99.8 | 81.1 | NR | 90.7 | 100 |
| LCSD | 18 Mar 08 | 88.9 | 85.3 | 101 | 84.4 | 65.2 | 124 | 65.5 | NR | 72.0 | 74.8 |
| Percent RPD |  | 5.8 | 19.8 | 18.8 | 18.7 | 44.9 | 21.6 | 21.3 |  | 23.0 | 28.8 |
| LCS | 8 Apr 08 | 95.3 | 116 | 118 | 86.0 | 102 | 117 | 105 | NR | 105 | 118 |
| LCSD | 8 Apr 08 | 92.9 | 118 | 117 | 102 | 107 | 125 | 113 | NR | 107 | 127 |
| Percent RPD |  | 2.6 | 1.7 | 0.9 | 17.0 | 4.8 | 6.6 | 7.3 |  | 1.9 | 7.3 |
| LCS | 19 May 08 | 70.0 | 95.7 | 81.8 | 67.1 | 84.0 | 78.2 | 88.9 | NR | 82.9 | 73.6 |
| LCSD | 19 May 08 | 90.0 | 112 | 106 | 84.5 | 99.0 | 103 | 103 | NR | 111 | 96.0 |
| Percent RPD |  | 25.0 | 15.7 | 25.8 | 23.0 | 16.4 | 27.4 | 14.7 |  | 29.0 | 26.4 |
| LCS | 10 June 08 | 91.8 | 91.0 | 94.6 | 101 | 96.3 | 90.4 | 92.5 | NR | 98.7 | 97.6 |
| LCSD | 10 June 08 | 85.0 | 97.3 | 93.1 | 103 | 100 | 93.8 | 83.6 | NR | 84.8 | 104 |
| Percent RPD |  | 7.7 | 6.7 | 1.6 | 2.0 | 3.8 | 3.7 | 10.1 |  | 15.1 | 6.3 |
| LCS | 26 June 08 | 129 | 102 | 89.5 | 70.7 | 103 | 93.9 | 91.8 | NR | 103 | 135 |
| LCSD | 26 June 08 | 80.7 | 100 | 72.8 | 75.6 | 94.7 | 108 | 102 | NR | 108 | 122 |
| Percent RPD |  | 46.1 | 2.0 | 20.6 | 6.7 | 8.4 | 14.0 | 10.5 |  | 4.7 | 10.1 |
| 1RPD, relative percent differences; LCS, laboratory control sample; LCSD, LCS duplicate; NR, not run |  |  |  |  |  |  |  |  |  |  |  |

| Table A5. QC for field duplicate samples (FD) and field blanks (FB). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sampling Date | 5 Jan 08 |  | 28 Feb 08 |  | 18 Mar 08 | 8 April 08 |
| Sample Type | Field sample | FD | Field sample | FD | FB | FB |
| Pyrethroids |  |  |  |  |  |  |
| Bifenthrin | FD only collected for pyrethroids at this date |  | nd | nd | FB only collected for OP analysis | nd |
| Cyfluthrin |  |  | nd | nd |  | nd |
| Cypermethrin |  |  | nd | nd |  | nd |
| Deltamethrin |  |  | nd | nd |  | nd |
| Esfenvalerate |  |  | nd | nd |  | nd |
| Fenpropathrin |  |  | nd | nd |  | nd |
| λ-Cyhalothrin |  |  | nd | nd |  | nd |
| Permethrin |  |  | nd | nd |  | nd |
| OPs |  |  |  |  |  |  |
| Azinphos methyl | nd | nd | FD only collected for OPs at this date |  | nd | FB only collected for pyrethroid analysis |
| Chlorpyrifos | nd | nd |  |  | nd |  |
| Diazinon | nd | nd |  |  | trace* |  |
| Dimethoate | nd | nd |  |  | nd |  |
| Disulfoton | nd | nd |  |  | nd |  |
| Malathion | nd | nd |  |  | nd |  |
| Methidathion | nd | nd |  |  | nd |  |
| Parathion, Methyl | nd | nd |  |  | nd |  |
| Phorate | nd | nd |  |  | nd |  |
| Phosmet | nd | nd |  |  | nd |  |
| *above the RL but below the MDL |  |  |  |  |  |  |

| Table A6. Holding times water samples collect for TSS (total suspended solids) analysis. Exceedances or areas of QC concern are bolded and highlighted in red. |  |  |  |
| --- | --- | --- | --- |
| Sample id | Sampling Date | Analysis Date | Sampling to Analysis (Days) |
| 103 | 18 Dec 07 | 27 Dec 07 | 9 |
| 107 | 18 Dec 07 | 27 Dec 07 | 9 |
| 109 | 18 Dec 07 | 27 Dec 07 | 9 |
| 112 | 26 Dec 07 | 27 Dec 07 | 1 |
| 117 | 5 Jan 08 | 7 Jan 08 | 2 |
| 121 | 5 Jan 08 | 7 Jan 08 | 2 |
| 121 | 5 Jan 08 | 7 Jan 08 | 2 |
| 123 | 5 Jan 08 | 7 Jan 08 | 2 |
| 116 | 14 Jan 08 | 16 Jan 08 | 2 |
| 127 | 14 Jan 08 | 16 Jan 08 | 2 |
| 131 | 14 Jan 08 | 16 Jan 08 | 2 |
| 135 | 20 Feb 08 | 21 Feb 08 | 1 |
| 139 | 20 Feb 08 | 21 Feb 08 | 1 |
| 143 | 28 Feb 08 | 29 Feb 08 | 1 |
| 147 | 28 Feb 08 | 29 Feb 08 | 1 |
| 150 | 28 Feb 08 | 29 Feb 08 | 1 |
| 153 | 18 Mar 08 | 21 Mar 08 | 3 |
| 155 | 18 Mar 08 | 21 Mar 08 | 3 |
| 158 | 18 Mar 08 | 21 Mar 08 | 3 |
| 163 | 8 Apr 08 | 11 Apr 08 | 3 |
| 165 | 8 Apr 08 | 11 Apr 08 | 3 |
| 168 | 8 Apr 08 | 11 Apr 08 | 3 |
| 175 | 19 May 08 | 17 July 08 | 59 |
| 179 | 19 May 08 | 17 July 08 | 59 |
| 189 | 10 June 08 | 17 Jul 08 | 37 |
| 192 | 10 June 08 | 17 Jul 08 | 37 |
| 193 | 10 June 08 | 17 Jul 08 | 37 |
| 183 | 26 June 08 | 17 Jul 08 | 21 |
| 184 | 26 June 08 | 17 Jul 08 | 21 |
| 213 | 26 June 08 | 17 Jul 08 | 21 |

| Table A7. RPD1 of TSS samples |  |  |  |
| --- | --- | --- | --- |
| Analysis Date | Sample Sediment Weight (mg L-1) | FD2 Sediment Weight (mg L-1) | RPD |
| 27 Dec 07 | 12.39 | 7.42 | 50.2% |
| 7 Jan 08 | 1766.45 | 1703.38 | 3.6% |
| 16 Jan 08 | 116 | 30.45 | 1.9% |
| 29 Feb 08 | 4.81 | 5.89 | 20.2% |
| 21 Mar 08 | 11.64 | 12.35 | 5.9% |
| 11 Apr 08 | 7.22 | 6.39 | 12.2% |
| 17 Jul 08 | 98.1 | 104.4 | 6.3% |
| 17 Jul 08 | 84.3 | 88.5 | 4.9% |
| 1Relative percent differences. 2Field duplicate |  |  |  |

# Appendix 2. QC Data for Sediment Samples

| Table A8. Holding times for sediment samples for chemical analysis collected from Del Puerto and Orestimba Creeks. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sediment Sample No. | Sampling Date | Date Received at CDFG Lab | Date Extracted | Sampling to Extraction (days) | Date analyzed | Sampling to Analysis (days)1 |
| 302 | 18 Dec 07 | 7 Jan 08 | 12 Mar 08 | 85 | 7 Apr 08 | 111 |
| 306 | 18 Dec 07 | 7 Jan 08 | 12 Mar 08 | 85 | 7 Apr 08 | 111 |
| 310 | 26 Dec 07 | 7 Jan 08 | 12 Mar 08 | 77 | 7 Apr 08 | 103 |
| 314 | 14 Jan 08 | 28 Feb 08 | 12 Mar 08 | 58 | 7 Apr 08 | 84 |
| 319 | 14 Jan 08 | 28 Feb 08 | 12 Mar 08 | 58 | 7 Apr 08 | 84 |
| 321 | 20 Feb 08 | 28 Feb 08 | 12 Mar 08 | 21 | 7 Apr 08 | 47 |
| 325 | 20 Feb 08 | 28 Feb 08 | 12 Mar 08 | 21 | 7 Apr 08 | 47 |
| 329 | 28 Feb 08 | 28 Feb 08 | 12 Mar 08 | 13 | 7 Apr 08 | 39 |
| 333 | 28 Feb 08 | 28 Feb 08 | 12 Mar 08 | 13 | 7 Apr 08 | 39 |
| 338 | 18 Mar 08 | 8 Apr 08 | 17 Apr 08 | 30 | 19 May 08 | 62 |
| 341 | 18 Mar 08 | 8 Apr 08 | 17 Apr 08 | 30 | 19 May 08 | 62 |
| 344 | 18 Mar 08 | 8 Apr 08 | 17 Apr 08 | 30 | 19 May 08 | 62 |
| 349 | 8 Apr 08 | 8 Apr 08 | 17 Apr 08 | 9 | 19 May 08 | 41 |
| 353 | 8 Apr 08 | 8 Apr 08 | 17 Apr 08 | 9 | 19 May 08 | 41 |
| 357 | 19 May 08 | 11 June 08 | 1 July 08 | 43 | 5 Aug 08. | 78 |
| 361 | 19 May 08 | 11 June 08 | 1 July 08 | 43 | 5 Aug 08 | 78 |
| 365 | 10 June 08 | 11 June 08 | 1 July 08 | 21 | 5 Aug 08 | 56 |
| 369 | 10 June 08 | 11 June 08 | 1 July 08 | 21 | 5 Aug 08 | 56 |
| 375 | 26 June 08 | 26 June 08 | 1 July 08 | 5 | 5 Aug 08 | 40 |
| 380 | 26 June 08 | 26 June 08 | 1 July 08 | 5 | 5 Aug 08 | 40 |
| 1Sediments for chemical analysis can be held up to 6 months at -20ºC |  |  |  |  |  |  |

| Table A9. Holding times for the sediment samples for TOC (total organic carbon) and grain size analysis. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sampling Date1 | TOC Analysis |  | Grain Size Analysis |  |
|  | Analysis Date | Sampling to Analysis Time (Months) | Analysis Date | Sampling to Analysis Time (Months) |
| 18 Dec 07 | 11 June 08 | 5.87 | 25 June 09 | 7.3 |
| 26 Dec 07 | 12 June 08 | 5.63 | 25 June 09 | 7.0 |
| 14 Jan 08 | 12 June 08 | 5.00 | 25 June 09 | 6.4 |
| 20 Feb 08 | 12 June 08 | 3.77 | 25 June 09 | 5.2 |
| 28 Feb 08 | 12 June 08 | 3.50 | 25 June 09 | 4.9 |
| 18 Mar 08 | 12 June 08 | 2.87 | 25 June 09 | 4.3 |
| 8 Apr 08 | 13 June 08 | 2.20 | 25 June 09 | 3.6 |
| 19 May 08 | 13 June 08 | 0.83 | 25 June 09 | 2.2 |
| 10 June 08 | 13 June 08 | 0.10 | 25 June 09 | 1.5 |
| 26 June 08 | 3 Nov 08 | 4.3 | 25 June 09 | 1.0 |
| 1Sediment samples were collected for TOC and grain size on the same date, and samples were collected from both Orestimba and Del Puerto Creeks. |  |  |  |  |

| Table A10. Percent recovery and RPD1 of surrogate samples in sediment and in QC samples. Acceptable range of surrogate recovery was 50 – 150%. Highlighted consecutive rows (of the same color) indicate samples that were also analyzed for RPD among the surrogates or duplicates. Acceptable range for RPD were < 25%. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Surrogate |  |  | Dibutylchlorendate |  | Dibromooctafluorobiphenyl |  |
| Sediment Sample Number2 | Sampling Date | Analysis Date | Percent Recovery | Percent RPD | Percent Recovery | Percent RPD |
| 302 | 18 Dec 07 | 7 Apr 08 | 115.0 | 12.0 | 92.7 | 1.3 |
| 302 LD | 18 Dec 07 | 7 Apr 08 | 102.0 |  | 93.9 |  |
| 306 | 18 Dec 07 | 7 Apr 08 | 95.3 |  | 88.5 |  |
| 310 | 26 Dec 07 | 7 Apr 08 | 102.0 |  | 89.6 |  |
| 314 | 14 Jan 08 | 7 Apr 08 | 89.8 |  | 87.0 |  |
| 319 | 14 Jan 08 | 7 Apr 08 | 96.0 |  | 103.0 |  |
| 321 | 20 Feb 08 | 7 Apr 08 | 106.0 |  | 98.7 |  |
| 325 | 20 Feb 08 | 7 Apr 08 | 106.0 |  | 102.0 |  |
| 329 | 28 Feb 08 | 7 Apr 08 | 114.0 |  | 92.2 |  |
| 333 | 28 Feb 08 | 7 Apr 08 | 99.8 |  | 95.5 |  |
| MB | NA | 7 Apr 08 | 103.0 |  | 100.0 |  |
| LCS | NA | 7 Apr 08 | 106.0 |  | 72.3 |  |
| MS | NA | 7 Apr 08 | 102.0 | 4.6 | 99.5 | 3.5 |
| MSD | NA | 7 Apr 08 | 97.4 |  | 96.1 |  |
| 338 | 18 Mar 08 | 19 May 08 | 91.3 |  | 97.8 |  |
| 341 | 18 Mar 08 | 19 May 08 | 88.0 |  | 97.4 |  |
| 349 | 8 Apr 08 | 19 May 08 | 78.5 | 7.5 | 89.9 | 2.2 |
| 349 LD | 8 Apr 08 | 19 May 08 | 84.6 |  | 91.9 |  |
| 353 | 8 Apr 08 | 19 May 08 | 90.2 |  | 92.8 |  |
| 344 | 18 Mar 08 | 19 May 08 | 87.3 |  | 87.4 |  |
| MB | NA | 19 May 08 | 90.1 |  | 87.9 |  |
| LCS | NA | 19 May 08 | 81.2 | 1.7 | 71.2 | 1.0 |
| LCSD | NA | 19 May 08 | 82.6 |  | 70.5 |  |
| MS | NA | 19 May 08 | 86.8 | 4.3 | 68.8 | 15.1 |
| MSD | NA | 19 May 08 | 90.6 |  | 80.0 |  |
| 357 | 19 May 08 | 5 Aug 08 | 103 |  | 89.2 |  |
| 361 | 19 May 08 | 5 Aug 08 | 92.6 | 3.1 | 94.4 | 0.8 |
| 361 LD | 19 May 08 | 5 Aug 08 | 95.5 |  | 95.2 |  |
| 365 | 10 June 08 | 5 Aug 08 | 97.5 |  | 100 |  |
| 369 | 10 June 08 | 5 Aug 08 | 73.1 |  | 71.3 |  |
| 375 | 26 June 08 | 5 Aug 08 | 96.6 |  | 95.2 |  |
| 380 | 26 June 08 | 5 Aug 08 | 98.3 |  | 96.3 |  |
| MB | NA | 5 Aug 08 | 86.5 |  | 80.9 |  |
| Table A10 continued. |  |  |  |  |  |  |
| Surrogate |  |  | Dibutylchlorendate |  | Dibromooctafluorobiphenyl |  |
| Sediment Sample Number | Sampling Date | Analysis Date | Percent Recovery | Percent RPD | Percent Recovery | Percent RPD |
| LCS | NA | 5 Aug 08 | 82.4 | 7.3 | 86.8 | 2.4 |
| LCSD | NA | 5 Aug 08 | 76.6 |  | 88.9 |  |
| MS | NA | 5 Aug 08 | 102 | 10.3 | 97.8 | 0 |
| MDS | NA | 5 Aug 08 | 92 |  | 97.8 |  |
| 1Relative percent differences 2MB, method blank; LCS, laboratory control sample; LCSD, LCS duplicate; MS, matrix spike; MSD, MS duplicate, LD, lab duplicate |  |  |  |  |  |  |

| Table A11. Percent recoveries of spiked QC sediment samples. RPD1 were calculated between the laboratory control spikes and laboratory control spike duplicates or between the matrix spikes and matrix spike duplicates and are highlighted in yellow in the row below the respective calculation. Acceptable range for RPD were < 25%. Exceedances or areas of QC concern are bolded and highlighted in red. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | -------- Percent Recovery or Percent RPD of QC Sediment Samples -------- |  |  |  |  |  |  |  |  |
| QC | Analysis Date | Bifenthrin | Cyfluthrin | Cypermethrin | Deltamethrin | Esfenvalerate | Fenpropathrin | Permethrin cis | Permethrin trans | λ-Cyhalothrin |
| LCS2 | 7 Apr 08 | 72.4 | 100.0 | 92.5 | 72.6 | 94.0 | 94.9 | 74.1 | 87.1 | 95.9 |
| MS | 7 Apr 08 | 116.0 | 105.0 | 107.0 | 99.7 | 106.0 | 99.5 | 92.6 | 102.0 | 97.8 |
| MSD | 7 Apr 08 | 117.0 | 106.0 | 104.0 | 89.6 | 104.0 | 101.0 | 90.1 | 84.6 | 100.0 |
| Percent RPD |  | 0.9 | 0.9 | 2.8 | 10.7 | 1.9 | 1.5 | 2.7 | 18.6 | 2.2 |
| LCS | 19 May 08 | 79.0 | 77.1 | 81.5 | 61.6* | 78.3 | 80.0 | 83.8 | 80.7 | 74.0 |
| LCSD | 19 May 08 | 77.3 | 76.7 | 78.1 | 56.7* | 75.8 | 78.2 | 80.9 | 95.8 | 73.0 |
| Percent RPD |  | 2.2 | 0.5 | 4.3 | 8.3 | 3.2 | 2.3 | 3.5 | 17.1 | 1.4 |
| MS | 19 May 08 | 91.8 | 99.8 | 101 | 79.3 | 99.6 | 93.0 | 78.7 | 91.1 | 104 |
| MSD | 19 May 08 | 103 | 86.5 | 102 | 75.6 | 94.9 | 91.2 | 83.5 | 83.7 | 104 |
| Percent RPD |  | 11.5 | 14.3 | 1.0 | 4.8 | 4.8 | 2.0 | 5.9 | 8.5 | 0.0 |
| LCS | 5 Aug 08. | 93.1 | 74.9 | 67.7* | 60.2* | 84.7 | 81.1 | 89.7 | 85.3 | 75.1 |
| LCSD | 5 Aug 08 | 98.9 | 71.5 | 71.4 | 68.1* | 82.9 | 76.2 | 94.0 | 91.6 | 74.5 |
| Percent RPD |  | 6.0 | 4.6 | 5.3 | 12.3 | 2.1 | 6.2 | 4.7 | 7.1 | 0.8 |
| MS | 5 Aug 08 | 95.9 | 106 | 119 | 96.9 | 124 | 99.6 | 103 | 104 | 125 |
| MSD | 5 Aug 08 | 94.3 | 100 | 112 | 116 | 126 | 97.4 | 104 | 102 | 120 |
| Percent RPD |  | 1.7 | 5.8 | 6.1 | 17.9 | 1.6 | 2.2 | 1.0 | 1.9 | 4.1 |
| 1Relative percent differences 2LCS, laboratory control sample; LCSD, LCS duplicate; MS, matrix spike; MSD, MS duplicate; RPD, relative percent difference *LCS/LCSD is outside of control limits (low recovery) |  |  |  |  |  |  |  |  |  |  |

| Table A12. QC results of laboratory and field sediment duplicates (for chemical analysis). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QC duplicate type | Lab |  | Lab |  | Lab |  | Field |  |
| Sampling date | 18 Dec 07 |  | 8 Apr 08 |  | 19 May 08 |  | 18 Mar 08 |  |
| Sample/aliquot No. | 302 |  | 349 |  | 361 |  | 338 | 341 |
|  | Aliquot 1 | Aliquot 2 | Aliquot 1 | Aliquot 2 | Aliquot 1 | Aliquot 2 | Aliquot 1 | Aliquot 1 |
|  | Concentration (ng g-1 dry weight) |  |  |  |  |  |  |  |
| Bifenthrin | nd | nd | nd | nd | 74.4* | 52.5* | nd | nd |
| Cyfluthrin, total | nd | nd | nd | nd | nd | nd | nd | nd |
| Cypermethrin, total | nd | nd | nd | nd | nd | nd | nd | nd |
| Deltamethrin | nd | nd | nd | nd | nd | nd | nd | nd |
| Esfenvalerate | trace‡ | trace‡ | nd | nd | nd | nd | nd | nd |
| Fenpropathrin | nd | nd | nd | nd | nd | nd | nd | nd |
| Permethrin, cis | nd | nd | nd | nd | nd | nd | nd | nd |
| Permethrin, trans | nd | nd | nd | nd | nd | nd | nd | nd |
| λ-Cyhalothrin, total | 10 | nd | nd | nd | nd | nd | nd | nd |
| * RPD for the two values, 34.5% ‡below the RL but above the MDL. |  |  |  |  |  |  |  |  |

| Table A13. QC for field and laboratory duplicates (TOC, grain size) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QC Type | -------------------Laboratory Duplicate -------------------- |  |  |  |  |  |  |  |  | Field Duplicate |  |  |  |  |
| ------------------------------------TOC Concentration (ppm) -------------------------------------- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sample No. | 315 |  | 342 |  | 350 |  | 354 | 362 |  | 339 |  | 341 |  |  |
| Aliquot 1 | 8,700 |  | 8,430 |  | 12,546 |  | 9,650 | 10286 |  | 7,819 |  | 7,685 |  |  |
| Aliquot 2 | 9,482 |  | 7,685 |  | 12,900 |  | 11,164 | 9,482 |  |  |  |  |  |  |
| RPD1 | 8.6% |  | 9.2% |  | 2.8% |  | 14.5% | 8.1% |  | 1.7% |  |  |  |  |
| ------------------------------------Grain Size Analysis -------------------------------------- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | % Pebble |  | % Granule |  | % Sand |  |  | % Silt |  | % Clay |  |  |  |
| 373 |  | 0.48 |  | 0.46 |  | 43.96 |  |  | 36.96 |  | 18.14 |  |  |  |
| 373 Lab Duplicate |  | 0.4 |  | 0.58 |  | 43.3 |  |  | 37.27 |  | 18.45 |  |  |  |
| RPD |  | 18.2% |  | 23.1% |  | 1.5% |  |  | 0.8% |  | 1.7% |  |  |  |
| 379 |  | 0.4 |  | 0.47 |  | 18.06 |  |  | 53.06 |  | 28.01 |  |  |  |
| 379 Lab Duplicate |  | 0.33 |  | 0.39 |  | 19.34 |  |  | 52.41 |  | 27.53 |  |  |  |
| RPD |  | 19.2% |  | 18.6% |  | 6.84% |  |  | 1.2% |  | 1.7% |  |  |  |
| 343 |  | 0.95 |  | 1.06 |  | 11.78 |  |  | 56.46 |  | 29.75 |  |  |  |
| 348 Field Duplicate |  | 0 |  | 0.06 |  | 7.98 |  |  | 60.74 |  | 31.22 |  |  |  |
| RPD |  | 200% |  | 178.6% |  | 38.5% |  |  | 7.3% |  | 4.8% |  |  |  |
| 1Relative percent difference (acceptable, < 25%) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Appendix 3. UCD ATL QC Data

| Table A14. Holding times of water samples for toxicity testing at UCD ATL. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sample Type1 |  | Sampling Date | Date received at UCD ATL | Sampling to Reception (days) | Date TOX test run | Sampling to TOX Test (days) |
| 1001 | P | 18 Dec 07 | 19 Dec 07 | 1 | 19 Dec 07 | 1 |
| 1002 | FD | 18 Dec 07 | 19 Dec 07 | 1 | 19 Dec 07 | 1 |
| 1003 | P | 18 Dec 07 | 19 Dec 07 | 1 | 19 Dec 07 | 1 |
| 1004 | FD | 18 Dec 07 | 19 Dec 07 | 1 | 19 Dec 07 | 1 |
| 1005 | FB | 18 Dec 07 | 19 Dec 07 | 1 | 19 Dec 07 | 1 |
| 1006 | P | 26 Dec 07 | 27 Dec 07 | 1 | 27 Dec 07 | 1 |
| 1008 | P | 5 Jan 08 | 5 Jan 08 | 0 | 5 Jan 08 | 0 |
| 1009 | P | 5 Jan 08 | 5 Jan 08 | 0 | 5 Jan 08 | 0 |
| 1010 | P | 14 Jan 08 | 14 Jan 08 | 0 | 14 Jan 08 | 0 |
| 1011 | P | 14 Jan 08 | 14 Jan 08 | 0 | 14 Jan 08 | 0 |
| 1013 | P | 20 Feb 08 | 20 Feb 08 | 0 | 20 Feb 08 | 0 |
| 1015 | P | 20 Feb 08 | 20 Feb 08 | 0 | 20 Feb 08 | 0 |
| 1017 | P | 28 Feb 08 | 28 Feb 08 | 0 | 28 Feb 08 | 0 |
| 1019 | P | 28 Feb 08 | 28 Feb 08 | 0 | 28 Feb 08 | 0 |
| 1021 | P | 18 Mar 08 | 18 Mar 08 | 0 | 19 Mar 08 | 1 |
| 1023 | P | 18 Mar 08 | 18 Mar 08 | 0 | 19 Mar 08 | 1 |
| 1025 | P | 8 Apr 08 | 8 Apr 08 | 0 | 9 Apr 08 | 1 |
| 1027 | P | 8 Apr 08 | 8 Apr 08 | 0 | 9 Apr 08 | 1 |
| 1029 | P | 19 May 08 | 19 May 08 | 0 | 20 May 08 | 1 |
| 1031 | P | 19 May 08 | 19 May 08 | 0 | 20 May 08 | 1 |
| 1034 | P | 10 June 08 | 10 June 08 | 0 | 11 June 08 | 1 |
| 1036 | P | 10 June 08 | 10 June 08 | 0 | 11 June 08 | 1 |
| 1038 | P | 26 June 08 | 26 June 08 | 0 | 27 June 08 | 1 |
| 1040 | P | 26 June 08 | 26 June 08 | 0 | 27 June 08 | 1 |
| 1P, primary sample; FD, field duplicate; FB, field blank |  |  |  |  |  |  |

| Table A15. Holding times of sediment samples for toxicity testing at UCD ATL. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sample Type1 |  | Sampling Date | Date received at UCD ATL | Sampling to Reception (days) | Date TOX test run | Sampling to TOX Test (days) |  |
| 501 | P | 18 Dec 07 | 19 Dec 07 | 1 | 19 Dec 07 | 1 |  |
| 502 | FD | 18 Dec 07 | 19 Dec 07 | 1 | 19 Dec 07 | 1 |  |
| 503 | FB | 18 Dec 07 | 19 Dec 07 | 1 | 19 Dec 07 | 1 |  |
| 504 | P | 18 Dec 07 | 19 Dec 07 | 1 | 19 Dec 07 | 1 |  |
| 505 | FD | 18 Dec 07 | 19 Dec 07 | 1 | 19 Dec 07 | 1 |  |
| 506 | P | 26 Dec 07 | 27 Dec 07 | 1 | 27 Dec 07 | 1 |  |
| 507 | P | 14 Jan 08 | 14 Jan 08 | 0 | 17 Jan 08 | 3 |  |
| 508 | P | 20 Feb 08 | 20 Feb 08 | 0 | 22 Feb 08 | 2 |  |
| 509 | P | 20 Feb 08 | 20 Feb 08 | 0 | 22 Feb 08 | 2 |  |
| 510 | P | 28 Feb 08 | 28 Feb 08 | 0 | 1 Mar 08 | 2 |  |
| 511 | P | 28 Feb 08 | 28 Feb 08 | 0 | 1 Mar 08 | 2 |  |
| 512 | P | 18 Mar 08 | 18 Mar 08 | 0 | 19 Mar 08 | 1 |  |
| 513 | P | 18 Mar 08 | 18 Mar 08 | 0 | 19 Mar 08 | 1 |  |
| 515 | P | 8 Apr 08 | 8 Apr 08 | 0 | 10 Apr 08 | 2 |  |
| 516 | P | 8 Apr 08 | 8 Apr 08 | 0 | 10 Apr 08 | 2 |  |
| 517 | P | 19 May 08 | 19 May 08 | 0 | 21 May 08 | 2 |  |
| 518 | P | 19 May 08 | 19 May 08 | 0 | 21 May 08 | 2 |  |
| 519 | P | 10 June 08 | 10 June 08 | 0 | 12 June 08 | 2 |  |
| 520 | P | 10 June 08 | 10 June 08 | 0 | 12 June 08 | 2 |  |
| 521 | P | 26 June 08 | 26 June 08 | 0 | 28 June 08 | 2 |  |
| 522 | P | 26 June 08 | 26 June 08 | 0 | 28 June 08 | 2 |  |
| 1P, primary sample; FD, field duplicate; FB, field blank |  |  |  |  |  |  |  |

| Table A16. QC for UCD ATL toxicity studies. |  |  |  |
| --- | --- | --- | --- |
| Source | Sample type | % Survival1 |  |
| Water Samples (C. dubia toxicity) |  |  |  |
| Lab | Control water | 100 | a |
| Del Puerto Creek | Field blank water | 100 | a |
| Orestimba Creek | Water primary sample | 100 | a |
| Orestimba Creek | Field duplicate water sample | 100 | a |
| Del Puerto Creek | Water primary sample | 100 | a |
| Del Puerto Creek | Field duplicate water sample | 100 | a |
| Sediment Samples (for H. azteca toxicity) |  |  |  |
| Lab | Control sediment | 85 | a |
| Del Puerto Creek | Field blank sediment | 75 | ab |
| Orestimba Creek | Sediment primary sample | 70 | ab |
| Orestimba Creek | Field duplicate sediment sample | 48 | bc |
| Del Puerto Creek | Sediment primary sample | 15 | d |
| Del Puerto Creek | Field duplicate sediment sample | 30 | cd |
| 1Numbers with the same letter are not significantly different from each other. |  |  |  |

# Appendix 4. Field Measurements and Non-chemical Laboratory Analysis Results.

| Table A17. Field measurements at the time of sampling Orestimba and Del Puerto Creeks. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling Date | Astronomical Time | Event Type |  | pH | DO1 (mg L-1) | EC (μS cm-1) | Water Temp (ºC) | Salinity (ppt) |
|  |  | Storm | Monthly |  |  |  |  |  |
| Orestimba Creek |  |  |  |  |  |  |  |  |
| 18 Dec 07 | 1040 | X |  | 7.39 | 6.23 | 835 | 7.8 | 0.4 |
| 26 Dec 07 | 0945 |  | X | 8.06 | 11.06 | 726 | 5.6 | 0.4 |
| 5 Jan 08 | 1010 | X |  | 7.50 | 9.39 | 325.5 | 9 | 0.2 |
| 14 Jan 08 | 1010 |  | X | 7.85 | 10.7 | 759 | 8.6 | 0.4 |
| 20 Feb 08 | 1115 | X |  | 7.57 | 9.4 | 793 | 11.8 | 0.4 |
| 28 Feb 08 | 1015 |  | X | 8.08 | 10.7 | 613 | 11.8 | 0.3 |
| 18 Mar 08 | 1010 |  | X | 7.80 | 8.03 | 1048 | 11.3 | 0.5 |
| 8 Apr 08 | 1030 |  | X | 8.16 | 9.6 | 1054 | 12.85 | 0.53 |
| 19 May 08 | 1100 |  | X | 7.75 | 6.43 | 1129 | 21.5 | 0.56 |
| 10 June 08 | 1000 |  | X | 8.07 | 8.09 | 1142 | 20.57 | 0.57 |
| 26 June 08 | 1000 |  | X | 7.94 | 7.43 | 1018 | 20.3 | 0.5 |
| mean |  |  |  | 7.83 | 8.82 | 858.4 | 12.8 | 0.43 |
| Del Puerto Creek |  |  |  |  |  |  |  |  |
| 18 Dec 07 | 1235 | X |  | 7.09 | 3.56 | 1346 | 13.2 | 0.7 |
| 5 Jan 08 | 1047 | X |  | 8.23 | 10.67 | 683 | 8.2 | 0.3 |
| 14 Jan 08 | 1125 |  | X | 7.72 | 5.2 | 1598 | 8.6 | 0.8 |
| 20 Feb 08 | 1245 | X |  | 8.12 | 9.88 | 795 | 13.2 | 0.4 |
| 28 Feb 08 | 1140 |  | X | 8.73 | 11.95 | 843 | 14.7 | 0.4 |
| 18 Mar 08 | 1125 |  | X | 8.62 | 10.09 | 1235 | 13.1 | 0.6 |
| 8 Apr 08 | 1145 |  | X | 8.86 | 12.7 | 1575 | 15.44 | 0.8 |
| 19 May 08 | 1230 |  | X | 8.79 | 10.41 | 669 | 24.79 | 0.32 |
| 10 June 08 | 1235 |  | X | 7.86 | 5.35 | 1553 | 21.59 | 0.78 |
| 26 June 08 | 1240 |  | X | 8.04 | 6.84 | 1204 | 20.87 | 0.6 |
| mean |  |  |  | 8.21 | 8.67 | 1150 | 15.4 | 0.57 |
| 1DO, dissolved oxygen; EC, electrical conductivity; Temp, temperature |  |  |  |  |  |  |  |  |

| Table A18. Total suspended sediments (TSS) in water samples collected at Orestimba and Del Puerto Creeks. |  |  |
| --- | --- | --- |
| Site1 | Sample Timing | Sediment Weight (mg/L) |
| ORC | Dec Storm | 11.38 |
| ORC | Dec Monthly | 21.18 |
| ORC | Jan Storm | 1766.45 |
| ORC | Jan Monthly | 30.45 |
| ORC | Feb Storm | 27.44 |
| ORC | Feb Monthly | 31.82 |
| ORC | Mar Monthly | 11.64 |
| ORC | Apr Monthly | 7.22 |
| ORC | May Monthly | 19.01 |
| ORC | June 10 | 98.10 |
| ORC | June 26 | 84.26 |
| DPC | Dec Storm | 12.39 |
| DPC | Jan Storm | 148.08 |
| DPC | Jan Monthly | 287.06 |
| DPC | Feb Storm | 56.45 |
| DPC | Feb Monthly | 4.81 |
| DPC | Mar Monthly | 62.7 |
| DPC | Apr Monthly | 30.42 |
| DPC | May Monthly | 20.27 |
| DPC | June 10 | 24.55 |
| DPC | June 26 | 8.89 |
| 1ORC, Orestimba Creek; DPC, Del Puerto Creek |  |  |

| Table A19. Grain size of sediment samples collected at Orestimba and Del Puerto Creeks. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Site1 | Sample Timing | Pebble (%) | Granule (%) | Sand (% ) | Silt (%) | Clay (%) |
| ORC | Dec Storm | 0.66 | 0.94 | 23.32 | 51.14 | 23.94 |
| ORC | Dec Monthly | 0.42 | 0.1 | 18.35 | 55.07 | 26.06 |
| ORC | Jan Monthly | 0 | 0.04 | 41.92 | 40.94 | 17.1 |
| ORC | Feb Storm | 0.47 | 0.06 | 74.65 | 18.35 | 6.47 |
| ORC | Feb Monthly | 0.28 | 0.1 | 84.36 | 10.72 | 4.54 |
| ORC | Mar Monthly | 0.4 | 0.58 | 43.3 | 37.27 | 18.45 |
| ORC | Apr Monthly | 0.93 | 0.81 | 37.17 | 40.99 | 20.1 |
| ORC | May Monthly | 0.74 | 0.51 | 32.84 | 43.85 | 22.06 |
| ORC | June 10 | 1.79 | 2.43 | 42 | 35.19 | 18.59 |
| ORC | June 26 | 1.04 | 0.87 | 25.74 | 45.08 | 27.27 |
| DPC | Dec Storm | 3.02 | 1.25 | 66.98 | 19.26 | 9.49 |
| DPC | Jan Monthly | 0 | 0 | 35.36 | 44.13 | 20.51 |
| DPC | Feb Storm | 0 | 0 | 18.05 | 50.89 | 31.06 |
| DPC | Feb Monthly | 0.41 | 0.11 | 13.56 | 55.35 | 30.57 |
| DPC | Mar Monthly | 0.95 | 1.06 | 11.78 | 56.46 | 29.75 |
| DPC | Apr Monthly | 1.76 | 0.33 | 23.35 | 49.8 | 24.76 |
| DPC | May Monthly | 0 | 1.06 | 17.35 | 50.53 | 31.06 |
| DPC | June 10 | 0.66 | 0.11 | 33.17 | 42.49 | 23.57 |
| DPC | June 26 | 0.33 | 0.39 | 19.34 | 52.41 | 27.53 |
| 1ORC, Orestimba Creek; DPC, Del Puerto Creek |  |  |  |  |  |  |

The 2008 water year is from October 1, 2007 through September 30, 2008. The 2008 water year is the time-frame of this study.

Non-production agriculture includes uses as applications to parks and recreation areas, golf courses, cemeteries, roadsides, railway rights-of-way, and structural pest control and is required to be reported in CDPR’s PUR database (CDPR, 2009a). Non-agricultural uses that are not reported in the PUR are mostly homeowner and most institutional and industrial uses.

Control limits are defined as 75 - 125% recovery for all surrogate, laboratory control samples, and matrix spikes as defined in CDPR 2009b.

Dormant season sampling occurred in December, January, and February. Irrigation season sampling occurred in March through June. In the 2008 WY, there was no rain after February 20 (CIMIS, 2008).