Table 2.   Beneficial uses of affected surface waters which will remain after removal of the potential MUN use. Water bodies shown in bold text are those which will have new rows added to the Basin Plan beneficial use table for surface waters (Table 2-1).

| HU No. | HYDRO-LOGIC UNIT/ SUBUNIT DRAINAGE FEATURE | WATERBODY CLASS ## MODIFIER | # BENEFICIAL USES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Receiving Water |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MUN | AGR | PRO | IND | GWR | FRSH | NAV | POW | REC-1 | REC-2 | COMM | AQUA | WARM | COLD | SAL | WILD | BIOL | RARE | MIGR | SPWN | WQE | FL D |  |
| 637.20 | Wendel Hot Springs | Hot Springs |  | x |  |  | x | x |  | x | x | x |  |  |  |  |  | x |  |  |  | x |  |  | Honey Lake |
| 637.20 | Amedee Hot Springs | Hot Springs |  | x |  |  | x | x |  |  | x | x | x |  | x | x |  | x |  |  |  |  |  |  | Honey Lake |
| 631.40 | Hot Creek | Perennial Stream |  | x |  |  | x |  |  |  | x | x | x |  | x | x |  | x |  |  |  |  |  |  | Little Walker River |
| 631.40 | Fales Hot Springs | Springs |  | x |  |  | x |  |  |  | x | x | x |  | x |  |  | x |  |  |  |  |  |  | Hot Creek |
| 603.10 | Little Hot Creek | Perennial Stream |  | x |  |  | x | x |  |  | x | x | x |  |  | x |  | x |  |  |  | x |  |  | Hot Creek or Owens River |
| 603.10 | Little Alkali Lake | Alkali Lake |  | x |  |  | x | x |  |  | x | x | x |  |  | x |  | x |  |  |  | x |  |  | Crowley Lake |
| 603.20 | Keough Hot Springs | Springs |  | x |  |  | x |  |  |  | x | x | x |  | x | x |  | x |  |  |  |  |  |  | Owens River |
| 609.00 | Amargosa River | Intermittent Stream |  | x |  |  | x |  |  |  | x | x |  |  | x |  | x | x | x | x |  | x |  |  | Amargosa Subarea GW |

Table 3. Water Quality Criteria for Inorganic Constituents of Concern for Lahontan Region Saline and Geothermal Waters. All concentrations in micrograms per liter (parts per billion) unless otherwise specified. Lahontan Basin Plan water quality objectives for surface waters designated MUN are the CA Dept. of Health Services MCLs.  Source: Central Valley RWQCB, 1998.

| Chemical Constituent | CA Dept. of Health Services Primary MCL | CA Dept. of Health Services Secondary MCL | CA Prop. 65 Regulatory Level | Agricultural Water Quality Goals | USEPA National Ambient Water Quality Criteria/ Freshwater Aquatic Life | USEPA National Ambient Water Quality Criteria/ Saltwater Aquatic Life | Proposed California Toxics Rule, Human Health | Proposed California Toxics Rule, Freshwater Aquatic Life | Other |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arsenic | 50 |  | 5 |  | 190 (4 day avg.) 360 (1 hr avg.) | 36 (4 day avg) 69 (1 hr. avg) |  | 150 (continuous limit) 340 (acute limit) | USEPA Drinking Water Health Advisory (SNARL) 0.02 |
| Beryllium | 4 |  | Carcinogen | 100 |  |  |  |  | USEPA Drinking Water Health Advisory (SNARL) 0.008 |
| Boron |  |  |  | 700/750 |  |  |  |  | USEPA Drinking Water Advisory (SNARL) 600 |
| Chloride |  | 250,000 |  | 106,000 | 230,000 (4 day avg) 860,000 (1 hr avg) |  |  |  |  |
| Copper | 1300 | 1000 |  | 200 | Depends on water hardness | 2.4 (4 day avg) 2.4 (1 hr avg) | 1300 | 11(continuous limit) 16 (acute limit) | CA Public Health Goal in Drinking Water 170 CA Toxics Rule proposed saltwater aquatic life criteria 4.8 (acute) 3.1 (continuous) |
| Fluoride | 1400 to 2400 |  |  | 1000 |  |  |  |  | CA Public Health Goal in Drinking Water 1000 |
| Lead | 15 |  | 0.25 Carcinogen, reproductive toxin | 5000 | Depends on water hardness | 8.1 (4 day avg) 210 (I hr avg) |  |  |  |
| pH |  |  |  |  |  | 6.5 to 8.5 units |  |  | Lahontan RWQCB objective 6.5-8.5 units |
| Sodium |  |  |  |  |  |  |  |  | USEPA Drinking Water Health Advisory SNARL 2000 |
| Specific conductance |  | 900 umho/cm |  | 700 umho/cm |  |  |  |  |  |
| Strontium |  |  |  |  |  |  |  |  | NAS 7 day suggested no adverse risk level 8400 |
| Sulfate |  | 250,000 |  |  |  |  |  |  |  |
| Total dissolved solids (TDS) |  | 500,000 |  | 450,000 |  |  |  |  | USEPA National Ambient Water Quality Criteria: Human Health- Taste and Odor or Welfare |