DETERMINATION OF FECAL COLIFORM VIOLATIONS OF LAHONTAN BASIN PLAN

WATER QUALITY OBJECTIVES

DATA REDUCTION PROCEDURE

Water Quality Objective:  from Log-normalized monthly data

RAW DATA

including multiple samples per day

DAILY MEANS - take geometric mean when there is

more than one sample per day

Natural Log of Daily Data

Moving Mean of Natural Log of Daily

Data within each 30-day period

Antilog of Mean Log Data to Yield Log Normalized

Mean of Fecal Coliform in 30 Day Period

BTW

Log Normalized 30-Day Fecal Coliform Mean Greater

Than 20/100 mL --> Violation

.