National Fish Tissue Study – Year Three (2002)

Electronic Data Deliverable (EDD)

## Data Dictionary

Worksheet Name: – “STATE ABBREVIATION”_RESULTS

Worksheet Description: – This worksheet stores the analytical results for all of the composite samples.

Column Name				Description

State	The full name of the state in which the sample was collected. An example of a record found in this field would be Alabama.

SDG	The sample delivery group associated with the particular sample.  This SDG number is used as a means of batching samples together in groups of 20 for analytical purposes.

Examples of records found in this field would be B4 or P2.

Composite_Sample_ID	This is the alpha-numeric code, ten characters in length, used to describe a sample.  The first two characters of the code are the states abbreviation; the third and fourth characters are the last two digits of the year in which the sample was collected; the fifth through eighth characters represent the lake ID number supplied by EPA; the ninth character represents the composite type (“B”  for bottom-dweller and “P” for predator); and the tenth, and final, character represents the sample type (“S” for standard and “D” for duplicate).  Examples of records found in this field would be AL000022PS or AL000022BD.

Method	The method number used by the laboratory to analyze the samples for a particular analyte of interest.  This field is limited to the following method numbers: 1613B, 1625C, 1631B, 1632A, 1656A, 1657A, and 1668A.

Sample				The unique 5-digit EPA Sample number assigned by the

sample prep laboratory to distinguish samples from one another.  For example, 56524, 56525, 56538, etc.

Episode	For the National Fish Tissue Study, each state was assigned an individual episode number.  This field is used mainly for internal tracking purposes by SCC and is used as a check to ensure the laboratories are preparing and analyzing the correct samples.  The field is also used in differentiating records within the electronic data submission from the laboratory.  For example, 6305, 6336, 6452, etc.

Analyte	The pollutant which is determined by the laboratory.

Worksheet Name: – “STATE ABBREVIATION”_RESULTS (cont.)

Column Name	Description

CAS_No	The unique Chemical Abstract Services (CAS) Number assigned to each analyte.  Please note that for those PCB congeners that co-elute, the CAS Number was left null.  Also, since Total Inorganic Arsenic does not have a CAS Number associated with it, this field was left null for this analyte.

Amount	The concentration of a particular analyte for which the data are being reported.

Replimit	The minimum level of quantitation (ML), adjusted for dilution or concentration, if necessary.

Unit	Unit of measure.  For example, NG/KG, UG/KG, UG/G, etc.

Dilution	The dilution factor used in the analysis.  The dilution factor can be calculated as follows (be certain that the units are consistent throughout the calculation):

Final volume used in analysis / initial sample volume

SCCCode	The column is used to represent SCC’s data considerations or “qualifiers”.  Examples of records found in this field include B, J, RNON, etc.  Please note that a Data Consideration Key is provided with each set of results and this key describes all of the SCC Codes that can be used.

Comments	A brief explanation or description of the SCC Code.  Examples of records found in this field include Blank Contamination, Estimated Value, Result Non-detect, etc.

Preparation	A description of how the sample was prepared.  The only two values to be found in this field are “Homogenized Whole” for the bottom-dwellers and “Filleted Prior to Homogenization” for the predators.

Composite_Type	This field describes the type of fish species sampled.  This field is limited to the records “Bottom-dweller” and “Predator”.

Site_Name	The location (lake, reservoir, etc.) where the samples were taken.  Examples of entries in this field include Unnamed Lake, Lewis Smith Lake, etc.

Worksheet Name: – “STATE ABBREVIATION”_RESULTS (cont.)

Column Name				Description

Percent_Lipids	Measure of the amount of lipid content of a particular sample; represented as a percentage of total sample weight, rounded to two decimal places.

Sample_Type	This field further describes the sample collected as either “Standard” or “Duplicate”.  The “Standard” sample type represents the samples collected as part of the routine sampling activities; and “Duplicate” sample type represents the samples taken during the re-sampling of the water body.  These “Duplicate” samples are used to measure the precision of the data within the sub-sampled water bodies.

Analysis	The full text description of the analysis performed on the sample.  This field is limited to the following: “Dioxins and Furans”, “Semi-volatile Organic Cmpds.”, “Metals (Hg + As Species)”, “Pesticides”, and “Polychlorinated Biphenyls”.

Worksheet Name: – “STATE ABBREVIATION”_SAMPLE_DESCRIPTION

Worksheet Description: – This worksheet describes the individual samples used to create the composite samples.

Column Name				Description

Composite_Sample_ID	This is the alpha-numeric code, ten characters in length, used to describe a sample.  The first two characters of the code are the states abbreviation; the third and fourth characters are the last two digits of the year in which the sample was collected; the fifth through eighth characters represent the lake ID number supplied by EPA; the ninth character represents the composite type (“B”  for bottom-dweller and “P” for predator); and the tenth, and final, character represents the sample type (“S” for standard and “D” for duplicate).  Examples of records found in this field would be AL000022PS or AL000022BD.

Sample_Type	This field further describes the sample collected as either “Standard” or “Duplicate”.  The “Standard” sample type represents the samples collected as part of the routine sampling activities; and “Duplicate” sample type represents the samples taken during the re-sampling of the water body.  These “Duplicate” samples are used to measure the precision of the data within the sub-sampled water bodies.

Fish_Species	The common name for the species of fish collected for analysis.  Examples of records found in this field include Yellow Bullhead, Largemouth Bass, Channel Catfish, etc.

Specimen_Number	This is a field of numbers used as a means to uniquely identify the individual fish samples that make up the composite sample.  For example, 1,2,3, etc.

Length (mm.)	The length of the individual samples in millimeters.

Specimen_Weight (g.)	The weight, in grams, of the individual samples that make up the composite sample.

Composite_Weight (g.)	The final weight, in grams, of the composite sample.

Location	This column is taken directly from the field sampling form filled-in by the field samplers.  This column gives a further explanation of where the fish samples were taken.  For example, “Off Peninsula 1”, “Confluence Of Hardridge Creek”, etc.

Sampling_Date	The date the samples were collected.

Worksheet Name: – “STATE ABBREVIATION”_SAMPLE_DESCRIPTION (cont.)

Column Name				Description

Sampling_Time	The time the samples were collected.

Notes	Additional comments made by the field samplers; taken directly from the field sampling forms.  Examples of records found in this field include “Lesions On L Side”, “Lake Level Down 2m. Due To Drought”, etc.

EPA_Sample_Number			The unique 5-digit EPA Sample number assigned by the

sample prep laboratory to distinguish samples from one another.  For example, 56524, 56525, 56538, etc.

State	The full name of the state in which the sample was collected. An example of a record found in this field would be Alabama.

Include_In_Composite	This field is a “Yes” or “No” field and it indicates whether or not the individual sample was used in creating the composite.  (Individual samples would not be used if they did not meet the 75% rule.)

Composite_Type	This field describes the type of fish species sampled.  This field is limited to the records “Bottom-dweller” and “Predator”.

Worksheet Name: – “STATE ABBREVIATION”_SAMPLER_INFO

Worksheet Description: – This worksheet describes the information related to the field sampler responsible for the collection of samples at a particular water body.

Column Name				Description

Episode	For the National Fish Tissue Study, each state was assigned an individual episode number.  This field is used mainly for internal tracking purposes by SCC and is used as a check to ensure the laboratories are preparing and analyzing the correct samples.  The field is also used in differentiating records within the electronic data submission from the laboratory.  For example, 6305, 6336, 6452, etc.

State	The full name of the state in which the sample was collected. An example of a record found in this field would be Alabama.

Lake_Number	The unique four digit code assigned to individual lakes, reservoirs, etc. by the EPA; used as a means to uniquely identify a particular site.  Examples of records found in this field include 0022, 0072,  0136, etc.

Sampling_Date	The date the samples were collected.

Sample_Type	This field further describes the sample collected as either “Standard” or “Duplicate”.  The “Standard” sample type represents the samples collected as part of the routine sampling activities; and “Duplicate” sample type represents the samples taken during the re-sampling of the water body.  These “Duplicate” samples are used to measure the precision of the data within the sub-sampled water bodies.

Sampler_Name	The full name of the person who collected the individual samples.

Sampler_Address	The street address associated with the person who collected the individual samples.

Sampler_City	The city associated with the person who collected the individual samples.

Sampler_State	The state associated with the person who collected the individual samples.

Sampler_Zip	The zip code for the city associated with the person who collected the individual samples.

Worksheet Name: – “STATE ABBREVIATION”_SAMPLER_INFO (cont.)

Column Name				Description

Sampler_Phone	The phone number associated with the person who collected the individual samples.

Sampler_Fax	The fax number associated with the person who collected the individual samples.

Site_Name	The location (lake, reservoir, etc.) where the samples were taken.  Examples of entries in this field include Unnamed Lake, Lewis Smith Lake, etc.

Worksheet Name :  – “STATE ABBREVIATION”_TRIP_INFO

Worksheet Description: – This worksheet describes the information related to the sampling trip made to a particular water body.

Column Name				Description

Composite_Sample_ID	This is the alpha-numeric code, ten characters in length, used to describe a sample.  The first two characters of the code are the states abbreviation; the third and fourth characters are the last two digits of the year in which the sample was collected; the fifth through eighth characters represent the lake ID number supplied by EPA; the ninth character represents the composite type (“B”  for bottom-dweller and “P” for predator); and the tenth, and final, character represents the sample type (“S” for standard and “D” for duplicate).  Examples of records found in this field would be AL000022PS or AL000022BD.

Composite_Type	This field describes the type of fish species sampled.  This field is limited to the records “Bottom-dweller” and “Predator”.

Sample_Type	This field further describes the sample collected as either “Standard” or “Duplicate”.  The “Standard” sample type represents the samples collected as part of the routine sampling activities; and “Duplicate” sample type represents the samples taken during the re-sampling of the water body.  These “Duplicate” samples are used to measure the precision of the data within the sub-sampled water bodies.

Sampling_Date	The date the samples were collected.

Collection_Method	The method/technique used to collect the individual samples.  Examples of records found in this field include “Angling Equipment (Hook And Line)”, “Electrofishing Unit”, “Trot Line”, etc.

Collector_Name	The name of the person who collected the individual samples.

Affiliation	The agency with whom the sample collector is associated.  For example, Al Dept. Of Environmental Management.

Water_Body_Type	The type of water body from which the individual samples were collected.  Records to be found in this field include Reservoir, Lake, etc.

pH	The pH of the water body from which the individual samples were taken.

Worksheet Name – “STATE ABBREVIATION”_TRIP_INFO (cont.)

Column Name	Description

Depth	The depth of the water body from which the individual samples were taken.

Units	The units of measure of depth of the water body.

Site_Name	The location (lake, reservoir, etc.) where the samples were taken.  Examples of entries in this field include Unnamed Lake, Lewis Smith Lake, etc.

State	The full name of the state in which the sample was collected. An example of a record found in this field would be Alabama.

County	The county in which the samples were collected.  For example, Lauderdale, Wilcox, etc.

Latitiude	The latitude (represented as a decimal value) associated with the sampling point at which the samples were collected.  For example, “33.9489”.

Longitude	The longitude (represented as a decimal value) associated with the sampling point at which the samples were collected.  For example, “-87.3314”.