Cal/EPA

Gray Davis

Governor

Summary of Water Quality Data for Selected Locations within the Salton Sea Watershed

Concentrations are Averages

|  | Alamo River - Outlet | Coachella Valley Stormwater Channel | Lower Colorado River | I.V. Drains | # I.V. Tilewater | Mexicali I Effluent | New River – Outlet | New River / IB | Salton Sea Midpoint |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total Nitrate as N (mg/L) | 8 | 12a | 0.2a |  | 9 | 0.2 | 5a | 0.2 | 0.1a |
| Total Phosphate as P (mg/L) | 0.7a | 1.a | 0.08a |  | 0.03b | 3.c | 0.9a | 2d | 0.34 a |
| BOD (mg/L) | 6 a | 3 a | 0.3 a |  | NA | 32c | 9a | 18d | 13 a |
| Fecal Coliform (MPN/100mL) | 17,600 a | 3000 a | 29 a |  | NA | 2,000,000c | 17,300a | 130,000d | <2 a |
| MBAS (mg/L) | 0.03 a | 0.07 a | 0.00 a |  | NA | 2c | 0.4 a | 1d | 0.2 a |
| TDS (mg/L) | 2400 a | 1600 a | 700 a |  | 4300, | 965c | 3000 a | 2600d | 44,000 a, |
| Total DDT (g/L) |  | 0.12 b | 0.00 b | 0.05 b | <0.01 b |  |  | 0.08 |  |
| Total DDT (fish) (ppb, wet weight) | 4400 (carp) | 170 (carp) | 40 (carp) |  | NA |  | 1640 (channel catfish) | 690 (carp) | 120 (corvina) |
| Selenium (ppb) | 8 | NA | 1-2 |  | 25f |  | 4 | 2 | 1 |
| Selenium (fish)j (ppm, wet weight) | 1 (carp) | 1 (carp) | 2 (carp) |  | NA |  | 1 (carp) | 1 (carp) | 3 (corvina) |
| Dacthal (fish)j (ppb, wet weight) | 560 (carp) | 650 (carp) | 0 (carp) |  | NA |  | 120 (carp) | 840 (carp) | 15 (corvina) |

Red text: data already included in other data source

NA Data not available.

? Regional Water Quality Control Board, Colorado River Basin Region.  Trend Monitoring and Surveillance for the Colorado River Basin Region.  1980-1993.

Regional Water Quality Control Board, Colorado River Basin Region. Draft Report on Non-Point Source Water Quality Impacts Upon Agricultural Drains within the Colorado River Basin Region.  September 1992.

CH2MHILL. Flow Monitoring and Wastewater Characterization for Mexicali, Baja California, Mexico (Samples from the Mexicali I Wastewater Treatment Plant were taken in September 1997).  1998.

Regional Water Quality Control Board, Colorado River Basin Region.  New River at the International Boundary, Monthly Monitoring Reports.  March 1997 through March 1998.

Regional Water Quality Control Board, Colorado River Basin Region.  1988.

Flow weighted average.

Approximate present concentration.

Total DDT = DDT and metabolites.

USGS.  Water Resources Investigations 79-86: Water-Quality Conditions in the New River, Imperial County, California.  1979.

California State Water Resources Control Board.  Toxic Substances Monitoring Program.  1978-1995.

USGS.  Water Resources Investigations 93-4014: Detailed Study of Water Quality, Bottom Sediment, and Biota Associated with Irrigation Drainage in the Salton Sea Area, California,  1988-90.  1993