Aliso Creek (901.130) – 303(d) Fact Sheet

1999 Biological Assessment Annual Report

No listing of Aliso Creek should be made based upon this data.

# Watershed Characteristics

Aliso Creek is a 7.20 mile waterway in the San Juan Watershed of Region 9.  It is classified inland surface water with the following beneficial uses: AGR, REC1, REC2, WARM and WILD1. The mouth of the creek is currently on the 1998 303(d) list because of high coliform counts from non-point sources.

# Water Quality Objectives not Obtained

Aliso Creek was sampled for aquatic insects and compared to other locations in San Diego County in the San Diego Regional Water Quality Control Board: 1999 Biological Assessment Annual Report

# Evidence of Impairment

Aliso Creek was sampled four times in a one-year period.

# Extent of Impairment

Sampling of benthic macroinvertebrate communities occurred on May, September, and November 1998 and May of 1999.  Two locations were sampled:  3 riffles upstream of Pacific Park Drive and at 5 riffles parallel to Country Club Road Upstream of Hwy 1.

Aliso Creek  Benthic Macroinvertebrate Index (BMI) rankings were below the overall study average (BMI < 0) for 7 of the 8 sampling events when compared against other sites in San Diego County. In May of 1998, the rankings were below the –25 score and one was the lowest of the rankings.  All other below average rankings were near the average.

Since the sampling events occurred only 4 times in a 12-month period, it is difficult to come to any conclusions about the watershed.  Also confounding the issue is that the rankings only represent a comparison among stations in San Diego and do not present an absolute determination of community health.  The seasonal nature of the data does not allow direct comparisons to be made between the dates.  The study represents a good baseline from which to judge all future assessments of the benthic community.  Further bioassessment and water quality data will be needed before any management decisions can be made.

# Potential Sources

Unknown

# TMDL Priority

# No TMDL is required at this time.

# Notes

# Information Sources

1 Water Quality Control Plan for the San Diego Basin (9), 1994