San Juan Creek (901.250 to 901.280) – 303(d) Fact Sheet

Toxic Substances Monitoring Program Data

San Juan Creek should be listed as threatened due to the presence of elevated levels organic pesticides in fish tissue.

# Watershed Characteristics

San Juan Creek is a 14.2-mile waterway in the San Juan Watershed of Region 9.  It is classified inland surface water with the following beneficial uses: AGR, IND, REC1, REC2, WARM, COLD and WILD1.

# Water Quality Objectives not Obtained

Data from the 2Toxic Substances Monitoring Program in 2000 show 3EPA screening levels for subsistence fisheries to be exceeded for Total DDT, Dieldrin and Total PCB in fish tissue. No direct link to water quality impairment could be established.

# Evidence of Impairment

Please see Table 1 for values in excess of the EPA screening values.  This data set only describes two points in time and is inconclusive in demonstrating impairment to water quality that may harm the beneficial uses.  It is also not clear if the EPA screening levels apply to whole fish or just the edible portion.  This water body is not listed as having a subsistence fishery, but the presence of organic pesticides in fish tissue at levels above the EPA screening levels is a concern and merit further research.

# Extent of Impairment

This is difficult to assess due to the migratory nature of fish.  Due to their high tolerance for temperature and other inhospitable conditions, red shiner inhabit most of the creek.  They have probably acquired small amounts of pollutants over time, as a single dose would most likely be fatal.  Therefore, the entire creek should be listed as threatened.

# Potential Sources

Unknown

# TMDL Priority

No TMDL is required at this time.

# Notes

The following chemicals were reviewed and represent all sub-classes of that chemical: Total Chlordane, Total DDT and Total PCB.  No standards could be found for Hexachlorobenzene, Oxadiazon and Chemical Group A, in fish tissue.

# Information Sources

1 Water Quality Control Plan for the San Diego Basin (9), 1994

2 Toxic Substances Monitoring Program, 1994-95, Data Report.

3 EPA Guidance for Assessing Chemical Contaminant Data for Use in Fish Advisories,

Volume 2, Risk Assessment and Fish Consumption Limits Third Edition, 2000, EPA 823-B-00-008