FEDERAL GEOGRAPHIC DATA COMMITTEE / NATIONAL BIOLOGICAL INFOMATION INFRASTRUCTURE METADATA TEMPLATE

There is no formal format for these sections of the metadata.  These sections are described in the metadata standard as free form text.  The sections abstract and methods may be cut and pasted from any existing documents including reports and work plans.  The remainder of the metadata is concerned with geographic extents and the data tables and structures.  The information you supply in this form will be added in the proper format to produce the entire metadata document that will be distributed with the data.

# ABSTRACT:

# Atmospheric Deposition of Pollutants to Santa Monica Bay.

There are numerous sources that contribute a variety of pollutants to Santa Monica Bay, CA.  Some sources, such as municipal wastewaters, are well-characterized and estimates of mass emissions to the Bay are relatively well-known.  However, other sources have not been characterized and their contribution to the Bay’s total pollutant load is unknown.  One such source is atmospheric deposition, which is of particular concern since air quality in the Los Angeles Air Basin is amongst the worst in the nation. Starting in January 1999 daily aerosol samples were collected every six days for regional air quality in the Santa Monica Bay watershed.  These data were supplemented with aerosol samples collected by the South Coast Air Quality Management District at similar times and by similar methods within the airshed, but outside of the watershed.  These samples were analyzed for particulate mass and trace metal concentrations.  In addition, samples of wet deposition (rainfall) were collected and analyzed for trace metals.  Finally, sea-surface microlayer samples were collected across a grid of stations in Santa Monica Bay to determine the amount of Bay surface area affected by atmospheric deposition.  Data were assessed for concentration and deposition changes on daily, seasonal, and annually variation.  These data demonstrated that contributions of trace metals from atmospheric deposition are a significant source of pollutants to the Bay by falling directly onto Bay water surfaces or by depositing onto terrestrial surfaces within the watershed and being washed off during subsequent storm events.

# PURPOSE:

There are a multitude of pollutant sources to Santa Monica Bay and it’s surrounding watershed (SMBRP 1998).  Some sources, such as treated wastewater, are well characterized (Raco-Rands 1997) and extensive effort has been spent assessing their impacts to the Bay (Dorsey et al 1995, Stull 1995).  Other sources, however, are virtually unmonitored and the magnitude of their contribution to the Bay’s overall pollutant load are unknown.  One example of this type of source is aerial deposition.  The Los Angeles metropolitan area exhibits amongst the worst air quality in the nation (EPA 1997).  Aerosols in the ambient air of greater Los Angeles are known to contain concentrations of toxic constituents, as well as potentially eutrophying nutrients, for marine ecosystems (Prastinis 1994, Arey et al 1989, Kawamura and Kaplan 1986, Young et al 1976).  Even so, little information currently exists for assessing the relative risk to the Bay from aerial deposition compared to other, better-characterized sources.

To better understand the contribution of pollutants and nutrients to Santa Monica Bay from aerial deposition, a unique collaboration among three multi-disciplinary organizations has been formed.  These organizations include the Santa Monica Bay Restoration Project (SMBRP), the Southern California Coastal Water Research Project  (SCCWRP), and the University of California at Los Angeles Institute of the Environment (UCLA).  The SMBRP is part of the US EPA’s National Estuary Program (NEP).  As part of their federal mandate, the SMBRP has the responsibility of helping to restore and protect the valuable resources of the Bay by promoting the development and implementation of environmental management actions within it’s watershed.  SCCWRP is a non-profit marine environmental research agency governed by a blend of large municipalities, state and regional water quality boards, and the US EPA.  Their mission is to understand the inputs, fates, and effects of pollutants on the marine environment in Santa Monica Bay and throughout the Southern California Bight.  UCLA is currently researching the multitude of physical and chemical interactions within the Bay’s watershed to understand the transport of pollutants across different media (air, land, water).  By joining forces, these three organizations represent the necessary spectrum of specialties to address pollutant impacts in the Bay from aerial deposition including resource management, atmospheric scientists, water quality experts, and marine biologists.

Understanding the transport and deposition of toxic contaminants and nutrients to Santa Monica Bay is a large and very complex proposition.  Therefore, the project has been divided into three major objectives including:

- To quantify the toxic materials and nitrogen emitted annually in the Los Angeles air basin that are subsequently transported to and deposited in the Bay and the Bay’s watershed;

- To evaluate the relative impacts of air deposition and the benefit of various emission reduction options; and

- To identify the sources of various airborne pollutants in the air basin and their relative contributions to total pollutant loading to the Bay; to recommend the most cost effective measures to control the identified sources.

Since each task builds one upon the other, the three organizations have decided to attempt only the first objective during the first year of study.  The following sections summarize the essential elements of this first year’s work plan. The research questions to be answered are first stated, followed by a discussion of the research approach to answering these questions.  Then, the technical components of the work plan are presented in detail.

# II.  Research Questions

The following three research questions have been identified:

- What is the total annual load of toxic contaminants and nutrients to Santa Monica Bay via atmospheric deposition, either directly or indirectly?

There are no regulatory criteria for loadings of toxic constituents via aerial depostion to the Bay.  However, answering this question will make it possible to compare the inputs from atmospheric deposition to current inputs from other sources.  This will provide resource managers charged with protecting the Bay the necessary information to evaluate the relative environmental risk from atmospheric deposition as a source of contaminants.  If atmospheric deposition contributes a large portion of the total load for a specific pollutant, then greater efforts may need to be expended in order to control this source.  If the contribution of that pollutant from atmospheric deposition is small, then less effort should be spent on aerial as a source of contamination and management efforts may be more efficiently directed elsewhere.  Figure 1 is a representation of how our data might be used to answer this research question.  It shows that atmospheric deposition of zinc represented approximately half of the total annual load in 1971 to the marine waters of the Southern California Bight.  Estimates of the total annual load were made for several constituents in the 1970’s from atmospheric deposition, as well as many other sources (SCCWRP 1973).  Estimates of historical pollutant loads from the 1970s and 1980s for atmospheric deposition will also enable us to evaluate temporal trends of inputs from this source.  Environmental managers can utilize this information to assess whether atmospheric deposition is a source of pollutants which has been increasing or decreasing over the last two decades once again enabling effective prioritization of resource issues and efficient management decisions.  Finally, if a management decision is made to reduce pollutants inputs from atmospheric deposition, the total annual load estimates generated during this study will provide the benchmark for assessing whether the management actions implemented were effective at reducing loads of pollutants to the Bay.

2.	What proportion of the annual load of toxic contaminants and nutrients from atmospheric deposition is contributed during different seasons or specific meteorological events?

Answering this question will determine if the annual load from atmospheric sources are primarily large, but episodic in nature, or if they are a low level, but chronic contributor of pollutants to the Bay.  To do this, we will estimate the relative contributions of pollutants during the winter and summer season, as well as specific events such as Santa Ana winds and rain storms.  Studies in southern California and other areas around the nation indicate that episodic events can increase deposition rates significantly and may be extremely important components of the total annual load (Young and Chan 1977, Offenberg and Baker 1997).  However, this assessment has not been accomplished within the Santa Monica Bay watershed and our vision of how the data may be used appears in Figure 2.  This understanding of when the greatest degree of inputs occurs is crucial to focusing future work including biological effect studies, source reduction strategies, or design of transport and deposition models.

3.	How do atmospheric concentrations of toxic contaminants and nutrients vary spatially between Santa Monica Bay and other regions of Los Angeles?

Answering this question will enable us compare the concentrations in the Santa Monica Bay watershed to the greater Los Angeles region, providing the larger spatial scale and adding context to the toxic contaminant loads being delivered by atmospheric deposition.  To this end, we will be coordinating our efforts with the South Coast Air Quality Management District (AQMD) TEP2000 Program, an intensive air quality monitoring study sampling many other locations within the airshed.

This question also recognizes that there are two separate categories of sources that contribute to atmospheric deposition loads which are segregated upon their transport to the Bay.  The first is from local sources within the watershed that may deposit either directly to the Bay water surfaces or indirectly by depositing onto land surfaces only to be washed into the ocean during the next rain event.  The second category of pollutant sources are from distant locations within the airshed, but outside the watershed that are subsequently transported to the Bay for deposition (either directly or indirectly).  The degree to which local sources contribute to the Bay's loading relative to distant sources will have a profound impact on how resource managers may approach control measures and implement management or regulatory actions for other transport mechanisms, most notably urban runoff.

METHODS:

III.  Research Approach

The major technical issue for this study is how to measure atmospheric deposition to land and water surfaces.  For both types of surfaces there are two general approaches:

- Direct measurement using deposition plates (for dry deposition) or rainwater collectors (for wet deposition).

- Indirect measurement in which a measured concentration in the air is multiplied by a theoretical “deposition velocity” which is a function of the local meteorology and the type of surface.

Both of these techniques have been used in previous studies of atmospheric deposition to water bodies and their watersheds (Zufall et al.  1998, Zufall and Davidson 1997, Clarke et al 1997, Hoff et al 1996.).  There is general agreement that collection of rainwater is a feasible method of measuring wet deposition and relatively standard methods have been developed for this purpose (NADP 1997).  In contrast, the direct measurement of dry deposition is more difficult because of the lack of geometric and fluid dynamic similitude between artificial collector surfaces and actual land and water surfaces.  For this reason, measurements of dry deposition are often times based on rates calculated from air concentrations.  Unfortunately, measurements of air concentrations over the Bay water surfaces have never been attempted and are logistically difficult.  Table 1 presents a comparison of the direct and indirect measurement techniques in the context of this study.

On the basis of these considerations the following combination of methodologies were selected:

- Wet deposition on land surfaces will be measured directly.

- Dry deposition on land surfaces will be estimated using measured air concentrations, segregated by particle size.  Theoretical deposition velocities for each particle size will be calculated using local meteorological conditions.  An aerodynamic surrogate surface will be deployed as a check of the relative magnitude of modeled deposition.

- The spatial pattern of deposition to the surface of Santa Monica Bay will be measured indirectly by collection of water samples from the surface microlayer using SCCWRP’s rotating drum system.  Bulk water samples will be collected at the same time.

- Wet and dry deposition in the watershed will be estimated using a model developed at UCLA to extrapolate measured and calculated deposition in this project.  The UCLA model will also be used to extrapolate over the Bay's water surfaces, using the microlayer measurements to assess spatial extent of deposition.

Using this approach also enables our study to best integrate the work being conducted by the AQMD at other locations within the airshed.  The AQMD samples air concentrations exclusively.  Further, this approach takes advantage of the atmospheric transport and deposition model being developed at UCLA.  The UCLA surface meteorology and ozone generation (SMOG) model has modules for meteorology, pollutant sources and dispersion, photochemistry and aerosol microphysics, and radiative processes (Lu et al 1994, 1995).  This model has been calibrated and verified using the Southern California Air Quality Study (SCAQ) (Lu et al 1996).

IV.  SAMPLE DESIGN

Sampling Locations

Our sampling design consists of a Master Site located within the Santa Monica Bay watershed (Figure 3).  This site will be at the UCLA campus on top of Buelter Hall where the atmospheric studies department is located.  This site has a long historical record of meteorological conditions and has been the site of many atmospheric studies by UCLA, AQMD, the California Air Resources Board (CARB), and others.  UCLA maintains and logs data at this site and is of sufficient quality for purposes of this study.  The building is four stories above ground level (approx. 12 m) where airflow is not impeded by any nearby structure; the surrounding area is comprised of parking lots, trees and shrubs, and other buildings.  This site also has a meteorological station which records wind speed and direction, precipitation, barometric pressure, and humidity.  Eight additional sampling sites, consisting of AQMD monitoring elements, are located throughout the airshed including downtown Los Angeles, Anaheim, Diamond Bar, Rubidoux, Fontana, Ontario, Costa Mesa, and Long Beach (Figure 3).  These sites provide an opportunity to extend the spatial scale of our study design.

Sea surface microlayer samples will be collected in a uniform grid pattern of nine sites extending over Santa Monica Bay (Figure 3).  The rationale for site selection was based upon the need to assess the spatial distribution of direct depostion to Bay water surfaces.  Sites were selected based upon predicted air flow patterns by the UCLA air transport model and preliminary work conducted by Stolzenbach, et al. (in prep).

Sampling Requirements

The NEP watershed management plan has identified the target analytes that are of concern in the Bay including trace metals, chlorinated hydrocarbons (DDTs and PCBs), and petroleum hydrocarbons (PAH).  These target analytes were chosen as constituents of concern based upon presence in discharges to the Bay or the presence of enriched concentrations in Bay sediments and/or fish tissues.  Table 2 identifies the target analytes to be selected in the project.

The sampling design has been devised to characterize three distinct time periods in our region which can affect deposition including the wet season, the dry season, and Santa Ana conditions (fall).  Sampling frequency will be as follows:

1)	Samples of airborne particulates will be obtained at our master site in the Santa Monica Bay watershed five times per month (every six days) during six months of the study period.  The days of sampling will be planned to match the sampling days of the AQMD study to maintain comparability at appropriate temporal scales.  The months to be sampled will be selected to obtain adequate measurements of seasonal deposition rates to include three sampling periods lasting two months (see schedule).

Two types of samples will be collected:

A)	Four of the monthly sample events will separate the samples onto 2.5 and 10 micron size fractions using a dichotomous sampler.  These samples will be analyzed for total mass and trace metals.

B)	One sample event per month will use a MOUDI Impactor to sample ten particle sizes (including 2.5 and 10 micron) to obtain the necessary breakdown by particle size to model deposition accurately.  These samples will be analyzed for total mass, trace metals, and trace organics.

2)	Sampling of wet and dry deposition will occur at our master site for six months of the study period.  Both wet and dry deposition samples will be collected, depending upon the prevailing weather.  Wet weather deposition will only be sampled during storm events and sample collection will follow each storm.  Dry weather deposition will only be sampled when there is no rain, or threat of rain.  The period of dry weather sampling will focus on the MOUDI sampling events so that matching size class data can be used.  These samples will be analyzed for trace metals, and trace organics.

3)	Samples of the microlayer and near-surface bulk water will be obtained in Santa Monica Bay at nine sites, once per month for six months.  These samples will be analyzed for trace metals only.  The timing of the microlayer and bulk water will coincide with the timing of MOUDI and dry deposition sampling.

4)	Appropriate meteorological data will be obtained from existing meteorological stations within the airshed during the study period, including offshore buoys.  The meteorolgical data within the airshed and offshore buoys will be generated under existing monitoring programs supported by the National Weather Service.  These stations, established by the National Oceanic and Atmospheric Administration (NOAA), have long-term data records of exceptional quality.

Air Sampling Standard Operation Procedures for Dichotomous Virtual Impactor and Micro-orifice Uniform Impactor

# Equipment

Dichotomous (Andersen Model 245)

Yellow filter cartridge

White filter cartridge

Two 37 mm diameter teflon filters with PMP ring

2 (1/4” ID vacuum tubes)

Dichotomous stand with PM10 inlet

Dichotomous housing with control module

MOUDI (MSP Model 110)

MOUDI

10 stages

10 impaction substrate holder

Rotator cabinet

11 (37 mm diameter teflon filters with PMP ring)

Vacuum pump (Leybold Heraeus type S4A 1/3 HP Rotary)

Dry gas meter

Stop watch

Absolute pressure meter

### Tubes (3/8” ID) and fasteners

Lab notebook with appropriate tables and pen

13 petri dishes

Temperature and humidity meter

Extension cord

6 outlet surge protector

# Dichotomous

- Power requirement: 100/115 VAC 50/60 Hz, 8 A

- Filters are pre-weighed with a microbalance (Mettler H54AR) and labeled according to the color of the filter cartridge for fine (white) and coarse (yellow) particle collection.  Temperature and humidity are taken concurrently and compared to those after the collection process.

- The filters are inserted into the corresponding filter cartridge using plastic tweezer.

- Start up procedure:

- Check flow rate of fine (15 lpm) and coarse (1.67 lpm) rotameter.

- Check automatic sampling program.

- Place filter cartridges with filters into appropriate slot.

- Record initial time and volume.

- Begin sampling.

# MOUDI

- Substrate preparation:

- A mixture ratio of 1:4 oleic acid to cyclohexane by volume is used to reduce bouncing.

- Turner and Hering (1987) suggested that 2 L oleic acid will be adequate for 8 mm diameter giving a thickness of approximately 30 um on 2 um pore teflon filters.

- Gelman teflon filters type R2PJ037 are used as impaction surfaces and after filter.

- The MOUDI substrate requires 28.6 mm of coating for a 37 mm substrate so 120 L of the mixture is applied using the micro pipet in drops of 20 L.

- Place impaction substrate in heat-resistant containers.

- The substrate with the oil is heated in an oven at 70C for 2 hours to evaporate the cyclohexane.

- When all solvent evaporated, let cool in closed petri dish for 30 min.

- Weigh oiled substrate and one un-oiled filter (to be used as after filter).

- Clean MOUDI substrate holder with acetone and wipe with kimwipes.

- Place substrate into MOUDI substrate holder and after-filter holder.

- MOUDI handling:

- Time (~2 minutes) should be allotted for the pressure in the MOUDI to equilibrate with atmospheric pressure at pump shut down to minimize back flow.

- If the final stage’s pressure drop increases while the flow remains constant then the nozzles are plugged.   Clean by removing nozzle from impactor (never push directly on center of nozzle plate) and soak in mild detergent followed by rinsing in distilled water and alcohol.

- When reassembling, apply vacuum grease to double o-rings between the stages to reduce friction during rotation.

- MOUDI setup:

- Power requirement: 	Rotator:  115 VAC 50/60 Hz, 0.3 A

Pump:     115 V  60 Hz, 6.7 A

- Make sure pump exhaust is connected to >12 ft long tubing downwind of sampler to prevent contamination.

- Verify flow rate of MOUDI: 30 + 1  lpm using dry gas meter, stop watch, and absolute pressure meter.  At 30 Lpm, pressure drop: inlet to stage 7 is 17.7 inches of H20 and stage 7 to filter base is 141 inches of H20.

- Record initial pressure drop, flow rate, and volume of dry gas meter.

- Start rotator if uniform deposition is required and began sampling when dichotomous starts.

End of day:

- Record final time and flow rates of course and fine particle collection for dichotomous.

- Transfer fine and coarse filters with the filter cartridge into appropriately labeled petri dishes for gravimetric analysis in lab.

- Calculate final flow rate for MOUDI using dry gas meter, stop watch, and absolute pressure meter.

- Record final pressure drop, flow rate, and volume of dry gas meter of MOUDI.

- Remove MOUDI from rotator apparatus, cover inlet and outlet with parafilm and transfer to lab, where the impaction substrates are transfer to appropriately labeled petri dishes and their final mass taken.  Any oil left on impaction substrate holder is noted.  After gravimetric analysis all filters are stored in freezer for pickup and chemical analysis.

- Download meteorological data for sampling time.

References:

Turner, J.R. and S.V. Hering (1987).  Greased and oiled substrates as bounce-free impaction surfaces.  J.  Aerosol Sci., 18,  215-224.

Microlayer Sampling

Standard Operation Procedures

##### I.  EQUIPMENT

#### Microlayer sampler

Drum

Stainless Steel Frame

2 Drive Motors

Motor wiring

Floats

4 eyebolts

Small skiff

Electric trolling motor

Two 12-volt deep cycle batteries

Sample containers

Sample container holder

Measure sea surface tension

#### Laboratory detergent (squirt bottle) and sponge

GPS (differential GPS recommended)

Pad of paper and pencils

## I.A.	A waterproof drum (18 by 24 inches, 1/8 inch thick) is made of 6061-T6 aluminum.  The drum is covered with a pressure-sensitive overlay coating: 1 mil (1/1000 inch) thick film of Teflon FEP with an 8 mil thick vinyl sheet backing.

I.B.	The frame is made of stainless steel tubing.  Wires from the motor to the battery run through the tubing.

I.C.	Two drive motors operate on 12 volts DC providing 35 in-lbs of torque to operate the drum at 9 RPM.  Motors are mounted so they turn in opposite directions.  The drum always rotates toward the wiper.  The motor are Dayton Permanent Magnet 12 volt DC; 9 F/L RPM; 1/120 input HP; 1.1 F/L amps; 35 in-lbs F/L torque; 267.5 to 1 gear ratio; Sleeve bearing Tenz class “A”; Model #4Z836.  Spare motors are recommended in case of damage during use.

I.D.	The wiper is a teflon or teflon coated blade with a 15 degree edge.  The wiper support assembly has stainless springs on either side for adjusting blade tension against drum.  The blade is held in place by 12 flathead screws (stainless steel or nylon).  A handmade PVC spigot is placed on the end of the wiper to direct the water into the sample container.

I.E.	Floats are foam Bogie Boards designed for easy replacement if damaged.  Floatation allows approximately 2 inches of the drum to sit beneath the water surface.

I.F.	Four ¾ inch eyebolts hold floats to the sampler frame and serve as supports for the sling used to lower and raise the sampler from the ship.  Eyebolts also allow sampler to be tied next to the skiff at sea.

I.G.	A small skiff is used to deploy the sampler at sea.  This reduces contamination from the primary vessel.  The skiff should be equipped with a 12 volt DC reversible trolling motor for propulsion and to reduce potential contamination from gasoline outboard motors.  Oars and life jackets are recommended for safety.

I.H.	Two 12 volt DC 90 amp deep cycle batteries are carried in the skiff.  One supplies power to the sampler and the other supplies power to the troll motor.  A battery charger is recommended to recharge batteries at the end of each day.

I.I.	The sampler is designed to accommodate a standard 4 liter amber reagent bottle that is held in place by the sample holder.  Sample containers smaller then 4 liter require a jig to hold them in place.  Jigs are easily made out of Styrofoam and should be of sufficient height that the spigot fits into the top of the sample bottle.  The Styrofoam jig is held in place by tape or a bungie cord fastened to the frame.

I.J.	Sea surface tension is measured with an oil-alcohol mixture referred here as the “Oil Drop Technique” (Adams 1937). This technique indirectly measures surface-active material present in the microlayer during sampling.

I.K.	A laboratory detergent is used to clean the drum prior to collecting samples at each station.  We use Mirco all-purpose liquid cleaner from American Scientific Products, a mixture of anionic and LABORATORY agents, or ALCONOX.  The detergent is kept in a squeez bottle for easy application.  A soft sponge is used to wash the drum.

# II.  PROCEDURES

II.A.	Sample Containers

II.A.1.	Organic Chemical Analysis

Containers used are re-cleaned 4 liter amber organic reagent bottles.  Labeling is removed.  The bottles and caps are washed with detergent (Micro), rinsed with tap water, then rinsed with deionized or distilled water.  Bottles are kilned for 4 hours at 565°C.  The caps are sonicated for 30 minutes in analytical grade methylchloride, and dried for 5 minutes in oven at greater than 60°C.  After bottles have cooled to room temperature, they are sealed with the caps.

II.A.2.	Trace Metal Analysis

Containers used are either re-cleaned or manufacturer certified clean (plastic or glass).  To re-clean containers, remove any labeling from it’s surface.  The bottles and caps are washed with detergent (Micro), rinsed with tap water, then rinsed with deionized or distilled water.  The bottles and caps are soaked in 6% nitric acid solution (prepared in deionized or distilled water) for a minimum of 24 hours.  The bottles and caps are rinsed with double distilled water and sealed with the caps.

II.A.3.	Miscellaneous Analysis

Sample containers must be cleaned by an appropriate method so little or no contamination of the sample can be attributed to the collection bottle.  Examples of miscellaneous analysis are total suspended solids, total organic carbon, bacteria, oil and grease, toxicity, and biology.

II.B.	Oil Drop Technique

II.B.1.	Equipment

MINERAL OIL

LAURYL ALCOHOL

DISPENSING BOTTLES

-Mineral oil (paraffin oil, liquid petrolatum) is white, light N.F.; Mallinckrodt 4 liter bottle; supplier is Baxter.

-Lauryl alcohol os 1-Dodecanol or Dodecyl alcohol; supplier is Sigma (No. L-5375).

-Eight 60 ml Nalgene low density polyethylene bottles with attached cap are used to dispense the solution; supplier is Baxter (No. 2411-0060)

II.B.2.	Procedure

The lauryl alcohol is placed in 26°C water bath until all material is in an aqueous state, rather than in solid phase.  Mineral oil is added (60 ml) to each Nalgene bottle.  Varying amounts of lauryl alcohol are pipetted into Nalgene bottles containing mineral oil according to the table below.  Each bottle is labeled with the percentage of lauryl alcohol and it’s representative surface pressure or tension.

Amount	Equivalent	Surface Pres.

Dispensed	Lauryl Alcohol	or Tension

µl	%	dyne/cm

30	0.05	clean water

42	0.07	1

60	0.1	3

120	0.2	6

180	0.3	12

300	0.5	16

420	0.7	19

600	1.0	22

Surface pressure or tension is estimated while the microlayer is sampled, usually by the skiff operator.  When the microlayer sample bottle is partially filled, place one drop of oil from the lowest concentration of laurly alcohol on the water surface.  If the drop does not spread, use successively higher concentrations.  One drop will spread a little and the next higher concentration will spread a lot; surface tension is somewhere between the two concentrations.  If the oil does not spread at 1.0% lauryl alcohol, the surface tension is greater than 22 dynes/cm.  Record the concentration.  It is important not to contaminate the microlayer samples with oil.  Surface tension should be measured at least twice during the sampling operation.

III.  METHOD

III.A.	Transportation

The microlayer sampler and accessory equipment are stored in a safe location where they are protected by a tarpaulin from falling objects and dust.  The equipment is transported by a work van.  Place the inflatable rubber boat on the mat, bow toward the driver’s seat.  Place accessory equipment (e.g., electric motor, rain gear, boots, sample bottles, etc.) inside the boat.  Place a 4 ft x 8 ft x 5/8 in sheet of plywood on top of the boat (supported by seats and pontoons).  Place sampler gently on top of the plywood.  Extreme care is taken as to not damage the rotating drum.  Secure sampler to the boat and van by rope tied through the eyebolts.  Place batteries in the rear of the van and secure them so they will not fall (spill acid) or move during transport.  Upon arrival at the dock, unload equipment in reverse order onto a research vessel.  Protect sampler from atmospheric dust and soot whenever possible by covering it.  Whenever possible, remove obvious sources of contamination (e.g., gasoline, oil, hydraulic fluid) from the vicinity of the equipment and operation.

III.B.	On Location

Prior to sampling, it is recommended that the inflatable boat be washed (inside and out) with detergent and rinsed well with seawater.  The boat is lowered over the side and the boat operator climbs aboard.  The electric motor is handed to the operator and subsequently secured to the stern of the inflatable.  One battery is placed in the stern for the electric motor; the other in the center section against one pontoon.  The accessory equipment (sample bottles, oil drop bottles, GPS, watch, pad of paper, pencil, and two-way radio) is handed to the operator and stowed.  The second person is assisted into the boat.  The electric motor is tested for proper operation and inflatable is cast off.

A 4-point sling is clipped to the eyebolts of the microlayer sampler.  The sampler is lowered over the side of the research vessel.  Care must be taken not to bump or damage the drum against the vessel.  The rubber boat is maneuvered to the microlayer sampler that is floating on the water.  Personnel in the inflatable unclamp the sling and secure the sampler to their boat close to the battery.  Maneuver the inflatable away from the mother ship at least 100m to the windward.  Squirt the drum and blade with LABORATORY detergent and wipe with a soft sponge while the drum is rotating.  Rotate drum for 2-3 minutes to rinse drum, blade and spigot with ambient seawater/microlayer.  Arrange sample containers, oil mixture, GPS, watch, and note pad for easy accessibility.

While the drum is rotating, place a sample container under the spigot and record the start time and GPS position.  The inflatable is driven slowly forward by electric motor at a constant speed.  The recommended speed is equivalent to the rotational speed of the drum (1 km/hr), but environmental factors (wind, ocean current, langmier cells) at each site determine actual sampling speed.

Sampling transects are determined by the operator.  These transects can follow a predetermined path, compass course, visual straight line, zigzag, circular, or biased toward sampling inside or outside a visible slick area.  Surface tension is estimated with the oil drop technique while the container is filling.  Transects should not cross a previously sampled area because the microlayer sample could be contaminated from the oil drops.  Transects should follow an up-wind direction to avoid this potential contamination problem.  The maximum recommended fill time for each bottle is one hour.

Record time, GPS position, and surface tension periodically during sampling and at completion.   Record ancillary information (e.g., particles or debris in the water or microlayer, slicks present, anything unusual), which could be valuable in interpreting analysis done in the laboratory.  While the inflatable boat is sampling, the skipper of the mother ship fills out a station occupation form.  It describes the general location and sea condition found at the site.

After sampling is completed, the operator maneuvers toward the mother ship or the mother ship comes along side the rubber boat.  The sling is clipped to the eyebolts of the sampler and it is carefully brought aboard the research vessel.  The rubber boat is maneuvered to the stern of the mother ship and the gear unloaded.  The inflatable boat is brought aboard and staged for easy deployment at the next station.

Any analyses and post-preparation of freshly collected microlayer sample is done at this time.  Micolayer samples are stored in a safe place.  Sample containers are prepared (labeled and arranged) for the next station.

IV.  END OF THE DAY

The microlayer drum is cleaned with detergent after the final sample is collected.  The sampler and rubber boat are rinsed with freshwater, covered, and secured for the trip back to port.  If no freshwater is available on board ship, the equipment is rinsed on the dock or at lab.  The equipment is loaded into the van and transported back to lab.  In the lab, the equipment is rinsed with freshwater, covered, and placed in storage.

V.  REFERENCE

Adam, N. K.  1937.  A rapid method for determining the lowering of tension of exposed water surfaces. with some observations on the surface tension of the sea and inland waters.  Proc. R. Soc. London (B) Biol. Sci. 122:134-139

# QAQC

As identified in Figure 4 (Figure number refers to Appendix A, Workplan), Dr. Keith Stolzenbach, Dept. of Civil Engineering, UCLA (Los Angeles, CA 90024, Tel: (310) 206-7624 and Mr. Ken Schiff, Southern California Coastal Water Research Project (SCCWRP, Westminster, CA 92683, Tel: (714) 894-2222) are the Principal Investigator and Co-Principal Investigator, respectively for this project.  They will be directly responsible to the Santa Monica Bay Restoration Project (SMBRP) for the quality and timely completion of this project.  They will also be responsible for data interpretation and for preparation and submission of reports to the SMBRP.

Dr. Keith Stolzenbach and Mr. Ken Schiff will be assisted by six task leaders as shown in Figure 4 (Figure number refers to Appendix A, Workplan).  Dr. Sheldon Friedlander of UCLA will be the task leader responsible for coordination and logistics of the atmospheric sampling activities and sample tracking and control.  Mr. Dario Diehl of SCCWRP will be the task leader responsible for coordination and logistics of the wet/dry deposition  and sea surface sampling activities and sample tracking and control.  Mr. Richard Gossett (CRG Laboratories, Terminal Island, CA 92   , Tel: (310) 519-4007)  will oversee all sample analysis and review and evaluate all analytical data generated during the project.  CRG Laboratories is an Independent Research Laboratory State Certified in the fields of solid waste, water and wastewater analysis.  Dr. Richard Turco of UCLA will oversee atmospheric modeling activities and review and evaluate modeling results.

Data management will be the responsibility of SCCWRP.  Kenneth Schiff will manage the storage, retrieval, and manipulation of data generated during the project.

11.	DATA QUALITY REQUIREMENTS AND ASSESSMENT

Quality Assurance Objectives

A vigilant program of quality assurance will be carried out in order to provide data of high quality. Quality assurance and quality control will be facilitated in this project by establishing data quality objectives (DQOs) for sampling and laboratory analysis.  These DQOs are intended to be benchmarks for achieving acceptable quality in the study progress including completeness, accuracy, and precision.  The DQOs established in this study are consistent with those established by the regionally-coordinated monitoring programs within Santa Monica Bay and has been adopted by NEP office and local regulatory agencies (SCBPP 1994).  Adhering to these DQOs will facilitate comparability among other measures in the region.

Table 3 (table # refers to Appendix A, Workplan) identifies each of the DQOs for both field sampling activities and laboratory activities.  These DQOs are focused upon completeness, accuracy, and precision criteria.  It is our intention to maintain sampling success for any of our logistics programs to be 95% or better.  In addition, analytical laboratory sample process success must meet or exceed those selected for field activities.  The DQOs for analytical laboratory precision and accuracy were performance-based measures established by a consortium of local laboratories participating in the regional monitoring activities in southern California (SCBPP 1994).  These performance-based measures represent initial and ongoing demonstrations of quality assurance including repeated analysis of locally-derived reference materials, repeated analysis of native samples (laboratory duplicates), repeated analysis of fortified native samples (matrix spikes), as well as negative controls (blanks) for laboratory and field activities.  These quality assurance activities shall occur at a frequency of not less than 5%.  The laboratory selected for this project has successfully participated in all of the regional monitoring interlaboratory intercalibrations.  The laboratory manager has served as the Quality Assurance Officer and Chair of the Analytical Chemistry Workgroup in previous regional monitoring projects.

Each collaborator is mandated to train personnel and maintain equipment.  Both UCLA and SCCWRP have training programs in place for new personnel and stock replacement equipment in the event of an equipment failure.  Training programs for field sampling activities have been built upon 30 years of curriculum at both SCCWRP and UCLA.  Training activities include safety, emergency response, equipment preparation and use, sample non-contamination, navigation, sample collection procedures, and sample storage.  Successful training performance evaluation is conducted primarily through sample collection audits and secondarily by review of logs and monitoring documentation.  For seagoing activities in particular, back-up plans are laid prior to departure in the event of equipment or weather related failures.  Each vessel to be used in this study maintains licensed captains with the US Coast Guard, including trained and able crew in the event of an emergency.  All vessel positioning will utilize satellite global positioning system (GPS) accurate to within 10m.

Representativeness refers to insuring that subsamples have the same constituents and concentration as the original sample from which they were taken.  Through carefully obtaining representative filter samples, judicious use of sample mixing and rinsing of volumetric glassware (i.e. microsyringes and pipettes), representativeness can be obtained without loss of target compound.  The representativeness of ambient air samples are assured through the use of 24 hour sampling to avoid short-term temporal variability and utilizing sampling sites that are distant from known mobile or stationary sources of air pollutants.

Comparability is particularly important for assessing relative risk among different types of emission sources and cross-media transport of pollutants.  In particular, we have designed a system of split sample analyses with the AQMD so that comparability can be evaluated with their TEP 2000 program.  This system of split samples involves the AQMD providing one set of filtered aerosols per month from each of their sampling sites to our project laboratory.  The rationale for this design is two-fold.  First, there is a need to assess comparability among laboratories that may be due to differences in analytical methodology or techniques (samples are collected in equivalent fashions).  Second, the sharing of samples from AQMD enables the evaluation of multiple sites that encompass a range of potential concentrations.  Evaluation of comparability will be assessed using control limits developed during the regional monitoring intercalibration exercises.  While the AQMD was not a participant in the intercalibrations, interlaboratory variability among 12 different labs was quantified and these control limits will be applied to the split samples.  If an inherent and consistent bias is evident in the split sample evaluation, the AQMD data will be normalized based upon the mean quotient of the two labs.  If no reasonable correction factor can be applied to the AQMD results, the study will rely upon the split sample analysis conducted in our laboratory.

Comparability will also be obtained through the careful monitoring of equipment output, frequent standard analysis, and well-maintained equipment.  Operator exchangeability will be insured through the development of and adherence to, operation protocols consistent with manufacturer and EPA specifications and requirements.

12.	Sampling and Laboratory Procedures

Sample collections will be performed at the various collection points by UCLA and SCCWRP personnel who will be provided with any necessary training.  Copies of Standard Operating Procedures for field collections of aerosols and microlayer samples are included in Appendix B.  Collected samples will be stored in hard plastic (e.g. Coleman) ice chests with blue-ice packs and transferred upon receipt to a refrigerator at 4C or freezer at -4C.  Table 12.1 contains a list of samples to be collected along with the type of filters and preservatives used for each sample.  The time between sampling and receipt of samples at SCCWRP is expected to be less than 8 hours.  Bottles and deposition surfaces will be washed using EPA procedures including acid for trace metals and scale plus solvents for trace organics.  Samples will be preserved and stored according to EPA or Standard Methods.  Travel, field and laboratory blanks will also be analyzed, see Section 16.

Tables 12.2 and 12.3 lists the details of the laboratory analysis parameters to be used.  We intend to use modified EPA methods ) for analysis of organic constituents.  The main modification of these methods relative to EPA protocols is the use of mass spectrometry detectors (GC-MS) compared to electron capture detectors (GC-ECD).  We have chosen the use of GC-MS to ensure accurate quantification of some target analytes; GC-ECD will often produce “false positives” for some target analytes, specifically for some PCB congeners.  Unfortunately, GC-MS methods are generally less sensitive than GC-ECD methods.  To compensate for the loss in sensitivity, the laboratory will increase the sample size when possible, and decrease final extract volumes to 500 L.  These modification concentrates sample masses thereby increasing our ability to detect target analytes at lower levels.  Table 12.4 documents detection limits attained in recent studies conducted by this laboratory using these techniques.

We intend to measure two target PAH analytes that are listed in table 12.2, but are not listed in the accompanying SOP for PAH analysis (see Appendix B).  These include 2-methylphenathrene and methylanthracene.  These analytes are measured using the same techniques as described in the accompanying SOP and have been successfully measured by the analytical laboratory in previous studies (see Table 12.4).  In a similar scenario, we have opted to measure the PCB congeners listed in table 12.2 rather than aroclors as described in the accompanying SOP for analysis of chlorinated hydrocarbons.  The PCB congeners listed are measured using the same techniques as described in the accompanying SOP for chlorinated hydrocarbons (see Appendix B) and have been successfully measured by the analytical laboratory in previous studies (see Table 12.4).

Two types of analysis identified in the Workplan (see Appendix A) have been removed from the target analyte list.  The first analysis is chlorinated hydrocarbons (DDTs and PCBs) from wet deposition.  The second type of analysis removed from the target analyte list in the Workplan was chlordane in aerosol samples.  The removal of chlorinated hydrocarbon and chlordane analyses was due to non-occurrence of these compounds in samples conducted by other investigators.

We intend to use EPA method 200.8 for analysis of trace metals in both liquid and solid matrices.  The SOP for analysis of trace metals in solid matrix using EPA method 200.8 is located in Appendix B.  We will use this same method and instrumentation for analysis of liquid matrix samples.  The modification from the SOP to accommodate liquid matrix concerns sample digestion.  Both solid and liquid matrices will be digested using concentrated nitric and hydrochloric acids.  Liquid samples, however, will not be heated or decanted as would a sediment sample.  Sediment samples require these steps to break down and extract particles from the sample matrix, steps that are not necessary for liquid matrices.

Mercury is not an analyte of primary concern in this study.  However, inductively coupled mass spectrometry (ICP-MS) analysis can produce mercury results if sufficient mass exists in the samples.  Therefore, we will use this method as a screening tool for mercury to assess the need for future studies using alternative methods.

Measurements of surface tension are made during field sampling of the sea surface microlayer.  Organic film thickness (i.e. microlayer thickness) is can be indirectly measured by surface tension.  Surface tension will be measured using the oil drop test by Adams (1937).  This measurement is conducted by placing drops of varying oil concentrations (dodecyl sodium sulfate:alcohol) until a sheening is encountered.  This test is conducted to the lee side of the sampling vessel so there are no interferences with the microlayer sampler.  Accuracy of this method is +/- 1 dyne.

Routine Assessment of Measurement Parameters

Precision will be assessed by performing duplicate analyses of single samples, then evaluating the resultant difference.  An action level is detailed in the attached workplan (Appendix A) for implementation in the monitoring phase.  Laboratory controls and spikes are expected to provide an estimate of the accuracy for the compounds which will be found in the samples.  Following an initial trial phase, we anticipate that the completeness of our analysis should approach 100%, and must exceed 95%, for the duration of the study.

Internal Quality Control Checks

EPA methods will be followed whenever possible; other techniques will be carefully scrutinized, checked, and calibrated prior to use.  Solvent blanks will be analyzed. Calibration standards will be prepared from registered standard sources according to EPA methods and marked with the analyst’s initials, date of preparation, and compound concentration.  Standards will be stored in Teflon sealed containers at either 4C for aqueous samples or -10C for standards in solvents.  They will be disposed of after exceeding shelf life, holding times exceedences, or compromise such as contamination.  Replicate samples will be collected and analyzed to insure the accuracy and validity of the data collected during this study. Instrument reliability will be monitored every day.

13.	SAMPLE CUSTODY PROCEDURES

A chain of custody has been developed which will provide an assurance that samples are not tempered with and maintain a written record tracing the samples through analysis and validation.  The Chain of Custody form is shown as Figure 13.1.  The chain of custody requires that each transfer be recorded with signature and date.  In addition, field logs will be maintained by Dario Diehl, field crew supervisor, which records observations and sampling events.  Once received by the QA officer the samples will be stored in a designated locked locality until ready for analysis.  At that time, the analyst will record his taking of the sample and be responsible for its final utilization while the QA officer retains the Chain of Custody form.  Any irregularities will be reported to the Task Manager.

14.	CALIBRATION PROCEDURES AND PREVENTIVE MAINTENANCE

All solvents and sample blanks will be tested just prior to actual use to insure purity and reproducibility of analysis.  Solvent batches of questionable purity will be redistilled and retested or outright rejected.  Blanks that produce high artifacts will be investigated and the cause of contamination removed.  Calibration and stock standards will be prepared from commercially available reference standards, and will be stored in the dark using cleaned vials with ground glass stoppers  at temperatures of 4C.  Those preparing calibration solutions will always include their name, the date, compound name and concentration, and will record the method of preparation source and reference number of the stock standard.  Calibrations will be carried out as specified in the appropriate EPA protocols and laboratory Standard Operating Procedure.  Sample quantification will be carried out using either the internal and/or external standard method, as is appropriate for the analysis being undertaken.

Routine preventive maintenance of each instrument is carried out by the analyst in accordance with manufacturer’s specifications although slight variations may be adopted for daily operating schedules.  Critical consumable and replacement parts are stocked within the laboratory to minimize downtime and most other parts can be ordered and delivered within 48 to 72 hours.

15.	DOCUMENTATION, DATA REDUCTION AND REPORTING

Documentation will be completed by raw data being placed in laboratory notebooks.  The notebook data will be placed in data management files on a computer spreadsheet for data reduction and reporting purposes.  All field data sheets for wet and dry deposition samples, and sea surface microlayer measurements will be maintained at SCCWRP.  These field data sheets (Appendix B) include parameters such as date/time, station code, station location, weather and sea conditions, activities performed, and samples collected.  These data sheets will be checked by the field sampling supervisor after each event.  All field data sheets for aerosol sampling will be maintained at UCLA.  These field data sheets (Appendix B) include parameters such as date/time, station code, station location, flow rates, and samples collected.  These data sheets will be checked by the field sampling supervisor after each event.  An independent check of the electronic data input will be reviewed by a designee of the project QA officer.  This electronic data review will occur following each sampling index period (every two months).

Data reduction will be carried out under the direction of the analyst who collected the data, or by the analyst himself.  Since several different types of analyses will be performed, no general data reduction technique is proposed here, however each type of analysis will follow EPA suggested guidelines or, lacking that, will employ those suggested by the system manufacturer.

Reporting of data will be completed in comprehensive reports to the project officer.

16.	DATA VALIDATION

Data validation refers to the myriad of activities which insure that procedures and quality control parameters are within acceptance criteria so that data can be reported with confidence.  In order to ensure the validity of our efforts and data we will undertake the following procedures:

1)	EPA approved methods will be our primary protocol source with other sources requiring in-house validation prior to analytical use on samples;

2)	Solvent blanks will be run on each fresh bottle of solvent to insure purity;

3)	Laboratory reagent water will be tested prior to use in blanks, the preparation of standards or in analytical workups;

4)	Travel, field and laboratory blanks will be utilized to monitor contamination interferences.  A field blank or equipment blank will be run for each matrix per sample day;

5)	Analytical processes will be carefully described as protocols to insure that the identical extraction and analytical procedures are followed throughout the study;

6)	A validation of instrument calibration and performance will be undertaken prior to any sample analysis;

7)	Replicates samples will be collected.  Duplicate samples will be collected at a frequency of not less than 10%;

8)	Refrigerated samples will be allowed to warm to room temperature prior to processing;

9)	The passage of highly concentrated samples will be confirmed through the use of solvent blanks until no memory effect is observed;

10)	Unusual observations or instrument performance will be carefully investigated, corrected and documented.  Following analysis, the raw data will be reviewed for validity by the analyst, then transcribed into our analytical master data base, but also retained on data sheets for the duration of the study.  Periodic data reviews and approvals will be carried out by the QA officer and laboratory Manager.

11)	The data generated by this project does not require a formal external validation.  However, data will be internally reviewed and verified through our information management system that includes range checks, duplicate record examination, data completeness, and a minimum 10% random raw data audit.

17.	Performance and System Audits

The analytical laboratory participates in both the EPA Check sample program and the State of California Environmental Laboratory Accreditation Program (ELAP).  The analytical laboratory is State Certified.  Each of these programs offer external check samples that are submitted blind on a periodic basis.  The samples to be included in the study covers general water quality parameters, See Table 12.2.  This program is coordinated through the QA officer who intends to include blind check samples of some pertinent compounds in selected samples.  System audits for other portions of the study are not required.

Table 12.1 Field Sampling Parameters

| Parameter | Sample Vol | Sample Container | Processing & Storage |
| --- | --- | --- | --- |
| Field Samples |  |  |  |
| Wind speed | n/a | N/a | logged to CR10 in mph every 15" |
| Wind direction | n/a | N/a | logged to CR10 in Deg N every 15" |
| Rainfall | n/a | N/a | logged to CR10 every 0.04" |
| Barometric Pressure | n/a | N/a | logged to CR10 in mBars every 15" |
| Surface Tension | n/a | N/a | Oil drop test +/- 1 dyne |
| Lat/Long | n/a | N/a | Differential GPS in 0.01' NAD83 |
| Lab Samples |  |  |  |
| Water/Microlayer Samples |  |  |  |
| Trace metals | 500 ml | HDPE | acidified with HNO3 < pH2;4C |
| Aerosol Samples |  |  |  |
| Trace metals | > 2,000 m3 | Teflon Filters | -4C |
| Trace organics | > 2,000 m3 | Teflon Filters | -4C |
| Dry Deposition |  |  |  |
| Trace metals | 6 days | Teflon Filters | -4C |
| Trace organics | 6 days | Teflon Filters | -4C |
| Wet Deposition |  |  |  |
| Trace metals | 500 ml | HDPE | acidified with HNO3< pH2;4C |
| Trace organics | 1,000 ml | Borosilicate Amber Glass with Teflon liners | 4C |
| Ammonia | 500 ml | HDPE | 4C |

TABLE 12.2  Reporting limits (RL) for target analytes by media type.

|  | Aerosol Particulates (Teflon Filters)a | Wet Deposition (Water) | Seasurface Microlayer (Seawater) |
| --- | --- | --- | --- |
| # Trace Metals (ppm, mg/kg, mg/L) |  |  |  |
| Cadmium | 0.2 | 0.2 | 0.2 |
| Chromium | 16 | 16 | 16 |
| Copper | 7 | 7 | 7 |
| Iron | 500 | 500 | 500 |
| Lead | 9.3 | 9.3 | 9.3 |
| Mercury | 0.03 | 0.03 | 0.03 |
| Nickel | 4.2 | 4.2 | 4.2 |
| Silver | 0.2 | 0.2 | 0.2 |
| Zinc | 30 | 30 | 30 |
| # Polycyclic Aromatic Hydrocarbons (ppb, g/kg, g/L) |  |  |  |
| 1-Methylnaphthalene | 50 | - | - |
| 1-Methylphenanthrene | 50 | - | - |
| 2,6-DimethylNaphthalene | 50 | - | - |
| 2,3,5-Trimethylnaphthalene | 50 | - | - |
| 2-Methylphenanthrene | 50 | - | - |
| Acenaphthene | 50 | - | - |
| Acenaphthylene | 50 | - | - |
| Anthracene | 50 | - | - |
| Benz[a]anthracene | 50 | - | - |
| Benzo[a]pyrene | 50 | - | - |
| Benzo[g,h,i]perylene | 100 | - | - |
| Benzo[k]fluoranthene | 50 | - | - |
| Biphenyl | 50 | - | - |
| Chrysene | 50 | - | - |
| Dibenz[a,h]anthracene | 100 | - | - |
| Fluoranthene | 50 | - | - |
| Fluorene | 50 | - | - |
| Methylanthracene | 50 | - | - |
| Indeno[1,2,3-c,d]pyrene | 100 | - | - |
| Naphthalene | 50 | - | - |
| Perylene | 50 | - | - |
| Phenanthrene | 50 | - | - |
| Pyrene | 50 | - | - |

Table 12.2  continued

|  | Aerosol Particulates (Teflon Filters)a | Wet Deposition (Water) | Seasurface Microlayer (Seawater) |
| --- | --- | --- | --- |
| Polychlorinated Biphenyls (ppb, g/kg, g/L) |  |  |  |
| Congeners 18, 28, 37, 44, 49, 52, 66, 70, 74, 77, 81, 87, 99, 101, 105, 110, 114, 118, 119, 123, 126, 128, 138, 149, 151, 153, 156, 157, 158, 167, 168, 169, 170, 177, 180, 183, 187, 189, 194, 201, 206 | 7.5 | - | - |
| Chlorinated Pesticides (ppb, g/kg, g/L) |  |  |  |
| o,p’-DDD | 1 | - | - |
| p,p’-DDD | 1 | - | - |
| o,p’-DDE | 1 | - | - |
| p,p’-DDE | 1 | - | - |
| o,p’-DDT | 1 | - | - |
| p,p’-DDT | 1 | - | - |
| Ammonia (ppm, mg/L) | - | 17 | - |

a  Aerosol concentrations (in mg/m3) are calculated by dividing the given concentration (mg/kg) by the total mass of the aerosol sample and volume of air sampled (kg/m3).

Table 12.3  Laboratory Analysis Parameters

| Parameter | Matrix | Units | Method | Storage and Hold Time |
| --- | --- | --- | --- | --- |
| Microlayer Samples |  |  |  |  |
| Trace metals | seawater | ug/l | Modified EPA 200.8 | 6 mos. acidified with HNO3 @ 4C |
| Aerosol Samples |  |  |  |  |
| Trace metals | Teflon Filters | ng/m3 | Modified EPA 200.8 | 6 mos @ -4C |
| Trace organics | Teflon Filters | ng/m3 | Modified SW846-8081 (Pesticides) SW846-Modified 8082 (PCB congeners) Modified SW846-8270 (PAHs) | 2 mos @ -4C |
| Dry Deposition |  |  |  |  |
| Trace metals | Teflon Filters | ng/m3/hr | Modified EPA 200.8 | 6 mos @ -4C |
| Trace organics | Teflon Filters | ng/m3/hr | Modified SW846-8081 (Pesticides) SW846-Modified 8082 (PCB congeners) SW846-8270 (PAHs) | 2 mos @ -4C |
| Wet Deposition |  |  |  |  |
| Trace metals | water | ug/l | Modified EPA 200.8 | 6 mos. acidified with HNO3 @ 4C 28 d for mercury |
| Ammonia | water | ug/l | SM 4500-NH3D | 24 hrs. @ 4C |

TABLE 12.4  Method detection limits (MDL) for target analytes by media type achieved by the participating laboratory on recent projectsa,b,c.

|  | Solid Matrix | Water Matrix | Seawater Matrix |
| --- | --- | --- | --- |
| # Trace Metals (ppm, mg/kg, mg/L) |  |  |  |
| Cadmium | 0.05 | 0.001 | 0.001 |
| Chromium | 0.05 | 0.001 | 0.001 |
| Copper | 0.05 | 0.001 | 0.001 |
| Iron | 10 | 0.050 | 0.050 |
| Lead | 0.05 | 0.001 | 0.001 |
| Mercury | 0.01 | 0.001 | 0.001 |
| Nickel | 0.05 | 0.001 | 0.001 |
| Silver | 0.01 | 0.001 | 0.001 |
| Zinc | 0.05 | 0.001 | 0.001 |
| # Polycyclic Aromatic Hydrocarbons (ppb, g/kg, g/L) |  |  |  |
| 1-Methylnaphthalene | 5 | - | - |
| 1-Methylphenanthrene | 5 | - | - |
| 2,6-DimethylNaphthalene | 5 | - | - |
| 2,3,5-Trimethylnaphthalene | 5 | - | - |
| 2-Methylphenanthrene | 5 | - | - |
| Acenaphthene | 5 | - | - |
| Acenaphthylene | 5 | - | - |
| Anthracene | 5 | - | - |
| Benz[a]anthracene | 5 | - | - |
| Benzo[a]pyrene | 5 | - | - |
| Benzo[g,h,I]perylene | 5 | - | - |
| Benzo[k]fluoranthene | 5 | - | - |
| Biphenyl | 5 | - | - |
| Chrysene | 5 | - | - |
| Dibenz[a,h]anthracene | 5 | - | - |
| Fluoranthene | 5 | - | - |
| Fluorene | 5 | - | - |
| Methylanthracene | 5 | - | - |
| Indeno[1,2,3-c,d]pyrene | 5 | - | - |
| Naphthalene | 5 | - | - |
| Perylene | 5 | - | - |
| Phenanthrene | 5 | - | - |
| Pyrene | 5 | - | - |

TABLE 12.4  continued

|  | Solid Matrix | Water Matrix | Seawater Matrix |
| --- | --- | --- | --- |
| Polychlorinated Biphenyls (ppb, g/kg, g/L) |  |  |  |
| Congeners 18, 28, 37, 44, 49, 52, 66, 70, 74, 77, 81, 87, 99, 101, 105, 110, 114, 118, 119, 123, 126, 128, 138, 149, 151, 153, 156, 157, 158, 167, 168, 169, 170, 177, 180, 183, 187, 189, 194, 201, 206 | 1 | - | - |
| Chlorinated Pesticides (ppb, g/kg, g/L) |  |  |  |
| o,p’-DDD | 1 | - | - |
| p,p’-DDD | 1 | - | - |
| o,p’-DDE | 1 | - | - |
| p,p’-DDE | 1 | - | - |
| o,p’-DDT | 1 | - | - |
| p,p’-DDT | 1 | - | - |
| Ammonia (ppm, mg/L) | - | 17 | - |

a  Solid matrix detection limits for inorganics and organics from the Southern California Bight Pilot Project, 1998.  Southern California Coastal Water Research Project, Westminster, CA

b  Freshwater matrix detection limits for inorganics from the Assessment of stormwater inputs from the Santa Ana River, California 1997/98.  Southern California Coastal Water Research Project, Westminster, CA

c  Seawater matrix detection limits for inorganics from the Assessment of Beneficial Use Impairments in Santa Monica Bay, 1997/98. Southern California Coastal Water Research Project, Westminster, CA

TABLE 2.  Analytes to be measured for estimating mass emissions from atmospheric depostion to Santa Monica Bay

Particulate Matter (PMx) e

Inorganic/Metals

Aluminum (Al) e, f, g, h

Arsenic (As) e, f, g, h

Cadmium (Cd) e, f, g, h

Chromium (Cr) e, f, g, h

Copper (Cu) e, f, g, h

Iron (Fe) e, f, g, h

Lead (Pb) e, f, g, h

Mercury (Hg) e, f, g, h

Nickel (Ni) e, f, g, h

Zinc (Zn) e, f, g, h

Organic/Pesticides

DDTa, e, f, g

PCBb, e, f, g

Chlordanec, e, f, g

Organic/Petroleum Hydrocarbons

PAHd, e, f, g

Nutrients

Ammonia g

a  includes o,p’ and p,p’ isomers of DDT, DDE, and DDD

b  41 PCB congeners used in monitoring of sediment and biota in SM Bay

c  alpha and gamma isomers of chlordane

d  24 PAH used in monitoring of sediments in SM Bay

e  analyzed in atmospheric particulates

f  analyzed in dry deposition

g  analyzed in wet depostion

h  analyzed in sea surface microlayer

TABLE 3. Control limits for analytical measurements of trace metals, polynuclear aromatic hydrocarbons (PAH), and chlorinated hydrocarbons (CHC).  Control limits from regional monitoring in the Southern California Bight (SCBPP 1994a).

|  | ###### Frequency | Control Limit |
| --- | --- | --- |
| Method Blank | 1/batch | < MDL for all analytes, or <5% of the measured sample concentration in samples for metals |
| Calibration Check |  |  |
| Metals | 10% |  10% true value for all metals |
| PAH | start&end per batch | < 20% RPD for 80% of all analytes of interest |
| CHC | start&end per batch | < 25% RPD for 80% of all analytes of interest |
| Matrix Spike and Matrix Spike Dups |  |  |
| Metals | 10% |  25% true value at <10x MDL,  20% true value at>10x MDL Dup analysis within 3 of control limit |
| PAH | 1/batch | 50 - 120% recovery for 80% of analytes at >10 MDL |
| CHC | 1/batch | 50 - 120% recovery for 80% of analytes at >10 MDL |
| CRM or LCM* |  |  |
| Metals | 1/batch | Within 3 of control limit |
| PAH | 1/batch | Within 3 of control limit |
| CHC | 1/batch |  30% true value for 80% of all analytes of interest |
| Laboratory Duplicates |  |  |
| Metals | 1/batch | < 25% RPD |
| PAH | NA |  |
| CHC | 1/batch | < 30% RPD |
| Surrogates |  |  |
| Metals | NA |  |
| PAH | 1/sample | Lab develop their own |
| CHC | 1/sample | Lab develop their own |

* Certified reference materials or Laboratory control materials

PREFERRED ANALYSIS METHODS:  Enter the recommended method or methods for analyzing the data here

a  Aerosol concentrations (in mg/m3) are calculated by dividing the given concentration (mg/kg) by the total mass of the aerosol sample and volume of air sampled (kg/m3).

USAGE:  Enter any caveats or additional information that you feel is important in using this data.

-99 = data not measured

NR = Not recorded

Tbl_Measurements  -  The corrected values field in the measurements table are the values calculated from subtracting the blank raw results.

The filter mass field represents the difference in the mass of the filter after being subtracted from its original (pre-exposure) filter weight.

KEYWORDS: List all keywords that you feel are important for this data set.  These words will be used by search engines to find you data set.

Aerosal

Dry Filter

Wet

Rainwater

Moudi Filter

Dichotomous Filter

UCLA

Trace Metals

Organics

|  | Aerosol Particulates (Teflon Filters)a | Wet Deposition (Rain Water) | Seasurface Microlayer (Seawater) |
| --- | --- | --- | --- |
| # Sampling Dates |  |  |  |
| October 20, 1998 | X |  |  |
| October 21, 1998 |  |  | X |
| January 11, 1999 |  |  | X |
| January 12, 1999 | X |  |  |
| January 13, 1999 | X |  |  |
| January 18, 1999 | X |  |  |
| January 24, 1999 | X |  |  |
| January 27, 1999 |  | X |  |
| January 30, 1999 | X |  |  |
| # Sampling Dates |  |  |  |
| February 1, 1999 |  | X |  |
| February 5, 1999 | X | X |  |
| February 10, 1999 |  | X |  |
| February 18, 1999 |  |  | X |
| February 20, 1999 | X |  |  |
| February 23, 1999 | X |  |  |
| February 25, 1999 | X |  |  |
| February 27, 1999 | X |  |  |
| March 1, 1999 | X |  |  |
| March 7, 1999 | X |  |  |
| March 10, 1999 |  | X |  |
| March 13, 1999 | X |  |  |
| March 16, 1999 |  | X |  |
| March 22, 1999 |  | X |  |
| March 26, 1999 |  | X |  |
| March 29, 1999 | X |  |  |
| July 5, 1999 | X |  |  |
| July 11, 1999 | X |  |  |
| July 12, 1999 |  |  | X |
| July 13, 1999 | X |  |  |
| July 17, 1999 | X |  |  |
| July 23, 1999 | X |  |  |
| July 29, 1999 | X |  |  |
| July 30, 1999 | X |  |  |
| August 4, 1999 | X |  |  |
| August 10, 1999 | X |  |  |
| August 16, 1999 | X |  |  |
| August 18, 1999 |  |  | X |
| August 20, 1999 | X |  |  |
| August 22, 1999 | X |  |  |
| August 28, 1999 | X |  |  |
| September 4, 1999 | X |  |  |
| September 9, 1999 | X |  |  |
| September 15, 1999 | X |  |  |
| September 27, 1999 | X |  |  |
| October 3, 1999 | X |  |  |
| October 9, 1999 | X |  |  |
| October 18, 1999 | X |  |  |
| October 21, 1999 |  |  | X |
| Sampling Dates |  |  |  |
| October 25, 1999 | X |  |  |
| October 27, 1999 | X |  |  |
| November 1, 1999 | X |  |  |
| November 2, 1999 | X |  |  |
| November 8, 1999 | X |  |  |
| November 9, 1999 |  |  | X |
| November 12, 1999 | X |  |  |
| November 20, 1999 | X |  |  |
| November 26, 1999 | X |  |  |
| December 6, 1999 | X |  | X |

SCBPP, 1994.  Quality Assurance Project Plan for the Southern California Bight Pilot Project.  Southern California Coastal Water Research Project, Westminster, CA.

Adams, N., 1937.  A rapid method for determining the lowering of tension of exposed water surfaces with some observations on the surface tension of the sea and inland waters.  Proc R. Soc. London (B) Biol. Sci. 122: 134-139.