Prepared by Brennan Ott

7/26/01

Loma Alta Creek

904.100

It is recommended that Loma Alta Creek not be listed.

# Watershed Characteristics

Loma Alta Creek is a coastal stream located in the Carlsbad Watershed.  It is 5.6 miles long.  Uses for the creek include contact and non-contact recreation, wildlife habitat, and a warm freshwater habitat as well.

Water Quality Objectives not Obtained

Loma Alta Creek was found to be in violation of inland surface water quality standards for total dissolved solids (TDS).

Evidence of Impairment

The water quality data used for Loma Alta Creek was taken on May 20, 1998.  The TDS concentration on this day was 2800 mg/l.  The standard set by the Basin Plan for inland surface waters for TDS is 500 mg/l.  The water in the Loma Alta Creek is in clear violation of this standard.

There is whole fish tissue data available for mosquitofish at this site.  The organic concentrations in the tissue are within allowable limits.  However, the heavy metal concentrations are suspect because there is no standards for heavy metal concentrations in fish tissue.  Since Heavy metal concentrations are not high in the water, it is assumed that they are not high in the fish too.

## Bio-assessments were conducted in September and November 1998 and May 1999.  Results from these bio-assessments indicate that the distribution (richness) of taxa between September and November is relatively the same, while May showed a decrease.  However, in May, the site displayed a greater abundance than the September and November samples.  November and September have similar levels of functional feeding groups whereas May exhibited a decrease in functional feeders from these two months.  At all sampling times, the site was dominated by a few disturbance tolerant insect and worm taxa and the functional feeding groups were composed mainly of collectors, filterers and grazers.  Shredders were rare.  The physical habitat quality at all three sampling periods was fair to good. The area sampled consisted of 5 riffles downstream of El Camino Road and 5 riffles downstream of College Blvd.

Extent of Impairment

Loma Alta Creek was sampled at station LAC-CB-T1.  Since there is only one water quality data point for this Creek, not much can be said about its impairment. There is no data given as to where exactly the sampling station is located along the Loma Alta Creek.

Potential Sources

There is not sufficient data to support any hard claims as to the source(s) of the pollution, but there is enough to warrant good assumptions.

Possible high TDS concentrations may due to urban runoff.

TMDL Priority

It is recommended that Loma Alta Creek not be placed on the TMDL list.

# Source References

Water quality standards were taken from the Water Quality Control Plan for the San Diego Basin.  Water quality data taken from Linda Pardy sheet 1.  Fish data taken from toxic substance monitoring program.  Bio-assessment data from San Diego Regional Water Quality Control Board: 1999 Biological Assessment Annual Report.