TOXIC SUBSTANCES MONITORING PROGRAM

1991 DATA REPORT

93-1WQ

1993

Prepared by

Del Rasmussen

Division of Water Quality

Field and Laboratory Operations Conducted by the

Water Pollution Control Laboratory

California Department of Fish and Game

STATE WATER RESOURCES CONTROL BOARD

CALIFORNIA ENVIRONMENTAL PROTECTION AGENCY

ACKNOWLEDGEMENTS

The State Water Resources Control Board wishes to acknowledge the contributions of the following individuals to this report:

Department of Fish and Game, Water Pollution Control Laboratory

Director:			Richard Hansen

Field Support:			Jack Linn

Karen Fothergill

Laboratory Support:		David Crane

Kathleen Regalado

Laurie Smith

State Water Resources Control Board

Graphic Support:		Dale Oliver

Toxic Substances Monitoring Program - 1991 Data Report

TABLE OF CONTENTS

Page

Acknowledgements	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	ii

List of Appendices	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	iv

List of Tables	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	v

List of Abbreviations	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	vii

1.  1991 PROGRAM

Introduction	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	1

Summary	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	1

2.  FIELD AND LABORATORY OPERATIONS	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	9

Substances Measured	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	9

Sample Size		.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	9

Wet and Lipid Weight Measurements	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	9

Station Numbers	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	10

3.  ADMINISTRATIVE AND COMPARATIVE CRITERIA	.	.	.	.	.	.	.	.	.	.	.	.	.	.	11

Maximum Tissue Residue Levels (MTRLs)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	11

FDA Action Levels and NAS Guidelines	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	12

Median International Standards (MIS)

for Trace Elements	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	12

Elevated Data Levels (EDL)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	12

4.	LITERATURE CITED	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	26

Toxic Substances Monitoring Program - 1991 Data Report

APPENDICES

Page

Appendix A -	Summary of 1991 Species Data	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	A-1

Appendix B -	Summary of 1991 Data:  Trace Elements in Sediment

(ppm, dry weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	B-1

Appendix C -	Summary of 1991 Data:  Organic Chemicals in Sediment

(ppb, dry weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	C-1

Appendix D -	Summary of 1991 Data:  Organic Chemicals in Freshwater Fish

Exceeding Maximum Tissue Residue Levels (MTRLs)

(ppb, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	D-1

Appendix E -	Summary of 1991 Data:  Trace Elements in Freshwater Fish

Exceeding Selected Criteria (ppm, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	E-1

Appendix F -	Summary of 1991 Data:  Organic Chemicals in Freshwater Fish

Exceeding Selected Criteria (ppb, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	F-1

Appendix G -	Summary of 1991 Data:  PCP and TCP in Freshwater Fish

(ppb, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	G-1

Appendix H -	Summary of 1991 Data:  Trace Elements in Marine Fish

Exceeding Selected Criteria (ppm, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	H-1

Appendix I -	Summary of 1991 Data:  Organic Chemicals in Marine Fish

Exceeding Selected Criteria (ppb, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	I-1

Appendix J -	Summary of 1991 Data:  Organic Chemicals in Freshwater Fish

Exceeding Lipid Weight EDL 85 and EDL 95

(ppb, lipid weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	J-1

Appendix K -	Summary of 1991 Data:  Organic Chemicals in Marine Fish

Exceeding Lipid Weight EDL 85 and EDL 95

(ppb, lipid weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	K-1

Appendix L -	Summary of 1991 Data:  Trace Elements in Fish, Crayfish,

and Turtles (ppm, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	L-1

Toxic Substances Monitoring Program - 1991 Data Report

APPENDICES (continued)

Page

Appendix M -	Summary of 1991 Data:  Organic Chemicals in Fish, Crayfish,

and Turtles (ppb, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	M-1

Appendix N -	Summary of 1991 Data:  Lipid Data in Fish

(ppb, lipid weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	N-1

Appendix O -	Station Sampling History	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	O-1

Appendix P -	1991 Station Descriptions	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	P-1

Appendix Q -	1991 Station Latitudes and Longitudes	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	Q-1

Appendix R -	Maps Showing 1991 Station Locations	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	R-1

Appendix S -	Field and Laboratory Operations	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	S-1

Appendix T -	Median International Standards	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	T-1

Appendix U -	Elevated Data Levels	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	U-1

LIST OF TABLES

Table  1.	1991 Toxic Substances Monitoring Program	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	3

Table  2.	1991 Freshwater Fish Code List	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	7

Table  3.	1991 Marine Fish Code List	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	8

Table  4.	1991 Non-Fish Species Code List	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	8

Table  5.	Maximum Tissue Residue Levels (MTRLs)

for Inland Surface Waters	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	14

Table  6.	NAS Guidelines and FDA Action Levels For

Toxic Chemicals in Fish (wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	15

Toxic Substances Monitoring Program - 1991 Data Report

LIST OF TABLES (continued)

Page

Table  7.	Median International Standards for Trace

Elements in Freshwater Fish and Marine

Shellfish (edible portion, ppm, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	16

Table  8.	TSMP EDL 85 and EDL 95 For Trace Elements

in Fish Livers.  Calculated Using 1978-1991

Data (ppm, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	17

Table  9.	TSMP EDL 85 and EDL 95 For Trace Elements

in Whole Fish.  Calculated Using 1978-1991

Data (ppm, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	18

Table 10.	TSMP EDL 85 and EDL 95 For Trace Elements

in Fish Filets.  Calculated Using 1978-1991

Data (ppm, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	19

Table 11.	TSMP EDL 85 and EDL 95 For Organic Chemicals in

Freshwater Fish Filets.  Calculated Using 1978-1991

Data (ppm, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	20

Table 12.	TSMP EDL 85 and EDL 95 for Organic Chemicals in

Marine Fish Filets.  Calculated Using 1978-1991

Data (ppb, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	21

Table 13.	TSMP EDL 85 and EDL 95 for Organic Chemicals in

Whole Freshwater Fish.  Calculated Using 1978-1991

Data (ppb, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	22

Table 14.	TSMP EDL 85 and EDL 95 for Organic Chemicals in

Whole Marine Fish.  Calculated Using 1978-1991

Data (ppb, wet weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	23

Table 15.	TSMP EDL 85 and EDL 95 for Organic Chemicals in

Filet and Whole Freshwater Fish.  Calculated

Using 1980-1991 Lipid Data (ppb, lipid weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	24

Table 16.	TSMP EDL 85 and EDL 95 for Organic Chemicals in

Filet and Whole Marine Fish.  Calculated

Using 1980-1991 Lipid Data (ppb, lipid weight)	.	.	.	.	.	.	.	.	.	.	.	.	.	.	.	25

Toxic Substances Monitoring Program - 1991 Data Report

LIST OF ABBREVIATIONS

DBP	Dichlorobenzophenone

DDD	Dichlorodiphenyldichloroethane

DDE	Dichlorodiphenyldichloroethylene

DDT	Dichlorodiphenyltrichloroethane

DDMS	Dichlorodiphenylmonochlorosaturatedethane

DDMU	Dichlorodiphenylmonochlorounsaturatedethane

DFG	California Department of Fish and Game

d/s	Downstream

EDL	Elevated Data Level(s)

FDA or (USFDA)	United States Food and Drug Administration

HCB	Hexachlorobenzene

HCH	Hexachlorocyclohexane

MIS	Median International Standard(s)

MTRL	Maximum Tissue Residue Level(s)

NAS	National Academy of Sciences

PAHs	Polynuclear Aromatic Hydrocarbons

PCBs	Polychlorinated Biphenyls

ppb	Parts Per Billion (ng/g)

ppm	Parts Per Million (ug/g)

Regional Water Boards	California Regional Water Quality Control Boards

State Water Board          	California State Water Resources Control Board

or SWRCB

TSMP	Toxic Substances Monitoring Program

USEPA	United States Environmental Protection Agency

u/s	Upstream

TOXIC SUBSTANCES MONITORING PROGRAM

1991

Introduction

The Toxic Substances Monitoring Program (TSMP) was initiated in 1976 by the California State Water Resources Control Board (State Water Board).  The TSMP was organized to provide a uniform statewide approach to the detection and evaluation of the occurrence of toxic substances in fresh, estuarine, and marine waters of the State through the analysis of fish and other aquatic life.  The TSMP primarily targets water bodies with known or suspected impaired water quality and is not intended to give an overall water quality assessment.  The California Department of Fish and Game (DFG) carries out the statewide TSMP for the State Water Board by collecting and analyzing samples.  The State Water Board provides funding for the program under an ongoing interagency agreement with the DFG.  Sampling stations are selected primarily by the nine Regional Water Quality Control Boards (Regional Water Boards) which are identified on the inside back cover.

The DFG reports annual sampling results to the State Water Board.  The information is transmitted to the Regional Water Boards and to other federal, State, and local agencies in the form of an annual TSMP report.  The report provides information on the statewide occurrence and levels of toxic substances and the data can be used by the Regional Water Boards and other agencies to identify waters impacted by toxic pollutants and to eventually abate such problems.  This report presents the results of the 1991 sampling and analysis program.  The raw data from the 1991 sampling program has already been released to the Regional Water Boards, other State agencies, and to the interested public.  This report is the formal report on the 1991 program.  The TSMP reports are routinely transmitted to the Office of Environmental Health Hazard Assessment of the California Environmental Protection Agency, which has responsibility for evaluating pollutant levels based on human health concerns and issuing fish consumption health advisories if indicated.

TSMP results are used by the State and Regional Water Boards in the statewide Water Quality Assessment/Clean Water Strategy.  Water bodies are classified from good to impaired water quality relative to each other and ranked according to this classification and resource value.  TSMP results are used to assist in the ranking process.  For example, water bodies that exceed human health criteria are considered more impaired than water bodies that only exceed environmental protection criteria.  TSMP results are also used in the regulatory activities of the Regional Water Boards and the Department of Pesticide Regulation.

Summary

Table 1 at the end of this section lists the 108 stations (94 water bodies) sampled in 1991 including 15 stations sampled as part of a special urban lake survey in Region 4.  Fifty-six of the stations were sampled for the first time.  A total of 159 samples were analyzed for trace elements (metals), pesticides, and PCBs (Appendix A).  Fish were collected at all but five of the stations.  Crayfish were collected at five stations in Regions 4, 5, and 8.  A spiny soft shelled turtle was collected at the Westmorland station on the New River in Region 7.  Along with aquatic organisms, sediment was analyzed from five stations in Regions 1 and 3 (Appendices B and C).  Arroyo chub, speckled dace, prickly sculpin, and Santa Ana sucker were collected and analyzed for the first time in the TSMP.  Species collected in 1991 are listed in Table 2 (freshwater fish), Table 3 (marine fish), and Table 4 (non-fish species) at the end of this section.

Sampling results were compared to criteria such as Maximum Tissue Residue Levels (MTRLs), U.S. Food and Drug Administration (FDA) action levels, Median International Standards (MIS), and the National Academy of Sciences (NAS) recommended guidelines for predator protection (see Administrative and Comparative Criteria section).  MTRLs are a new criterion developed from water quality objectives in the November 1992 California Inland Surface Waters Plan (SWRCB 1992).  MTRLs were exceeded at 13 water bodies from Regions 3, 4, 6, 7, and 9 (Appendix D).  The FDA action level for mercury was exceeded in white bass from two stations on Lake Nacimiento in Region 3 (Appendix E).  The FDA action level for chlordane was exceeded in a carp sample from Harbor Park Lake in Region 4 (Appendix F).  Mercury exceeded the MIS at seven water bodies in Regions 1, 4, and 5 (Appendix E).  Selenium exceeded the MIS at 2 stations; Suisun Bay in Region 2 and Lindero Lake in Region 4 (Appendix E).  The NAS guidelines for organic chemicals were exceeded at 17 stations (15 water bodies) in Regions 3, 4, 7, and 8 (Appendix F).  In addition to the regular chemical scan, four samples from Region 1 were analyzed for PCP and TCP and one sample of arroyo chub from the Valencia station on the Santa Clara River (Region 4) were analyzed for polynuclear aromatic hydrocarbons (PAHs).  PCP and TCP results can be found in Appendix G.  PAHs were not detected at the Valencia station.

The pesticide diazinon was found at the three highest concentrations found to date statewide.  Diazinon was detected at 260 and 180 ppb in whole samples of red shiner from San Diego Creek near Michelson Drive in Region 8.  Diazinon was also found at 180 ppb in another whole sample of red shiner from Peters Canyon Channel, a tributary to San Diego Creek.  The previous high diazinon value was 140 ppb in a 1990 whole sample of red shiner from Peters Canyon Channel.  The second highest levels of arsenic, lead, and dieldrin were also found in 1991.  A liver sample of grey smoothhound shark from Mugu Lagoon (Region 4) contained 20.9 ppm arsenic.  Grey smoothhound shark collected from this station in 1988 contained the highest level of arsenic at 29 ppm.  Lead was found at 1.2 ppm in a whole sample of fathead minnow from Belvedere Park Lake, an urban lake in Region 4.  This concentration is second only to a whole sample of California killifish collected in 1990 from Famosa Slough in Region 9.  Dieldrin was found at the second and third highest levels (1,100 and 1,000 ppb) in whole samples of threespine stickleback from Blanco Drain near the Salinas River in Region 3.  The highest concentration of dieldrin (1,700 ppb) occurred in a whole sample of threespine stickleback from Watsonville Slough (Region 3) in 1984.  DDT continues to be found in high concentrations with the third and fourth highest concentrations yet detected in the TSMP.  The two stickleback samples from Blanco Drain contained 13,019 and 12,299 ppb DDT.  The highest DDT concentration found statewide is 19,270 ppb in a 1989 goldfish filet sample from Rio de Santa Clara in Region 4.

Additional tabular summaries of chemistry data are provided in Appendices H through N.  Marine fish samples exceeding criteria are summarized in Appendix H (trace elements) and Appendix I (organic chemicals).  Lipid weight data exceedances are summarized in Appendix J (freshwater) and Appendix K (marine).  Summaries of all chemistry data are provided in Appendix L (trace elements), Appendix M (organic chemicals), and Appendix N (lipid weight data).  A complete TSMP sampling history is provided in Appendix O.  Station location descriptions and latitude and longitude information can be found in Appendices P and Q, respectively.  Regional maps showing 1991 station locations are in Appendix R.

TABLE 1

1991 Toxic Substances Monitoring Program

____________________________________________________________________________________________________________

Station                                           Sample                 Collection      Analyses

Name                                                                       Date

____________________________________________________________________________________________________________

Region 1

Beaughton Creek/d/s Highway 97 Bridge     Brown Trout               (BN)      09/19/91     Metals, Organics,

PCP, TCP

Carrville Pond*                           Sediment                  (SED)     09/18/91     Metals, Organics

Estero Americano*                         Pacific Staghorn Sculpin  (STG)     07/16/91     Cu, Hg

Estero de San Antonio*                    Prickly Sculpin           (PCP)     07/16/91     Metals

Klamath River/d/s Iron Gate Reservoir*    Speckled Dace             (DC)      09/19/91     Metals, Organics,

PCP, TCP

Lake Mendocino                            Largemouth Bass           (LMB)     09/09/91     Hg

Lake Pillsbury                            Largemouth Bass           (LMB)     09/10/91     Metals

Lake Sonoma                               Largemouth Bass           (LMB)     09/09/91     Hg

Lost River/Tule Lake                      Tui Chub                  (TC)      09/20/91     Metals, Organics

McDaniel Slough                           Threespine Stickleback    (STB)     09/16/91     Metals

Russian River/Duncans Mills*              Prickly Sculpin           (PCP)     07/17/91     Metals, Organics

Shasta River                              Speckled Dace             (DC)      09/19/91     Metals, PCP, TCP

Trinity River/d/s Burnt Ranch             Rainbow Trout             (RBT)     09/17/91     Organics

Trinity River/East Fork                   Rainbow Trout             (RBT)     09/18/91     Metals, Organics,

PCP, TCP

Trinity River/Willow Creek                Sculpin                   (SCP)     09/16/91     Metals, Organics

Region 2

Alameda Creek/Niles Canyon Road*          Sculpin                   (SCP)     07/09/91     Metals, Organics

Napa River/Napa                           Hitch                     (HCH)     07/11/91     Hg, Se, Organics

Stevens Creek                             Rainbow Trout             (RBT)     07/09/91     Metals, Organics

Suisun Bay                                White Sturgeon            (WST)     01/12/92     Metals, Organics

Walker Creek*                             Pacific Staghorn Sculpin  (STG)     07/16/91     Metals, Organics

Walnut Creek*                             Green Sunfish             (GSF)     07/10/91     Metals, Organics

Region 3

Aptos Creek*                              Prickly Sculpin           (PCP)     08/07/91     Metals, Organics

Blanco Drain/Salinas River                Threespine Stickleback    (STB)     09/04/91     Organics

Carmel Lagoon                             Threespine Stickleback    (STB)     08/01/91     Metals, Organics

Sediment                  (SED)     08/01/91     Metals, Organics

Corcoran Lagoon*                          Pacific Staghorn Sculpin  (STG)     08/07/91     Metals, Organics

El Estero*                                Bluegill                  (BG)      08/02/91     Metals, Organics

Lake Nacimiento/Dip Creek                 White Bass                (WHB)     07/31/91     Metals

Sediment                  (SED)     07/31/91     Metals

Lake Nacimiento/Las Tablas                White Bass                (WHB)     07/31/91     Metals

Sediment                  (SED)     07/31/91     Metals

Moran Lake*                               Threespine Stickleback    (STB)     08/06/91     Metals, Organics

Neary's Lake*                             Sacramento Sucker         (SSKR)    08/06/91     Hg, Se, Organics

Roberts Lake*                             Sacramento Perch          (SP)      08/01/91     Metals, Organics

Santa Maria River/Mouth*                  Sediment                  (SED)     07/31/91     Organics

Schwann Lake*                             Largemouth Bass           (LMB)     08/07/91     Metals, Organics

____________________________________________________________________________________________________________

*  Stations sampled for the first time.

TABLE 1 (continued)

1991 Toxic Substances Monitoring Program

____________________________________________________________________________________________________________

Station                                           Sample                 Collection      Analyses

Name                                                                       Date

____________________________________________________________________________________________________________

Region 4

Alamitos Bay*                             California Corbina        (CCB)     06/12/91     Metals, Organics

Arroyo Conejo                             Green Sunfish             (GSF)     06/19/91     Metals, Organics

Arroyo Simi*                              Fathead Minnow            (FHM)     06/19/91     Metals, Organics

Belvedere Park Lake*#                     Fathead Minnow            (FHM)     04/18/91     Metals, Organics

Calabasas Lake*#                          Largemouth Bass           (LMB)     04/20/91     Metals, Organics

Calleguas Creek                           Goldfish                  (GF)      06/18/91     Organics

Conejo Creek*                             Mosquitofish              (GAM)     06/19/91     Metals, Organics

Echo Park Lake #                          Largemouth Bass           (LMB)     04/19/91     Metals, Organics

Eleanor Lake*#                            Black Bullhead            (BLB)     04/22/91     As

Goldfish                  (GF)      04/22/91     As, Hg, Se,

Organics

El Dorado Park Lake*#                     Largemouth Bass           (LMB)     04/21/91     Metals, Organics

Harbor Park Lake #                        Carp                      (CP)      06/15/91     Organics

Hollenbeck Park Lake*#                    Red Swamp Crayfish        (PROI)    04/18/91     Metals, Organics

Legg Lake #                               Largemouth Bass           (LMB)     04/17/91     Metals, Organics

Lincoln Park Lake #                       Largemouth Bass           (LMB)     04/18/91     Metals, Organics

Lindero Lake*#                            Largemouth Bass           (LMB)     04/22/91     Metals, Organics

Los Angeles River/Sepulveda Basin*        Goldfish                  (GF)      05/15/91     Hg, Se, Organics

Malibu Creek                              Bluegill                  (BG)      06/18/91     Metals, Organics

Malibou Lake*#                            Largemouth Bass           (LMB)     04/23/91     Metals, Organics

Mugu Lagoon                               Gray Smoothhound Shark    (GSS)     06/17/91     Metals

Peck Road Lake #                          Largemouth Bass           (LMB)     04/17/91     Metals, Organics

Puddingstone Reservoir #                  Largemouth Bass           (LMB)     06/11/91     Metals, Organics

Rio de Santa Clara/Oxnard Drain           Mosquitofish              (GAM)     06/17/91     Organics

San Gabriel River                         Mozambique Tilapia        (TLM)     06/16/91     Metals

Santa Clara River/Santa Paula             Santa Ana Sucker          (SAKR)    06/20/91     Hg, Se

Santa Clara River/Valencia*               Arroyo Chub               (AC)      06/11/91     Organics, PAHs

Sherwood Lake*#                           Largemouth Bass           (LMB)     04/22/91     Metals, Organics

Ventura River                             Carp                      (CP)      06/20/91     Metals, Organics

Westlake Lake*#                           Largemouth Bass           (LMB)     04/23/91     Metals, Organics

Region 5

American River/d/s Folsom Reservoir*      Largemouth Bass           (LMB)     10/03/91     Hg

American River/d/s Watt Avenue Bridge     Sacramento Sucker         (SSKR)    10/16/91     Hg

Feather River/d/s Highway 99 Bridge       Channel Catfish           (CCF)     10/09/91     Hg

Feather River/d/s Oroville Reservoir*     Sucker                    (SKR)     11/05/91     Hg

Franks Tract*                             Crayfish                  (PACI)    10/21/91     Metals

Sacramento River/Hood                     White Catfish             (WCF)     10/11/91     Hg

White Catfish             (WCF)     11/21/91     Hg

Crayfish                  (PACI)    10/21/91     Metals

Sacramento River/u/s I-5 Overcrossing*    Crayfish                  (PACI)    10/11/91     Metals

San Joaquin River/Vernalis                Channel Catfish           (CCF)     10/30/91     Hg

Yuba River/N.F./d/s Bullards Bar Res*     Smallmouth Bass           (SMB)     10/15/91     Hg

____________________________________________________________________________________________________________

*  Stations sampled for the first time.

#  Urban Lake Survey

TABLE 1 (continued)

1991 Toxic Substances Monitoring Program

____________________________________________________________________________________________________________

Station                                           Sample                 Collection      Analyses

Name                                                                       Date

____________________________________________________________________________________________________________

Region 6

Bishop Creek Canal/d/s Bishop*            Brown Trout               (BN)      07/23/91     Metals, Organics

Carson River/W.F./d/s Paynesville*        Brown Trout               (BN)      09/27/91     Metals, Organics

Donner Lake*                              Kokanee                   (KOK)     10/23/91     Metals, Organics

Gull Lake*                                Sacramento Perch          (SP)      07/24/91     Metals

Haiwee Reservoir*                         Smallmouth Bass           (SMB)     07/24/91     Metals

Little Rock Creek Reservoir*              Black Bullhead            (BLB)     08/14/91     Metals

Sabrina Lake*                             Brown Trout               (BN)      07/23/91     Metals

Silver Creek*                             Sucker                    (SKR)     09/27/91     Metals

Silverwood Lake*                          Largemouth Bass           (LMB)     08/14/91     Metals

Squaw Creek                               Brown Trout               (BN)      10/22/91     Metals, Organics

Region 7

Colorado River/Needles                    Carp                      (CP)      08/20/91     Hg, Se, Organics

Colorado River/u/s Imperial Dam           Largemouth Bass           (LMB)     08/18/91     Metals, Organics

Fig Drain                                 Sailfin Molly             (MOL)     08/17/91     Se, Organics

Mayflower Drain*                          Mosquitofish              (GAM)     08/16/91     Se, Organics

New River/International Boundary          Carp                      (CP)      12/18/91     Se, Hg, Organics

New River/Westmorland                     Channel Catfish           (CCF)     08/15/91     Se, Organics

Spiny Soft Shelled Turtle (SST)     08/15/91     Se, Organics

Orange Drain*                             Mosquitofish              (GAM)     08/17/91     Se, Organics

Palo Verde Outfall Drain                  Carp                      (CP)      08/19/91     Se, Organics

Peach Drain*                              Mosquitofish              (GAM)     08/17/91     Se

Reservation Main Drain                    Redbelly Tilapia          (TLZ)     08/18/91     Se, Organics

Rose Drain                                Mosquitofish              (GAM)     08/17/91     Se, Organics

Salton Sea/North                          Orangemouth Corvina       (ORC)     05/30/91     Metals, Organics

Orangemouth Corvina       (ORC)     06/18/91     Se, Organics

Sargo                     (SAR)     05/30/91     Metals, Organics

Salton Sea/South                          Orangemouth Corvina       (ORC)     05/15/91     Metals, Organics

Region 8

El Modena Channel/u/s Walnut Avenue Brg*  Red Shiner                (PRS)     05/16/91     Metals, Organics

Huntington Harbour/Anaheim Bay            White Croaker             (WCK)     11/21/91     Metals, Organics

Newport Bay                               Spotted Sand Bass         (SSB)     06/20/91     Metals, Organics

Peters Canyon Channel                     Red Shiner                (PRS)     05/16/91     Metals, Organics

San Diego Creek/Barranca Parkway          Red Shiner                (PRS)     05/16/91     Metals, Organics

San Diego Creek/Michelson Drive           Red Shiner                (PRS)     05/16/91     Metals, Organics

Santa Ana River/Prado Dam                 Largemouth Bass           (LMB)     05/14/91     Metals, Organics

Red Swamp Crayfish        (PROI)    05/14/91     Metals, Organics

Santa Ana River/USGS Gage                 Santa Ana Sucker          (SAKR)    05/14/91     Hg, Se

____________________________________________________________________________________________________________

*  Stations sampled for the first time.

TABLE 1 (continued)

1991 Toxic Substances Monitoring Program

____________________________________________________________________________________________________________

Station                                           Sample                 Collection      Analyses

Name                                                                       Date

____________________________________________________________________________________________________________

Region 9

Chollas Creek/Main Street*                Longjaw Mudsucker         (LJM)     06/14/91     Metals, Organics

Keys Creek*                               Green Sunfish             (GSF)     06/13/91     Metals, Organics

Rainbow Creek*                            Black Bullhead            (BLB)     06/14/91     Metals, Organics

San Luis Rey River/Foussat Road*          Largemouth Bass           (LMB)     06/13/91     Metals, Organics

San Luis Rey River/Highway 15*            Largemouth Bass           (LMB)     06/13/91     Metals, Organics

San Luis Rey River/Highway 76*            Largemouth Bass           (LMB)     06/13/91     Metals, Organics

Santa Margarita River/Willow Glen Road*   Green Sunfish             (GSF)     06/14/91     Metals, Organics

____________________________________________________________________________________________________________

*  Stations sampled for the first time.

TABLE 2

Toxic Substances Monitoring Program

1991 Freshwater Fish Code List*

Species	Common	Species	Family

Code	 Name	 Name	Name

AC	Arroyo Chub**	Gila orcutti	Cyprinidae

BG	Bluegill	Lepomis macrochirus	Centrarchidae

BLB	Black Bullhead	Ameiurus melas	Ictaluridae

BN	Brown Trout	Salmo trutta	Salmonidae

CCF	Channel Catfish	Ictalurus punctatus	Ictaluridae

CP	Carp	Cyprinus carpio	Cyprinidae

DC	Speckled Dace**	Rhinichthys osculus	Cyprinidae

FHM	Fathead Minnow	Pimephales promelas	Cyprinidae

GAM	Mosquitofish	Gambusia affinis	Poeciliidae

GF	Goldfish	Carassius auratus	Cyprinidae

GSF	Green Sunfish	Lepomis cyanellus	Centrarchidae

HCH	Hitch	Lavinia exilicauda	Cyprinidae

KOK	Kokanee	Oncorhynchus nerka	Salmonidae

LJM	Longjaw Mudsucker	Gillichthys mirabilis	Gobiidae

LMB	Largemouth Bass	Micropterus salmoides	Centrarchidae

MOL	Sailfin Molly	Poecilia latipinna	Poeciliidae

PCP	Prickly Sculpin**	Cottus asper	Cottidae

PRS	Red Shiner	Cyprinella lutrensis	Cyprinidae

RBT	Rainbow Trout	Oncorhynchus mykiss	Salmonidae

SAKR	Santa Ana Sucker**	Catostomus santaanae	Catostomidae

SCP	Sculpin	Cottus sp.	Cottidae

SKR	Sucker	Catostomus sp.	Catostomidae

SMB	Smallmouth Bass	Micropterus dolomieu	Centrarchidae

SP	Sacramento Perch	Archoplites interruptus	Centrarchidae

SSKR	Sacramento Sucker	Catostomus occidentalis	Catostomidae

STB	Threespine Stickleback	Gasterosteus aculeatus	Gasterosteidae

STG	Pacific Staghorn Sculpin	Leptocottus armatus	Cottidae

TC	Tui Chub	Gila bicolor	Cyprinidae

TLM	Mozambique Tilapia	Tilapia mossambica	Cichlidae

TLZ	Redbelly Tilapia	Tilapia zillii	Cichlidae

WCF	White Catfish	Ameiurus catus	Ictaluridae

WHB	White Bass	Morone chrysops	Percichthyidae

WST	White Sturgeon	Acipenser transmontanus	Acipenseridae

*Common and scientific names were obtained from Robins et al. (1991).  List includes fish that inhabit both fresh and estuarine waters.

**Collected for the first time.

TABLE 3

Toxic Substances Monitoring Program

1991 Marine Fish Code List*

Species	Common	Species	Family

Code	 Name	 Name	Name

CCB	California Corbina	Menticirrhus undulatus	Sciaenidae

GSS	Gray Smoothhound Shark	Mustelus californicus	Carcharhinidae

ORC	Orangemouth Corvina	Cynoscion xanthulus	Sciaenidae

SAR	Sargo	Anisotremus davidsoni	Haemulidae

SSB	Spotted Sand Bass	Paralabrax maculatofasciatus	Serranidae

WCK	White Croaker	Genyonemus lineatus	Sciaenidae

*Common and scientific names were obtained from Robins et al. (1991).

TABLE 4

Toxic Substances Monitoring Program

1991 Non-Fish Species Code List

Species	Common	Species	Family

Code	 Name	 Name	Name

PACI	Crayfish	Pacifastacus leniusculus	Astacidae

PROI	Red Swamp Crayfish	Procambarus clarki	Astacidae

SST	Spiny Soft Shelled Turtle	Trionyx spiniferus	Trionychidae

FIELD AND LABORATORY OPERATIONS

The presence of many toxic substances in fresh waters is determined by analyzing tissues from fish and other aquatic organisms.  Concentrations of these substances in water are often too low or transitory to be reliably detected through the more traditional methods of analysis of water samples.  Also, many toxic substances are not water soluble, but can be found associated with sediment or organic matter.  Fish and other aquatic organisms are sampled because they bioaccumulate and bioconcentrate toxic substances to levels which may be many hundreds of times the levels actually in the water.  This concentration factor facilitates detection of toxic pollutants.  The following is a general overall discussion of field and laboratory procedures.  A detailed discussion is provided in Appendix S.

Substances Measured

A total of 10 trace elements (metals) and 45 pesticides and PCBs (organic chemicals) are analyzed in the TSMP on a regular basis.  Additional substances, such as polynuclear aromatic hydrocarbons (PAHs), pentachlorophenol (PCP), and tetrachlorophenol (TCP), are looked for on a request basis only.  Not every sample is analyzed for all metals or organic chemicals.  Each sample at each station is handled individually.  The requesting agency, usually the Regional Water Boards, will specify the type of analysis for each sample.  All metals, except mercury and selenium, are routinely analyzed in liver tissue.  Mercury, selenium, and all organic chemicals are analyzed in muscle tissue (filet).  When only very small fish are available, metal or organic chemical analysis is performed on a whole-body composite of larger than usual numbers of individual fish.

Sample Size

Composite samples, using six fish of each species, are collected whenever possible.  The number and size uniformity of the fish in each composite depends upon their availability.  Replicate composites are collected and analyzed to measure the variability of toxicant concentrations in single species composites collected at the same time and place.  Collection of the same species from all stations is desirable to minimize possible variation in the data due to differences in pollutant uptake between species.  However, this is not possible over the entire State due to the variety of habitat sampled and limited collection time available in the program.  All reasonable efforts  are made to maintain both station-to-station and year-to-year uniformity in collections.  In general, predator fish are desired from all stations.  Forage fish are desired as supplemental samples at stations where pollution problems are known to exist, or as substitute samples where predatory fish are not available.

Wet and Lipid Weight Measurements

Tissue concentrations of metals and organic chemicals are measured on a wet weight basis.  Metal data are presented in parts per million (ppm), while organic chemical data are presented in parts per billion (ppb).  In addition to wet weight measures, organic chemicals are also expressed on a lipid weight basis.  Lipid weight measurements offer several advantages.  Because chlorinated hydrocarbons are much more soluble in lipids (fat tissues) than in water, they partition into lipid-rich tissues of aquatic organisms (Stout and Beezhold 1981).  Animals with higher proportions of lipid in their tissue usually have had higher concentrations of chlorinated hydrocarbon pollutants (Phillips 1980).  Factors such as season, water temperature, health of the organism, stress on the organism, and type of species can affect the lipid levels of samples collected for analysis and can, therefore, cause variability in results.  Use of lipid weight measurements may reduce this source of variability, although disadvantages have also been noted (Phillips 1980).  As a result, lipid weight values may represent a more realistic measure of environmental availability of chlorinated hydrocarbons than wet weight values.  Wet weight measures, however, remain the preferred measure for most readers because all standards for human health and for predator protection are based on wet weight measures.  Also, wet weight measures better reflect the exposure of predators or humans to the actual concentration in freshly caught fish.

Station Numbers

Each TSMP station is identified by a unique seven digit number derived from the State Water Board's hydrologic basin planning maps.  The first digit of a station number signifies one of the nine Regional Water Boards.  The second and third digits represent a hydrologic area, while the fourth and fifth digits identify a hydrologic subarea.  The sixth and seventh digits represent the distance in miles above the downstream hydrologic boundary.  For example, station 519.21.01 is in Region 5, hydrologic area 19, subarea 21, and is one mile upstream from the hydrologic unit boundary.  Not all mileage indicators are accurate, however.  In certain instances, it was necessary to assign an arbitrary mileage indicator.  For example, the arbitrary designation is used when two or more stations within the same hydrologic subarea are located within the same number of miles of the hydrologic boundary, resulting in the same station number.  In this case, one or more of the stations is arbitrarily assigned a mileage designator from 90 to 99.

ADMINISTRATIVE AND COMPARATIVE CRITERIA

In this report, as in previous TSMP reports, the term "criteria" is used to refer to the criteria against which a particular metal or organic chemical is being compared.  As more than one criterion may apply to any one metal or organic compound, a hierarchy was established.  The intent of the hierarchy is to compare data against the more important criterion.  In general, FDA action levels and the "Median International Standards" (MIS), human health-related criteria, are considered more important or critical.  Following human health criteria are NAS guidelines, predator protection criteria.  Last in the hierarchy are "elevated data levels" (EDL).  New to this report are Maximum Tissue Residue Levels (MTRLs), also human health related criteria.  All appropriate 1991 data will be compared separately to MTRLs in addition to following the usual hierarchy.  The criteria mentioned above are discussed below.

In interpreting the TSMP data by any of the criteria provided, the reader is cautioned that there is no simple relationship between concentrations of toxic substances observed in tissue samples and actual concentrations in water.  Different aquatic organisms tend to bioaccumulate a given toxic substance in water to different levels;  however, the differences usually do not prevent a general interpretation of the data.  The reader is cautioned that the limited number of samples obtained and analyzed at each station in a single year is generally too small to provide a statistically sound basis for making absolute statements on toxic substance concentrations.  The values reported herein should be accepted as indicators of relative levels of toxic pollution in water, not as absolute values.  In this sense, trends over time and ranking values of a toxic substance in a particular species provide only an indication of areas where fish are evidently accumulating concentrations which are above "normal".

Maximum Tissue Residue Levels (MTRLs)

MTRLs were developed from human health water quality objectives in Table 2 of the State Water Board's November 1992 California Inland Surface Waters Plan (SWRCB 1992).  The objectives represent concentrations in water that protect against drinking water and consuming fish or shellfish that contain substances at levels which could result in significant human health problems.  MTRLs are used as alert levels or guidelines indicating water bodies with potential human health concerns and are an assessment tool and not compliance or enforcement criteria.  MTRLs are compared only to filet or edible tissue samples and should not be compared to whole body or liver samples.  Table 5 at the end of this section lists MTRLs for those substances monitored in the TSMP.  The MTRLs for 10 of the 15 carcinogens listed in Table 5 are below the current tissue detection limit for those substances.  Only MTRLs for arsenic, DDT, HCB, gamma-HCH, and PCP are above the detection limits.

The MTRLs were calculated by multiplying the human health water quality objectives by the bioconcentration factor (BCF) for each substance as recommended in the USEPA Draft Assessment and Control of Bioconcentratable Contaminants in Surface Waters (USEPA 1991).  BCFs were taken from the USEPA 1980 Ambient Water Quality Criteria Documents for each substance.  MTRLs were not calculated for objectives that are based on maximum contaminant levels (MCLs) or taste and odor criteria.

FDA Action Levels and NAS Guidelines

The U.S. Food and Drug Administration (FDA) has established maximum concentration levels for some toxic substances in human foods (USFDA 1985).  The levels are based on specific assumptions of the quantities of food consumed by humans and upon the frequency of their consumption.  The FDA limits are intended to protect humans from the chronic effects of toxic substances consumed in foodstuffs. The National Academy of Sciences (NAS) has established recommended maximum concentrations of toxic substance concentrations in freshwater fish tissue (NAS 1973).  They were established not only to protect the organisms containing the toxic compounds, but also to protect the species that consume these contaminated organisms.  The specific action levels and guidelines used in this report are shown in Table 6 at the end of this section.

Median International Standards (MIS) for Trace Elements

The Food and Agriculture Organization of the United Nations has published a survey of health protection criteria used by member nations (Nauen 1983).  These criteria vary somewhat in the tissues to be analyzed or the level of protection desired, but may be compared qualitatively.  Table 7 at the end of this section summarizes these standards as an indication of what other countries have determined to be unsafe levels of trace elements.  Though the standards do not apply within the United States, they provide an indication of what other nations consider to be an elevated concentration of trace elements in fish tissues.  Even so, the reader is reminded that most TSMP metal analyses are done in liver, rather than in edible portions.  To date, only mercury and selenium are routinely measured in edible portions in the TSMP.  Measurements in liver should not be compared to Median International Standards.  A description of how the Median International Standards were compiled is provided in Appendix T.

Elevated Data Levels

The "elevated data level" (EDL) was introduced in 1983 as an internal comparative measure which ranks a given concentration of a particular substance with previous data from the TSMP.  The EDL is calculated by ranking all of the results for a given chemical from the highest concentration measured down to and including those records where the chemical was not detected.  From this, a cumulative distribution is constructed and percentile rankings are calculated.  For example, the 50th percentile corresponds to the median or "middle" value rather than to the mean.  With a large number of records, the median can be approximately compared to the mean.

Starting in 1990, EDL calculations were modified to reflect the growing number of marine species analyzed in the TSMP.  In the past, EDL calculations for wet weight measures were grouped by similar tissue types, such as filet or whole-body samples.  In 1990, the EDL calculations were further split into freshwater and marine fish types.  Now when any sample is compared to an EDL, it is compared to the EDL calculated from the same fish and tissue types (i.e. freshwater fish filets are compared only to other freshwater fish filets, etc.).  The substance most affected by the change in the EDL calculations was arsenic.  The EDL criteria for arsenic in freshwater fish livers and whole samples were lowered by approximately half from 1978-1989 calculations.  A separate copper EDL is calculated for salmonid liver tissue because trout are known to accumulate copper to higher levels than other species.  White bass also seem to accumulate copper and other trace elements to higher levels.  Starting in 1988, white bass are not included in the EDL calculations.  White bass are found only in a few locations in California and further sampling of this species will be avoided whenever possible.  In calculating the EDLs for lipid weight measures of organic chemicals, all tissue types are combined because lipid weight measures in different tissue types tend to be far more similar than wet weight measures (Phillips 1980).  However, like wet weight measures, EDL lipid weight calculations were also split into freshwater and marine fish types.  The 1978-1991 EDLs and the number of data points used to calculate each EDL are provided in Tables 8 through 16 at the end of this section.

The 85th percentile (EDL 85) was chosen as an indication that a chemical is elevated from the median.  The 85th percentile corresponds to measures used by the U.S. Fish and Wildlife Service in their National Contaminant Biomonitoring Program and would represent approximately one and one-half standard deviations from the mean, if the data were normally distributed.  The 95th percentile (EDL 95) was chosen to indicate values that are highly elevated above the median.  The 95th percentile would represent two standard deviations from the mean, if the data were normally distributed.  When used along with other information, these measures provide a useful guideline to determine if a chemical has been found in unusually high concentrations.  A more detailed description of EDL rankings is provided in Appendix U.  The reader is  again cautioned that EDLs are not directly related to potentially adverse human or animal health effects; they are only a way to compare findings in a particular area with the larger data base of findings from all over the state.

TABLE 5

Toxic Substances Monitoring Program

Maximum Tissue Residue Levels (MTRLs) for Carcinogens in Inland Surface Waters

Water Quality Objective a	BCF b	MTRL c

Substance	(μg/l)	(l/kg)	(μg/kg, ppb)

aldrin	0.00013	d	0.05

arsenic	5.0 e	44	200.0 (0.2 ppm)

chlordane (total)	0.00008	14100	1.1

DDT (total)	0.00059	53600	32.0

dieldrin	0.00014	4670	0.65

heptachlor	0.00016	11200	1.8

heptachlor epoxide	0.00007	11200	0.8

hexachlorobenzene (HCB)	0.00066	8690	6.0

hexachlorocyclohexane (HCH), alpha	0.0039	130	0.5

hexachlorocyclohexane (HCH), beta	0.014	130	1.8

hexachlorocyclohexane (HCH), gamma	0.019	130	2.5

PAHs (total)	0.0028	30	0.08

PCBs (total)	0.00007	31200	2.2

pentachlorophenol (PCP)	0.28	11	3.1

toxaphene	0.00067	13100	8.8

Maximum Tissue Residue Levels (MTRLs) for Non-carcinogens in Inland Surface Waters

Water Quality Objective a	BCF b	MTRL c

Substance	(mg/l)	(l/kg)	(mg/kg, ppm)

cadmium	0.01	64	0.64

endosulfan (total)	0.0009	270	0.25 (250 ppb)

endrin	0.0008	3970	3.0 (3,000 ppb)

mercury	0.000012	f	1.0

nickel	0.6	47	28.0

a.From Table 2, Human Health Water Quality Objectives, "California Inland Surface Waters Plan" (SWRCB 1992).  MTRLs were not developed for objectives based on maximum contaminant levels (MCLs) or taste and odor criteria.

b.Bioconcentration Factors taken from the USEPA 1980 Ambient Water Quality Criteria Documents for each substance.

c.MTRLs were calculated by multiplying the Water Quality Objective by the BCF, except for aldrin, arsenic, and mercury.

d.Aldrin MTRL is derived from a combination of aldrin and dieldrin risk factors and BCFs as recommended in the USEPA 1980 "Ambient Water Quality Criteria for Aldrin/Dieldrin" (USEPA 1980).

e.Arsenic MTRL was calculated from the formula NSRL  (WI/BCF) + FC = MTRL.  [NSRL (California's No Significant Risk Level for arsenic) = 10 μg/d, WI (Water Intake) = 2 l/d, FC (daily fish consumption) = 0.0065 kg/d].

f.The MTRL for mercury is the FDA action level.  The water quality objective for mercury in the Inland Surface Waters Plan is based on the FDA action level as recommended in the USEPA 1985 "Ambient Water Quality Criteria for Mercury" (USEPA 1985).

TABLE 6

NAS Guidelines and FDA Action Levels for Toxic Chemicals in Fish

(wet weight)

NASa	FDAb

Recommended Guideline	Action Level for

for Freshwater Fish	Freshwater and Marine Fish

(Whole Fish)	(Edible Portion)

Chemical	ug/g (ppm)	ng/g (ppb)	ug/g (ppm)	ng/g (ppb)

Mercury	0.5	500	1.0d	1,000

DDT (total)	1.0	1,000	5.0	5,000

PCB (total)	0.5	500	2.0e	2,000

aldrin	0.1c	100	0.3	300

dieldrin	0.1c	100	0.3	300

endrin	0.1c	100	0.3	300

heptachlor	0.1c	100	0.3	300

heptachlor epoxide 	0.1c	100	0.3	300

chlordane (total)	0.1c	100	0.3	300

lindane	0.1 	100	 -	-

hexachlorocyclo-

hexane (total)	0.1c	100	 -	-

endosulfan (total)	0.1c	100	 -	-

toxaphene	0.1c	100	5.0	5,000

aNational Academy of Sciences-National Academy of Engineering.  1973.  Water Quality Criteria, 1972 (Blue Book).  U.S. Environmental Protection Agency, Ecological Research Series.

bU. S. Food and Drug Administration.  1984.  Shellfish Sanitation Interpretation:  Action Levels for Chemical and Poisonous Substances, June 21, 1984.  U.S.F.D.A., Shellfish Sanitation Branch, Washington, D.C.

cIndividually or in combination.  Chemicals in this group under NAS Guidelines are referred to as Chemical Group A in this report.

dAs methyl mercury.

eA tolerance, rather than an action level, has been established for PCBs (21CFR 109, published May 29, 1984).  An action level is revoked when a regulation establishes a tolerance for the same substance and use.

TABLE 7

Median International Standards for Trace Elements

in Freshwater Fish and Marine Shellfish a

(edible portion, ppm, wet weight)

Number of Countries

Element	Fish	Shellfish	Range	with Standards

Antimony	1.0	1.0	1.0 to 1.5	3

Arsenic	1.5	1.4	0.1 to 5.0	11

Cadmium	0.3	1.0	0.05 to 2.0	10

Chromium	1.0	1.0	 1.0	1

Copper	20.0	20.0	10 to 100	8

Fluoride	150.0	-	 150.0	1

Fluorine	17.5	-	10 to 25	2

Lead	2.0	2.0	0.5 to 10.0	19

Mercury	0.5	0.5	0.1 to 1.0	28

Selenium	2.0	0.3	0.3 to 2.0	3

Tin	150.0	190.0	50 to 250	8

Zinc	45.0	70.0	40 to 100	6

a	Based on:Nauen, C. C., Compilation of Legal Limits for Hazardous Substances in Fish and Fishery Products, Food and Agriculture Organization of the United Nations, 1983.

TABLE 8.  TSMP EDL 85 AND EDL 95 for Trace Elements in Fish Livers

Calculated Using 1978 - 1991 Data.

(ppm, wet weight)

Freshwater Fish

Element	Fish	EDL 85	EDL 95	Number of

Type*	Samples

Arsenic	ALL	0.22	0.74	498

Cadmium	ALL	0.36	1.00	512

Chromium	ALL	0.03	0.08	495

Copper	SALMO	170.00	230.00	113

Copper	NON	13.00	32.00	400

Lead	ALL	0.10	0.20	493

Nickel	ALL	<0.10	0.31	496

Selenium**	ALL	3.44	4.98	104

Silver	ALL	0.25	0.68	496

Zinc	ALL	28.00	38.00	494

Marine Fish

Element	EDL 85	EDL 95	Number of

Samples

Arsenic	7.10	18.94	28

Cadmium	1.15	3.38	28

Chromium	<0.02	0.03	26

Copper	17.20	25.00	28

Lead	<0.10	0.16	28

Nickel	<0.10	0.16	28

Selenium**	ID	ID	3

Silver	0.18	0.69	28

Zinc	40.00	44.60	28

*	Non = Includes all non-salmonid species.  Salmo = Salmonids.

All=All fish species.

**	Selenium analysis in liver was discontinued starting in 1985.

<	=	EDL lies below the indicated detection limit.

ID	=	Insufficient number of data points to calculate the EDL.

TABLE 9.  TSMP EDL 85 AND EDL 95 for Trace Elements in Whole Fish

Calculated Using 1978 - 1991 Data.

(ppm, wet weight)

Freshwater Fish

Element	EDL 85	EDL 95	Number of

Samples

Arsenic	0.48	0.85	93

Cadmium	0.08	0.15	93

Chromium	0.19	0.34	93

Copper	3.41	4.14	93

Lead	0.20	0.77	93

Mercury	0.08	0.15	94

Nickel	0.20	0.46	94

Selenium	1.50	2.06	114

Silver	0.03	0.04	93

Zinc	40.00	44.35	93

Marine Fish

Element	EDL 85	EDL 95	Number of

Samples

Arsenic	ID	ID	2

Cadmium	ID	ID	2

Chromium	ID	ID	2

Copper	ID	ID	4

Lead	ID	ID	2

Mercury	ID	ID	4

Nickel	ID	ID	2

Selenium	ID	ID	2

Silver	ID	ID	2

Zinc	ID	ID	2

ID = Insufficient number of data points to calculate the EDL.

TABLE 10.  TSMP EDL 85 AND EDL 95 for Trace Elements in Fish Filets

Calculated Using 1978 - 1991 Data.

(ppm, wet weight)

Freshwater Fish

Element	EDL 85	EDL 95	Number of

Samples

Arsenic	0.10	0.20	30

Cadmium	<0.01	0.01	16

Chromium	<0.02	<0.02	16

Copper	0.70	0.81	16

Lead	<0.10	<0.10	16

Mercury	0.83	1.80	1070

Nickel	<0.10	<0.10	16

Selenium	1.10	2.00	384

Silver	<0.02	<0.02	16

Zinc	23.40	32.80	16

Marine Fish

Element	EDL 85	EDL 95	Number of

Samples

Arsenic	ID	ID	2

Cadmium	ID	ID	1

Chromium	ID	ID	1

Copper	ID	ID	1

Lead	ID	ID	1

Mercury	0.17	0.68	28

Nickel	ID	ID	1

Selenium	3.64	3.96	44

Silver	ID	ID	1

Zinc	ID	ID	1

<	=	EDL lies below the indicated detection limit.

ID	=	Insufficient number of data points to calculate the EDL.

TABLE 11

TSMP EDL 85 AND EDL 95 For Organic Chemicals in Freshwater Fish Filets

Calculated Using 1978 - 1991 Data.

(ppb, wet weight)

Chemical                                  EDL 85        EDL 95        Number of

Samples

Aldrin                                         <5.0          <5.0            682

Chemical Group A                              439.0        1263.6            702

Chlordene, Alpha                               <5.0          <5.0            578

Chlordene, Gamma                               <5.0          <5.0            578

Cis-chlordane                                  14.0          38.0            687

Cis-nonachlor                                   6.1          18.2            578

Oxychlordane                                   <5.0          <5.0            686

Trans-chlordane                                 8.5          21.0            687

Trans-nonachlor                                19.0          45.0            658

Total Chlordane                                43.0         121.6            687

Chlorpyrifos                                  <10.0          19.0            682

Dacthal                                        12.0         316.0            688

DDD, o,p'                                      12.0          36.0            687

DDD, p,p'                                      95.8         260.0            687

DDE, o,p'                                      <5.0          25.7            687

DDE, p,p'                                     636.0        2000.0            688

DDMS, p,p'                                    <30.0         <30.0            687

DDMU, p,p'                                    <15.0          43.3            687

DDT, o,p'                                     <10.0          18.0            685

DDT, p,p'                                      31.0         120.0            687

Total DDT                                     823.6        2534.2            688

Diazinon                                      <50.0         <50.0            663

Dichlorobenzophenone, p,p'                      ID            ID               6

Dicofol (Kelthane)                           <100.0        <100.0            682

Dieldrin                                       11.0          37.5            669

Endosulfan I                                   <5.0          25.0            688

Endosulfan II                                 <70.0          94.2            228

Endosulfan sulfate                            <85.0         126.0            228

Total Endosulfan                                5.5          63.2            688

Endrin                                        <15.0         <15.0            685

HCH, Alpha                                     <2.0          <2.0            685

HCH, Beta                                     <10.0         <10.0            685

HCH, Delta                                     <5.0          <5.0            685

HCH, Gamma (Lindane)                           <2.0           3.6            685

Total HCH                                        *            5.1            685

Heptachlor                                     <5.0          <5.0            682

Heptachlor Epoxide                             <5.0          <5.0            682

Hexachlorobenzene                              <2.0           6.3            685

Methoxychlor                                  <15.0         <15.0            680

Oxadiazon                                      <5.0          11.8            148

Parathion, Ethyl                              <10.0         <10.0            663

Parathion, Methyl                             <10.0         <10.0            663

PCB-1248                                      <50.0         <50.0            717

PCB-1254                                      <50.0         161.5            717

PCB-1260                                       66.0         191.5            717

Total PCB                                     137.8         372.6            717

Pentachlorophenol                               2.8           5.0             20

2,3,5,6-tetrachlorophenol                      <2.0           1.7             20

Toxaphene                                     270.0        1100.0            700

< = EDL lies below the indicated detection limit.

ID = Insufficient number of data points to calculate the EDL.

* = EDL lies below the detection limit.

TABLE 12

TSMP EDL 85 AND EDL 95 For Organic Chemicals in Marine Fish Filets

Calculated Using 1978 - 1991 Data.

(ppb, wet weight)

Chemical                                  EDL 85        EDL 95        Number of

Samples

Aldrin                                         <5.0          <5.0             33

Chemical Group A                                7.6          36.8             33

Chlordene, Alpha                               <5.0          <5.0             32

Chlordene, Gamma                               <5.0          <5.0             32

Cis-chlordane                                  <5.0          <5.0             33

Cis-nonachlor                                  <5.0           8.5             32

Oxychlordane                                   <5.0          <5.0             33

Trans-chlordane                                <5.0          <5.0             33

Trans-nonachlor                                <5.0          13.1             33

Total Chlordane                                  *           23.4             33

Chlorpyrifos                                  <10.0         <10.0             33

Dacthal                                        21.4          30.7             33

DDD, o,p'                                     <10.0         <10.0             33

DDD, p,p'                                      16.0          20.1             33

DDE, o,p'                                     <10.0         <10.0             33

DDE, p,p'                                     222.0         288.0             33

DDMS, p,p'                                    <30.0         <30.0             33

DDMU, p,p'                                    <15.0         <15.0             33

DDT, o,p'                                     <10.0         <10.0             33

DDT, p,p'                                     <10.0         <10.0             33

Total DDT                                     236.8         308.9             33

Diazinon                                      <50.0         <50.0             33

Dichlorobenzophenone, p,p'                      ID            ID               0

Dicofol (Kelthane)                           <100.0        <100.0             33

Dieldrin                                       <5.0          <5.0             33

Endosulfan I                                   <5.0          <5.0             33

Endosulfan II                                 <70.0         <70.0             20

Endosulfan sulfate                            <85.0         <85.0             20

Total Endosulfan                                 *             *              33

Endrin                                        <15.0         <15.0             33

HCH, Alpha                                     <2.0          <2.0             33

HCH, Beta                                     <10.0         <10.0             33

HCH, Delta                                     <5.0          <5.0             33

HCH, Gamma (Lindane)                           <2.0          <2.0             33

Total HCH                                        *             *              33

Heptachlor                                     <5.0          <5.0             33

Heptachlor Epoxide                             <5.0          <5.0             33

Hexachlorobenzene                              <2.0          <2.0             33

Methoxychlor                                  <15.0         <15.0             33

Oxadiazon                                      <5.0          <5.0             16

Parathion, Ethyl                              <10.0         <10.0             33

Parathion, Methyl                             <10.0         <10.0             33

PCB-1248                                      <50.0         <50.0             33

PCB-1254                                      <50.0         141.0             33

PCB-1260                                       59.8         127.0             33

Total PCB                                      96.2         266.6             33

Pentachlorophenol                               ID            ID               0

2,3,5,6-tetrachlorophenol                       ID            ID               0

Toxaphene                                    <100.0        <100.0             33

< = EDL lies below the indicated detection limit.

ID = Insufficient number of data points to calculate the EDL.

* = EDL lies below the detection limit.

TABLE 13

TSMP EDL 85 AND EDL 95 For Organic Chemicals in Whole Freshwater Fish

Calculated Using 1978 - 1991 Data.

(ppb, wet weight)

Chemical                                  EDL 85        EDL 95        Number of

Samples

Aldrin                                         <5.0          <5.0            119

Chemical Group A                             1716.1        3751.0            119

Chlordene, Alpha                               <5.0           5.5            119

Chlordene, Gamma                                6.5          10.1            119

Cis-chlordane                                  48.0          70.2            119

Cis-nonachlor                                  20.1          30.2            119

Oxychlordane                                   14.0          20.0            119

Trans-chlordane                                28.1          40.0            119

Trans-nonachlor                                52.3          82.2            119

Total Chlordane                               162.6         246.6            119

Chlorpyrifos                                   34.3          82.9            119

Dacthal                                       120.0         483.5            119

DDD, o,p'                                      59.8         240.5            119

DDD, p,p'                                     360.0        1200.0            119

DDE, o,p'                                      28.1          61.2            119

DDE, p,p'                                    2145.0        5365.0            119

DDMS, p,p'                                    <30.0         <30.0            119

DDMU, p,p'                                     80.2         182.5            119

DDT, o,p'                                      56.2         190.5            119

DDT, p,p'                                     201.5         729.0            119

Total DDT                                    3568.6        7348.7            119

Diazinon                                      <50.0          84.8            118

Dichlorobenzophenone, p,p'                      ID            ID               0

Dicofol (Kelthane)                           <100.0        <100.0            119

Dieldrin                                      126.0         584.0            118

Endosulfan I                                   13.0          56.1            119

Endosulfan II                                 <10.0          84.0             80

Endosulfan sulfate                            180.0         240.0             80

Total Endosulfan                              127.4         328.7            119

Endrin                                        <15.0          58.0            119

HCH, Alpha                                     <2.0           2.0            119

HCH, Beta                                     <10.0         <10.0            119

HCH, Delta                                     <5.0          <5.0            119

HCH, Gamma (Lindane)                            3.1           8.0            119

Total HCH                                       4.1           9.6            119

Heptachlor                                     <5.0          <5.0            119

Heptachlor Epoxide                              5.4          13.1            119

Hexachlorobenzene                               4.8          10.1            119

Methoxychlor                                  <15.0         <15.0            119

Oxadiazon                                     331.0        1800.0             53

Parathion, Ethyl                              <10.0         <10.0            118

Parathion, Methyl                             <10.0         <10.0            118

PCB-1248                                      <50.0         <50.0            120

PCB-1254                                      180.0         520.0            120

PCB-1260                                      100.0         210.0            120

Total PCB                                     283.0         840.0            120

Pentachlorophenol                               ID            ID               5

2,3,5,6-tetrachlorophenol                       ID            ID               5

Toxaphene                                    1200.0        2465.0            119

< = EDL lies below the indicated detection limit.

ID = Insufficient number of data points to calculate the EDL.

TABLE 14

TSMP EDL 85 AND EDL 95 For Organic Chemicals in Whole Marine Fish

Calculated Using 1978 - 1991 Data.

(ppb, wet weight)

Chemical                                  EDL 85        EDL 95        Number of

Samples

Aldrin                                          ID            ID               1

Chemical Group A                                ID            ID               1

Chlordene, Alpha                                ID            ID               1

Chlordene, Gamma                                ID            ID               1

Cis-chlordane                                   ID            ID               1

Cis-nonachlor                                   ID            ID               1

Oxychlordane                                    ID            ID               1

Trans-chlordane                                 ID            ID               1

Trans-nonachlor                                 ID            ID               1

Total Chlordane                                 ID            ID               1

Chlorpyrifos                                    ID            ID               1

Dacthal                                         ID            ID               1

DDD, o,p'                                       ID            ID               1

DDD, p,p'                                       ID            ID               1

DDE, o,p'                                       ID            ID               1

DDE, p,p'                                       ID            ID               1

DDMS, p,p'                                      ID            ID               1

DDMU, p,p'                                      ID            ID               1

DDT, o,p'                                       ID            ID               1

DDT, p,p'                                       ID            ID               1

Total DDT                                       ID            ID               1

Diazinon                                        ID            ID               1

Dichlorobenzophenone, p,p'                      ID            ID               0

Dicofol (Kelthane)                              ID            ID               1

Dieldrin                                        ID            ID               1

Endosulfan I                                    ID            ID               1

Endosulfan II                                   ID            ID               1

Endosulfan sulfate                              ID            ID               1

Total Endosulfan                                ID            ID               1

Endrin                                          ID            ID               1

HCH, Alpha                                      ID            ID               1

HCH, Beta                                       ID            ID               1

HCH, Delta                                      ID            ID               1

HCH, Gamma (Lindane)                            ID            ID               1

Total HCH                                       ID            ID               1

Heptachlor                                      ID            ID               1

Heptachlor Epoxide                              ID            ID               1

Hexachlorobenzene                               ID            ID               1

Methoxychlor                                    ID            ID               1

Oxadiazon                                       ID            ID               0

Parathion, Ethyl                                ID            ID               1

Parathion, Methyl                               ID            ID               1

PCB-1248                                        ID            ID               1

PCB-1254                                        ID            ID               1

PCB-1260                                        ID            ID               1

Total PCB                                       ID            ID               1

Pentachlorophenol                               ID            ID               0

2,3,5,6-tetrachlorophenol                       ID            ID               0

Toxaphene                                       ID            ID               1

ID = Insufficient number of data points to calculate the EDL.

TABLE 15

TSMP EDL 85 AND EDL 95 for Organic Chemicals in

Filet and Whole Freshwater Fish

Calculated Using 1980 - 1991 Lipid Data

(ppb, lipid weight)

Chemical                                  EDL 85         EDL 95       Number of

Samples

Aldrin                                          *              *             721

Chemical Group A                            26256.8       101486.2           742

Chlordene, Alpha                                *              *             694

Chlordene, Gamma                                *            106.1           694

Cis-chlordane                                 928.7         2456.6           727

Cis-nonachlor                                 326.8         1105.1           694

Oxychlordane                                    *            237.8           727

Trans-chlordane                               452.0         1214.5           727

Trans-nonachlor                              1453.3         4041.0           727

Total Chlordane                              3481.4         8295.0           727

Chlorpyrifos                                    *           1855.7           721

Dacthal                                      1530.2        19636.9           728

DDD, o,p'                                     748.5         3015.3           727

DDD, p,p'                                    6620.7        22038.7           727

DDE, o,p'                                     182.3         1082.5           727

DDE, p,p'                                   51040.1       140072.5           728

DDMS, p,p'                                      *              *             727

DDMU, p,p'                                    490.8         2623.9           727

DDT, o,p'                                       *           1895.5           726

DDT, p,p'                                     785.6         5313.2           726

Total DDT                                   63165.6       185564.2           728

Diazinon                                        *              *             701

Dichlorobenzophenone, p,p'                     ID             ID               6

Dicofol (Kelthane)                              *              *             721

Dieldrin                                      823.2         3675.1           708

Endosulfan I                                  189.0         1830.5           728

Endosulfan II                                   *           2680.9           307

Endosulfan sulfate                              *          10571.4           307

Total Endosulfan                              350.4         6496.8           728

Endrin                                          *              *             725

HCH, Alpha                                      *              *             725

HCH, Beta                                       *              *             725

HCH, Delta                                      *              *             725

HCH, Gamma (Lindane)                            *            289.4           725

Total HCH                                      41.7          601.8           725

Heptachlor                                      *              *             721

Heptachlor Epoxide                              *              *             721

Hexachlorobenzene                              51.5          442.4           725

Methoxychlor                                    *              *             720

Oxadiazon                                       *           3984.0           202

Parathion, Ethyl                                *              *             702

Parathion, Methyl                               *              *             702

PCB-1248                                        *              *             756

PCB-1254                                     2258.9        10877.2           756

PCB-1260                                     2143.9        13999.5           756

Total PCB                                    7949.5        37288.3           756

Pentachlorophenol                             253.6          376.4            19

2,3,5,6-tetrachlorophenol                       *            234.3            19

Toxaphene                                   14557.0        76808.6           740

ID = Insufficient number of data points to calculate the EDL.

* = EDL lies below the detection limit.

TABLE 16

TSMP EDL 85 AND EDL 95 for Organic Chemicals in

Filet and Whole Marine Fish

Calculated Using 1980 - 1991 Lipid Data

(ppb, lipid weight)

Chemical                                  EDL 85         EDL 95       Number of

Samples

Aldrin                                          *              *              34

Chemical Group A                              537.4         1611.9            34

Chlordene, Alpha                                *              *              33

Chlordene, Gamma                                *              *              33

Cis-chlordane                                   *            284.4            34

Cis-nonachlor                                   *            317.3            33

Oxychlordane                                    *              *              34

Trans-chlordane                                 *            182.8            34

Trans-nonachlor                                 *            390.4            34

Total Chlordane                                 *           1155.2            34

Chlorpyrifos                                    *              *              34

Dacthal                                      1761.7         2433.5            34

DDD, o,p'                                       *              *              34

DDD, p,p'                                     936.1         1841.7            34

DDE, o,p'                                       *              *              34

DDE, p,p'                                   28582.9        36517.9            34

DDMS, p,p'                                      *              *              34

DDMU, p,p'                                      *            499.3            34

DDT, o,p'                                       *              *              34

DDT, p,p'                                       *              *              34

Total DDT                                   30431.6        36517.9            34

Diazinon                                        *              *              34

Dichlorobenzophenone, p,p'                     ID             ID               0

Dicofol (Kelthane)                              *              *              34

Dieldrin                                        *              *              34

Endosulfan I                                    *              *              34

Endosulfan II                                   *              *              21

Endosulfan sulfate                              *              *              21

Total Endosulfan                                *              *              34

Endrin                                          *              *              34

HCH, Alpha                                      *              *              34

HCH, Beta                                       *              *              34

HCH, Delta                                      *              *              34

HCH, Gamma (Lindane)                            *              *              34

Total HCH                                       *              *              34

Heptachlor                                      *              *              34

Heptachlor Epoxide                              *              *              34

Hexachlorobenzene                               *              *              34

Methoxychlor                                    *              *              34

Oxadiazon                                       *              *              16

Parathion, Ethyl                                *              *              34

Parathion, Methyl                               *              *              34

PCB-1248                                        *              *              34

PCB-1254                                        *           5274.3            34

PCB-1260                                     3753.0        20741.1            34

Total PCB                                    8519.5        26018.3            34

Pentachlorophenol                              ID             ID               0

2,3,5,6-tetrachlorophenol                      ID             ID               0

Toxaphene                                       *              *              34

ID = Insufficient number of data points to calculate the EDL.

* = EDL lies below the detection limit.

LITERATURE CITED

Adrian, W.J.  1971.  A New Wet Digestion Method for Biological Material Utilizing Pressure.  Atom. Absorption Newsletter 10 (4):96.

(DFG) Department of Fish and Game.  1990.  Laboratory Quality Assurance Program Plan

(March 14, 1990).  Environmental Services Division, California Department of Fish and Game.  Sacramento, California.

Evans, S.J., M.S. Johnson, and R.T. Leah.  1986.  Determination of Mercury in Fish Tissue, a Rapid,

Automated Technique for Routine Analysis.  School of Biology, University of Liverpool, U. K.  In Varian Pamphlet No. AA-60, May 1986.  Walnut Creek, Calif.

(NAS) National Academy of Sciences-National Academy of Engineering.  1973.  Water Quality Criteria

1972 (Blue Book).  EPA Ecological Research Series.  EPA-R3-73-033.  U.S. Environmental Protection Agency, Washington, D.C.

Nauen, C.E.  1983.  Compilation of Legal Limits for Hazardous Substances in Fish and Fishery Products,

Circular No. 764.  FIRI/C764, Food and Agricultural Organization of the United Nations.

Phillips, D.J.H.  1980.  Quantitative Aquatic Biological Indicators.  Applied Science Publishers Ltd, Ripple

Road, Barking, Essex, England.  Pages 38-90.

Robins, C.R., R.M. Bailey, C.E. Bond, J.R. Brooker, E.A. Lachner, R.N. Lea, and W.B. Scott.  1991.

Common and Scientific Names of Fishes from the United States and Canada.  American Fisheries Society Special Publication 20, Bethesda, Maryland.

Stout, V.F. and F.L. Beezhold.  1981.  Chlorinated hydrocarbon levels in fishes and shellfishes of the

northeastern Pacific Ocean including the Hawaiian Islands.  Mar. Fish. Rev. 43:1:1-12.

(SWRCB) State Water Resources Control Board.  1992.  Water Quality Control Plan for Inland Surface

Waters of California.  November 1992.  State Water Resources Control Board, California Environmental Protection Agency, Sacramento, California.

(USEPA) U.S. Environmental Protection Agency.  1991.  "Draft" Assessment and Control of Bioconcentratable Contaminants in Surface Waters.  March 1991.  Office of Water, U.S. EPA, Washington, D.C.

(USEPA) U.S. Environmental Protection Agency.  1985.  Ambient Water Quality Criteria for Mercury.

EPA 440/5-84-026.  U.S. EPA, Office of Water Regulations and Standards, Washington, D.C.

LITERATURE CITED (continued)

(USEPA) U.S. Environmental Protection Agency.  1980.  Ambient Water Quality Criteria for

Aldrin/Dieldrin.  EPA 440/5-80-019.  U.S. EPA, Office of Water Regulations and Standards, Washington, D.C.

(USFDA) U.S. Food and Drug Administration, Bureau of Foods.  1985.  Action Levels for Poisonous or

Deleterious Substances in Human Food and Animal Feed.  USFDA, Washington, D.C.

Uthe, J.F., H.C. Freeman, J.R. Johnson, and P. Michalik.  1974.  Comparison of wet ashing and dry ashing for the determination of arsenic in marine organisms, using methylated arsenals for standards. Assoc.  Official Analytical Chem., J., 57(6):1363-1365.

APPENDIX S

Field and Laboratory Operations

FIELD AND LABORATORY OPERATIONS

Sample Collection

Sample collections were obtained using a Smith-Root Model VII and Model XIA Portable Electrofishers; a Smith-Root SR-16E electrofishing boat; variable mesh, woven, and monofilament gill nets; baited hoop nets measuring three feet in diameter with one inch square mesh; or beach seines of varying lengths, widths, and material. Collected fish were kept in clean stainless steel buckets until they could be double-wrapped in extra-heavy duty aluminum foil (dull side inward), labeled, and packed in dry ice where they were frozen.

Laboratory Analysis

A detailed description of procedures and techniques discussed below can be found in the Department of Fish and Game's (DFG) Laboratory Quality Assurance Program Plan (DFG 1990).  The following is a summary of the 1991 Quality Assurance/Quality Control (QA\QC) results provided by the DFG's Water Pollution Control Laboratory.  Copies of the Laboratory Quality Assurance Program Plan and QA\QC results are available upon request.

Trace Elements Analytical Techniques in Tissues

A Varian Model Spectra 30 atomic absorption spectrophotometer and a Varian Model VGA-76 Hydride Generator were used for techniques employing conventional (flame) atomic absorption spectrophotometry (copper and zinc), hydride generation (arsenic and selenium), and cold vapor technique for mercury (Adrian 1971; Uthe et al. 1974; and Evans et al. 1986).  A Perkin-Elmer Model 3030 Zeeman atomic absorption spectrophotometer equipped with a HGA-600 graphite furnace and an AS-60 autosampler was used for techniques requiring a graphite furnace (cadmium, chromium, nickel, lead, and silver).  All analytical values were corrected using procedural blanks.  Trace element analytical and digestion techniques along with their detection limits are presented in Table S-1.  All digestion techniques, except for mercury, are the same as those used since 1988.

Samples were weighed into pre-cleaned 200mm x 25mm glass tubes which had been checked for trace element contamination.  Digestion of the sample was accomplished by adding concentrated nitric acid and heating the tube in an aluminum block to reflux the acid.  The acid was allowed to reflux until the evolution of NOx (brown fumes) were no longer apparent (about 2 hours).  The block temperature was increased to reduce the volume in the tube by evaporation.  When the volume in the tube reached about 0.5 ml the tube was removed and allowed to cool.  The digestate was diluted to 40.0 ml with Type II water.  The digestate was mixed on a vortex mixer and transferred to a clean polyethylene bottle.

In addition to routine trace element analyses, 10 percent of the samples were analyzed in duplicate to determine precision.  The results of duplicate laboratory sample analyses are presented in Table S-2.  To protect sample integrity, all materials contacting samples during laboratory operations were analyzed for trace element content.  To ensure accuracy, reference materials from the National Institute of Standards and Technology (NIST) and the National Research Council of Canada were analyzed (Table S-3).

Synthetic Organic Compounds Analytical Techniques in Tissues

A 10 gram sample of the flesh-water (1:1) paste was spiked with nonachlorobiphenyl (PCB congener No. 206) and extracted twice with acetonitrile by shaking for two minutes. The sample extracts were combined, filtered, and partitioned with petroleum ether.  An aliquot of the petroleum ether extract was eluted through a FlorisilR column.  The FlorisilR columns were eluted with petroleum ether (Fraction 1), six percent ethyl ether (Fraction 2), and 15 percent ethyl ether (Fraction 3).  Fractions 2 and 3 were spiked with nonachlorobiphenyl and all of the fractions were concentrated to an appropriate volume in a ZymarkR Turbovap concentrator prior to analysis by gas chromatography. The nonachlorobiphenyl was used as an internal standard to determine relative retention times and gas chromatograph operation.  A mixture of synthetic standards was eluted through the FlorisilR column to determine the recovery and separation characteristics of the column.  The distribution of synthetic organic compounds in the three fractions is listed in Table S-4.  The detection levels for synthetic organics in flesh are presented in Table S-5.

At stations where the TSMP had previously detected endosulfan, samples were analyzed for endosulfan I, endosulfan II and endosulfan sulfate.  This required an additional elution through FlorisilR with 50 percent ethyl ether in petroleum ether (Fraction 4, Table S-4).  All other stations were initially analyzed for endosulfan I only.  This fraction was also spiked with nonachlorobiphenyl prior to the concentration step.  Due to the high lipid content of the fraction all of the 50 percent extracts were diluted with iso-octane by a factor of ten prior to analysis by gas chromatography.

As part of quality control, 10 percent of the samples were duplicated in the laboratory (Table S-6).  All materials and solutions contacting the sample after initial extraction were analyzed for organic contamination.  To preclude errors due to contamination, a vertical solvent blank was passed through each set of glassware and analyzed before introducing a new sample.

Instrument and Analytical Conditions for Chlorinated Hydrocarbons

1991

Chlorinated hydrocarbons were determined with a Varian Model 3500 gas chromatograph equipped with a model 8035 autosampler, temperature programmable on-column injector, and dual Ni63 electron capture detectors.  A 30 meter J&W DB1 fused silica capillary column is connected to the temperature programmable injector, the column effluent is split using a press-fit "Y" connector to a 30 meter J&W DB5 and a 30 meter J&W DB17 column.  The DB5 and DB17 columns are connected to the electron capture detectors.  All three columns have a 0.25 mm ID and a 25 um liquid phase thickness.  Helium was used as the carrier gas at a linear velocity of 35 cm/sec and nitrogen was used as the detector makeup gas at a flow of 25 ml/min.  Chromatographic data was acquired and processed with a Perkin-Elmer Model 7700 professional computer using Chromatographics 3 software.

All samples were analyzed using a single injection for each extract under the following conditions:

Injector temperature program:

Initial temperature - 50 oC

Program rate - 300 oC/min

Final temperature - 280oC

Final temperature hold time - 57 min

Column temperature program:

Initial temperature - 50oC

Program rate 1 - 15oC/min to 210oC

Program rate 2 - 2oC/min to 280oC

Final temperature hold time - 0 min

Detector temperature:	330oC

Analytical Techniques for Polynuclear Aromatic Hydrocarbon Compounds (PAHs) in Flesh

Sample extraction procedures for PAHs were similar to those used for chlorinated hydrocarbons and are described below.  A 10 gram sample of the flesh water (1:1) paste was homogenized with acetonitrile in an all-glass blender with stainless steel blades and filtered.

Sample extracts were analyzed using a Varian Saturn II Ion Trap GC-MS. One  microliter of sample extract was injected into a J&W Scientific DB-5, 30 meter x 0.25 mm I.D. fused silica capillary column having a 0.25 um film thickness.  The GC oven temperature was initially held at 70oC for two minutes.  The temperature ramp was 15oC per minute until the oven reached 150oC.  The second temperature ramp was 2oC per minute to a final temperature of 280oC and held for 5 minutes.  Initial injector temperature was 70o and was programmed to 280o at 300o/min immediately after injection.  The GC carrier gas was helium at a linear velocity of 37 cm/sec.   Detection limits of the PAHs are reported in Table S-7.

Procedure for Lipid Determination

As synthetic organic concentrations in organisms may vary with lipid content, it is customary to provide lipid data when reporting tissue concentrations.  A thoroughly homogenized sample weighing approximately 5 g (wet weight) is macerated and dried with anhydrous granular Na2SO4.  The dried sample is transferred to a blender with 150 ml of petroleum ether and blended for two minutes at high speed.  The liquid is vacuum-filtered into a 250 ml filter flask through a 10 cm Buchner funnel containing Whatman #1 filter paper.  The sample is blended once more with an additional 150 ml of petroleum ether and filtered.  The filtrate is concentrated to approximately 25 ml with heat (steam bath) and nitrogen steam.  The remaining filtrate is then quantitatively transferred into a 50 ml pre-weighed planchet.  The petroleum ether is evaporated, the planchet containing the residue is reweighed, and the percent lipid is calculated.

TABLE S-1

Toxic Substances Monitoring Program

1991 Digestion Techniques and Detection Limits in Fish Tissue

Detection Limits

Element	Digestion Techniques	Instrumental Analysis	(ug/g wet weight)

Arsenic	Dry Ash w/Mg(NO3)2.6H2O	NaBH4 Reduction A.A.	0.05

Mercury	HNO3 reflux	Cold Vapor A.A.	0.02

Copper	HNO3 reflux	Flame A.A.	0.02

Zinc	HN03 reflux	Flame A.A.	0.05

Cadmium	HNO3 reflux	Graphite Furnace	0.01

(Ammonium phosphate/magnesium nitrate)

Chromium	HNO3 reflux	Graphite Furnace                 	0.02

Lead	HNO3 reflux	Graphite Furnace	0.10

(Ammonium phosphate/magnesium nitrate)

Nickel	HNO3 reflux	Graphite Furnace	0.10

Selenium	Dry Ash w/Mg(NO3)2.6H2O	NaBH4 Reduction A.A.	0.05

Silver	HNO3 reflux	Graphite Furnace	0.02

TABLE S-2

Toxic Substances Monitoring Program

Results of Duplicate Sample Analysis:  1991 Trace Metal Quality Control

(ug/g wet weight)

Station       Station                     Species  Tissue   Arsenic  Cadmium   Chromium   Copper   Lead   Mercury   Nickel  Selenium   Silver  Zinc

Number         Name                        Code*

515.40.31  Feather River/D/S Oroville Res.   SKR      F                                                     0.31                                      515.40.31  Feather River/D/S Oroville Res.   SKR      F                                                     0.30

515.40.31  Feather River/D/S Oroville Res.   SKR      F                                                     0.34

515.40.31  Feather River/D/S Oroville Res.   SKR      F                                                     0.35

519.22.04  Sacramento R/U/S I-5 Overcross    PACI     F                                                     0.09

519.22.04  Sacramento R/U/S I-5 Overcross    PACI     F                                                     0.08

510.00.30  Sacramento River/Hood             PACI     F        0.20                                                            0.14

510.00.30  Sacramento River/Hood             PACI     F        0.18                                                            0.14

510.00.30  Sacramento River/Hood             PACI     F                0.05       0.02      11.    <0.1              <0.1                0.02   14.

510.00.30  Sacramento River/Hood             PACI     F                0.05       0.02      11.    <0.1              <0.1                0.02   14.

510.00.30  Sacramento River/Hood             WCF      F                                                     0.54

510.00.30  Sacramento River/Hood             WCF      F                                                     0.54

723.10.02  New River/Westmorland             CCF      F                                                                        1.0

723.10.02  New River/Westmorland             CCF      F                                                                        1.0

723.10.58  New River/International Boundary  CP       F                                                     0.47

723.10.58  New River/International Boundary  CP       F                                                     0.46

728.00.90  Salton Sea/South                  ORC      L        2.0    <0.01      <0.02      18.    <0.1              <0.1                0.08   34.

728.00.90  Salton Sea/South                  ORC      L        2.1    <0.01      <0.02      17.    <0.1              <0.1                0.08   34.

309.82.08  Lake Nacimiento/Las Tablas        WHB      F                                                     1.3

309.82.08  Lake Nacimiento/Las Tablas        WHB      F                                                     1.3

111.63.14  Lake Pillsbury                    LMB      L        0.07

111.63.14  Lake Pillsbury                    LMB      L        0.07

402.10.02  Ventura River                     CP       W       <0.05    0.05       0.07       0.82  <0.1              <0.1      0.54     <0.02   43.

402.10.02  Ventura River                     CP       W       <0.05    0.06       0.08       0.83  <0.1              <0.1      0.55     <0.02   41.

405.21.16  Los Angeles R/Sepulveda Basin     GF       F                                                     0.08               0.51

405.21.16  Los Angeles R/Sepulveda Basin     GF       F                                                     0.08               0.51

* Tables 2, 3, and 4 list code names for species.             L = Liver.     F = Filet.     W = Whole Body.

TABLE S-2 (continued)

Toxic Substances Monitoring Program

Results of Duplicate Sample Analysis:  1991 Trace Metal Quality Control

(ug/g wet weight)

Station       Station                     Species  Tissue   Arsenic  Cadmium   Chromium   Copper   Lead   Mercury   Nickel  Selenium   Silver  Zinc

Number         Name                        Code*

903.12.06  Keys Creek                        GSF      F                                                                         0.60

903.12.06  Keys Creek                        GSF      F                                                                         0.61

903.17.07  San Luis Rey River/HWY 15         LMB      F                                                     0.08

903.17.07  San Luis Rey River/HWY 15         LMB      F                                                     0.07

204.30.11  Alameda Creek/Niles Canyon Road   SCP      W                0.01       0.12       1.9   <0.1               0.2               <0.02   17.    204.30.11  Alameda Creek/Niles Canyon Road   SCP      W                0.01       0.10       2.6   <0.1               0.1               <0.02   17.

728.00.03  Reservation Main Drain            TLZ      F                                                                         0.20

728.00.03  Reservation Main Drain            TLZ      F                                                                         0.21

405.52.01  Puddingstone Reservoir            LMB      L        0.67    0.15      <0.02       6.5   <0.1              <0.1               <0.02     19.

405.52.01  Puddingstone Reservoir            LMB      L        0.66    0.15      <0.02       6.9   <0.1              <0.1               <0.02     19.

105.50.35  Beaughton Creek/D/S HWY 97 Bridge BN       F                                                    <0.02

105.50.35  Beaughton Creek/D/S HWY 97 Bridge BN       F                                                    <0.02

207.10.90  Suisun Bay                        WST      L        1.5     1.         0.05      51.    <0.1               1.2                0.80     63.

207.10.90  Suisun Bay                        WST      L        1.5     1.         0.05      52.    <0.1               1.2                0.77     63.

403.11.91  Mugu Lagoon                       GSS      F                                                                         0.39

403.11.91  Mugu Lagoon                       GSS      F                                                                         0.39

403.11.91  Mugu Lagoon                       GSS      L       21.      3.5        0.02       3.4   <0.1              <0.1                0.67     14.

403.11.91  Mugu Lagoon                       GSS      L       21.      3.5        0.02       3.3   <0.1              <0.1                0.67     15.

114.32.00  Lake Mendocino                    LMB      F                                                     0.32

114.32.00  Lake Mendocino                    LMB      F                                                     0.33

801.11.96  Peters Canyon Channel             PRS      W        0.10                                                             1.2

801.11.96  Peters Canyon Channel             PRS      W        0.10                                                             1.3

110.00.90  McDaniel Slough                   STB      W        0.36   <0.01       0.22       3.6   <0.1               0.4      0.22      0.03     37.

110.00.90  McDaniel Slough                   STB      W        0.36   <0.01       0.16       3.5   <0.1               0.4      0.22      0.03     38.

205.50.94  Stevens Creek                     RBT      F                                                                        0.88

205.50.94  Stevens Creek                     RBT      F                                                                        0.88

635.20.04  Donner Lake                       KOK      L                0.04      <0.02     120.    <0.1              <0.1                0.49     41.

635.20.04  Donner Lake                       KOK      L                0.04      <0.02     130.    <0.1              <0.1                0.52     41.

* Tables 2, 3, and 4 list code names for species.             L = Liver.     F = Filet.     W = Whole Body.

TABLE S-2 (continued)

Toxic Substances Monitoring Program

Results of Duplicate Sample Analysis:  1991 Trace Metal Quality Control

(ug/g wet weight)

Station       Station                     Species  Tissue   Arsenic  Cadmium   Chromium   Copper   Lead   Mercury   Nickel  Selenium   Silver  Zinc

Number         Name                        Code*

304.12.90  Schwann Lake                      LMB      W        0.08                                                            0.15

304.12.90  Schwann Lake                      LMB      W        0.08                                                            0.14

603.20.41  Sabrina Lake                      BN       F                                                     0.10

603.20.41  Sabrina Lake                      BN       F                                                     0.11

603.20.24  Bishop Creek Canal/D/S Bishop     BN       F                                                     0.12

603.20.24  Bishop Creek Canal/D/S Bishop     BN       F                                                     0.10

603.20.24  Bishop Creek Canal/D/S Bishop     BN       L        0.13    0.02       0.02     230.    <0.1              <0.1                0.38     32.

603.20.24  Bishop Creek Canal/D/S Bishop     BN       L        0.14    0.01       0.02     240.    <0.1              <0.1                0.39     33.

603.30.05  Haiwee Reservoir                  SMB      F                                                     0.12

603.30.05  Haiwee Reservoir                  SMB      F                                                     0.13

626.80.03  Little Rock Creek Reservoir       BLB      F                                                     0.31               0.07

626.80.03  Little Rock Creek Reservoir       BLB      F                                                     0.28               0.06

626.80.03  Little Rock Creek Reservoir       BLB      L       <0.05   <0.01      <0.02       2.5   <0.1              <0.1               <0.02     20.

626.80.03  Little Rock Creek Reservoir       BLB      L       <0.05    0.01      <0.02       2.3   <0.1              <0.1               <0.02     20.

628.20.02  Silverwood Lake                   LMB      F                                                                        0.39

628.20.02  Silverwood Lake                   LMB      F                                                                        0.39

405.12.00  Alamitos Bay                      CCB      F                                                     0.05

405.12.00  Alamitos Bay                      CCB      F                                                     0.05

304.13.92  Aptos Creek                       PCP      W                0.03       0.06       0.98  <0.1     0.14     <0.1               <0.02     17.  304.13.92  Aptos Creek                       PCP      W                0.03       0.07       0.98  <0.1     0.13     <0.1               <0.02     17.

309.82.08  Lake Nacimiento/Las Tablas        Sed               4.1     0.41      63.        18.    12.      0.48     67.       0.34      0.07     53.   309.82.08  Lake Nacimiento/Las Tablas        Sed               4.1     0.56      68.        18.    13.      0.48     68.       0.32      0.08     54.

309.82.08  Lake Nacimiento/Las Tablas        Sed                       0.52      69.        20.    13.               67.                 0.07     58.

309.82.04  Lake Nacimiento/Dip Creek         Sed                       0.37      44.        10.    14.      0.09     39.                <0.04     36.   309.82.04  Lake Nacimiento/Dip Creek         Sed                       0.36      46.        15.    15.      0.10     38.                <0.04     38.

309.82.04  Lake Nacimiento/Dip Creek         Sed                       0.37      45.        11.    15.               37.                <0.04     35.

307.00.01  Carmel Lagoon                     Sed                       0.23       4.0        2.3    0.57    0.03      2.5               <0.04      8.0

307.00.01  Carmel Lagoon                     Sed                       0.16       3.6        2.0    0.76    0.03      2.4               <0.04      8.6

307.00.01  Carmel Lagoon                     Sed                       0.19       4.3        2.7    0.74              3.6               <0.04      8.9

* Tables 2, 3, and 4 list code names for species.             L = Liver.     F = Filet.     W = Whole Body.

TABLE S-2 (continued)

Toxic Substances Monitoring Program

Results of Duplicate Sample Analysis:  1991 Trace Metal Quality Control

(ug/g wet weight)

Station       Station                     Species  Tissue   Arsenic  Cadmium   Chromium   Copper   Lead   Mercury   Nickel  Selenium   Silver  Zinc

Number         Name                        Code*

106.40.12  Carrville Pond                    Sed                       0.07     320.        62.     0.90    0.11    790.                 0.06     25.

106.40.12  Carrville Pond                    Sed                       0.07     320.        59.     0.90    0.10    750.                 0.07     25.

106.40.12  Carrvelle Pond                    Sed                      <0.03     330.        62.     0.79            760.                 0.05     27.

* Tables 2, 3, and 4 list code names for species.             L = Liver.     F = Filet.     W = Whole Body.

TABLE S-3

Toxic Substances Monitoring Program

1991 Trace Metal Analysis of Reference Materials (ug/g dry weight)*

REFERENCE

MATERIAL**              AG           AS            CD            CR           CU             HG             NI          PB            SE            ZN

NBS-1577a                       0.047+0.015                                                                                       0.73+0.10

(Bovine Liver)                 (0.047+0.006)                                                                                     (0.71+0.07)

DOLT-1                                         4.47+0.56     0.44+0.24     20.3+1.5                     0.27+0.14     1.40+0.68                  94.1+5.2

(Dogfish Liver)                               (4.18+0.28)   (0.40+0.07)   (20.8+1.2)                   (0.26+0.06)   (1.36+0.29)                (92.5+2.3)

DORM-1                           17.2+0.36    0.106+0.037    3.92+1.7      4.57+1.5      0.787+0.11     1.20+0.32     0.37+0.18   1.61+0.19      19.5+1.2 (Dogfish Muscle)                (17.7+2.1)   (0.086+0.012)  (3.60+0.40)   (5.22+0.33)   (0.798+0.07)   (1.20+0.30)   (0.40+0.12) (1.62+0.12)    (21.3+1.0)

NBS 1566a           1.50+0.40                  4.23+0.67     1.16+0.50     63.1+2.3                     2.19+0.75    0.315+0.100                  835+48.

(Oyster)           (1.63+0.15)                (4.15+0.38)   (1.43+0.46)   (66.3+4.3)                   (2.25+0.44)  (0.371+0.014)                (830+57)

*  Sample values are given first, followed by reference values in parentheses, both values include 95% confidence interval.

**  NBS refers to the National Bureau of Standards; DOLT-1 and DORM-1 are from the National Research Council of Canada; NIES 6 is from the National

Institute for Environmental Studies of Japan.

TABLE S-4

Toxic Substances Monitoring Program

Distribution of Synthetic Organic Compounds Among

Four Fractions of a Standard FlorisilR Column

(0%) Fraction 1	(6%) Fraction 2	(15%) Fraction 3

HCH, alpha*	HCH, alpha*	dacthal

aldrin	HCH, beta	diazinon

chlordene, alpha	HCH, gamma	dichlorobenzophenone, p,p'

chlordene, gamma	HCH, delta	dieldrin

DDE, o,p'	chlorbenside	endosulfan I

DDE, p,p'	cis-chlordane	endrin

DDMU, p,p'	trans-chlordane	malathion

DDT, o,p'	chlorpyrifos	oxadiazon

DDT, p,p'*	DDD, o,p'	parathion, ethyl

heptachlor	DDD, p,p'	parathion, methyl

hexachlorobenzene	DDMS, p,p'	tetradifon (tedion)

trans-nonachlor	DDT, p,p'*

PCB 1248	dicofol (kelthane)

PCB 1254	ethion

PCB 1260	heptachlor epoxide

methoxychlor	(50%) Fraction 4

cis-nonachlor

oxychlordane	endosulfan II

toxaphene	endosulfan sulfate

*	 Found in both 0% and 6% fractions.

TABLE S-5

Toxic Substances Monitoring Program

Synthetic Organic Compounds Analyzed

and Their Detection Limits in Flesh

Compound	Detection Limit

(ng/g, ppb wet weight)

aldrin	5

chlorbenside	50

cis-chlordane	5

trans-chlordane	5

chlordene, alpha	5

chlordene, gamma	5

chlorpyrifos	10

dacthal	5

DDD, o,'p	10

DDD, p,p'	10

DDE, o,p'	10

DDE, p,p'	5

DDMS, p,p'	30

DDMU,p,p'	15

DDT, o,p'	10

DDT, p,p'	10

diazinon	50

dichlorobenzophenone-p,p'	30

dicofol (Kelthane)	100

dieldrin	5

endosulfan I	5

endosulfan II	70

endosulfan sulfate	85

endrin	15

ethion	20

HCH, alpha	2

HCH, beta	10

HCH, gamma	2

HCH, delta	5

heptachlor	5

heptachlor epoxide	5

HCB	2

methoxychlor	15

cis-nonachlor	5

trans-nonachlor	5

oxadiazon	5

oxychlordane	5

parathion, ethyl	10

parathion, methyl	10

PCB 1248	50

PCB 1254	50

PCB 1260	50

pentachlorophenol*	2

2,3,5,6-tetrachlorophenol*	2

tetradifon (Tedion)	10

toxaphene	100

*  Analyzed only when requested.

TABLE S-6

Toxic Substances Monitoring Program

Results of Duplicate Sample Analysis:  1991 Synthetic Organic Compounds Quality Control

(ng/g wet weight)

Station Name                 Newport Bay                       Calleguas Creek                    Conejo Creek                      Alameda Creek/

Niles Canyon Road

Station No.                  801.11.97                         403.12.06                          403.12.07                         204.30.11

Species*                     SSB                               GF                                 GAM                               SCP

REPLICATE                    1        2                        1        2                         1        2                        1        2

COMPOUNDS

cis-chlordane                                                                                                                      8.2      7.2

cis-nonachlor                                                                                                                      9.3      8.6

gamma-chlordene

oxychlordane                                                                                   13.       14.

trans-chlordane

trans-nonachlor                                               5.9      9.2                     37.       45.                      <5.0      7.3

chlorpyrifos                                                                                  <10.       10.

dacthal                                                      30.      24.                     120.      120.

DDD, o,p'                                                    12.      11.                      10.       12.

DDD, p,p'                  12.      18.                     100.      84.                      95.       95.

DDE, o,p'                                                                                      29.       17.

DDE, p,p'                  98.      95.                     950.    1100.                    1700.     1800.                      10.      13.

DDT, o,p'                                                    20.      26.                      56.       56.

DDT, p,p'                                                    88.      91.                     480.      450.

DDMU,p,p'                                                   <15.      26.                      52.       54.

diazinon                                                                                       64.       70.

dieldrin                   <5.0      6.2                                                       39.       34.

endosulfan I

endosulfan II

endosulfan sulfate                                                                            210.      210.

hexachlorobenzene

alpha-HCH

gamma-HCH                                                                                       7.9       8.4

heptachlor epoxide

oxadiazon                  <5.0      9.5                                                                                          21.      26.

PCB 1248                                                                                      <50.       54.

PCB 1254                   78.      71.                     <50.      79.                     302.      130.

PCB 1260                   57.      53.                                                        54.       53.

toxaphene                                                   440.     340.                    2000.     1700.

percent moisture           76.4     76.5                     80.0     79.9                      76.2     76.8                     77.3     77.0

percent lipid               1.52     1.64                     0.397    0.295                     4.04     4.02                     4.42     4.86

* Tables 2, 3, and 4 list code names for species.

< Below detection limit.

TABLE S-6 (continued)

Toxic Substances Monitoring Program

Results of Duplicate Sample Analysis:  1991 Synthetic Organic Compounds Quality Control

(ng/g wet weight)

Station Name                 Suisun Bay                        Huntington Harbor/                 Lost River/Tule Lake              Donner Lake

Anaheim Bay

Station No.                  207.10.90                         801.11.00                          105.92.01                         635.20.04

Species*                     WST                               WCK                                TC                                KOK

REPLICATE                    1        2                        1        2                         1        2                        1        2

COMPOUNDS

cis-chlordane                                                10.      10.                                                         <5.0      5.0

cis-nonachlor                                                11.      12.                                                         10.      11.

gamma-chlordene

oxychlordane                                                                                                                       7.8      9.0

trans-chlordane                                               6.8      6.9

trans-nonachlor                                              15.      15.                                                          8.4      8.4

chlorpyrifos

dacthal

DDD, o,p'

DDD, p,p'                                                    28.      32.

DDE, o,p'

DDE, p,p'                  31.      19.                     340.     390.                       <5.0      5.5                     23.      26.

DDT, o,p'

DDT, p,p'

DDMU,p,p'

dieldrin

endosulfan I

endosulfan II

endosulfan sulfate

hexachlorobenzene

alpha-HCH

gamma-HCH

heptachlor epoxide

oxadiazon

PCB 1254                                                    120.     150.                                                        100.      110.

PCB 1260                  <50.      60.                     140.     160.                                                         65.       74.

toxaphene

percent moisture           81.8     81.5                     75.9     76.2                      79.5     79.4                     78.0     78.1

percent lipid               0.270    0.229                    3.73     3.54                      3.00     3.17                     3.03     3.15

* Tables 2, 3, and 4 list code names for species.

< Below detection limit.

TABLE S-6 (continued)

Toxic Substances Monitoring Program

Results of Duplicate Sample Analysis:  1991 Synthetic Organic Compounds Quality Control

(ng/g wet weight)

Station Name                 Santa Maria River/

Mouth

Station No.                  312.10.00

Species*                     Sed

REPLICATE                    1        2

COMPOUNDS

cis-chlordane

cis-nonachlor

gamma-chlordene

oxychlordane

trans-chlordane

trans-nonachlor

chlorpyrifos

dacthal

DDD, o,p'

DDD, p,p'

DDE, o,p'

DDE, p,p'

DDT, o,p'

DDT, p,p'

DDMU,p,p'

dieldrin

endosulfan I

endosulfan II

endosulfan sulfate

hexachlorobenzene

alpha-HCH

gamma-HCH

heptachlor epoxide

oxadiazon

PCB 1254

PCB 1260

toxaphene

percent moisture           68.3     69.4

percent lipid

* Tables 2, 3, and 4 list code names for species.

TABLE S-7

Toxic Substances Monitoring Program

Polynuclear Aromatic Hydrocarbons (PAHs) Analyzed

and Their Detection Limits in Flesh

Detection Limit

Compound	(ng/g, ppb wet weight)

1991

naphthalene	100

1-methylnaphthalene	100

2-methylnaphthalene	100

biphenyl	100

2,6-dimethylnaphthalene	100

acenaphthylene	100

acenaphthene	100

2,3,5-trimethylnaphthalene	100

fluorene	100

phenanthrene	100

anthracene	100

1-methylphenanthrene	100

fluoranthene	100

pyrene	100

benz[a]anthracene	100

chrysene	100

benzo[b]fluoranthene	100

benzo[k]fluoranthene	100

benzo[e]pyrene	100

benzo[a]pyrene	100

perylene	100

indeno[1,2,3-cd]pyrene	100

dibenz[a,h]anthracene	100

benzo[ghi]perylene	100