FRESHWATER SAMPLES FROM SAN DIEGO CREEK

(DRY AND WET WEATHER EVENTS)

San Diego Creek Sampling

For wet weather, samples were collected from the Campus Drive bridge.  To take the samples, a metal davit was attached to the bridge.  From the davit a torpedo sampler was lowered to the creek.  Inside the sampler was a 1 gallon glass bottle.  At each sampling period, multiple drops were made and the water was transferred to a 5 gallon polycarbonate bottle until it was full.  Samples for all analyses were then aliquoted from this composite.  Samples for dissolved metals were taken from the composite by an ISCO sampler with a 0.45 micron filter attached using EPA suggested clean sampling techniques.  Samples for total and dissolved metals analysis were transferred to cleaned plastic 500 ml or 1 liter bottles.  Total suspended solids aliquots were put into 1 liter plastic bottles.  Storm rainfall data and stream flow rates during sample collection are presented on Figures 1 and 2.

For dry weather, samples were collected directly from the creek bed at the base of the bridge using an ISCO sampler.  For each collection event, a sample was taken in the morning and then another was taken 4 to 5 hours later.  The samples for metals analysis went straight from the creek into their respective containers (either through the filter or not).  The remaining samples were taken from a composite made in a 5 gallon polycarbonate bottle.  All sample containers were the same as described above.

All samples were stored on ice until transported back to SCCWRP where they were stored at 5 C.  Samples for chemical analysis were delivered to the chemistry lab within 24 hrs of collection.

Figure 1.  Hydrograph of San Diego Creek at Campus Drive for wet weather event in March 7, 2002.  Rainfall measured 0.36 inches at John Wayne Airport.  Dashed lines indicate when samples were taken; at stream flows of 238 cfs and 170 cfs, respectively.

Figure 2.  Hydrograph of San Diego Creek at Campus Drive for wet weather event in November 8, 2002.  Rainfall measured 0.54” on 11/8, 0.54” on 11/9, and 0.07” on 11/10 at John Wayne Airport.  Dashed lines indicate when samples were taken; 9 cfs and 27 cfs, respectively.

Table 1.  Concentrations of total and dissolved metals from San Diego Creek wet weather runoff samples collected March 7, 2002.  Results are reported in ug/L, except where noted.

1100 hrs		1425 hrs

| Constituent/Metal | MDL | Total | Dissolved | Total | Dissolved |
| --- | --- | --- | --- | --- | --- |
| Aluminum | 0.01 | 1255 | 160 | 872 | 260 |
| Antimony | 0.01 | 1.58 | 1.41 | 2.18 | 1.86 |
| Arsenic | 0.01 | 4.38 | 3.9 | 3.91 | 3.62 |
| Barium | 0.5 | 46.2 | 34.2 | 35.6 | 25.0 |
| Beryllium | 0.005 | <0.5 | <0.5 | <0.5 | <0.5 |
| Cadmium | 0.005 | 0.15 | <0.1 | 0.18 | <0.1 |
| Chromium | 0.005 | 5.20 | 3.66 | 2.44 | 1.36 |
| Cobalt | 0.005 | 1.17 | <0.5 | <0.5 | <0.5 |
| Copper | 0.005 | 15.7 | 10.4 | 15.6 | 10.6 |
| Iron | 0.01 | 1036 | 198 | 804 | 236 |
| Lead | 0.005 | 2.02 | 0.30 | 2.44 | 0.54 |
| Manganese | 0.005 | 116 | 83.4 | 65.2 | 42.0 |
| Mercury | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 |
| Molybdenum | 0.005 | 17.8 | 21.0 | 9.12 | 10.1 |
| Nickel | 0.005 | 5.04 | 4.09 | 4.78 | 3.87 |
| Selenium | 0.01 | 10.6 | 10.2 | 4.30 | 4.16 |
| Silver | 0.005 | <0.1 | <0.1 | <0.1 | <0.1 |
| Strontium | 0.01 | 862 | 855 | 392 | 414 |
| Thallium | 0.005 | <0.5 | <0.5 | <0.5 | <0.5 |
| Tin | 0.005 | 0.11 | <0.5 | 0.15 | <0.5 |
| Titanium | 0.005 | 46.1 | 11.1 | 44.7 | 16.6 |
| Vanadium | 0.005 | 12.6 | 9.88 | 10.6 | 8.84 |
| Zinc | 0.005 | 55.0 | 30.2 | 51.6 | 27.2 |
| TSS mg/L |  | 62 |  | 38 |  |
| Alkalinity (mg/L) |  | 129 |  | 85 |  |
| Hardness (mg/L) |  | 393 |  | 183 |  |

Table 2.  Concentrations of total PCBs from San Diego Creek wet weather runoff samples collected March 7, 2002.  Concentrations are in ng/L.

|  | 1100 hrs | 1425 hrs |
| --- | --- | --- |
| PCB018 | <1.0 | <1.0 |
| PCB028 | <1.0 | <1.0 |
| PCB031 | <1.0 | <1.0 |
| PCB033 | <1.0 | <1.0 |
| PCB037 | <1.0 | <1.0 |
| PCB044 | <1.0 | <1.0 |
| PCB049 | <1.0 | <1.0 |
| PCB052 | <1.0 | <1.0 |
| PCB066 | <1.0 | <1.0 |
| PCB070 | <1.0 | <1.0 |
| PCB074 | <1.0 | <1.0 |
| PCB077 | <1.0 | <1.0 |
| PCB081 | <1.0 | <1.0 |
| PCB087 | <1.0 | <1.0 |
| PCB095 | <1.0 | <1.0 |
| PCB097 | <1.0 | <1.0 |
| PCB099 | <1.0 | <1.0 |
| PCB101 | <1.0 | <1.0 |
| PCB105 | <1.0 | <1.0 |
| PCB110 | <1.0 | <1.0 |
| PCB114 | <1.0 | <1.0 |
| PCB118 | <1.0 | <1.0 |
| PCB119 | <1.0 | <1.0 |
| PCB123 | <1.0 | <1.0 |
| PCB126 | <1.0 | <1.0 |
| PCB128 | <1.0 | <1.0 |
| PCB138 | <1.0 | <1.0 |
| PCB141 | <1.0 | <1.0 |
| PCB149 | <1.0 | <1.0 |
| PCB151 | <1.0 | <1.0 |
| PCB153 | <1.0 | <1.0 |
| PCB156 | <1.0 | <1.0 |
| PCB157 | <1.0 | <1.0 |
| PCB158 | <1.0 | <1.0 |
| PCB167 | <1.0 | <1.0 |
| PCB168/132 | <1.0 | <1.0 |
| PCB169 | <1.0 | <1.0 |
| PCB170 | <1.0 | <1.0 |
| PCB177 | <1.0 | <1.0 |
| PCB180 | <1.0 | <1.0 |
| PCB183 | <1.0 | <1.0 |
| PCB187 | <1.0 | <1.0 |
| PCB189 | <1.0 | <1.0 |
| PCB194 | <1.0 | <1.0 |
| PCB200 | <1.0 | <1.0 |
| PCB201 | <1.0 | <1.0 |
| PCB206 | <1.0 | <1.0 |
| Total Detectable PCB | <1.0 | <1.0 |

Table 3.  Concentrations of total chlorinated pesticides from San Diego Creek wet weather runoff samples collected March 7, 2002.  Concentrations are in ng/L.

|  | 1100 hrs | 1425hrs |
| --- | --- | --- |
| Toxaphene | <10 | <10 |
| 2,4'-DDD | <1.0 | <1.0 |
| 2,4'-DDE | <1.0 | <1.0 |
| 2,4'-DDT | <1.0 | <1.0 |
| 4,4'-DDD | <1.0 | <1.0 |
| 4,4'-DDE | <1.0 | <1.0 |
| 4,4'-DDT | <1.0 | <1.0 |
| Total Detectable DDT | <1.0 | <1.0 |
| Aldrin | <1.0 | <1.0 |
| BHC-alpha | <1.0 | <1.0 |
| BHC-beta | <1.0 | <1.0 |
| BHC-delta | <1.0 | <1.0 |
| BHC-gamma | <1.0 | <1.0 |
| Chlordane-alpha | <1.0 | <1.0 |
| Chlordane-gamma | <1.0 | <1.0 |
| Dieldrin | <1.0 | <1.0 |
| Endosulfan Sulfate | <1.0 | <1.0 |
| Endosulfan-I | <1.0 | <1.0 |
| Endosulfan-II | <1.0 | <1.0 |
| Endrin | <1.0 | <1.0 |
| Endrin Aldehyde | <1.0 | <1.0 |
| Heptachlor | <1.0 | <1.0 |
| Heptachlor Epoxide | <1.0 | <1.0 |
| Methoxychlor | <1.0 | <1.0 |
| Mirex | 1 | <1.0 |
| trans-Nonachlor | <1.0 | <1.0 |

Table 4.  Concentrations of total organophosphorus pesticides in runoff from San Diego Creek wet weather samples collected March 7, 2002.  Concentrations are in ng/L.  Note:  data table is based on verbal communication with chemistry laboratory.

| Compound | Sample 1100 | Sample 1425 |
| --- | --- | --- |
| Bolstar (Sulprofos) | <10 | <10 |
| Chlorpyrifos | <10 | <10 |
| Coumaphos | <10 | <10 |
| Demeton | <10 | <10 |
| Diazinon | <10 | <10 |
| Disulfoton | <10 | <10 |
| Ethoprop (Ethoprofos) | <10 | <10 |
| Fenchlorophos (Ronnel) | <10 | <10 |
| Fensulfothion | <10 | <10 |
| Fenthion | <10 | <10 |
| Guthion | <10 | <10 |
| Merphos | <10 | <10 |
| Methyl Parathion | <10 | <10 |
| Mevinphos (Phosdrin) | <10 | <10 |
| Phorate | <10 | <10 |
| Tetrachlovinphos (Stirofos) | <10 | <10 |
| Tokuthion | <10 | <10 |
| Trichloronate | <10 | <10 |

Table 5.  Concentrations of total and dissolved metals from San Diego Creek dry weather runoff samples collected May 2, 2002.  Results are reported in ug/L, except where noted.

|  |  | 0900 hrs |  | 1405 hrs |  |
| --- | --- | --- | --- | --- | --- |
| Constituent/Metal | MDL | Total | Dissolved | Total | Dissolved |
| Aluminum | 0.01 | 1050 | 220 | 1100 | 200 |
| Antimony | 0.01 | 1.30 | 1.46 | 1.32 | 1.48 |
| Arsenic | 0.01 | 6.44 | 5.27 | 6.51 | 5.22 |
| Barium | 0.5 | 73.5 | 58.2 | 74.3 | 59.2 |
| Beryllium | 0.005 | <0.5 | <0.5 | <0.5 | <0.5 |
| Cadmium | 0.005 | 0.16 | <0.1 | 0.17 | <0.1 |
| Chromium | 0.005 | 2.15 | 0.74 | 2.14 | 0.74 |
| Cobalt | 0.005 | 0.88 | <0.6 | 0.90 | <0.6 |
| Copper | 0.005 | 4.68 | 2.81 | 5.60 | 2.77 |
| Iron | 0.01 | 1345 | 299 | 1390 | 284 |
| Lead | 0.005 | 1.49 | 0.21 | 1.66 | 0.20 |
| Manganese | 0.005 | 130 | 15.8 | 120 | 19.0 |
| Mercury | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 |
| Molybdenum | 0.005 | 37.4 | 46.2 | 37.0 | 46.0 |
| Nickel | 0.005 | 4 | 3.08 | 3.96 | 3.01 |
| Selenium | 0.01 | 18.4 | 18.4 | 19.8 | 18.6 |
| Silver | 0.005 | <0.1 | <0.1 | <0.1 | <0.1 |
| Strontium | 0.01 | 1500 | 1515 | 1495 | 1505 |
| Thallium | 0.005 | <0.5 | <0.5 | <0.5 | <0.5 |
| Tin | 0.005 | 0.16 | 0.16 | 0.15 | 0.13 |
| Titanium | 0.005 | 55.4 | 14.0 | 61.2 | 13.0 |
| Vanadium | 0.005 | 16.7 | 11.4 | 16.6 | 11.8 |
| Zinc | 0.005 | 10.8 | 4.54 | 12.6 | 4.42 |
| TSS mg/L |  | 80 |  | 83 |  |
| Alkalinity (mg/L) |  | 261 |  | 232 |  |
| Hardness (mg/L) |  | 681 |  | 666 |  |

Table 6.  Concentrations of total PCBs from San Diego Creek dry weather runoff samples collected May 2, 2002.  Concentrations are in ng/L.

|  | 0900 hrs | 1405 hrs |
| --- | --- | --- |
| PCB018 | <1.0 | <1.0 |
| PCB028 | <1.0 | <1.0 |
| PCB031 | <1.0 | <1.0 |
| PCB033 | <1.0 | <1.0 |
| PCB037 | <1.0 | <1.0 |
| PCB044 | <1.0 | <1.0 |
| PCB049 | <1.0 | <1.0 |
| PCB052 | <1.0 | <1.0 |
| PCB066 | <1.0 | <1.0 |
| PCB070 | <1.0 | <1.0 |
| PCB074 | <1.0 | <1.0 |
| PCB077 | <1.0 | <1.0 |
| PCB081 | <1.0 | <1.0 |
| PCB087 | <1.0 | <1.0 |
| PCB095 | <1.0 | <1.0 |
| PCB097 | <1.0 | <1.0 |
| PCB099 | <1.0 | <1.0 |
| PCB101 | <1.0 | <1.0 |
| PCB105 | <1.0 | <1.0 |
| PCB110 | <1.0 | <1.0 |
| PCB114 | <1.0 | <1.0 |
| PCB118 | <1.0 | <1.0 |
| PCB119 | <1.0 | <1.0 |
| PCB123 | <1.0 | <1.0 |
| PCB126 | <1.0 | <1.0 |
| PCB128 | <1.0 | <1.0 |
| PCB138 | <1.0 | <1.0 |
| PCB141 | <1.0 | <1.0 |
| PCB149 | <1.0 | <1.0 |
| PCB151 | <1.0 | <1.0 |
| PCB153 | <1.0 | <1.0 |
| PCB156 | <1.0 | <1.0 |
| PCB157 | <1.0 | <1.0 |
| PCB158 | <1.0 | <1.0 |
| PCB167 | <1.0 | <1.0 |
| PCB168/132 | <1.0 | <1.0 |
| PCB169 | <1.0 | <1.0 |
| PCB170 | <1.0 | <1.0 |
| PCB177 | <1.0 | <1.0 |
| PCB180 | <1.0 | <1.0 |
| PCB183 | <1.0 | <1.0 |
| PCB187 | <1.0 | <1.0 |
| PCB189 | <1.0 | <1.0 |
| PCB194 | <1.0 | <1.0 |
| PCB200 | <1.0 | <1.0 |
| PCB201 | <1.0 | <1.0 |
| PCB206 | <1.0 | <1.0 |
| Total Detectable PCB | <1.0 | <1.0 |

Table 7.  Concentrations of total chlorinated pesticides from San Diego Creek dry weather runoff samples collected May 2, 2002.  Concentrations are in ng/L.

|  | 0900 hrs | 1405 hrs |
| --- | --- | --- |
| Toxaphene | <10 | <10 |
| 2,4'-DDD | <1.0 | <1.0 |
| 2,4'-DDE | <1.0 | <1.0 |
| 2,4'-DDT | <1.0 | <1.0 |
| 4,4'-DDD | <1.0 | <1.0 |
| 4,4'-DDE | <1.0 | <1.0 |
| 4,4'-DDT | <1.0 | <1.0 |
| Total Detectable DDT | <1.0 | <1.0 |
| Aldrin | <1.0 | <1.0 |
| BHC-alpha | <1.0 | <1.0 |
| BHC-beta | <1.0 | <1.0 |
| BHC-delta | <1.0 | <1.0 |
| BHC-gamma | <1.0 | <1.0 |
| Chlordane-alpha | <1.0 | <1.0 |
| Chlordane-gamma | <1.0 | <1.0 |
| Dieldrin | <1.0 | <1.0 |
| Endosulfan Sulfate | <1.0 | <1.0 |
| Endosulfan-I | <1.0 | <1.0 |
| Endosulfan-II | <1.0 | <1.0 |
| Endrin | <1.0 | <1.0 |
| Endrin Aldehyde | <1.0 | <1.0 |
| Heptachlor | <1.0 | <1.0 |
| Heptachlor Epoxide | <1.0 | <1.0 |
| Methoxychlor | <1.0 | <1.0 |
| Mirex | <1.0 | <1.0 |
| trans-Nonachlor | <1.0 | <1.0 |

Table 8.  Concentrations of total organophosphorus pesticides in runoff from San Diego Creek wet weather samples collected May 2, 2002.  Concentrations are in ng/L.  Note:  data table is based on verbal communication with chemistry laboratory.

| Compound | Sample 0900 | Sample 1405 |
| --- | --- | --- |
| Bolstar (Sulprofos) | <10 | <10 |
| Chlorpyrifos | <10 | <10 |
| Coumaphos | <10 | <10 |
| Demeton | <10 | <10 |
| Diazinon | <10 | <10 |
| Disulfoton | <10 | <10 |
| Ethoprop (Ethoprofos) | <10 | <10 |
| Fenchlorophos (Ronnel) | <10 | <10 |
| Fensulfothion | <10 | <10 |
| Fenthion | <10 | <10 |
| Guthion | <10 | <10 |
| Merphos | <10 | <10 |
| Methyl Parathion | <10 | <10 |
| Mevinphos (Phosdrin) | <10 | <10 |
| Phorate | <10 | <10 |
| Tetrachlovinphos (Stirofos) | <10 | <10 |
| Tokuthion | <10 | <10 |
| Trichloronate | <10 | <10 |

Table 9.  Concentrations of total and dissolved metals from San Diego Creek dry weather runoff samples collected August 12, 2002.  Results are reported in ug/L, except where noted.

|  | 0922 hrs |  |  |  | 1400 hrs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Constituent/Metal |  | MDL | Total | Dissolved |  | Total | Dissolved |
| Aluminum |  | 0.01 | 538 | <0.01 |  | 392 | <0.01 |
| Antimony |  | 0.01 | 1.08 | 1.09 |  | 1.04 | 1.12 |
| Arsenic |  | 0.01 | 8.74 | 7.64 |  | 8.25 | 7.92 |
| Barium |  | 0.5 | 53.4 | 41.2 |  | 51.6 | 43.2 |
| Beryllium |  | 0.005 | <0.005 | <0.005 |  | <0.005 | <0.005 |
| Cadmium |  | 0.005 | 0.14 | <0.005 |  | 0.10 | <0.005 |
| Chromium |  | 0.005 | 0.92 | <0.005 |  | <0.005 | <0.005 |
| Cobalt |  | 0.005 | 0.70 | 0.51 |  | 0.62 | 0.51 |
| Copper |  | 0.005 | 3.62 | 1.60 |  | 3.06 | 1.96 |
| Iron |  | 0.01 | 758 | 110 |  | 526 | 109 |
| Lead |  | 0.005 | 1.16 | <0.005 |  | 0.86 | <0.005 |
| Manganese |  | 0.005 | 89.1 | 22.0 |  | 69.2 | 19.6 |
| Mercury |  | 0.005 | <0.005 | <0.005 |  | <0.005 | <0.005 |
| Molybdenum |  | 0.005 | 53.1 | 55.9 |  | 51.4 | 55.8 |
| Nickel |  | 0.005 | 3.32 | 2.76 |  | 3.12 | 2.99 |
| Selenium |  | 0.01 | 17.3 | 16.4 |  | 16.4 | 16.4 |
| Silver |  | 0.005 | <0.005 | <0.005 |  | <0.005 | <0.005 |
| Strontium |  | 0.01 | 1625 | 1585 |  | 1550 | 1585 |
| Thallium |  | 0.005 | <0.005 | <0.005 |  | <0.005 | <0.005 |
| Tin |  | 0.005 | 0.40 | 0.42 |  | 0.36 | 0.36 |
| Titanium |  | 0.005 | 33.6 | 0.64 |  | 21.8 | 0.57 |
| Vanadium |  | 0.005 | 18.0 | 15.6 |  | 17.6 | 17.0 |
| Zinc |  | 0.005 | 7.91 | 2.02 |  | 7.16 | 2.16 |
| TSS mg/L |  |  | 61 |  |  | 41 |  |
| Alkalinity (mg/L) |  |  | 282 |  |  | 293 |  |
| Hardness (mg/L) |  |  | 524 |  |  | 692 |  |

Table 10.  Concentrations of total PCBs from San Diego Creek dry weather runoff samples collected August 12, 2002. Concentrations are in ng/L.

|  | 0922 hrs | 1400 hrs |
| --- | --- | --- |
| PCB018 | <1.0 | <1.0 |
| PCB028 | <1.0 | <1.0 |
| PCB031 | <1.0 | <1.0 |
| PCB033 | <1.0 | <1.0 |
| PCB037 | <1.0 | <1.0 |
| PCB044 | <1.0 | <1.0 |
| PCB049 | <1.0 | <1.0 |
| PCB052 | <1.0 | <1.0 |
| PCB066 | <1.0 | <1.0 |
| PCB070 | <1.0 | <1.0 |
| PCB074 | <1.0 | <1.0 |
| PCB077 | <1.0 | <1.0 |
| PCB081 | <1.0 | <1.0 |
| PCB087 | <1.0 | <1.0 |
| PCB095 | <1.0 | <1.0 |
| PCB097 | <1.0 | <1.0 |
| PCB099 | <1.0 | <1.0 |
| PCB101 | <1.0 | <1.0 |
| PCB105 | <1.0 | <1.0 |
| PCB110 | <1.0 | <1.0 |
| PCB114 | <1.0 | <1.0 |
| PCB118 | <1.0 | <1.0 |
| PCB119 | <1.0 | <1.0 |
| PCB123 | <1.0 | <1.0 |
| PCB126 | <1.0 | <1.0 |
| PCB128 | <1.0 | <1.0 |
| PCB138 | <1.0 | <1.0 |
| PCB141 | <1.0 | <1.0 |
| PCB149 | <1.0 | <1.0 |
| PCB151 | <1.0 | <1.0 |
| PCB153 | <1.0 | <1.0 |
| PCB156 | <1.0 | <1.0 |
| PCB157 | <1.0 | <1.0 |
| PCB158 | <1.0 | <1.0 |
| PCB167 | <1.0 | <1.0 |
| PCB168/132 | <1.0 | <1.0 |
| PCB169 | <1.0 | <1.0 |
| PCB170 | <1.0 | <1.0 |
| PCB177 | <1.0 | <1.0 |
| PCB180 | <1.0 | <1.0 |
| PCB183 | <1.0 | <1.0 |
| PCB187 | <1.0 | <1.0 |
| PCB189 | <1.0 | <1.0 |
| PCB194 | <1.0 | <1.0 |
| PCB200 | <1.0 | <1.0 |
| PCB201 | <1.0 | <1.0 |
| PCB206 | <1.0 | <1.0 |
| Total Detectable PCB | <1.0 | <1.0 |

Table 11.  Concentrations of chlorinated pesticides from San Diego Creek dry weather runoff samples collected August 12, 2002.  Concentrations are in ng/L.

|  | 0922 hrs | 1400 hrs |
| --- | --- | --- |
| Compound | Sample | Sample |
| Toxaphene | <10 | <10 |
| 2,4'-DDD | <1.0 | <1.0 |
| 2,4'-DDE | <1.0 | <1.0 |
| 2,4'-DDT | <1.0 | <1.0 |
| 4,4'-DDD | <1.0 | <1.0 |
| 4,4'-DDE | <1.0 | <1.0 |
| 4,4'-DDT | <1.0 | <1.0 |
| Total Detectable DDT | <1.0 | <1.0 |
| Aldrin | <1.0 | <1.0 |
| BHC-alpha | <1.0 | <1.0 |
| BHC-beta | <1.0 | <1.0 |
| BHC-delta | <1.0 | <1.0 |
| BHC-gamma | <1.0 | <1.0 |
| Chlordane-alpha | <1.0 | <1.0 |
| Chlordane-gamma | <1.0 | <1.0 |
| Dieldrin | <1.0 | <1.0 |
| Endosulfan Sulfate | <1.0 | <1.0 |
| Endosulfan-I | <1.0 | <1.0 |
| Endosulfan-II | <1.0 | <1.0 |
| Endrin | <1.0 | <1.0 |
| Endrin Aldehyde | <1.0 | <1.0 |
| Heptachlor | <1.0 | <1.0 |
| Heptachlor Epoxide | <1.0 | <1.0 |
| Methoxychlor | <1.0 | <1.0 |
| Mirex | <1.0 | <1.0 |
| trans-Nonachlor | <1.0 | <1.0 |

Table 12.  Concentrations of total organophosphorus pesticides in runoff from San Diego Creek wet weather samples collected August 12, 2002.  Concentrations are in ng/L.  Note:  data table is based on verbal communication with chemistry laboratory.

| Compound | Sample 0922 | Sample 1400 |
| --- | --- | --- |
| Bolstar (Sulprofos) | <10 | <10 |
| Chlorpyrifos | <10 | <10 |
| Coumaphos | <10 | <10 |
| Demeton | <10 | <10 |
| Diazinon | <10 | <10 |
| Disulfoton | <10 | <10 |
| Ethoprop (Ethoprofos) | <10 | <10 |
| Fenchlorophos (Ronnel) | <10 | <10 |
| Fensulfothion | <10 | <10 |
| Fenthion | <10 | <10 |
| Guthion | <10 | <10 |
| Merphos | <10 | <10 |
| Methyl Parathion | <10 | <10 |
| Mevinphos (Phosdrin) | <10 | <10 |
| Phorate | <10 | <10 |
| Tetrachlovinphos (Stirofos) | <10 | <10 |
| Tokuthion | <10 | <10 |
| Trichloronate | <10 | <10 |

Table 13.  Concentrations of total and dissolved metals from San Diego Creek wet weather runoff samples collected November 8, 2002.  Results are reported in ug/L, except where noted.

|  | 0100 hrs |  |  | 0430 hrs |  |
| --- | --- | --- | --- | --- | --- |
| Element | MDL | Total | Dissolved | Total | Dissolved |
| Metals |  |  |  |  |  |
| Aluminum | 0.01 | 380 | 19.5 | 460 | 19.6 |
| Antimony | 0.01 | 1.04 | 1.20 | 1.05 | 1.22 |
| Arsenic | 0.01 | 5.54 | 4.86 | 5.68 | 5.12 |
| Barium | 0.5 | 52.8 | 48.8 | 53.4 | 48.0 |
| Beryllium | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 |
| Cadmium | 0.005 | 0.12 | <0.005 | 0.14 | <0.005 |
| Chromium | 0.005 | 1.70 | 0.90 | 1.82 | 1.04 |
| Cobalt | 0.005 | 0.58 | <0.005 | 0.62 | <0.005 |
| Copper | 0.005 | 3.46 | 1.84 | 3.66 | 1.885 |
| Iron | 0.01 | 666 | 202 | 766 | 202 |
| Lead | 0.005 | 1.11 | <0.005 | 1.16 | <0.005 |
| Manganese | 0.005 | 50.2 | 11.5 | 54.9 | 12.1 |
| Mercury | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 |
| Molybdenum | 0.005 | 53.5 | 60.4 | 50.4 | 60.0 |
| Nickel | 0.005 | 3.78 | 3.27 | 3.96 | 3.26 |
| Selenium | 0.01 | 24.5 | 22.9 | 24.9 | 24.1 |
| Silver | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 |
| Strontium | 0.01 | 1775 | 1690 | 1750 | 1680 |
| Thallium | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 |
| Tin | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 |
| Titanium | 0.005 | 17.2 | 2.11 | 17.3 | 2.37 |
| Vanadium | 0.005 | 16 | 12.8 | 16.2 | 13.0 |
| Zinc | 0.005 | 9.26 | 3.14 | 10.4 | 3.04 |
| TSS mg/L |  | 48.9 |  | 55.3 |  |
| Alkalinity (mg/L) |  | 293 |  | 292 |  |
| Hardness (mg/L) |  | 768 |  | 783 |  |

Table 14.  Concentrations of total and particulate PCBs from San Diego Creek wet weather runoff samples collected November 8, 2002.  Concentrations are in ng/L.

|  | 0100 hrs |  | 0430 hrs |  |
| --- | --- | --- | --- | --- |
| Compound | Total | Particulate | Total | Particulate |
| PCB018 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB028 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB031 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB033 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB037 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB044 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB049 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB052 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB066 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB070 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB074 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB077 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB081 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB087 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB095 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB097 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB099 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB101 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB105 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB110 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB114 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB118 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB119 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB123 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB126 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB128 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB138 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB141 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB149 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB151 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB153 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB156 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB157 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB158 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB167 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB168/132 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB169 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB170 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB177 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB180 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB183 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB187 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB189 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB194 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB200 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB201 | <1.0 | <1.0 | <1.0 | <1.0 |
| PCB206 | <1.0 | <1.0 | <1.0 | <1.0 |
| Total Detectable PCB | <1.0 | <1.0 | <1.0 | <1.0 |

Table 15.  Concentrations of total and particulate chlorinated pesticides from San Diego Creek wet weather runoff samples collected November 8, 2002. Concentrations are in ng/L.

|  | 0100 hrs |  | 0430 hrs |  |
| --- | --- | --- | --- | --- |
| Compound | Total | Particulate | Total | Particulate |
| Toxaphene | <10 | <10 | <10 | <10 |
| 2,4'-DDD | <1.0 | <1.0 | <1.0 | <1.0 |
| 2,4'-DDE | <1.0 | <1.0 | <1.0 | <1.0 |
| 2,4'-DDT | <1.0 | <1.0 | <1.0 | <1.0 |
| 4,4'-DDD | <1.0 | <1.0 | <1.0 | <1.0 |
| 4,4'-DDE | 3.2 | <1.0 | <1.0 | <1.0 |
| 4,4'-DDT | <1.0 | <1.0 | <1.0 | <1.0 |
| Total Detectable DDT | 3 | <1.0 | <1.0 | <1.0 |
| Aldrin | <1.0 | <1.0 | <1.0 | <1.0 |
| BHC-alpha | <1.0 | <1.0 | <1.0 | <1.0 |
| BHC-beta | <1.0 | <1.0 | <1.0 | <1.0 |
| BHC-delta | <1.0 | <1.0 | <1.0 | <1.0 |
| BHC-gamma | <1.0 | <1.0 | <1.0 | <1.0 |
| Chlordane-alpha | <1.0 | <1.0 | <1.0 | <1.0 |
| Chlordane-gamma | <1.0 | <1.0 | <1.0 | <1.0 |
| Dieldrin | <1.0 | <1.0 | <1.0 | <1.0 |
| Endosulfan Sulfate | <1.0 | <1.0 | <1.0 | <1.0 |
| Endosulfan-I | <1.0 | <1.0 | <1.0 | <1.0 |
| Endosulfan-II | <1.0 | <1.0 | <1.0 | <1.0 |
| Endrin | <1.0 | <1.0 | <1.0 | <1.0 |
| Endrin Aldehyde | <1.0 | <1.0 | <1.0 | <1.0 |
| Heptachlor | <1.0 | <1.0 | <1.0 | <1.0 |
| Heptachlor Epoxide | <1.0 | <1.0 | <1.0 | <1.0 |
| Methoxychlor | <1.0 | <1.0 | <1.0 | <1.0 |
| Mirex | <1.0 | <1.0 | <1.0 | <1.0 |
| trans-Nonachlor | <1.0 | <1.0 | <1.0 | <1.0 |

Table 16.  Concentrations of total and particulate organophosphorus pesticides in runoff from San Diego Creek wet weather samples collected November 8, 2002.  Concentrations are in ng/L.

|  | Sample 0100 |  | Sample 0430 |  |
| --- | --- | --- | --- | --- |
| Compound | Total | Particulate | Total | Particulate |
| Bolstar (Sulprofos) | <10 | <10 | <10 | <10 |
| Chlorpyrifos | <10 | <10 | <10 | <10 |
| Coumaphos | <10 | <10 | <10 | <10 |
| Demeton | <10 | <10 | <10 | <10 |
| Diazinon | 34.9 | <10 | 33.2 | <10 |
| Disulfoton | <10 | <10 | <10 | <10 |
| Ethoprop (Ethoprofos) | <10 | <10 | <10 | <10 |
| Fenchlorophos (Ronnel) | <10 | <10 | <10 | <10 |
| Fensulfothion | <10 | <10 | <10 | <10 |
| Fenthion | <10 | <10 | <10 | <10 |
| Guthion | <10 | <10 | <10 | <10 |
| Merphos | <10 | <10 | <10 | <10 |
| Methyl Parathion | <10 | <10 | <10 | <10 |
| Mevinphos (Phosdrin) | <10 | <10 | <10 | <10 |
| Phorate | <10 | <10 | <10 | <10 |
| Tetrachlovinphos (Stirofos) | <10 | <10 | <10 | <10 |
| Tokuthion | <10 | <10 | <10 | <10 |
| Trichloronate | 25.5 | <10 | <10 | <10 |

Table 17.  Sample and site location information.

| Station Location | Station | Date | Time | Depth (m) | Lat Degree | Lat Min. | Long Degree | Long Min. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| San Diego Creek | Campus Dr. | 3/7/02 | 1100 | N/A | 33 | 39.314 | -117 | 50.722 |
| San Diego Creek | Campus Dr. | 3/7/02 | 1425 | N/A | 33 | 39.314 | -117 | 50.722 |
| San Diego Creek | Campus Dr. | 5/2/02 | 900 | N/A | 33 | 39.314 | -117 | 50.722 |
| San Diego Creek | Campus Dr. | 5/2/02 | 1405 | N/A | 33 | 39.314 | -117 | 50.722 |
| San Diego Creek | Campus Dr. | 8/12/02 | 900 | N/A | 33 | 39.314 | -117 | 50.722 |
| San Diego Creek | Campus Dr. | 8/12/02 | 1350 | N/A | 33 | 39.314 | -117 | 50.722 |
| San Diego Creek | Campus Dr. | 11/8/02 | 100 | N/A | 33 | 39.314 | -117 | 50.722 |
| San Diego Creek | Campus Dr. | 11/8/02 | 430 | N/A | 33 | 39.314 | -117 | 50.722 |