SAN LUIS REY RIVER

Hydrologic Unit # 903.11

NEW 303(d) LISTINGS

Chloride, Sulfate and Total Dissolved Solids (TDS)

# PREVIOUS 303(d) LISTINGS

None

# WATERSHED CHARACTERISTICS

The San Luis Rey River is located in the San Luis Rey Watershed in the north end of San Diego County, California. The San Luis Rey River originates from Lake Henshaw.  In the lower segment, it runs parallel to Highway 76 all the way to the pacific coastline.

The San Luis Rey River is classified an inland surface water.  It is designated with the following beneficial uses: MUN, AGR, IND, REC1, REC2, WARM, WILD and RARE1.

## WATER QUALITY OBJECTIVES NOT ATTAINED

Chloride	The Basin Plan1 objective is 250 mg/L.

Sulfate	The Basin Plan1 objective is 250 mg/L.

TDS		The Basin Plan1 objective is 500 mg/L.

## EVIDENCE OF IMPAIRMENT

Chloride	Data collected in October 1997 to November 2000 by the City of Oceanside Water Utilities Laboratory2 showed 3 locations along the San Luis Rey River to exceed 250 mg/L.  Three locations in the City of Oceanside were sampled quarterly for chloride.  At Bonsall Bridge, 8 of 11 (73%) samples exceeded the Basin Plan objective, with a mean concentration of 306.9 mg/L and a median of 297.0 mg/L.  At Douglas Bridge, 7 of 10 (70%) samples exceeded the Basin Plan objective, with a mean concentration of 296.1 mg/L and a median of 306.0 mg/L.  At Benet Road, 8 of 10 (80%) samples exceeded the Basin Plan objective, with a mean concentration of 413.5 mg/L and a median of 346.0 mg/L.  See graph below for trend.

Chlorides may impart a salty taste to drinking water in concentrations between 100 – 700 mg/L.  The secondary drinking water standard for chlorides is 500 mg/L.  The measured values may be impairing the MUN beneficial use.

Elevated concentrations in waters used for industrial process and supply can significantly increase the corrosion rate of steel and aluminum.  The observed concentrations may be impairing the IND beneficial use.

High chloride concentrations can be toxic to plant life.  A safe concentration of chloride of irrigation waters is considered to be in the range of 100 – 140 mg/L.  Irrigation with water containing 140 – 350 mg/L of chloride may cause slight to moderate plant injury.1  The measured concentrations can be expected to impair the AGR beneficial use.  Damage to native flora could also impair the WARM, WILD and RARE beneficial uses.

Sulfate	Data collected in October 1997 to November 2000 by the City of Oceanside Water Utilities Laboratory2 showed 3 locations along the San Luis Rey River to exceed 250 mg/L.  Three locations in the City of Oceanside were sampled quarterly.  At Bonsall Bridge, 10 of 10 samples exceeded the Basin Plan objective, with a mean concentration of 426.2 mg/L and a median of 450.0 mg/L.  At Douglas Bridge, 9 of 9 samples exceeded the Basin Plan objective, with a mean concentration of 366.7 mg/L and a median of 361.9 mg/L.  At Benet Road, 10 of 10 samples exceeded the Basin Plan objective, with a mean concentration of 424.2 mg/L and a median of 420.0 mg/L.  See graph below for trend.

High concentrations of sulfate in drinking water can cause laxative effects1 and would impair the MUN beneficial use.

TDS	Data collected in October 1997 to November 2000 by the City of Oceanside Water Utilities Laboratory2 showed 3 locations along the San Luis Rey River to exceed 500 mg/L.  Three locations in the City of Oceanside were sampled quarterly.  At Bonsall Bridge, 11 of 11 samples exceeded the Basin Plan objective, with a mean concentration of 1612.5 mg/L and a median of 1680.0 mg/L.  At Douglas Bridge, 10 of 10 samples exceeded the Basin Plan objective, with a mean concentration of 1469.2 mg/L and a median of 1515.0 mg/L.  At Benet Road, 10 of 10 samples exceeded the Basin Plan objective, with a mean concentration of 1701.7 mg/L and a median of 1670.0 mg/L.  See graph below for trend.

Sampling by the Regional Water Quality Control Board, San Diego Region in May and June of 19983 also contain evidence of elevated concentrations of TDS.  One sample at Foussat Rd had a concentration of 850 mg/L and one sample at Old Highway 395 had a concentration of 970 mg/L.

Total Dissolved Solids may consist of carbonates, bicarbonates, chlorides, sulfates, phosphates, nitrates, magnesium, sodium, iron and manganese.  The most frequent constituents are usually salts (sodium, chloride, boron, etc.)  Most of the problem can be traced to human impacts, and therefore, can be mitigated. Geologic conditions help to define the natural levels of many of these constituents. High concentrations of TDS are expected to impact the AGR beneficial use directly through irrigation waters or indirectly through adverse effects on soil permeability.  TDS values between 450 to 2000 mg/L are expected to have a slight to moderate restriction on use of waters for irrigation of crops1.

### EXTENT OF IMPAIRMENT

Chloride	Sampling occurred at 3 locations on the San Luis Rey River: at Bonsall Bridge, at Douglas Bridge and at Benet Road.  All 3 locations are in or near the City of Oceanside, in the lower section of the river.  The entire lower extent of the river is impaired for chloride.

Sulfate	Sampling occurred at 3 locations on the San Luis Rey River: at Bonsall Bridge, at Douglas Bridge and at Benet Road.  All 3 locations are in or near the City of Oceanside, in the lower section of the river.  The entire lower extent of the river is impaired for sulfate.

TDS	Sampling occurred at 3 locations on the San Luis Rey River: at Bonsall Bridge, at Douglas Bridge and at Benet Road.  All 3 locations are in or near the City of Oceanside, in the lower section of the river.  Sampling also occurred at Foussat Rd and at Old Highway 395.  The entire lower extent of the river is impaired for TDS.

POTENTIAL SOURCES

### Chloride	Urban runoff, other point sources and non-point sources

#### Sulfate	Urban runoff, other point sources and non-point sources

TDS		Urban runoff, other point sources and non-point sources

TMDL PRIORITY

Chloride		Medium

Sulfate		Medium

TDS			Medium

### INFORMATION SOURCES

Water Quality Objectives and Watershed Characteristics

1 Water Quality Control Plan for the San Diego Basin (9), 1994.  California Regional

Water Quality Control Board, San Diego Region.

### Data Sources

2 Quarterly Monitoring Reports for the City of Oceanside. 1997- 2000.  City of

Oceanside, CA.

3 SDRWQCB In-House Monitoring.  1998.  California Regional Water Quality Control

Board, San Diego Region.