San Luis Rey River

903.11 at Foussat Rd

It is recommended that the Lower San Luis Rey River not be listed on 2002 303(d) list for nutrients or total dissolved solids.

Watershed Characteristics

The San Luis Rey River is located in the San Luis Rey Watershed in north end of San Diego County, California. The San Luis Rey River originates from Lake Henshaw and runs parallel to Highway 76 in the lower segment all the way to the pacific coastline.  The area examined for this summary was described as where Foussat Rd. crosses the river.

The San Luis Rey River is designated an Inland Surface Water.  It is used for municipal and domestic, agricultural, and industrial water supply, contact and non-contact recreation, warm freshwater habitat, wildlife habitat, and rare, threatened, and endangered species.

# Water Quality Objectives not Obtained

On May 20, 1998, the San Luis Rey River at Foussat Rd. was not in compliance for the following water quality parameters: Total Phosphorus and Total Dissolved Solids (TDS).

# Evidence of Impairment

The allowed upper limit for these constituents along with the day, site and exceeded value for each constituent are given in the attached tables.

# Extent of Impairment

The data available for San Luis Rey River at Foussat Rd. shows only one sampling date, May 20, 1998.  Other sampling occurred on June 11, 1998 upstream at the Old Highway 395 crossing of the San Luis Rey River (Pardy Table 1).  The river is also sampled quarterly at Bonsall Bridge, Douglas Bridge, and Benet Road by the City of Oceanside. All these sites are located in Oceanside except at Old Highway 395, which is in Bonsall (Gonzales 1997-2000). Benet Road is located roughly 6 miles south of the Douglas Bridge and directly south of the Oceanside Municipal Airport, about 5 miles east of the ocean. The entire stretch of the river within this area is of concern.

# Potential Sources

The San Luis Rey Watershed Guidelines indicates that land use of the watershed is 50% undeveloped, 37% agriculture, 5% Residential, 5% Parks and Recreation, Water 2% and Industrial, Transportation, Commercial and Military comprising the remaining 1% (San Luis Rey Watershed Guidelines).

TDS consists of manganese, iron, bicarbonate, chloride, sulfate, and other substances.  All of these listed components were found to be in violation of the water quality standards at Foussat Rd and at other stations along the San Luis Rey River.  Several sand mining operations conduct business in and around the San Luis Rey River and could be possible source for the TDS exceedence.  Other potential sources are the input from agricultural activities, recreation associated with the adjacent parks and golf courses, urban runoff, and light industry.

Total phosphate was another exceedence noted at Foussat Rd. Although the exceedence was over the Basin Plan concentration of 0.05 mg/l, it met the Nitrogen:Phosphate ratio of 10:1

Phosphate is often associated with fertilizer used in agriculture.  Since agriculture is the predominant use in the San Luis Rey watershed, it is the most likely source in the river.  Other sources include urban runoff, recreation associated with the golf courses and parks located in proximity of the river channel, and light industry.

# TMDL Priority

Although TDS and Total Phosphate exceeded Basin Plan water quality parameters at the time of sampling and a similar trend was observed at other locations along the river, it is not a high priority at this time.  Supporting information comes from a preliminary bioassessment study that was conducted on in San Diego County in 1998 and 1999.  In the report titled “San Diego Regional Water Quality Control Board: 1999 Biological Assessment Report.”  In this preliminary study, it was noted that the San Luis Rey watershed is a potential reference site for San Diego County.

It is recommended that this stretch of the San Luis Rey River not have a 303(d) listing and be considered a low priority, but continued water quality monitoring and bioassessments is still warranted.

# Source References

All water quality standards were taken from the Water Quality Control Plan for the San Diego Basin.

San Luis Rey River Watershed Guidelines, 2000

Linda Pardy Table 1

Harrington, James. “San Diego Regional Water Quality Control Board: 1999 Biological Assessment Report.” California Department Fish and Game. 1999.

Gonzalez, Mary.  Quarterly Monitoring Reports for the City of Oceanside. 1997-2000.