San Diego River in Santee (907.110) – 303(d) Fact Sheet

San Diego Baykeeper - EnvironMatrix

This data does not lead to a listing recommendation.

## Watershed Characteristics

The Lower San Diego River is a 6.0-mile waterway in the San Diego River Watershed of Region 9.  It is classified inland surface water with the following beneficial uses: MUN, AGR, IND, PROC, REC1, REC2, WARM, COLD and WILD1.

# Water Quality Objectives not Obtained

TDS	The Basin Plan objective is 1000 mg/L.

# Evidence of Impairment

Sampling occurred at 4 locations along the San Diego River over 2 days. Baykeeper claims that total coliform, fecal coliform and total dissolved solids (TDS) were in excess of Basin Plan Standards1.

Total Coliform	Objectives for total coliform concentrations apply only where shellfish harvesting for human consumption is designated.

Fecal Coliform	With only two samples, neither Basin Plan Objective for fecal coliform can be directly applied.  However, the objective of 400 coliform forming units (CFU) / 100 mL can be used as a guide to the severity of the contamination.  The two samples did not exceed this objective.

## TDS	All 3 samples were above the Basin Plan Objective of 1000 mg/L.  Average and median values are not applied to samples from different sampling locations.  Three samples are not enough to warrant 303(d) listing.

See Table 1 for a summary of their results and Basin Plan Standards1.

# Extent of Impairment

Sampling occurred at Friars Rd YMCA (furthest west location), Mission Valley Golf Course, Forrester Creek at Trolley Station and at Mission Dam at Mission Trails.

# Potential Sources

# Unknown

# TMDL Priority

No TMDL is required at this time.

# Information Sources

1 Water Quality Control Plan for the San Diego Basin (9), 1994